CCNP Exam objectives answers:

Before you start reading this document you should be aware of the following things. This document only covers the FCS test, there are currently not any exam objectives published for the CIT exam. The objectives that appear in the published document are simply regurgitated marketology from some non-technical person at Cisco. Many of these exam objectives have no relevancy to the test, or even worse have any relevancy to the day to day operation of the cisco products. I have prepared another guide which is the FCSNOTES.doc which is what I would recommend you use to study from. This document will probably never be complete as it covers almost every concept in internetworking in some way. This does not mean that you should completely ignore this document, I would just not try to memorize every fact from it, there is just too much here.

-Brian Horakh

CiscoTests.com

- Describe the major features of the catalyst switches

Catalyst 5000 features:

Modular chassis 2, 5, 13 slot model

Supervisor engine: hardware based switching backplane with tri level priority, bridge address table for 16000 Mac addresses, network management processor, two 100mbps fast etherlink uplinks.

Switching modules with choice of interface options.

AC or DC power supply (Catalyst 5000/5500/5505 option) [secondary optional, hot swappable], dual power cord availability

NetFlow switching provides layer 2 and layer 3 support

Route-switch module multiprotocol layer 3 routing

Media-independent backplane, Ethernet, TokenRing, FDDI, ATM

Three levels of priority on the switching backplane, accommodates technologies that use priorities such as ATM, Token Ring, enables user to define priorities on a per interface basis, ensures server traffic priority, accommodates bursty traffic, prevents higher layer protocols from timing out by giving the interface the highest possible priority on the backplane until the buffer content is reduced.

Switching backplane: 1.2gbs on 5000, 5005 > 1million pps, 3.6gb on 5500, 5505

Memory: 4mb flash (supervisor engine 1), 8mb flash (supervisor engine II), 8mb+PC card (supervisor III), 16mb dram (supervisor II), 4mb or 20mb (supervisor 1), 128kb NVRAM (1), 256kb NVRAM (2, 3)

The Catalyst 2900 Series switch supports the following standard features:

14-port fixed configuration chassis, which includes a single power supply (2901 and 2902)

26-port fixed configuration chassis, which includes two power supplies (2926T/F)

Low-latency 1.2-Gbps switching backplane with tri-level priority

Handles up to 16,000 active MAC addresss

VLANs in the bridge lookup table

25-MHz 68EC040 network management processor

Two Fast Ethernet interfaces (full or half duplex), which can be Inter Switch Link (ISL) trunks

Two Fast Ethernet interfaces (full or half duplex), which support Fast Ether Channel (FEC) (2926T/F)

Fully integrated support for 1024 VLANs

VLAN ISL support

Virtual Trunking Protocol (VTP) for distributing VLAN configuration information

Dynamic ISL Protocol providing SNMP instrumentation for VTP

Embedded RMON, which provides the following four RMON groups: statistics, history, alarms, events, and Switched port analyzer functionality, which allows monitoring of a single port or VLAN via a single monitor port. Segment traffic is "mirrored" to a user-designated port.

Load sharing via spanning-tree priority on a per-VLAN basis on ISL trunks between Catalyst 2900 and Catalyst 5000s. Multiple ports can be used to achieve 800 Mbps and higher performance.

Cisco Group Management Protocol (CGMP), which enables efficient distribution of layer 3 multicasts

10/100 Fast Ethernet interface with autosensing speed detection for easy upgrade

- Describe the architecture and major components of the Catalyst switches

Check your Cisco product guide,

You need to know how many slots in a 7513 chassis (13)

You need to know that you need a supervisor engine, which is covered later.

You neeed to know about the various ASIC’s and processors (also covered later)

- Place a catalyst switch in a network for optimal performance benefit

The switch should be placed where it would be best used, take into consideration all the facts, you want to reducing broadcast traffic, give clients faster access to their respective servers, etc.

4. Use the command-line or menu-driven interface to configure the catalyst series switches and they’re switching modules.

You should already know how to use the IOS because your CCNA exam.

- Use the command-line or menu-driven interface to configure trunks, virtual LANs, and ATM LAN Emulation

6. Describe the advantages of LAN Segmentation

LAN segmentation keeps local traffic from going out on the WAN or backbone. Broadcast messages do not leave a local LAN segment. LAN segmentation using routers provides a method for grouping computers based on logical address.

7. Describe LAN Segmentation using bridges

The simplest form of segmentation, uses layer 2 of the OSI model. A bridge learns which network a device is located on based on the incoming port and source address of a packet. Once a bridge has learned which port a MAC address is located on then only traffic that is destined for the remote address is passed through the bridge (and then only to the specific port which the device is on). In addition all broadcast traffic, and traffic to known destinations is passed to all ports, when a packet goes to all ports it is referred to as flooding.

8. Describe LAN Segmentation using routers

Routers use Layer 3 (Network Layer) to filter traffic. Traffic is only passed through a router when the router knows that is has the best (or only) route to a remote destination. It has the opposite rule of bridging, so if the destination is not known then the router discards the packet.

9. Describe LAN Segmentation using switches

Switches are primarily the same as bridges. Generally speaking switches will be more equipped for dealing in LAN to LAN segments than bridges.  Switches provides the same services as bridges except at Wire Speed.

10. Name and describe two switching methods

(There are actually three)

Store-and-forward is the traditional mode of operation for a bridge. In store-and-forward mode, the port adapter reads the entire frame into memory and then determines if the frame should be forwarded. At this point, the frame is examined for any errors (frames with errors are not forwarded). If the frame contains no

Errors, it is sent to the destination port for forwarding. While store-and-forward mode reduces the amount of error traffic on the LAN, it also causes a delay in frame forwarding that is dependent upon the length of the frame.

Cut-through mode is a faster mode of forwarding frames. In cut-through mode, the switch transfers

Non broadcast packets between ports without buffering the entire frame into memory. In cut-through mode the switch receives the first few bytes of a frame, it analyzes the packet header to determine the destination of the frame, establishes a connection between the input and output ports, and, when the token becomes available, it transmits the frame onto the destination ring.

In certain circumstances, however, the cut-through mode cannot be applied and the switch must buffer frames into memory. For example, buffering must be performed in the following circumstances:

The switch has more than one packet to transmit to the same network.

A packet is switched between 4- and 16-Mbps rings (token ring) or 10/100mb Ethernet.

The destination ring is beaconing (token ring).

Adaptive cut-through mode uses a combination of store-and-forward and cut-through modes. Adaptive cut-through mode, the user can configure the switch to automatically use the best forwarding mode based on user-defined thresholds. In adaptive cut-through mode, the ports operate in cut-through mode unless the number of forwarded frames that contain errors exceeds a specified percentage. When this percentage is exceeded, the switch automatically changes the mode of the port to store-and-forward. Then, once the

Number of frames containing errors falls below a specified percentage, the operation mode of the ports is once again set to cut through.

11. Describe full –and—half-duplex Ethernet operation

Half Duplex is the most common network. Half Duplex means that a station can be either receiving or sending at the same time. Full duplex means that a station can be both sending and receiving at the same time. Full duplex requires that no other computers can be sharing the media, Full duplex typically is used when a host connects directly to a switch. On the test Cisco references half duplex to being like a narrow bridge in which only one car can be going any direction at the same time.

12. Describe token ring switching concepts

The traditional method of connecting multiple Token Ring segments is to use a source-routing bridge (SRB). For example, bridges are often used to link workgroup rings to the backbone ring. However, the introduction of the bridge can significantly reduce performance at the user's workstation. Further problems may be introduced by aggregate traffic loading on the backbone ring. To maintain performance and avoid overloading the backbone ring, you can locate servers on the same ring as the workgroup that needs to access the server. However, dispersing the servers throughout the network makes them more difficult to back up, administer, and secure than if they are located on the backbone ring. Dispersing the servers also limits the number of servers that particular stations can access.

Collapsed backbone routers may offer greater throughput than bridges, and can interconnect a larger number of rings without becoming overloaded. Routers provide both bridging and routing functions between rings and have sophisticated broadcast control mechanisms. These mechanisms become increasingly important as the number of devices on the network increases.

The main drawback of using routers as the campus backbone is the relatively high price per port and the fact that the throughput typically does not increase as ports are added. A Token Ring switch is designed to provide wire speed throughput regardless of the number of ports in the switch. In addition, the Catalyst 3900 Token Ring switch can be configured to provide very low latency between Token Ring ports by using cut-through switching.

As a local collapsed backbone device, a Token Ring switch offers a lower per-port cost and can incur lower interstation latency than a router. In addition, the switch can be used to directly attach large numbers of clients or servers, thereby replacing concentrators. Typically, a Token Ring switch is used in conjunction with a router, providing a high-capacity interconnection between Token Ring segments while retaining the broadcast control and wide-area connectivity provided by the router.

History of Token Ring Switching

The term switching was originally used to describe packet-switch technologies such as Link Access Procedure, Balanced (LAPB), Frame Relay, Switched Multimegabit Data Service (SMDS), and X.25. Today, LAN switching refers to a technology that is similar to a bridge in many ways.

Like bridges, switches connect LAN segments and use information contained in the frame to determine the segment to which a datagram needs to be transmitted. Switches, however, operate at much higher speeds than bridges, and can support new functionality, such as virtual LANs (VLANs).

Token Ring switches first appeared in 1994. The first-generation Token Ring switches can be divided into two basic categories:

Processor-based switches: These switches use reduced instruction set computer (RISC) processors to switch Token Ring frames. Although they typically have a lot of function, they are slow and relatively expensive. These switches have been deployed mainly as backbone switches because of their high cost.

Application-specific integrated circuit (ASIC)-based switches with limited functionality:

These switches are fast and relatively inexpensive, but have very limited function. Typically, they offer little to no filtering, limited management information, limited support for bridging modes, limited VLANs. Today, although these switches are less expensive than processor-based switches, they are still too expensive and limited for widespread use of dedicated Token Ring to the desktop.

In 1997, a second generation of Token Ring switches was introduced. Cisco's second-generation Token Ring switches use ASIC-based switching, but they provide increased functionality resulting in a higher speed and lower cost. They also provide a wider variety of function than their predecessors, including support for multiple bridging modes, Dedicated Token Ring (DTR) on all ports, high port density, high-speed links, filtering, Remote Monitoring (RMON) management, broadcast control, and flexible VLANs.

The family of second-generation Token Ring switches can be used for backbone switching, workgroup microsegmentation, and dedicated Token Ring to the desktop. Token Ring switches currently being offered include:

The Catalyst 3900, which is a stackable workgroup switch that provides support for all switching modes, filtering, RMON, DTR, and SNMP management, as well as support for Asynchronous Transmission Mode (ATM) and Inter-Switch Link (ISL).

The Catalyst 5000, which is a modular switch that supports Ethernet, Fast Ethernet, Fiber Distributed Data Interface (FDDI), ATM, and now Token Ring.

13. Name seven reasons to create VLANS

Broadcast control--Just as switches isolate collision domains for attached hosts and only forward appropriate traffic out a particular port, VLANs refine this concept further and provide complete isolation between VLANs. A VLAN is a bridging domain, and all broadcast and multicast traffic is contained within it.

Security--VLANs provide security in two ways: High-security users can be grouped into a VLAN, possibly on the same physical segment, and no users outside of that VLAN can communicate with them.

Because VLANs are logical groups that behave like physically separate entities, inter-VLAN communication is achieved through a router. When inter-VLAN communication occurs through a router, all the security and filtering functionality that routers traditionally provide can be used because routers are able to look at OSI Layer 3 information. In the case of nonroutable protocols, there can be no inter-VLAN communication. All communication must occur within the same VLAN.

Performance--The logical grouping of users allows, for example, an engineer making intensive use of a networked CAD/CAM station or testing a multicast application to be assigned to a VLAN that contains just that engineer and the servers he or she needs. The engineer's work does not affect the rest of the engineering group, which results in improved performance for the engineer (by being on a dedicated "LAN") and improved performance for the rest of the engineering group (whose communications are not slowed down by the engineer's use of the network).

Network management--The logical grouping of users, divorced from their physical or geographic locations, allows easier network management. It is no longer necessary to pull cables to move a user from one network to another. Adds, moves, and changes are achieved by configuring a port into the appropriate VLAN. Expensive, time-consuming recabling to extend connectivity in a switched LAN environment is no longer necessary because network management can be used to logically assign a user from one VLAN to another.

Scalability lets you get past the standard flat network using hubs and routers.

Because it sells Cisco products (anybody got any other reasons)??

14. Describe the role that switches play in the creation of VLANS

Switches are one of the core components of VLAN communications. They are the entry point for end-station devices into the switched fabric and for communication across the enterprise. Switches provide the intelligence to group users, ports, or logical addresses into common communities of interest. Each switch has the intelligence to make filtering and forwarding decisions by packet, based on VLAN metrics defined by network managers, and to communicate this information to other switches and routers within the network. And while today LAN switches are installed between shared segment hubs and routers located within the backbone, they will take on a larger, more significant role for VLAN segmentation and low-latency forwarding as they are deployed in the wiring closet. LAN switches offer significant increases in performance and dedicated bandwidth across the network, with the intelligence necessary for VLAN segmentation.

15. Describe VLAN frame filtering and frame tagging

(also VLAN Colors)

VLAN Frame Filtering:

The most common approaches for logically grouping users into administratively-defined VLANs include packet filtering and packet identification. Packet filtering is a technique that examines particular information about each packet based on user-defined offsets. Packet identification (tagging)

uniquely assigns a user-defined ID to each packet. Both of these techniques examine the packet when it is either received or forwarded by the switch. Based on the set of rules defined by the administrator, these techniques determine where the packet is to be sent, filtered, and/or broadcast. These control mechanisms can be centrally administered (with network management software) yet are easily deployed throughout the

network. The concepts of packet filtering are very similar to those commonly used for routers. A filtering table is developed for each switch. This provides a high level of administrative control, because it can examine many attributes of each packet. Network managers can group users based upon MAC station addresses, network layer protocol types, and/or application types. Table entries are compared with the packets filtered by the switch. The switch takes the appropriate action based on the entries.

Packet filtering typically provides an additional layer of switch processing prior to forwarding each packet to another port or switch within the network, and it becomes more apparent as you filter deeper into each packet. This additional processing can effect switch latency and overall network performance. In addition, maintaining address tables adds an extra layer of administration per switch and requires synchronizing tables between switches.

VLAN Frame Tagging:

VLAN switching is accomplished through frame tagging where traffic originating and contained within a particular virtual topology carries a unique VLAN identifier (VLAN ID) as it traverses a common backbone or trunk link. The VLAN ID enables VLAN switching devices to make intelligent forwarding decisions based on the embedded VLAN ID. Each VLAN is differentiated by a color, or VLAN identifier. The unique VLAN ID determines the frame coloring for the VLAN. Packets originating and contained within a particular VLAN carry the identifier that uniquely defines that VLAN (by the VLAN ID).

The VLAN ID allows VLAN switches and routers to selectively forward packets to ports with the same VLAN ID. The switch that receives the frame from the source station inserts the VLAN ID and the packet is switched onto the shared backbone network. When the frame exits the switched LAN, a switch strips header and forwards the frame to interfaces that match the VLAN color. If you are using a Cisco network

management product such as VlanDirector, you can actually color code the VLANs and monitor VLAN graphically.

VLAN Colors:

VLAN switching is accomplished through frame tagging where traffic originating and contained within a particular virtual topology carries a unique VLAN identifier (VLAN ID) as it traverses a common backbone or trunk link. The VLAN ID enables VLAN switching devices to make intelligent forwarding decisions based on the embedded VLAN ID. Each VLAN is differentiated by a color, or VLAN identifier. The unique VLAN ID determines the frame coloring for the VLAN. Packets originating and contained within a particular VLAN carry the identifier that uniquely defines that VLAN (by the VLAN ID).

The VLAN ID allows VLAN switches and routers to selectively forward packets to ports with the same VLAN ID. The switch that receives the frame from the source station inserts the VLAN ID and the packet is switched onto the shared backbone network. When the frame exits the switched LAN, a switch strips header and forwards the frame to interfaces that match the VLAN color. If you are using a Cisco network

management product such as VlanDirector, you can actually color code the VLANs and monitor VLAN graphically.

VLAN packet identification (packet tagging) is a relatively new approach that has been specifically developed for switched communications. This approach places a unique identifier in the header of each packet as it is forwarded throughout the switch fabric. The identifier is understood and examined by each switch prior to any broadcasts or transmissions to other switches, routers, or end-station devices. When the packet exits the switch fabric, the switch removes the identifier before the packet is transmitted to the target end station. Over the past two years, packet identification has gained acceptance as switches have increased in popularity; packet identification functions at layer 2 and requires little processing or administrative overhead

The overall benefits of both approaches (packet filtering and packet identification) allow VLAN architectures that are nonintrusive to end-node applications and communication protocols. Switches provide all of the filtering, identification, and forwarding without any modification to the attached end station devices. This delivers a VLAN architecture that easily integrates with existing LAN applications while offering scalability and migration to ATM networks.

16. Describe how switches can be used with hubs

A Cisco Ethernet LAN switch can be especially effective when used as a switch of hubs. The need to accommodate network growth has forced network managers to connect several hubs in daisy-chain fashion in order to accommodate more devices. The result of this configuration is that all devices share a single 10-Mbps data path, reducing overall network efficiency.

Network growth can be accommodated more easily with a Cisco Ethernet LAN switch. For example, a Catalyst 3100 switch can be deployed in the wiring center to link multiple Ethernet hubs and significantly increase network throughput.

17. Name the five components of VLAN implementation

I have not been able to find the answer to this question, and have not seen any questions on the test which reference a count of 5, so we’ll just ignore this one.

18. Describe static and dynamic VLANs

Static VLANs are the most common type of VLAN. A dynamic VLAN will reconfigure itself based on the MAC address which is connected to it (obviously this only works if you have one mac address per switch port)

19. Describe the VLAN technologies

vLANs allow logical network topologies to overlay the physical switched infrastructure such that any

arbitrary collection of LAN ports can be combined into an autonomous user group or community of interest. VLAN’s logically segment the network into separate layer 2 "broadcast domains" whereby packets are then only switched between ports designated to be within the same virtual LAN. This offers significant benefits in terms of efficient use of bandwidth, flexibility, performance and security. With LAN Switching, this is achieved via a technique known as "frame-tagging" where traffic originating and contained within a particular virtual topology carries a vLAN ID as it traverses a common backbone or trunk link and this then enables vLAN Switching devices to make intelligent forwarding decisions based upon the embedded vLAN ID.

In campus ATM Switching, LAN Emulation is used to define virtual LAN networks. Using LANE, the ATM fabric emulates the broadcast-capable, connectionless characteristics of local area networks and users are combined together to form virtual LANs by all connecting to the same emulated LAN within the ATM backbone. Cisco Systems offers the industry's most comprehensive vLAN capabilities across its strategic product family and supports multiple vLAN tagging protocols optimized for different media

Cisco's Inter Switch Link (ISL) for 100BaseT Fast Ethernet (Gigabit Ethernet planned)

IEEE 802.10 Standard for FDDI backbones and via LAN Emulation over ATM.

802.1Q Virtual LANs Standard

By definition vLANs perform network partitioning and traffic separation at Layer 2. Communications between different virtual LANs requires a Layer 3 routing or a Layer 2 translation function. IOS has full support for routing and translation between two vLANs vLANs, or between vLAN trunking and non-vLAN interfaces/

IOS supports full routing of IP, Novell IPX, Appletalk, DECnet, Vines and XNS protocols over ISL and ATM LANE virtual LANs, as well as IP, Novell IPX and Appletalk routing of IEEE 802.10 based vLANs. A switched vLAN domain corresponds to a routed subnet/network number and is represented in the Cisco IOS(tm) via a vLAN subinterface. Standard routing attributes such as network advertisements, secondary addresses, helper addresses etc. are applicable and vLAN routing is Fast Switched.

Virtual LAN translation happens between different Virtual LANs (which use the same or different vLAN protocols), or between vLAN and non-vLAN encapsulating interfaces at layer 2. This functionality is typically used for selective inter-vLAN switching of non-routable protocols and to extend a single vLAN topology across hybrid switching environments. It is also possible to simply bridge ISL and IEEE 802.10 vLANs (i.e. where the vLAN encapsulating header is preserved) on the main/first interface.

The Cisco IOS(tm) runs an IEEE 802.1 Spanning Tree per layer 2 vLAN to enhance scalability and prevent topology changes in one switched vLAN domain from impacting a different virtual LAN. VLAN translation was introduced in Cisco IOS(tm) release 11.1 and is Fast Switched.

The Hot Standby Router Protocol (HSRP) is a Cisco proprietary protocol which provides excellent fault tolerance and enhanced routing performance for IP networks. HSRP allows routers to monitor each other's operational status and very quickly assume packet forwarding responsibility should the current forwarder in the HSRP group fail or be taken down for maintenance. This mechanism remains transparent to the attached hosts and can be deployed on any LAN type. With Multi-Group Hot Standby, routers can simultaneously provide redundant backup and perform load-sharing across different IP subnets.

Cisco IOS(tm) vLAN routers are able to take advantage of mHSRP for vLAN subnets defined via IEEE 802.10 on FDDI and ATM LAN Emulation for Ethernet starting from software release 11.2. With Cisco IOS 11.3.

Where a separate HSRP group is configured for each vLAN subnet so that Cisco IOS(tm) router A could be the primary/forwarder for virtual LANs 10 and 20 shown, while also acting as backup for vLANs 30 and 40. Conversely routerB acts as the primary/forwarder for ISL virtual LANs 30 and 40 as well as secondary/backup for distributed vLAN subnets 10 and 20.

Distributed Switching:

In order to meet anticipated performance and scalability requirements for inter-vLAN routing, Cisco has implemented distributed switching of ISL vLAN encapsulated IP on the C7500 platform in Cisco IOS 11.3 (future software releases will add additional vLAN and Layer 3 protocol support). Thus in typical "router-on-a-stick" scenarios, where an aggregation route performs inter-vLAN routing for multiple vLAN switches, traffic between virtual LANs can be switched autonomously on-card or between cards rather than via the central Route Switch Processor (RSP).

This architecture allows incremental capacity increases by simply adding additional Versatile Interface Processor (VIP) cards and with the C7513 accommodating up to twelve distributed switching VIPs, the platform can scale to over one million inter-vLAN routed packets per second. In multi-protocol environments, having the majority IP vLAN traffic switched within the VIP cards (NB. the default mode with Cisco IOS(tm) 11.3) also significantly increases routing performance for the other protocols since the RSP off-loads IP and can then be dedicated to switching the non-IP protocols.

IEEE 802.1Q based vLANs as are currently available with ISL, IEEE 802.10 and LAN Emulation based virtual LANs.

21. Describe Cisco’s VLAN architecture

VLANs provide the following features:

- Simplification of moves, adds, and changes

- Workgroup and network security

- Controlled broadcast activity

- Centralized administration control

VLANs can be used to provide security firewalls, restrict individual user access, flag any unwanted intrusion to the network, and control the size and composition of the broadcast domain.

You can increase security by segmenting the network into distinct broadcast groups. This enables you to

- Restrict the number of users in a VLAN.

- Prevent another user from joining a VLAN without first receiving approval from the VLAN network management application.

- Configure all unused ports to a default low-service VLAN.

22. Describe demand nodes and resource nodes

Demand nodes are workstations, resource nodes are servers.

23. Describe configuration rules for demand nodes and resource nodes

Put your servers on the backbone, put your workstations on segments where they will be close to the servers they use the most. (duh)

24. Describe local resources and remote resources

Local resources are printers, remote resources are things that you access that don’t appear on the local LAN (duh again).

25. Describe configuration rules for local resources and remote resources

You should provide a quick and easy way to get to both. Don’t worry the only questions on the test regarding this stuff is really obvious.

26. Name five applications for Catalyst 5000 series switches

I can think of more than 5, but you should know the applications on Catalyst 5000 series switches.

27. Catalyst 5000 series switch product evolution

Sounds like Marketology, No Clue.

28. Catalyst 5002 product features

See Question #1

29. Catalyst 5500 product features

The only major thing you need to know here is that the 5500 has a ATM Switch processor, along with interface modules and port adaptors.

30. Describe the architecture and function of the major components of a cisco 5000 series switch:

See #31, and #32

31. Processors: NMP, MCP, LCP

NMP - 25-MHz Motorola MC68EC040 Network Management Processor (NMP)

MCP

7000 series processors: http://www.cisco.com/warp/public/733/ip/index.shtml

LCP

- Logic Units: LTL, CBL, Arbiter, and EARL

EARL: Enhanced Address Recognition Logic

LTL: Local Target Logic

CBL: Color Blocking Logic

Arbiter Overview

The arbiter is a printed circuit board that is mounted to the front (non-IP side) of the backplane. The arbiter arbitrates traffic across the CxBus by prioritizing access requests from IPs to ensure that each request is processed. The arbiter provides the

following services for the system: CxBus clock generation--The arbiter generates the 16.667 MHz clock and provides a private copy of the clock to the

SP and each IP.

CxBus arbitration--The arbiter arbitrates IP requests to transmit commands on the CxBus; arbitration is based on a  round-robin priority scheme to ensure that all IPs have access to a known portion of the CxBus bandwidth.

Global lock arbitration--The arbiter arbitrates IP and SP requests for the global lock, a synchronization primitive used to control SP and IP access to shared data structures.

- ASICS: SAINT, SAGE, SAMBA, and PHOENIX

SAINT & SAGE: The Catalyst architecture has three basic blocks: the Bus Arbitration and Encoded Address Recognition Logic, the Port Interface, and the Network Management Processor. The Bus Arbitration and Encoded Address Recognition Logic are shared among all ports. Together they govern access to the data switching bus and control the destination of packet transfers. Each Ethernet Port Interface comprises a custom ASIC, called the SAINT, with an integrated 10/100 Ethernet MAC controller. Other media ports make use of a second custom ASIC, called the SAGE, without an integrated MAC. To maximize current Cisco Internetwork Operating System(TM) (Cisco IOS(TM)) software investments, the Catalyst 5000 uses an integrated Motorola 68040 family processor.

- Trace a frame’s progress through a catalyst 5000 series switch

- Describe the hardware features, functions and benefits of Catalyst 5000 series switches

- Describe the hardware features and functions of the Supervisor engine

- Describe the hardware features and functions of the modules in the catalyst 5000 series switches

- Establish a serial connection

- Use the catalyst 5000 CLI to:

- Enter Privileged Mode

After you log in successfully, the system automatically enters normal mode, which gives you access to normal-mode commands only. You can enter privileged mode by issuing the enable command followed by a second password. Privileged mode is indicated by (enable) immediately after the system prompt. To return to normal mode, enter the disable command at the prompt.

- Set system information

Although not required, several system parameters should be set as part of the initial system setup. To set the system parameters, perform the following tasks in privileged mode:

set system name [name_string]

set time [day_of_week] [mm/dd/yy] [hh:mm:ss]

set prompt prompt_string

set password

set enablepass

set system contact [contact_string]

set system location [location_string]

to verify do a “show system”

42. Configure interface types

To set the interface type, perform the following steps in privileged mode:

| Task | Command |
| --- | --- |
| If you are using a local network connection to the console port, set the logical port sc0. Assign the Catalyst 5000 IP address to a VLAN. | set interface sc0 up set interface sc0 ip_address [netmask [broadcast]] set interface sc0 vlan_num ip_address |
| If you are using a SLIP connection to the console port, set the slip port sl0. Figure 3-2 for an example. | set interface sl0 up set interface slip_address dest_address |
| Configure static routes. For example, you need to configure static routes if your Telnet station or SNMP network management workstation is on a different network from the switch. | set ip route destination gateway [metric] |
| Configure a default route, if desired. See Figure 3-2 for an example. | set ip route destination gateway metric |
| Check the status of the configuration of the switch. See Figure 3-3 for an example. | show interface |
| Display the route table entries of the configuration. See Figure 3-4 for an example. | show ip route |

43. Prepare network connections

This means you need to know how to identify the connector type.

- Features in the catalyst series software release 2.2

Supervisor Resilience

The Catalyst 5500 series switch offers the option of using two Supervisor Engine modules to provide fault tolerance in mission-critical environments and to ensure switching resilience.

Note The Supervisor Resilience feature is available on Supervisor Engine II only.

Broadcast/Multicast Suppression

The Broadcast/Multicast Suppression feature provides a means of limiting broadcast from any port and can prevent a switched port from being disrupted by a broadcast storm.

Cisco Group Management Protocol (CGMP)

CGMP manages multicast traffic in Catalyst 5000 series switches by allowing directed switching of IP multicast traffic within a network at rates greater than one million packets per second.

Secure Port Filtering

The Secure Port Filtering feature allows Ethernet and Fast Ethernet switching modules to block input to an Ethernet or Fast Ethernet port when the Media Access Control (MAC) address of a station attempting to access the port is different from the configured or learned MAC address.

Terminal Access Controller Access Control System Plus (TACACS+) Protocol

The TACACS+ Protocol controls access to the Supervisor Engine module and determines the identity of a user by verifying a username with a password.

System Message Logs

The System Message Log feature allows you to redirect system messages from the Supervisor Engine module, as well as output from asynchronous events such as interface transition, to other destinations, such as a Trivial File Transfer Protocol (TFTP) server.

Network Time Protocol

Network Time Protocol (NTP) time-synchronizes Catalyst 5000 series switches in a network by downloading the system time to all users.

Domain Name System

Domain Name System (DNS) allows the resolution of host names through the DNS protocol from a DNS server.

Login Banner

The Login Banner feature allows you to create a single-line or multiline message banner that appears on your screen before session login.

Multiple Module Download

Multiple module download allows you to perform a single TFTP download that updates all modules of the same type on  a Catalyst 5000 series switch.

New Hardware Supported

Software Release 2.2(1) supports the following hardware features:

Supervisor Engine module II

Catalyst 5002 switch

Catalyst 5500 switch

WS-X5223 Group Switching Fast Ethernet switching module (100BaseTX, 24 Port)

WS-X5213A 10/100-Mbps Fast Ethernet switching module (10/100BaseTX, 12 Port)

- Features in the catalyst series software release 2.3

- Fast EtherChannel: Allows two to four Fast Ethernet links to be grouped together to enable higher bandwidth channels for improved network performance.

- Dynamic VLAN Assignment

Allows the Catalyst 5000 series switch to assign ports to a VLAN dynamically, based on the media-access control (MAC) address of the host connected to that port.

- Virtual Management Policy Server (VMPS)

Allows network administrators to define the VLAN membership policies for their network in a central database so that the Catalyst 5000 series switch fabric automatically configures user ports to the correct VLAN.

- VLAN Trunk Protocol (VTP) Pruning

Optimizes the utilization of network trunk bandwidth by reducing unnecessary broadcast, multicast, and flooded-unicast traffic. Also allows the Catalyst 5000 series switch fabric to prune the VLAN forwarding tree automatically in order to contain traffic from each defined VLAN to only those switches that require the traffic.

- Cisco Group Management Protocol (CGMP) Fast Leave Processing

Detects leave messages from Internet Group Management Protocol (IGMP) version 2 hosts and immediately prunes ports from the specified multicast forwarding tree to reduce processing latency for nodes leaving a multicast group.

- IP Permit List

Allows network administrators to create a list of IP addresses or subnets that are permitted to access the system console via Telnet and Simple Network Management Protocol (SNMP) agent services.

- Remote Monitoring (RMON) Agent Enhancements

The RMON agent has been enhanced to automatically restore the last configuration of the embedded RMON agent for each port after a system reset. A further enhancement sends all RMON traps to all trap hosts configured in the Catalyst 5000 console trap-receiver table.

- Support for New Hardware Modules

Software release 2.3 provides support for the following new Catalyst 5000 series modules:

- Ethernet Switching Module (10BaseT 48 Port)--WS-X5012

- Fast EtherChannel Switching Module (100BaseFX 12 Port)--WS-X5201

- 10/100-Mbps Fast EtherChannel Switching Module (10/100BaseTX 12 Port)--WS-X5203

- Route Switch Module (RSM) (Catalyst 5000 and Catalyst 5500 switches only)--WS-X5302

The RSM is a RSP2-class router module for the Catalyst 5000 series switches. The RSM provides high-performance, multilayer switching and routing services between switched virtual LANs (VLANs) and emulated LANs (ELANs). Initial protocol support includes IP, IPX, AppleTalk, and DECnet. For more information, refer to the Catalyst 5000 Series Switch Route Switch Module Installation and Configuration Note (documentation part number 78-4058-01).

- Virtual LANs

VLANs are an effective tool for maintaining throughput and controlling broadcasts in large multilayer switched networks. Cisco's VTP substantially reduces VLAN setup and administration across multi-LAN networks by automating the setup and distribution of each VLAN across the network. VTP eliminates configuration errors and the potentially complex diagnostics required to track these errors. Moreover, VTP substantially decreases the time required by network managers to add new VLANs and reduces the complexity of mapping between different technologies.

- VLAN Trunking

VLAN trunking is basically any connection between two switches or a switch and router which is designated as a VLAN trunk. Read the next objective for more info.

- VLAN trunk protocol

The VTP maintains VLAN configuration consistency throughout the network. VTP manages the addition, deletion, and modification of VLANs at the system level, automatically communicating this information to all the other switches in the network. In addition, VTP minimizes these possible configuration inconsistencies that can result in security violations:

- VLANs can become cross-connected when duplicate names are used.

- VLANs can become internally disconnected when they are incorrectly mapped between one LAN type and the other.

You can use the CLI, console menus, or the MIB (when using a SNMP management station) to modify a VLAN configuration when the switch is in either server or transparent modes.

A switch configured in VTP server mode advertises VLAN configuration to neighboring switches through its trunks and learns new VLAN configurations from those neighbors. Use the server mode to add or delete VLANs and to modify VLAN information by using either the VTP MIB, the CLI, or the console. For example, when you add a VLAN, VTP advertises the new VLAN, and both servers and clients prepare to receive traffic on their trunk ports.

After the switch automatically transitions to VTP client mode, it transmits advertisements and learns new information from advertisements. However, you cannot add, delete, or modify a VLAN through the MIB, the CLI, or the console. The VTP client does not maintain VLAN information in nonvolatile storage; when it starts, it learns the configuration by receiving advertisements from the trunk ports.

In VTP transparent mode, the switch does not advertise or learn VLAN configurations from the network. When a switch is in VTP transparent mode, you can modify, add, or delete VLANs through the console, the CLI, or the MIB.

Consistent with the overall value of switches---easy to install, configure, and manage---Cisco has developed a VLAN administration and configuration protocol that automates the setup of VLANs across the network. This protocol, known as the VLAN Trunk Protocol (VTP) enables VLAN communication across the campus from a centralized network management platform, thus minimizing the amount of administration that is required when adding or changing VLANs anywhere within the network. VTP completely eliminates the need to administer VLANs on a per-switch basis, an essential characteristic as the number of a network's switches and VLANs grows and reaches a point where changes can no longer be reliably administered on individual components. VTP allows for greater scalability because it eliminates complex VLAN administration tasks across every switch.

VTP provides global advertisements to all Cisco switches when a new VLAN is added, changes its name, or is deleted. VTP advertisements are also sent to any new switches and links that have been added or changed, thus eliminating the need to reconfigure a switch for VLANs when topologies change. Further, VTP maps VLANs across different backbones, including Fiber Distributed Data Interfaces (FDDI), Fast Ethernet, and Asynchronous Transfer Mode (ATM). VTP mapping is significant because VLANs can now be configured as one continuous broadcast domain irrespective of the technology, and results in much wider distribution of VLANs across the network without any constraints based on backbone technology type. VTP maps ISL (Fast Ethernet) and 802.10 (FDDI) packet-tagging schemes with ATM LAN Emulation (LANE). Cisco plans to extend the use of VTP to support the 802.1Q first and second level packet-tagging schemes, offering seamless migration to 802.1Q.

VTP ensures network connectivity and reliability by maintaining VLAN configuration consistency across the network. This eliminates the need for configuration error checking and problem detection, which is one the biggest concerns when adding VLANs to a network. Moreover, VTP maintains security between VLANs because it configures a unique name, index number, and VLAN identifier for each VLAN. And it does not allow network mangers to use the same values for any other VLAN. VTP prevents network managers from easily confusing VLAN names and indexes which would result in configuration and security problems that could become difficult to detect and resolve. VTP maintains configuration accuracy across the network and provides a centralized configuration checkpoint for network managers.

VTP is essential for minimizing the reconfiguration costs associated with moves, adds, and changes. The fourth phase of Cisco's VLAN development road map specifies a dynamic VLAN architecture that automatically configures relocated users to VLANs. VTP ensures that VLANs exist across the network, and that all switches know about every VLAN. This roadmap allows a user to move anywhere in the network and, if permitted access to the VLAN, to join by simply configuring the appropriate port on the switch to the VLAN where they belong. VTP handles the setup of the VLAN to all the other switches participating in that VLAN, as shown in Figure 1. The setup includes configuring the links between the switches, as well as between the access node switches (see Figure 2).

VTP is a switch-to-switch, switch-to-router, and switch-to-server VLAN configuration advertisement protocol. It synchronizes all of the devices in the network to ensure consistent and up-to-date VLAN configuration information. Each device is VLAN aware and is kept up to date by VTP advertisements. The advertisements are sent to a multicast address that includes all neighboring devices. Advertisements from each switch contain the VLANs the switch knows about as well as the configuration revision number, which is used to synchronize each device to the network's latest VLAN configurations.

Typically, network managers set up VLANs from a common point of entry using a network management application. The application provides the field information necessary to describe a VLAN. This information is then distributed throughout the network as a VTP advertisement and goes to all switches, routers, and servers that are participating as VLAN-configurable components. Each device is categorized as either a VTP server or client. As a server device, the switch can advertise new VLANs and effect changes. VTP servers are points of entry for adding new VLANs and commonly receive information from the network management application. VTP servers can store the latest VLAN configurations and changes.

Multiple VTP servers within the network ensure VLAN configuration redundancy and reliability. These servers continuously synchronize their data and maintain consistent VLAN configuration data. If one part of the network goes down, the active switches maintain the latest VLAN configuration data and keep track of these revisions by using the configuration revision numbers. When the malfunctioning section on the network recovers, the switches within it receive the latest VLAN advertisements and the segment is updated to the latest network configuration. Thus recovery time is increased and the need to manually keep track of the changes that occurred during the outage are eliminated. This redundancy capability makes the VLAN switching fabric very robust and reliable. To ensure security, VTP uses "keyed-MD5" message authentication code to prevent spoofing.

VTP is an industry first, is supported within the CatalystTM 5000 as part of the 2.1 software release, and will be available on the Catalyst 3000 a part of the 1.3 software release. VTP also will also be supported in Cisco's Internetwork Operating System software (Cisco IOSTM) in the second half of 1996.

- VLAN configuration

- Broadcast/multicast control

To configure broadcast/multicast suppression:

Set port boradcast mod_num/port_num threshold%

- Dynamic VLANs

Covered later.

- Out-of-band management (console port)

You can view the following items

Switch Statistics,  Port Statistics,  Address table, Spanning Tree Bridge Information, Current Spanning Tree Information, Catalyst VLAN Statistics, CDP Neighbor Display,  VTP Statistics, IEEE 802.3 Statistics , Diagnostic Test Results, Message Log Information, ATM Port Info/Statistics, Switch Statistics Screen,

- In-band management (network connection using SNMP)

This is using SNMP or telnet

To configure SNMP:

| Task | Command |
| --- | --- |
| Step 1 Configure the SNMP community strings and access types. | set snmp community {read-only ¦ read-write ¦ read-write-all} [community_string] |
| Step 2 Assign a trap receiver address and community. | set snmp trap rcvr_address rcvr_community |
| Step 3 Enable SNMP traps. | set snmp trap enable [all ¦ module ¦ chassis ¦ bridge ¦ repeater ¦ auth ¦ vtp ¦ ippermit] |
| Step 4 Verify the SNMP configuration. | show snmp |

Example session:

Console> (enable) set snmp community read-only Company

SNMP read-only community string set.

Console> (enable) set snmp community read-write Engineering

SNMP read-write community string set.

Console> (enable) set snmp community read-write-all Sysadmin

SNMP read-write-all community string set.

Console> (enable) set snmp trap 172.20.52.3 Sysadmin

SNMP trap receiver added.

Console> (enable) set snmp trap enable all

All SNMP traps enabled.

Console> (enable) show snmp

RMON: Disabled

Traps Enabled: Port,Module,Chassis,Bridge,Repeater,Vtp,Auth,ippermit

Port Traps Enabled: 1/1-2,2/1-24

Community-Access     Community-String

----------------     --------------------

read-only            Company

read-write           Engineering

read-write-all       Sysadmin

Trap-Rec-Address                           Trap-Rec-Community

----------------------------------------   --------------------

172.20.52.3                                Sysadmin

Console> (enable)

- RMON

Remote Monitoring

Configuring RMON

To configure RMON:

| Task | Command |
| --- | --- |
| Step 1 Activate RMON support. | set snmp rmon enable |
| Step 2 Verify that RMON is enabled. | show snmp |

Example session:

Console> (enable) set snmp rmon enable

SNMP RMON support enabled.

Console> (enable) show snmp

RMON: Enabled

Traps Enabled: Port,Module,Chassis,Bridge,Repeater,Vtp,Auth,ippermit

Port Traps Enabled: 1/1-2,2/1-24

Community-Access     Community-String

----------------     --------------------

read-only            Company

read-write           Engineering

read-write-all       Sysadmin

Trap-Rec-Address                           Trap-Rec-Community

----------------------------------------   --------------------

172.20.52.3                                Sysadmin

Console> (enable)

- SPAN

The Switched Port Analyzer allows "mirroring" of all traffic passing through any port onto a single designated monitoring port. All transmit and receive activity on one of the eight Ethernet ports can be monitored through this monitor port.

To configure SPAN:

| Task | Command |
| --- | --- |
| Step 1 Specify the SPAN source and destination, and the type of traffic to monitor. | set span {src_mod/src_port ¦ src_vlan} dest_mod/dest_port [rx ¦ tx ¦ both] |
| Step 2 Enable SPAN. | set span enable |
| Step 3 Verify the SPAN configuration. | show span |

Example session:

Console> (enable) set span 2/1 2/24 tx

Enabled monitoring of ports 2/1 transmit traffic by ports 2/24.

Console> (enable) set span enable

Port monitoring enabled.

Console> (enable) show span

Source        Destination   Direction         Status

------------  ------------  ----------------  --------

Port 2/1      Port 2/24     transmit          enabled

Console> (enable)

- CWSI

CiscoWorks for Switched Internetworks (CWSI) is a comprehensive suite of applications designed for managing Cisco Catalyst® and LightStream® switches. CWSI offers extensive configuration, traffic, logical viewing and performance management capabilities on a device- and network-wide basis to dramatically ease network management of switched internetworks.

CWSI is a suite of applications that features:

- Graphical display of the physical devices and links in your network

- RMON and protocol analysis, as well as troubleshooting of protocol related problems

- Graphical display of ATM switches, physical links, permanent and switched virtual circuits, as well as performance monitoring of ATM switches and links and traffic analysis of RMON-enabled ATM links

- End-station and user ID information and tracking database as well as configuration information from MAC-VLAN mapping

- Graphical display for configuring, modifying, and managing VLANs

- Graphical representation of individual devices useful in configuring and monitoring chassis, port, and interface information.

- Describe the approach for troubleshooting Catalyst switches

- Describe the physical-layer problem areas

- Use the show commands to troubleshoot problems

- Describe the switch hardware status

- Describe the major features and functions of the catalyst 5000 FDDI/CDDI module

- Describe IEEE 802.10 VLANS

The IEEE 802.10 protocol provides connectivity between VLANs. Originally developed to address the growing need for security within shared LAN/MAN environments, it incorporates authentication and encryption techniques to ensure data confidentiality and integrity throughout the network. Additionally, by functioning at Layer 2, it is well suited to high-throughput, low-latency switching environments. IEEE 802.10 protocol can run over any LAN or HDLC serial interface.

- Configure the Catalyst 5000 FDDI/CDDI module

- Describe the LAN Emulation components

- Describe the start-up produce of a LAN Emulation Client

- Describe how one LEC establishes communication with another LEC

- Define LAN Emulation

- Outline the performance ratings for the ATM bus and the switching bus

- Describe how to access the CLI for the LANE

- List the features of the Catalyst 5000 LANE modules

- Describe the SIMPLE server redundancy Protocol (SSRP)

- Explain ATM address structure

- Describe how ATM addresses are automatically assigned

- Describe the rules for assigning ATM components to interfaces

- Configure LANE components on a Catalyst 5000 switch

- Describe the major features and benefits of the Catalyst 1900 and Catalyst 2820 switches

- Describe the hardware components and their function s of the catalyst 1900 and catalyst 2820 switches

- Describe the architecture and operation of the catalyst 1900 and catalyst 2820 switches

- Switching modes

- Virtual LANs

- Multicast packet filtering and registration

- Broadcast storm control

- Management support, CDP, and CGMP

- Trace a frame’s progress through a catalyst 1900 or catalyst 2820 switch

- Use the catalyst 1900 and catalyst 2820 switch menus for configuration

- Configure IP addresses and ports on the catalyst 1900 and catalyst 2820 switches

- Describe catalyst 3000 series LAN switch products

- Describe catalyst 3000 series LAN switch products differences

- Perform initial setup of a catalyst 3000 series switch

- Configure the switch for management

- Configure port parameters

- Configure VLANs and trunk links

- Configure the ATM LANE module

- Perform basic router module configuration

- Describe the post and diagnostic messages on the catalyst 1900 and catalyst 2820 switches

The 13 tests in POST detect nonfatal and fatal failures when the switch is powered up..

If a nonfatal failure occurs, the SYSTEM LED turns amber.

Catalyst 2820 Management Console

Copyright (c) Cisco Systems, Inc.  1993-1998

All rights reserved.

Standard Edition Software

Ethernet address:      00-E0-1E-7E-B4-40

PCA Number: 73-2239-01

PCA Serial Number: SAD01200001

Model Number: WS-C2822-A

System Serial Number: FAA01200001

-------------------------------------------------

*** Power On Self Test (POST) failed ***

*** Failed Test(s): 1

*** Failed Port(s): 1 2 3 4 5 6 7 8 A

1 user(s) now active on Management Console.

Press any key to continue.

- Describe the cabling guidelines for the catalyst 1900 and catalyst 2820 switches

- Use the statistics and reports to maintain the catalyst 1900 and catalyst 2820 switches

- Describe the firmware upgrade procedures for the catalyst 1900 and catalyst 2820 switches

- Troubleshoot the catalyst 3000 series switch subsystems

It is important to try to isolate a problem to a specific area, then try to isolate the problem to a specific component. By comparing what the system is doing to what it should be doing will usually identify the cause and help solve the problem.

When problem solving, or "troubleshooting" the Catalyst 3000 as a system, determine which one of the following subsystems has a problem:

- Power system - Input power, AC power cable, and power supply.

- Cooling system - The fans should go on when power is applied.

- System cables - This includes all of the interface cables that connect the equipment to the network.

Interpreting LEDs

The LEDs on the Catalyst 3000 and expansion modules indicate the operating state of the equipment. If you are experiencing problems, the following table may help to find the cause. With any of these errors, if these basic checks do not resolve the problem, contact the Cisco Technical Assistance Center (TAC).

| LED | State | What To Do |
| --- | --- | --- |
| PWR | Off | If the PWR LED is off and both of the 2 fans are not running, be sure the Catalyst 3000 is receiving power. |
| DIAG | ON | The DIAG LED is on during the self test. The self-test runs after power-on or reset. The LED should not remain on after this test. |
| FAULT | ON | The FAULT LED should remain off during normal operation. If it is on, an error has occurred. "Power cycle" the Catalyst 3000. The FAULT LED should not come back on. |
| LNK/FDX | OFF | Check all of the port connections. Check the cables and all of the cable connectors. Re-check configuration. |
| XMIT RCV | Not blinking | The XMIT and RCV LEDs should blink or be on as packets are forwarded to other ports. If the LEDs do not blink, verify that the physical network configuration is correct. Check the console for proper configuration and operation. |

Check the console for errors

Check the segment for communciation (ping a server)

Check the Virtual LAN segment lengths (make sure they are within the 802.3 specifications using a cable tester - Be sure to use a cable tester or TDR. Do not rely on the physical measurement of cables between stations and wiring closets because cables are often routed through the ceiling. )

Cable length specifications are listed in the following table:

| Cable | Length |
| --- | --- |
| 10Base-T | 328 feet (100 meters) |
| Thin Ethernet | 607 feet (185 meters) |
| Thick Ethernet | 1640 feet (500 meters) |

Using a TDR or other such cable-checking device, verify that the cable has no opens or shorts.

- Troubleshooting network interfaces and connections

- Use the switch LED’s to isolate problems

- Isolate network segment problems

- Configure cisco routers for scaleable operation in large or growing multiprotocol Internetwork

- Describe the key requirements of a scaleable Internetwork

- Select a cisco IOS features as a solution for a given Internetwork requirement

- Describe causes of network congestion

- List solutions for controlling network congestion

- Configure IP standard access lists

- Limit virtual terminal access

- Configure IP extended access lists

- Verify access list operation

- Configure an alternative to using access lists

- Configure an IP helper address to manage broadcasts

- Describe ipx/spx traffic management issues

- Filter ipx traffic using ipx access lists

- Manage ipx/spx traffic over wan connections

- Verify ipx/spx filter operation

- Describe the need for queuing in a large network

- Describe weighted fair queuing operation

- Configure priority queuing

- Configure custom queuing

- Verify queuing operation

- List the key information routers need to route data

- Compare distance vector and link-state protocol operation

link state routing protocol, is designed to recalculate routes quickly in the face of topological changes. This is called Fast Network Convergence. It uses a minimum of routing protocol traffic, so it provides better convergence time. It also allows different subnets of the same IP network number to have different masks. This is called Variable Length Subnetting. It provides support for discontiguous networks.

- given an IP address, use VLSMs to extend the use of the IP address

- given a network plan that includes IP addressing, explain if route summarization is or is not possible.

- Define private addressing and determine when it can be used

- Define network address translation and determine when it can be used

- Explain why OSPF is better than RIP in large Internetwork

See #124.

OSPF algorithm is sometimes referred to as the Dijkstra algorithm. OSPF is a link state protocol. Routers send LSA (Link State Advertisements) to its neighbors. Routers use LSA's (which contain routing information) to build a hierachy and compute the shortest path. OSPF supports VLSM & route summarization. the OSPF router ID is the number by which the router is known, the ID is important  in establishing neighbor relationships. By default the router ID is the highest number on any interface, unless a loopback address has been assigned, if the interface goes down the router must resend all its routing information about its interfaces, the default ID can be overridden using a loopback address. You can set a loopback address by specifying "Interface loopback 0", then specify an ip address by "ip address 10.10.1.1 255.255.0.0". OSPF uses a hello protocol to identify and establish relationships with ADJACENT routers. Routers on broadcast networks send hello packets every 10 seconds, non-broadcast networks use every 30 seconds. OSPF has two components to its rapid convergence: detection of topology changes, recalculation of routes. OSPF uses two mechanisms to detect topology changes: the first mechanism is interface status changes (eg: CD loss), the other mechanism is when it does not receive a hello packet from its neighbor within a timing window called a "dead timer". you can dead timer by specifing the "ip ospf dead-interval" at the config prompt, the default value of the dead timer is 4 times the value of the hello interval. LSA's are event driven. LSA's are also generated every 30 minutes as a method of ensuring topological database synchronization. At the 30 minute interval the router broadcasts the LSA to its neighbors, if the neighbors tables are already correct then nothing happens, otherwise they neighbors send LSA's to their neighbors.

There are three types of network connections in OSPF: Point to Point, non-broadcast multiaccess, multiaccess. Point to point is where each router can directly contact its neighbors. non-broadcast multiaccess routers may not communicate with all other routers (neighbor relationships must be manually configured in a non-broadcast network.) In a multiaccess network, the hello protocol elects a backup designated router (BDR), and a designated router (DR). the DR is responsible for generating LSAs for the entire network. The designed router approach decreases the amount of traffic and the size of the database. If the BDR doesn't receive the LSA in the specified time period it becomes the DR and the hello protocol elects a new BDR. Domain = AS. Rouers that operate soley within an area are called internal routers. Routers that have multiple interfaces can participate in multiple areas. These routers, which are called Area Border Routers (ABRs) maintain separate logical databases for each area.  All ABRs are connected together to form a OSPF backbone, the backbone itself is an OSPF area.

OSPF area guidelines, an area should not have more than 50 routers, and areas with unstable links should have even less. OSPF uses a CPU intensive algorithm. OSPF floods all Link State Changes to all routers in an area. ABRs should not be in more than three areas (since it has to recompute the route if something changes). Every ABR is in at least two areas (the backbone, and one area).  OSPF backbones must be contiguous, all routers in a backbone should be directly connected to other backbone routers. OSPF includes the concept of virtual links, a virtual link connects two ABRs that are not directly connected, you can use a virtual link to heal a partitioned backbone, however it is not a good idea to design a network that requires virtual links. You should also avoid placing hosts, such as workstations, file servers, or other shared resources, in the backbone area (keeping them out of the backbone simplifies internetwork expansion and creates a more stable environment). Areas must be contigous, which means you can trace a path from any router on the network to any other router on the network. You cannot use virtual links to reconnect a partition area. Areas can have a single ABR or multiple ABRs -- obviously multiple ABRs are better since you have less chance of the area becoming disconnected from the network.

OSPF areas can be categorized into a number of different types: stub areas, stub areas without summaries, non-stub areas. stub areas are areas that carry default route, intra-area routes, and interarea routes, but they do NOT carry external route advertisements. Stub areas without summaries carry only default route, and intraarea routes. non-stub areas carry default-routes, intraarea routes, interarea routes, and external routes. An area router must be a non-stub router if it carries any other protocols such as RIP, such a router is known as a Autonomous System Border Router (ASBR). A router must also be a non-stub area when a virtual link is configured across the area.

There are different type of LSA's: Router link advertisements (RLAs) describe the collected states of the routers links to a specific area. RLAs are flooded through the entire area, but nowhere else. NLA's (network link advertisements) are sent by designed routers they describe all the routers attached to a multiaccess router.  NLA's are flooded throughout the area containing the multiaccess network. RLAs and NLA's cause the full SPF algorith to be run. SLA's are generated by ABR's they summarize routes to destinations outside the area, but within the AS. Only intra-area routes are advertised to the backbone. Both intraarea and interarea routes are advertised into other areas. Finally, AS external links advertisements describe a route to a destination that is external to the AS. They are generated by AS boundary routers, this is the ONLY type of advertisement that is forwarded throughout the entire AS. All OSPF packets start with a 24 byte header.

Version is one byte, type byte denotes packet type: hello, database description packet, link-state request, link-state update, link-state acknowledgement.

The hello packet is sent at regular intervals to maintain neighbor relationships

database description packets describe the contents of the topology database, they are exchanged when an adjacency is being initialized.

link state request packet request a piece of a topology database from a neighbor.

link state update packets are responses to link state requests, they are also used in the regular dispersal of LSA's, several LSA's can be included in a single packet.

the link state acknowledgement is used to acknowledge link state updates, they must be acknowledged to ensure that link-state is flooding throughout the network reliable.

The two byte packet length specifies the length of the packet (including the headers 24 bytes).

The router id uses four bytes to identify a packets source.

The area id (also four bytes) identifies the area to which the packet belongs. All OSPF packets are associated with a single area.

The checksum field is used for error checking. (two bytes)

The two byte authentication field indiciates what type of authentication is used.  All OSPF protocol exchanges are authenticated. Authentication is configured on a per area basis. For example simple password is one type.

The eight byte authentication field contains the actual authentication information (for example the password)

Route summarization reduces the traffic and memory requirements for the routing protocol. To use route summarization you must have a hierarchial design. Route summarization is disabled by default in OSPF. You can assign numbers using many different options: unique numbers, NIC authorized numbers using VLSM and bitwise subnetting, Private addressing with buffer zone  to the official internet world. The first one is basically simple, large networks, easy to remember lots of wasted address space. If you use privately addressed IP nodes you will have limited access to the Internet. Use a DMZ (Demilitarized zone) – same as NAT. Route summarization increases the stability of the network. Keep in mind OSPF route summarization occurs at ABR’s, OSPF supports VLSM, OSPF requires manual route summarization.  There are four types of information that may need to be handled within an area: default, intra-area, inter-area, external. Intra-area routes are routes for all networks or subnetworks within an area. Inter-area are routes to other networks within in the same AS but not in this area. When designing an OSPF internetwork for efficient route selection there are three things to consider: tuning OSPF metrics, controlling inter-area traffic, load balancing. Default metric for OSPF is bandwidth, each link is assigned a number that is the inverse of its bandwidth. The metric for a route is the sum of all metrics for all links in the route. When route summarization is enabled OSPF uses the metric of best route in the summary. Link metrics are normalized, to give FDDI a value of 1, however your network might have ATM which is faster than FDDI but by default it will be assigned the same value as FDDI. Given an environment with both FDDI and a link faster than FDDI you must reconfigure your network so that the faster link has the lower value, you enter “ip ospf cost” at the interface prompt. There are two forms of external metrics – type 1 and type 2. If you use external type 1 metrics, routes are calculated based on adding the internal route metric to the external route metric, using type 2 the internal route metric is not added in. External type 1 is generally preferred. It is important to control inter-area traffic, when an area has only one ABR all traffic that does not belong in the area is sent to that ABR. In areas with more than one ABR two choices are available, use the ABR closest to the originator, or use the ABR closest to the destination. If ABR’s inject only the default route into the area, the traffic goes to the ABR closest to the source, this is convenient when the backbone has much faster speeds available. If you want to use the ABR closest to the destination then ABR’s should inject summary information, most networks prefer to avoid assymetric routing. Asymetric routing means the packets going from A to B take a different path than the packets going from B to A. Typically networks are designed to provide redundant routes, these routes are also handy for providing relief for high traffic lines, if equal cost paths exist in OSPF it automatically load balances. Cisco routes can automatically use up to 4 equal cost paths, packets can be distributed on a packet by packet basis, or by destination. Distributing packets based on destination is the default on Cisco routers, this is recommended for line speeds faster than 56k. You can use the “no ip route-cache” at the interface to disable distribution based on destination.

- Explain how OSPF discovers, chooses, and maintains routes

- Configure OSPF for proper operation

- Verify OSPF operation

- Describe the issues with interconnecting multiple areas and how OSPF addresses each

- Explain the differences between the possible types of areas, routers, and LSAs.

- Configure a multiarea OSPF network

- Verify OSPF operation

- Describe enhanced IGRP features and operation

EIGRP is compatible with IGRP routers. EIGRP features include fast convergence, partial and bounded updates, variable length subnet masks, multiple network layer support. EIGRP routers use the Diffusing Update Algorith (DUAL). The routers DUAL finite state machine is responsbile for determinig which direction packets are sent, it selects routes based on "feasible successor", a feasible successor is a neighboring router which has the least cost path to a destination that is guaranteed not to be part of a routing loop. Each EIGRP router keeps the routing table of its neighbors. EIGRP updates its routing tables ONLY when it needs to. When a EIGRP router loses a connection it examines the routing tables of its neighbors to determine a feasible successor. If a feasible successor cannot be determined it queries its neighbors until it finds one, this helps for rapid convergence. Routing information is automatically bounded so it only sends what needs to be updated. By default EIGRP uses a flat (non-hierarchical topology). EIGRP supports TCP/IP, Appletalk, IPX.

The functionality of EIGRP can be divided into two areas: tasks that are shared between protocols, tasks that are separate for each supported protocol. For each protocol a separate routing table must be maintained. For IP it uses OSPF, RIP, IS-IS, EGP, BGP. IPX uses SAP & RIP, Appletalk uses RTMP. Relevant data is passed to the DUAL which makes the decisions. Update packets use RTP to be reliable except for some that don't require reliable transport (eg: hello packets). EIGRP supports the following message types: hello, acknowledgment, updates, queries, replies, requests. Each EIGRP router that receives a query packet must unicast a reply, they indicate that the orginator of the query packet does not need to recompute the route because there are feasible sucessors. It can occur that the router receiving a packet does not know how to reach the destination, in such a case it sends a similar query to its neighbors, reply packets ensure that query packets are broadcast reliably. Reply packets are also broadcast reliably so they must be responded to, request packets are used to get specific information from neighbors, they are used in route server applications, they can be either unicast or multicast and are transmitted unreliably.

Every EIGRP router remains aware of its neighbors by maintain a "neighbor table" for each protocol, the tables are based on the reception of hello packets from the neighbors, when a router sends a hello packet it advertises a hold time, this is the amount of time that a router can be assumed to be operational, this way the router must broadcast another hello packet before the hold time expires.

EIGRP uses a topology table to keep its neighbors routing tables. the DUAL finite state machine uses the topology tables to build its routing tables. Destinations listed in a topology table can have two states: active and passive. A destination is in the passive state provide that it is in operation, a successor can be identified from the list of possible successors. If no successor can be located then it is in an active state, if a route is in an active state the router broadcasts queries to see if a feasible successor can be found.

DUAL uses the following method to determine a feasible successor. DUAL determines which neighbors advertise the lowest cost route to the destination. DUAL calculates the minimum cost to the destination. DUAL determines which neighbors offer a cost that is equal to the minimum cost to the destination. Once a successor is identified DUAL updates the routing table.

IP EIGRP can redistribute RIP, OSPF, ISIS EGP, and BGP.

To begin an EIGRP conversion start on the perhipery and work towards the center of the network. If your network contains only one AS then route redistribution is automatic.

config ; router eigrp 109 ; network 172.16.0.0 ; copy runn start

The only packet types which are transmitted reliably: update, query, reply [non-reliably: hello, acknowledgement, request]

Hello packets are used to discover neighbors, they are always multicast.

Reply packets are sent in response to query packets

Acknowledgement packets are sent in response to other types of packets that must be transmitted reliably.

items you would find in a topology table: destination address, neighbor offers the best route to a destination, other neighbors associate with that destination, the metrics each neighbor advertises for reaching that destination.

- configure enhanced IGRP

- verify enhanced IGRP operation

- select and configure the different ways to control route update traffic

- configure route redistribution in a network that does not have redundant paths between dissimilar routing processes

- configure route redistribution in a network that has redundant paths between dissimilar routing processes

- resolve path selection problems that result in a redistributed network

- verify route redistribution

- Describe when to use BGP to connect to an ISP

When the connection is Multihomed, or when it is necessary to accept a small portion of an internetwork into the routing tables (filtering).

- Describe methods to connect to an ISP using static and default routes, and BGP

If you don’t know how to do static routes, then you’ve got some real problems.

There are lots of great BGP tutorials out there, I’m not going to reinvent the wheel.

- Compare the differences between WAN connection types: dedicated, asynchronous dial-in, dial-on-demand and packet-switched services

Dedicated = leased line, T1

Asynchronous dial-in = modem, off a regular phone line

Dial-on-demand = can use either an asynchronous interface (modem) or a ISDN BRI circuit

Packet-switched = can be either frame relay or X.25 networks

- Determine when to use PPP, HDLC, LAPB, and IETF encapsulation types

Encapsulation methods are set according to the type of protocol or application you configure on your communication server.

IETF is used on Frame Relay links when you want to connect to another non-cisco router.

HDLC encapsulation is used for serial lines by default. This encapsulation method provides the synchronous framing and error detection functions of HDLC without windowing or retransmission

You can use PPP on either Synchronous or Asynchronous (modem) lines.

LAPB is typically used with X.25 networks

- List at least four common issues to be considered when evaluating a WAN service

Whew, I can think of a lot more than four.

Bandwidth, reliability, scalability, and ???

- Describe the components that make up ISDN connectivity

See #179

- Configure ISDN BRI

Specify the Interface and Its IP Address

To specify an ISDN Basic Rate Interface (BRI) and enter interface configuration mode, perform the following task in global configuration mode:

| Task | Command |
| --- | --- |
| Specify the interface and enter interface configuration mode. | interface bri number interface bri slot/port (Cisco 7200 series) |
| Specify an IP protocol address for the interface. | IP address IP-address mask |

PPP encapsulation is configured for most ISDN communication.  Each ISDN B channel is treated as a synchronous serial line and supports HDLC and PPP encapsulation. The default serial encapsulation is HDLC. To configure PPP encapsulation, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Configure PPP Encapsulation | encapsulation PPP |

The router might need to communicate with devices that require a different encapsulation protocol or the router might send traffic over a Frame Relay or X.25 network. For more information, see the "Configuring Frame Relay on ISDN D and B Channels" and "Configuring X.25 on ISDN D and B Channels" chapters of this manual.

To configure the router for automatic detection of encapsulation type on incoming calls, or to configure encapsulation for Combinet compatibility, see the "Configuring Special ISDN Signaling" chapter of this manual.

Configure Network Addressing. The steps in this section support the primary goals of network addressing:

- Define which packets are interesting--and will thus cause the router to make an outgoing call.

- Define the remote host where the calls are going.

- Specify whether broadcast messages will be sent.

- Specify the dialing string to use in the call.

Intermediate steps that use shared argument values tie the host identification and dial string to the interesting packets to be transmitted to that host.

To configure network addressing, complete the following tasks beginning in interface configuration mode:

| Task | Command |
| --- | --- |
| Step 1 (Most locations) Define the remote recipient's protocol address, host name, and dialing string; optionally, provide the ISDN subaddress; set the dialer speed to 56 or 64 kbps, as needed. Or (Germany) Use the command keyword that enables ISDN semipermanent connections. | dialer map protocol next-hop-address name hostname speed 56¦64 dial-string[:isdn-subaddress] dialer map protocol next-hop-address name hostname spc [speed 56 ¦ 64] [broadcast] dial-string[:isdn-subaddress] |
| Step 2 Assign the interface to a dialer group to control access to the interface. | dialer-group group-number |
| Step 3 Associate the dialer group number with an access list number. | dialer-list dialer-group list access-list-number |
| Step 4 Define an access list permitting or denying access to specified protocols, sources, or destinations. Permitted packets cause the router to place a call to the destination protocol address. | access-list access-list-number {deny ¦ permit} protocol source address source-mask destination destination-mask |

Specify ISDN Service Profile Identifiers (SPIDs)

Some service providers use service profile identifiers (SPIDs) to define the services subscribed to by the ISDN device that is accessing the ISDN service provider. The service provider assigns the ISDN device one or more SPIDs when you first subscribe to the service. If you are using a service provider that requires SPIDs, your ISDN device cannot place or receive calls until it sends a valid, assigned SPID to the service provider when accessing the switch to initialize the connection.

Currently, only the DMS-100 and NI-1 switch types require SPIDs. The AT&T 5ESS switch type may support a SPID, but we recommend that you set up that ISDN service without SPIDs. In addition, SPIDs have significance at the local access ISDN interface only. Remote routers are never sent the SPID.

A SPID is usually a seven-digit telephone number with some optional numbers. However, service providers may use different numbering schemes. For the DMS-100 switch type, two SPIDs are assigned, one for each B channel.

To define the SPIDs and the local directory number (LDN) on the router, perform the following tasks in interface configuration mode:

| Task | Command |
| --- | --- |
| Specify a SPID and local directory number for the B1 channel. | isdn spid1 spid-number [ldn] |
| Specify a SPID and local directory number for the B2 channel. | isdn spid2 spid-number [ldn] |

The LDN is optional but might be necessary if the router is to answer calls made to the second directory number.

Configure Calling Line Identification Screening

This task applies only to Cisco 2500 series, Cisco 3000 series, and Cisco 4000 series routers that have a BRI. Calling line identification (CLI, also called caller ID) screening adds a level of security by allowing you to screen incoming calls. You can verify that the calling line ID is from an expected origin. CLI screening requires a local switch that is capable of delivering the CLI to the router.

To configure caller ID screening, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Configure caller ID screening. | isdn caller number |

Note If caller ID screening is configured and the local switch does not deliver caller IDs, the router rejects all calls.

Configure Called Party Number Verification: When multiple devices are attached to an ISDN BRI, you can ensure that only a single device answers an incoming call by verifying the number or subaddress in the incoming call against the device's configured number or subaddress or both.

You can specify that the router verify a called-party number or subaddress number in the incoming setup message for ISDN BRI calls, if the number is delivered by the switch. You can do so by configuring the number that is allowed. To configure verification, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Specify that the router verify a called-party number or subaddress number in the incoming setup message. | isdn answer1 [called-party-number][:subaddress] |

Verifying the called-party number ensures that only the desired router responds to an incoming call. If you want to allow an additional number for the router, you can configure it, too.

To configure a second number to be allowed, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Specify that the router verify a second called-party number or subaddress number in the incoming setup message. | isdn answer2 [called-party-number][:subaddress] |

Configure ISDN Calling Number Identification

A router with an ISDN BRI interface might need to supply the ISDN network with a billing number for outgoing calls. Some networks offer better pricing on calls in which the number is presented. When configured, this information is included in the outgoing call Setup message.

To configure the interface to identify the billing number, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Specify the calling party number. | isdn calling-number calling-number |

This command can be used with all switch types except German 1TR6 ISDN BRI switches.

Configure the Line Speed for Calls Not ISDN End-To-End

When calls are made at 56 kbps but delivered by the ISDN network at 64 kbps, the incoming data can be corrupted.

However, on ISDN calls, if the receiving side is informed that the call is not an ISDN call from end to end, it can set the line speed for the incoming call. To set the speed for incoming calls recognized as not ISDN end-to-end, complete the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Set the speed to be used for incoming calls recognized as not ISDN end-to-end. | isdn not-end-to-end {56 ¦ 64} |

Configure a Fast Rollover Delay: Sometimes a router attempts to dial a call on an ISDN B channel before a previous, failed call is completely torn down. The fast rollover fails because the second call is made to a different number before the B-channel is released from the unsuccessful call. This might occur in ISDN configurations where:

- The two B-channels of the BRI are not configured as a hunt group, but have separate numbers defined, and

- The B-channel is not released by the ISDN switch until after Release Complete signal is processed.

You need to configure this delay if a BRI on a remote peer has two phone numbers configured one for each B-channel, you are dialing into this BRI, you have a dialer map for each phone number, and the first call succeeds but a second call fails with no channel available.

To configure a fast rollover delay, complete the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Define a fast rollover delay. | isdn fast-rollover-delay seconds |

A delay of 5 seconds should cover most cases. Configure sufficient delay to make sure the ISDN RELEASE_COMPLETE message has been sent or received before making the fast rollover call. Use the debug isdn q931 command to display this information.

This pattern of failed second calls is a rare occurrence.

Monitor and Maintain ISDN Interfaces: Use the following commands to monitor and maintain ISDN interfaces:

| Task | Command |
| --- | --- |
| Display information about the physical attributes of the ISDN BRI B and D channels. | show interfaces bri number show interfaces bri slot/port (Cisco 7200 series) |
| Display protocol information about the ISDN B and D channels. | show controllers bri number how controllers bri slot/port (Cisco 7200 series) |
| Display information about calls, history, memory, status, and Layer 2 and Layer 3 timers. | show isdn {active ¦ history ¦ memory ¦ status ¦ timers} |
| Obtain general diagnostic information about the specified interface. | show dialer interface bri number |

- Configure legacy dial-on-demand routing (DDR)

DDR Commands

This chapter describes the function and displays the syntax of each dial-on-demand routing command. For more information about defaults and usage guidelines, see the corresponding chapter of the Router Products Command Reference publication.

[no] backup delay {enable-delay | never} {disable-delay | never}

To define how much time should elapse before a secondary line status changes after a primary line status has changed, use the backup delay interface configuration command. To return to the default, which means as soon as the primary fails, the secondary is brought up without delay, use the no form of this command.

enable-delay :Number of seconds that elapse after the primary line goes down before the router activates the secondary line. The default is 0 seconds.

disable-delay Number of seconds that elapse after the primary line goes up before the router deactivates the secondary line. The default is 0 seconds.

never  Prevents the secondary line from being activated or deactivated.

[no] backup interface type number

To configure the serial interface as a secondary or dial backup line, use the backup interface interface configuration command. To disable this feature, use the no form of this command.

[no] backup load {enable-threshold | never} {disable-load | never}

To set traffic load threshold for dial backup service, use the backup load interface configuration command. To return to the default value, use the no form of this command.

enable-threshold  Percentage of the primary line's available bandwidth.

disable-load  Percentage of the primary line's available bandwidth.

never  Sets the secondary line to never be activated due to traffic load.

[no] chat-script script-name expect-send

Use the chat-script global configuration command to create a script that will place a call over a modem. Use the no form of this command to disable the specified chat script.

script-name  Name of the chat script.

expect-send  Content of the chat script.

clear dialer [interface type number]

clear dialer [interface serial slot/port] (Cisco 7000 series only)

To clear the values of dialer statistics for one or more serial or BRI interfaces configured for DDR, use the clear dialer privileged EXEC command.

Definitions:

interface  (Optional) Indicates that one interface will be specified.

type  Interface type, either serial or bri.

number  Interface number.

slot/port  On the Cisco 7000 series, specifies the slot and port numbers.

clear snapshot quiet-time interface

To end the quiet period on a client router within two minutes, use the clear snapshot quiet-time EXEC command.

[no] dialer caller number

To configure caller ID screening, use the dialer caller interface configuration command. To disable this feature, use the no form of this command.

number - Telephone number for which to screen. Specify an x to represent a single "don't-care" character. The maximum length of each number is 25 characters.

[no] dialer dtr

To enable DDR on an interface and specify that the serial line is connected by non-V.25bis modems using EIA signaling only (the data terminal ready [DTR] signal), use the dialer dtr interface configuration command. To disable dial-on-demand routing for the interface, use the no form of this command.

dialer enable-timeout seconds

no dialer enable-timeout

Use the dialer enable-timeout interface configuration command to set the length of time an interface stays down after a call has completed or failed before it is available to dial again. Use the no form of this command to reset the enable timeout value to the default.

seconds - Time in seconds that the router waits before the next call can occur on the specific interface. Acceptable values are positive, nonzero integers. The default is 15 seconds.

dialer fast-idle seconds

no dialer fast-idle

Use the dialer fast-idle interface configuration command to specify the amount of time that a line for which there is contention will stay idle before the line is disconnected and the competing call is placed. Use the no form of this command to return to the default value.

seconds  Idle time, in seconds, that must occur on an interface before the line is disconnected. Acceptable values are positive, nonzero integers. The default is 20 seconds.

dialer hold-queue packets

no dialer hold-queue [packets]

To allow "interesting" outgoing packets to be queued until a modem connection is established, use the dialer hold-queue interface configuration command. To disable a dialer hold queue, use the no form of this command.

packets  Number of packets, in the range 0 to 100 packets, to hold in the queue. This argument is optional with the no form of this command.

dialer idle-timeout seconds

no dialer idle-timeout

Use the dialer idle-timeout interface configuration command to specify the idle time before the line is disconnected. Use the no form of this command to reset the idle timeout to the default value.

seconds  Idle time, in seconds, that must occur on an interface before the line is disconnected. Acceptable values are positive, nonzero integers. The default is 120 seconds.

dialer in-band [no-parity | odd-parity]

no dialer in-band

Use the dialer in-band interface configuration command to specify that DDR is to be supported. Use the no form of this command to disable dial-on-demand routing for the interface.

no-parity  (Optional) Indicates that no parity is to be applied to the dialer string that is sent out to the modem on synchronous interfaces.

odd-parity  (Optional) Indicates that the dialed number has odd parity (7-bit ASCII characters with the eighth bit the parity bit) on synchronous interfaces.

dialer load-threshold load

no dialer load-threshold

To configure bandwidth on demand by setting the maximum load before the dialer places another call to a destination, use the dialer load-threshold interface configuration command. To disable the setting, use the no form of this command.

load  Interface load beyond which the dialer will initiate another call to the destination. This argument is a number between 1 and 255.

[no]dialer map protocol next-hop-address [name hostname] [spc] [speed 56 | 64] [broadcast]

[modem-script modem-regexp] [system-script system-regexp]

[dial-string[:isdn-subaddress]]

[no] dialer map protocol next-hop-address [name hostname] [spc]

[speed 56 | 64] [broadcast] [dial-string[:isdn-subaddress]]

[no] dialer map bridge [name hostname] [spc] [broadcast] [dial-string[:isdn-subaddress]]

[no] dialer map protocol next-hop-address dial-string [name hostname]

[modem-script modem-regexp] [system-script system-regexp]

[broadcast]

dialer map snapshot sequence-number dial-string

no dialer map snapshot [sequence-number]

To define a dialer map for Cisco's snapshot routing protocol on a client router connected to a DDR interface, use the dialer map snapshot interface configuration command. To delete one or more previously defined snapshot routing dialer maps, use the no form of this command.

sequence-number  Number in the range from 1 to 254, inclusive, that uniquely identifies a dialer map.

dial-string  Telephone number of a remote snapshot server to be called during an active period.

dialer priority number

no dialer priority

To set the priority of an interface in a dialer rotary group use the dialer priority interface configuration command. Use the no form of this command to revert to the default setting.

number  Specifies the priority of an interface in a dialer rotary group; the highest number indicates the highest priority. A number from 0 to 255. The default value is 0.

dialer rotary-group number

Use the dialer rotary-group interface configuration command to include an interface in a dialer rotary group.

number  Number of the dialer interface in whose rotary group you want this interface included. An integer that you select  that indicates the dialer rotary group; defined by the interface dialer command. A number from 0 to 255.

dialer string dial-string

no dialer string

Use the dialer string interface configuration command to specify the string (telephone number) to be called for interfaces calling a single site. Use the no form of this command to delete the dialer string specified for the interface.

dial-string  String of characters to be sent to a DCE.

dialer wait-for-carrier-time seconds

no dialer wait-for-carrier-time

Use the dialer wait-for-carrier-time interface configuration command to specify how long to wait for a carrier. Use the no form of this command to reset the carrier wait time value to the default.

seconds  Number of seconds that the interface waits for the carrier to come up when a call is placed. Acceptable values are positive, nonzero integers. The default is 30 seconds.

dialer-group group-number

no dialer-group

To control access, use the dialer-group interface configuration command. To remove an interface from the specified dialer access group, use the no form of this command.

group-number  Number of the dialer access group to which the specific interface belongs. This access group is defined using the dialer-list command. Acceptable values are nonzero, positive integers between 1 and 10.

[no] dialer-list dialer-group list access-list-number

To group access lists, use the dialer-list list global configuration command. To disable automatic dialing, use the no form of this command.

dialer-group   Specifies the number of a dialer access group identified in any dialer-group interface configuration command.

access-list-number  Specifies the access list number specified in any IP or Novell IPX access lists including Novell IPX extended, Service Access Point (SAP) access lists and bridging type.

dialer-list dialer-group protocol protocol-name {permit | deny |

list access-list-number | access-group}

no dialer-list dialer-group [protocol protocol-name [list

access-list-number | access-group]]

To define a DDR dialer list to control dialing by protocol or by a combination of protocol and access list, use the dialer-list protocol global configuration command. To delete a dialer list, use the no form of this command.

dialer-group  Number of a dialer access group identified in any dialer-group interface configuration command.

protocol-name   One of the following protocol keywords: AppleTalk, bridge, clns, clns_es, clns_is, decnet_router-L1, decnet_router-L2, decnet_node, IP, ipx, vines, or xns.

permit  (Optional) Permits access to an entire protocol.

deny   (Optional) Denies access to an entire protocol.

list    Specifies that an access list will be used for defining a granularity finer than an entire protocol.

access-list-number

Access list number. Access list numbers include any Decnet, Banyan VINES, IP, Novell IPX, or

XNS standard or extended access lists, plus Novell IPX extended, Service Access Point (SAP) access

lists and bridging types. See the "Dialer-List Supported Access List Types and Numbers" table in the

Router Products Command Reference publication.

access-group   Filter list name used in the clns filter-set and clns access-group commands.

encapsulation PPP - Use the encapsulation PPP interface configuration command to configure Point-to-Point Protocol (PPP) encapsulation.

interface dialer number

Use the interface dialer global configuration command to define a dialer rotary group.

number - Number of the dialer rotary group. It can be number in the range 0 through 255.

PPP authentication chap [if-needed]

no PPP authentication chap

To enable Challenge Handshake Authentication Protocol (CHAP) on a serial interface, use the PPP authentication chap interface configuration command. Use the no form of this command to disable this feature.

(Optional) CHAP authentication is not done on this line if the user has already authenticated.

PPP authentication pap [if-needed]

no PPP authentication pap

To enable Password Authentication Protocol (PAP) on a serial interface, use the PPP authentication pap interface configuration command. To disable this encapsulation, use the no form of this command.

(Optional) PAP authentication is not done on this line if the user has already authenticated.

script dialer regexp

no script dialer

To specify a default modem chat script, use the script dialer line configuration command. Use the no form of this command to disable this feature.

regexp  Specifies the set of modem scripts that might be executed. The first script that matches the argument regexp will be used.

show dialer [interface type number]

To obtain a general diagnostic display for serial interfaces configured for DDR, use the show dialer EXEC command.

interface  (Optional) Information for only the interface specified by the arguments type and number is to be displayed.

type   (Optional) Interface type.

number  (Optional) Interface unit number.

show snapshot [interface]

To display snapshot routing parameters associated with an interface, use the show snapshot EXEC command.

interface   (Optional) Interface type and number.

[no] snapshot client active-time quiet-time

[suppress-statechange-updates] [dialer]

To configure a client router for snapshot routing, use the snapshot client interface configuration command. To disable a client router, use the no form of this command.

active-time  Amount of time, in minutes, that routing updates are regularly exchanged between the client and  server routers. This can be an integer in the range 5 to 100. There is no default value. A typical value would be 5 minutes.

quiet-time  Amount of time, in minutes, that routing entries are frozen and remain unchanged between active periods. Routes are not aged during the quiet period, so they remain in the routing table as if they

were static entries. This argument can be an integer from 8 to 100000. There is no default value. The minimum quiet time is generally the active time plus 3.

suppress-statechange-updates  (Optional) Disables the exchange of routing updates each time the line protocol goes from "down" to "up" or from "dialer spoofing" to "fully up."

dialer  (Optional) Allows the client router to dial up the remote router in the absence of regular traffic.

[no] snapshot server active-time [dialer]

To configure a server router for snapshot routing, use the snapshot server interface configuration command. To disable a server router, use the no form of this command.

active-time  Amount of time, in minutes, that routing updates are regularly exchanged between the client and server routers. This can be an integer in the range 5 to 100. There is no default value. A typical value would be 5 minutes.

dialer   (Optional) Allows the client router to dial up the remote router in the absence of regular traffic.

username name password secret

Use the username password command to specify the password to be used in Challenge Handshake Authentication Protocol (CHAP) caller identification and Password Authentication Protocol (PAP).

name  Host name, server name, user ID, or command name.

secret  For CHAP authentication: specifies the secret for the local router or the remote device. The secret is encrypted when it is stored on the local router. This prevents the secret from being stolen. The secret can consist of any string of up to 11 printable ASCII characters. There is no limit to the number of username/password combinations that can be  specified, allowing any number of remote devices to be authenticated.

- Configure dialer profiles

To configure a dialer profile, perform the tasks in the following sections. The first and last are required. Map-class configuration is optional.

Configure a dialer interface - Any number of dialer interfaces can be created for a router. Each dialer interface is the complete configuration for a destination subnetwork and any networks reached through it. The router on the destination subnetwork sends traffic on to the appropriate shadowed networks.

To configure a dialer interface, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Create a dialer interface. | interface dialer number |
| Step 2 Specify the IP address and mask of the dialer interface as a node in the destination network to be called. | IP address address mask |
| Step 3 Specify PPP encapsulation. | encapsulation PPP |
| Step 4 Specify the remote router CHAP authentication name. | dialer remote-name name |
| Step 5 Specify the remote destination to call and the map class that defines characteristics for calls to this destination. | dialer string string class class-name |
| Step 6 Specify the dialing pool to use for calls to this destination. | dialer pool number |
| Step 7 Assign the dialer interface to a dialer group. | dialer-group number |
| Step 8 Specify an access list by list number or by protocol and list number to define the "interesting" packets that can trigger a call. | dialer-list dialer-group protocol protocol-name {permit ¦ deny ¦ list access-list-number} |

Configure a Map Class - Map class configuration is optional but allows you to specify different characteristics for different types of calls on a per-call-destination basis. For example, you can specify higher priority and a lower wait-for-carrier time for an ISDN-calls map class than for a modem-calls map class. You can also specify a different speed for some ISDN calls than for other ISDN calls.

A specific map class is tied to a specific call destination by the use of the map-class name in the dialer-string command with the class keyword.

To specify a map class and define its characteristics, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Specify a map class and enter map-class configuration mode. | map-class dialer class-name |
| Specify the fast idle timer value. | dialer fast-idle seconds |
| Specify the idle time before the calls in this map class are disconnected. | dialer idle-timeout seconds |
| Specify the length of time to wait for a carrier when dialing out to the dial string associated with the map class. | dialer wait-for-carrier-time seconds |
| For ISDN only, specify the bit rate used on the B channel associated with a specified map class or specify that an ISDN semipermanent connection is to be used for calls associated with this map. | dialer isdn [speed speed] [spc] |

Configure the Physical Interfaces - To configure a physical interface, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Specify the physical interface. | interface type number |
| Step 2 Enable PPP encapsulation. | encapsulation PPP |
| Step 3 Specify PPP CHAP authentication, if you also want to receive calls on this interface. | PPP authentication chap |
| Step 4 Put the interface in a dialing pool and, optionally, assign the interface a priority. For ISDN interfaces, you can also optionally specify the minimum number of channels reserved and maximum number of channels that can be for this dialing pool. | dialer pool-member number [priority priority] dialer pool-member number [priority priority] [min-link minimum] [max-link maximum]1 |
| Step 5 (Optional) Repeat command if you want to put the interface in additional dialing pools. | dialer pool-member number [priority priority] or dialer pool-member number [priority priority] [min-link minimum] [max-link maximum] |

- Verify DDR operation

- show interface BRI0 - To see if serial is up. It will be in “standby” mode if it is up.

- debug isdn events - To see ISDN activity occurring on the user side of the ISDN interface.

- debug isdn q931 - To check the ISDN connections as users dial-in to see what is happening with the ISDN call (for example, if the call is being dropped).

- debug PPP negotiation - To see if PPP traffic is passing across the link; this is when you will check for address negotiation.

- debug PPP authentication - To see if a client is passing authentication. If you are using a version prior to Cisco IOS™ release 11.2, use the debug PPP chap command instead.

- debug PPP error - To display protocol errors and error statistics associated with PPP connection negotiation and operation.

- debug dialer - To see dial-on-demand routing information.

- Configure dial backup

To specify a backup interface for a primary serial interface or subinterface, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Select a backup interface. | backup interface type number or backup interface type slot/port (For the Cisco 7500 series) or backup interface type slot/port-adapter/port (Cisco 7200 series) |

Note When you use a Basic Rate Interface (BRI) for a dial backup, neither of the B channels can be used while the interface is in a standby mode. In addition, when a BRI is used as a backup interface and the BRI is configured for Legacy DDR, only one B channel is usable. Once the backup is initiated over one B channel, the second B channel is unavailable. When the backup interface is configured for Dialer Profiles, both B channels can be used.

You can configure dial backup to activate the secondary line based on the traffic load on the primary line. The software monitors the traffic load and computes a 5-minute moving average. If this average exceeds the value you set for the line, the secondary line is activated and, depending upon how the line is configured, some or all of the traffic will flow onto the secondary dialup line.

To define how much traffic should be handled at one time on an interface, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Define the traffic load threshold as a percentage of the primary line's available bandwidth. | backup load {enable-threshold ¦ never} {disable-load ¦ never} |

Define Backup Line delays - You can configure a value that defines how much time should elapse before a secondary line status changes after a primary line status has changed. This means that you can define two delays:

- A delay that applies after the primary line goes down but before the secondary line is activated

- A delay that applies after the primary line comes up but before the secondary line is deactivated

To define these delays, perform the following task in interface configuration mode:

| Task | Command |
| --- | --- |
| Define backup line delays. | backup delay {enable-delay ¦ never} {disable-delay ¦ never} |

Dial Backup Using an Asynchronous Interface Example

The following is an example for dial backup using interface async 1, which is configured for DDR:

interface serial 0

IP address 172.30.3.4 255.255.255.0

backup interface async1

backup delay 10 10

!

interface async 1

IP address 172.30.3.5 255.255.255.0

dialer in-band

dialer string 5551212

dialer-group 1

async dynamic routing

!

dialer-list 1 protocol IP permit

!

chat-script sillyman "" "atdt 5551212" TIMEOUT 60 "CONNECT"

!

line 1

modem chat-script sillyman

modem inout

speed 9600

Dial Backup Using DDR and ISDN Example - The following example of uses an ISDN interface to back up a serial interface.

Note When you use a BRI interface for dial backup, neither of the B channels can be used while the interface is in standby mode.

Interface BRI 0 is configured to make outgoing calls to one number. This is a legacy DDR spoke example.

interface serial 1

backup delay 0 0

backup interface bri 0

ip address 1.2.3.4 255.255.255.0

!

interface bri 0

IP address 1.2.3.5 255.255.255.0

dialer string 5551212

dialer-group 1

!

dialer-list 1 protocol IP permit

Note Dialing will occur only after a packet is received to be output on BRI 0. Cisco recommends using the dialer-list command with the protocol and permit keywords specified to control access for dial backup. Using this form of access control specifies that all packets are interesting.

Dial Backup Service When the Primary Line Goes Down Example

The following example configures serial 1 as a backup line that becomes active only when the primary line (serial 0) goes down. The backup line will not be activated because of load on the primary line.

interface serial 0

backup interface serial 1

backup delay 30 60

The backup line is configured to activate 30 seconds after the primary line goes down and to remain on for 60 seconds after the primary line is reactivated.

The same example on the Cisco 7500 would be as follows:

interface serial 1/1

backup interface serial 2/0

backup delay 30 60

Dial Backup Service When the Primary Line Reaches Threshold Examples

The following example configures the secondary line (serial 1) to be activated only when the load of the primary line reaches a certain threshold:

interface serial 0

backup interface serial 1

backup load 75 5

In this case, the secondary line will not be activated when the primary goes down. The secondary line will be activated when the load on the primary line is greater than 75 percent of the primary's bandwidth. The secondary line will then be brought down when the aggregate load between the primary and secondary lines fits within 5 percent of the primary bandwidth.

The same example on the Cisco 7500 would be as follows:

interface serial 1/1

backup interface serial 2/2

backup load 75 5

Dial Backup Service When the Primary Line Exceeds Threshold Examples. The following example configures the secondary line to activate once the traffic threshold on the primary line exceeds 25 percent:

interface serial 0

backup interface serial 1

backup load 25 5

backup delay 10 60

Once the aggregate load of the primary and the secondary lines return to within 5 percent of the primary bandwidth, the secondary line is deactivated. The secondary line waits 10 seconds after the primary goes down before activating, and remains active for 60 seconds after the primary returns and becomes active again.

The same example on the Cisco 7500 is as follows:

interface serial 1/0

backup interface serial 2/0

backup load 25 5

backup delay 10 60

- Verify dial backup operation

- show interface BRI0 - To see if serial is up. It will be in “standby” mode if it is up.

- debug isdn events - To see ISDN activity occurring on the user side of the ISDN interface.

- debug isdn q931 - To check the ISDN connections as users dial-in to see what is happening with the ISDN call (for example, if the call is being dropped).

- debug ppp negotiation - To see if PPP traffic is passing across the link; this is when you will check for address negotiation.

- debug ppp authentication - To see if a client is passing authentication. If you are using a version prior to Cisco IOS™ release 11.2, use the debug ppp chap command instead.

- debug ppp error - To display protocol errors and error statistics associated with PPP connection negotiation and operation.

- debug dialer - To see dial-on-demand routing information.

- Configure multilink PPP

The Multilink Point-to-Point Protocol (PPP) feature provides load balancing functionality over multiple WAN links, while providing multivendor interoperability, packet fragmentation and proper sequencing, and load calculation on both inbound and outbound traffic. Cisco's implementation of Multilink PPP supports the fragmentation and packet sequencing specifications in RFC 1717.

Multilink PPP allows packets to be fragmented and the fragments to be sent at the same time over multiple point-to-point links to the same remote address. The multiple links come up in response to a dialer load threshold that you define. The load can be calculated on inbound traffic, outbound traffic, or on either, as needed for the traffic between the specific sites. MLP provides bandwidth on demand and reduces transmission latency across WAN links.

Multilink PPP is designed to work over single or multiple interfaces of the following types that are configured to support both dial-on-demand rotary groups and PPP encapsulation:

- Asynchronous serial interfaces

- Basic Rate Interfaces (BRIs)

- Primary Rate Interfaces (PRIs)

Configure Multilink PPP on Asynchronous Interfaces

To configure Multilink PPP on asynchronous interfaces, you configure the asynchronous interfaces to support DDR and PPP encapsulation, then you configure a dialer interface to support PPP encapsulation, bandwidth on demand, and Multilink PPP.

To configure an asynchronous interface to support DDR and PPP encapsulation, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Specify an asynchronous interface. | interface async number |
| Step 2 Specify no IP address for the interface. | no ip address |
| Step 3 Enable PPP encapsulation. | encapsulation ppp |
| Step 4 Enable DDR on the interface. | dialer in-band |
| Step 5 Include the interface in a specific dialer rotary group. | dialer rotary-group number |

Repeat this step for additional asynchronous interfaces, as needed.

At some point, adding more asynchronous interfaces does not improve performance, With the default MTU size, Multilink PPP should support three asynchronous interfaces using V.34 modems. However, packets might be dropped occasionally if the MTU is small or large bursts of short frames occur.

To configure a dialer interface to support PPP encapsulation and Multilink PPP, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Define a dialer rotary group. | interface dialer number |
| Step 2 Specify no IP address for the interface. | no ip address |
| Step 3 Enable PPP encapsulation. | encapsulation ppp |
| Step 4 Enable DDR on the interface. | dialer in-band |
| Step 5 Configure bandwidth on demand by specifying the maximum load before the dialer places another call to a destination. | dialer load-threshold load [inbound ¦ outbound ¦ either] |
| Step 6 Enable Multilink PPP. | ppp multilink |

To enable Multilink PPP on a single Integrated Services Digital Network (ISDN) BRI interface, you are not required to define a dialer rotary group separately because ISDN interfaces are dialer rotary groups by default.

To enable PPP on an ISDN BRI interface, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Specify an interface. | interface bri number |
| Step 2 Provide an appropriate protocol address for the interface. | ip address ip-address mask |
| Step 3 Enable PPP encapsulation. | encapsulation ppp |
| Step 4 (Optional) Specify a dialer idle timeout. | dialer idle-timeout seconds |
| Step 5 Specify the dialer load threshold for bringing up additional WAN links. | dialer load-threshold load |
| Step 6 Configure the ISDN interface to call the remote site. | dialer map protocol next-hop-address [name hostname] [spc] [speed 56 ¦ 64] [broadcast] [dial-string[:isdn-subaddress]] |
| Step 7 Control access to this interface by adding it to a dialer access group. | dialer-group group-number |
| Step 8 (Optional) Enable PPP authentication. | ppp authentication pap |
| Step 9 Enable Multilink PPP on the dialer rotary group | ppp multilink |

If you do not use PPP authentication procedures (Step 8), your telephone service must pass caller ID information.The load threshold number is required.

When Multilink PPP is configured and you want a multilink bundle to be connected indefinitely, use the dialer idle-timeout command to set a very high idle timer. (The dialer-load threshold 1 command no longer keeps a multilink bundle of n links connected indefinitely and the dialer-load threshold 2 command no longer keeps a multilink bundle of two links connected indefinitely.)

To enable Multilink PPP on multiple ISDN BRI interfaces, you set up a dialer rotary interface and configure it for Multilink PPP and then you configure the BRIs separately and add them each to the same rotary group. To set up the dialer rotary interface for the BRI interfaces, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Specify the dialer rotary interface. | interface dialer number |
| Step 2 Specify the protocol address for the dialer rotary interface. | ip address address mask |
| Step 3 Enable PPP encapsulation. | encapsulation ppp |
| Step 4 Specify in-band dialing. | dialer in-band |
| Step 5 (Optional) Specify the dialer idle timeout period, using the same timeout period as the individual BRI interfaces. | dialer idle-timeout seconds |
| Step 6 Map the next-hop protocol address and name to the dial string needed to reach it. | dialer map protocol next-hop-address [name hostname] [spc] [speed 56 ¦ 64] [broadcast] [dial-string[:isdn-subaddress]] |
| Step 7 Specify the dialer load threshold, using the same threshold as the individual BRI interfaces. | dialer load-threshold load |
| Step 8 Control access to this interface by adding it to a dialer access group. | dialer-group group-number |
| Step 9 (Optional) Enable PPP Challenge Handshake Authentication Protocol (CHAP) authentication. | ppp authentication chap |
| Step 10 Enable Multilink PPP. | ppp multilink |

If you do not use PPP authentication procedures (Step 10), your telephone service must pass caller ID information. To configure each of the BRIs to belong to the same rotary group, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Specify one of the BRI interfaces. | interface bri number |
| Step 2 Specify that it does not have an individual protocol address. | no ip address |
| Step 3 Enable PPP encapsulation. | encapsulation ppp |
| Step 4 Set the dialer idle timeout period, using the same timeout for each of the BRI interfaces you configure. | dialer idle-timeout seconds |
| Step 5 Add the interface to the rotary group. | dialer rotary-group group-number |
| Step 6 Specify the dialer load threshold for bringing up additional WAN links. | dialer load-threshold load |

Repeat Steps 1 through 6 for each BRI you want to belong to the same dialer rotary group.

For an example of configuring Multilink PPP on multiple ISDN BRI interfaces, see the "Multiple ISDN Interfaces Configured for Multilink PPP Example" section later in this chapter.

When Multilink PPP is configured and you want a multilink bundle to be connected indefinitely, use the dialer idle-timeout command to set a very high idle timer. (The dialer load-threshold 1 command no longer keeps a multilink bundle of n links connected indefinitely and the dialer load-threshold 2 command no longer keeps a multilink bundle of two links connected indefinitely.)

Configure MLP Interleaving and Queuing for Real-Time Traffic

Interleaving on Multilink PPP allows large packets to be multilink encapsulated and fragmented into a small enough size to satisfy the delay requirements of real-time traffic; small real-time packets are not multilink encapsulated and are transmitted between fragments of the large packets. The interleaving feature also provides a special transmit queue for the smaller, delay-sensitive packets, enabling them to be transmitted earlier than other flows.

Weighted fair-queuing on Multilink PPP works on the packet level, not at the level of multilink fragments. Thus, if a small real-time packet gets queued behind a larger best-effort packet and no special queue has been reserved for real-time packets, the small packet will be scheduled for transmission only after all the fragments of the larger packet are scheduled for transmission.

Weighted fair-queuing is now supported on all interfaces that support Multilink PPP, including Multilink PPP virtual access interfaces and virtual interface templates. Weighted fair-queuing is enabled by default.

Fair-queuing on Multilink PPP overcomes a prior restriction. Previously, fair-queuing was not allowed on virtual access interfaces and virtual interface templates. Interleaving provides the delay bounds for delay-sensitive voice packets on a slow link that is used for other best-effort traffic.

Restrictions

Interleaving applies only to interfaces that can configure a multilink bundle interface. These include virtual-templates, dialer interfaces, and ISDN BRI or PRI interfaces.

Multilink and fair queuing are not supported when a multilink bundle is off-loaded to a different system using Multichassis Multilink PPP. Thus, interleaving is not supported in Multichassis Multilink PPP (MMP) networking designs.

MLP Interleaving Configuration Tasks

Multilink PPP support for interleaving can be configured on virtual-templates, dialer interfaces, and ISDN BRI or PRI interfaces. To configure interleaving, complete the following tasks:

Step 1 Configure the dialer interface, BRI interface, PRI interface, or virtual template, as defined in the relevant chapters of this manual.

Step 2 Configure Multilink PPP and interleaving on the interface or template.

Note Fair queueing, which is enabled by default, must remain enabled on the interface.

To configure Multilink PPP and interleaving on a configured and operational interface or virtual interface template, perform the following tasks in interface configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Enable Multilink PPP. | ppp multilink |
| Step 2 Enable real-time packet interleaving. | ppp multilink interleave |
| Step 3 Optionally, configure a maximum fragment delay. | ppp multilink fragment-delay milliseconds |
| Step 4 Reserve a special queue for real-time packet flows to specified destination UDP ports, allowing real-time traffic to have higher priority than other flows. | ip rtp reserve lowest-UDP-port range-of-ports [maximum-bandwidth] |
| Step 5 For virtual templates only, apply the virtual template to the multilink bundle. | multilink virtual-template 1 |

1 This step is not used for ISDN or dialer interfaces.

Interleaving statistics can be displayed by using the show interfaces command, specifying the particular interface on which interleaving is enabled. Interleaving data is displayed only if there are interleaves. For example, the following line shows interleaves:

Output queue: 315/64/164974/31191 (size/threshold/drops/interleaves)

- Verify Multilink PPP operation

To monitor and maintain virtual interfaces, you can perform any of the following tasks:

| Task | Command |
| --- | --- |
| Display MLP and MMP bundle information. | show ppp multilink |

- Configure snapshot routing

In this snaphot routing configuration, Router 1 is acting as a client and Router 2 a server (master). The active time is 5 minutes and quiet time is 20 minutes. Interior Gateway Routing Protocol (IGRP) is the routing protocol being used. Broadcasts are all FFFFs at layer two and layer three.

You need the following:

Identify which router will be the client and which one will be the server.

The IP Addresses for both routers.

The ISDN phone numbers for both locations.

Snapshot client/server commands.

Active/quiet period timers on the Basic Rate Interface (BRI).

Two access servers.

ISDN switch information.

Configuration on the “Master” Side

hostname router2

!

username all

username router1 password foo   <-- for CHAP athentication

!

isdn switch-type basic-net3

!

interface Ethernet0

ip address 192.168.60.34 255.255.255.0

!

interface BRI0

ip address 172.16.13.134 255.255.255.0

encapsulation ppp

no keepalive

dialer idle-timeout 300

dialer map ip 172.16.13.133 name router1 broadcast 5552070

dialer hold-queue 10                    <-- snapshot itself will not use this

dialer-group 1

snapshot server 5

! make sure active time "5" matches the active time on client side

ppp authentication chap

!

router igrp 33

network 172.16.0.0

network 192.168.60.0

!

access-list 101 deny igrp 0.0.0.0 255.255.255.255 0.0.0.0 255.255.255.255

access-list 101 deny ip 198.92.60.149 0.0.0.0 0.0.0.0 255.255.255.255

access-list 101 permit ip 0.0.0.0 255.255.255.255 0.0.0.0 255.255.255.255

! Snapshot routing does not need to de defined in the access-list.

!

dialer-list 1 list 101

router igrp 33

network 172.16.0.0

!

access-list 101 deny   igrp any any

access-list 101 permit ip any any

!

dialer-list 1 list 101

Configuration on the “Client” Side

hostname router1

!

username router2 password foo

!

isdn switch-type basic-net3

!

interface Ethernet0

ip address 133.11.12.133 255.255.255.0 secondary

ip address 198.92.60.133 255.255.255.0

!

interface BRI0

ip address 172.16.13.133 255.255.255.0

encapsulation ppp

no keepalive

dialer map snapshot 1 5552050

! or use alternatively: dialer map snapshot 1 name router2 broadcast 5552050

dialer map ip 172.16.13.134 name router2 broadcast 5552050

dialer hold-queue 10

dialer-group 1

snapshot client 5 20 dialer

! the dialer option means we allow snapshot to use the dialer interface.

ppp authentication chap

!

Debug and Show Commands to Use for Troubleshooting

debug ppp authentication - To see if a client is passing authentication. If you are using a version prior to Cisco IOS™ release 11.2, use the debug ppp chap command instead.

debug ppp negotiation - To see if a client is passing PPP negotiation; this is when you will check for address negotiation.

debug ppp error - To display protocol errors and error statistics associated with PPP connection negotiation and operation.

debug isdn q931 - To check ISDN connections as users dial in to see what is happening with the ISDN call (for example, if the connection is being dropped)

- Configure ipx spoofing

To enable or disable spoofing for IPX watchdog packets, use the set ipx spoofing command. Spoofing is used to prevent unnecessary packets from being transmitted across the ISDN line.

SEt IPX SPoofing minutes | OFf

Cisco IOS software allows the router to respond to a server's Watchdog requests on behalf of a remote client. This process is called Network Control Protocol (NCP) or IPX spoofing. Benefits of IPX spoofing include:

Local acknowledgment of queries

No link responses

Efficient use of the link

Minimal operator intervention

When using IPX spoofing, NetWare servers are lead to believe that a session is still active when it is not. When the number of IPX or SPX sessions are limited, this can cause connectivity problems by denying logins to legitimate users.

Syntax Description

Minutes :  Enables IPX spoofing for an idle ISDN connection for a specified number of minutes. The range is 1 to 32,000 minutes.

off  :  Disables IPX spoofing.

Default: Off (disabled)

IPX routing must be enabled for any profile on which you wish to enable spoofing.

Example

The following example enables spoofing for one hour on the profile LAN:

Host:LAN> set ipx spoofing 60

Note IPX routing must be enabled for spoofing to function.

- Define routable and non-routable protocols and give an example of each

Very simple, non routable protocols do not contain the necessary information to discern the different between a host and a network. Non-routable protocols do not implement layer 3 (network) layer.

SNA and Netbios are examples of a non-routable protocol.

IP, IPX, AppleTalk are examples of a routable protocol.

- Define various bridging types and describe when to use each type

Bridging happens at the data-link layer, which controls data flow, handles transmission errors, provides physical addressing, and manages access to the physical medium. Bridges analyze incoming frames, make forwarding decisions based on those frames, and forward the frames to their destinations. Sometimes, like in source-route bridging, the frame contains the entire path to the destination. In other cases, like transparent bridging, frames are forwarded one hop at a time toward the destination.

Bridges can be either remote or local. Local bridges provide direct connections between many LAN segments in the same area. Remote bridges connect LAN segments in different areas, usually over telecommunication lines.

Transparent bridging is found primarily in Ethernet environments, and is mostly used to bridge networks which have the same media types. Bridges keep a table of destination addresses and outbound interfaces.

source-route transparent (SRT) bridging: can forward traffic from both transparent and source-route end nodes and form a common spanning tree with transparent bridges, thereby allowing end stations of each type to communicate with end stations of the same type in a network of arbitrary topology.

Translational bridging - used to bridge data between different media types. This is typically used to go between Ethernet and FDDI or Token Ring to Ethernet.

Source-route bridging (SRB) - found primarily in Token Ring environments. Bridges only forward frames based on the routing indicator contained in the frame. End stations are responsible for determining and maintaining the table of destination addresses and routing indicators.

remote source-route bridging (RSRB) allows you to bridge traffic between two Token Rings from separate routers (same as SRB but over WAN links)

Source-route translational bridging (SR/TLB)- a combination of source-route bridging and transparent bridging that allows communication in mixed Ethernet and Token Ring environments. (Translational bridging without routing indicators between Token Ring and Ethernet is also called source-route transparent bridging.)

Concurrent Routing and Bridging (CRB)  enables a user to both route and bridge a protocol on separate interfaces within a single router. However, routed traffic is confined to the routed interfaces, and bridged traffic is confined to the bridged interfaces.

Integrated Routing and Bridging (IRB) enables routing and bridging between VLANs, and includes support for IPX with Subnetwork Access Protocol (SNAP) and Service Advertising Protocol (SAP) encapsulations and for AppleTalk. It was designed to enhance CRB.

- Configure transparent bridging

The following lines would setup transparent briding between the serial interface and the ethernet interface, you would have to do the same lines on the remote router.

Router(config)# bridge 1 protocol ieee

Router(config)# interface e 0

Router(config-if)# bridge-group 1

Router(config-if)# interface s 0

Router(config-if)# bridge-group 1

- Configure Integrated routing and bridging (IRB)

Overview

Cisco Systems leads the industry in providing solutions for both switching (LAN and Asynchronous Transfer Mode [ATM]) and routing within the campus. In addition, we have the most complete implementation of virtual LANs (VLANs) in terms of tagging (Inter-S witch Link [ISL], 802.10,

and LAN Emulation [LANE]) and aggregation. However, routing between VLANs is currently limited to IP and Internetwork Packet Exchange (IPX), with Novell-Ethernet encapsulation for IPX only. This scenario has created selling and i mplementation problems in the field. Integrated routing and bridging (IRB) enables routing and bridging between VLANs, and includes support for IPX with Subnetwork Access Protocol (SNAP) and Service Advertising Protocol (SAP) encapsulations and for AppleT alk.

Introduction to IRB

IRB was designed to enhance concurrent routing and bridging (CRB), announced in Version 11.0 of Cisco Internetwork Operating System (Cisco IOS) software. In the past, the router could either route or bridge a protocol, but not both. CRB enables a user to both route and bridge a protocol on separate interfaces within a single router. However, routed traffic is confined to the routed interfaces, and bridged traffic is confined to the bridged interfaces. In other words, for any given protocol, t he traffic may be either routed or bridged on a given interface, but not both.

With CRB the routed interfaces do not communicate with the bridged interfaces. In order to connect a bridged interface to a routed interface, an external connection is needed; connection cannot be made internally.

Introducing an external connection between a bridged interface and a routed interface enables the router to communicate between the routed and bridged domains. The router still cannot perform this function internally, and two ports are needed to provide this functionality.

With IRB, a protocol can be routed either between both routed interfaces and bridged interfaces or between different bridge groups internal to the router. The traffic that remains in the bridge group (the bridged traffic) will be bridged among the bridged interfaces, and the traffic that needs to go out to

another network (the routed traffic) will be routed internally to the appropriate output routed interf ace. An external connection is no longer needed to provide this functionality. For example, all the local-area transport (LAT) traffic is bridged within the bridge group and stays local to the bridged interfaces. The IPX traffic destined f or a different network is routed from the bridged interface to the destination routed

interface. In order to implement the internal routing and bridging functionality, IRB uses a new interface, the bridged virtual interface (BVI), a virtual interface that represents the whole bridge group to the routed world. Instead of allowing every bridged interfa ce of the same bridge group to have a unique

path to the routed interfaces, there is only one BVI per bridge group, and the BVI represents all the interfaces within that bridge group. Bridging occurs within the bridge group, so bridged traffic has no need for the BVI. The interface number of the BVI is the same number as the bridge group it represents, creating a link between the two. For example, BVI 1 relates to bridge group 1. When configuring an Ethernet interface to map to the BVI, you need to config ure the bridge group number in the Ethernet interface and create a BVI to match the bridge group. The router will create the connection between the Ethernet interface and the BVI. If three Ethernet interfaces belong to the same bridge group, they use the same

BVI to route traffic. IRB routes through the BVI and bridges within the bridge group. The configurations later in this paper show the mapping between physical interfaces, bridge groups, and the BVIs.

Several areas should be noted when considering whether to use IRB or not. First, IRB is not supported on the Route Processor/Switch Processor (RP/SP) for the Cisco 7000 or the Cisco Advanced Gateway Server (AGS)+. It is currently supported on the Cisco 75 00 series, will be supported on the Cisco 7200 series, and will be supported on the Route Switch Processor (RSP) on the Cisco 7000 series. IRB supports IP, Internetwork Packet Exchange (IPX), and AppleTalk in both fast switching and process switching mode s. IRB is not supported over X.25 or Integrated Services Digital Network (ISDN) bridged interfaces.

When IRB is enabled, the default behavior in a bridge group is to bridge all packets. If you want to route a given protocol, you need to configure IRB to route that protocol traffic. If you want only to route (and not bridge) a protocol, you need to confi gure IRB to a) route that protocol, and b) not bridge that protocol. Configuring IRB to route a protocol does not automatically mean that bridging is disabled for that protocol. For nonroutable protocols such as local-area transport (LAT) and NetBIOS, the traffic will always be bridged; bridging for these protocols cannot be disabled.

Implementing IRB with ISL VLANs

Today, IP can be routed over any form of VLAN encapsulation, that is, LANE, ISL, and 802.10. However, some shortcomings exist when other protocols are running in the network. Cisco routers support IPX over ISL with Novell-ether encapsulations only. The other encapsulations, EtherII, SNAP, and SAP, are slated for Release 11.3, which will be available in Q1 '97. AppleTalk support over ISL will also be available with

this release. Customers can use IRB to route AppleTalk and IPX over ISL VLANs. However, IRB does not work with 802.10 VLANs; it works only with ISL VLANs.

With IRB enabled, IPX and AppleTalk traffic will enter the Fast Ethernet port, but will be seen on the BVI, not on the Fast Ethernet subinterface. The Fast Ethernet subinterface must have both ISL and a bridge group configured. The ISL VLAN number needs to map to the bridge group number. For example, you can configure the Fast Ethernet subinterface 0/0/0.1 to belong to ISL 1 and bridge group 1. How the bridge group maps to the BVI has been discussed; interface BVI 1 is configured to map to bridge group 1. The Fast Ethernet subinterface will put the data into a bridge group, and the BVI, not the Fast Ethernet subinterface, will act as the routed interface. A configuration example of using both ISL and IRB to route is given in the "IRB Configuration" section.

With the mapping between ISL/bridge group/BVI, IRB can route all the IPX encapsulations, AppleTalk, and IP over ISL. IRB can route the traffic between BVIs back to the Fast Ethernet subinterfaces or to other routed interfaces.  For each ISL VLAN that is configured, that ISL VLAN number should match both the bridge group number and the BVI number. A unique BVI with a corresponding bridge group should exist for that ISL VLAN. See more details in the section "IRB Configuration."

In networks that run only IP and IPX Novell, the use of IRB for inter-VLAN routing is unnecessary. But if the customer has another type of encapsulation for IPX (SNAP or SAP) or is running AppleTalk, then IRB can be used to route those protocols, and the Fast Ethernet subinterface can be used as the routed interface to route IP between ISL VLANs.

To obtain the best IRB over VLAN performance using ISL, use of the Versatile Interface Processor 2 (VIP2) card with the FE-PA in the Cisco 7500 series routers instead of just the Fast Ethernet Interface Processor (FEIP) is strongly recommended. There is only one option in the Cisco 7200 series. The VIP2 card has the ISL hardware assist on it, and therefore the ISL cyclic redundancy check (CRC) is calculated in hardware instead of software, which provides much faster performance. With the FEIP, the ISL CRC is calculated in software. The Cisco 7200 series has the hardware assist as well.

IRB Configuration

The following steps should be taken to configure IRB to run with ISL:

Step 1 Correctly configure the router to route the appropriate protocols, that is, ip/ipx/appletalk routing.

Step 2 Configure the specific media interface to belong to a bridge group if you want that interface to participate in IRB.

Step 3 For protocol addressing (IP, IPX, AppleTalk), configure the specific addresses, along with the IPX encapsulation and AppleTalk zone if appropriate, in the BVI interface.

Step 4 Turn on IRB, and configure each bridge group to bridge and/or route the appropriate protocol.

Step 5 Configure the ISL VLAN on the Fast Ethernet media interface, not the BVI (if inter-VLAN routing is desired; if you are using IRB without ISL, skip this step.)

Following is a sample configuration of IRB with ISL.

Use Fast Ethernet with ISL to route IP, but use IRB to route and bridge IPX (SNAP) and AppleTalk

The general IRB command to route is

bridge <bridge-group-number> route <protocol>

The configuration looks like:

!

appletalk routing

ipx routing 0000.0c40.0cb8

!

interface Fast Ethernet 1/0/0

no ip address

!

interface Fast Ethernet 1/0/0.1

encapsulation isl 1

ip address 198.18.1.1 255.255.255.0

bridge-group 1

!

interface Fast Ethernet 1/0/0.2

encapsulation isl 2

ip address 198.18.2.1 255.255.255.0

bridge-group 2

!

interface BVI1

no ip address

appletalk cable-range 1-1 1.253

appletalk zone irb-test

ipx encapsulation snap (or novell, or sap, or etherII)

ipx network c6120101

!

interface BVI2

no ip address

appletalk cable-range 2-2 2.253

appletalk zone irb-test

ipx encapsulation snap

ipx network c6120201

!

router eigrp 123

network 198.18.1.0

network 198.18.2.0

!

bridge irb

bridge 1 protocol ieee

bridge 1 route ipx

bridge 1 route appletalk

bridge 2 protocol ieee

bridge 2 route ipx

bridge 2 route appletalk

!

To see the IRB characteristics, you can type:

#show interface <interface> irb

To show the traffic that crosses the ISL subinterfaces or the IRB (labeled as bridged) interfaces, you can type:

#show vlan

- Describe the basic functions of source-route bridging (SRB)

SRB is the original method of bridging used to connect Token Ring segments. A source-route bridge makes all forwarding decisions based upon data in the routing information field (RIF)  It does not learn or look up Media Access Control (MAC) addresses. Therefore, SRB frames without a RIF are not forwarded.

With SRB, each port on the switch is assigned a ring number and the switch itself is assigned one or more bridge numbers. This information is used to build RIFs and to search them to determine when to forward a frame.

Clients or servers that support source routing typically send an explorer frame to determine the path to a given destination. There are two types of explorer frames: all-routes explorer (ARE) and spanning-tree explorer (STE). SRB bridges copy ARE frames and add their own routing information. For frames that are received from or sent to ports that are in the spanning-tree forwarding state, bridges copy STE frames and add their own routing information. Because ARE frames traverse all paths between two devices, they are used in path determination. STE frames are used to send datagrams because the spanning tree ensures that only one copy of an STE frame is sent to each ring.

- Configure SRB

- Configure source-route transparent bridging (SRT)

SRT bridging is an IEEE standard that combines source-route bridging and transparent bridging. An SRT bridge forwards frames that do not contain a RIF based on the destination MAC address. Frames that contain a RIF are forwarded based on source routing. There are two possible problems with using SRT.

Some protocols, such as SNA, attempt to establish a connection using a frame without a RIF. In the SNA case, this test frame is sent to see if the destination is on the same ring as the source. If no response is received from this test frame, then an ARE test frame with a RIF is sent. If SRT bridging is used, the first test frame without a RIF is forwarded through the bridge to the destination. The destination responds,

and the spanning-tree path through the bridges is used. Although this path will work, it may be undesirable. The network may be configured with parallel backbones with the intent that traffic is to be distributed across the backbones. This works well if source-routing is used. However, if the spanning-tree path is used, then only one of the backbones will carry traffic. The other backbone will not be used unless     there is a failure.

The use of duplicate SNA gateway MAC addresses can cause a problem. SNA requires the user to enter the destination MAC address of  the gateway (for example, IBM 3745 Token Ring interface coupler [TIC]). To prevent the user from having to enter a backup address in the case of a gateway failure, many SNA network designers put another gateway on a different ring with the same MAC address. This works with source routing and allows automatic recovery of a failed gateway. However, SRT does not allow the same MAC address to be on two different rings.

- Configure source-route translational bridging (SR/TLB)

You may bridge packets between a source-route bridging domain and a transparent bridging domain. Using this feature, a software "bridge" is created between a specified virtual ring group and a transparent bridge group. To the source-route station, this bridge looks like a standard source-route bridge. There is a ring number and a bridge number associated with a "ring" that actually represents the entire transparent bridging domain. To the transparent bridging station, the bridge represents just another port in the bridge group.

When bridging from the source-route (typically, Token Ring) domain to the transparent bridging (typically, Ethernet) domain, the source-route fields of the frames are removed. This RIF is cached for use by subsequent return traffic.

When bridging from the transparent bridging domain to the source-route bridged domain, the packet is checked to see if it has a multicast or broadcast destination, or a unicast (single host) destination. If it has a multicast, the packet is sent as a spanning-tree explorer. If it has a unicast destination, a RIF path is looked up to the destination in the RIF cache. If a path is found, it will be used, otherwise the packet will be sent as a spanning-tree explorer.

Note The spanning-tree protocol messages, used to prevent loops in the transparent bridging domain, are not passed between the source-route domain and the transparent bridging domain. Therefore, you must not set up multiple paths between the source-route and transparent bridging domains.

Configuring SR/TLB

A new subcommand of the global source-bridge configuration command is used to establish bridging between transparent bridging and source-route bridging.

Note Before using this command, you must have previously completely configured your router using multiport source-bridge and transparent bridging.

The command syntax is:

source-bridge transparent ring-group pseudo-ring bridge-num tb-group
no source-bridge transparent ring-group pseudo-ring bridge-num tb-group

The ring-group argument is a virtual ring group created by the source-bridge ring-group command. This is the source-bridge virtual ring to associate with the transparent bridge group.

The pseudo-ring argument is the ring number used to represent the transparent bridging domain to the source-route bridged domain. This number must be a unique number, not used by any other ring in your source-route bridged network.

The bridge-num argument is the bridge number of the bridge which leads to the transparent bridging domain.

The tb-group argument is the number of the transparent bridge group that you wish to tie into your source-route bridged domain. The no version of the command disables this feature.

The steps involved in configuring SR/TLB are to:

Step 1: Configure source-route bridging and transparent bridging.

Step 2: Choose a pseudo-ring number for the transparent bridging domain.

Step 3: Choose a bridge-number for the path to the transparent bridging domain.

Step 4: Enter the source-bridge transparent command in the configuration.

Assume the following configuration for source-route bridging and transparent bridging existed before the desire to enable SR/TLB:

!

interface tokenring 0

source-bridge 1 1 2

!

interface tokenring 1

source-bridge 2 1 1

!

interface Ethernet 0

bridge-group 1

!

interface Ethernet 0

bridge-group 1

!

bridge 1 protocol dec

One aspect of this configuration must change immediately: the two Token Ring interfaces are communicating with two-port local source-route bridging. The addition of SR/TLB creates a third ring, so these two interfaces must be reconfigured to communicate through a virtual ring, as shown:

!

source-bridge ring-group 10

!

interface tokenring 0

source-bridge 1 1 10

!

interface tokenring 1

source-bridge 2 1 10

!

interface ethernet 0

bridge-group 1

!

interface ethernet 1

bridge-group 1

!

bridge 1 protocol dec

At this point, the configuration is ready for the addition of the source-bridge transparent bridge command. You must determine two things:

- A ring number for the pseudo-ring. It must be unique throughout the source-route bridged network. For the example configuration above, use a 3.

- A bridge number for the path to the pseudo-ring. For the example configuration above, use a 1.

Once you have determined the ring number and the bridge number, the command can then be added, giving this final configuration:

source-bridge ring-group 10

source-bridge transparent 10 3 1 1

!

interface tokenring 0

source-bridge 1 1 10

!

interface tokenring 1

source-bridge 2 1 10

!

interface ethernet 0

bridge-group 1

!

interface ethernet 1

bridge-group 1

!

bridge 1 protocol dec

Source-Bridge Command Field Descriptions

| Field | Description |
| --- | --- |
| 10 | The ring-group to which the command applies; corresponds to the ring-group number given in the source-bridge commands. |
| 3 | The ring number assigned to the pseudo-ring. |
| 1 (first) | The bridge number leading to the pseudo-ring. |
| 1 (second) | The transparent bridge-group tied into this source-route bridged ring group; corresponds to the bridge number given in the transparent bridging commands. |

The following notes and caveats apply to all uses of SR/TLB:

- There cannot exist multiple paths between the source-route bridged domain and the transparent bridged domain. Such paths can lead to data loops in the network, as the spanning-tree packets used to avoid these loops in transparent bridged networks do not traverse the source-route bridged network.

- Many higher-level protocols, such as IP with its ARP frames, and XNS and Novell IPX with their addressing structure, violate protocol boundaries by putting MAC-level addresses into the data portion of the frame. SR/TLB, in translating between media, cannot hope to translate these addresses in the data portion of the frame. This will lead to confusing situations on end hosts. Therefore, while routing of these frames between Token Ring and Ethernet will work, bridging these frames will not. In general, it is best to assume that no normally routable protocol can be bridged with SR/TLB.

- Any access filters applied on any frames apply to the frames as they appear on the media to which the interface with the access control applies. This is important because in the far most common use of SR/TLB, Ethernet and Token Ring connectivity, the bit ordering of the MAC addresses in the frame is swapped.

- Verify SRB operation

debug source-bridge

The debug source-bridge command provides informational displays of source bridging activity.

Samples of messages displayed include the following:

In the following sample display, SRBn or RSRBn denotes a message associated with interface Token Ring n. An n of 99 denotes the remote side of the network.

SRBn: no path, s: <src MAC addr>d: <dst MAC addr>rif: <rif>

A bridgeable packet came in on interface Token Ring n but there was no where to send it. This is most likely a configuration error. For example, an interface has source bridging turned on but it is not connected to another source bridging interface or a ring group.

SRBn: direct forward (srn <ring>bn <bridge>trn <ring>)

A bridgeable packet has been forwarded from Token Ring n to the target ring. The two interfaces are directly linked.

SRBn: br dropped proxy XID,  <address>for <address>, wrong vring (rem)

SRBn: br dropped proxy TEST, <address>for <address>, wrong vring (rem)

SRBn: br dropped proxy XID,  <address>for <address>, wrong vring (local)

SRBn: br dropped proxy TEST, <address>for <address>, wrong vring (local)

SRBn: br dropped proxy XID,  <address>for <address>, no path

SRBn: br dropped proxy TEST, <address>for <address>, no path

A proxy explorer reply was not generated because there was no way to get there from this interface. The packet came from the node with the first <address>.

SRBn: br sent proxy XID,  <address>for <address>[<rif>]

SRBn: br sent proxy TEST, <address>for <address>[<rif>]

An appropriate proxy explorer reply was generated on behalf of the second <address>. It is sent to first <address>.

RSRB: sent RingXreq to <ring group>/<ip addr>

A Ring exchange request was sent to the indicated peer. This tells the remote side which rings this node has and requests a reply indicating which rings that side has.

RSRB: <label>: sent <op>to <ring group>/<ip addr>

A message has been sent to the indicated remote peer. Where <label> may be AHDR (active header), PHDR (passive header), HDR (normal header), or DATA (data exchange). Where <op> may be Forward, Explorer, Ring Xchg, Req, Ring Xchg, Rep, Unknown Ring Group, Unknown Peer, and Unknown Target Ring.

RSRB: removing bn <bridge>rn <ring>from <ring group>/<ip addr>

RSRB: added bridge <bridge>, ring <ring>for <ring group>/<ip addr>

The remote bridge and ring pair have been removed from or added to the local ring group table because the remote peer has changed.

RSRB: peer <ring group>/<ip addr>closed [last state n

RSRB: passive open <ip addr>(remote port) -><local port>

RSRB: CONN: opening peer <ring group>/<ip addr>, attempt n

RSRB: CONN: Remote closed <ring group>/<ip addr>on open

RSRB: CONN: peer <ring group>/<ip addr>open failed, <reason>[code]

Miscellaneous remote peer connection establishment messages.

RSRBn: sent local explorer, bridge <bridge>trn <ring>, [rif]

An explorer packet was propagated onto the local ring from the remote ring group.

RSRBn: ring group <ring group>not found

RSRBn: explorer rif [rif] not long enough

The remote source-route bridging code found the packet to be in error.

For each of the following debug commands there is a corresponding undebug command that stops the output. Enter these commands at the EXEC prompt.

debug source-event

The debug source-event command enables an interesting subset of the source-bridge debugging messages, including bridged packets rejected and remote peer connection activities.

debug token-event

The debug token-event command provides informational displays about significant Token Ring hardware events.

debug token-ring

The debug token-ring command invokes verbose Token Ring hardware debugging. This includes detailed displays as traffic arrives and departs the unit. Cisco Systems recommends using this feature only on router/bridges with light loads.

Following is an alphabetical summary of the global configuration commands for source-route bridging.

[no] access-list list {permit|deny} address mask

Used to configure a vendor code access list. This is a global bridge access-list command for IEEE 802 address access lists.

[no] access-list list {permit|deny} type-code wild-mask

Configures the access list mechanism for filtering frames by protocol type. The argument list is a user-selectable number in the range 200 - 299, inclusive, that identifies the list. The keyword permit permits the frame; the keyword deny denies the frame.

[no] lnm snmp-only

Prevents any LNM stations from modifying parameters in the Cisco router.

[no] netbios access-list bytes name {permit|deny} offset pattern

Defines the offset and patterns. The NetBIOS byte offset access list contains a series of offsets and hexadecimal patterns with which to match byte offsets in NetBIOS packets. The argument name is the name of the access list being defined. The argument offset is a decimal number indicating the number of bytes into the packet where the byte comparison should begin. An offset of zero points to the beginning of the NetBIOS delimiter string (0xffef) at the start of each NetBIOS packet. The argument pattern is a hexadecimal string of digits representing a byte pattern. The argument pattern must conform to certain conventions.

[no] netbios access-list host name {permit|deny} pattern

The NetBIOS station access list contains the station name with which to match, along with a permit or deny condition. Use the netbios access-list host global configuration command to assign the name of the access list to a station or set of stations on the network. The argument name is the name of the access list being defined. The argument pattern is a set of characters. The characters can be the name of the station, or a combination of characters and pattern matching symbols that establish a pattern for a set of NetBIOS station names.The no version removes an entire list, or just a single entry from a list, depending upon the argument given for pattern.

rif MAC-address [RIF-string] [interface-name|ring-group ring] 
no rif MAC-address [interface-name|ring-group ring]

Inserts or removes an entry into the RIF cache. If RIF-string is present, it is checked for validity and used. Otherwise, the entry is flagged as having no RIF. An interface-name or appropriate ring-group ring may also be specified to indicate the direction from which this RIF entry would have arrived. The no version of this command removes an entry from the cache.

[no] rif timeout minutes

Determines the period of inactivity allowed before unused RIF cache entries are removed. The no version of the command resets the RIF timeout period to its default of 15 minutes.

[no] source-bridge enable-80d5

Enables or disables the conversion of frames between Token Ring LLC2 and Ethernet Type II 0x80d5 format. When disabled, Token Ring LLC2 frames are converted to IEEE 802.3 format LLC2 frames. The list of frames undergoing this conversion may be customized by DSAP address with the source-bridge sap-80d5 command. This command only applies to frames going through the Source-Route Translational Bridge (SR/TLB), and not to frames going across a Source-Route Transparent (SRT) bridge.

[no] source-bridge largest-frame ring-group size

Defines the largest frame size to communicate with all peers in the ring group.

[no] source-bridge old-oui

Causes the OUI code in Token Ring frames translated to and from Ethernet Type II to be 0x000000. The default OUI value, used when this command is not given, is 0x0000F8. This default value is compatible with the draft value used in various SRT protocol documents. The no version of the command restores the default value.

[no] source-bridge passthrough ring-number

Specifies that frames destined to ring-number should never be terminated with Local Acknowledgment. The no version allows frames to be terminated with Local Acknowledgment.

[no] source-bridge remote-peer ring-group interface interface-name [lf size] [version number]

Defines or removes a serial interface over which to run bridged Token Ring traffic. The keyword lf and the size argument define the largest frame size to communicate with all peers in the ring group.The keyword version specifies the forced RSRB protocol version number for the remote peer.

[no] source-bridge remote-peer ring-group tcp ip-address [lf size][local-ack] [version number]

Defines or removes a remote peer for the specified ring group. We would make a TCP connection to carry bridged Token Ring traffic. The keyword lf and the size argument define the largest frame size to communicate with all peers in the ring group. The keyword local-ack specifies that Local Acknowledgment should be used for LLC2 session going to this remote peer.The keyword version specifies the forced RSRB protocol version number for the remote peer.

[no] source-bridge ring-group ring-number

Establishes or removes a ring group.

[no] source-bridge sap-80d5 SAP

When used in conjunction with the source-bridge enable-80d5 command, enables or disables the translation of Token Ring LLC2 frames to Ethernet Type 2 80d5 format frames. Frames whose destination SAP is given with one of these commands will undergo the transformation into 80d5 frames when entering an Ethernet. If the
source-bridge enable-80d5 command is not given, this command has no effect. As many of these commands as are needed may be given, one SAP per line.

[no] source-bridge tcp-queue-max number

Sets the maximum output TCP queue length, in packets, that will be enqueued to remote source-route bridge peers. The no version of the command restores the default queue length of 100.

[no] source-bridge transparent ring-group pseudo-ring bridge-number tb-group

Establishes a Source-Route Translational Bridging (SR/TLB) link between the source-route bridged ring-group and the transparent bridge group tb-group. The pseudo-ring is the ring number associated with the transparent bridging domain, and bridge-number is the pseudo-bridge that attaches the ring-group to the pseudo-ring.

Source-Route Bridge Interface Subcommand Summary

Following is an alphabetical summary of the interface subcommands for source-route bridging.

[no] access-expression {in|out} expression

Defines an access expression for a given interface--for Token Ring only.

[no] lnm alternate number

Enables an LRM other than the default LRM to change parameters. The number parameter must be an integer between 0 and 3.

[no] lnm crs

Enables the Configuration Report Server (CRS), which keeps track of the current logical configuration of a Token Ring, and reports any changes to LNM. Also reports on various other activities such as the change of the Active Monitor on a Token Ring.

[no] lnm loss-threshold number

A configurable number which expresses the percentage loss rate in hundredths of a percent of number of frames dropped compared to number of frames forwarded.

[no] lnm password number string

Assigns a password to a reporting link to prevent unauthorized access from an LRM to a bridge, and to control access to the different reporting links.

[no] lnm rem

Enables the Ring Error Monitor (REM), which monitors errors reported by any station on the ring. Also monitors whether the ring is in a functional state or in a failure state.

[no] lnm rps

Enables the Ring Parameter Server (RPS), which ensures that all stations on a ring are using a consistent set of reporting parameters, and report to LNM when any new station joins a Token Ring.

[no] lnm softerr number

Controls the frequency of error reports sent from stations on a Token Ring to the Ring Error Monitor. The number is a time interval in tens of milliseconds between error messages. The no version of the command restores the timer value to its default of 200, or two seconds.

mac-address ieee-address

Sets the MAC layer address. The argument ieee-address is a 48-bit IEEE MAC address written as a dotted triple of four-digit hexadecimal numbers. This command forces the use of a different MAC address on the specified interface, thereby avoiding the TI MAC firmware problem. It is up the network administrator to ensure that no other host on the network is using that MAC address.

[no] multiring {protocol-keyword|all|other}

Enables or disables the specified interface's ability to collect and use source-route (RIF) information for routable protocols. The argument protocol-keyword is one of: apollo, appletalk, clns, decnet, ip, vines, or xns. The all keyword enables the multiring for all frames; the other keyword enables the multiring for any frame not included in the previous list.

[no] netbios input-access-filter bytes name

Defines an access list filter on incoming messages. The argument name is the name of a NetBIOS access filter previously defined with one or more of the netbios access-list bytes global configuration commands. Use the no version of the command with the appropriate name to remove the entire access list.

[no] netbios input-access-filter host name

Defines an access list filter on incoming messages. The argument name is the name of a NetBIOS access filter previously defined with one or more of the netbios access-list host global configuration commands. Use the no version of the command with the appropriate argument to remove the entire access list.

[no] netbios output-access-filter bytes name

Defines an access list filter on outgoing messages. The argument name is the name of a NetBIOS access filter previously defined with one or more of the netbios access-list bytes global configuration commands. Use the no version of the command to remove the entire access list.

[no] netbios output-access-filter host name

Defines an access list filter on outgoing messages. The argument name is the name of a NetBIOS access filter previously defined with one or more of the netbios access-list global configuration commands. Use the no version of the command to remove the entire access list.

[no] source-bridge local-ring bridge-number target-ring

Enables and disables source bridging on a specific interface.

[no] source-bridge input-address-list list

Assigns an access list to a particular interface for filtering the Token Ring or IEEE 802 source addresses. Use this variation of the source-bridge interface subcommand to do this. The argument list is the access list number. The no version of this command removes the application of the access list.

source-bridge input-lsap-list list

Filters IEEE 802-encapsulated packets on input. The access list specifying the type codes to be filtered is given by this variation of the source-bridge interface subcommand. The argument list is the access list number. This access list is applied to all IEEE 802 frames received on that interface prior to the source-routing process. Specifying a zero disables the filter.

source-bridge input-type-list list

Filters SNAP-encapsulated packets on input. Use the access list specifying the type codes to be filtered The argument list is the access list number. This access list is then applied to all SNAP frames received on that interface prior to the source-routing process. Specify a zero for list to disable the application of the access list on the bridge group.

[no] source-bridge max-hops count

Limits the maximum number of source-route bridge hops of your network. The argument count determines the number of bridges an explorer packet may traverse. The command no source-bridge max-hops resets the count back to the maximum value.

[no] source-bridge old-sna

Enables or disables a workaround for some source-route bridging behavior exhibited by older SNA nodes.

[no] source-bridge output-address-list list

Assigns an access list to a particular interface for filtering the Token Ring or IEEE 802 destination addresses. Use this variation of the source-bridge interface subcommand to do this. The argument list is the access list number. The no version of this command removes the application of the access list.

source-bridge output-lsap-list list

Filters IEEE 802-encapsulated packets on output. The access list specifying the type codes to be filtered is given by this variation of the source-bridge interface subcommand. The argument list is the access list number. This access list is applied just before sending out a frame to an interface. Specifying a zero disables the filter.

source-bridge output-type-list list

Filters SNAP-encapsulated on output. Use the access list specifying the type codes to be filtered. The argument list is the access list number. This access list is then applied just before sending out a frame to an interface. Specify a zero for list to disable the application of the access list on the bridge group.

[no] source-bridge proxy-explorer

Enables and disables the proxy explorer function. The default is disabled.

[no] source-bridge route-cache

Enables fast switching to allow for faster implementations of local source-route bridging between 4/16 Megabit Token Ring cards in the same Cisco router/bridge. By default, the system enables fast switching in the source-route bridging software. The no version of the command disables this feature.

[no] source-bridge spanning

Manually changes the forwarding state of spanning explorer packets; the no form disables forwarding

- List the driving factors and enabling technologies that characterize access servers

This was obviously placed in the exam objectives by somebody who has no idea what this means. I’m going to assume this question refers to all the technologies mentioned before and after this question. Pure marketology.

- Describe the single user-to-LAN and LAN-to-LAN remote access scenarios

User-to-lan use a Cisco 7xx series router, LAN to Lan use a Cisco 3600 series router, this is marketology. User-to-lan is when you don’t have a network the router.

- List and describe the access services provided by cisco access servers

More marketology here. Nothing too technical. Cisco seems to have a product offering for every different configuration.

- Define the difference between basic telephone service and ISDN services

If you can’t do this then you shouldn’t even be thinking about taking this test. ISDN = digital, basic telephone service = Analog.

- Describe the functional differences between BRI and PRI facilities

BRI = 2B+1D, PRI = 23B+1D

The ISDN Physical Layer is specified by the ITU I-series and G-series documents. The U interface provided by the telco for BRI is a 2-wire, 160 kbps digital connection. Echo cancellation is used to reduce noise, and data encoding schemes (2B1Q in North America, 4B3T in Europe) permit this relatively high data rate over ordinary single-pair local loops.

2B1Q

2B1Q (2 Binary 1 Quaternary) is the most common signalling method on U interfaces. This protocol is defined in detail in 1988 ANSI spec T1.601. In summary, 2B1Q provides:

- two bits per baud

- 80 kbaud per second

- transfer rate of 160 kbps

This means that the input voltage level can be one of 4 distinct levels (note: 0 Volts is not a valid voltage under this scheme). These levels are called Quaternaries. Each quaternary represents 2 data bits, since there are 4 possible ways to represent 2 bits, as in the table above.

Frame Format

Each U interface frame is 240 bits long. At the prescribed data rate of 160 kbps, each frame is therefore 1.5 msec long. Each frame consists of:

- Frame overhead - 16 kbps

- D channel - 16 kbps

- 2 B channels at 64 kbps - 128 kbps

| Sync 18 bits | 12 * (B1 + B2 + D) 216 bits | Maintenance 6 bits |
| --- | --- | --- |

- The Sync field consists of 9 Quaternaries (2 bits each) in the pattern +3 +3 -3 -3 -3 +3 -3 +3 -3.

- (B1 + B2 + D) is 18 bits of data consisting of 8 bits from the first B channel, 8 bits from the second B channel, and 2 bits of D channel data.

- The Maintenance field contains CRC information, block error detection flags, and "embedded operator commands" used for loopback testing without disrupting user data.

Data is transmitted in a superframe consisting of 8 240-bit frames for a total of 1920 bits (240 octets). The sync field of the first frame in the superframe is inverted (i.e. -3 -3 +3 +3 +3 -3 +3 -3 +3).

The ISDN Data Link Layer is specified by the ITU  Q.920 through Q.923. All of the signalling on the D channel is defined in the Q.921 spec. (All you need to remember is that the Q series defines the data link layer)

Link Access Protocol - D channel (LAP-D) is the Layer 2 protocol used. This is almost identical to the X.25 LAP-B protocol.

Terminal Endpoint Identifiers (TEIs) are unique IDs given to each device (TE) on an ISDN S/T bus. This identifier can be dynamic; the value may be assigned statically when the TE is installed, or dynamically when activated.

| TEI | Description |
| --- | --- |
| 0-63 | Fixed TEI assignments |
| 64-126 | Dynamic TEI assignment (assigned by the switch) |
| 127 | Broadcast to all devices |

Establishing the Link Layer

The Layer 2 establishment process is very similar to the X.25 LAP-B setup, if you are familiar with it.

- The TE (Terminal Endpoint) and the Network initially exchange Receive Ready (RR) frames, listening for someone to initiate a connection

- The TE sends an Unnumbered Information (UI) frame with a SAPI of 63 (management procedure, query network) and TEI of 127 (broadcast)

- The Network assigns an available TEI (in the range 64-126)

- The TE sends a Set Asynchronous Balanced Mode (SABME) frame with a SAPI of 0 (call control, used to initiate a SETUP) and a TEI of the value assigned by the network

- The network responds with an Unnumbered Acknowledgement (UA), SAPI=0, TEI=assigned.

At this point, the connection is ready for a Layer 3 setup.

The ISDN Network Layer is also specified by the ITU Q-series documents Q.930 through Q.939. Layer 3 is used for the establishment, maintenance, and termination of logical network connections between two devices.

SPIDs

Service Profile IDs (SPIDs) are used to identify what services and features the telco switch provides to the attached ISDN device. SPIDs are optional; when they are used, they are only accessed at call setup time. The format of the SPID is usually the 10-digit phone number of the ISDN line, plus a prefix and a suffix that are sometimes used to identify features on the line. If an ISDN line requires a SPID, but it is not correctly supplied, then Layer 2 initialization will take place, but Layer 3 will not, and the device will not be able to place or accept calls. See ITU spec Q.932 for details.

Information Field Structure

The Information Field is a variable length field that contains the Q.931 protocol data.

| Information Field |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Protocol Discriminator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | Length of CRV |  |  |  |  |  |  |
| Call Reference Value (1 or 2 octets) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | Message Type |  |  |  |  |  |  |  |  |  |  |  |  |
| Mandatory & Optional Information Elements (variable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

These are the fields in a Q.931 header: 
Protocol Descriminator (1 octet) - identifies the Layer 3 protocol. If this is a Q.931 header, this value is always 0816. 
Length (1 octet) - indicates the length of the next field, the CRV. 
Call Reference Value (CRV) (1 or 2 octets) - used to uniquely identify each call on the user-network interface. This value is assigned at the beginning of a call, and this value becomes available for another call when the call is cleared. 
Message Type (1 octet) - identifies the message type (i.e., SETUP, CONNECT, etc.). This determines what additional information is required and allowed. 
Mandatory and Optional Information Elements (variable length) - are options that are set depending on the Message Type.

Layer 3 Call Setup

These are the steps that occus when an ISDN call is established. In the following example, there are three points where messages are sent and received; 1) the Caller, 2) the ISDN Switch, and 3) the Receiver.

- Caller sends a SETUP to the Switch.

- If the SETUP is OK, the switch sends a CALL PROCeeding to the Caller, and then a SETUP to the Receiver.

- The Receiver gets the SETUP. If it is OK, then it rings the phone and sends an ALERTING message to the Switch.

- The Switch forwards the ALERTING message to the Caller.

- When the receiver answers the call, is sends a CONNECT message to the Switch

- The Switch forwards the CONNECT message to the Caller.

- The Caller sends a CONNECT ACKnowledge message to the Switch

- The Switch forwards the CONNECT ACK message to the Receiver.

- Done. The connection is now up.

- List the primary ISDN reference points and their functions

Integrated Services Digital Network (ISDN) is a set of CCITT standards for digital transmission over ordinary telephone copper wire as well as over other media. Home and business users who install ISDN adaptors in place of theirmodems can see highly-graphic Web pages arriving very quickly (up to 128kbps). ISDN requires adapters at both ends of the transmission so your access provider also needs an ISDN adapter. ISDN is generally available from your phone company in most urban areas in the United States and Europe.

There are two levels of service: the Basic Rate Interface (BRI), intended for the home and small enterprise, and the Primary Rate Interface (PRI), for larger users. Both rates include a number of b-channels and d-channels. The B channels carry data, voice, and other services. The D channel carries control and signaling information.

The Basic Rate Interface consists of two 64 Kbps B channels and one 16 Kbps D channel. Thus, a Basic Rate user can have up to 128 Kbps service. The Primary Rate consists of 23 B channels and one 64 Kpbs D channel in the United States or 30 B channels and 1 D channel in Europe.

Integrated Services Digital Network in concept is the integration of both analog or voice data together with digital data over the same network. Although the ISDN you can install is integrating these on a medium designed for analog transmission, broadband  ISDN will extend the integration of both services throughout the rest of the end-to-end path using fiber optic and radio media. Broadband ISDN will encompass frame relay service for high-speed data that can be sent in large bursts, the Fiber Distributed-Data Interface (FDDI), and the Synchronous Opical Network (SONET). BISDN will support transmission from 2 Mbps up to much higher, but as yet unspecified, rates.

ISDN Standards: E deals with telephone network standards (between ISDN devices), I deals with concepts and terminology, Q deals with switching. (you need to know this stuff)

ISDN reference points:

TE1: Terminal Equipment type 1. This is the ISDN telephone, computer, ISDN FAX machine. Or whatever it is that you've hooked up to the ISDN phone line.

<the wire between a TE1 and a NT2 is called reference point S>

TE2: Terminal Equipment type 2. This is the old analog telephone, old-style fax machine, modem. Or whatever you used to hook up to the analog phone line. It can also be other communications equipment that is handled by a TA (see below).

<the wire between a TE2 and a TA is reference point R>

TA: Terminal Adaptor. This lets old, TE2 stuff talk to the ISDN network. It also adapts other kinds of equipment, like ethernet interfaces , to ISDN. It is typically called a POTS (Plain old Telephone system) port.

<the wire between a TE2 an a NT2 is reference point S>

NT2: Network Termination type 2. In most homes, this won't exist. If you were a big company with your own private telephone system, then this would be the guts of that telephone system.

<the wire between a NT2 and an NT1 is reference point T>

NT1: Network Termination type 1. This is the end of the line for the local phone company, and the beginning of your house's phone network.

<the wire between a NT1 and the LT is reference point U>

LT: Line Termination. This is the physical connection to the phone company.

<the wire between a LT and the ET is reference point V>

ET: Exchange Termination. This is the local phone company's logical connection from your telephones to "the phone network".

LAPD is the Link Access Protocol for the ISDN D channel, as defined by CCITT Recommendation Q.921. It is a layer 2, or data link layer, protocol which is used for communication between ISDN Terminal Equipment (TE, i.e. the router) and Network Equipment (NT, i.e. the ISDN exchange). LAPD is responsible for providing addressing, flow control, and error detection for higher layer users of the ISDN D channel. LAPD is similar to LAPB (layer 2 of X.25), with the addition multiple logical connections, allowing a single D channel to support multiple layer 3 entities. LAPD is not used on ISDN B channels.

In normal operation the LAPD module will not require any configuring since the default configuration will allow it to function fully. The default for BRI interfaces is to operate with automatic TEI assignment. PRI interfaces have TEI values assigned by CCITT Recommendation Q.921.

- Describe the call setup and teardown process for ISDN services

These are the steps that occus when an ISDN call is established. In the following example, there are three points where messages are sent and received; 1) the Caller, 2) the ISDN Switch, and 3) the Receiver.

- Caller sends a SETUP to the Switch.

- If the SETUP is OK, the switch sends a CALL PROCeeding to the Caller, and then a SETUP to the Receiver.

- The Receiver gets the SETUP. If it is OK, then it rings the phone and sends an ALERTING message to the Switch.

- The Switch forwards the ALERTING message to the Caller.

- When the receiver answers the call, is sends a CONNECT message to the Switch

- The Switch forwards the CONNECT message to the Caller.

- The Caller sends a CONNECT ACKnowledge message to the Switch

- The Switch forwards the CONNECT ACK message to the Receiver.

- Done. The connection is now up.

- Describe the T1 carrier technology

I’m going to assume they are referring to T1, E1 differences.

T1 or E1

In the early sixties engineers at Bell Labs created a voice multiplexing system that first digitized a voice signal into a 64 kbps data stream (representing 8000 voltage samples a second with each sample expressed in 8 bits) and then organized twenty four of them into a framed data stream, with some conventions for figuring out which 8 bit slot went where at the receiving end. The resulting frame was 193 bits long, and created an equivalent data rate of 1.544 Mbps. The structured signal was called DS1, but it has acquired an almost colloquial synonym -- T1 -- which also describes the raw data rate, regardless of framing or intended use. AT&T deployed DS1 in the interoffice plant starting in the late sixties (almost all of which has since been replaced by fiber), and by the mid-seventies was using DS1 in the feeder segment of the outside loop plant.

In Europe, and at CCITT (now ITU), the collection of world PTTs other than ATT modified Bell Labs original approach, as they were wont to do, and defined E1, a multiplexing system for 30 voice channels running at 2.048 Mbps. In Europe E1 is the only designation, and stands for both the formatted version and the raw data rate.

Until recently, T1 and E1 circuits were implemented over copper wire by using crude transceivers with a self-clocking Alternate Mark Inversion (AMI) protocol. AMI requires repeaters 3000 feet from the central office and every 6000 feet thereafter, and takes 1.5 MHz of bandwidth, with a signal peak at 750 kHz (U.S. systems). To a transmission purist, this is profligate and ugly, but it has worked for many years and hundreds of thousands of lines (T1 and E1) exist in the world today.

Telephone companies originally used T1/E1 circuits for transmission between offices in the core switching network. Over time they tariffed T1/E1 services and offered them for private networks, connecting PBXs and T1 multiplexors together over the Wide Area Network (WAN). Today T1/E1 circuits can be used for many other applications, such as connecting Internet routers together, bringing traffic from a cellular antenna site to a central office, or connecting multimedia servers into a central office. An increasingly important application is in the so-called feeder plant, the section of a telephone network radiating from a central office to remote access nodes that in turn service premises over individual copper lines. T1/E1 circuits feed Digital Loop Carrier (DLC) systems that concentrate 24 or 30 voice lines over two twisted pair lines from a central office, thereby saving copper lines and reducing the distance between an access point and the final subscriber.

Note, however, that T1/E1 is not a very suitable service for connecting to individual residences. First of all, AMI is so demanding of bandwidth, and corrupts cable spectrum so much, that telephone companies cannot put more than one circuit in a single 50 pair cable, and must put none in any adjacent cables. Offering such a system to residences would be equivalent to pulling new wire to most of them. Secondly, until recently no application going to the home demanded such a data rate. Thirdly, even now, as data rate requirements accelerate with the hope of movies and high speed data for everyone, the demands are highly asymmetric -- bundles downstream to the subscriber, and very little upstream in return -- and many situations will require rates above T1 or E1. In general, high speed data rate services to the home will be carried by ADSL or VDSL (or similar types of modems over CATV lines).

HDSL -- High data rate Digital Subscriber Line

HDSL is simply a better way of transmitting T1 or E1 over twisted pair copper lines. It uses less bandwidth and requires no repeaters. Using more advanced modulation techniques, HDSL transmits 1.544 Mbps or 2.048 Mbps in bandwidths ranging from 80 kHz to 240 kHz, depending upon the specific technique, rather than the greedy 1.5 MHz absorbed by AMI. HDSL provides such rates over lines up to 12,000 feet in length (24 ga), the so-called Carrier Serving Area (CSA), but does so by using two lines for T1 and three lines for E1, each operating at half or third speed.

Most HDSL will go into the feeder plant, which connect subscribers after a fashion, but hardly in the sense of an individual using a phone service.

Typical applications include PBX network connections, cellular antenna stations, digital loop carrier systems, interexchange POPs, Internet servers, and private data networks. As HDSL is the most mature of DSL technologies with rates above a megabit, it will be used for early-adopter premises applications for Internet and remote LAN access, but will likely give way to ADSL and SDSL in the near future.

SDSL -- Single line Digital Subscriber Line

On its face SDSL is simply a single line version of HDSL, transmitting T1 or E1 signals over a single twisted pair, and (in most cases) operating over POTS, so a single line can support POTS and T1/E1 simultaneously. However, SDSL has the important advantage compared to HDSL that it suits the market for individual subscriber premises which are often equipped with only a single telephone line. SDSL will be desired for any application needing symmetric access (such as servers and power remote LAN users), and it therefore complements ADSL (see below). It should be noted, however, that SDSL will not reach much beyond 10,000 feet, a distance overwhich ADSL achieves rates above 6 Mbps.

- Describe the two 56-kbps modem technologies: X2 and K56

X2 is a proprietary standard developed and marketed by US robotics

56K Flex is a standard developed by several companies including lucent and rocketwell.

Neither the 56k nor the X2 standards are compatible with eachother. (An interesting fact that really doesn’t matter since it isn’t on the test, but it is interesting none the less is that the modems rarely work at 56k, you really should read up on the details of how to provide 56k access, needless to say that you need a digital endpoint so you must be dialing into modem attached to a BRI or PRI to even get 56k access. Do some research on the net and you’ll see what I’m talking about)

V.90 is the new standard which is the 56k standard for communications, currently both Cisco and other RAS vendors are working towards this standard.

- Define xDSL and its applications and compare the various implementations

Be sure to read question #177 because HDSL and SDSL are also types of xDSL implementations.

DSL -- Digital Subscriber Line

The basic acronyms for all DSL arrangements came from Bellcore, so we must blame them for the basic confusion between a line and its modems. In general we say that DSL signifies a modem, or a modem pair, and not a line at all. Yes, a modem pair applied to a line creates a digital subscriber line, but when a telephone company buys DSL, or ADSL, or HDSL, it buys modems, quite apart from the lines, which they already own. So, DSL is a modem, not a line. This confusion becomes quite important to avoid when we talk about prices. A "DSL" is one modem; a line requires two.

DSL itself, apart from its later siblings, is the modem used for Basic Rate ISDN. A DSL transmits duplex data, i.e., data in both directions simultaneously, at 160 kbps over copper lines up to 18,000 feet of 24 ga wire. The multiplexing and demultiplexing of this data stream into two B channels (64 kbps each), a D channel (16 kbps), and some overhead takes place in attached terminal equipment. By modern standards DSL does not press any transmission thresholds, but its standard implementation (ANSI T1.601 or ITU I.431) employs echo cancellation to separate the transmit signal from the received signal at both ends, a novelty at the time DSL first found its way into the network.

DSL modems use twisted-pair bandwidth from 0 to about 80 kHz. (Some European systems use 120 kHz of bandwidth.) They therefore preclude the simultaneous provisioning of analog POTS. However, DSL modems are being used today for so-called pair gain applications, in which DSL modems convert a single POTS line to two POTS lines, obviating the physical installation of the second line wiring. The telephone company just installs the analog/digital voice functions at the customer premises for both lines, and presto, two from one.

ADSL -- Asymmetric Digital Subscriber Line

ADSL followed on the heels of HDSL, but is really intended for the last leg into a customer's premises. As its name implies, ADSL transmits an asymmetric data stream, with much more going downstream to the subscriber and much less coming back. The reason for this has less to do with transmission technology than with the cable plant itself. Twisted pair telephone wires are bundled together in large cables. Fifty pair to a cable is a typical configuration towards the subscriber, but cables coming out of a central office may have hundreds or even thousands of pairs bundled together. An individual line from a CO to a subscriber is spliced together from many cable sections as they fan out from the central office (Bellcore claims that the average U.S. subscriber line has twenty-two splices). Alexander Bell invented twisted pair wiring to minimize the interference of signals from one cable to another caused by radiation or capacitive coupling, but the process is not perfect. Signals do couple, and couple more so as frequencies and the length of line increase. It turns out that if you try to send symmetric signals in many pairs within a cable, you significantly limit the data rate and length of line you can attain.

Happily, the preponderance of target applications for digital subscriber services are asymmetric. Video on demand, home shopping, Internet access, remote LAN access, multimedia access, specialized PC services all feature high data rate demands downstream, to the subscriber, but relatively low data rates demands upstream. MPEG movies with simulated VCR controls, for example, require 1.5 or 3.0 Mbps downstream, but can work just fine with no more than 64 kbps (or 16 kbps) upstream. The IP protocols for Internet or LAN access push upstream rates higher, but a ten to one ratio of down to upstream does not compromise performance in most cases.

So ADSL has a range of downstream speeds depending on distance:

Up to 18,000 feet	1.544 Mbps (T1)

16,000 feet       2.048 Mbps (E1)

12,000 feet       6.312 Mbps (DS2)

9,000 feet	8.448 Mbps

Upstream speeds range from 16 kbps to 640 kbps. Individual products today incorporate a variety of speed arrangements, from a minimum set of 1.544/2.048 Mbps down and 16 kbps up to a maximum set of 9 Mbps down and 640 kbps up. All of these arrangement operate in a frequency band above POTS, leaving POTS service independent and undisturbed, even if a premises ADSL modem fails.

As ADSL transmits digitally compressed video, among other things, it includes error correction capabilities intended to reduce the effect of impulse noise on video signals. Error correction introduces about 20 msec of delay, which is much too much for LAN and IP-based data communications applications. Therefore ADSL must know what kind of signals it is passing, to know whether to apply error control or not (this problem obtains for any wire-line transmission technology, over twisted pair or coaxial cable). Furthermore, ADSL will be used for circuit switched (what we have today), packet switched (such as an IP router) and, eventually, ATM switched data. ADSL must connect to personal computers and television set top boxes at the same time. Taken together, these application conditions create a complicated protocol and installation environment for ADSL modems, moving these modems well-beyond the functions of simple data transmission and reception.

VDSL -- Very high data rate Digital Subscriber Line

VDSL began life being called VADSL, because at least in its first manifestations, VDSL will be asymmetric transceivers at data rates higher than ADSL but over shorter lines. While no general standards exist yet for VDSL, discussion has centered around the following downstream speeds:

12.96 Mbps	(1/4 STS-1)	4,500 feet of wire

25.82 Mbps	(1/2 STS-1)	3,000 feet of wire

51.84 Mbps	(STS-1)	        1,000 feet of wire

Upstream rates fall within a suggested range from 1.6 Mbps to 2.3 Mbps. The principal reason T1E1.4 decided against "VADSL" was the implication that VDSL would never be symmetric, when some providers and suppliers hope for fully symmetric VDSL someday, recognizing that line length will be compromised.

In many ways VDSL is simpler than ADSL. Shorter lines impose far fewer transmission constraints, so the basic transceiver technology is much less complex, even though it is ten times faster. VDSL only targets ATM network architectures, obviating channelization and packet handling requirements imposed on ADSL. And VDSL admits passive network terminations, enabling more than one VDSL modem to be connected to the same line at a customer premises, in much the same way as extension phones connect to home wiring for POTS.

However, the picture clouds under closer inspection. VDSL must still provide error correction, the most demanding of the non-transceiver functions asked of ADSL. As public switched network ATM has not begun deployment yet, and will take decades to become ubiquitous, VDSL will likely be asked to transmit conventional circuit and packet switched traffic. (Indeed, a recent telephone company RFQ describes a VDSL-type transceiver with three circuit-switched video channels and a single ATM channel.) And passive network terminations have a host of problems, some technical, some regulatory, that will surely lead to a version of VDSL that looks identical to ADSL (with inherent active termination) except its capability for higher data rates.

VDSL will operate over POTS and ISDN, with both separated from VDSL signals by passive filtering.

- Describe the major Cisco IOS features used for dialup applications

DDR – Dial on Demand Routing; allows a connection to be initiated when interesting traffic passes by.

BOD – bandwidth-on-demand: (1) A service provided by some data communications networks which can devote more network resources to an application needing to transport data at a now higher throughput. Typically bandwidth on demand is available from X.25 networks at up to 56 Kb/s, from frame relay networks at up to 1.5 Mb/s, and from ATM networks at up to 150 Mb/s. (2) A service provided by some data communications networks and/or CPE which can devote more network resources in exact increments of S6 or 64 Kb/s to an application needing to transportdata at a now higher throughput. Bandwidth-On-Demand INteroperability Group (BONDing): A multi-vendor agreement specifying the methods used for inverse multiplexing of N individual DSOs to establish a wide-area connection with bandwidth equivalent to N x DSOs. These caneither be 56 Kb/s or 64 Kb/s DSOs. BONDing typically is used to provide higher-bandwidth connections for videoconferencing applications.

- Describe the applications for which each of these features is used

See question #180.

- Describe line types and how they work

I’m going to assume they are referring to line encoding types, since we’ve already covered line types (eg: T1, ISDN) by now.

Typical Line Format/Encoding pairs: ESF/B8ZS, D4/AMI

The framing used by the physical layer of the T1 line may be D4 or ESF. D4 format, also known as the superframe format. This format consists of 12 consecutive frames, separated by framing bits. ESF specifies the extended superframe format. This format consists of 24 consecutive frames, separated by framing bits. The ISDN specification advises that you use ESF with ISDN D-channel signaling, when in reality you cannot use ISDN signaling with D4 framing; otherwise, false framing and Yellow Alarm emulation can result.

The encoding value sets the layer-1 line encoding used for the physical links, which affects the way in which data is  represented by the digital signals on the line. Your carrier can tell you which encoding to use.

ESF: Extended Super Frame. A technique of combining 24 DS-1 (T-1) frames so that the repeating 24 framing bits (F-bits) can be used to carry encoded network management data as well as a CRC-6 frame check sequence.

B8ZS: Binary 8 Zero Substitution . A technique used with DS-1 signals in order to maintain ones density, where a special code is substituted for eight consecutive zeros and marked by two bipolar violations. The use of B8ZS allows the use of a full 64kb/s DS-0 for data transmission. Not compatible with older "AMI" equipment. (Referred to Bipolar 8 Zero Substitution by early AT&T ref.)

D4:  Same as ESF except it uses 12 frames instead of  24.

AMI: Alternate Mark Inversion. Used in the DS-1 digital signal, where the first mark "1" is a positive voltage and the next mark "1" is a negative voltage. See bipolar. Often used to refer to a DS-1 with robbed bit signaling and without B8ZS coding.

- Describe how to configure passwords

Hostname(config)# enable password secretword

- Describe error logging/syslog facilities and commands, and identify the useful elements

- Using an access server, power up and perform either an automatic or manual initial configure of the cisco software

- Describe forward and reverse terminal connections

- Describe how terminal connections are managed using suspend, resume, show, disconnect and exit command

- Describe how to configure user menus

- Describe how to perform terminal connections for telnet

- Describe TN3270 operation with an access server

- Describe commands used to configure an access server for modems

- Describe the commands used to configure modems for dial-out operation

- Configure an external modem attached to a cisco access server for basic asynchronous operations

- Describe how modem auto-configuration can be used

The Cisco IOS software can issue initialization strings automatically for most types of modems. A modem initialization string is a series of parameter settings that are sent to your modem to configure it to interact with the access server in a specified way. The Cisco IOS software defines seven initialization strings that have been found to properly initialize most modems so that the modems function properly with Cisco access servers. These initialization strings have the following names:

- Codex_3260

- Usr_courier

- Usr_sportster

- Hayes_optima

- Global_village

- Viva

- Telebit_t3000

If you do not know which of these modem strings is appropriate for your modems, issue the modem autoconfigure discovery line configuration command, as shown in the following example:

2511# configure terminal

2511(config)# line 1 16

2511(config-line)# modem autoconfigure discovery

2511(config-line)# Ctrl-Z

2511# copy running-config startup-config

The Cisco IOS software first tries the first of these strings to see if the modem initializes properly. If not, the Cisco IOS software cycles to the next string and repeats the process until the appropriate string is found. If none of the strings properly initializes the modem, you must manually configure the modem. If you know that your modem can be configured using an initialization string from one of these scripts, you can issue the modem autoconfigure type type command, where type is one of the strings in the preceding list. If you list a specific modem type, initialization proceeds more quickly.

The following example shows how to enter line configuration mode and issue the modem autoconfigure type type command for a US Robotics Sportster modem.

2511(config-line)# modem autoconfigure type usr_sportster

- Give an example of when modem auto-configurations can be used

Duh, When you have a modem that Cisco knows how to autoconfigure!

- Identify configuration commands required to enable modem auto-configuration

See question #194

- Execute the commands used to verify and troubleshoot modem auto-configuration operation by performing the auto-configuration lab

See question #194

- Describe the differences between remote-node and remote control

Remote node = PPP connection

Remote control = PC Anywhere connection

If you don’t know the difference then you shouldn’t be taking this test.

- Describe and configure cisco access servers for PPP sessions

- Describe PPP options-callback, authentication, compression, and multilink

- Describe how to configure multilink PPP

- Describe major functions and features of Windows 95 dialup networking

- Configure a dialup windows 95 connection and connect to an access server

- Describe asynchronous routing features

- Configure and use dialer rotary groups in access servers

- Establish access control and timer control to a dial-on-demand routing (DDR) interface

- Configure and use a dialer map for DDR to multiple destinations

- Describe how to configure Cisco access servers for dial backup

- Specify default, static, redistribution, and passive interface routing configuration methods for ISDN

- Identify dialer map, protocol, DDR, address, name, and connection criteria and options for ISDN

Dialer profiles allow the configuration of physical interfaces to be separated from the logical configuration required for a call. With dialer profiles, the logical and physical configurations are dynamically bound on a per-call basis.

A dialer interface is a logical entity that uses a per-destination dialer profile. All configuration settings specific to the destination go into the dialer interface configuration. Multiple dial strings can be specified for the same dialer interface. A dialer string can be associated with different per-call parameters defined with each dialer map-class.

The dialer map-class defines specific characteristics for any call to the specified dial string. For example, the map-class for one destination might specify ISDN speed 56 kbps. The map-class for a different destination might specify ISDN speed 64 kbps.

Each dialer interface uses a dialer pool. A dialer pool is a group of one or more physical interfaces. A physical interface can belong to multiple dialer pools. Contention for a specific physical interface is resolved with a configurable priority.

Dialer profiles currently support PPP and HDLC encapsulation on the physical interface. All other settings are part of a logical configuration applied to the physical interface as needed for specific calls.

When dialer profiles are used to configure DDR, a physical interface has no configuration settings except encapsulation and the dialer pools to which the interface belongs.

To configure dialer profiles, perform the following tasks:

- Configure one or more dialer interfaces.

- Configure a map-class to specify different characteristics for different types of calls on a per-call-destination basis.

- Configure the physical interfaces.

These tasks are shown in the sample configuration below:

interface Dialer1

ip address 1.1.1.1 255.255.255.0

encapsulation ppp

dialer remote-name Smalluser

dialer string 5554540

dialer pool 3

dialer-group 1

!

interface Dialer2

ip address 2.2.2.2 255.255.255.0

encapsulation ppp

dialer remote-name Mediumuser

dialer string 5264540

dialer load-threshold 50 either

dialer pool1

dialer-group 2

!

interface Dialer3

ip address 3.3.3.3 255.255.255.0

encapsulation ppp

dialer remote-name Poweruser

dialer string 41156884540 class Eng

dialer hold-queue 10

dialer load-threshold 80

dialer pool 2

dialer-group 2

!

map-class dialer Eng

isdn speed 56

You can configure any number of dialer interface for a router. Each dialer interface is the complete configuration for a destination.

Use the dialer remote-name user-name command to specify the remote destination. This is the remote router name passed for authentication.

The dialer string command has been modified to support map classes.

Use the dialer pool number command to bind a dialer interface to a dialer pool that has been configured with the dialer remote-name command.

The dialer-group command is used to reference a dialer list. The dialer list protocol command specifies an access list number or a protocol that defines "interesting" packets to trigger a call.

You can use the map-class dialer class-name command to specify a map-class and enter the map-class configuration mode.

In this example below, the dialer isdn speed 56 command specifies an ISDN bit rate of 56 kbps for use in the map-class. You can set the speed to 56; 64 is the default value. Several other map-class commands are available as well:

| Command | Description |
| --- | --- |
| dialer isdn [speed <56>] ¦ [no-spc] | Specifies the ISDN line speed 56 Kbps. Note: 64 Kbps is the default. The speed parameter is used only with 56 Kbps line speed; 64 is not a valid option. The ¦ [no-spc] parameter is for ISDN only (not async). It lets you specify that an ISDN semipermanent connection is not to be used for calls associated with this map. |
| dialer idle-timeout number | Specifies the idle timer values to use when placing a call. |
| dialer fast-idle number | Specifies the fast idle timer values to use when placing a call. |
| dialer wait-for-carrier-time number | Specifies the carrier time value to use when placing a call. |

Configuring Physical Interfaces

Use the dialer pool-member number command to assign a physical interface to a dialer pool. You can assign an interface to multiple dialer pools by using this interface configuration command to specify several dialer pool numbers.

Use the priority option of this command to set the interface's priority within a dialer pool.

You can use a combination of synchronous, serial, BRI, or PRIs with dialer pools.

The dialer pool-member optional command parameters include:

| Parameter | Description |
| --- | --- |
| number | Sets the dialer pooling number. This is a decimal value from 1 to 255. |
| priority number | Sets the priority of the physical interface within the dialer pool. Interfaces with a priority number are selected first for dial out. This is a decimal value from 1 to 255. |
| min-link number | Sets the minimum number of ISDN B channels on an interface reserved for this dialer pool. This is a number from 1 to 255. (Used for dialer backup.) |
| max-link number | Sets the maximum number of ISDN B channels on an interface reserved for this dialer pool. This is a number from 1 to 255. |

Dialer Profiles Configuration Example

The configuration in this example is for a site that backs up two leased lines using one BRI.

interface Dialer0

ip unnumbered Loopback0

encapsulation ppp

dialer remote-name Remote0

dialer pool 1

dialer string 5551212

dialer-group 1

!

interface Dialer1

ip unnumbered Loopback0

encapsulation ppp

dialer remote-name Remote1

dialer pool 1

dialer string 5551234

dialer-group 1

!

interface BRI0

encapsulation PPP

dialer pool-member 1

ppp authentication chap

!

interface Serial0

ip unnumbered Loopback0

backup interface Dialer0

backup delay 5 10

!

interface Serial

ip unnumbered Loopback0

backup interface Dialer1

backup delay 5 10

Only the dialer interfaces are visible to the upper layer protocols, not the physical interfaces making up the dialing pool. Since one dialer interface maps to one destination, addressing access lists, and static routes can be specified on a per-destination basis, regardless of which interface actually carries out the call.

Dialer commands can be configured under the dialer interface directly. The same command may appear more than once, possibly with different parameters. The order of precedence is (from highest to lowest):

- map-class parameters

- interface parameters

Verifying Dialer Profiles Operation

The show dialer interface bri0 command displays information in the same format as the legacy DDR statistics on incoming and outgoing calls:

Router #show dialer interface bri 0

BRI0 - dialer type = ISDN

DIAL String     Successess    Failures   Last called   Last status

0 incoming call(s) have been screened.

BRI0: B-Channel 1

Idle timer (120 secs), Fast idle timer (20 secs)

Wait for carrier (30 secs), Re-enable (15 secs)

Dialer state is data link layer up

Dial reason: ip (s=6.1.1.8, d=6.1.1.1)

Interface bound to profile Dialer0

Time until disconnect 102 secs

Current call connected 00:00:19

Connected to 5773872 (system1)

BRI0: B-Channel 2

Idle timer (120 secs), Fast idle timer (20 secs)

Wait for carrier (30 secs), Re-enable (15 secs)

Dialer state is idle

The "Dialer state is data link layer up" line in the output above suggests that the dialer came up properly.

A "physical layer up" message in the output indicates that the Link Control Protocol (LCP) came up, but the Network Control Program (NCP) did not.

The source and destination address of the packet that initiated the dialing are shown in the "Dial reason" line of the output.

The show interface command provides similar information.

- Identify ISDN PRI configuration commands for setting the controller type, framing type and line coding, group time slot and speed to use, and DDR thesholds

- Configure an ISDN call, open a circuit, pass data over the interface, and monitor and analyze the ISDN operation between the devices

- Specify common methods of cost management

A primary-vendor Cisco solution has offered many users the benefits of reduced vendor and product implementation complexity realized by using a common operating software base that is shared across Cisco's broad router, switch, and access server product lines out to the customer premises. Several interviewed providers indicated that by streamlining their managed CPE service to a market leader like Cisco, their sales and marketing team message could be focused on their integrated solution, and not diluted into a hardware component option discussion.

- Describe how access list design can minimize access or service options

Using access lists to filter traffic provides a way to determine which traffic is “interesting”. Interesting traffic is traffic which has the ability to bring the session up.

- Identify why the following features are important: IPX spoofing, SPX spoofing, Appletalk NBP filtering (You also need to know HOW to do the following)

Answer: IPX Spoofing, SPX Spoofing, Appletalk NBP filtering are important since they make it possible to provide a seemless connection to a network without actually having a link established.

To enable or disable spoofing for IPX watchdog packets, use the set ipx spoofing command. Spoofing is used to prevent unnecessary packets from being transmitted across the ISDN line.

set IPX SPoofing minutes | OFf

Cisco IOS software allows the router to respond to a server's Watchdog requests on behalf of a remote client. This process is called Network Control Protocol (NCP) or IPX spoofing. Benefits of IPX spoofing include:

- Local acknowledgment of queries

- No link responses

- Efficient use of the link

- Minimal operator intervention

When using IPX spoofing, NetWare servers are lead to believe that a session is still active when it is not. When the number of IPX or SPX sessions are limited, this can cause connectivity problems by denying logins to legitimate users.

Syntax Description

| Minutes | Enables IPX spoofing for an idle ISDN connection for a specified number of minutes. The range is 1 to 32,000 minutes. |
| --- | --- |
| Off | Disables IPX spoofing. |

Default

Off (disabled)

Command Mode

Profile mode

Usage Guidelines

IPX routing must be enabled for any profile on which you wish to enable spoofing.

Example

The following example enables spoofing for one hour on the profile LAN:

Host:LAN> set ipx spoofing 60

Modifying the Keepalive Timers for the SPX Protocol

Application programs use the SPX protocol rather than IPX when they require guaranteed, sequenced delivery of packets. Examples of these types of programs include console and printer applications such as Novell's Remote Console (RCONSOLE) and Remote Printer (RPRINTER), gateway applications such as NetWare for Systems Application Architecture (SAA), and some remote database applications.

When an SPX connection is established, both ends of the connection exchange keepalive requests to track periods when no data is transmitted. By default, these keepalives are exchanged every three seconds for NetWare 3.x and every six seconds for NetWare 4.x on the server. Timers on the server cannot be changed; however, keepalive timers can be changed on the client side by editing the NET.CFG file and modifying the SPX Verify Timeout and SPX Abort Timeout parameters.

The default value for the SPX Abort Timeout parameter is 540 ticks, and for the SPX Verify Timeout parameter, 54 ticks. One tick equals 1/18th of a second. Values for these parameters can go up to 65,000 ticks, or approximately one keepalive every hour.

Enabling SPX Keepalive Spoofing

Another way to prevent SPX keepalive exchanges, which can consume significant bandwidth and run up costs on pay-per-packet WAN links, is to enable SPX spoofing. This is a Cisco IOS software feature that enables the Cisco router, rather than the SPX application, to respond to the keepalive packets. SPX spoofing keeps the packets off the WAN link yet maintains the NetWare connection. The configuration in Figure 6 illustrates that Router A has an Ethernet connection to Network 100 and an Integrated Services Digital Network (ISDN) Basic Rate Interface (BRI) to Network 1.

Both Router A and Router B need to spoof SPX keepalives. Router A is spoofing on Network 100 and Router B is spoofing on Network 200. The ISDN interface connects Router A to Router B and its remote clients.

Enable SPX spoofing using the ipx spx-spoof command. Note that route caching must be turned off before enabling this feature. Use the ipx spx-idle-time command to set the time in seconds that must elapse before spoofing of keepalive packets can occur.

Filtering NetWare Serialization Packets

To make sure that licensed copies of NetWare are not being improperly copied to other servers, the NetWare operating system has a built-in copy protection scheme. Each NetWare server transmits a packet (a unicast to all file servers) containing its unique serialization number at roughly 66-second intervals. When a server detects duplicate serialization identifiers, it broadcasts a copyright violation message to all users and the console log.

One solution to the problem of serialization packets keeping the link up is to filter them. This solution has some drawbacks because it could allow users to bypass Novell's licensing scheme. Although inelegant, filtering is the only solution to the NetWare serialization packet problem supported by the Cisco IOS software. It is done using the Cisco IOS software access-list command.

Defining an Access List to Filter NBP Packets

The following example adds entries to access-list number 607 to allow forwarding of NBP packets from specific sources and deny forwarding of NBP packets from all other sources. The first command adds an entry that allows NBP packets from all printers of type LaserWriter. The second command adds an entry that allows NBP packets from all AppleTalk file servers of type AFPServer. The third command adds an entry that allows NBP packets from all applications called HotShotPaint. For example, there might be an application with a zone name of Accounting and an application with a zone name of engineering, both having the object name of HotShotPaint. NBP packets forwarded from both applications will be allowed.

The final access-list other-nbps command denies forwarding of NBP packets from all other sources.

access-list 607 permit nbp 1 type LaserWriter

access-list 607 permit nbp 2 type AFPServer

access-list 607 permit nbp 3 object HotShotPaint

access-list 607 deny other-nbps

To use this access list to filter NBP packets on Ethernet interface 0, apply it to the interface using the following commands:

appletalk routing

interface ethernet 0

appletalk cable-range 55-55

appletalk zone No Parking

appletalk access-group 607

Note An NBP filter is applied against the inbound traffic when the filter is used with the "access-group" command. When used with dialer lists, the NBP filter is applied against outbound traffic.

The following example adds entries to access-list number 608 to deny forwarding of NBP packets from two specific servers whose fully-qualified NBP names are specified. It permits forwarding of NBP packets from all other sources.

access-list 608 deny nbp 1 object ServerA

access-list 608 deny nbp 1 type AFPServer

access-list 608 deny nbp 1 zone Bld3

access-list 608 deny nbp 2 object ServerB

access-list 608 deny nbp 2 type AFPServer

access-list 608 deny nbp 2 zone Bld3

access-list 608 permit other-nbps

access-list 608 permit other-access

To use this access list to filter NBP packets on Ethernet interface 0, apply it to the interface using the following commands:

appletalk routing

interface ethernet 0

appletalk cable-range 55-55

appletalk zone No Parking

appletalk access-group 608

Note An NBP filter is applied against the inbound traffic when the filter is used with the access-group command. When used with dialer lists, the NBP filter is applied against outbound traffic.

- Describe the basic hardware and software features of the cisco 700 series routers

Product Features

Dial-On-Demand Routing

SPID and Switch Automatic Detection

TFTP Client

Profile-Based User Name and Password

Bandwidth Allocation Control Protocol and Bandwidth Allocation Protocol

NetBIOS Over TCP/IP Packet Filter

NetBIOS Name Spoofing

Automatic Configuration From BOOTP

DHCP Server

DHCP Relay Agent

Port Address Translation

IP Address Assignment Through IPCP

IP RIP Update Linkup

Force Dynamically Negotiated IP Addresses

Fee Pulse Mode

Call Transfer

Call Forwarding (Sweden and Finland)

Three-Way Call Conferencing

Data Call Button (Cisco 770 Series Router Only)

Second Number Fail-Over

IPX Support

IPX Ping

Default IPX Route

IPX SAP Filtering

Service Advertising Protocol (SAP) Helper

Route Summarization

Data Compression

Security Features

Leased Line Authentication Requirement Removed

SNMP Support

SNMP Community Names

Supported MIBs

Supported RFCs

Router Panels

Specifications

Overview of Cisco 700 Series Routers

Cisco 700 series routers connect Ethernet LANs to other networks over Integrated Services Digital Network (ISDN) Basic Rate Interface (BRI) lines. Cisco 700 series routers offer multiprotocol routing capability between WAN and LAN ports and can function as transparent bridges.

This chapter contains the following sections:

Product Features

This section describes the major features of Cisco 700 series routers.

Dial-On-Demand Routing

Cisco 700 series routers include dial-on-demand routing (DDR). This allows the router to dynamically initiate calls to remote devices across ISDN BRI lines as traffic demands. The router also terminates ISDN connections based on the level of the traffic demanded on the ISDN line and the dynamic routing parameters.

SPID and Switch Automatic Detection

The automatic service profile identifier (SPID) and switch detection feature simplifies the use of ISDN terminal equipment and makes the equipment easier to use with fewer parameters to enter. With this feature, the user only has to enter the local directory numbers with area code. No SPID number or switch type is required from the user. The command used is the set autodetection command.

The automatic detection feature only applies to ISDN switches for the USA and Canada, usually an AT&T 5ESS Custom, DMS-100, NI-1, or NI-2 switch. The automatic detection mechanism might not work with any other switch, such as a Siemens switch. The automatic detection feature is only implemented in the U. S. image.

TFTP Client

Cisco 700 series routers support the use of a Trivial File Transfer Protocol (TFTP) client. This feature increases TFTP file transfer efficiency and reliability. The swl tftp command loads the TFTP client software from the console port.

Profile-Based User Name and Password

Remote users often have multiple service providers. Connecting to different service providers might require different user names and passwords. Previous releases of Cisco 700 software used the same user name and password for all profiles when making outgoing Point-to-Point Protocol (PPP) connections. Cisco 700 series router software Release 4.1(1) allows different profiles to have different user names and passwords.

In previous releases, the set systemname command set the system prompt and the PPP user name when making outgoing PPP connections. Different profiles can now use different user names when dialing out. A new profile level command, the set ppp clientname command, has been created for this purpose.

Bandwidth Allocation Control Protocol and Bandwidth Allocation Protocol

The Bandwidth Allocation Control Protocol (BACP) and the Bandwidth Allocation Protocol (BAP) define a set of rules to control dynamic bandwidth allocation gracefully by managing the number of links in a PPP multilink bundle. BACP consists of a network control protocol that negotiates once per PPP multilink bundle while BAP defines a set of request and response messages to manage the links.

Cisco 700 series router software Release 4.0(2) already supports dynamic management of both B channels with the demand feature. The implementation of BACP and BAP allows Cisco 700 series routers to coordinate and negotiate the actual allocation and deallocation of the second channel. The parameters are set using the demand command and the timeout command.

BACP is only implemented on a BRI interface. A Multilink PPP has to be negotiated for BACP to be functional.

If a data call is bumped to accommodate a voice call and if BAP negotiation is enforced, the user might experience a short audio idle period before a B channel becomes available. The user might not be able to connect the voice call if the peer declines to give up the link. This happens when the set ppp bacp linkdrop negotiation command is set to ON.

NetBIOS Over TCP/IP Packet Filter

A generic filter capability on the Transaction Control Protocol/User Datagram Protocol (TCP/UDP) packets allows users to filter NetBIOS packets generated by application programs. Use the set ip filter command to control this feature.

NetBIOS Name Spoofing

When a WINS client sends out a NetBIOS Name query, the router can answer a NetBIOS name query in place of the WINS server. If the router cannot answer, the router forwards the query packets to the server, which forwards the response to the client. If spoofing is set, the router keeps a local database of up to 100 Domain Name System (DNS) name entries and has an aging scheme to age out the unused name entries.

Automatic Configuration From BOOTP

The automatic configuration feature allows the Cisco 700 series router to obtain a configuration file from a remote server using BOOTP protocol. Upon power-up, the router examines the Auto_Config flag stored in NVRAM. If the Auto_Config flag is FALSE, the router initiates a BOOTP request for an IP address from the BOOTP server. If the BOOTP server finds a matching MAC address, the server returns an IP address and a configuration file name. The router initiates TFTP to download a configuration file.

After successfully loading the configuration file, the router sets the Auto_Config flag to TRUE in NVRAM. In subsequence power-up operations, if the Auto_Config is TRUE, the router does not initiate BOOTP. The set default command resets the Auto_Config flag to FALSE.

The automatic configuration feature only works on the Ethernet interface. If the router does not receive a BOOTP response from the server within 5 seconds of the first BOOTP request, the router gives up and the Internet Protocol (IP) information must be configured manually.

DHCP Server

DHCP automates IP addressing and reduces the number of IP addresses a site might require. Cisco 700 series routers can function as a dynamic host configuration protocol (DHCP) server. (The router can also act as a DHCP relay, but not both at the same time.)

When a DHCP server is enabled and configured, it assigns and manages IP addresses from a specified address pool to DHCP clients. The options supported by this server are sufficient for Windows 95 clients. If more IP addresses or options are required, a commercial DHCP server (for example, Windows NT) should be used.

DHCP Relay Agent

Cisco 700 series routers can function as a DHCP relay agent. (The router can also act as a DHCP server, but not both at the same time.) When configured, Cisco 700 series routers relay DHCP requests and responses between DHCP clients and a specified DHCP server. Use the set dhcp relay command, where the IP address is the address of the server to which DHCP requests will be forwarded.

Port Address Translation

Cisco 700 series routers support port address translation (PAT) allowing a designated private IP network to communicate with the outside world. When configured, Cisco 700 series routers translate source addresses from an IP private network to a single, global, unique IP address before forwarding the packets to the outside world.

IP Address Assignment Through IPCP

The router can be assigned an IP address from the remote device using Internet Protocol Control Protocol (IPCP) address negotiation. The implementation is based on RFC1332. It supports IPCP options 1 and 3. (It does not support option 2, TCP/IP Header Compression.) IP unnumbered is also supported. IPCP address negotiation is on by default in any profile configured for IP routing. This feature does not support assigning addresses to remote devices.

IP RIP Update Linkup

The IP Routing Information Protocol (RIP) Update Linkup option for IP RIP enables better interoperability between the router and Cisco IOS software. If the IP RIP update is set to link up for a WAN profile, the IP RIP data is sent as soon as a connection is established and again every 30 seconds for as long as the connection exists. If the connection does not exist, no RIP data is sent. This prevents the line from being brought up by RIP packets where the update is periodic.

Force Dynamically Negotiated IP Addresses

When this feature is on, the negotiated IP address is assigned to the Standard or the user-defined profile (whichever profile is used to place the call). It also tells the software to try any IP address configured for this profile in the IP address negotiation.

When this feature is off and the internal profile does not contain an address, the negotiated IP address is automatically assigned to the Internal profile by the system. If the Internal profile contains an address, it will be assigned to the user-defined profile from which the call was initiated. The parameters are set using the set ppp address negotiation command.

Fee Pulse Mode

Fee Pulse Mode manages the ISDN connection based on the paid periods of time. If other thresholds indicate a call should be dropped due to low traffic, Fee Pulse Mode maintains the connection until the current paid period has expired. Therefore, you are not paying full price for part of a connection period. This feature is available only for NET3 (same as ETSI) switch types, and you must subscribe to Advice of Charge-During Active Call (AOC-D) supplementary service.

If the feature is enabled and idle time expires, the router checks the remaining time in the current paid period. The router maintains the connection until the end of the paid period, minus the disconnect time required to terminate the PPP and ISDN links.

If the idle time expires too close to the end of the paid period to close the connections before the end of the paid period, the router extends the connection to the end of the next paid period, minus disconnect time.

Call Transfer

Cisco 700 series routers can transfer an active voice call to another telephone number. For more information on configuring call transfer, refer to the chapter "ISDN and Analog Telephone Devices."

Call Forwarding (Sweden and Finland)

Call forwarding supplementary service has been added for Sweden and Finland (NET3). Call forwarding is implemented differently in the European countries. The version implemented here uses keypad procedures and is only partly ETSI-compliant.

Three-Way Call Conferencing

Cisco 700 series routers can add another party to an active voice call. You can connect three people using this feature. For more information on configuring three-way call conferencing, refer to the chapter "ISDN and Analog Telephone Devices."

Data Call Button (Cisco 770 Series Router Only)

The data call button, located on the front panel of Cisco 770 series routers, provides an alternative to establishing or disconnecting data calls through the command interface. The data call button performs in Make or Break mode. If no B channels are connected for data, and at least one B channel is available, the button works in Make mode and attempts to establish a call. When at least one B channel is connected for data, the button works in Break mode, disconnecting the call. Table 1-1 summarizes the operation of the Cisco 770 series data call button.

Table  1-1: Cisco 770 Series Data Call Button Operation Summary Current Status  No call up

1 data call only

2 data calls

1 voice call only

2 voice calls

1 data and 1 voice call

Switch Operation  Make a data call  Disconnect the data call  Disconnect both data calls  Make a data call  No action  Disconnect the data call

Second Number Fail-Over

Cisco 700 series routers can place a call to a backup telephone number when an outgoing call is not successful because the receiving side is busy or does not answer. Cisco 700 series routers support second number fail-over for manual and auto-data calls. The parameters are set using the set backupnumber command.

IPX Support

To improve performance, only the services defined in Novell IPX Router Specification 107-000029-001 are supported, and a maximum of 400 IPX route entries and 200 IPX SAPs are allowed. If you need more route and SAPs entries, software Release 4.0(1) and higher supports set ipx sap helper and set ipx gateway commands.

IPX Ping

The IPX ping feature allows you to determine if the specified IPX node is alive and reachable. Cisco 700 series routers only generate Novell IPX format ping packets when IPX ping is sourced from the router. The response by the Cisco 700 series router to a Novell IPX ping packet is in Novell IPX format. The response by the Cisco 700 series router to Cisco IPX ping packets is in Cisco IPX format. (If a Novell NetWare server is pinged, the specified server must be running the IPXPING.NLM to respond to the ping. This NLM [NetWare Loadable Module] is part of an IPX Upgrade [IPXRTR.EXE] to NetWare servers.) The parameters are set using the ipxping address command.

Default IPX Route

Cisco 700 series routers can route IPX packets to an established default gateway if no learned or static route can be found for the destination in the routing table. The parameters are set using the set ipx gateway command.

IPX SAP Filtering

IPX SAP filtering controls which services are added to the Cisco 700 series router Service Advertisement Protocol (SAP) table. You can specify filters per type range or socket range. Before a service is added to the SAP table, the service type and socket number for the service is checked against the existing filters. If the type and socket ranges match any of the existing filters, service is permitted or denied based on the action specified in the action field of the filter. Otherwise (no filter matches the incoming service), the service is blocked. If no filter is configured, all SAPs are permitted. The parameters are set using the set ipx filter sap command. The maximum number of SAP filters is 20.

Service Advertising Protocol (SAP) Helper

Cisco 700 series routers can forward to a specified router or server (instead of responding to them) all SAP get-nearest-service request broadcast packets and get general service request broadcast packets to an IPX router or IPX server as unicast packets. Cisco 700 series routers forward these request packets without looking in the local SAP table. The parameters are set using the set ipx sap helper command.

Route Summarization

Route summarization is provided on IP RIP Version 2 (only Version 2 provides the required subnet mask information). When route summarization is set to auto, the router summarizes routes that cross the classful boundary to a single route based on the classful boundary. When route summarization is set to off, the router uses the normal RIP Version 2 routes with variable-length subnet masks (VLSMs). The default setting for RIP summarization is OFF. Use the set ip rip summarization command to control this feature.

Data Compression

Cisco 700 series routers support data compression using the compression algorithm QIC-122 standard, Stacker LZS. Data compression is a software configuration option that optimizes the ISDN line bandwidth. Packets are compressed before being sent onto the ISDN line. After they arrive at their destination, the packets are decompressed and sent on to the remote LAN.

Security Features

Cisco 700 series routers provide the following security features:

PPP authentication support, including Password Authentication Protocol (PAP) and Challenge Handshake Authentication Protocol (CHAP)

Password security for local and remote configuration access

IP filtering based on source and destination addresses, source and destination ports, and packet types

Leased Line Authentication Requirement Removed

For 64-kbps or 128-kbps leased line connections, previous versions of the software required PAP/CHAP authentication to identify the corresponding profile. In software Release 4.0(1) and higher, the authentication sequence is no longer required for leased line connections.

To eliminate the need for authentication, a user-defined profile named leasedline must be present and defined. If this profile is not present upon call connect, the router requires authentication to select the correct profile. If the call cannot be authenticated, the router defaults to the Standard profile.

Within the user-defined profile called leasedline, verify that PPP authentication is set to none. The switch types that support this feature are PERM64 and PERM128.

SNMP Support

Cisco 700 series routers support Simple Network Management Protocol (SNMP).

SNMP Community Names

Cisco 700 series routers support the following SNMP community names:

Public,proxy,private,regional,core

These community names are read-only and cannot be changed. Cisco 700 series routers do not support SNMP set commands.

Supported MIBs

Cisco 700 series routers support the following SNMP Management Information Bases (MIBs):

MIB II

IEEE 802.1d Bridge MIB

MIB II

Cisco 700 series routers support MIB II standards as follows:

System

Interfaces (all objects, except that a connection is considered to be an interface)

Address translation

Internet Protocol (IP)

Transmission Control Protocol (TCP)

Internet Control Message Protocol (ICMP)

User Datagram Protocol (UDP)

SNMP

IEEE 802.1d Bridge MIB

Cisco 700 series routers support IEEE 802.1d MIB standards as follows:

Base

Transparent bridging

Static

Supported RFCs

Cisco 700 series routers support the following Request For Comments (RFC) documents:

RFC 1058--Routing Information Protocol (RIP)

RFC 1332--PPP Internet Protocol Control Protocol (IPCP)

RFC 1334--PPP Authentication Protocols

RFC 1541--Dynamic Host Configuration Protocol (DHCP)

RFC 1552--PPP Internetwork Packet Exchange

RFC 1570--PPP Link Control Protocol (LCP) Extensions

RFC 1582--Extensions to RIP to Support Demand Circuits

RFC 1618--PPP Over ISDN

RFC 1638--PPP Bridging Control Protocol (BCP)

RFC 1661--Point-to-Point Protocol (PPP)

RFC 1717--PPP Multilink Protocol (MP)

RFC 1723--Routing Information Protocol (RIP) Version 2--Carrying Additional Information

Router Panels

Figure 1-1 and Figure 1-2 show the front panels of Cisco 760 series and Cisco 770 series routers. Instructions for connecting the routers are provided in the Cisco 700 Quick Reference Guide.

- List the installation steps for a cisco 700 series router

Starting Point

This section shows how to set the router to default values and provide basic configuration information used in all configurations. All of the example configurations assume default values unless otherwise indicated.

Step 1 Use the set default command as follows to be sure that you begin with all default values when configuring your router:

> set def

System-level parameters and the parameters in the permanent profiles are set to their default values. Existing user-defined profiles are deleted and the router reboots.

Step 2 Enter the set directorynumber command (a ten-digit local number with no spaces or dashes) to set the ISDN directory numbers.

>set 1 directory number 4085551234

>set 2 directory number 4085551235

You are ready to set Service Profile Identifier (SPID) automatic detection. A SPID is a number provided by the ISDN carrier to identify the line configuration of the BRI service. Each SPID points to line setup and configuration information.

Setting a SPID is described in the section "Setting SPID Autodetection" in this chapter (recommended if you are connecting to a service provider where the ISDN switch type is DMS-100 or National ISDN-1 (NI-1)) or to enter the SPIDs manually, as described in the section "Setting SPIDs Manually" in this chapter.

Setting SPID Autodetection

If the service provider ISDN switch type is DMS-100 or National ISDN-1 (NI-1), the router supports an automatic SPID detection feature. This section describes how to set the autodetection feature. If the switch type is 5ESS Custom PPP, do not enter SPIDs; go to the next section.

Enable automatic SPID detection feature as follows:

>set autodetection on

Once you enable the autodetection feature, wait for the router to complete the process. This process might take several minutes to complete. When autodetection is successful, the following messages display:

>L76   1             Auto Spid Detect Successful 5ESS

>L76   1  055512340  Auto Spid Detect Successful

>L76   2  055512350  Auto Spid Detect Successful

No additional procedures are required. You can now enter specific configuration information.

If autodetection fails, the following message displays:

>L84  Manually enter spids and set autodetection off

Setting SPIDs Manually

SPIDs can be detected automatically using the set autodetection command, or SPIDs can be entered manually, as described in this section.

If the service provider switch type is 5ESS Custom PPP, you do not need to enter SPIDs; go to the next section. If the service provider switch type is DMS-100, National (NI-1), or 5ESS Multipoint, continue with this section.

To enter the SPIDs assigned by your ISDN service provider, take the following steps:

Step 1 Enter the set switch command to configure the ISDN switch type that is being used with your ISDN line:

> set switch 5Ess

Step 2 Enter the set spid command to set the router's SPID numbers:

> set 1 spid 0510198765430

> set 2 spid 0510187654320

You are ready to configure the router for a specific routing environment.

An AT&T 5ESS switch can support up to eight SPIDs per BRI line. Because multiple SPIDs can be applied to a single B channel, multiple services can be supported simultaneously. For example, the first B channel can be configured for data, and the second B channel can be configured for both voice and data. In this scenario, the second B channel can support an ISDN telephone in addition to supporting data connections. For 5ESS switches, the SPID is usually the 10-digit ISDN number beginning with "01" and ending with "0." For example: ISDN number, 4085551212; SPID, 0140855512120. (There is no standard format for SPIDs. As a result, SPID values can vary, depending on the switch vendor and the carrier.)

DMS-100 and NI-1 switches support only two SPIDs, with only one B channel per SPID. If both B channels will be used for data only, enter the two SPIDs (one for each B channel). An issue comes up when trying to run data and voice over the same B channel. Assuming the first SPID is applied to the first B channel for data traffic and is limited to that B channel only, this leaves only one other SPID for the second B channel.

Consequently, the second B channel can be used for either data or voice, but not both simultaneously. The absence or presence of the second SPID in the configuration dictates whether the second B channel can be used for data or voice. This is an example of SPID values for DMS-100 and NI-1 switches: ISDN number, 4085551212; SPID 1, 408555121201; SPID 2, 408555121202. In this case the SPID is the 10-digit ISDN number ending with a "01" for SPID 1 and a "02" for SPID 2.

Bridging with a Cisco 700 Series Router

This section describes how to bridge a Cisco 700 series router over an ISDN line to another router. Bridging is used in cases where you do not need a lot of filtering to manage the network. Basically, you are relying on the nodes on the LAN side of the router to determine if a packet should be accepted or dropped. (If you turn routing on, you can filter the packets on the WAN side, reducing your traffic on the LAN side.)

--------------------------------------------------------------------------------

Note Bridging over an ISDN line is not an efficient use of ISDN bandwidth. Routing over the ISDN line helps optimize ISDN bandwidth by reducing traffic to the WAN.

--------------------------------------------------------------------------------

You are going to establish a basic connection with another router, relying primarily on the defaults. In this example configuration, a simple user-defined profile is created in your Cisco 700 series router to bridge over an ISDN network to another router.

By default, the Cisco 700 series router automatically "learns" the MAC addresses that exist locally and remotely across the WAN. The router stores the MAC addresses in a MAC address table, so it knows if the unicast packets should remain on the LAN or forwarded across the ISDN line. In a bridging scenario, the router does not need an IP address and bridging occurs regardless. The IP address is used when the router is being managed remotely by a Telnet session or participating in SNMP.

Both sides of the WAN must be configured with PPP host names, secrets, and passwords for authentication. Each profile must also include dialing information.

Cisco 700 Series Router Bridging Instructions

The IP address and subnet mask are not entered. IP address assignments are not necessary in a bridged network; they are used only if the router is being pinged or accessed through Telnet or SNMP. Take the following steps to configure the Cisco 700 series router to bridge to the CPA902 router at the central site:

Step 1 Enter the set system command to create a system name for the Cisco 700 series router:

>set system CPA765

CPA765>

This is a system-level command. The system name is case sensitive and can be from 1 to 60 characters. Notice that the prompt changes to reflect the name of the router.

Step 2 Enter the set user command to create a user-defined profile:

CPA765>set user CPA902

CPA765:CPA902>

The user-defined profile matching the central router user name is created. Notice that the prompt changes to reflect the name of the router and the profile. The appearance of the profile name indicates you are working in the specified profile. You can have up to 16 profiles.

Step 3 Enter the set active command to activate the user-defined profile:

CPA765:CPA902> set active

When you initiate a call, the router reviews the user-defined profile information to determine where to send the data. The router reviews the active profiles, looking for a match. When a matching profile is found, communications are established, based on the profile.

The router slows down when a call is initiated and the router has to search a large number of active profiles to discover the profile to use for establishing a call. If there are only two active profiles, the response time is faster. When receiving calls, the state of the profile does not matter. The router searches all profiles to find a match.

Step 4 Enter the set ppp secret client command to configure the client secret for CHAP authentication:

765:CPA902> set ppp secret client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

This is required if the router you are connecting to requires a secret for CHAP authentication. If the router does not require a secret, this entry is not necessary. Note that despite the prompt indicating a password entry, the entry is actually a secret.

When your router attempts communications with another router, it receives the host secret of the other router. Your router looks at the value in the profile it has for the calling router. If the secrets match, authentication passes and communications are allowed. If the secrets do not match, the call is dropped. This example uses cisco for the secret. The characters are not echoed on the terminal when you type the secret.

Step 5 Enter the set ppp password client command to configure the client password for PAP authentication:

765:CPA902> set ppp password client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

This is required if the router you are connecting to requires a PAP password for authentication. If the router does not require a PAP password, this entry is not necessary.

The PPP client password is compared to the connected router host password during PAP authentication. This example uses cisco for the password. The difference between a password and a secret is that a secret is encrypted, but a password is not encrypted.

Step 6 Enter the reboot command to enable your configuration changes:

CPA765:CPA902> reboot

Just as you reboot your PC to activate major configuration changes, the router is rebooted.

CPA902 Bridging Command Summary

The following commands are all that is necessary to configure the CPA902 router at the central site for user-defined profile bridging:

CPA902> set user CPA765

CPA902> set ppp password host <cisco>

CPA902> set ppp secret host <cisco>

CPA902> set ppp password client <cisco>

CPA902> set ppp secret client <cisco>

The commands create a user profile for your router and uses the defaults. If there is no profile for a router attempting to connect to a Cisco 700 series router (or a CPA902) using software Release 4.1(1) or higher, the call is dropped.

Routing TCP/IP with Cisco 700 Series Router to an ISP

This section describes how to configure a Cisco 700 series router to route to an ISP using Internet Protocol (IP). Figure 3-2 illustrates the configuration used in this example. By default, PPP incoming authentication is on and outgoing authentication is off.

For an incoming call with authorization required, the router asks the calling router to supply a user name, password and secret for the specified authorization type (CHAP or PAP). If authorization fails, the call is dropped. If authorization passes, the corresponding profile is activated.

For an outgoing call with authorization required, the router asks the switch it calls to identify itself. (If you call an Ascend device, the Ascend device drops the call if your router attempts authentication. In this case, it is best to set the authentication to none.)

The example also uses unnumbered IP addresses, a common practice used to conserve IP addresses.

In this example, a user-defined profile named central is created, representing the ISP router. Your router uses the central profile to initiate the call to the ISP router.

When the Cisco 700 series router calls the ISP router, it sends the ISP the PPP host name, the CHAP secrets, and PAP passwords, depending upon what the other router requires to authenticate the call.

If authentication is required and the call fails authentication or a profile cannot be found, the call is dropped. If authentication is not required and a profile cannot be found, the router uses the Standard profile to establish the connection.

Routing a Cisco 700 Series Router to an ISP Instructions

Do the following to configure the system-level commands for a remote Cisco 700 series router to connect to an ISP:

Step 1 Enter the set system command to configure your router with a system name:

> set system 765

The system name is case sensitive and can be from 1 to 60 characters used to identify the router.

Take the following steps to configure the permanent profile, LAN:

Step 1 Enter the change user command to select the LAN profile:

765> cd lan

The LAN profile is a permanent profile that manages the LAN (10BaseT) side of your router. You do not have to create this profile or activate it.

Step 2 Enter the set ip address command to configure the IP address for the 10BaseT port (or unmanaged hub):

765:LAN> set ip address 172.16.17.9

Step 3 Enter the set ip netmask command to configure the IP subnet mask for the 10BaseT port (or unmanaged hub):

765:LAN> set ip netmask 255.255.255.248

IP networks can be divided into smaller units called subnets. Subnets provide extra flexibility for network administrators. The number of bits used for the subnet address varies. To specify how many bits are used, IP provides the subnet mask.

Step 4 Enter the set ip routing command to enable IP routing on the 10BaseT port (or unmanaged hub):

765:LAN> set ip routing on

Step 5 Enter the change user command without any parameters to return to the system level:

765:LAN> cd

Take the following steps to configure the user-defined profile, central:

Step 1 Enter the set user command to create a user-defined profile named central for the router at the ISP:

765> set user central

Step 2 Enter the set number command to configure the ISDN telephone number the router dials to connect to the ISP:

765:central> set number 5558011

Step 3 Enter the set ppp password client command to configure the client secret if needed:

765:central> set ppp password client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

The router only supplies the PAP password or the CHAP secret it is asked for. If another router asks the Cisco 700 series router for the password, it sends only the password, not the secret. Setting all the passwords and secrets avoids a failed connection.

Step 4 Enter the set ppp secret client command to configure the client secret if needed:

765:central> set ppp secret client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

Step 5 Enter the set ip routing command to enable IP routing to the ISP:

765:central> set ip routing on

Previously, you turned IP routing on for the LAN side of your router. Here, you are enabling the IP routing engine on the WAN side of the router.

Step 6 Enter the set ip route and set ip netmask commands to use IP unnumbered routing.

765:central> set ip route destination 0.0.0.0/0 gateway 0.0.0.0 propagate on

765:central> set ip netmask 0.0.0.0

765:central> set bridge off

If you are not using IP unnumbered routing (the ISP has assigned a specific IP address to you), the following example shows the commands:

765:central> set ip address 172.16.125.2

765:central> set ip netmask 255.255.255.248

765:central> set ip route destination 0.0.0.0/0 gateway 172.17.125.1 propagate on

These are filtering parameters.

Step 7 Enter the set timeout command to terminate the ISDN connection after 360 seconds of no ISDN line activity:

765:central> set timeout 360

Your router listens to the ISDN connection. If there is no traffic for 360 seconds (6 minutes), your router drops the line. If you are paying for the time you are connected through the ISDN line, dropping the line when there is no traffic saves you money.

Step 8 Enter the set active command to activate the user-defined profile:

765:central> set active

Step 9 Enter the reboot command to enable your configuration changes:

765:central> reboot

Just like you would reboot your PC to activate major configuration changes, the router is rebooted.

Routing IP On-Demand

This section describes how to configure a remote Cisco 765 router and a central site CPA902 router for on-demand IP routing using PPP.

PPP addresses issues that include the assignment and management of IP addresses, asynchronous (start/stop) and bit-oriented synchronous encapsulation, network protocol multiplexing, error detection, and option negotiation.

PPP addresses these issues by providing an extensible Link Control Protocol (LCP) and a family of Network Control Protocols (NCPs) to negotiate optional configuration parameters and facilities. PPP supports IP and IPX.

Routing IP On-Demand--Example Configuration

In this example, the Cisco 765 router is used to establish a connection through the ISDN service provider to the corporate network at a central site. A CPA902 represents the router at the central site (although the central site router could be any router).

Routing IP On-Demand Instructions

Take the following steps to configure the Cisco 700 series router for on-demand IP routing using PPP:

Step 1 Enter the set system command to configure the router with a system name.

> set system remote765

remote765>

This is a system-level command. The system name is case sensitive and can be from 1 to 60 characters. Notice that the prompt changes to reflect the name of the router.

Step 2 Enter the set multidestination command:

remote765> set multidestination on

The set multidestination command enables the router to dial to multiple destinations. This is not necessary if you are only calling one site.

Take the following steps to configure the permanent profile, LAN:

Step 1 Enter the cd command to configure the LAN profile:

remote765> cd LAN

The user-defined profile matching the central router user name is created. Notice that the prompt changes to reflect the name of the router and the profile. The appearance of the profile name indicates you are working in the specified profile. You can have up to 16 profiles.

Step 2 Enter the set ip address command to configure the IP address of the connection to the local Ethernet LAN:

remote765:LAN> set ip address 172.16.17.9

Step 3 Enter the set ip netmask command to configure the IP subnet mask of the connection to the local Ethernet LAN:

remote765:LAN> set ip netmask 255.255.255.0

IP networks can be divided into smaller units called subnets. Subnets provide extra flexibility for network administrators. The number of bits used for the subnet address varies. To specify how many bits are used, IP provides the subnet mask.

Step 4 Enter the set ip routing command to enable IP routing to the local Ethernet LAN (the 10BaseT port):

remote765:LAN> set ip routing on

Turning IP routing on enables the IP routing engine.

Step 5 Enter the set ip rip update command to send IP Routing Information Protocol (RIP) packets to the local Ethernet LAN every 30 seconds:

remote765:LAN> set ip rip update periodic

The RIP sends information about your router to other routers. Each entry in a RIP routing table provides a variety of information, including the ultimate destination and the next hop on the way to that destination.

Take the following steps to configure the user-defined profile, central902:

Step 1 Enter the change user command without any parameters to return to the system level:

remote765:LAN> cd

Step 2 Enter the set user command to create a user-defined profile for the central site, matching the system name of the central site router:

remote765> set user central902

When receiving a call, the Cisco 765 router searches its user profiles, looking for a match between the user-defined profile name and the system name of the calling router. This is part of the authentication process. When it finds a match, it uses the parameters in the profile to communicate with the calling device. If it does not find a match, the Cisco 765 router drops the call.

Step 3 Enter the set ppp password client command to configure the PAP password:

765:central> set ppp password client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

The router only supplies the PAP password or the CHAP secret it is asked for. If another router asks the Cisco 700 series router for the password, it sends only the password, not the secret. Setting all the passwords and secrets avoids a failed connection.

Step 4 Enter the set ppp secret client command to configure the client secret:

765:central> set ppp secret client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

Step 5 Enter the set ip address command to configure the IP address of the ISDN (WAN) interface (this could also be unnumbered):

remote765:central902> set ip address 10.48.125.7

Step 6 Enter the set ip netmask command to configure the IP subnet mask of the ISDN interface used by the ISDN service provider:

remote765:central902> set ip netmask 255.255.255.0

Step 7 Enter the set ip routing command to enable IP routing to the central site CPA902:

remote765:central902> set ip routing on

Step 8 Enter the set ip rip update command to send IP RIP packets to the central site CPA902 only when a change occurs in the RIP routing table:

remote765:central902> set ip rip update demand

Demand is unique to Cisco 700 series routers. It is only used where you are connecting to another router running Cisco IOS-700. If you are connecting to any other router, you should use static or periodic RIP update.

Step 9 Enter the set ip route command to configure a static route to the destination network and the IP address of the gateway:

remote765:central902> set ip route destination 0.0.0.0/0

gateway 10.32.125.4

Use 0.0.0.0 if you are doing unnumbered IP routing.

Step 10 Enter the set number command to configure the ISDN telephone number the router dials to connect to the central site CPA902:

remote765:central902> set number 5550143

Step 11 Enter the set timeout command to terminate the ISDN connection after 360 seconds of no ISDN

line activity:

remote765:central902> set timeout 360

Your router listens to the ISDN connection. If there is no traffic for 360 seconds (6 minutes), your router drops the line. If you are paying for the time you are connected through the ISDN line, dropping the line when there is no traffic saves you money.

Step 12 Enter the set active command to activate the user-defined profile:

765:central> set active

When you initiate a call, the router reviews the user-defined profile information to determine where to send the data. The router reviews the active profiles, looking for a match. When a matching profile is found, communications are established, based on the profile.

Step 13 Enter the reboot command to reset the router and enable your configuration changes:

remote765:central902> reboot

Central Site IP Routing Command Summary

Following is the command summary for configuring the central site CPA902 router for on-demand IP routing with PPP:

set system central902

set multidest on

set ppp auth in chap

set ppp secret client

cd lan

set ip address 172.15.1.100

set ip netmask 255.255.255.0

set ip routing on

set ip rip update periodic

cd

set user remote765

set ppp auth out chap

set ppp secret host

set bridging off

set ip address 10.48.125.4

set ip netmask 255.255.255.0

set ip routing on

set ip rip update demand

set number 5553693

set timeout 360

set active

reboot

Routing IP and IPX On-Demand--Example Configuration

The Cisco 765 router is used to establish a connection through the ISDN service provider to the corporate network at a central site using PPP. A CPA902 represents the router at the central site; however, it could be any router. The Cisco 765 router has three permanent profiles: LAN, Internal and Standard. This example uses the LAN profile and a user-defined profile.

On-Demand IP and IPX Routing with PPP Instructions

Take the following steps to configure the remote Cisco 765 router for on-demand IP and IPX routing using PPP:

Step 1 Enter the set system command to configure the router with a system name:

> set system remote765

Step 2 Enter the set multidestination command to enable the router to dial to multiple remote destinations:

remote765> set multidestination on

This is not necessary if you are only calling one site.

Take the following steps to configure the permanent profile, LAN:

Step 1 Enter the change user command to configure the LAN profile:

remote765> cd LAN

Step 2 Enter the set ipx routing command to enable IPX routing to the local Ethernet LAN:

remote765:LAN> set ipx routing on

Step 3 Enter the set ipx network command to configure the IPX network number for the local Ethernet LAN:

remote765:LAN> set ipx network 1478

Step 4 Enter the set ipx framing command to configure IPX framing for the local Ethernet LAN:

remote765:LAN> set ipx framing 802.2

Framing is the process of inserting start and stop signals before and after data being transmitted. These framing elements delimit the data. They allow the receiver to determine the sender's timing, because the duration of the start bit indicates the bit interval size being used by the sender.

A framing error occurs when the receiver incorrectly identifies the start and stop signals, or the framing, in a transmission.

Step 5 Enter the set ipx rip update command to send IPX RIP updates onto the local Ethernet LAN every 60 seconds:

remote765:LAN> set ipx rip update periodic

The RIP sends information about your router to other routers. Each entry in a RIP routing table provides a variety of information, including the ultimate destination and the next hop on the way to that destination.

Step 6 Enter the set ip address command to configure the IP address of the router connection to the local Ethernet LAN:

remote765:LAN> set ip address 172.16.17.9

Step 7 Enter the set ip netmask command to configure the IP subnet mask of the router's connection to the local Ethernet LAN:

remote765:LAN> set ip netmask 255.255.255.0

Step 8 Enter the set ip routing command to enable IP routing to the local Ethernet LAN:

remote765:LAN> set ip routing on

Step 9 Enter the set ip rip update command to send IP RIP packets onto the local Ethernet LAN every 30 seconds:

remote765:LAN> set ip rip update periodic

Step 10 Enter the change user command without parameters to return to the system level:

remote765:LAN> cd

Take the following steps to configure the user-defined profile, central902:

Step 1 Enter the set user command to create a user-defined profile for the central site CPA902:

remote765> set user central902

Step 2 Enter the set ppp password client command to configure the PAP password:

765:central> set ppp password client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

The router only supplies the PAP password or the CHAP secret it is asked for. If another router asks the Cisco 700 series router for the password, it sends only the password, not the secret. Setting all the passwords and secrets avoids a failed connection.

Step 3 Enter the set ppp secret client command to configure the client secret:

765:central> set ppp secret client

Enter new Password: <cisco>

Re-Type new Password: <cisco>

Step 4 Enter the set ipx routing command to enable IPX routing to the central site CPA902:

remote765:central902> set ipx routing on

Step 5 Enter the set ipx network command to configure the network number for the ISDN segment between the routers:

remote765:central902> set ipx network 32125

Step 6 Enter the set ipx rip update command to send IPX RIP packets to the central site CPA902 only when changes are made to the routing tables:

remote765:central902> set ipx rip update demand

Step 7 Enter the set ipx spoof command to have the router respond to the local server's watchdog packets on behalf of the remote workstation:

remote765:central902> set ipx spoof 10

Cisco IOS software provides a feature that allows the router to respond to a server's Watchdog requests on behalf of a remote client. This process is called NCP or IPX spoofing. Benefits of IPX spoofing include:

Local acknowledgment of queries

No link responses

Efficient use of the link

Minimal operator intervention

When using IPX spoofing, NetWare servers are lead to believe that a session is still active when it is not. When the number of IPX or SPX sessions are limited, this can cause connectivity problems by denying logins to legitimate users.

Step 8 Enter the set ip address command to configure the IP address for the ISDN (WAN) interface:

remote765:central902> set ip address 10.32.125.7

Step 9 Enter the set ip netmask command to configure the IP subnet mask for the ISDN (WAN) segment:

remote765:central902> set ip netmask 255.255.255.0

Step 10 Enter the set ip routing command to enable IP routing to the central site CPA902:

remote765:central902> set ip routing on

Step 11 Enter the set ip rip update command to send IP RIP packets to the central site CPA902 only when a change occurs in the RIP routing table:

remote765:central902> set ip rip update demand

The RIP sends information about your router to other routers. Each entry in a RIP routing table provides a variety of information, including the ultimate destination and the next hop on the way to that destination.

Step 12 Enter the set ip route command to configure a static route to the destination network:

remote765:central902> set ip route destination 0.0.0.0/0

gateway 10.32.125.4

Step 13 Enter the set number command to configure the ISDN telephone number the router dials to connect to the central site CPA902 router:

remote765:central902> set number 5550143

Step 14 Enter the set timeout command to terminate the ISDN connection after 360 seconds of no ISDN line activity:

remote765:central902> set timeout 360

Your router listens to the ISDN connection. If there is no traffic for 360 seconds (6 minutes), your router drops the line. If you are paying for the time you are connected through the ISDN line, dropping the line when there is no traffic saves you money.

Step 15 Enter the set active command to activate the user-defined profile:

765:central> set active

When you initiate a call, the router reviews the user-defined profile information to determine where to send the data. The router reviews the active profiles, looking for a match. When a matching profile is found, communications are established, based on the profile.

Step 16 Enter the reboot command to reset the router and enable your configuration changes:

remote765:Central902> reboot

Just like you would reboot your PC to activate major configuration changes, the router is rebooted.

Central Site On-Demand IP and IPX Routing with PPP Commands

Following is the command summary for configuring the central site CPA902 router for on-demand IP and IPX routing using PPP:

set system central902

set multidestination on

set ppp auth in chap

set ppp secret client <cisco>

cd lan

set ipx routing on

set ipx network 73146

set ipx framing 802.2

set ipx rip update periodic

set ip address 172.15.1.100

set ip netmask 255.255.255.0

set ip routing on

set ip rip update periodic

cd

set user REMOTE765

set bridging off

set ipx network 32125

set ipx routing on

set ipx rip update demand

set ipx spoof 10

set ip address 10.32.125.4

set ip netmask 255.255.255.0

set ip routing on

set ip rip update demand

reboot

Define a profile and describe the types of profiles used with the Cisco 700 series

Profile Overview

There are two modes in which you can set parameters, the system mode and the profile mode. System mode parameters affect the configuration on a global level. Profiles are individual parameters, maintained in configuration sets. Profile mode parameters affect how the router handles the connection to a device.

You do not have to reconfigure the router every time you connect to a different device. Instead of using one set of configuration parameters for all devices, you can use different profiles to communicate with a variety of devices.

For example, you can create a user-defined profile called 2500 that contains the parameters to be used when communicating with a Cisco 2500 series router over the WAN. You can customize your Cisco 700 series router to maintain up to 17 user-defined profiles. Profiles are saved in Cisco 700 series router's nonvolatile random access memory (NVRAM).

In addition to user-defined profiles, there are three permanent profiles, Internal, LAN, and Standard. The Internal profile stores parameters used to communicate between the LAN and WAN ports on the Cisco 700 series router. The LAN profile stores parameters that configure the LAN port on the router. The Standard profile is the default profile. If authentication is not required and the destination device you are connecting to does not have a user-defined profile, the router uses the Standard profile.

Profiles and Connections

Profiles are either active or inactive. An active profile creates a virtual connection to the remote device associated with the profile. A virtual connection is a connection without physical channels. After creating a virtual connection, an on-demand call can be made to the associated remote device to establish a physical connection.

A physical connection is a dynamically created pipeline of packets from the Cisco 700 series router to a switch on the WAN. All connections are associated with the profile that defines the configuration of the connection.

Virtual and physical connections behave similarly; the difference is that physical connections forward packets to the WAN. Virtual connections monitor packet traffic on the LAN until a demand filter "sees" that a packet is destined for the WAN and initiates a call to the switch, opening the physical connection. Once the call is established, the virtual connection becomes an active physical connection, and the packets move through the pipeline.

System and Profile Parameters

The system is composed of both system mode parameters, user-defined profiles, and permanent profiles. System mode parameters can be changed only in system mode. The prompt indicates you are in system mode by displaying nothing or the router name. An example of the prompt is shown below:

Router_name>

If you are in profile mode, the profile name appears on the prompt, separated from the system name by a colon (:). An example of the prompt is shown below:

Router_name:Profile>

All profiles are based on the profile template and inherit the system-level values. When you create a new profile, its default values are taken from the profile template.

System Mode Parameter Set

System mode parameters affect the router as a system. nonexistent-markerlists the system parameters.

System Parameter Set

| Caller ID parameters | Call waiting | PPP1 parameters |
| --- | --- | --- |
| Date and time | Country group | Screen length |
| Directory number | Address age time | Screen echo |
| Delay time | Local and remote access | SNMP2 parameters |
| Forwarding mode | Phone 1 and 2 | SPIDs3 |
| Multidestination dialing | PPP client password | Switch type |
| Numbering plan | PPP client secret | System password |
| Patterns | Voice priority | Power Source 1 detect |
| Passthru |  | System name |

Profile Mode Parameter Set

Changes made to profile mode parameters in system mode affect the profile template. When a profile is created, it inherits the matching system mode parameters from the profile template. Any changes to parameters in profile mode apply only to that profile. Changes made to profile parameters in system mode are stored in the profile template. When you use the set profile command to create a user-defined profile, the default parameters for the new profile are taken from system mode.

Profile Parameters

| Bridging | Line speed | PPP authentication (outgoing) |
| --- | --- | --- |
| Ringback number | Auto calling | All IP parameters, including filters |
| Compression | Demand | PAP password (client and host) |
| Passthrough | Timeout | All IPX parameters, including filters |
| Learning | Called number | CHAP secret (client and host) |
| Subnet mask | Encapsulation | Bridge filters (address, type, and user-defined) |
| Protocol | Loopback |  |

Permanent Profiles

Cisco 700 series routers contain three permanent profiles. Permanent profiles can be modified, but they cannot be deleted. The permanent profiles are as follows:

| LAN | Determines how data is passed from the router to the LAN. This profile is commonly used for connections made directly to the local network. |
| --- | --- |
| Internal | Determines how data is passed between the bridge engine and the IP/IPX router. |
| Standard | A default profile that supports bridging, but it does not support routing. If authentication is set to none and a profile does not exist for the WAN switch, the router uses the Standard profile by default. If authentication is required and no profile is found, the call is dropped. |

The decision to use the LAN or Internal profile involves some knowledge of your network design and whether you are bridging or routing to remote sites (or a combination of both). It is best to use the LAN profile instead of the Internal profile to simplify the configuration. You can easily associate the LAN profile with the Ethernet interface and the user-defined profiles with the ISDN interface.

Sometimes situations arise (very infrequent) where you must route a protocol to one site and bridge the SAME protocol to another site. Simply leave the LAN profile as a bridging profile and use the Internal profile for all routed protocol information.

Creating and Modifying Profiles

A new profile is created with the set user command. When you create a new profile, you automatically enter profile mode for that profile. The following example creates a user profile called tomd. Enter the set user command to create a profile using the profile template for the default values of the parameters, as follows:

Host>set user tomd

Host:tomd>

Notice that the profile mode is indicated by the prompt, which appears as the system name and the profile name, separated by a colon. While this prompt is displayed, modifications to the parameters only affect the parameters in the profile. The changes do not affect system mode parameters or other profiles.

The cd command allows you to change to the system mode or to another profile. Following is an example of the cd command used to change to a permanent profile called LAN:

Router_name> cd LAN

Router_name:LAN>

Note that the prompt includes the name of the profile. You can now modify the LAN profile parameters.

Displaying Profile Configurations

The show commands display the values associated with a profile parameter in profile mode. The commands work in system mode to show the values associated with parameters in the profile template.

In profile mode, some show commands only display profile parameters. Parameter values that have been redefined in profile mode are indicated with a <*>. All other parameter values are inherited from the profile template.

Removing Profile-Based Values

You can remove any parameter value within a profile with the unset command. The parameter you removed inherits its value from the system mode.

In the following example, the profile parameter number is removed from the profile using the unset command:

Host:Profile> unset number

Deleting Profiles

The reset user command deletes a user-defined profile from the router. The three permanent profiles (LAN, Internal and Standard) cannot be deleted. This command also closes any connection associated with the profile.

In the following example, the tomd profile is removed from the system using the reset user command:

Host:Profile> reset user tomd

Changing Profile Names

The set profile user command changes the name of an existing profile. Enter this command while in profile mode for the profile you want to affect. In the following example, the profile name is being changed from 2500 to 4500:

766:2500> set profile user 4500

766:4500>

Incoming Calls

When the router receives an incoming call, the router searches both active and inactive profiles for a profile with the same name as the calling device. If it finds a profile with the matching user ID, the router uses the configuration parameters of that profile while communicating with the remote device. If the profile is inactive, it is automatically activated for the duration of the connection.

When the call is finished, the physical link between the two devices is disconnected. However, the virtual connection to the remote router might be configured to remain active.

If the profile is configured to remain active after a link disconnects, a virtual connection remains. The virtual connection monitors the LAN traffic. If packets destined for the WAN are detected, the router opens up the physical connection and forwards the packets.

If the profile is configured to become inactive after a link disconnects, both the physical link and the virtual connection to the remote router are disconnected until another call is received from the same remote router.

Outgoing Calls

Outgoing calls require that the associated user-defined profiles be set to active, the set auto command must be on and there must be a phone number to call stored in the profile. If the profile is inactive, a number to dial is not available to the router.

Describe the difference between connecting two Cisco 700 series routers and connecting a Cisco 700 to a Cisco IOS router

700 Series: IP Static Routing To Another 700 Series Router

This configuration describes the commands necessary to configure two 700 series routers for routing IP traffic between two sites using ISDN. This configuration does not use the RIP routing protocol. Instead, it uses static IP route entries.

Configuration for the Atlanta router:

set systemname Atlanta

set ppp secret client

gocisco1

gocisco1

set encapsulation ppp

cd lan

set bridge off

set ip routing on

set ip address 10.1.1.1

set ip netmask 255.0.0.0

set user Boston

set bridge off

set ip routing on

set ip address 0.0.0.0

set ip route destination 20.0.0.0/8 gateway 20.1.1.1

set ip framing none

set ip propagate off

set 1 number 16175553333

set 2 number 16175554444

set ppp secret host

cisco

cisco

set timeout 120

Configuration for the Boston router:

set systemname Boston

set ppp secret client

gocisco1

gocisco1

set encapsulation ppp

cd lan

set bridge off

set ip routing on

set ip address 20.1.1.1

set ip netmask 255.0.0.0

set user Boston

set bridge off

set ip routing on

set ip address 0.0.0.0

set ip route destination 10.0.0.0/8 gateway 10.1.1.1

set ip framing none

set ip propagate off

set 1 number 14045551111

set 2 number 14045552222

set ppp secret host

gocisco1

gocisco1

set timeout 120

700 Series: IP; Static Routing To An IOS Router

This sample configuration describes the commands necessary for connecting a 700 Series router to an IOS router. The configuration uses static IP routes to route IP traffic between the two sites over ISDN.

IOS "Atlanta" Configuration

version 11.2

service udp-small-servers

service tcp-small-servers

!

hostname Atlanta

!

enable secret cisco

!

username Boston password gocisco1

ip subnet-zero

no ip domain-lookup

isdn switch-type basic-ni1

!

interface Ethernet 0

ip address 10.1.1.1 255.0.0.0

!

interface BRI0

encapsulation ppp

ppp authentication chap

ppp multilink

dialer load-threshold 1

isdn spid1 01404555111101

isdn spid2 01404555222201

ip unnumbered ethernet 0

dialer map ip 20.1.1.1 name Boston speed 56 broadcast 16175553333

dialer map ip 20.1.1.1 name Boston speed 56 broadcast 16175554444

dialer-group 1

!

router rip

network 10.0.0.0

!

ip route 20.0.0.0 255.0.0.0 20.1.1.1

ip route 20.1.1.1 255.255.255.255 bri 0

dialer-list 1 protocol ip permit

!

line con 0

password console

login

line vty 0 4

password telnet

login

!

end

Cisco 700 "Boston" Configuration

set system Boston

set switch 5ess

set 1 spid 01617555333301

set 1 directorynumber 5553333

set 2 spid 01617555444401

set 2 directorynumber 5554444

set ppp authentication incoming chap

set ppp authentication outgoing chap

set ppp secret client

gocisco1

gocisco1

cd lan

set ip address 20.1.1.1

set ip netmask 255.0.0.0

set ip routing on

set ip rip update periodic

set bridging off

cd

set user Atlanta

set encapsulation ppp

set timeout 300

set 1 number 14045551111

set 2 number 14045552222

set ppp secret host

gocisco1

gocisco1

set ip routing on

set ip framing none

set ip route destination 0.0.0.0/0 gateway 0.0.0.0

set ip rip update off

set bridging off

cd

set active Atlanta

Describe the operation of a DHCP server and Agent

DHCP Description

DHCP is a client-server protocol that allows devices on an IP network (the DHCP clients) to request configuration information from a DHCP server. DHCP allocates network addresses from a central pool on an as-needed basis. DHCP is useful for assigning IP addresses to hosts connected to the network temporarily, or for sharing a limited pool of IP addresses among a group of hosts that do not need permanent IP addresses.

DHCP allows for increased automation and fewer network administration problems by:

Eliminating the need for the manual configuration of individual computers, printers, and shared file systems.

Preventing the simultaneous use of the same IP address by two clients.

Allowing configuration from a central site.

DHCP Server Application Notes

The following are application notes for DHCP server:

DHCP relay and DHCP server are mutually exclusive.

When DHCP server is initialized, default addresses are used if no LAN or internal address exists. The Cisco 700 series router picks up the DHCP client's default gateway, netmask, and starting DHCP addresses using the LAN IP address, if one exists. If a LAN address does not exist, the router uses the internal IP address. If neither exists, it uses the default settings: 10.0.0.1 as the LAN IP address (default gateway for DHCP clients), 255.0.0.0 as the subnet mask, and 10.0.0.2 as the starting DHCP client addresses.

For the DHCP values to be automatically generated based on the LAN or internal IP address, each DHCP value must be set to 0.0.0.0 or none, for the new values to take effect.

DHCP Relay Application Notes

The following are application notes for DHCP relay:

The IP address in the Internal profile must be on the same network as that of the DHCP clients.

A firewall configuration (where there is an Internal profile and LAN IP address) does not work with DHCP relay.

A configuration where PAT is on and DHCP relay is enabled is not valid. DHCP relay will attempt to cross from a public to a private domain. PAT prevents access to the private domain. DHCP relay fails because it must reference the router's private address.

DHCP relay and DHCP server are mutually exclusive. The Cisco 700 series router can function as one or the other, but not both.

Describe the operation of PAT

PAT Description

Cisco 700 series routers provide PAT, enabling local hosts on a private IP network to communicate externally. Packets destined for an external address have their private IP address plus port number translated to the router's external IP address before the IP packet is forwarded to the WAN. IP packets returning to the router have their external IP addresses (plus port number) translated back to the private IP addresses and the packets are forwarded to the LAN. When PAT is enabled, the transmission of RIP packets is automatically disabled to prevent a broadcast of the private IP addresses externally.

PAT Application Notes

A configuration where PAT is on and DHCP relay is enabled is not valid. DHCP relay will attempt to cross from a public to a private domain. PAT prevents access to the private domain. DHCP relay fails because it must reference the router's private address.

Since PAT takes away the end-to-end significance of an IP address, limitations exist when PAT is enabled.

PING from an outside host to a host in the private network ends at the Cisco 700 router, and is not forwarded to the inside host. Telnet from an outside host to a host in the private network also ends at the Cisco 700 router if there is no telnet port handler defined.

Only one inside or private WWW server is supported. WWW linkages with other inside hosts or servers are not translated. Only one FTP Server, Telnet Server and so forth is supported in the inside network.

Packets destined for the Cisco 700 router itself, for example DHCP, SNMP, PING and TFTP, are not subject to PAT.

A maximum of 12 personal computers can simultaneously boot in the inside network. If more than 12 personal computers try to boot up simultaneously, one or more can get an error message about not being able to access the server.

400 PAT entries are allocated for sharing among the inside machines. If TCP connections are setup and TCP timeouts are set to keepalive, no more than 400 machines can get to the outside world.

The Cisco 700 PAT feature does not handle fragmented FTP packets.

Multidestination with unnumbered links does not work for PAT and is not recommended.

When a packet is received from the outside, PAT compares the port number with an internally configured port handler list (15 entries maximum), and if there is a port handler defined for this port, it routes the packet to the appropriate port handler. Otherwise, if there is a default port handler defined it routes the packet there. If it fails to find entries for the above two cases, the Cisco 700 handles the packet itself.

Some well known ports cannot have port handlers defined. They include the DHCP client port, used by the Cisco 700 in getting DHCP server responses, and the WINS NetBIOS ports, used by Inside Win95 PCs to get WINS information.

Install and configure a Cisco 700 series product to communicate with an access server

See the previous question

Configure a cisco 700 series router as a DHCP server and relay agent

Cisco 700 series routers can function as a DHCP relay agent. (The router can also act as a DHCP server, but not both at the same time.) When configured, Cisco 700 series routers relay DHCP requests and responses between DHCP clients and a specified DHCP server. Use the set dhcp relay command, where the IP address is the address of the server to which DHCP requests will be forwarded.

Describe how Network Address Translation (NAT) operates

Two of the key problems facing the Internet are depletion of IP address space and scaling in routing. Network Address Translation (NAT) is a feature that allows an organization's IP network to appear from the outside to use different IP address space than what it is actually using. Thus, NAT allows an organization with nonglobally routable addresses to connect to the Internet by translating those addresses into globally routable address space. NAT also allows a more graceful renumbering strategy for organizations that are changing service providers or voluntarily renumbering into CIDR blocks. NAT is also described in RFC 1631.

NAT has several applications. Use it for the following purposes:

You want to connect to the Internet, but not all your hosts have globally unique IP addresses. NAT enables private IP internetworks that use nonregistered IP addresses to connect to the Internet. NAT is configured on the router at the border of a stub domain (referred to as the inside network) and a public network such as the Internet (referred to as the outside network). NAT translates the internal local addresses to globally unique IP addresses before sending packets to the outside network.

You must change your internal addresses. Instead of changing them, which can be a considerable amount of work, you can translate them by using NAT.

You want to do basic load sharing of TCP traffic. You can map a single global IP address to many local IP addresses by using the TCP load distribution feature.

As a solution to the connectivity problem, NAT is practical only when relatively few hosts in a stub domain communicate outside of the domain at the same time. When this is the case, only a small subset of the IP addresses in the domain must be translated into globally unique IP addresses when outside communication is necessary, and these addresses can be reused when no longer in use.

A significant advantage of NAT is that it can be configured without requiring changes to hosts or routers other than those few routers on which NAT will be configured. As discussed previously, NAT may not be practical if large numbers of hosts in the stub domain communicate outside of the domain. Furthermore, some applications use embedded IP addresses in such a way that it is impractical for a NAT device to translate. These applications may not work transparently or at all through a NAT device. NAT also hides the identity of hosts, which may be an advantage or a disadvantage.

A router configured with NAT will have at least one interface to the inside and one to the outside. In a typical environment, NAT is configured at the exit router between a stub domain and backbone. When a packet is leaving the domain, NAT translates the locally significant source address into a globally unique address. When a packet is entering the domain, NAT translates the globally unique destination address into a local address. If more than one exit point exists, each NAT must have the same translation table. If the software cannot allocate an address because it has run out of addresses, it drops the packet and sends an ICMP Host Unreachable packet.

A router configured with NAT must not advertise the local networks to the outside. However, routing information that NAT receives from the outside can be advertised in the stub domain as usual.

As mentioned previously, the term inside refers to those networks that are owned by an organization and that must be translated. Inside this domain, hosts will have address in the one address space, while on the outside, they will appear to have addresses in a another address space when NAT is configured. The first address space is referred to as the local address space while the second is referred to as the global address space.

Similarly, outside refers to those networks to which the stub network connects, and which are generally not under the organization's control. As will be described later, hosts in outside networks can be subject to translation also, and can, thus, have local and global addresses.

To summarize, NAT uses the following definitions:

Inside local address--The IP address that is assigned to a host on the inside network. The address is probably not a legitimate IP address assigned by the Network Information Center (NIC) or service provider.

Inside global address--A legitimate IP address (assigned by the NIC or service provider) that represents one or more inside local IP addresses to the outside world.

Outside local address--The IP address of an outside host as it appears to the inside network. Not necessarily a legitimate address, it was allocated from address space routable on the inside.

Outside global address--The IP address assigned to a host on the outside network by the host's owner. The address was allocated from globally routable address or network space.

Give on example of when NAT could be used

NAT has several applications. Use it for the following purposes:

You want to connect to the Internet, but not all your hosts have globally unique IP addresses. NAT enables private IP internetworks that use nonregistered IP addresses to connect to the Internet. NAT is configured on the router at the border of a stub domain (referred to as the inside network) and a public network such as the Internet (referred to as the outside network). NAT translates the internal local addresses to globally unique IP addresses before sending packets to the outside network.

You must change your internal addresses. Instead of changing them, which can be a considerable amount of work, you can translate them by using NAT.

You want to do basic load sharing of TCP traffic. You can map a single global IP address to many local IP addresses by using the TCP load distribution feature.

As a solution to the connectivity problem, NAT is practical only when relatively few hosts in a stub domain communicate outside of the domain at the same time. When this is the case, only a small subset of the IP addresses in the domain must be translated into globally unique IP addresses when outside communication is necessary, and these addresses can be reused when no longer in use.

Identify the configure commands necessary to enable NAT

NAT Configuration Task List

Before configuring any NAT translation, you must know your inside local addresses and inside global addresses. The following sections discuss how you can use NAT to perform optional tasks:

Translate Inside Source Addresses

Use this feature to translate your own IP addresses into globally unique IP addresses when communicating outside of your network. You can configure static or dynamic inside source translation as follows:

Static translation establishes a one-to-one mapping between your inside local address and an inside global address. Static translation is useful when a host on the inside must be accessible by a fixed address from the outside.

Dynamic translation establishes a mapping between an inside local address and a pool of global addresses.

Steps that occur:
The user at Host 1.1.1.1 opens a connection to Host B.

The first packet that the router receives from Host 1.1.1.1 causes the router to check its NAT table.

If a static translation entry was configured, the router goes to Step 3.

If no translation entry exists, the router determines that source address (SA) 1.1.1.1 must be translated dynamically, selects a legal, global address from the dynamic address pool, and creates a translation entry. This type of entry is called a simple entry.

The router replaces the inside local source address of Host 1.1.1.1 with the translation entry's global address, and forwards the packet.

Host B receives the packet and responds to Host 1.1.1.1 by using the inside global IP destination address (DA) 2.2.2.2.

When the router receives the packet with the inside global IP address, it performs a NAT table lookup by using the inside global address as a key. It then translates the address to the inside local address of Host 1.1.1.1 and forwards the packet to Host 1.1.1.1.

Host 1.1.1.1 receives the packet and continues the conversation. The router performs Steps 2 through 5 for each packet.

Configure Static Translation

To configure static inside source address translation, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Establish static translation between an inside local address and an inside global address. | ip nat inside source static local-ip global-ip |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

The previous steps are the minimum you must configure. You could configure multiple inside and outside interfaces.

Configure Dynamic Translation

To configure dynamic inside source address translation, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Define a pool of global addresses to be allocated as needed. | ip nat pool name start-ip end-ip {netmask netmask ¦ prefix-length prefix-length} |
| Define a standard access list permitting those addresses that are to be translated. | access-list access-list-number permit source [source-wildcard] |
| Establish dynamic source translation, specifying the access list defined in the prior step. | ip nat inside source list access-list-number pool name |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

Note The access list must permit only those addresses that are to be translated. (Remember that there is an implicit "deny all" at the end of each access-list.) An access list that is too permissive can lead to unpredictable results.

The following example translates all source addresses passing access list 1 (having a source address from 192.168.1.0/24) to an address from the pool named net-208. The pool contains addresses from 171.69.233.208 to 171.69.233.233.

ip nat pool net-208 171.69.233.208 171.69.233.233 netmask 255.255.255.240

ip nat inside source list 1 pool net-208

!

interface serial 0

ip address 171.69.232.182 255.255.255.240

ip nat outside

!

interface ethernet 0

ip address 192.168.1.94 255.255.255.0

ip nat inside

!

access-list 1 permit 192.168.1.0 0.0.0.255

Overload an Inside Global Address

You can conserve addresses in the inside global address pool by allowing the router to use one global address for many local addresses. When this overloading is configured, the router maintains enough information from higher-level protocols (for example, TCP or UDP port numbers) to translate the global address back to the correct local address. When multiple local addresses map to one global address, each the TCP or UDP port numbers of each inside host distinguish between the local addresses.

illustrates NAT operation when one inside global address represents multiple inside local addresses. The TCP port numbers act as differentiators.

The router performs the following process in overloading inside global addresses.. Both Host B and Host C think they are talking to a single host at address 2.2.2.2. They are actually talking to different hosts; the port number is the differentiator. In fact, many inside hosts could share the inside global IP address by using many port numbers. The user at Host 1.1.1.1 opens a connection to Host B.

The first packet that the router receives from Host 1.1.1.1 causes the router to check its NAT table.

If no translation entry exists, the router determines that address 1.1.1.1 must be translated, and sets up a translation of inside local address 1.1.1.1 to a legal global address. If overloading is enabled, and another translation is active, the router reuses the global address from that translation and saves enough information to be able to translate back. This type of entry is called an extended entry.

The router replaces the inside local source address 1.1.1.1 with the selected global address and forwards the packet.

Host B receives the packet and responds to Host 1.1.1.1 by using the inside global IP address 2.2.2.2. 
When the router receives the packet with the inside global IP address, it performs a NAT table lookup, using the protocol, inside global address and port, and outside address and port as a key, translates the address to inside local address 1.1.1.1, and forwards the packet to Host 1.1.1.1. 
Host 1.1.1.1 receives the packet and continues the conversation. The router performs Steps 2 through 5 for each packet.

To configure overloading of inside global addresses, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Define a pool of global addresses to be allocated as needed. | ip nat pool name start-ip end-ip {netmask netmask ¦ prefix-length prefix-length} |
| Define a standard access list. | access-list access-list-number permit source [source-wildcard] |
| Establish dynamic source translation, identifying the access list defined in the prior step. | ip nat inside source list access-list-number pool name overload |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

Note The access list must permit only those addresses that are to be translated. (Remember that there is an implicit "deny all" at the end of each access list.) An access list that is too permissive can lead to unpredictable results.

The following example creates a pool of addresses named net-208. The pool contains addresses from 171.69.233.208 to 171.69.233.233. Access list 1 allows packets having the source address from 192.168.1.0 to 192.168.1.255. If no translation exists, packets matching access list 1 are translated to an address from the pool. The router allows multiple local addresses (192.168.1.0 to 192.168.1.255) to use the same global address. The router retains port numbers to differentiate the connections.

ip nat pool net-208 171.69.233.208 171.69.233.233 netmask 255.255.255.240

ip nat inside source list 1 pool net-208 overload

!

interface serial0

ip address 171.69.232.182 255.255.255.240

ip nat outside

!

interface ethernet0

ip address 192.168.1.94 255.255.255.0

ip nat inside

!

access-list 1 permit 192.168.1.0 0.0.0.255

Translate Overlapping Addresses

The NAT overview discusses translating IP addresses, perhaps because your IP addresses are not legal, officially assigned IP addresses. Perhaps you chose IP addresses that officially belong to another network. The case of an address used both illegally and legally is called overlapping. You can use NAT to translate inside addresses that overlap with outside addresses. Use this feature if your IP addresses in the stub network are legitimate IP addresses belonging to another network, and you want to communicate with those hosts or routers.

The router performs the following process when translating overlapping addresses:

The user at Host 1.1.1.1 opens a connection to Host C by name, requesting a name-to-address lookup from a DNS server.

The router intercepts the DNS reply and translates the returned address if there is an overlap (that is, the resulting legal address resides illegally in the inside network). To translate the return address, the router creates a simple translation entry mapping the overlapping address 1.1.1.3 to an address from a separately configured, outside local address pool.

The router examines every DNS reply from everywhere, ensuring that the IP address is not in the stub network. If it is, the router translates the address.

Host 1.1.1.1 opens a connection to 3.3.3.3.

The router sets up translations mapping inside local and global addresses to each other, and outside global and local addresses to each other.

The router replaces the source address with the inside global address and replaces the destination address with the outside global address.

Host C receives the packet and continues the conversation.

The router does a lookup, replaces the destination address with the inside local address, and replaces the source address with the outside local address.

Host 1.1.1.1 receives the packet and the conversation continues, using this translation process.

Configure Static Translation

To configure static outside source address translation, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Establish static translation between an outside local address and an outside global address. | ip nat outside source static global-ip local-ip |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

Configure Dynamic Translation

To configure dynamic outside source address translation, perform the following tasks beginning in global configuration mode.

| Task | Command |
| --- | --- |
| Define a pool of local addresses to be allocated as needed. | ip nat pool name start-ip end-ip {netmask netmask ¦ prefix-length prefix-length} |
| Define a standard access list. | access-list access-list-number permit source [source-wildcard] |
| Establish dynamic outside source translation, specifying the access list defined in the prior step. | ip nat outside source list access-list-number pool name |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

Note The access list must permit only those addresses that are to be translated. (Remember that there is an implicit "deny all" at the end of each access list.) An access list that is too permissive can lead to unpredictable results.

In the following example, the addresses in the local network are being used legitimately by someone else on the Internet. An extra translation is required to access that external network. Pool net-10 is a pool of outside local IP addresses. The statement ip nat outside source list 1 pool net-10 translates the addresses of hosts from the outside overlapping network to addresses in that pool.

ip nat pool net-208 171.69.233.208 171.69.233.223 prefix-length 28

ip nat pool net-10 10.0.1.0 10.0.1.255 prefix-length 24

ip nat inside source list 1 pool net-208

ip nat outside source list 1 pool net-10

!

interface serial 0

ip address 171.69.232.192 255.255.255.240

ip nat outside

!

interface ethernet0

ip address 192.168.1.94 255.255.255.0

ip nat inside

access-list 1 permit 192.168.1.0 0.0.0.255

Provide TCP Load Distribution

Another use of NAT is unrelated to Internet addresses. Your organization may have multiple hosts that must communicate with a heavily used host. Using NAT, you can establish a virtual host on the inside network that coordinates load sharing among real hosts. Destination addresses that match an access list are replaced with addresses from a rotary pool. Allocation is done in a round-robin basis, and only when a new connection is opened from the outside to the inside. Non-TCP traffic is passed untranslated (unless other translations are in effect).

The router performs the following process when translating rotary addresses:

The user on Host B (9.6.7.3) opens a connection to virtual host at 1.1.1.127.

The router receives the connection request and creates a new translation, allocating the next real host (1.1.1.1) for the inside local IP address.

The router replaces the destination address with the selected real host address and forwards the packet.

Host 1.1.1.1 receives the packet and responds.

The router receives the packet, performs a NAT table lookup using the inside local address and port number, and the outside address and port number as the key. The router then translates the source address to the address of the virtual host and forwards the packet.

The next connection request will cause the router to allocate 1.1.1.2 for the inside local address.

To configure destination address rotary translation, perform the following tasks beginning in global configuration mode. This allows you to map one virtual host to many real hosts. Each new TCP session opened with the virtual host will be translated into a session with a different real host.

| Task | Command |
| --- | --- |
| Define a pool of addresses containing the addresses of the real hosts. | ip nat pool name start-ip end-ip {netmask netmask ¦ prefix-length prefix-length} type rotary |
| Define an access list permitting the address of the virtual host. | access-list access-list-number permit source [source-wildcard] |
| Establish dynamic inside destination translation, identifying the access list defined in the prior step. | ip nat inside destination list access-list-number pool name |
| Specify the inside interface. | interface type number |
| Mark the interface as connected to the inside. | ip nat inside |
| Specify the outside interface. | interface type number |
| Mark the interface as connected to the outside. | ip nat outside |

Note The access list must permit only those addresses that are to be translated. (Remember that there is an implicit "deny all" at the end of each access list.) An access list that is too permissive can lead to unpredictable results.

In the following example, the goal is to define a virtual address, connections to which are distributed among a set of real hosts. The pool defines the addresses of the real hosts. The access list defines the virtual address. If a translation does not already exist, TCP packets from serial 0 (the outside interface) whose destination matches the access list are translated to an address from the pool.

ip nat pool real-hosts 192.168.15.2 192.168.15.15 prefix-length 28 type rotary

ip nat inside destination list 2 pool real-hosts

!

interface serial 0

ip address 192.168.15.129 255.255.255.240

ip nat outside

!

interface ethernet 0

ip address 192.168.15.17 255.255.255.240

ip nat inside

!

access-list 2 permit 192.168.15.1

Change Translation Timeouts

By default, dynamic address translations time out after some period of non-use. You can change the default values on timeouts, if necessary. When overloading is not configured, simple translation entries time out after 24 hours. To change this value, perform the following task in global configuration mode:

| Task | Command |
| --- | --- |
| Change the timeout value for dynamic address translations that do not use overloading. | ip nat translation timeout seconds |

If you have configured overloading, you have finer control over translation entry timeout because each entry contains more context about the traffic that is using it. To change timeouts on extended entries, perform one or more of the following tasks in global configuration mode:

| Task | Command |
| --- | --- |
| Change the UDP timeout value from 5 minutes. | ip nat translation udp-timeout seconds |
| Change the DNS timeout value from 1 minute. | ip nat translation dns-timeout seconds |
| Change the TCP timeout value from 24 hours. | ip nat translation tcp-timeout seconds |
| Change the Finish and Reset timeout value from 1 minute. | ip nat translation finrst-timeout seconds |

Monitor and Maintain NAT

By default, dynamic address translations will time out from the NAT translation table at some point. You can clear the entries before the timeout by performing one of the following tasks in EXEC mode:

| Task | Command |
| --- | --- |
| Clear all dynamic address translation entries from the NAT translation table. | clear ip nat translation * |
| Clear a simple dynamic translation entry containing an inside translation, or both inside and outside translation. | clear ip nat translation inside global-ip local-ip [outside local-ip global-ip] |
| Clear a simple dynamic translation entry containing an outside translation. | clear ip nat translation outside local-ip global-ip |
| Clear an extended dynamic translation entry. | clear ip nat translation protocol inside global-ip global-port local-ip local-port [outside local-ip local-port global-ip global-port] |

You can display translation information by performing one of the following tasks in EXEC mode:

| Task | Command |
| --- | --- |
| Display active translations. | show ip nat translations [verbose] |
| Display translation statistics. | show ip nat statistics |

NAT Configuration Examples

The following are NAT configuration examples.

Dynamic Inside Source Translation Example

The following example translates all source addresses passing access list 1 (having a source address from 192.168.1.0/24) to an address from the pool named net-208. The pool contains addresses from 171.69.233.208 to 171.69.233.233.

ip nat pool net-208 171.69.233.208 171.69.233.233 netmask 255.255.255.240

ip nat inside source list 1 pool net-208

!

interface serial 0

ip address 171.69.232.182 255.255.255.240

ip nat outside

!

interface ethernet 0

ip address 192.168.1.94 255.255.255.0

ip nat inside

!

access-list 1 permit 192.168.1.0 0.0.0.255

Overloading Inside Global Addresses Example

The following example creates a pool of addresses named net-208. The pool contains addresses from 171.69.233.208 to 171.69.233.233. Access list 1 allows packets having the source address from 192.168.1.0 to 192.168.1.255. If no translation exists, packets matching access list 1 are translated to an address from the pool. The router allows multiple local addresses (192.168.1.0 to 192.168.1.255) to use the same global address. The router retains port numbers to differentiate the connections.

ip nat pool net-208 171.69.233.208 171.69.233.233 netmask 255.255.255.240

ip nat inside source list 1 pool net-208 overload

!

interface serial0

ip address 171.69.232.182 255.255.255.240

ip nat outside

!

interface ethernet0

ip address 192.168.1.94 255.255.255.0

ip nat inside

!

access-list 1 permit 192.168.1.0 0.0.0.255

Translating Overlapping Address Example

In the following example, the addresses in the local network are being used legitimately by someone else on the Internet. An extra translation is required to access that external network. Pool net-10 is a pool of outside local IP addresses. The statement ip nat outside source list 1 pool net-10 translates the addresses of hosts from the outside overlapping network to addresses in that pool.

ip nat pool net-208 171.69.233.208 171.69.233.223 prefix-length 28

ip nat pool net-10 10.0.1.0 10.0.1.255 prefix-length 24

ip nat inside source list 1 pool net-208

ip nat outside source list 1 pool net-10

!

interface serial 0

ip address 171.69.232.192 255.255.255.240

ip nat outside

!

interface ethernet0

ip address 192.168.1.94 255.255.255.0

ip nat inside

!

access-list 1 permit 192.168.1.0 0.0.0.255

TCP Load Distribution Example

In the following example, the goal is to define a virtual address, connections to which are distributed among a set of real hosts. The pool defines the addresses of the real hosts. The access list defines the virtual address. If a translation does not already exist, TCP packets from serial 0 (the outside interface) whose destination matches the access list are translated to an address from the pool.

ip nat pool real-hosts 192.168.15.2 192.168.15.15 prefix-length 28 type rotary

ip nat inside destination list 2 pool real-hosts

!

interface serial 0

ip address 192.168.15.129 255.255.255.240

ip nat outside

!

interface ethernet 0

ip address 192.168.15.17 255.255.255.240

ip nat inside

!

access-list 2 permit 192.168.15.1

Describe the commands necessary to monitor NAT operation

By default, dynamic address translations will time out from the NAT translation table at some point. You can clear the entries before the timeout by performing one of the following tasks in EXEC mode:

| Task | Command |
| --- | --- |
| Clear all dynamic address translation entries from the NAT translation table. | clear ip nat translation * |
| Clear a simple dynamic translation entry containing an inside translation, or both inside and outside translation. | clear ip nat translation inside global-ip local-ip [outside local-ip global-ip] |
| Clear a simple dynamic translation entry containing an outside translation. | clear ip nat translation outside local-ip global-ip |
| Clear an extended dynamic translation entry. | clear ip nat translation protocol inside global-ip global-port local-ip local-port [outside local-ip local-port global-ip global-port] |

You can display translation information by performing one of the following tasks in EXEC mode:

| Task | Command |
| --- | --- |
| Display active translations. | show ip nat translations [verbose] |
| Display translation statistics. | show ip nat statistics |

Describe how a Virtual Private Dialup Network (VPDN) operates

How VPDN Works

Virtual private dialup networking enables users to configure secure networks that take advantage of Internet Service Providers that tunnel the company's remote access traffic through the ISP cloud.

Remote offices or mobile users can connect to their home network using local dialup services of third parties. The dialup service provider agrees to forward the company's traffic from the ISP POP to a company-run home gateway. Network configuration and security remains in the control of the client. The dialup service provider provides a virtual pipe between the company's sites.

Note The MMP feature uses L2F to connect multiple PPP sessions for which individual dial-in calls have arrived on different stack group members. L2F provides speed and reliability for the setup and shutdown of Multilink PPP.

A VPDN connection between a remote user and the home LAN is accomplished in the following steps:

The remote user initiates a PPP connection to the ISP using the analog telephone system or ISDN.

The ISP network access server accepts the connection.

The ISP network access server authenticates the end user with CHAP or PAP. The username is used to determine whether the user is an VPDN client. If the user is not a VPDN client, the client accesses the Internet or other contacted service.

The tunnel endpoints--the network access server and the home gateway--authenticate each other before any sessions are attempted within a tunnel.

If no L2F tunnel exists between the network access server and the remote users' home gateway, a tunnel is created. Once the tunnel exists, an unused slot within the tunnel is allocated.

The home gateway accepts or rejects the connection. Initial setup can include authentication information required to allow the home gateway to authenticate the user.

The home gateway sets up a virtual interface. Link-level frames can now pass through this virtual interface through the L2F tunnel.

illustrates a VPDN connection from a remote user, who makes a local call, to the corporate network, through an end-to-end L2F tunnel (shown by the dotted line). The user can even be sent directly to a restricted part or a restricted set of servers on the corporate based on the user's authentication. In, the restriction placed on this user is suggested by the arc isolating a part of the corporate network cloud.

Give one example of when VPDN can be used

See Question #228

Identify the configuration commands necessary to enable VPDN

Configure a Virtual Template and Create a Virtual Template Interface

To configure a virtual template for interfaces on a home gateway access server, perform the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Create a virtual template interface, and enter interface configuration mode. | interface virtual-template number |
| Step 2 Identify the virtual template interface type and number on the LAN. | ip unnumbered ethernet 0 |
| Step 3 Enable PPP encapsulation on the virtual template interface. | encapsulation ppp |
| Step 4 Enable PPP authentication on the virtual template interface. | ppp authentication chap |

Configure Incoming VPDN Connections

To configure virtual private dialup networking on a home gateway router or access server, perform the following tasks in global configuration mode:

| Task | Command |
| --- | --- |
| Enable virtual private networking. | vpdn enable |
| Specify the remote host (the network access server), the local name (the home gateway) to use for authenticating, and the virtual template to use. | vpdn incoming nas-name hgw-name virtual-template number |

Configure VPDN on the Network Access Server

You can configure the router to authenticate users and also to select the outgoing tunnel based on the home gateway's host name or based on the DNIS information in the incoming calls. In addition, you can configure the tunnel search order.

Configure the Network Access Server to Authenticate Users

You can configure the network access to authenticate users before forwarding each connection to the home gateway. This allows you to detect unauthorized users and possibly to lessen the amount of traffic sent over the tunnels. To enable the network access server to authenticate users, complete the following task in global configuration mode:

| Task | Command |
| --- | --- |
| Enable the network access server to authenticate users. | vpdn local-authentication |

Configure VPDN Tunnel Lookup Based on Domain Name

To configure a network access server to make outgoing L2F connections to a home gateway based on domain name, complete the following tasks in global configuration mode:

| Task | Command |
| --- | --- |
| Enable virtual private networking. | vpdn enable |
| Specify the remote host that is to accept L2F connections. | vpdn outgoing domain-name local-name ip ip-address |

Configure VPDN Tunnel Lookup Based on Dialed Number Information

The network service provider can select a specific VPDN tunnel for outgoing calls from a dial-in user by using the Dialed Number Information Service (DNIS) information provided on ISDN lines.

The ability to select a tunnel based on DNIS provides additional flexibility to network service providers who offer VPDN services and to the corporations that use the services. Instead of having to use only the domain name for tunnel selection, tunnel selection can be based on the dialed number.

With this feature, a corporation--which might have only one domain name--can provide multiple specific phone numbers for users to dial in to the network access server at the service provider's point of presence. The service provider can select the tunnel to the appropriate services or portion of the corporate network based on the dialed number.

To configure a network access server to select outgoing L2F tunnel connections based on DNIS information for virtual private dialup networking, perform the following tasks in global configuration mode:

Example

The following example establishes a terminal session with an IBM host named "finance:"

Router> tn3270 finance

To terminate an active TN3270 session, you can issue any of the following commands:

exit
quit
logout

Describe the protocol translation sequence

This is an extremely vague question, Read #234 and #235, you should be able to use these to determine exactly what is happening in each possible scenario (I’m not going to cover each scenario since protocol translation is barely even touched on the actual test)

Basically the protocol translation software attempts to provide transparent protocol translation between systems running disparate protocols. It enables terminal users on one network to access hosts on another network, despite differences in the native protocol stacks associated with the originating device and the target host.

A communication or access server supports virtual terminal connections in both directions between the protocols in the following list. You can configure the server to translate automatically between them. This is called the one-step translation method.

X.25 and Local Area Transport (LAT)

X.25 and Telnet sessions using the Transmission Control Protocol (TCP)

LAT and TCP/Telnet

The software supports limited connections in both directions between the following protocols. Connecting between these protocols requires that you first connect to a protocol translator, then to the host to which you want to connect. This is called the two-step translation method.

XRemote to X.25 PAD environments (XRemote must use the two-step method)

TN3270 to LAT, X.25, and TCP/Telnet (TN3270 must use the two-step method)

The following sections describe the two-step and the one-step translation methods.

Define the asynchronous mobility environment

If you are a mobile user, it is often impractical to dial into your "home" access server from a remote site. Asynchronous mobility allows you to dial into different server products elsewhere on the internetwork while experiencing the same server environment that you would if you were connecting directly to your "home" access server.

This host mobility is accomplished by packet "tunneling," a technique by which raw data from the dial-in user is encapsulated and transported directly to the host site where your "home" access server performs the actual protocol processing.

You enable asynchronous mobility by entering the tunnel command to set up a network layer connection to a specified host. This task is described in the following section.

After a connection is established, you will receive an authentication dialog or prompt from your "home" access server and can proceed as if you are connected directly to that server. When communications are complete, the network connection can be closed and terminated from either end of the connection.

To set up a network layer connection to an access server, enter the following command:

tunnel hostname

Example: The following example establishes a network layer connection with an IBM host named "mktg:"

Router> tunnel mktg

Describe virtual asynchronous interfaces

Using Cisco IOS software, you can configure asynchronous protocol features, such as PPP and SLIP, on VTY lines. PPP and SLIP normally function only on asynchronous interfaces, not on VTY lines. When you configure a VTY line to support asynchronous protocol features, you are creating virtual asynchronous interfaces on the VTY lines. One practical benefit of virtual asynchronous interfaces is the ability to tunnel PPP and SLIP across X.25, TCP, or LAT networks on VTY lines. You tunnel PPP and SLIP using the protocol translation facility.

One practical benefit of enabling virtual asynchronous interfaces is the ability to tunnel PPP and SLIP over X.25, thus extending remote node capability into the X.25 area. You can also tunnel PPP and SLIP over Telnet or LAT on virtual terminal lines. You can tunnel PPP and SLIP over X.25, LAT, or Telnet, but you do so by using the protocol translation feature in the Cisco IOS software.

To tunnel incoming dial-up SLIP or PPP connections over X.25, LAT, or TCP to an IP network, you can use one-step protocol translation or two-step protocol translation, as follows:

If you are tunneling SLIP or PPP using the one-step method, you do not need to enter the vty-async command. Using the translate command with the SLIP or PPP keywords for one-step connections automatically enables asynchronous protocol functions on a per-VTY basis.

If you are tunneling SLIP or PPP using the two-step method, you must first enter the vty-async command on a global basis. Next, you perform a two-step connection process.

Configure and test an access router for protocol translation

The protocol translation feature provides transparent protocol translation between systems running different protocols. It enables terminal users on one network to access hosts on another network, despite differences in the native protocol stacks associated with the originating device and the targeted host.

Protocol translation is a resourceful facility for many business applications. For example,  a remote PC dialing through an IP network and connecting an X.25 host. The PC's TCP packets undergo a TCP-to-X.25 protocol translation by the router.

Whether to Use One-Step or Two-Step Protocol Translation, since IOS supports virtual terminal connections in both directions between the protocols in the following list. You can configure the router to translate automatically between them. This is called one-step translation, which is the most popular translation method.

X.25 and local-area transport (LAT)

X.25 and Telnet sessions using the Transmission Control Protocol (TCP)

LAT and TCP/Telnet

On outgoing connections, you can also use the one-step protocol translation facility to tunnel SLIP or PPP to IP and IPX networks or ARA to AppleTalk networks across X.25, LAT, or IP (on outgoing connections only).

Cisco IOS software supports limited connections in both directions between the protocols listed below. Connecting between these protocols requires that you first connect to a router, then to the host to which you want to connect. This is called two-step translation, which is the least popular translation method.

XRemote to SLIP/PPP and X.25 PAD environments (XRemote must use the two-step method)

LAT, X.25, SLIP/PPP, and TCP (Telnet) to TN3270 (TN3270 must use the two-step method)

Use the one-step method when network users repeatedly log on to the same remote network hosts through a router. This connection is more efficient and enables the device to have more knowledge of the protocols in use because the router acts as a network connection rather than as a terminal. The one-step method provides transparent protocol conversion. When connecting to the remote network host, the user enters the connection command to the remote network host but does not need to specify protocol translation. The network administrator has already created a configuration that defines a connection and the protocols to be translated. The user performs only one step to connect with the host.

When you make a one-step connection to the router, the Cisco IOS software determines which host the connection is for and which protocol that host is using. It then establishes a new network connection using the protocol required by that host.

A disadvantage of the one-step protocol translation method is that the initiating computer or user does not know that two networking protocols are being used. This means that parameters of the foreign network protocols cannot be changed after connections are established. The exception to this limitation is any set of parameters common to both networking protocols. Any parameter common to both can be changed from the first host to the final destination.

To configure the one-step method of protocol translation, set up the following protocols and connection options in the configuration file:

The incoming connection--The configuration includes the protocol to be used--LAT, X.25, or TCP/IP (Telnet)--the address, and any options such as reverse charging or binary mode that are supported for the incoming connection.

The outgoing connection--The outgoing connection is defined in the same way as the incoming connection, except that SLIP, PPP (including IP and IPX on PPP sessions), and ARA are also supported.

The connection features global options--You can specify additional features for the connection to allow, for example, incoming call addresses to match access list conditions or limit the number of users that can make the connection.

Use two-step protocol translation for one-time connections or when you use the router as a general-purpose gateway between two types of networks (for example, X.25 PDN and TCP/IP). As with the one-step method, Cisco recommends that you configure virtual templates for this feature.

With the two-step connection process, you can modify the parameters of either network connection, even while a session is in process. This process is similar to connecting a group of terminal lines from a PAD to a group of terminal lines from a TCP server. The difference is that you do not encounter the wiring complexity, unreliability, management problems, and performance bottlenecks that occur when two devices are connected via asynchronous serial lines.

To create one-step protocol translation connection specifications, perform the following task in global configuration mode:

| Task | Command |
| --- | --- |
| Create the connection specifications for one-step protocol translation. | translate protocol incoming-address [in-options] protocol outgoing-address [out-options] [global-options] |

Configure a Virtual Template for One-Step Protocol Translation

To configure a virtual interface template to enable tunneling of PPP or SLIP across an X.25, TCP, or LAT

WAN, first create and configure a virtual interface template, then apply it as the single outgoing option to the translate command.

To enable tunneling of PPP or SLIP across an X.25, TCP, or LAT WAN by using one-step protocol translation, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Create a virtual interface template, and enter interface configuration mode. | interface virtual-template number |
| Step 2 Assign an IP address to the virtual interface template. | ip unnumbered ethernet 0 |
| Step 3 Enable encapsulation on the virtual interface template. | encapsulation {ppp ¦ slip} |
| Step 4 Assign an IP address to the device connecting to the virtual access interface | peer default ip address {ip-address ¦ dhcp ¦ pool [pool-name]} |
| Step 5 Exit back to global configuration mode. | exit |
| Step 6 Assign the virtual interface template to a protocol translation session. | translate {lat ¦ tcp ¦ x25} incoming-address [in-options] virtual-template number [global-options] |

Mapping Outgoing Translate Command Options to Interface Commands

| Translate Command Options | Corresponding Interface Configuration Command |
| --- | --- |
| ip-pool | peer default ip address {ip-address ¦ dhcp ¦ pool [poolname]} |
| header-compression | ip tcp header compression [on ¦ off ¦ passive] |
| routing | ip routing or ipx routing |
| mtu | mtu |
| keepalive | keepalive |
| authentication {chap ¦ pap} | ppp authentication {chap ¦ pap} |
| ppp use-tacacs | ppp use-tacacs |
| ipx loopback | ipx ppp-client loopback number |

Configure Two-Step Protocol Translation

To translate using the two-step method, perform the following task beginning in global configuration mode. The first step is required only if you are tunneling SLIP or PPP using the two-step protocol translation facility:

| Task | Command |
| --- | --- |
| Step 1 Establish an incoming connection to the router running protocol translation. | connect lat pad telnet tunnel |
| Step 2 Establish the outgoing connection from the router supporting protocol translation to another network host. | connect lat pad telnet tunnel ppp slip |

The Cisco IOS software supports the two-step method in both directions for protocols other than PPP and SLIP (for example, from Telnet to PAD, and vice versa). PPP and SLIP are supported on outgoing connections only.

Configure a Virtual Template for Two-Step Protocol Translation

If you are tunneling PPP or SLIP using two-step protocol translation with virtual interface templates, you still use the vty-async command, just as before implementation of virtual templates. However, virtual asynchronous interfaces are not created as they were before virtual interface templates. Virtual access interfaces are created dynamically when a tunnel connection is established.

To create and configure a virtual interface template and apply it to a two-step protocol translation session, complete the following tasks beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Step 1 Create a virtual interface template, and enter interface configuration mode. | interface virtual-template number |
| Step 2 Assign an IP address to the virtual interface template. | ip unnumbered ethernet 0 |
| Step 3 Enable encapsulation on the virtual interface template. | encapsulation {ppp ¦ slip} |
| Step 4 Assign an IP address to the device connecting to the virtual access interface (such as the PC in Figure 66). | peer default ip address {ip-address ¦ dhcp ¦ pool [pool-name]} |
| Step 5 Exit back to global configuration mode. | exit |
| Step 6 Create a virtual asynchronous interface. | vty-async |
| Step 7 Apply the virtual template to the virtual asynchronous interface. | vty-async virtual-template number |

Because each protocol translation session uses a virtual terminal (VTY) line, you need to increase the number of VTY lines to increase the number of protocol translation sessions. That is, if your router has ten VTY lines, you can have up to ten protocol translation sessions. The default number of VTY lines is 5 (lines 0 through 4). To increase the number of lines, and thus the maximum number of protocol translation sessions, perform the following tasks, as appropriate, beginning in global configuration mode:

| Task | Command |
| --- | --- |
| Increase the number of virtual terminal lines, and thus, the maximum number of protocol translation sessions. | line vty number |
| Decrease the number of virtual terminal lines, and thus, the maximum number of protocol translation sessions. | no line vty number |

Protocol translation is a CPU-intensive task. Increasing the number of protocol translation sessions while routing is enabled can impact available memory. The amount of memory available depends on the platform type, the amount of DRAM available, the activity of each translation session, and the speed of the link. If you are using the maximum number of sessions and have problems with memory, you might need to decrease the number of protocol translation sessions.