16/11/98

# Chillnz CCNA Test prep ver 1.0

## OSI Reference

## 1)  Identify and describe functions of the layers of the OSI Reference Model

## 7.) Applications:

Where the user applications software lies.

Such issues as file access and transfer, virtual terminal emulation, nterprocess ommunication and the like are handled here.

6.) Presentation:  			Data representation

Differences in data representation are dealt with at this level.  For example, UNIX-style line endings (CR only) might be converted to  MS-DOS style  (CRLF), or EBCIDIC to ASCII character sets.

Data formats and data structures.

5.) Session:  				Inter-host communications

As its name implies, the session layer establishes, manages, and terminates sessions between applications. Sessions consist of dialogue between two or more presentation entities (recall that the session layer provides its services to the presentation layer). The session layer synchronizes dialogue between presentation layer entities and manages their data exchange. In addition to basic regulation of conversations (sessions), the session layer offers provisions for data expedition, class of service, and exception reporting of

session-layer, presentation-layer, and application-layer problems.

E.G  NFS, SQL, RPC, X

Co-ordinates communication.  Service requests, service replies.

4.) Transport: 				End to end connections

Segments upper layer apps.

Establish end to end connection

Send segments

Optionally ensure data reliability

Multiplexing, connection synchronisation, flow control, error recovery, reliability through windowing.

This is the lower layer that provides local user services.

3.) Network: 				Address and best path

This layer makes certain that a packet sent from one device to another actually gets there in a reasonable period of time.  Routing and flow control is performed here.  This is the lowest layer of the OSI model that can remain ignorant of the physical network.

2.) Data Link: 				Access to media

This layer deals with getting data packets on and off the wire, error detection and correction and retransmission.  This layer is generally broken into two sub-layers: The LLC (Logical Link Control) on the upper half, which does the error checking, and the MAC (Medium Access Control) on the lower half, which deals with getting the data on and off the wire.

Provides reliable transit of data across a physical link. The data-link layer is concerned with physical addressing, network topology, line discipline, error notification, ordered delivery of frames, and flow control. (From CCO)

- Physical: 				Binary transmission

Access to the wires of the network

Here is where the cable, connector and signalling specifications are defined.

2)  Connection-oriented vs. connectionless service

Connection-oriented: a virtual circuit is established

Connection establishment and termination required

Sequenced, acknowledged data delivery

Built-in error detection/retransmission

Sliding window flow control

Connectionless:      Data transfer without virtual circuit

No message sequencing

No delivery guarantee

Higher layer is responsible for error recovery,

flow control, and reliability

3)  Data-Link vs. network addresses

Link Layer:    MAC address (see 60) below)

Network Layer: Virtual/logical address, unique within a

hierarchical address space, contains a network

and a host part

4) Identify 3 reasons for using a layered model

Reduce complexity - divide interrelated aspects into less complex elements

Standardise interfaces

Facilitates modular engineering

Ensures interoperable technology

Accelerates evolution

Simplifies teaching and learning

5) Explain 5 conversions steps of data encapsulation

Application/Presentation/Session Layer:

Accepts user input, converts into	data

Transport Layer:   Adds segment (TCP or UDP) header 	segment

Network Layer:     Adds network header with			packet

Link Layer:        Adds frame Header				frame

Physical Layer:    Converts frame into bit stream		bits

6)  Define flow control and describe three basic methods used in networking

Technique for ensuring that a transmitting entity, such as a modem, does not overwhelm a receiving entity with data. When the buffers on the receiving device are full, a message is sent to the sending device to suspend the transmission until the data in the buffers has been processed.

Stop/Go method	eg XON/XOFF character based, CTS/RTS hardware based

Window 		eg ack after so many segments

Acknowledgement techniques

7) Key internetwork functions of OSI Network layer and how they are performed

in a router

Network Layer:  Routing, provide dynamic control over connectivity,

flexibility, performance, and management

Network layer handles best path determination and deals with addressing

## WAN Protocols

8) Differentiate between the following WAN services Frame Relay, ISDN/LAPD,

HDLC, PPP

Frame Relay: Variation of HDLC, uses PVC (Private Virtual Circuit)

Can do multipoint

ISDN/LAPD:   variation of a subset of HDLC - ABM mode.

Data link signalling

HDLC:        High level data link control - superset of SDLC

PPP:         Extended SDLC with protocol indicator (for multi-protocol

support) and LCP (Link Control Protocol) for establishing,

configuring, maintaining, and terminating connections

Point to point

Usage:

SDLC			Host (eg mainframe) to Router

HDLC

LAPB			Router  to  Router

PPP

X25

ISDN			Router  to  Network     (public switched network)

Frame Relay

9) Recognise Frame relay terms and features

VC:   Virtual Circuit (PVC, SVC), SVC can be over ISDN

DLCI: Data-link connection ID, identifies a PVC

LMI:  Local Management Interface, control protocol for PVC setup

and management.  Extends frame relay protocol - allows

multicast and xon/xoff flow control, global addresses.

LMI types : ansi		Annex D

Q933a		Annex A (CCITT)

cisco

Uses Statistical multiplexing

High performance - efficient.  Assumes reliable physical layer.

No error correction/retransmission - just checksum.

No explicit flow control, just congestion notification

Flow control and error retransmission left to upper layers.

Wide range of speeds from 56k over T1 (1.5Kbps) to DS3 (45 Mbps)

10) List commands to configure Frame Relay LMIs, maps, and subinterfaces

int ser0

encapsulation frame-relay [ietf | cisco]   (default = cisco)

Optional:

frame-relay lmi-type [ansi | cisco | q933a]  (default = cisco)

keepalive 10

Static mapping of dlci:

frame-relay map <protocol> <address> <dlci> [broadcast] [ietf|cisco]

(map a DLCI to a network for a given protocol, allows routing updates

only if "broadcast" is defined: frame map ip 192.168.0.1 70.  Used if

other router doesn’t use Inverse ARP, or the protocol doesn’t support it.)

Dynamic mapping of dlci doesn't need anything - just uses Inverse ARP.

int s0.<sub-number] (point-to-point | multi-point)

frame-relay interface-dlci <dlci> (broadcast)

The following example illustrates how to configure two routers for static mode.

Configuration for Router 1

interface serial 0

ip address 131.108.64.2 255.255.255.0

encapsulation frame-relay

keepalive 10

frame-relay map ip 131.108.64.1 43

Configuration for Router 2

interface serial 0

ip address 131.108.64.1 255.255.255.0

encapsulation frame-relay

keepalive 10

frame-relay map ip 131.108.64.2 43

Basic Subinterface Examples

interface serial 0

encapsulation frame-relay

interface serial 0.1 point-to-point

ip address 10.0.1.1 255.255.255.0

frame-relay interface-dlci 42

!

interface serial 0.2 multipoint

ip address 10.0.2.1 255.255.255.0

frame-relay map 10.0.2.2 18

11) List commands to monitor Frame Relay operations in the router

show int ser0

debug frame-relay lmi

show frame-relay lmi

show frame-relay pvc

show frame-relay traffic

show frame-relay map

12) Identify PPP operations to encapsulate WAN data on routers

username <name> password <password>  (to set CHAP password)

int ser0

encapsulation ppp

ppp authentication (pap | chap)

ppp multilink

13) Relevant use and context for ISDN

All digital service for voice, data, video, telecommuting etc.

Fast connection setup

Higher bandwidth than traditional modem

Use as a backup link

Dial on deman

14) ISDN protocols, functions groups, reference points, and channels

E protocols - Telephone network standards

I protocols - Concepts, terminology, and general methods

Q protocols - Switching and signalling

Functions are devices or hardware functions:

TA    - Terminal adapter

TE1|2 - Terminal end point 1 (integrated TA) or 2 (needs TA)

NT1   - Network termination ("signal converter")

NT2   - found in PBXs etc, combines layer 2 and 3 functions

NT1/NT2 combined units

LT    - Local termination (access point at phone company)

ET    - Exchange termination (communicates with other ISDn components)

Reference points are interfaces (between functions)

Phone --+-- TA --+-- NT1 --+-- ISDN Switch

R       S/T        U

PC --+--  NT1 --+-- ISDN Switch

S/T         U

Bearer channel:	Used for data transfer (voice or data)

Data channel:		Used for control/signaling information using LAPD

15) Describe Cisco's implementation of ISDN BRI

Is a TE1 (i.e. has a built-in ISDN TA)

Available in several routers

SNMP support

PPP with compression

Multiprotocol support

supports DDR (dial on demand routing),

supports call screening

multiple bearer channels

bandwidth on demand

isdn switch-type <type>               (telco dependant)

ip route <network> <mask> <next-hop>  (setup static route)

dialer-list <group> protocol <prot-name> [permit|deny]   (access list)

int bri0

encapsulation [ppp|hdlc]

ip address <address> <mask>

dialer-group <group-number>

dialer idle-timeout 300

dialer-map <protocol> <next-hop-address> [name <name>] [speed <speed>]

<dial-string> [broadcast]

isdn spid1 <number>  (to set up Service Profile ID, provider specific)

isdn spid2 <number>  (same for 2nd bearer channel)

Rate adaptation is the setting of the speed parameter in the dialer-map command.

Sub addresses are used if there is more than one device connected to the NT1 eg fax/phone. Use :X appended to the ‘phone number’, where X is the device number.

Bandwidth on demand – use load-threshold to specify when to use extra channel via multilink PPP.  Can be based on traffic (in | out | either).

Call screening use			isdn caller <number>

Number which interface responds to: isdn answer1 <called-party-number>

Both can use sub-addresses.

The following example enables Multilink PPP on BRI 0:

interface BRI0

description Enables PPP Multilink on BRI 0

ip address 7.1.1.1 255.255.255.0

encapsulation ppp

dialer map ip 7.1.1.2 name starbuck 14195291357

dialer map ip 7.1.1.3 name roaster speed 56 14098759854

ppp authentication chap

ppp multilink

dialer load-threshold either 10

dialer-group 1

Monitoring ISDN

Show controller bri00		bri unit and D channel info

Show interface bri0		interface info

Show dialer				dialer group info

## IOS

16) Login to router in both user and privileged modes

Use 'enable' to enter privileged mode.

Use ‘disable’ to exit privileged mode.

17) Use the context sensitive help facility

Symbolic translation

Keyword completion, use "?"

History (see below 9)

Command prompts

Syntax checking

18) Use the command history and editing features

^P (for last command)

^N (for next command)

<tab> (for command completion)

show history (for display of history buffer)

term history size number-of-lines

<no> terminal editing (to dis-/enable history)

19) Examine router elements (RAM, ROM, CDP, show)

RAM:   Holds config, IOS, routing tables.  Has a shared,packet,or IO memory for buffers.

ROM:   Holds ROM monitor and boot image

NVRAM: Holds startup-config file

Flash: Holds config files, or new boot images.

CDP:   Cisco Discovery Protocol, get info about other devices on network

show version			display IOS information

show flash				display information on flash memory

show memory				display information on tables and buffers

show buffers

show stacks

show processes CPY            shows processes running in RAM

show protocols			show protocols in RAM

cdp enable	interface config command to turn on CDP on the interface

show cdp				show CDP settings

show cdp interfaces		shows CDP for the routers interfaces

show cdp entry routerB		show info on routerB gained from CDP

show cdp neighbours 		show list of neighbours

show cdp neighbours detail	show detail on the neighbour

20) Manage configuration files from the privileged exec mode.

show startup-config				show conf

show running-config				write term

erase startup-config				erase mem

configure [ terminal | memory ]		conf t

21) Control router passwords, identification, and banner

Passwords:

Set privileged mode password:

enable password  PASSWORD		For older systems

enable secret PASSWORD		Takes precedence

Line passwords, for logins. con is console port, vty is telnet

The login command after it means a password is prompted for, without it there would be no password needed to access the router.

line con 0

password PASSWORD

login

line vty 0 4

password PASSWORD

login					login enables password prompt

Router hostname

hostname NAME

Interface Identification:

Router[config]# interface s0

Router[Config-if]# description Serial link to remote office

Bammers:

First an motd banner is displayed, then a login banner, then once logged in either an exec banner if serial connection, or incoming banner if a telnet connection.

banner [exec|incoming|login|motd]  [delimiter character]

lines of banner  ending with delimiting character

eg banner login ^

sod off!

^

22)  Identify main commands for Cisco startup

EXEC command:

reload				(reboot Cisco)

ROM monitor command:

boot					(boots from ROM - usual default)

boot flash				(boots from flash)

boot filename ipaddress		(boots via tftp)

Configuration commands - global

boot system flash

boot system rom

boot system [tftp | rcp] filename ipaddress

Configuration Register – global configuration

config-register 0x10X

Where X is a hex digit for boot options:

0x0		Use ROM monitor – manual boot

0x1		Boot from ROM

0x2 – 0xF	Look in NVRAM for ‘boot system’ commands

To check current value of config-register do

show version

23) Enter an initial configuration using the setup command

Asks for standard questions - follow menu:

- hostname

- enable secret (password)

- enable password

- enable virtual terminal password

- SNMP

- IP (routing etc.)

- DECNet, XNS, Novell, Apollo, AppleTalk, Vines

- Bridging

- Interfaces

Then asks if you want to use this config. If Yes then saves

to NVRAM, else process begins again

Ctrl-C stops/cancels it.

? at a prompt gives help.

24) Copy and manipulate configuration files

copy running-config startup-config   		write mem

copy [tftp | rcp] running-config 		conf network

copy [tftp | rcp] startup-config		conf overwrite-network

copy running-config [tftp | rcp]		write network

erase startup-config				erase mem

25) List the commands to load Cisco IOS from flash, tftp, ROM.

Configuration commands - global

boot system flash (from flash)

boot system rom (from ROM)

boot host [tftp | rcp] [filename] [ip address]

boot network [tftp | rcp] [filename] [ip address]   - for network configs,

gets two files

After 'boot host | network' a 'service config' command after it will make the router use that source when it reboots.

26) Prepare to backup, upgrade, and load a backup IOS image

Preparation:

show flash			display free memory and filename!

ping ipaddress		check IP connectivity to tftp server

- check image name on tftp server

Backup:

copy flash tftp   	prompts for IP address and filename

Upgrade:

copy tftp flash   	prompts for IP address and filename and confirm,

and erase flash confirm.

To load from backup is the same as an upgrade, except that it will warn that the image file is being overwritten (if using same filename).

27) Prepare initial configuration and enable IP.

Setup

Add static IP routes with

ip route network [mask] {address | interface } [distance]

ip default-network network

For monitoring:

show ip protocol

show ip route

show ip interfaces

## Network Protocols

28) Monitor Novell IPX operations on the router

show ipx interface (status & parameters)

show ipx servers   (known servers)

show ipx route     (routing table)

show ipx traffic   (number & type of packets)

debug ipx routing activity

debug ipx sap

ping ipx network.node

29) Define the two parts of network addressing, then identify the parts in specific protocol address examples

Network address	path part used by router

Host address	device on the network

e.g 	IPX		010a.0123.0123.0123	010a		0123.0123.0123

IP		130.111.123.123		130.111	123.123

X.25		12349876543210		1234		9876543210

30) Create the different classes of IP addresses (and subnetting)

A - Very large sites,   0-127, 16,777,214 hosts,       126 networks

B - Large sites,      128-191,     65,534 hosts,    16,384 networks

C - Small sites,      192-223         254 hosts, 1,097,152 networks

D - Multicast,        224-239

E - Experimental,     240-255

31) Configure IP addresses

conf t

int eth0

ip address xx.xx.xx.xx y.y.y.y [secondary]

x.x.x.x is address, y.y.y.y is netmask

Secondary is for multiple addresses - virtual networks.

ip domain-lookup			Enable DNS

ip name-server			Set DNS server

32) Verify IP addresses

telnet		application level

ping			icmp level

trace			check path packet is taking

33) List the required IPX address and encapsulation type

show ipx int eth0   (both displayed on 2nd line)

Name			Cisco type			Header format

ethernet II		arpa				Ethernet  			ipx

eth 802.2		sap				802.3  802.2 LLC   	ipx

eth SNAP		snap				802.3	 802.2 LLC  snap  ipx

eth 802.3		novell-ether		802.3    			ipx

token ring		token

token ring SNAP	snap

hdlc			hdlc				hdlc				ipx

34) Enable Novell IPX and configure interfaces

Novell addresses are composed of network address - 32 bit number, and a node address 48 bit number (MAC address) represented by dotted triplets of 4 hexadecimal numbers. Eg. 4a.0000.0c00.23fe, where 4a is the network. Leading zeros are not needed.  Encapsulation type is optional.

SAP is service advertisement protocol (server announcements)

GNS is clients looking for a server.

ipx routing				(enables RIP and SAP)

int eth0

ipx network 1a [encapsulation-type]

Default encapsulation types:

For Ethernet: novell-ether

For Token Ring: sap

For FDDI: snap

Also subinterfaces can be used if there's multiple networks on the same wire.

Alternatively, the 'secondary' qualifier on the end could have been added, like in the IP example.

ipx routing

ipx maximum-paths 2

interface ethernet 0.1

ipx network 1 encapsulation novell-ether

interface ethernet 0.2

ipx network 2 encapsulation snap

interface ethernet 0.3

ipx network 3 encapsulation arpa

ip maximum-paths paths   is for load sharing over multiple equal metric paths, eg two serial links to same place.

35) Identify functions of the TCP/IP transport-layer protocols

UDP: Connectionless communication - unreliable

TCP: Connection-oriented communication - reliable

36) Identify functions of the TCP/IP network-layer protocols

IP:   Connectionless datagram delivery

RIP:  Routing (also other protocols like OSPF, [E]IGRP)

ICMP: see below

ARP:  MAC address resolution

RARP: IP address resolution, usually for bootp

37) Identify the functions performed by ICMP

Control and messaging functions:

Error messaging

Router discovery

Diagnostics/testing

38) Configure IPX access lists and SAP filters to control basic Novell traffic

Standard lists are 800-899, extended lists 900-999, SAP lists are 1000-1099

The Cisco router does not forward SAP broadcasts, but constructs its

own SAP table and broadcasts that (every 60 seconds [standard])

A –1 for service/port specifies ‘any’ service/port.

Standard access-list:		(use number 800-899)

Basically filter on network or node address only.

access-list <number> (deny|permit) src-net[.srcnode[srcnodemask]]  [dst-net[.dstnode][dstnodemask]]

int eth0

ipx access-group <list-number> {in | out]

Extended access list:		(use number 900-999)

Basically has protocol and socket numbers.  Also log option.

access-list <number> (deny|permit) protocol [src-net[.srcnode] [srcnetmask.srcnodemask]] src-socket [dst-net[.dstnode][dstnetmask.dstnodemask]]

dst-socket [log]

int eth0

ipx access-group <list-number> {in | out]

SAP:					(use number 1000-1099)

To create list -

access-list <number> (permit|deny) network[.node] [network-mask node-mask] [service type[server-name]]

Apply filter(s) to interface -

int eth0

ipx input-sap-filter <number>  (filter before constructing SAP table)

ipx output-sap-filter <number> (filter when sending SAP table)

ipx router-sap-filter <number>  (filter adverts from a certain router)

## Routing

39) Add RIP to configuration

router rip

network xx.xx.xx.xx

network yy.yy.yy.yy

The network command specifies the interfaces that RIP info is sent and received.  Also only those interfaces will be advertised.

debug ip rip		(for debugging)

40) Add IGRP to conf

router igrp <autonomous system number>

network xx.xx.xx.xx

network yy.yy.yy.yy

The network command specifies the interfaces that IGRP info is sent and received.  Also only those interfaces will be advertised.

41) Explain the services of seperate and integrated multiprotocol routing

Seperate:   "ships-in-the-night" routing, i.e. no protocol is

affected any other protocol

Integrated: Replaces native routing algorithm, exchanges common

routing info, still maintains seperate routing tables,

saves resources, simplifies admin's tasks

E.G EIGRP does IP,IPX, appletalk.

42) List problems each routing type encounters when dealing with topology changes and describe techniques to reduce the number of these problems

Distance Vector limitations:

Time to convergence, e.g. distance vector (hop-count) updates

occur from router to router (not broadcast)

PROBLEM: Routing loops (slow detection of topology change, incorrect routing table/updates)

SOLUTION 1: Split horizon - If you learn a route on one interface do not send back that info on that same interface – so as not to cause loops.

SOLUTION 2: Route poisoning - If net unreachable set route metric to infinity.

PROBLEM:  Network topology keeps changing giving inconsistent network.

SOLUTION: Hold-down timers - Ignore updates for a set period so as not to screw up convergence if an interface goes up and down many times in a short period.

PROBLEM: Counting to infinity (incorrect routing table/updates)

SOLUTION: Define a maximum metric ("max. hop count +1 = infinity"). If metrics get too high due to routing loops.

Link state routing problems:

Processing power, e.g. link-state (OSPF) needs lots of processing

power to rebuild the routing database (tree)

Network bandwidth, e.g. initial (multicast) link-state info floods the network

PROBLEM: Unsynchronised updates can arrive at different time based

on bandwidth (slow and fast links)

SOLUTION: Use time stamps, update numbering and counters

PROBLEM: Synchronisation of large networks

SOLUTION: Use link state area hierarchy for topology

SOLUTION: Exchange route summaries at area borders

PROBLEM: Bandwidth and resources usage

SOLUTION: Dampen the periodic update (longer intervals)

SOLUTION: Use targeted multicast (not flood)

PROBLEM: partitioned regions – slow parts separated from fast parts

SOLUTION: manage network using area hierarchy

43) Describe the benefits of network segmentation with routers

## Network Security

44) Configure standard and extended access lists to filter IP traffic

Used to permit or deny a complete protocol suite based on the source

network/subnet/host address

Are in the range of 1-99 (IP).

Are processed top-down, i.e. first matching rule preempts further

processing

Only one access list per port per direction per protocol

Uses a wildcard mask to define which bits of the network address are

relevant (0 = check this bit, 1 = ignore [inverted subnet mask])

Place standard lists close to the destination, place extended list close

to the source

Standard access lists

access-list <number> (permit|deny) <src> [<src wildcard>]

int eth0

ip access-group <number> (in|out)

Extended access lists

access-list <number> (permit|deny) prot src src-wildcard src-port dst dst-wildcard dst-port [est] [log]

int eth0

ip access-group <number> (in|out)

45) Monitor and verify access lists

show ip interfaces 				(to see applied access lists)

show ipx interfaces

show access-lists <number>

## LAN Switching

46) Describe the advantages of LAN segmentation

Smaller broadcast domains (via routers)

Smaller collision domains (via switching)

Dedicated circuits for high-load devices

Relieve congestion

47) Describe LAN segmentation using bridges

Connects several LAN together, either local or wide-area

Link-layer function

48) Describe LAN segmentation using routers

Connects LANs together, but seperates broadcasts and possibly

protocols

Each segment is a broadcast domain of it's own

Can connect networks that use different media

49) Describe LAN segmentation using switches

Connects several LAN together locally, same protocol

Link-layer function

Each segment is a collision domain of it's own

50) Name and describe two switching methods

Cut-through and store and forward. see below 57)

51) Full- and half-duplex operation

Full:  Send and receive data concurrently in both directions - no collisions

on the segment

Half:  Only one can send at any given time

52) Describe the network congestion problem in Ethernet networks

Collisions:  Too many devices in one segment, too many want

to "talk" at the same time

53) Describe the benefits of network segmentation with bridges

54) Describe the benefits of network segmentation with switches

Dedicated bandwidth to each port. Full duplex.

55) Describe the features and benefits of Fast Ethernet

100 Mbps, faster than 10 Mbps

Proven technology, supported by known architecture

Price, performance, ease of migration

High reliability, wide industry support

100 Base-T standard

Allows for ether channels (bundling of lines)

Coexists with 10 Base-T

56) Describe the guidelines and distance limitations of Fast Ethernet

CAT-5 cabling required

Maximum 100 meter cable run for UTP, 2000 meters for fibre

(to stay within 512 bit times)

57) Distinguish between cut-through and store-and-forward switching

Cut-through:        Determine destination after first 64 bytes

Packet switching approach that streams data through a switch so that the leading edge of a packet exits the switch at the output port before the packet finishes entering the input port. A device using cut-through packet switching reads, processes, and forwards packets as soon as the destination address is looked up  and the outgoing port determined. Also known as on-the-fly packet switching.

Some routers have two different versions, one called FastForward forwards after the first bit is received.  The other is FragFree which receives 64 bytes to make sure it isn’t a collision fragment and then starts forwarding - still has a low latency but with less errors than the FastForward.

Compare with Store-and-forward: Switch pulls in the complete packet first and checks CRC and destination and then forwards it.

58) Describe the operation of Spanning Tree Protocol and it’s benefits

Ensures the existence of a loop-free topology in networks that

contain parallel bridges/switches.  Uses 802.1d protocol between bridging devices to communicate.  Bridges regularly check neighbours to see they're still up, if not a spanning tree is recalculated.  Bridges put interfaces which would create loops into a standby mode - ie don't forward traffic.

59) Describe the benefits of virtual LANs

Broadcast control

Performance - put power users on their own lan

Security - isolate a part of the network or restrict access to it via

firewalling router

Network management - no longer need to repatch to change network.

60)  Define and describe a MAC address

Link layer address to uniquely identify a device/port in a LAN

segment, is 48 bits (= 6 bytes) long, first 3 bytes identify a

vendor code

Notes:

Telnet escape sequence is <ctrl><shift><6><x>

Logging:

no logging buffered		log to console

terminal monitor			log to telnet session

logging on				log to UNIX syslog

logging 202.36.149.2

show logging

logging buffered			logs are kept in buffers in the router

When troubleshooting:

>Show interface ser0

serial0 is up, line protocol is up		working

serial0 is up, line protocol is down	connection problem – no keepalives

serial0 is down, line protocol is down	interface problem – no carrier

serial0 is administratively down		interface disabled

Physical layer deals with carrier, data link layers deals with keepalives and control and user information.