1. Show version

2. show processes - you can check router load in the previous 5 seconds, 1 minute and 5 minutes. the PID is the process ID, the Q is the queue priority (L = low, M=medium, H=high) . Ty  is a status test of the processes. PC is the current program counter. Runtime is the total CPU time the process has used. Invoked is the number of times the process has been invoked. uSecs is the number of microseconds for each process invocation. Stacks tells you the low watermark, and the total memory used (in bytes). TTY tells which terminal owns the process. Process displays the name of the process.

3. show memory

4. show buffers

5. show flash

6. show protocols

7. show ip protocol

8. debug ip rip

9. show ip route

10. debug ipx routing activity

11. no debug all

You can also use undebug all, or the southern version of “u all” (This actuall works)

12. show frame-relay pvc

13. cdp timer 30

14. show cdp neighbors

15. show cdp entry *

16. show cdp interface ethernet0

17. show access-lists

18. show debugging

19. show cdp neighbors detail

20. show frame-relay lmi interface serial0

21. show frame-relay route

22. debug frame-relay lmi

23. show version

24. show ip ospf 100

25. show clock

26. cdp holdtime 120

(the default is 180 seconds)

27. cdp timer 30

28. clock rate 56000

29. encapsulation ppp

30. encapsulation frame-relay ietf

31. description "Internet Connection"

32. ip address 172.16.10.1 255.255.0.0 secondary

33. router rip

34. access-list 101 deny tcp any 172.16.10.1 0.0.0.0 eq 23

access-list 101 permit ip any any

(or) access-list 101 deny tcp any 172.16.10.1 0.0.0.0 eq telnet

access-list 101 permit ip any any

35. exit

36. access-list 1000 deny -1

37. router rip, timers basic 60

38. configure terminal ; interface serial 0 ;  no shutdown

39. copy running-config tftp

40. write mem

(or) copy running-config startup-config