50 Concepts you should understand before taking the exam

- Routing

- Switching

- Bridging

- What Routing Protocols are and what they do.

- Installation Procedure: AutoInstall procedure steps: first the router sends a SLARP or BOOTP asking for its IP address, then it asks the server who answered for a file named network-confg via TFTP. the network-config contains the routers hostname, finally it uses the hostname to request a file named hostname-config that contains its specific hardware entries, the TFTP server downloads the new configuration file to the router. If the autoinstall doesn't provide a hostname, then it sends a DNS request to a name server, if the TFTP doesn't have a file named hostname-confg then it will send router-confg, then you can telnet to the router and make specific changes, a new router can install from an existing router using a TFTP server using e, to, f interfaces.

- CDP works at the data-link layer, so it only works when two Cisco routers are directly connected. It allows any device using CDP to know about all devices on the same network (regardless of upper layer protocols)  CDP broadcasts every 60 seconds, and an entry expires after 180.

show cdp interface to display a summary of CDP information such as encapsulation

show cdp entry routername : displays a single cached CDP entry, this displays all layer 3 address such as IP, appletalk and CLNS.

show cdp neighbors : displays cdp updates received by the local router

show cdp neighbor detail will give you even more detail.

7. Carrier Detect signal refers to the physical layer, line protocol is triggered by keep-alive frames which are in the datalink layer. If line is up, and line protocol is down then there is a connection or clocking problem. If line is down and line protocol is down then there is a interface problem.

8. RIP is specified in 1723, which updates 1058. The 1723 allows RIP to carry additional information and add some security features. The "router invalid timer" determines the length of time that a route is deemed invalid. The "route flush timer" when this expires the route is removed from the routing table. Before a route may be removed from a table the router must notify its neighbors that the route is invalid.

the HOLD down time should be just greater than the time it takes to make the entire network aware of any changes that might take place.

9. IGRP (Interior Gateway Routing Protocol) was developed by CISCO for routing within Autonomous systems (AS). IGRP improves on RIP by offering a Larger hop count, multiple metrics, so it much more suitable for compex networks. IGRP has a max hop count of 255. Cisco ported IGRP to run in OSI CLNP (Conectionless Network Protocol) IGRP is a distance vector protocol. distance vector protocols send all or part of their routing tables in routing update messages. Periodic routing table messages are sent at regular intervals, using this routers can calculate the distance to all nodes in an AS. IGRP uses the following metrics: internetwork delay, bandwidth, reliability, load, which are all set by the Network administrator. For example the reliability and load can take any value between 1 and 255 (255 being the best), saturation is also between 1-255 however 255 is completely saturated. Bandwidth can represent speeds between 1200bps to 10gbps, the delay metric uses units of 10ms. IGRP also uses hold downs, split horizon, and poison reverse updates. IGRP uses a series of timers, they include an update timer, invalid timer, and hold-down period, a flush timer. The update timer specifies how frequently the routing messages should be sent. the IGRP default for this is 90 seconds, the invalid timer specifies how long a router should wait (without receiving any updates) before it marks a route invalid. The IGRP default for the invalid timer is 3x the update period. The IGRP default for the flush timer is 7x the update period. IGRP advertises three types of routes: System, Interior, Exterior. Interior routes are routes between subnets in the network attached to a router interface. If the network attached to a router is not subnetted IGRP does not advertise its route. System routes are routes within an AS. Exterior routes are considered gateways of last resort. Before you can enable IGRP you must disable rip.To turn on IGRP use the command "router igrp 59" (59 is the AS number)

10. Split horizons never send the information back the way they came, this way it avoids routing loops.

SAP packets correspond to the application, presentation, and session layers.

802.3, FDDI, and Token Ring do not map onto the Logical link control sublayer, Arcnet and PPP do.

ISDN Basic Rate Interface Service defines digital service to the user. That is the connection from the ISDN office to the user location provides for access to three channels. The channels are two 64Kb B-channels and one 16Kb D-channel.

The B-channels and the D-channel provide the user with access to the circuit switched network (this is the telephone network we are familiar with) and also provides access to the X.25 packet network. The channels can be provisioned as follows.

B1 - Voice, Data, B channel Packet (high speed 64Kb)

B2 - Voice, Data, B Channel Packet (high speed 64Kb)

D - Signaling, D Channel Packet (low speed 16Kb)

Primary Rate Interface

ISDN Primary Rate Interface service provides digital access via a T1 line. A T1 line provides a 1.544 bandwidth. This bandwidth is divided into 24 64Kb channels. The ISDN PRI service uses 23 of the T1 channels to provide B channel access and uses the 24th channel for signaling purposes. ISDN call control procedures use packetized messages to initiate, monitor, and release connections. In a BRI call these messages are routed via the D channel. On the PRI service the call control messages are routed over the 24th channel which is the D channel in this instance.

WAN standards describe physical layer  and data-link layer. Physical layer covers electrical, mechanical, operational, and functional connections for wide area network services. DCE is the service provider, DTE is the customer. Examples of DTE's are modems or CSU/DSU's. WAN data-link protocols describe how frames are carried between systems on a single data link, they include protocols designed to work for point to point, multipoint, dedicated facilities, and multiaccess switched services such as frame relay.

Common datalink encapsulations with synchronous serial lines are SDLC (Synchronous Data Link Control), PPP (Point to Point), HDLC (HighLevel Data Link Control), LAPB (Link Access Protocol Balanced).

SDLC is a bit oriented network designed by IBM for SNA networks, it defines a a multipoint WAN environment that allows several stations to connect to a dedicated facility. SDLC defines a primary station, and one or more secondary stations. Communcations are ALWAYS between a primary and secondary station. SDLC frames always contain the address of the secondary station involved, it does not contain the address of the primary location.

PPP is defined in RFC 1548, it was derived from SDLC. PPP uses LCP (Link Control Protocol) to establish, configure, maintain and terminate the point to point connection. LCP phases are: Link Establishment & Configuration, Link Quality determination, network layer protocol configuration negotiation, link termination. LCP works very much like the 802.2 LLC.

HDLC (HighLevel data link control) is an ISO standard, it is also the default for Cisco routers. The cisco HDLC frame uses a proprietary type field that acts as a protocol field - that makes it possible for multiple network layer protocols to share the same link. HDLC supports both Multipoint and point to point configurations.

LAPB (Link Access Procedure Balanced)  is primarily used with X.25 but can also be used as a simple data-link transport. LAPB uses the same frame format as SDLC.  LAPB is capable of detecting out-of-sequence frames, detecting missing frames, exchanging frames, retransmitting frames, acknowledging frames.

Bandwidth on demand, DDR = dial on demand routing, advanced DDR (access lists, rotary group, ddr for IPX), dial backup, dial backup using ISDN. DDR in-band calls start when the router receives a packet marked interesting, interesting packets are ones that pass the access lists. You can use different DDR access lists for each destination.

Isdn switch-type basic-net3

# basic-net3 is the service providers switch type.

Interface ethernet0

Interface bri0

Encapsulation ppp

Ip address 172.16.46.1 255.255.255.0

Dialer idle-timeout 60

Dialer map ip 172.16.46.2 8064325

# dialer map is used to associate one or many ip’s with a single location

# the next hop is the number of the router address

# 8064325 is the phone number of the remote location.

Dialer-group 1

Dialer-list 1 protocol ip permit

# valid protocol names are IP and IPX

Ip route 172.16.192.0 255.255.255.0 172.16.46.2

Additional notes:

Valid dial group numbers are 1 – 10

Dialer load threshold [load], can be used to specify a maximum load before the dialer places another call to the destination.

Dialer fast-idle [seconds], is used so that if the link is idle and another active connection needs to dial it will drop the existing connection, the seconds is the amount of time the link must be idle before another active connection can use the interface.

Dialer idle-timeout [seconds], number of idle seconds before a call is disconnected

Access lists can be used with DDR to help clarify what is “interesting”, however they may be used to specify what is interesting for dialing, once the session is up the access list must be applied to the interface to filter.

Username dublin password secret

Access-list 101 deny igrp any any

Access-list 101 deny icmp any 172.16.192.0 0.0.0.255 echo

Access-list 101 permit ip any 172.16.192.0 0.0.0.255 eq ftp

Dialer-list 1 list 101

Ip route 172.16.192.0 255.255.255.0 172.16.46.2

Cisco routers support both PAP and CHAP

Use the command “Ppp authentication chap” to enable chap on a ppp enabled interface

Use the command “username dublin password secret” the name keyword can be the host name, server name, user ID, or command name.

Use the command “dialer map ip 172.16.46.2 name dublin 8064325” to specify a username and password combination for dialing out. Valid protocols are appletalk, bridge, clns, decnet, ip, ipx, novell, snapshot, vinesbanyan, xns

Defining Criteria for Forwarding or Blocking Packets

When creating an access list, you define criteria which are applied to each packet that is processed by the router; the

router decides whether to forward or block each packet based on whether or not the packet matches the criteria.

Typical criteria you define in access lists are packet source addresses, packet destination addresses, or upper-layer

protocol of the packet. However, each protocol has its own specific set of criteria that can be defined.

For a single access list, you can define multiple criteria in multiple, separate access list statements. Each of these

statements should reference the same identifying name or number, to tie the statements to the same access list. You

can have as many criteria statements as you want, limited only by the available memory. Of course, the more

statements you have, the more difficult it will be to comprehend and manage your access lists.

The Implied "Deny All Traffic" Criteria Statement

At the end of every access list is an implied "deny all traffic" criteria statement. Therefore, if a packet does not match

any of your criteria statements, the packet will be blocked.

CPE

customer premises equipment. Terminating equipment, such as terminals,

telephones, and modems, supplied by the telephone company, installed at

customer sites, and connected to the telephone company network.

Demarc

Demarcation point between carrier equipment and CPE

CO

central office. Local telephone company office to which all local loops

in a given area connect and in which circuit switching of subscriber

lines occurs.

DTE

data terminal equipment. Device at the user end of a user-network

interface that serves as a data source, destination, or both. DTE

connects to a data network through a DCE device (for example, a modem)

and typically uses clocking signals generated by the DCE. DTE includes

such devices as computers, protocol translators, and multiplexers.

Compare with DCE.