Article 20996 of sci.electronics:

Path: discus!technion!barilvm!frmop11!psuvm!cunyvm!caen!zaphod.mps.ohio-state.edu!

qt.cs.utexas.edu!cs.utexas.edu!utgpu!watserv1!watmath!mwtilden

From: mwtilden@watmath.waterloo.edu (Mark W. Tilden)

Newsgroups: sci.electronics

Subject: Re: Solar cells and capacitors

Message-ID: <1991Dec3.150139.24012@watmath.waterloo.edu>

Date: 3 Dec 91 15:01:39 GMT

References: <1991Dec2.122934.27134@doug.cae.wisc.edu>

Organization: University of Waterloo

Lines: 58

In article <1991Dec2.122934.27134@doug.cae.wisc.edu> writes:

>I have a little problem.  What I really want to do is this: Make a very low

>frequency relaxation oscillator using solar cells.  You see, I want some

>solar cells to charge up a fat capacitor, and then dump all the capacitor's

>power to a load until the capacitor is almost empty, and finally start the

>whole cycle over again.

Try this:

|-------------------O-------------O-----------------|

|                   |             |                 |

|               |-------|         |                 |

|               <  mot. >-        |               + |

|               |-------|         |                ---

p      2.2k-10k     |           + |              -------

n|--O---/\/\/------O           -----  >2200uF     ---    >2v solar

P    |              |           -----  cap       -------      cell.

|    |              N           - |                ---

|----|------------|p              |              -------

|              n             |               - |

/-/             |             |                 |

^ Zener        |             |                 |

| mc4742       |             |                 |

|--------------O-------------O-----------------|

Legend:

|                      |

collector-     P                      N                      |

base-      -|n     PNP trans.     -|p      NPN trans.    ----O----  wire join.

emitter-       p                      n

|                      |

Essentially it's a modified SCR design with supercritical feedback.  The

zener starts clamping at vcc-.7v and will eventually trigger the PNP,

triggering the NPN, and will lock so long as the motor is moving.  The

beauty of this design is it's incredible high impedance until tripped.

For transistors, I've used 3904s and 3906s, but for higher power designs,

the NPN must be replaced with something like cascaded 2n2222s, or high

gain power transistors, like TIP 31s.  The zener should be rated to trip

at 2 volts below your solar-cell voltage rating to get maximum power.

This circuit is called the BEAM solarengine.  I developed it for the

Robot Olympics, used to build a solaroller device which covers a 1

meter distance in pure sunlight using only a cell 2-1/2" by 1/2".  I

promote it in my BEAM robotics package because of it's simplicity

and cheapness of parts.  What's your application?

Is all.

--

Mark Tilden: _-_-_-__--__--_      /(glitch!)  M.F.C.F Hardware Design Lab.

-_-___       |              \  /\/            U of Waterloo. Ont. Can, N2L-3G1

|__-_-_-|               \/               (519) - 885 - 1211 ext.2454,

"MY OPINIONS, YOU HEAR!? MINE! MINE! MINE! MINE! MINE! AH HAHAHAHAHAHAHAHAHA!!"