| System Unit | Dimensions (HxWxD) | 2.1 x 12.2 x 9.6 in/5.38 x 31.1 x 24.5 cm |  |
| --- | --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |
| Weight |  |  |  |
|  | Armada 1560DM | 7.6 lb/3.45 kg |  |
|  | Armada 1571DM | 7.68 lb/ 3.45 kg |  |
|  | Armada 1573DM | 7.68 lb/ 3.45 kg |  |
|  | Armada 1590DMT | 7.46 lb/3.38 kg |  |
|  | Armada 1592DMT | 7.5 lb/3.4 kg |  |
|  | Armada 1598DMT | 7.97 lb/3.57 kg |  |
|  | Stand-Alone Power Requirements |  |  |
|  | Nominal Operating Voltage (Li-Ion) | 14.2 VDC |  |
|  | Nominal Operating Voltage (NiMH) | 9.6 |  |
|  | Maximum Operating Power | 35 W |  |
|  | Peak Operating Power | 40 W |  |
|  | Temperature |  |  |
|  | Operating | 5 to 95° F/10 to 40° C |  |
|  | Nonoperating | -4 to 140° F/-30 to 60° C |  |
|  | Relative Humidity (noncondensing) |  |  |
|  | Operating | 10 to 90% |  |
|  | Nonoperating | 5 to 95% |  |
|  | Shock |  |  |
|  | Operating | 10 G, 11 ms, half sine |  |
|  | Non operating | 240 G, 2 ms, half sine |  |
|  | Vibration |  |  |
|  | Operating | .55 G, .25 Oct/Min sweep rate |  |
|  | Nonoperating | 1.5 G, .5 Oct/Min sweep rate |  |
|  | Maximum Altitude (unpressurized) |  |  |
|  | Operating | 10,000 ft/3.15 km |  |
|  | Nonoperating | 30,000 ft/10.14 km |  |
| 12.1-Inch Color TFT SVGA Display | Dimensions (HxW) Diagonal Size | 7.24 x 9.7 in/18.4 x 24.6 cm 12.1 in/30.7 cm |  |
|  | Mounting | Internal |  |
|  | Number of Colors |  |  |
|  | Armada 1590DMT and 1592DMT | 16 M |  |
|  | Contrast Ratio | 100:1 |  |
|  | Brightness |  |  |
|  | Armada 1590DMT and 1592DMT | over 100 cd/m2 (typical) |  |
|  | Pixel Resolution |  |  |
|  | Pitch | .30 x .30 mm |  |
|  | Format | 800 x 600 |  |
|  | Configuration | RGB Stripe |  |
|  | Backlight | Edge Lit |  |
|  | Character Display | 80 x 25, 80 x 50 |  |
|  | Total Power Consumption (with backlight) | 6 W (max) |  |

| 12.1-Inch Color STN SVGA Display | Dimensions (HxW) Diagonal Size | 7.24 x 9.7 in/18.4 x 24.6 cm 12.1 in/30.7 cm |
| --- | --- | --- |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |
| Mounting | Internal |  |
|  | Number of Colors |  |
|  | Armada 1560DM | 16 M |
|  | Contrast Ratio | 30:1 (typical) |
|  | Brightness |  |
|  | Armada 1560DM | 100 cd/m2 (typical) |
|  | Response Time |  |
|  | Armada 1560DM | 300 ms |
|  | Pixel Resolution |  |
|  | Pitch | 0.30 x 0.30 mm |
|  | Format | 800 x 600 |
|  | Configuration | RGB Stripe |
|  | Backlight | Edge Lit |
|  | Character Display | 80 x 25, 80 x 50 |
|  | Total Power Consumption (with backlight) | 6 W (max) |
| 12.1-Inch Fast Response Color STN SVGA Display | Dimensions (HxW) Diagonal Size | 7.24 x 9.7 in/18.4 x 24.6 cm 12.1 in/30.7 cm |
|  | Mounting | Internal |
|  | Number of Colors |  |
|  | Armada 1571DM and 1573DM | 16 M |
|  | Contrast Ratio | 35:1 (typical) |
|  | Brightness |  |
|  | Armada 1571DM and 1573DM | 100 cd/m2 (typical) |
|  | Responce Time |  |
|  | Armada 1571DM and 1573DM | 150 ms |
|  | Pixel Resolution |  |
|  | Pitch | 0.30 x 0.30 mm |
|  | Format | 800 x 600 |
|  | Configuration | RGB Stripe |
|  | Backlight | Edge Lit |
|  | Character Display | 80 x 25, 80 x 50 |
|  | Total Power Consumption (maximum) | 6 W (max) |
| 13.3-Inch Color TFT XGA Display | Dimensions (HxW) Diagonal Size | 10.6 x 7.9 in/26.9 x 20.1 cm 13.3 in/33.8 cm |
|  | Mounting | Internal |
|  | Number of Colors | 64K |
|  | Contrast Ratio | 125:1 |
|  | Brightness |  |

Armada 1598DMT105 +/- 10 cd/m2

| 13.3-Inch Color TFT | Pixel Resolution |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XGA Display (cont’d) | Pitch |  | 0.264 x 0.264 mm |  |  |
|  | Format |  | 1024 x 768 |  |  |
|  | Configuration |  | RGB stripe |  |  |
|  | Backlight |  | Edge Lit |  |  |
|  | Character Display |  | 80 x 25 |  |  |
|  | Total Power Consumption (maximum) |  | 4.0 W |  |  |
| Hard Drives1 |  |  | 2.1-GB | 3.2-GB | 4.0-GB |
|  | Capacity Per Drive |  | 2.1 GB | 3.2 GB | 4.0 GB |
|  | Drive Type |  | 65 | 65 | 65 |
|  | Transfer Rate |  |  |  |  |
|  | Media |  | 39.1 to 61.8 Mb/s | 51.7 to 83.4 Mb/s | 51.7 to 83.4 Mb/s |
|  | Interface |  | 16.6 MB/s | 16.6 MB/s | 16.6 MB/s |
|  | Sector Interleave |  | 1:1 | 1:1 | 1:1 |
|  |  | Seek Times (typical read, including settling) |  |  |  |
|  |  | Track-to-track | 4 ms | 4 ms | 4 ms |
|  | Average |  | 13 ms | 13 ms (read) 14 ms (write) | 13 ms (read) 14 ms (write) |
|  | Maximum |  | 23 ms | 23 ms | 23 ms |
|  | Disk Rotational Speed |  | 4009 rpm | 4009 rpm | 4009 rpm |
|  | Physical Configuration |  |  |  |  |
|  | Cylinders |  | 4975 | 6975 | 6975 |
|  | Heads |  | 6 | 5 | 6 |
|  | Sectors per track |  | 110-180 | 144-240 | 144-240 |
|  | Bytes per sector |  | 512 | 512 | 512 |
|  | Logical Configuration |  |  |  |  |
|  | Cylinders |  | 4200 | 6304 | 7944 |
|  | Heads |  | 16 | 16 | 16 |
|  | Sectors per track |  | 63 | 63 | 63 |
|  | Bytes per sector |  | 512 | 512 | 512 |
|  | Buffer Size |  | 96 KB | 512 KB | 512 KB |
| Diskette Drive | Diskette Size |  | 3.5-inch |  |  |
|  | Light |  | None |  |  |
|  | Height |  | .82 in/2.08 cm |  |  |
|  | Bytes per Sector |  | 512 |  |  |
|  | Sectors per track |  |  |  |  |
|  | High density |  | 18 (1.44-MB)/15 (1.2-MB) |  |  |
|  | Low density |  | 9 |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |  |
| Tracks per side |  |  |  |  |  |
|  | High density |  | 80 (1.44-MB)/80 (1.2-MB) |  |  |
|  | Low density |  | 80 |  |  |

Read/Write heads2

| Diskette Drive (cont’d) | Access Times |  |  |  |
| --- | --- | --- | --- | --- |
|  | Track-to track (high/low) | 3 ms/6 ms |  |  |
|  | Average (high/low) | 94 ms/174 ms |  |  |
|  | Settling time | 15 ms |  |  |
|  | Latency average | 100 ms |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| Integrated AC Adapter | Weight | .66 lb/.3 kg |  |  |
|  | Power Supply (Input) |  |  |  |
|  | Operating Voltage | 90 to 260 VAC RMS |  |  |
|  | Operating Current | 1.10 A RMS |  |  |
|  | Operating Frequency Range | 47 to 63 Hz AC |  |  |
|  | Maximum Transient | 4/50 kV |  |  |
| Li-Ion Battery Pack | Dimensions (HxWxL) Weight | .82 x 3.2 x 5.8 in/2.08 x 8.13 x 14.73 cm .85 lb/.39 kg |  |  |
|  | Energy Voltage | 14.4V |  |  |
|  | Amp-hour capacity | 2.8 Ah |  |  |
|  | Watt-hour capacity | 40 Wh |  |  |
|  | Temperature Operating Nonoperating | 50° to 104° F/10° to 40° C -12° to 140°F/-30° to 60°C |  |  |
| NiMH Battery Pack | Dimensions (HxWxL) Weight | .82 x 3.2 x 5.8 in/2.08 x 8.13 x 14.73 cm 1.2 lb/.54 kg |  |  |
|  | Energy Voltage Amp-hour capacity Watt-hour capacity | 9.6 V 4.0 Ah 36 Wh |  |  |
|  | Temperature Operating Nonoperating | 50° to 104° F/10° to 40° C -22° to 140°F/-30° to 60°C |  |  |
| Convenience Base | Dimensions (HxWxD) | 4.9 x 14.2 x 14.7 in/12.46 x 36.07 x 37.34 cm |  |  |
|  | Weight | 4.25 lb/1.93 kg |  |  |
|  | Power Supply (Input) Operating Voltage Operating Current Rated Voltage Rated Current Line Frequency | 100 to 240 VAC 1.10 Amp Maximum 100 to 240 VAC 1.0 Amp Maximum 47 to 63 Hz |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| Temperature Operating Nonoperating | 50 to 95°F/10 to 35°C -4 to 140°F/20 to 60°C |  |  |  |
|  | Relative Humidity Operating Nonoperating | 10 to 90% 5 to 95% |  |  |
|  | Altitude Operating Nonoperating | 10,000 ft/3.15 km 30,000 ft/10.14 km |  |  |
| Convenience Base (cont’d) | Shock Operating Nonoperating | 10 G, 11 ms, half sine 140 G, 2 ms, half sine |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| Vibration Operating Nonoperating | 0.25 G, 5 to 500 Hz, 0.5 octave/min sweep rate 1.0 G, 5 to 500 Hz, 0.5 octave/min sweep rate |  |  |  |
|  | Monitor Support | Up to 55 lb/24.95 kg |  |  |
| 20X Max2 CD-ROM Drive | Applicable Disc | CD-DA, CD-ROM, CD-ROM XA (except ADPCM), CD-1, Photo CD (Multi Session), Video CD, CD-Extra (CD+), CD-G |  |  |
|  | Center Hole Diameter | 15 mm |  |  |
|  | Disc Diameter Disc Thickness | 12 cm, 8 cm 1.2 mm |  |  |
|  | Track Pitch | 1.6 µm |  |  |
|  | Laser |  |  |  |
|  | Output power | 0.24 +/- 0.1 mW |  |  |
|  | Type | GaAiAS double hetero laser diode |  |  |
|  | Wave length | 770-795 nm at Ph-2.7 mW |  |  |
|  | Access Times (typical, including settling) Audio Output Level | 150 ms |  |  |
|  | Line out | 0.8-1.2 Vrms |  |  |
| . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . |  |  |  |  |
| Headphone | none |  |  |  |
|  | Cache Buffer | 128 KB |  |  |
|  | Data Transfer Rate (typical, including settling) |  |  |  |

Sustained (16X)2400 KB/s
150 KB/sNormal PIO Mode 4 (single burst)16.66 MB/sStartup TimeLess than 8 seconds

1 Hard drive specifications may vary slightly depending on vendor.

2 Data transfer rates vary from 1276 KB/s to 3000 KB/s.

Compaq, registered United States Patent and Trademark Office. Armada, Armada Station, Ministation, MultiLock, QuickBlank, QuickLock, are trademarks and/or service marks of Compaq Computer Corporation.

Other product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1998 Compaq Computer Corporation. All rights reserved. Printed in the U.S.A.