Models . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Deskpro 4000

Model 5166X/2400 (16 MB)

27060x-xx2

Model 5200X/2400
(32 MB)

27064x-xx4

Model 5233X/3200/CDS 
(32 MB)

27076x-xx5

Model 6233X/2400
(32 MB)

27065x-xx4

Model 6266X/3200/CDS
(32 MB)

27068x-xx3

Model 6300X/3200/CDS
(32 MB)

27077x-xx6

Model 6333X/3200/CDS 
(32 MB)

32270x-xx3

Standard Features

Processor

Intel Pentium® Processors with MMX™ technology  166, 200 or 233 MHz or Intel Pentium II Processors 
233, 266, 300 or 333 MHz

Upgradability

Allows for upgrades of processor, memory and hard drives

Graphics

Pentium Processor with MMX™ technology models:  S3 ViRGE/GX Enhanced 64-bit Graphics PCI card (2-MB SGRAM standard with maximum resolution 1280 x 1024 with 256 colors, upgradable to 4 MB-SGRAM with 1280 x 1024x 65K colors)

Pentium II Processor models:  Matrox MGA 1064/SG Graphics Accelerator PCI card (2-MB SGRAM standard with maximum resolution of 1280 x 1024 x 256 colors, upgradable to 4-MB SGRAM providing 1600 x 1200 x 256 colors)

Memory

Models standard with 16-MB or 32-MB SDRAM expandable to 384 MB

Architecture

PCI/ISA

Expansion Slots/Bays

Pentium Processor with MMX™ technology models:  five-slot/four-bay desktop with two PCI, two ISA and one shared PCI/ISA slot; three externally accessible drive bays and one internal drive bay

Pentium II Processer models:  five-slot/four-bay with two PCI, one ISA, one shared, one AGP; three external and one internal drive bay

Diskette Drive

All Models:  One 1.44-MB diskette drive

CD-ROM Drive/Audio

CD-ROM models preinstalled with a 24X Max1 
CD-ROM Drive IDE (slot-load) on Pentium II Processor models and 16X Max2 CD-ROM Drive IDE (slot-load) on Pentium Processor with MMX™ technology models; Enhanced Business Pro Audio3 input/output jacks and internal speaker

Network Controller

Compaq Netelligent 10/100 TX PCI UTP 
Controller (embedded)

Hard Drives

Model 2400:  2.4-GB Ultra ATA hard drive

Model 3200:  3.2-GB Ultra ATA hard drive

Cache

Standard with 512-KB pipeline burst synchronous level 2

Intelligent Manageability

Intelligent Manageability combines innovative hardware technology with PC LAN management

tools from Compaq and other leading vendors to

make Compaq computers easier to inventory, trouble shoot, protect, and maintain. Intelligent Manageability includes: Asset Management, Fault Management, Security Management, Configuration Management, and Integration Management.

Service-Friendly Design

Includes Quick Release Cover Latches for easy access, Removable Expansion Board Cage, Slide Out Drive Cage and Slide Out System Board

Interfaces

ECP parallel, serial (2), infrared, keyboard, RJ-45, USB (2), mouse and video

Security

Smart Cover Lock, Memory Change Alert, Ownership Tag, Configuration Control Hardware, Smart Cover Sensor, Setup Password, Power-On Password, QuickLock/QuickBlank, Removable Media Boot Control, Removable Media Write Control, Serial Control, Parallel Control, Infrared Control, USB Port Control, and Cable Lock Provision

Energy Conservation

Energy-saver modes for monitor, hard drives processor, power supply, and internal fan, all Energy Star compliant (Energy Star compliant on Pentium Processor with MMX™ technology models; Pentium II Processor models not Energy Star compliant)

Ease of Use

Plug and Play compliant

Power Supply

200 Watts

Software

Windows 95 on Pentium Processor with MMX™ technology models, Windows NT 4.0 on Pentium II Processor models; Insight Personal Edition,

Standard Features (cont’d)

| Software (cont’d) |  |  |  |
| --- | --- | --- | --- |
| Computer Setup Utility, Diagnostics for Windows and Intelligent Manageability |  |  |  |
| Service and Support |  |  |  |
| Protected by Compaq Services including a three-year4 limited warranty with first year on-site, hardware technical phone support and online support through CompuServe, Prodigy, America Online and the Internet. |  |  |  |
| Options . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. |
| Memory |  |  |  |
| 8-MB Memory Kit (1 x 8-MB SDRAM DIMM, 66 MHz) |  |  | 242802-B21 |
| 16-MB Memory Kit (1 x 16-MB SDRAM DIMM, 66 MHz) |  |  | 242803-B21 |
| 32-MB Memory Kit (1 x 32-MB SDRAM DIMM, 66 MHz) |  |  | 242804-B21 |
| 64-MB Memory Kit (1 x 64-MB SDRAM DIMM, 66 MHz) |  |  | 242805-B21 |
| 128-MB Memory Kit (1 x 128-MB SDRAM DIMM, 66 MHz) |  |  | 270812-B21 |
| 16-MB Memory Kit (bulk) (1 x 16-MB SDRAM DIMM, 66 MHz)5 |  |  | 294283-B21 |
| 32-MB Memory Kit (bulk) (1 x 32-MB SDRAM DIMM, 66 MHz)5 |  |  | 294284-B21 |
| 64-MB Memory Kit (bulk) (1 x 64-MB SDRAM DIMM, 66 MHz)5 |  |  | 294285-B21 |
| 16-MB Memory Kit (1 x 16-MB SDRAM ECC DIMM, 66 MHz)6 |  |  | 270840-B21 |
| 32-MB Memory Kit (1 x 32-MB SDRAM ECC DIMM, 66 MHz)6 |  |  | 270841-B21 |
| 64-MB Memory Kit (1 x 64-MB SDRAM ECC DIMM, 66 MHz)6 |  |  | 270842-B21 |
| 128-MB Memory Kit (1 x 128-MB SDRAM ECC DIMM, 66 MHz)6 |  |  | 270843-B21 |
| 16-MB Memory Kit (bulk) (1 x 16-MB SDRAM ECC DIMM, 66 MHz)5,6 |  |  | 294286-B21 |
| 32-MB Memory Kit (bulk) (1 x 32-MB SDRAM ECC DIMM, 66 MHz) 5,6 |  |  | 294287-B21 |
| 64-MB Memory Kit (bulk) (1 x 64-MB SDRAM ECC DIMM, 66 MHz) 5,6 |  |  | 294288-B21 |
| Monitors |  |  |  |
| V50 Color7 |  | 264150-001 |  |
| V70 Color7 |  | 255606-001 |  |
| V90 Color7 |  | 305609-001 |  |
| P50 Color7 |  | 264200-001 |  |
| P70 Color7 |  | 255650-001 |  |
| P110 Color7 |  | 284956-001 |  |
| P1610 Color7 |  | 305708-001 |  |
| TFT500 Flat Panel7 |  | 285006-001 |  |

| Graphics |  |  |  |
| --- | --- | --- | --- |
| 2-MB SGRAM Module Memory Kit |  | 270834-B21 |  |
| Matrox MGA Millennium II Graphics Controller (PCI) |  | 270246-B21 |  |
| 4-MB WRAM Module (for Matrox Millennium II) |  | 270831-B21 |  |
| 12-MB WRAM Module (for Matrox Millennium II) |  | 270833-B21 |  |
| MPEG1 Decoder |  | 294145-B21 |  |
| Diskette Drives/Hard Drives |  |  |  |
| LS-120 Drive |  | 185061-001 |  |
| 2.4-GB Ultra ATA Hard Drive |  | 296604-B21 |  |
| 4.3-GB Ultra ATA Hard Drive |  | 296606-B21 |  |
| 2.1-GB UltraSCSI Hard Drive |  | 185251-001 |  |
| 4.3-GB UltraSCSI Hard Drive |  | 185252-001 |  |
| Storage/Controllers and Adapters |  |  |  |
| Plug and Play SCSI Controller |  | 187749-001 |  |
| PCI UltraSCSI Controller |  | 185250-001 |  |
| Tape Drives |  |  |  |
| TR4 4-GB IDE Tape Drive |  | 186703-001 |  |
| Multimedia |  |  |  |
| 16X Max2 CD-ROM Drive IDE (slot-load) |  | 270706-001 |  |
| 8X PD-CD Drive (IDE) |  | 185261-001 |  |
| Business Multimedia 24X Kit |  | 294147-B21 |  |
| Business Wavetable Sound Card8 |  | 294146-B21 |  |
| Enhanced Business Pro Audio3 |  | 185430-002 |  |
| Multimedia Sound System (speakers) |  | 294106-001 |  |
| Communications |  |  |  |
| Serial/Parallel Interface Board |  | 108406-002 |  |
| Netelligent 10/100 TX PCI UTP |  | 169845-002 |  |
| Netelligent 10 T PCI UTP |  | 242500-001 |  |
| Netelligent 10 T ISA UTP |  | 265615-001 |  |
| Netelligent 10 T/2 PCI UTP/Coax |  | 292856-B21 |  |
| Netelligent 10 T/2 ISA UTP/Coax |  | 265616-001 |  |
| Netelligent 4/16 TR PCI UTP/STP |  | 268009-001 |  |
| Netelligent 4/16 TR ISA UTP/STP |  | 265435-001 |  |
| Netelligent 16/4 TR PCI IBM UTP/STP |  | 301206-B21 |  |
| Netelligent 16/4 TR ISA IBM UTP/STP Controller |  | 301207-B21 |  |
| Netelligent 56K ISA Fax Modem9 |  | 294900-001 |  |
| 336 ISA Fax Modem |  | 259213-002 |  |
| External Infrared Transceiver10 |  | 220687-001 |  |

| Options (cont’d) . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .. |
| --- |
| Other |  |  |
| Mouse | 143315-001 |  |
| Keyboard | 110562-001 |  |
| Windows 95 Space Saver Keyboard | 235212-001 |  |
| Floppyless Bezel | 297051-001 |  |
| TFT500 Arm Option | 285040-B21 |  |
| Ultra ATA 80-Conductor Cable Kit11 | 294162-B21 |  |

1	Data transfer rate will vary from 150 KB/s to 3600 KB/s.

2	Data transfer rate will vary from 150 KB/s to 2400 KB/s.

3	Includes internal speaker.

4	Certain restrictions and exclusions apply. Consult the Compaq Customer Support Center for details.

5	Bulk memory skus are priced individually but must be ordered in 	lots of 50. Only orders of 50 are accepted.

6	There is no exchange or return program available for SDRAM when replacing a standard SDRAM non-ECC based system with SDRAM-ECC memory.

7	AssetControl monitor. AssetControl features are accessible with Compaq desktops featuring Intelligent Manageability.

8	Supports external speakers, speakers not included.

9	The K56flex protocol is designed only to allow faster down loads from K56flex compliant digital sources. Maximum achievable download transmission rates may not reach 56 Kb/s and will vary with line conditions.

10Requires disabling of one Serial port and an optional External 	Infrared Transceiver.

11Use this kit for optimum performance with multiple Ultra ATA 	hard drives.