# Model number information for ESL9000 Tape Libraries

The Model number defines the tape library characteristics and is set at the factory. The model number should only be changed when upgrading to a different drive type or in the case that the Application software doesn’t recognize the Compaq Inquiry string; in this case change the model no. to the ATL model number as shown below.

The home screen will always display either ESL9198 or ESL9326 regardless of the model no. selected; the Standard Product ID. Will reflect either “ESL9000 Series” or “PX000 model-no”(ATL model no.) depending on the model no. selected. The Inquiry page CC will contain the specific model name of the product. Reference the table below for more information.

## ESL9198 libraries

| Model no | Library Model Name (also Inq page CC) | Displayed Name | Std. Vendor ID | Std. Product ID | Max No. of storage bins/drives | Drive Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6318280 * | ESL9198DLX | ESL9198 | Compaq | ESL9000 Series | 198/8 | DLT8000 |
| 6318281 | ESL9198DLX | ESL9198 | Compaq | P2000 6318281 | 198/8 | DLT8000 |

| 6318285 * | ESL9198SL | ESL9198 | Compaq | ESL9000 Series | 198/8 | SDLT |
| --- | --- | --- | --- | --- | --- | --- |
| 6318286 | ESL9198SL | ESL9198 | Compaq | P2000 6318286 | 198/8 | SDLT |

ESL9326 libraries

| Model no | Library Model Name (and Inq page CC) | Displayed Name | Std. Vendor ID | Std. Product ID | Max No. of storage bins/drives | Drive Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6310080 * | ESL9326D | ESL9326 | Compaq | ESL9000 Series | 326/16 | DLT7000 |
| 6310081 | ESL9326D | ESL9326 | Compaq | ESL9000 Series | 170/16 | DLT7000 |
| 6310082 | ESL9326D | ESL9326 | Compaq | P3000 6310082 | 326/16 | DLT7000 |
| 6310085 * | ESL9326DX | ESL9326 | Compaq | ESL9000 Series | 326/16 | DLT8000 (HVD) |

## ESL9326 Mixed Media capable libraries

| Model no | Library Model Name and Inq page CC | Displayed Name | Std. Vendor ID | Std. Product ID | No. of storage bins/drives | Drive Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6313080 * | ESL9326SL | ESL9326 | Compaq | ESL9000 Series | 326/16 | SDLT |
| 6313081 | ESL9326SL | ESL9326 | Compaq | ESL9000 Series | 170/16 | SDLT |
| 6313082 | ESL9326SL | ESL9326 | Compaq | P3000 6313082 | 326/16 | SDLT |

Notes:

- * indicates a default model no.

- Model no’s highlighted in green are available from the GUI, all other model no’s are available through the diagnostic software (may need to use 6310xxx if the model no. isn’t listed).

- All drives are LVD unless otherwise noted.