exercise

20

Electroencephalography

Brain Wave Patterns and the Electroencephalogram

Any physiological investigation of the brain can emphasize and expose only a very minute portion of its activity. Higher brain functions, such as consciousness and logical reasoning, are extremely difficult to investigate. It is obviously much easier to do experiments on the brain’s input-output functions, some of which can be detected with appropriate recording equipment. Still, the ability to record brain activity does not necessarily guarantee an understanding of the brain.

The electroencephalogram (EEG), a record of the electrical activity of the brain, can be obtained through electrodes placed at various points on the skin or scalp of the head. This electrical activity, which is recorded as waves (Figure 20.1), is not completely understood at present but may be regarded as action potentials generated by brain neurons.

Certain characteristics of brain waves are known. They have a frequency of 1 to 30 hertz (Hz) or cycles per second, a dominant rhythm of 10 Hz, and an average amplitude (voltage) of 20 to 100 microvolts (mV). They vary in frequency in different brain areas, occipital waves having a lower frequency than those associated with the frontal and parietal lobes.

The first of the brain waves to be described by scientists were the alpha waves (or alpha rhythm). Alpha waves have an average frequency range of 8 to 13 Hz and are produced when the individual is in a relaxed state with the eyes closed. Alpha block, suppression of the alpha rhythm, occurs if the eyes are opened or if the individual begins to concentrate on some mental problem or visual stimulus. Under these conditions, the waves decrease in amplitude but increase in frequency. Under conditions of fright or excitement, the frequency increases still more.

Beta waves, closely related to alpha waves, are faster (14 to 25 Hz) and have a lower amplitude. They are typical of the attentive or alert state.

Very large (high-amplitude) waves with a frequency of 4 Hz or less that are seen in deep sleep are delta waves. Theta waves are large, abnormally contoured waves with a frequency of 4 to 7 Hz. Although theta waves are normal in children, they represent emotional problems or some sort of neural imbalance in adults.

Brain waves change with age, sensory stimuli, brain pathology, and the chemical state of the body. (Glucose deprivation, oxygen poisoning, and sedatives all interfere with the rhythmic activity of brain output by disturbing the metabolism of neurons.) Sleeping individuals and patients in a coma have EEGs that are slower (lower frequency) than the alpha rhythm of normal adults. Fright, epileptic seizures, and various types of drug intoxication are associated with comparatively faster cortical activity. Thus impairment of cortical function is indicated by neuronal activity that is either too fast or too slow; unconsciousness occurs at both extremes of the frequency range.

Because spontaneous brain waves are always present, even during unconsciousness and coma, the absence of brain waves (a “flat” EEG) is taken as clinical evidence of death. The EEG is used clinically to diagnose and localize many types of brain lesions, including epileptic lesions, infections, abscesses, and tumors. l

Activity 1:

Observing Brain Wave Patterns Using an Oscilloscope or Physiograph

If one electrode (the active electrode) is placed over a particular cortical area and another (the indifferent electrode) is placed over an inactive part of the head, such as the earlobe, all of the activity of the cortex underlying the active electrode will, theoretically, be recorded. The inactive area provides a zero reference point, or a baseline, and the EEG represents the difference between “activities” occurring under the two electrodes.

1.	Connect the EEG lead-selector box to the oscilloscope preamplifier, or connect the high-gain preamplifier to the physiograph channel amplifier. Adjust the horizontal sweep and sensitivity according to the directions given in the instrument manual or by your instructor.

2.	Prepare the subject. The subject should lie undisturbed on a cot or on the lab bench with eyes closed in a quiet, dimly lit area. (Someone who is able to relax easily makes a good subject.) Apply a small amount of electrode gel to the subject’s forehead above the left eye and on the left earlobe. Press an electrode to each prepared area and secure each by (1) applying a film of collodion gel to the electrode surface and the adjacent skin or (2) using a long elastic EEG strap (knot tied at the back of the head). If collodion gel is used, allow it to dry before you continue.

3.	Connect the active frontal lead (forehead) to the EEG lead-selector box outlet marked “L Frontal.” Connect the lead from the indifferent electrode (earlobe) to the ground outlet (or to the appropriate input terminal on the high-gain preamplifier).

4.	Turn the oscilloscope or physiograph on, and observe the EEG pattern of the relaxed subject for a period of 5 minutes. If the subject is truly relaxed, you should see a typical 
alpha-wave pattern. (If the subject is unable to relax and the alpha-wave pattern does not appear in this time interval, test another subject.) Discourage all muscle movement during the monitoring period.*

5.	Abruptly and loudly clap your hands. The subject’s eyes should open, and alpha block should occur. Observe the immediate brain wave pattern. How do the frequency and amplitude of the brain waves change?

Would you characterize this as beta rhythm?

Why?

6.	Allow the subject about 5 minutes to achieve complete relaxation once again, then ask him or her to compute a number problem that requires concentration (for example, add 3 and 36, subtract 7, multiply by 2, add 50, etc.). Observe the brain wave pattern during the period of mental computation.

Observations:

7.	Once again allow the subject to relax until alpha rhythm resumes. Then, instruct him or her to hyperventilate for 3 minutes. Be sure to tell the subject when to stop hyperventilating. Hyperventilation rapidly flushes carbon dioxide out of the lungs, decreasing carbon dioxide levels in the blood and producing respiratory alkalosis.

Observe the changes in the rhythm and amplitude of the brain waves occurring during the period of hyperventilation.

Observations:

8.	Think of other stimuli that might affect brain wave patterns. Test your hypotheses. Describe what stimuli you tested and what responses you observed.

n

Activity 2:

Electroencephalography Using Biopac®

In this laboratory, the EEG of the subject will be recorded during a relaxed state, first with the eyes closed, then with the eyes open while silently counting to ten, and finally with the eyes closed again.

Setting up the Equipment

1.	Connect the BIOPAC® unit to the computer and turn the computer ON.

2.	Make sure the Biopac® MP30 (or MP35) unit is OFF.

3.	Plug in the equipment as shown in Figure 20.2.

•	Electrode lead set—CH 1

4.	Turn the Biopac® MP30 (or MP35) unit ON.

5.	Attach three electrodes to the subject’s scalp and ear as shown in Figure 20.3. Follow these important guidelines to assist in effective electrode placement:

•	Select subjects with the easiest access to the scalp.

•	Move as much hair out of the way as possible.

•	Apply a dab of electrode gel to the spots where the electrodes will be attached.

•	Apply pressure to the electrodes for one minute to ensure attachment.

•	Use a swimcap or supportive wrap to maintain attachment.

•	Do not touch the electrodes while recording.

•	The earlobe electrode may be folded under the lobe itself.

6.	When the electrodes are attached, the subject should lie down and relax with eyes closed for 5 minutes before recording.

7.	Start the BIOPAC® Student Lab program on the computer by double-clicking the icon on the desktop or by following your instructor’s guidance.

8.	Select lesson L03-EEG-1 from the menu, and click OK.

9.	Type in a filename that will save this subject’s data on the computer hard drive. You may want to use subject last name followed by EEG-1 (for example, SmithEEG-1), then click OK.

10.	During this preparation, the subject should be very still and in a relaxed state with eyes closed. Allow the subject to relax with minimal stimuli.

Calibrating the Equipment

1.	Check to make sure that the electrodes remain firmly attached to the surface of the scalp and earlobe. The subject should remain absolutely still and try to avoid movement of the body or face.

2.	With the subject in a relaxed position, click Calibrate.

3.	A prompt will request that you check electrode attachment one final time. When ready, click OK; the computer will record for 15 seconds and stop automatically.

4.	Observe the recording of the calibration data, which should look like Figure 20.4.

•	If the data look very different, click Redo Calibration and repeat the steps above.

•	If the data look similar, proceed to the next section.

Recording the Data

1.	The subject should remain relaxed with eyes closed.

2.	After clicking Record, the “director” will instruct the subject to keep his or her eyes closed for the first 10 seconds of recording, then open the eyes and mentally (not verbally) count to ten, then close the eyes again and relax for 10 seconds. The director will insert a marker by pressing the F9 key (PC) or ESC key (Mac) when the command to open eyes is given, and another marker when the subject reaches the count of ten and closes the eyes. Click Stop 10 seconds after the subject re-closes the eyes.

3.	Observe the recording of the data, which should look similar to the data in Figure 20.5.

•	If the data look very different, click Redo and repeat the steps above. If the subject moved too much during the recording, it is likely that artifact spikes will appear in the data.

•	If the data look similar, proceed to the next step.

4.	Click the buttons for each of the different EEG rhythms at the top of the data in the following order: alpha, beta, delta, and theta. When you click on each button, the software will extract and display each of the specific frequency bands.

•	In order to properly view each of the waveforms, you may have to click the Display menu and select 
Autoscale Waveforms. This function will re-scale the data for the rhythm band that is selected.

5.	Observe the alpha rhythm band of data. The intensity of the alpha signal should decrease during the “eyes open” phase of the recording.

•	If the data do not demonstrate this change, it is possible that the electrodes were not properly attached and you should redo the recording.

6.	When finished, click Done. If you are certain you want to stop recording, click YES. Remove the electrodes from the subject’s scalp.

7.	A pop-up window will appear. To record from another subject select Record from another subject and return to step 5 under Setting up the Equipment. If continuing to the Data Analysis section, select Analyze current data file and proceed to step 2.

Data Analysis

1.	If just starting the BioPac® program to perform data analysis, enter Review Saved Data mode and choose the file with the subject’s EEG data (for example, SmithEEG-1).

2.	Observe how the channel numbers are designated: 
CH 1—raw EEG; CH 2—alpha; CH 3—beta; CH 4—delta; and CH 5—theta.

3.	To set up the display for optimal viewing, hide Channel 1. To do this, hold down the control key (PC) or option key (Mac) while using the cursor to click on channel box 1 (the small box with a 1 in the upper left of the screen).

4.	To analyze the data, set up the first four pairs of channel/measurement boxes at the top of the screen by selecting the following channels and measurement types from the drop-down menus:

Channel	Measurement

CH 2	stdev

CH 3	stdev

CH 4	stdev

CH 5	stdev

stdev (standard deviation): this is a statistical calculation that estimates the variability of the data in the area highlighted by the I-beam cursor. This function minimizes the effects of extreme values and electrical artifacts that may unduly influence interpretation of the data.

5.	Use the arrow cursor and click on the I-beam cursor box on the lower right side of the screen to activate the “area selection” function. Using the activated I-beam cursor, highlight the first 10-second segment of EEG data, which represents the subject at rest with eyes closed (Figure 20.6).

6.	Observe that the computer automatically calculates the stdev for each of the channels of data (alpha, beta, delta, and theta).

7.	Record the data for each rhythm in the chart below, rounding to the nearest 1/100 mV.

8.	Repeat steps 5–7 to analyze and record the data for the next two segments of data, with eyes open, and with eyes re-closed. The triangular markers that were inserted at the top of the data should provide guidance for highlighting.

9.	To continue the analysis, change the settings in the first four pairs of channel/measurement boxes. Select the following channels and measurement types:

Channel	Measurement

CH 2	Freq

CH 3	Freq

CH 4	Freq

CH 5	Freq

Freq (frequency): will give the frequency in Hertz (Hz) of an individual wave that is highlighted by the I-beam cursor.

10.	In order to view an individual wave from among the high frequency waveforms, the zoom function must be used. To activate the zoom function, use the cursor to click on the magnifying glass in the lower right corner of the screen (near the I-beam cursor box). The cursor will become a magnifying glass.

11.	To examine individual waves within the alpha data, click on that band with the magnifying glass until it is possible to observe the peaks and troughs of individual waves within Segment 1.

•	In order to properly view each of the waveforms, you may have to click the Display menu and select Autoscale Waveforms. This function will re-scale the data for the rhythm band that is selected.

12.	At this time, focus on alpha waves only. Reactivate the I-beam cursor by clicking on its box in the lower right corner. Highlight a single alpha wave from peak to peak as shown in Figure 20.7.

13.	Read the calculated frequency (in Hz) in the measurement box for CH 2, and record this as the frequency of Wave 1 for Alpha in the chart below.

14.	Use the I-beam cursor to select two more individual alpha waves and record their frequencies in the table.

15.	You will now perform the same frequency measurements for three waves in each of the beta data (CH 3), the delta data (CH 4), and the theta data (CH 5). Record these measurements in the chart.

16.	Calculate the average of the three waves measured for each of the brain rhythms, and record the average in the chart.

17. When finished, answer the following questions and then exit the program by going to the File menu at the top of the page and clicking Quit.

Look at the waveforms you recorded and carefully examine all three segments of the alpha rhythm record. Is there a difference in electrical activity in this frequency range when the eyes are open versus closed? Describe your observations.

Carefully examine all three segments of the beta rhythm record. Is there a difference in electrical activity in this frequency range when the eyes are open versus closed? Describe what you observe.

This time, compare the intensity (height) of the alpha and beta waveforms throughout all three segments. Does the intensity of one signal appear more varied than the other in the record? Describe your observations.

Examine the data for the delta and theta rhythms. Is there any change in the waveform as the subject changes states? If so, describe the change observed.

The degree of variation in the intensity of the signal was estimated by calculating the standard deviation of the waves in each segment of data. In which time segment (eyes open, eyes closed, or eyes re-closed) is the difference in the standard deviations the greatest?

n

Objectives

1.	To define electroencephalogram, and to discuss its clinical significance.

2.	To describe or recognize typical tracings of the most common brain wave patterns (alpha, beta, theta, and delta waves), and to indicate the conditions under which each is most likely to be predominant.

3.	To indicate the source of brain waves.

4.	To define alpha block.

5.	To monitor electroencephalography and recognize alpha rhythm.

6.	To describe the effect of a sudden sound, mental concentration, and alkalosis on brain wave 
patterns.

Materials

q	Oscilloscope and EEG lead-selector box or physiograph and high-gain preamplifier

q	Cot (if available) or pillow

q	Electrode gel

q	EEG electrodes and leads

q	Collodion gel or long elastic EEG straps

q	BIOPAC® Apparatus: BIOPAC® MP30 (or MP35) data acquisition unit, PC or Macintosh (Mac) computer with at least 4MB of RAM available above and beyond the operating system needs and any other programs that are running, BIOPAC® Student Lab Software v3.0 or greater, wall transformer, serial cable, electrode lead set, disposable vinyl electrodes, Lycra® swim cap (such as Speedo® brand) or supportive wrap (such as 3M CobanTM Self-adhering Support Wrap) to press electrodes against head for improved contact, and a cot or lab bench and pillow.##	Exercise 20

Figure 20.1  Electroencephalography and brain waves. (a) To obtain a recording of brain wave activity (an EEG), electrodes are positioned on the patient’s scalp and attached to a recording device called an electroencephalograph. (b) Typical EEGs. Alpha waves are typical of the awake but relaxed state; beta waves occur in the awake, alert state; theta waves are common in children but not in normal awake adults; delta waves occur during deep sleep.(a)* Note that 60-cycle “noise” (appearing as fast, regular, low amplitude waves superimposed on the more irregular brain waves) may interfere with the tracings being made, particularly if the laboratory has a lot of electrical equipment.	Electroencephalography#

Figure 20.2  Setting up the BIOPAC® MP30 (or MP35) unit. Plug the electrode set into Channel 1.				#	Exercise 20

Figure 20.4  Example of calibration data.

Figure 20.3  Placement of electrodes and the appropriate attachment of electrode leads by color.	Electroencephalography#

Figure 20.6  Highlighting the first data 
segment.

Figure 20.5  Example of EEG data.Standard Deviations (stdev) of Signals in Each Segment

Eyes Closed	Eyes Open	Eyes Re-closed
		Segment 1	Segment 2	Segment 3
Rhythm	Channel	Seconds 0-10	Seconds 10–20	Seconds 20-30

Alpha	CH 2

Beta	CH 3

Delta	CH 4

Theta	CH 5#	Exercise 20

Figure 20.7  Highlighting a single alpha wave from peak to peak.Frequencies of Waves for Each Rhythm (Hz)

Rhythm	Channel	Wave 1	Wave 2	Wave 3	Average

Alpha	CH 2

Beta	CH 3

Delta	CH 4

Theta	CH 5	Electroencephalography#