| # Application | File, print, message, application, database |
| --- | --- |
| Presentation | Encrypt, decyrpt, compress, expand, format |
| Session | Dialog Control, coordinating communication |
| Transport | End to end connection, transparent data transfer, flow control, acknowledging, window, session establishment, tear down of virtual circuits, |
| Network | Routing |
| Datalink | Framing |
| Physical | Bit synchronization |

10 Base 5 is 100mt

CSMA/CD 802.3

Full duplex requires: FD NIC, Loopback and collision detection disabled

100 Base T spec 802.3u

Spanning Tree spec 802.1d

| Catalyst 5000 | Type slot/port |
| --- | --- |
| Catalyst 7000 | Type slot/port adapter/port |

| Directly Connected | C | 0 |  |
| --- | --- | --- | --- |
| Static Route | S | 1 |  |
| BGP External | B, EX | 20 |  |
| EIGRP Internal | D | 90 | not classful |
| IGRP | I | 100 | classful |
| OSPF | O | 110 | not classful |
| IS-IS | i | 115 |  |
| RIP | R | 120 | classful |
| EGP | E | 140 |  |
| NLSP | N/a | ? | not classful |
| Unknown | ? | 255 |  |

Traceable protocols are: Apple Talk, CLNS, IP, Vines, Old Vines

IPX encapsulations:

| # Ethernet | 802.3 | Novel-ether (default) |
| --- | --- | --- |
|  | 802.2 | Sap |
|  | Ethernet_II | Arpa |
|  | Ethernet_snap | Snap |
| Token Ring | TokenRing | Sap (default) |
|  | Token-Ring_snap | Snap |
| FDDI | Fddi_snap | Snap (default) |
|  | Fddi_802.2 | Sap |
|  | Fddi_raw | Novell_fddi |

| # Standart IP access-list | 1-99 | # Standart IPX access-list | 800-899 |
| --- | --- | --- | --- |
| Extended IP access-list | 100-199 | Extended IPX access-list | 900-999 |
|  |  | SAP IPX access-list | 1000-1099 |

| Frame Relay encapsulation types | 1) Cisco (default) 2) ietf |
| --- | --- |
| Frame Relay LMI types (LMI is autodetected after IOS 11.2) | 1) Cisco (default) 2) ANSI 3) Q.933a |

ISDN Standards:

E deals with telephone network standards

I deals with concepts and terminology,

Q deals with switching.

TE1: Native ISDN interface

TE2: Non- Native ISDN interface

NT1: ISDN modem

LT: Local Termination

TA: Terminal Adapter

ET: Exchange Termination

v: ISDN cloud

NT2

|

t

|

TE1-------------s-------NT1-----------u-------LT-----------v------------ET

|

|

TE2---r-----TA--- |

ISDN device may raise its priority level when all the other devices in the D channel have had a chance to send a D message.

PRI circuit types: E1 uses CRC4 or no CRC4, T1 uses SF or ESF

Dialer-list is used to filter what traffic is interesting

You can have a maximum of 6 name servers.

You should avoid using more than 2 VLSMs on any network (according to Cisco)

How to setup an ISDN/Pri Interface on cisco 7500

Isdn switch-type switch-type

Controller t1 slot/port [or] Controller t1 number

Framing esf

Linecode b8zs

Pri-group [timeslots/range]

Enhanced Editing features: CTRL-A returns to the beginning of a line,  CTRL-B is a backspace, CTRL H, is a destructive backspace,  CTRL-P cycles through the command history,  CTRL-L reloads the previous line, CTRL-K clears the line, ESC-B moves back one word at a time. Enhanced Editing is enabled by default.