Concepts

IP Quality of Service

Queuing

Bridge Group Virtual Interfaces

Bridge Group Virtual Interfaces

interface bvi

Use the interface bvi interface configuration command to create the bridge-group virtual interface (BVI) that represents the specified bridge group to the routed world and links the corresponding bridge group to the other routed interfaces. Use the no form of this command to delete the BVI.

interface bvi bridge-group
no interface bvi bridge-group

Syntax Description

| bridge-group | Bridge-group number. It must be the same number specified in the bridge protocol command. |
| --- | --- |

Default

No BVI is created.

Command Mode

Interface configuration

Usage Guidelines

This command first appeared in Cisco IOS Release 11.2.

You must enable IRB before attempting to create a BVI.

When you intend to bridge and route a given protocol in the same bridge group, you must configure the network-layer attributes of the protocol on the BVI. Do not configure protocol attributes on the bridged interfaces. No bridging attributes can be configured on the BVI.

Example

The following example creates a bridge-group virtual interface and associates it with bridge group 1:

interface bvi 1

Related Command

bridge irb

Priority Output Queuing

Original Location: http://www.cisco.com/warp/public/614/1.html

Priority output queuing allows a network administrator to define four priorities of traffic -- high, normal, medium, and low -- on a given interface. As traffic comes into the router, it is assigned to one of the four output queues. Packets on the highest-priority queue are transmitted first. When that queue empties, traffic on the next highest-priority queue is transmitted, and so on. This mechanism assures that during congestion, the highest-priority data does not get delayed by lower-priority traffic. However, if the traffic sent to a given interface exceeds the bandwidth of that interface, lower-priority traffic can experience significant delays.

Custom Queuing

For networks that need to provide a guaranteed level of service for all traffic, Cisco offers custom queuing. Custom queuing allows a customer to reserve a percentage of bandwidth for specified protocols. Customers can define up to 10 output queues for normal data and an additional queue for system messages such as LAN keepalive messages (routing packets are not assigned to the system queue). Cisco routers service each queue sequentially, transmitting a configurable percentage of traffic on each queue before moving on to the next one. Custom Queuing guarantees that mission-critical data is always assigned a certain percentage of the bandwidth, but also assures predictable throughput for other traffic.

To provide this feature, Cisco routers determine how many bytes should be transmitted from each queue, based on the interface speed and the configured percentage. When the calculated byte count from a given queue has been transmitted, the router completes transmission of the current packet and moves on to the next queue, servicing each queue in a round-robin fashion.

Independent tests of Cisco routers have shown that when a line is saturated with traffic from multiple protocols, traffic carried on the line maintains allocated bandwidth percentages to within a few percentage points of what was specified in the configuration.

A key advantage of Cisco's "bandwidth reservation" technique is that unused bandwidth can be dynamically allocated to any protocol that requires it. For example, if SNA is allocated 50 percent of the bandwidth but uses only 30 percent, the next protocol in the queue can take up the extra 20 percent until SNA requires it. Additionally, Custom Queuing maintains the predictable throughput of dedicated lines by efficiently using packet-switching technologies such as Frame Relay.

Traffic Delineation

To make effective use of any prioritization scheme, network administrators must have flexibility in how they allocate traffic to a queue. For example, if an enterprise has both SNA and NetBIOS traffic on a Token Ring LAN, it may be imperative to prioritize SNA traffic (which is often mission-critical) over the NetBIOS traffic. In providing this granularity, Cisco allows prioritization queues to be specified by LAN service access point (SAP), the field in a LAN frame that specifies which protocol is being carried in the frame.

In addition, Cisco routers support prioritization of SDLC traffic, any routed protocol suite, and any TCP/IP port. TCP/IP port prioritization enables network administrators to prioritize Telnet traffic ahead of FTP.

In environments with traditional SNA, prioritizing at an even more granular level is possible. For example, it may be necessary to prioritize traffic for certain logical units (LUs) to assure that terminal traffic takes precedence over printer traffic. Cisco supports the capability to prioritize by LU address for a given physical unit (PU) when transmitting either SDLC or Token Ring SNA traffic. Additionally, enterprises may need to prioritize selected SDLC lines over other lines to assure, for example, that customer service traffic takes precedence over administrative traffic.

Summary

Either Custom Queuing or Priority Output Queuing may be appropriate prioritization schemes, depending on network requirements.

- Priority Output Queuing is designed for environments that focus on mission-critical data, excluding or delaying less critical traffic during periods of congestion.

- Custom Queuing is designed for environments that need to guarantee a minimal level of service to all protocols.

With either type of queuing, Cisco has the ability to prioritize at a granular level when determining which traffic belongs in each queue.

IP Quality of Service

(Original Location: http://www.cisco.com/warp/public/728/3800/ipqs_ds.htm)

Rapid growth in intranet and extranet deployment has resulted in massive increases in demand for network bandwidth, performance, and flexibility to support both existing and emerging applications and services.

Introduction

In response to these demands, new quality-of-service (QoS) features have been developed to augment the extensive set of existing QoS capabilities of the Cisco IOSTM software.

The new features will enable enterprises to deploy and operate large-scale IP networks that efficiently handle mission-critical and rapidly growing voice over IP traffic as well as bandwidth-hungry Web/multimedia applications. Enterprise network managers can also leverage complementary QoS features being deployed by network service providers to build end-to-end networking capabilities that meet their business policies and requirements.

Scaling QoS Performance

As enterprise IP WAN networks scale to DS3/E3 speeds and beyond, Cisco is delivering new, high-performance QoS capabilities. The keys to scalable QoS performance include:

- A network architecture that distributes functionality between cooperating edge/aggregation and core/backbone nodes

- Fully distributed switching and services on the router

High-touch policies such as packet classification and granular Layer 3 measurements are applied at the edge of the network. The network backbone is then responsible for high-speed switching and transport and policy enforcement via the congestion control and queueing techniques discussed in this document.

These new QoS capabilities include committed access rate (CAR) and distributed versions of Weighted Random Early Detection (WRED) and Weighted Fair Queueing (WFQ). These capabilities are implemented as fully distributed services on the Cisco 7500 series of routers. Versatile Interface Processors (VIPs) serve as intelligent line cards with local execution of both switching decisions and services application, and they provide performance up to 155 Mbps per interface.

Packet Classification

The new packet classification feature CAR enables network managers to specify policies that partition network traffic into multiple priority levels or classes of services (COSs). The network manager can define up to six classes using the three precedence bits in the type-of-service (TOS) field in the IP packet header. When classified, the network ensures that packets within a class receive appropriate service levels in terms of criteria such as bandwidth allocation, delay bounds, and packet loss.

Packets are classified by CAR based on criteria including physical port, IP address, application port, protocol type, or other criteria specified by either access control lists (ACLs) or extended access lists (EACLs).

Bandwidth Management

CAR also provides bandwidth management functionality. The network manager can use CAR to designate traffic handling policies when traffic either conforms to or exceeds specified rate limits. CAR rate-limit policies can be based on physical port, Media Access Control (MAC) address, IP address, application port, or other criteria specified by ACLs or EACLs. CAR rate limits can be applied to both input and output traffic and Asynchronous Transfer Mode (ATM) and Frame Relay subinterfaces. CAR can also be utilized to specify more complex bandwidth management policies via cascaded rate limits, thus providing network managers very fine-grained network resource control.

Congestion Avoidance

The Random Early Detection (RED) and the new WRED features provide network managers with powerful congestion avoidance capabilities. RED works cooperatively with TCP traffic sources to maximize network throughput and capacity utilization while minimizing packet loss and delay. WRED provides preferential treatment for premium traffic classes under congestion situations by allowing network managers to specify a different RED policy per traffic class.

Congestion Management

The new distributed WFQ feature segregates packet traffic into either flows or classes and then schedules packet output to meet specified bandwidth allocation or delay bounds. WFQ classes may be assigned either by IP precedence, application ports, IP protocol, or incoming interface.

Granular Layer 3 Measurements

In addition to these new capabilities is NetFlow on the Cisco 7500, which provides a highly scalable measurement vehicle for billing/accounting, planning, and monitoring of QoS-enabled IP networks. NetFlow generates granular Layer 3 measurements on a per-flow basis, including: IP addresses, application ports, packet and byte counts, flow start and end time stamps, and packet classification.

Existing Cisco IOS QoS Capabilities

Complementing these new capabilities are existing Cisco IOS software features that provide QoS capabilities across all Cisco router platforms, including low-end and midrange routers. These features include policy routing, Resource Reservation Protocol (RSVP), WRED, WFQ, custom queueing, priority queueing, generic traffic shaping and Frame Relay traffic shaping, plus link fragmentation/interleaving, and RTP header compression for improving multiservice performance and efficiency on slow links.

Policy Based Routing

Policy routing provides the network manager with the means to define customized routing paths for selected packets based on criteria (such as source address and application port) not normally considered by destination-based routing protocols. Thus, particular traffic types such as voice traffic may be sent over special routes that minimize hop counts and other delay characteristics to ensure high-quality service characteristics. Also, policy routing can be used on low-end and midrange routers to classify packets and mark the packet via the IP precedence field, enabling backbone routers to give priority treatment to voice packets when congestion occurs.

Resource Reservation Protocol

RSVP enables applications with real-time traffic needs (for example, voice and multimedia) to dynamically request and reserve network resources necessary to meet their specific QoS requirements. Through proxy RSVP capabilities, Cisco routers utilize RSVP to request resources on behalf of applications that are not yet RSVP-enabled.

WRED, WFQ, Custom Queueing, Priority Queueing

Like their VIP-distributed versions on the Cisco 7500, WRED and WFQ provide techniques to control congestion and to allocate resources to packet flows in a network. Custom and priority queueing provide additional means to schedule packets on a congested link.

Link Fragmentation and Interleaving

Voice over IP traffic is susceptible to increased latency and jitter on slow speed links when large packets are queued. Link fragmentation and interleaving reduces delay and jitter on slower speed links by breaking up large datagrams and interleaving low delay traffic with the resulting smaller packets.

A TrCRF is a logical grouping of ports.Within the TrCRF, source-route switching is used for forwarding based on either MAC addresses or route descriptors. Frames can be switched between ports within a single TrCRF.

A TrCRF is specified on a router with a ring number and a unique VLAN ID number for the local ring on the router's interface. On switches, TrCRFs comprise one or more physical Token Ring ports. A TrCRF does not contain any physical ports on a router. A TrCRF is confined to the router's physical chassis and is not distributed across a TRISL link. However, if only one TrCRF is defined for a TrNET VLAN, it may be distributed.

A TrBRF on the router can have only one TrCRF. If routing is performed with RIF, the multiring trcrf-vlan command is used to specify the TrCRF for the local ring. If there is a need to cache RIFs for routed traffic bridged across a subinterface, the multiring command is used.

A TrBRF is a logical grouping of TrCRFs. The TrBRF is used to join different TrCRFs existing in a network of routers and switches that are connected via high-speed links on a TRISL backbone.

A TrBRF is specified with a bridge number and VLAN ID number. The bridge number is used to identify the bridge which interconnects all logical rings (TrCRFs) that have the same parent TrBRF.

A TrBRF can function as an SRB or SRT bridge running either the IBM or IEEE Spanning-Tree Protocol. If SRB is used, duplicate MAC addresses can be defined on different logical rings.

A standard Token Ring ISL packet contains a frame that is encapsulated with a header that transports VLAN IDs between routers and switches. A Token Ring ISL packet must have a VLAN ID for each logical ring (TrCRF) and a VLAN ID for the entire Token Ring bridged network (TrBRF).

A packet can originate from one TrCRF and have a destination to a different TrCRF that is contained within the same TrBRF. This second level of VLAN IDs requires the ISL header to contain a destination VLAN ID and a source VLAN ID. The destination VLAN ID is used for forwarding and the source VLAN ID is used for learning. The destination VLAN ID can be a TrBRF or a TrCRF.

The Token Ring ISL frame encapsulation is 35 bytes.

The Cisco Duplicate Ring Protocol (DRiP) runs on Cisco routers and switches that support switched VLAN networking and is used to identify active Token Ring VLANs (TrCRFs).

Through packet advertisements, DRiP maintains the status of TrCRFs. It then uses this information to determine whether there are multiple TrCRFs active in a TrBRF. A single DRiP database exists in a Cisco router.

DRiP information is used for the following:

- All-routes explorer (ARE) filtering

To enable the switch to filter out excessive ARE frames, the switch must be aware of the TrCRFs that are attached to the distributed bridge (TrBRF). The DRiP information is used in conjunction with the local configuration to determine which of the TrCRFs configured within a TrBRF have active ports. This information is used on the base switch to correctly filter AREs and on the ISL module to discard AREs that have already been on an attached ring.

- Detecting the configuration of duplicate TrCRFs across routers and switches, which would cause a TrCRF to be distributed across ISL trunks

The DRiP information is used in conjunction with the local configuration information to determine which TrCRFs are already active on the switches. If a TrCRF is enabled on more than one switch, the ports associated with the TrCRF are disabled on all switches. A router will not disable the port, instead it generates the following error message indicating that two identical TrCRFs are existing:

DRIP conflict with CRF <vlan-id>

Note As VLAN IDs are unique throughout the Token Ring VLAN (TrNetVLAN) network, DRiP does not need to understand the parent/child relationship of a TrCRF to a TrBRF.

How DRiP Works

DRiP sends advertisements to a multicast address so the advertisements are received by all neighboring devices (but they are not forwarded by normal bridging procedures). The advertisement includes VLAN information for the source device only. The DRiP database in the router is initialized when TRISL encapsulation is configured, at least one TrBRF is defined, and the interface is configured for SRB or for routing with RIF.

On any subinterface configured with TRISL encapsulation, there can be two different CRF VLANs. If SRB is enabled in the router with TRISL, a VLAN ID is specified for the CRF of the virtual ring. If routing is enabled on the router, a VLAN ID is specified for the CRF of the pseudo-ring.

When a switch receives a DRiP advertisement from a router, it compares the information in the advertisement with its local configuration to determine which TrCRFs have active ports and then denies any configuration that would allow a TrCRF that is already active on another box to be configured on the local switch.

If there is a conflict between two identical TrCRFs, all ports attached to the conflicting TrCRFs are shut down in the switches and the router's ports remain active. If a TrCRF is configured on two devices that share the same TrBRF for bridging between them, and if ports are assigned to these TrCRFs simultaneously, each switch or router will send a DRiP advertisement. The switch with the higher MAC address will disable the port and the router displays an error message.

If a trunk connection is lost and the router does not get DRiP advertisements on that trunk port for two minutes, all entries associated with that trunk port are removed.

DRiP Advertisements

A DRiP advertisement is sent at periodic intervals (30 seconds) by the router. Even if no change in status or configuration has taken place, the configuration revision number is updated. The periodic message will indicate that nothing has changed and the new revision number is provided.

A router also generates a DRiP advertisement when one of the following occurs:

- TRISL configuration is deleted from a router interface. The router updates its configuration revision number and the DRiP advertisement is sent on all ISL trunks.

- The periodic timer for 30 seconds expires. The DRiP advertisement is sent on all TRISL trunks.

- The router receives a DRiP advertisement with a revision number less than its own and the advertisement contains conflicting information about a TrCRF that is in use on that trunk port. The router does not update its revision number; it generates its own advertisement, sends it on the ISL trunk from which the original DRiP advertisement was received, and updates its database.

How DRiP Advertisements are Transmitted

Advertisements are transmitted on the fast Ethernet port default VLAN (VLAN1) which corresponds to the type of trunk link. Thus, only one copy is transmitted on a trunk port, no matter how many VLANs are defined.

DRiP Frame Format

DRiP is assigned the Cisco HDLC protocol type value 0x0102. A Cisco-proprietary SNAP value is used.

The SNAP format is as follows:

- LLC--0xAAAA03

- Org ID--0x00000C

- HDLC protocol type--0x0102

Benefits

Following are the benefits of TRISL:

- Expands the capability of Cisco routers to support routing between Token Ring VLANs.

- With TRISL support, no change is required to existing Token Ring end stations, applications, protocol stacks, or source-route bridges to take advantage of a 100 Mbps backbone. Clients and servers continue to use Token Ring and SRB protocols without change, minimizing implementation cost and protecting investments in installed equipment.

- TRISL provides a high-speed link between routers and switches or between switches. TRISL forwards the data across the high-speed link without breaking the frames. The entire frame is sent intact across the TRISL connection.

- TRISL is less expensive than ATM and avoids the need for LAN emulation (LANE) services. It is primarily intended for network managers who do not want an ATM backbone for their campus.

- Broadcasts are controlled by groups of client and server end stations across TRISL by creating multiple broadcast domains via multiple VLANs. Catalyst 5000 switching modules and the Catalyst 3900 stackable Token Ring switches support media-speed filtering by protocol on their Token Ring switch ports. As a result, it is possible to completely direct broadcast traffic between clients and servers using broadcast control by VLAN across a switched campus and using protocol filtering on each Token Ring port within the VLAN.

Other crap that is interesting but won’t be on the test:

I saw the word netflow a lot in the material so I thought I would look it up:

NetFlow technology efficiently provides the metering base for a key set of applications including network traffic accounting, usage-based network billing, network planning, network monitoring, outbound marketing, and data mining capabilities for both service provider and enterprise customers. Cisco provides a set of NetFlow applications to collect NetFlow export data, perform data volume reduction, post-processing, and provide to end-user applications easy access to NetFlow data. Cisco is currently working with a number of partners to provide customers with comprehensive solutions for NetFlow-based billing, planning and monitoring. NetFlow also provides the measurement base for Cisco's new Internet Quality of Service (QoS) initiatives. NetFlow captures the traffic classification or precedence associated with each flow, enabling differentiated charging based on Quality of Service.