Cisco Internetwork Troubleshooting

69 questions 60 minutes

1. When troubleshooting a problem connecting a modem to a router which of the following is a useful diagnostic step?

Use the show line command to verify adequate buffering

Issue the show line command to check modem state

Show serial s0 queue to monitor traffic on the port

Upgrade to the latest modem drivers

2. The follow lines are part of the output from a show interfaces command:

Ethernet 0 is up, Line protocol is up

Hardware is MCI Ethernet aa00.00400.0134 BIA 0000.0c00.4365

Ethernet address 131.0.8.11 subnet 255.255.255.0

Which hardware address will this station respond to

Aa00.00400

0000.0c00.4365

131.0.8.11

255.255.255.0

3. What does the SNAP mean in the following packet

DLC Header

Frame I arrived 15:05:33:3:8:9

Frame size is 62	0x3e8 hex

AC:Frame Priority 0

Reservation Priority 0

Monitor Count: 0

FC: LLC Frame PFC

Attention Code = none

FS addr recognized indicator –0

Frame copied indicator 00

Destination = station cisco a05903

Source = station ibm 0a8591

Llc header

dsap ==aa

Source sap = aa

Command unnumbered frame relay

Dsap header type 0x800

- subnetwork access protocol

- super non application process

- serial network application

- second node appearance

Capturing appletalk packets with your sniffer with your    encounter the following counter: 279 frames accepted

What is the meaning of this field

- the analyzer has transmitted 279 well formed packets

- the analyzer has seen 279 well formed packets

- 279 packets have met the routers access list criteria

- the router has received 279 packets that met the capture filter criteria

- the analyzer has seen 279 packets with its address as the destination

Which problem will cause an increasing number of transitions occurred when you do a show interface token ring

- Broadcast storms

- The ring is repeatedly going down and coming back up

- Malformed packet arriving on the token ring interface

- Excessive ring poll processes on the network

Which tool is best suited for testing signals across a PC / Modem interface

- Network monitor

- Breakout box

- Time domain reflectometer

- Null modem cable

After capturing a variety of IPX traffic from your network how can you view only the packets using the 802.3 frame type.

1. Set a router access list that prevent other frame types on the network

- switch the analyzers connection to thin ethernet

- configure a display filter

- set the interface card in the analyzer to accept on the ethernet II frame type

What tool is useful for certification of your Lan infrastructure

- a bert/wart

- cable tester

- packet driver

- network management system

What command monitors RTMP route aging    acquisition advertisements

- display router errors

- monitors route aging    advertisements and acquisition

- starts a log of all appletalk traffic

- starts a log of NBP traffic

You have noticed a few of your IPX clients on a serverless segment are having a problem connecting to their router since you’ve updated your router from an older IOS version to the newest version. What action can you take to resolve this problem

- enable old style novell broadcasts

- issue the ipx gns-round-robin

- increase the ipx-gns-reponse-delay time on the router

- enable gns on the cisco router

- turn off gns on the novell server

What command displays the entries in the routing table

Show ip route

Show ip protocol

Show ip arp

Ping

What can be derived from the following section of a    packet:

RTMP packet dump.

Choose 1

This is a connection oriented protocol

This is a    routing information packet

The transport layer is udp

This is a decnet protocl

This protocol uses IP protocol as a transport

The frametype is 802.3 llc

While capturing ip traffic with your network sniffer

You get the message 2279 frames accept what does this mean?

- the analyzer has seen 2279 well formed packets

- the analyzer has 2279 well formed packets that met the capture criteria

- the analyzer can display 2279 well formed packets that meet the capture filter criteria

- the analyzer has captured 2279 well formed packets

Examine the graphic by clicking on the exhibit:

It’s a show processes on a cisco catalyst

Relevant info:

CPU Utilization 5 sec: 100%    1 min: 84% 5 min: 74%

Based on the output of the show processes command what problem is currently affecting the router

- Several ospf and iso protocols have stopped running

- The tcp timer has expired

- Excessive high network bandwidth utilization

- Excessive high cpu utilization

What command can you use to obtain a routers firmware version?

Show config

Show version

Show controller

What is an advantage of a connectionless protocol?

- high reliability

- preferrable and lengthy exchanges of data

- less network traffic

- fast error recovery and flow control

what is a common term cisco systems uses when referring to the frame type novell describers as ethernet 802.3

choose (1)

- e2

- arpa

- ethernet_ii

- snap

- ethernet_snap

- etherner_802.2

- novell-ether

- sap

- ISO 1

What problem is likely to result in a users inability to see zones or services outside their own network?

- clients are not configured with a default gateway

- phase 1 / phase ii incompability

- incorrect appletalk encapsulation

- too many zones configured for a single network

results of a show controllers token command on one of your routers includes the information

“bridge capab1e”

show controllers token is the command

the ring bridge has passed the ring bridge self test

source bridge routing is enabled

the interface is bridge capable but not configured for bridging

the router has source route bridging buffers available

What command can you use to obtain a routers configuration for your tech support engineer

Show firmware

Show controllers

config

Show running-config

What does IPX internal network command do?

Set the ipx    interface as non seed

Adds an additional network address to an interface

Assigns the address for IPX when it is tunneled over IP network

Assigns the address that is advertised by NLSP and IPX wan on all router interfaces

show interfaces fddi 3/0 displays an upstream neighbor value of 0000 0 f8 0000 what is the signifcance of this address

- it is the software address of the upstream neighbor

- it is the hardware address of the upstream neighbor

- the identity of the upstream neighbor is unknown

- the address upstream neighbor is illegal

What which step of the troubleshooting process contains identifying a set of symptoms and associated causes?

- logging the trouble ticket

- verifying the information

- defining the problem

- isolating the problem to the device level

Which services are not available through Cisco online? (Choose two)

- Bug Navigator Query program

- Cisco Documentation

- Ordering information

- Online distribution of software

- Cisco q&a database

- File drop facilities

When trying to isolate and resolve a problem on a token ring network you issue a “show controllers token” one part of the result is listed below

Tx-beacon 150/0

After making changes you believe you solved the problem you issue the show controllers token command again and get the following result:

Tx-becaon 20/150

Trying the command later gives you following results

Tx-becoan 0/170

What was the result of your attempts to correct the problem?

- beaconing has increased

- beaconing has decreased

- beaconing has stopped

- no change

which tool can be used to verify cable length?

- ethernet analyzer

- tdr

- snmp agent

- volt/ohm meter

Which field in the partial display of the packet listed below indicates available space for incoming data below:

- tcp header

- tcp

- tcp: source port = 1339

- tcp: destination port = 23

- tcp: initial sequence number = 0

- tcp: data offset = 20 bytes

- tcp: flags = 20

- tcp: 0 = no urgent point

- tcp: 0 = no acknowledge

- tcp: 0 = no push

- tcp: 0 = no reset

- tcp: 1 sent

- tcp: 0 = no fin

- tcp: tcp window = 512

- tcp: checksum = e43e correct

- tcp: no options

what is the source of a common novell network error referred to as “Configuration mistmatch”

- servers or routers have assigned different network addresses to a common network

- routers do not contain support for proprietary novell frame types

- inconsistent frame types used by the clients and servers

- netware servers on the same network using different frame types

what command reports the discover of new zones?

Debug apple errors

Debug apple routing

Show apple interfaces

Debug apple zip

when diagnosing a problem the output of the show interfaces serial command includes the following line:

serial s0 is administratively down, line protocol is down

what does this mean?

- the serial line is in an idle state

- the remote router is down

- the interface is in an idle dial on demand link

- the interface has been taken down by an administrator

Which cisco product provides an RMON console    to analyze traffic and enable proactive management of switched internetworks

- Cisco works

- Rmon agent

- Traffic director

- Cisco net manager

- Cisco rmon manager

Regarding the following frame which statements are true? (choose 2)

Ipx: ipx header

Ipx: no align

Ipx: checksum = ffff

Ipx: length = 224

Ipx: transport control = 00

Ipx: 0000 = reservered ipx

Ipx: ….. 0000 = hop count

Ipx : dest network.node = 1000.fffffffffffff socket = 1106    (sap)

Ipx: src network.node = 100.02.60 c2 fc 79 socket = 1106 (sap)

This is a broadcast packet

This frame uses a snap format

This is uses a novell ether frame

This a portion of a network layer protocol

The ipx portion is a connection oriented protocol

An IP ping command displays a series of five exclamation points in the output. What is the significance of the exclamation points.

- Network unreachable

- a level five error was returned

- indicates a wait time before a reply was received

- five replies were received

- five receive buffers available.

Use the sample sniffer    internetwork analyzer sample decode, select the attributes that characterize this communciation (choose 3)

I believe it was a dump from a breakout box, it had the following columns:

Summary	delta	pdst/source	Dce	Dte	Hdlc	sabm		p/f = 1

Select the attributes that characterize this communication

The data is using token ring framing

The stations are using hdlc framing (given because hdlc appears in the dump)

this is a communication between a mainframe and a terminal

this is a serial communication

Which of the following protocols are always connectionless (choose 3)

- IP

- Tcp

- Arp

- 802.3

- 802.2

what is the show buffers command used for?

- Showing trends that indicate the link is a bottleneck

- Baseline application performance

- Proof of interface funcationality

- Verifying that all protocols are operational

What counter in the show interfaces serial command output indicates a modem or line problem exists?

- input errors

- rs/los

- carrier transitions

- rx/los

- queue overruns

- keepalive set

what type of device is typically used to test physical connectivity?

- protocol analyizer

- digital multimeter

- breakout box

- packet driver

Which tool is most useful for analysis of redesign, reconfiguration, or stress testing?

- network analayizer

- network management system

- simulation and modeling tools

- network monitor

what type of protocol requires the application to request retransmission of lost, missing or corrupt packets?

- connectionless

- connection oriented

- host to host

- client/server

which step follows isolating a problem isolating a problem to a specific device (router)

- swapping out the router and observe the result

- run a diagnostic on all interfaces and observe the result

- isolate the likely problem source to a single variable

- upgrade all software to the current release levels?

Where should you first look for errors when troubleshooting a serial wan problem?

- interface operation with the show interface command

- the physical connections by checking the cable interface connections

- the protocol operation by viewing a packet trace of link traffic

- network host reachability with the ping utility?

Where are the output of the debug and system error messages sent by default?

- console

- output configuration requires a tftp server to put files

- output is written to a syslog server

- error logging automatically invokes debug output to a designated ftp server

What is the troubleshooting engine?

- the report generator for cisco debug commands

- the name of the cisco documentation cd

- a cisco roadshow highlighting internetwork troubleshooting solutions

- a customer engineered tool that uses case based reasoning to solve common problems

A show interfaces serial command indicates a high number of interface resets, what is the significance of the interface resets counter?

- the router restart several times

- key packets were not sent within several seconds time

- there have been several attempts to restart the interface with the protocol down but the interface up

- this counter reflects the number of times the reset button the back of the router    was pushed

Which command should you use on an appletalk network if you suspect problems and want to monitor neighbors becoming reachable or unreachable and interfaces coming up or down.

- show apple rtmp

- debug apple traffic

- protocol apple verbose

- debug apple events

When should you use the write core command?

- to obtain a core dump after a router has crashed?

- To write a backup of the configuration to the routers memory?

- To obtain a memory image of a malfunctioning router

- to write the os image to nvram

when troubleshooting a wan connection what command displays hardware type and version information?

Show interfaces serial

Show version

Show controller

Show running-config

The output of show buffers indicates an increasing number of trends under very big buffers what does this indicate?

- too many frames are being sliced because the receive buffer is too small

- outgoing frames are being truncated because the very big buffer    size is too small

- the number of very big output buffers is being reduced?

- Unused very big buffers are being reduced to smaller buffers

You have implementing an action plan to resolve a problem. What should your next action be if your previous action did not solve the problem?

- gather additional facts

- undue the changes that didn’t work

- repeat the process modifying additional variables till a solution is reached

- evaluate the problem definition for validty

what is the purpose of the debug arp command?
1. Determines if the router is sending and receiving arp requests?

2. displays the arp cache contents

3. send an arp request to all attached router neighbors

4. place a new list of ip hardware address pairs for all stations on attached segments into the routers cache.

How do you use the show processes command to see when processes are causing an excessive load on the router?

- issue the command at select time throughout the workday to note the variations in the values over time

- issue the command multiple times because the fast clock of the router does not provide an accurate snapshot of the router with a single invocation

- redirect output to a file since the high number of processes running on a router results in multiple screens of information

- issue the command several times on a busy router because high utilization spikes truncate the output of the command.

If certain ip data is being unintentionally being filtered by one or more router interfaces the likely cause of the problem is:

- ip rip filters

- ip access list

- buffer sizes is configured to small

- foreign protocol conflicts with IP

which type of troubleshooting tool is commonly used to display packet data?

- Digital multimeter

- Network monitor

- Fox box

- Protocol analyzer

55. which command restricts logging to the console only?

- no logging on

- log console

- logging monitor only

- logging monitor

if users can access some hosts on a local and remote host on some segment but not others what is the likely cause of the problem?

- router failures

- access list configuration

- addressing and subnet mask problems

- high utilization

- overloaded hosts

regarding the following show interfaces command output which of the following statements is true?

2200000 packets input, 200000000 bytes 0 no buffer

192000 broadcasts, 134 runts, 0 giants

3 input errors, 390 crc, 0 frame, 0 overrun, 0 ignore, 0 abort

0 input packets with dribble condition detected

23000230 packets output 30239203 bytes 0 no buffers

output packets: 436000 collisions

4 choices:

- the network attached is experiencing broadcast storms

- the network exceeds router capacity

- the network is experiencing excessive collisions

- malformed packets are causing router degradation

what command should you use to display debugging information about ip packets?

- debug ip packet

- show ip buffers

- show ip access

- debug ip traffic

which of the following    error logging methods produces the lowest overhead

- console

- remote terminal

- syslog server

which device commonly uses the rmon mib for    internetwork troubleshooting

protocol analzyer

network monitor

cable tester

packet driver

global statistics screen of a internetwork anaylzer says that there is a large amount of SNMP traffic. What is the signfiicance of SNMP?

The routers are exchanging large routing tables

The management overhead consititues a large portion of the link traffic

A lot of email is exchanged across the link

A remote device is informing the management console that there is a problem with the link.

What will be indicated in the show interfaces command output if more than 3 consecutive keepalives have been missed by the e0 interface?

- Ethernet 0 is    up, line protocol down

- Ethernet 0 is down, line protocol is up

- Ethernet 0 is down, line protocol is down

- Ethernet 0 is not responding

- Ethernet 0 is down

Which is the possible cause of a host being unable offnet hosts through a router

- no default gateway specified

- the local host and a remote station have the same address

- the router between different hosts uses the same frame type

- incompatible link type between the local host and remote router

which of the following problems is likely to cause certain tcp/ip protocols to be routed but not others?

- Routers with duplicate addresses

- Two separate networks with the same addresses

- Invalid subnet mask configuration

- Misconfigured access lists

what fact can be arrived from the following decoded packet
full dump includes: dlc header info

it’s a broadcast packet fffffffff

definitally an arp packet

You have to pick the correct address.

What tool is used to verify a optical fiber installation?

Protocol analyzer

Volt/ohm meter

Network monitor

Otdr

Digital multimeter

What is the purpose of the debug ip icmp command?

- verbose explanation of icmp ping results

- display if the router is sending or receiving    icmp messages

- troubleshoot problems with icmp protocol stack

- send icmp packets to all neighboring routers

The show Ipx traffic displays the bad hop count increasing and the routes disappear and reappear often. Which of the problems is likely a source of the symptoms?

- increased routing throughput

- better error recovery with redundant links

- routes unpredicatably appearing and disappearing

- non routable protocols could not reach remote destinations.

Which cisco product works with sun net manager and hp openview?

- cisco netmanager

- cisco works

- cisco rmon manager

- cisco traffic manager