- Which command will display the settings of the routers ARP cache learned through the Ethernet interface?

Show interface ethernet 0 arp

Show arp interface ethernet 0

Show arp

Show ip arp

- Which of the following commands can be run from user level?

Show aliases

Show arp

Show users

Show ppp

Show interface serial 0

Show version

Show running-config

- How would you set the 3rd port, on the supervisor module installed in slot 1 on a Cisco 5000 to high priority?

Set port level 1/3 high

Set port level 0/2 high

Set 1/3 level high

Set 0/2 level high

There is no such available setting in the Cisco Switching IOS

- How many classes of addresses are available for use on the internet?

1

2

3

4

5

- Identify the correct classifications of address classes: (2)

A – uses two octets to make up the netmask

B – uses three octets to make up the netmask

C – uses a netmask prefix of 26

D – used for broadcasting

E – used for testing

- type the command to instruct the router to load the IOS from Read Only Memory?

Load system rom

boot system rom

run rom

boot system default

- Identify the statement which would permit only smtp mail to access the host 209.76.25.1

- Access-list 101 deny ip any any

Access-list 101 permit tcp any 209.76.25.1 eq smtp

- Access-list 101 permit tcp any 209.76.25.1 eq smtp

Access-list 101 deny ip any 209.76.25.1

- Access-list 101 permit tcp 209.76.25.1 any eq smtp

Access-list 101 deny ip any any

- Access-list 101 permit ip 209.76.25.1 any  eq 25

Access-list 101 deny ip 209.76.25.1 any

- What is the correct netmask (in decimal dot notation) for the address 209.76.25.1/26

255.255.255.192

255.255.255.224

255.255.255.254

255.255.255.0

- What is the correct network address for the host 209.76.25.72/26

209.76.25.64

209.76.25.65

209.76.25.127

209.76.25.0

- What is the correct broadcast address for the host 172.16.64.128 using a netmask of 255.255.192.0

172.16.128.255

172.16.127.255

172.16.64.0

172.16.0.128

- Which routing protocols are classful?

BGP

OSPF

RIP

Static routing

- type the command to instruct the router to load the IOS from the file named "ios11.2.img" saved on flash?

Load boot  flash ios.11.2.img

boot system flash ios11.2.img

load flash ios11.2.img

run flash ios.11.2.img

- What which of the following is an invalid host address using a netmask of 255.255.255.192

10.1.1.1

10.1.1.66

10.1.1.127

10.1.1.130

- When designing a network you can typically use the

80/20 rule

70/30 rule

50/50 rule

each network is unique and does not have any rules

- Identify the true statements about the following access list (2):

Access-list Ip 101 deny ip 192.10.172.0 0.0.0.255 any eq 23

This access list is invalid

The netmask on the this access list is reversed

This access list filters telnet access

This access list prevents the host 192.10.172.0 from telneting

This access list denies any telnet traffic to 192.10.172.0

This access list prevents any telnet traffic from 192.10.172.0

- A ISDN BRI circuit can be described as a

3B

2B+1C

2B+1D

none of the above

- On an ISDN circuit the D channel speed is

16kbps

32kbps

56kbps (Europe) or 64kbps (US)

128kbps

- On Cisco catalyst 5000 how would you set the second port on the controller in the first slot to full duplex?

Set port duplex 0/1 full

Set port duplex 1/1 full

Set port duplex 0/2 full

Set port duplex 1/2 full

- Identify the true statements (2)

Store and forward switching creates variable latency through the switch

Cut through switching creates variables latency through the switch

Store and forward switching works at wire speed

Cut through switching works at wire speed

- Choose the best answers: Which of the following is the session layer concerned with? (2)

Managing exchanges of data between presentation layer entities

Flow Control and windowing

Establishing, managing and  and terminating sessions

Verifying data encoding formats

- Which ISDN specification series deals with the concepts and terminology surrounding ISDN? (1)

I Series

Q Series

E Series

There is no specific series

- Identify which access list valid? (1)

Access-list 101 deny any any 10.1.1.1 255.255.0.0 eq 23

Access-list 101 permit ip host 1.2.3.4 10.1.1.1 255.255.0.0

Access-list 99 permit icmp any any

Access-list 99 permit tcp any 10.1.1.1 255.255.0.0 eq telnet

- Which of the following options makes sure a routing packet never leaves the same interface it was learned from?

Poison reverse

Split horizon

Spanning tree

Hold downs

- Which command shows all serial interfaces with IPX configured?

Show ipx interface serial

Show serial ipx

Show serial interfaces with ipx

Show int serial ipx

- Which command shows what static IP to name entries have been created?

Show hostnames

Show hosts

Show ip hosts

Show host mapping

- What does the command “CDP Timer 150” do?

Gives CDP a administrative metric of 150

Configures the rate at which CDP packets are sent

Changes the CDP hold down timer

Sets the CDP hold down timer to 150

- Which command would set the encrypted secret password to CISCO?

Enable secret cisco encrypt

Enable secret 5 cisco

Enable secret cisco

Enable-secret cisco

Enable password cisco

- How can you get to user exec mode from the console?

Type “enable”

Type “config terminal”

Press the enter key on the keyboard

Press CTRL-Z key on the keyboard

- How do you get more flash memory in a 2500 series router? (2)

You can’t its on the system board

Call your local cisco sales representative

Move all permanent images to the system rom

Delete extra unused images from the router

You should order more RAM, some will automatically be converted to flash

Flash memory is not included on a 2500 series router

- Which of the following are valid IPX addresses? (1)

4a.0000.0c00.23fe

4a00000.0c0023fe

4a000000c0023fe

4a.00.00.00.c0.23.fe

- Which of the following protocols are connection oriented?

ICMP

RARP

IP

RIP

BGP

- What is the purpose of ARP?

Resolves IP addresses to host names

Resolves MAC addresses to IP addresses

Resolves IP addresses to mac address

Resolves MAC addresses to hostnames

- The command to turn off enhanced editing :

DISABLE EDIT

TERMINAL NO EDITING

NO EDIT

NO TERMINAL EDIT

- Choose the correct definitions matching the Transport Level: (Choose 3)

IP - Route determination

FTP - Transfers of Files

TCP - Provides Flow Control and Error Checking

SMTP - Provides Mail Exchange and

UDP - Provides Connectionless datagrams service

TCP - Provides Connection Oriented Services

- And the was one for network level: (Choose Two)

IP - Route Determination

RCP - End to End Communication

ICMP - Provides Network Testing and Verification

TCP - Provides Flow Control and Error Checking

NFS - Provides Connectivity via the Internetwork.

- What three occurrences will reset the holddown timer after a triggered update?

HD Timer expires

Infinity is finally defined as some max number

Another update is received indicating net status changed

Another update is received indicating a better metric

The router receives a processing task proportional to the number of links in the internetwork

The router detects fault LSP’s propagating through the internetwork

- What command displays IPX routing update packets received or transmitted between a router

Sh rip

Disp rip

Sh ipx route

Sh table ipx

Disp ipx route

Disp ipx routing activity

Debug ipx routing activity

Disp ipx interface activity

- 3 statements describe default encapsulation and LMI type configuration.

IETF encap must be configured unless the connecting routers are both cisco

The LMI type config term options

The default LMI is Cisco

In release 11.3 the LMI type is autosensed

There are only four encapsulations and 3 LMI type options

- Which 3 commands will display the IPX address on int e0

Sh ipx e0

Sh ipx int

Disp ipx int

Sh int e0

Sh ipx int e

Sh ipx int ethernet0

Sh ipx address

Sh ipx address ethernet

Sh int e0 ipx addr

Disp ipx address int e0

- Which is true regarding half duplex Ethernet operation?

Half Duplex transmission between stations is achieved using Point to Point Ethernet & Fast Ethernet

With Half Duplex transmission logically circuits feed into a single cable creating a situation similar to a one way bridge

Half duplex transmission between stations is achieved by using point to multipoint Ethernet and Fast Ethernet

Half Duplex ethernet technology provides a transmit circuit connection wired directly to the receiver circuit at the other end

- What statement is true about the Message of the Day (MOTD)

MOTD banners are displayed at login

You must start a motd banner with a delimiting character of your choice

The banner motd command is used to add a description to a specific interface

The interface description command must end with a delimiting character

- Which is true regarding presentation layer?

Segments & Reassembles data

Provides error detection, correction, and flow control

Determines the best way to move data from one place to another

Provides system independent process or program services to end users

Provides data transfer, control, management between cooperating application processes over session connection

Governs electrical signals

Provides a common representation of application information while information is in transit between two cooperating Operating systems.

- Session layer standards (you must choose 3)

Ascii & EBCDIC

NFS & SQL

PICT & JPEG
	RPC & Netbios

MIDI & Mpeg

- Which three functions are characteristic of the data link LLC layer.

Provides logical address

Provides hw addr

Capable of flow control and sequencing

Enables media independence for upper layers

Provides saps that mac sublayer functions use

- Which three Novell IPX encapsulations names are matched? Choose 2 (there is a bug because you must choose three)

Ethernet II – ARPA
	Ethernet 802.3 – SAP

Ethernet 802.2 – LLC
	Ethernet_snap – SNAP

Token_ring_SNAP – SNAP

- Which two statements accurately describe the log option for IPX extended access lists (2)

Records type of sap service forwarded

records each time a sap is recorded in the SAP table

Records the socket name or # to which a packet is sent

Records the socket name or # from which a packet is sent

Records the # of times a packet is matched to an ACL statement

Records every time a SAP service matching the parameter is forwarded

Records the ACL control list violation when packet matches

- Describe End to End network services

Flow Control & Data Integrity

Best Route selection

Accomplished Segment by Segment, each segment is autonomous

Best efforts packet delivery

- What output would the following commands give the output:

ROUTER#CL

Ambiguous Command “CL”

CL -?

Help CL

CL?

? CL
CL ?

- Which command would display the configured DLCIs

Disp dlci

Sh prot frame relay S1

Sh frame-relay s1

Sh s1 encapsulation frame relay

Sh frame-relay pvc

- What information can you get from CDP info

Hostname

Software platform

Hardware platform

One address per protocol

Two addresses per protocol

Incoming/outgoing port

As many addresses as are configured for each protocol

The same info as show version

- True/False the correct syntax to setup sub interfaces for series 7000 or 7500 is:

Interface e0 1/0/0  slot/port adapter/port number

True

- Which types of serial communications are characterized by PVC and layer 2 identifiers

Frame relay

X.25

PPP

SLIP

I chose frame relay since it’s the only WAN protocol I could find that was related to PVC, there were 1 or 2 other choices.

- What 2 commands verify end to end communications

Telnet

Ping

Trace

I chose ping and trace there were other choices but as I remember they were off the wall.

- What command to disable edit mode

End edit

Disable edit

Disable 0

Edit mode is not enabled by default

- What is shown with the IP route command? Choose 3

Ip route 154.4.55.0 255.255.255.0 195.23.55.1 85

154.4.55.0 is source address

154.4.55.0 is the source network

85 is administrative distance

154.4.55.0 is the destination network

195.23.55.0 is source network

85 is the autonomous system number

- What is the syntax for the enable banner command

Router (config)# banner motd .

Router (config)# motd

Router (config)# set motd .

- What 3 sources can you enter configuration commands

TFTP server

Console

Vterminal

Nvram

HP Openview

- What is the purpose and default value of the CDP timer command

60 seconds; interval between updates

90 seconds; interval between updates

60 seconds; interval before an entry expires

90 seconds; interval before an entry expires

- What key stokes will bring you back to the beginning of the command line

Ctrl A

Ctrl B

Ctrl P

Ctrl N

- Which of the follow do not belong to the customer (2 answers):

CPE

Demarc

CO

DTE

DCE

Router

- Which command would you use to find the parameters for the clock?

cl?

cl ?

show cl?

show cl ?

- What are the characteristics of Store and forward switching

Reads entire frame and checks CRC before forwarding

Latency is increased

Has no effect on latency

Reads header the immediately forward packet

Packet may be applied to filters

I chose the first 2 choices

- Describe Connection oriented sessions

Is typically implemented on x.25 networks
Single path is determined, resources reserved, data is sent and received sequentially, link terminated

- What command displays interfaces with access-lists set

Show IP interface

Show ip access-lists

show ip access lists

show access lists

show access-lists

- What are the 2 functions of the data link Mac layer

Handles access to shared media

Provides SAPs for higher level protocols

Allows multiple devices to uniquely identify one another on the data link layer

Manages protocol access to the physical network medium

- What does the IPX maximum path command do

Sets the maximum metric that can appear in the routing table

Configures round robin load sharing over multiple equal metric paths (parallel paths)

Allows you to disable the TTL on an IPX packet

This parameter is only used in NLSP routing.

- What command displays IPX routing update packets received and transmitted between routers

Sh ipx routing all

Debug ipx routing

Debug ipx updates

Debug ipx routing activity

- What command has the proper syntax to specify a sub interface #4

Interface s0.4

Interface e4.0

Interface e0 sub int 1

Interface s0 sub 4

- What command sets up a secondary IP address of 192.15.1.7 with a class C netmask.

Ip address 192.15.1.7 255.255.255.0

Ip address 192.15.1.7 255.0.0.0 secondary

Ip secondary address 192.15.1.7 0.255.255.255

Ip address 192.15.1.7 255.255.255.0 secondary

Secondary ip address 192.15.1.7 255.255.255.0

- Deny telnet inbound with access list

Access-list 101 deny ip any eq 23

Access-list 101 deny tcp 192.15.72.5 255.255.255.0  eq 23

Access-list 101 permit ip any except telnet

Distribute-list 101 deny any eq telnet

- What Command loads an configuration file from tftp server

Copy tftp startup-config

Load tftp ram

Copy tftp running-config

Load tftp startup-config

- What command will display the characteristics of encapsulation and LMI type configuration options on serial interface 0

Dis s0

Sh int s0

Debug  frame-relay verbose

Sh dlci int s0

- What command Displays RIP routing updates send and received at router

Sh ip rip 10

Debug rip updates

Debug ip rip

Debug ip routing

This may be reverse. The question may have been what command do you use to Displays RIP routing updates send and received at router

- 3 ways to provide login access to router

TFTP

Console

Telnet

Rlogin

Auxiliary Port

RADIUS

- What command would you use to set the clock rate to 56k

- What is the appropriate line to define a ISDN switch type of basic-net3

- What is the command to set the encapsulation to ppp on a serial interface

- set the number of seconds to 60 before a call is disconnected

- map the ip address 1.1.1.1 to the phone number 5551212

- how many valid dialer groups are there?

- Specify a maximum usage of 100/255 before the dialer dials

- Construct the line which would enable the router to drop the active call in 10 seconds because another dialout is pending.

- Specify an idle disconnect time of 30 seconds

- What is the word used to describe traffic which will allow the dialer to dialout

- Construct an access list #101 which would disable any IGRP traffic

- Construct an access list #101 which would permit any host to ftp 1.1.1.1

- what command would show the basic status of an ISDN port 0

- what command would you use to show more detailed information about ISDN layers 1,2,3

- What command sets up a rotary dialer group #0

- What command includes an interface in a dialer rotary group

- You should assign a high ___ count for the route to the external network via a serial interface so that it will only be used if the primary link is down.

- What is the name of the unique identifier for a PVC

- The ____ DLCI field identifies a frame as an LMI frame instead of a regular Frame relay frame.

- Using the ______ ______ feature make a Frame relay network appear more like a LAN.

- _______ groups are designated by DLCI port numbers between 1019 and 1022, these special packets are sent to all nodes that belong to a specific group.

- ________ ___ must be used if the remote router does not support inverse ARP.

- A static map links a specified next hop address to a specific _____.

- The three types of  messages sent and received on the LMI are Status Messages, Status Updates, and Status enquiry messages. (True/False)

- The ____ model makes  all routes connected by VC’s peers on the same network, Frame Relay routers must copy all broadcasts and transmit on each VC.

- Routers attempting to implement NBMA may have trouble if the routing protocol does not have ____ ____ disabled, this is because the router will distribute changes back to the interface it received them from.

- In configuration the __________ follows the physical interface and is separated by a period. \

- Before you can configure sub interfaces you must have enabled frame relay encapsulation and a physical interface. (True/False)

- Correct Syntax: interface type number.subinterface  [multipoint | point to point ] (true/false)

- __________ sub-interfaces can be dynamically resolved using inverse arp, or can be statically mapped using the Frame-Relay map command.

- For ______ _ ______ sub interfaces using the frame-relay interface dlci command

- What are the three optional parameters when assigning a frame relay DLCI to a subinterface on a route

- Give the statement that would assign DLCI #5 to interface s0 using ietf encapsulation

- DLCI’s on both sides of the frame relay connection are always the same (true/false)

- Do the following steps:

- select serial 0 as the port to modify

- set the encapsulation to frame relay

- set the lmi type to cisco

- set sub interface 1 on serial 0 to point to point

- You are in subinterface configuration mode, now set the ip address for the subaddress to 1.1.1.1 with a netmask of 255.0.0.0

- specify DLCI 10 to the subinterface 1 on serial 0 and specify that broadcasts can be forwarded out through it.

- A minimum of __ and a maximum of ___ dlci's must be in place for prioritization to take place

- The router learns the network address of each router by sending and receiving _______ arp messages on each DLCI.

- Syntax to enable PVC switching on a frame relay DCE, DTE or Network to Network Interface (NNI)

- Specify a static DLCI route for a packet received on DLCI 10 to go out GRE tunnel 0  on its way to DLCI 15

- What are the three valid  frame-relay intf-type

- Syntax that will display a list of frame relay maps

- Name the different priority levels for frame relay

- One neighbor entry must be included for each _______ neighbor.

- The ______ bit tells the router that the frame relay packet encountered congestion.