The THC Hack/Phreak Archives: PSNINTRO.DOC (842 lines)

Note: I did not write any of these textfiles.  They are being posted from

the archive as a public service only - any copyrights belong to the

authors.  See the footer for important information.

==========================================================================

% X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X %

X**=======================================================================**X

%!!                     Phreakers/Hackers/Anarchists                      !!%

X!!                     -++--++--++--++--++--++--++-                      !!X

%!!                                                                       !!%

X!!           THE COMPLETE INTRODUCTORY GUIDE TO SPRINTNET AND            !!X

%!!                    SIMILAR PACKET SWITCHED NETWORKS                   !!%

X**=======================================================================**X

% X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X %

X**=======================================================================**X

%!! P/H/A - Written By Doctor Dissector On Sunday, April 22, 1990 - P/H/A !!%

X**=======================================================================**X

% X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X % X %

Part I: Disclaimer

------------------

The sole purpose of this document is to educate. Neither the author nor

the sponsor group (Phreakers/Hackers/Anarchists) will be held responsible

for the reader's actions before, during, and following exposure to this

document as well as the validity or accuracy of the information contained

within this document.

Part II: Introduction

---------------------

Packet switching networks can be said to be the most useful tool for both

the inexperienced and the experienced hack. When I first learned about

PSNs (SprintNet/Telenet in general), I discovered that there were not any

good "full length" introductions or guides to the use of these systems. In

effect, scrounging around for a small file here and another there was not

very productive in any sense. So, I decided to compile a "complete"

introduction and guide, as I know it, to the "world" of the packet switched

network. Enjoy!

Doctor Dissector - PHA

Part III: Table Of Contents

---------------------------

Part    Description

-----   -------------------------------------------------------------

I      Disclaimer

II     Introduction

III    Table Of Contents

IV     What Is A Packet Switched Network?

V      Network Protocols

VI     PAD Security

VII    Connection To The SprintNet PAD

VIII   X.121 International Address Format

IX     Network User Identification

X      Setting PAD ITI/X.3 Parameters

XI     Disconnect Code Sequence

XII    Misc Network Notes

XIII   Appendix

XIV    Conclusion And Closing Notes

XV     Greets, Hellos, Etc....

Appendix  Description

--------  -----------------------------------------------------------

A      Hunt/Confirm Sequence Codes

B      PAD Command Summary

C      ITI/X.3 Parameter Summaries

D      International DNIC/PSN List

E      Overseas PSNs Which Accept Collect Calls

F      Network Protocol List

G      Glossary

Part IV: What Is A Packet Switched Network?

-------------------------------------------

A packet switched network can be accessed through any local POTS

dialup/port. Systems known as "hosts" on the PSN pay for connection to

the PSN depending on transmission speed and protocol type. PSNs offer

more efficient data transfer and less rates as compared to the typical

circuit switched call. Thus, to anyone who would be interested in

transferring large amounts of data over either the PSN or the circuit

system, the PSN would result in an increase of convenience due to the

reduction of data transmission error and cost.

Another feature of the PSN is the speed and data translation which

takes place between the PSN's PAD (Packet Assembler/Disassembler) and

the host. For example, one could connect to the PSN's PAD at 1200 bps

and the PAD could connect to the host system at 9600 bps and still

allow the user to receive error free transmission. This "flow control"

is done by the actual increase or decrease of the data packet between

the PAD and the user or the PAD and the host.

PSNs also have the ability to interconnect through special gateways

which might allow one user who dialed one PSN's PAD and then connected

to another PSN's PAD through a system which was accessible by the first.

Almost every PSN in the world can be accessed through gateways on one

PSN to another PSN, through subsequent gateways until the target PSN

is achived; of course, there are always exceptions, some private or

small data networks may not be reachable through gateways, these systems

can only be reached, usually, through direct dialins.

Some PSNs allow the caller to execute "collect calls" to host

systems which accept them, although the majority of the hosts on any

given PSN do not accept collect calls. To connect to a host system which

does not accept collect calls, one must possess a network user identifier

(NUI) or access to a private system on the PSN which accepts collect

calls and has the ability to access another PSN with its own identifier.

These will be discussed further into this document.

Part V: Network Protocols

-------------------------

The PSN utilizes several communications protocols similar to the

communications protocols used by typical asynchronous modems. However,

MOST PSNs utilize synchronous communications and the X type protocols

versus the typical modem's asynchronous V protocols. As a result, the

PAD of any PSN also serves as a synchronous/asynchronous translator

between the synchronous netowrk and the asynchronous modem.

Most PSNs offer network speeds from snail's pace baud rates of

300 bps (asynchronous) to the lightning of 48,000 bps (synchronous).

The most common data protocol used by PSNs today is the X.25 protocol,

thus if one were able to access a private PAD which offered support for

the X.25 protocol, one could access virtually any network user address

(NUA) from that PAD. SprintNet PADs support the X.25 protocol, so if

one had an NUI of sorts, one also could access any NUA from the SprintNet

PAD. See appendix F for a list of network protocols.

Part VI: PAD Security

---------------------

SprintNet PADs and most dialin PADs in general have no "immediate"

form of telephone security common within their systems. Plainly, SprintNet

and most PSN dialin PADs cannot trace on the fly, as they do not have

their own equiptment to trace incomming calls. HOWEVER, this does not

mean that they CANNOT trace; SprintNet can, and will, upon probable

cause, cooperate with the telco to trace calls. Notice that tracing

usually is premeditated and one-time abusers have a very slim chance

of being caught. Also note that most PAD activities are logged and if

abuse is suspected, the PSN owners would most likely suspect the abuser

as originating from the local area, since the POTS dialin/port is also

located in the same area.

Once online, security from "calling" hosts which do not accept collect

calls is enforced by the presence of the NUI. Without an NUI, one would

usually be stuck, only able to call systems accepting collect calls, sans

the use of another system's NUI.

There is one more aspect of seucurity worth mentioning. Whenever a

packet of data is sent to a host system, a header of data is sent stating

where the originating "call" is being placed by. Thus, if you were

connecting to "312312" from your local POTS dialin/port that owned an

address of "20231H," the system at 312312 would know the call was being

originated from 20231H. Once again, if someone were abusing any system on

the PSN and that system saved a log of the originating addresses accessing

that system, the owners of the abused system could easily determine which

POTS dialin/port number the abuser was using, and then inform the PSN

security of possible abuse in that dialin's local area. Because of this

ability to "trace" the originating address, there is one way to foil this.

One could connect to another PAD, and then, from that PAD connect to

the target system. Thus, the POTS dialin/port address will be sent to

the connected PAD, and the connected PAD would intercept the POTS address

and send the connected PAD's address to the target system instead of

the POTS address. SO, if the target system was abused and the owners

attempted to "trace" the originating address, they would receive the

address of the connected PAD. For example: you dial your local POTS

dialin/port which had an address of "71516G," log into another PAD at

"415100," connect from 415100 to "213213." The system at 213213 if

"traced" would find that you were originating from 415100, not 71516G.

See how it works? Good... Notice that the system 213213 would still

know that you were originating from 71516G, but the folks you were

genuinely abusing wouldn't know that!

Part VII: Connection To The SprintNet PAD

-----------------------------------------

The following procedure outlines the methods used to connect to

and through the SprintNet PAD.

Step    Procedures                           Network/Operator Response

----    ----------                           -------------------------

1      Turn on your terminal. Make sure

it's Online.

2      Dial your local SprintNet access

number.

3      For data sets Bell 103 & 113 type,

depress the DATA button.

4      Enter the hunt/confirm sequence      <CR> <CR>

for your baud/parity type. For

E,7,1 1200/2400, type <CR> twice.

For hunt/confirm sequences, see

appendix A.

5      SprintNet will identify itself,      TELENET

its port address, and then send      909 14B

a TERMINAL= prompt for terminal

identification. "D1" specifies       TERMINAL=D1<CR>

dumb terminal.

6      NUI Input: After SprintNet gives

the "@" prompt, type "ID ;" and      @ID ;ABCD<CR>

then your ID code, follwed by a      PASSWORD=123456<CR>

<CR>. Then enter your password

followed by another <CR>. If you

don't have an NUI, you can always

access systems which allow collect

calls.

7      At the "@" prompt, you can enter     @02341123456790<CR>

the network user address (NUA) of

the desired host. If, during the

connection attempt wish to abort

the attempt, a BREAK signal will

bring you back to the "@" prompt.

8      SprintNet will respond with a        (address) CONNECTED

connection message, or an error

message.

9      To disconnect from your computer,    (address) DISCONNECTED

log off as usual. SprintNet will

send a disconnect message. To

disconnect off of a system without

logging off, typing "<CR>@<CR>" will

bring you back to the "@" prompt.

Part VIII: X.121 International Address Format

---------------------------------------------

Most PSNs around the world follow the X.121 format for access to both

domestic and international hosts. SprintNet does not require some parts

of the format for domestic connection, which will be discussed below.

+-----------------------------------------    Zero Handler For SprintNet

|                                             (Formats The X.121 Address)

|

|

|

|       +---------------------------------    Data Network Identifier

|       |                                     Code (DNIC)

|       |

|       |

|       |       +-------------------------    Area Code of Host

|       |       |

|       |       |

|       |       |         +---------------    DTE Address of Host

|       |       |         |

|       |       |         |

|       |       |         |       +--------    Port Address

|       |       |         |       |

|       |       |         |       |

|0|   |DDDD|   |AAA|    |HHHHH|   |PP|

|

+-------    Optional 'Subaddress'

Field for Packet Mode

DTE

For a complete list of DNICs/PSNs according to country, please see

appendix D.

On SprintNet, a "0" MUST lead the NUA, although on other PSNs, this

may not be necessary.

On SprintNet, the DNIC is defaulted to 3110. Any host entered at the

"@" prompt, if domestic to Telenet/USA, will not require the input of

zero handler or the 3110 DNIC. For example:

Domestic           X.121          SprintNet Int'l

----------     --------------     ---------------

2129966622     31102129966622     031102129966622

212869         311021200869       0311021200869

21244          311021200044       0311021200044

Part IX: Network User Identification

------------------------------------

Network user identifiers (NUIs) offer full SprintNet PAD use for

any distance or amount of time for any host accessible by the PAD in

question. Think of the NUI as a /<-/<00l Kode for calling long

distance. Any systems that you call are logged, and each call is charged.

At the end of the month, the owner of the NUI is billed. So, it is

possible to hack out NUIs and use them, but like k0dez, abuse kills.

NUIs can be entered into SprintNet in two ways. The first method is to

type "ID ;xxxx" where xxxx can be from 4-? charachters in length, both

alphabetic and numeric. Then, at the password prompt, enter a password.

The second method for entering an NUI is in conjunction to the NUA

you are accessing. The format is "<NUA>,<ID>,<PW>" where at the "@"

prompt you would type the desired NUA, followed by a comma, then your

ID followed by a comma, and then your password. Your password will not

be echoed.

Part X: Setting PAD ITI/X.3 Parameters

--------------------------------------

Online PAD parameter modification may be desired for certain

applications, connections, or data transfers. See appendix C for brief

summaries of these parameters. Modification of these parameters can be

done by the following procedure at the "@" prompt:

X.3 Parameters

--------------

To display current parameters: "PAR?<CR>"

The PAD will respond with: "PAR1:<VALUE>,2:<VALUE>,..."

To modify parameter(s): "SET? <PARM>:<VALUE>,<PARM>:<VALUE>,..."

The PAD will respond with: "PAR<PARM>:<VALUE>,..."

ITI Parameters

--------------

To display current parameters: "PAR? 0,<PARM>,<PARM>,..."

The PAD will respond with: "PAR<PARM>:<VALUE>,<PARM>:<VALUE>,..."

To modify parameter(s): "SET? 0:33,<PARM>:<VALUE>,<PARM>:<VALUE>,..."

The PAD will respond with: "PAR0:33,<PARM>:<VALUE>,..."

Part XI: Disconnect Code Sequence

---------------------------------

When disconnected off of any host on SprintNet, a disconnect coding

sequence with a string of data will be sent to your terminal. The

following is a translation format for the disconnect coding.

<NUA> DISCONNECTED AA BB TT:TT:TT:TT CCC DD

Where:

<NUA> is the NUA of the given host system.

AA is the clearing code.

BB is the diagnositc code.

TT:TT:TT:TT is the time spent on the host.

CCC is the number of frames received.

DD is the number of frames sent.

Part XII: Misc Network Notes

----------------------------

Just a few things one might want to know when using PSNs:

1) When using/abusing a private PAD, try to use it after business

hours, as the operators will not tend to discover your presence

as quickly.

2) When hacking or abusing ANY system on ANY PSN, if anything seems

different or suspicious, logoff, disconnect, or HANG-UP

IMMEDIATELY! Much better SAFE than SORRY!

3) For a complete and updated list of POTS dialin/ports, dial the

IN-WATS number at 1-800-546-1000 or 1-800-546-2000, type "MAIL,"

and for user name and password, enter "PHONES." You will be

diverted to the SprintNet dialing directory & a menu. From then on

you will have plenty of info about POTS dialins and port numbers.

4) For international information concerning SprintNet and other PSNs,

get to a SprintNet "@" prompt and type "MAIL." Then, for the user

name, enter "INTL/ASSOCIATES." For the password, type "INTL," and

you will be diverted to the international information menu.

5) For even more info on SprintNet and PCP, the NUA for the PCP

support BBS is 311090900631 (909631 domestic).

6) Some 2400 bps and 2400+ bps PADs have problems recognizing 8,N,1

connections. Sometimes they only allow E,7,1 transmissions.

Experimentation or inquiry may yeild results. SprintNet's customer

information line is at 1-800-336-0437, overseas is 1-703-689-6400.

7) PCP outdials and other outdial systems are abundant on the PSNs

throughout the world. If you have any NUAs to these or find any,

they utilize the typical Hayes AT command set, so they should be

easy to figure out. MOST of the time, they ONLY allow dialing of

local (to the oudial's area code) numbers, but some have been known

to allow interstate and even international calls. Experimentation,

again, is always necessary.

8) Domestically, the "AAA" (Area Code) portion of the NUA is usually

the same as the area code (NPA) of the same calling area. However,

some area codes are shared on the network and some non-existant

area codes such as 909, 223, 224 and others contain hosts.

9) On any PAD, the data transmission rates may be slowed, due to the

assembley/disassembley time, called packet delay. Depending on which

system, baud, and transfer protocol used, pad delay can differ from

almost none to noticable fractions of seconds. PCP oudials are

notorious for LLOONNGG pad delays....

Part XIII: Appendix

-------------------

Appendix A: Hunt/Confirm Sequence Codes

=======================================

Bits  Stop  Parity  Modem Baud  Duplex  Sequence

----  ----  ------  ----------  ------  --------

7     1     EVEN    300-1200    FULL   <CR><CR>

7     1     EVEN    300-1200    HALF   <CR>;<CR>

7     1     EVEN      2400      FULL     @<CR>

7     1     EVEN      2400      HALF     @;<CR>

8     1     NONE    300-1200    FULL   <CR>D<CR>

8     1     NONE    300-1200    HALF   <CR>H<CR>

8     1     NONE      2400      FULL     @D<CR>

8     1     NONE      2400      HALF     @H<CR>

At BPS speeds 2400+, wait 1/2 a second BEFORE and AFTER the

"@" sign in the sequence above.

Appendix B: PAD Command Summary

===============================

The following is a list of commands usable from the "@" prompt on the

SprintNet PSN.

Command      Description

-----------  -------------------------------------------------------------

<NUA>        Connects to the host specified by that NUA.

C <NUA>      Connects to the host specified by that NUA.

STAT         Displays the network port address (NUA of the port).

FULL         Sets duplex to full.

HALF         Sets duplex to half.

DTAPE        Prepares the PSN for bulk file transfers.

CONT         Continues the current connected session/connect attempt.

BYE          Aborts connect attempt/disconnects from current session.

D            Aborts connect attempt/disconnects from current session.

HANGUP       Logs you off from the SprintNet PAD.

TERM <TERM>  Changes the terminal specification to that of <TERM>.

MAIL         Request connection to SprintNet Telemail.

TELEMAIL     Request connection to SprintNet Telemail.

ID ;<ID>     Enter NUI, <ID> is your ID. This is followed by a PASSWORD

prompt. Password will not be echoed.

TEST CHAR    Test if you are receiving garbled output. If so, adjust

parity or data bits, and then try again. If errors persist,

be sure to complain to SprintNet customer service!

TEST ECHO    Test if your input is being garbled by Telenet. Similar

otherwise as TEST CHAR.

Appendix C: ITI/X.3 Parameter Summaries

=======================================

Para-                               Para-

meter  Description (Default Value)  meter  Description (Default Value)

-----  ---------------------------  -----  ---------------------------

1    Line feed Insertion (0)       31+   Interrupt Character (0)

2    Network Message Display (0)   32    Automatic Hang-up (0)

3    Echo (1)                      33+   Flush Output (0)

4    Echo Mask (163)               34    Transmit on Timers (1)

5    Transmit Mask (2)             35    Idle Timer (80)

6*   Buffer Size (0)               36    Interval Timer (0)

7*   Command Mask (127)            37    Network Usage Display (0)

8*   Command Mask (3)              38    Carriage Return PAD (Variable)

9    Carriage Return PAD (Fixed)   39    Padding Options (1)

10    Linefeed Padding              40    Insert on Break (0)

11    Tab Padding                   41    PAD-Terminal Flow Control (0)

12    Line Width                    42    PAD-Terminal XON Character (17)

13    Page Length (0)               43    PAD-Terminal XOFF Character (19)

14    Line Folding (1)              44*   Generate Break (INV)

15    Page Wait (0)                 45*   APP on Break (0)

16    Interrupt on Break (0)        46    Input Unlock Option (0)

17    Break Code (0)                47    Input Unlock Timer (0)

18    NVT Options (0)               48    Input Unlock Character (0)

19    Initial Keyboard State (0)    49    Output Lock Option (2)

20    Half/Full Duplex              50    Output Lock Timer (10)

21    Real Character Code           51    Output Lock Option (0)

22    Printer Style                 53*   Break Options (0)

23    Terminal Type                 54    Terminal-PAD Flow Control (0)

24    Permanent Terminal (0)        55    Terminal-PAD XON Character (17)

25    Manual or Auto Connect (0)    56    Terminal-PAD XOFF Character (19)

26    Rate                          57    Connection Mode (2)

27    Delete Character (127)        58    Escape to Command Mode (1)

28    Cancel Character (24)         59*   Flush Output on Break (0)

29    Display Character (18)        60    Delayed Echo

30+   Abort Output Character (0)    63    Eight-bit Transparency (1)

64+   Early ACK (0)

65    More-Data Bit Generation (3)

66    Defer Processing of User (0)

67    ESP Packetizing Option (0)

68    Escape Sequence Timer (0)

69    Escape Sequence Maximum Length (0)

70    Escape Sequence Initiator (0)

71    Parameter Reset on Disconnect (0)

Note: - All Telenet Parameters must follow the National Option Marker

(Parameter 0, value '21' Hex) in PAD Messages.

- Parameters marked with "*" should not be used.

- Parameters marked with "+" should be used with caution.

Appendix D: International DNIC/PSN List

=======================================

Note: This is not a complete list!

COUNTRY                     NETWORK                             DNIC

-------                     -------                             ----

ALASKA                      ALASCOM                             3135

ANTIGUA                     ANTIGUA                             3443

ARGENTINA                   ARPAC                               7220

ARGENTINA                   ARPAC                               7222

AUSTRIA                     DATEX-P                             2322

AUSTRIA                     RA                                  2329

AUSTRALIA                   AUSPAC                              5052

AUSTRALIA                   MIDAS                               5053

BAHAMAS                     BATELCO                             3640

BAHRAIN                     IDAS                                4263

BARBADOS                    IDAS                                3423

BELGIUM                     DCS                                 2062

BELGIUM                     DCS-TELEX                           2068

BELGIUM                     DCS-PSTN                            2069

BERMUDA                     IPSD                                3503

BRAZIL                      INTERDATA                           7240

BRAZIL                      RENPAC                              7241

BRAZIL                      RENPAC                              7249

BRAZIL                      RENPAC                              7248

CAMEROON                    CAMPAC                              6242

CANADA                      DATAPAC                             3020

CANADA                      GLOBEDAT                            3025

CANADA                      CNCP                                3028

CANADA                      TYMNET CANADA                       3106

CAYMAN ISLANDS              IDAS                                3463

CHILE                       ENTEL                               7302

CHILE                       ENTEL                               3104

CHINA                       PTELCOM                             4600

COLUMBIA                    DAPAQ                               3107

COSTA RICA                  RACSADATOS                          7120

COSTA RICA                  RACSAPAC                            7122

COSTA RICA                  RACSAPAC                            7128

COSTA RICA                  RACSAPAC                            7129

COTE D'IVOIRE               SYTRANPAC                           6122

DENMARK                     DATAPAK                             2382

DEMMARK                     DATAPAK                             2383

DOMINICAN REPUBLIC          UDTS                                3700

EGYPT                       ARENTO                              6020

FINLAND                     FINNPAK                             2442

FRANCE                      TRANSPAC                            2080

FRANCE                      N.T.I.                              2081

FRANCE                      TRANSPAC                            9330

FRANCE                      TRANSPAC                            9331

FRANCE                      TRANSPAC                            9332

FRANCE                      TRANSPAC                            9333

FRANCE                      TRANSPAC                            9334

FRANCE                      TRANSPAC                            9335

FRANCE                      TRANSPAC                            9336

FRANCE                      TRANSPAC                            9337

FRANCE                      TRANSPAC                            9338

FRANCE                      TRANSPAC                            9339

FRENCH ANTILLES             DOMPAC                              3400

FRENCH GUYANA               DOMPAC                              7420

GABON                       GABONPAC                            6282

GERMANY                     DATEX-P                             2624

GREECE                      HELPAK                              2022

GREENLAND                   DATAPAK                             2901

GUAM                        LSDS-RCA                            5350

GUATEMALA                   GUATEL                              7040

HONDURAS                    HONDUTEL                            7080

HONG KONG                   IDAS                                4542

HONG KONG                   DATAPAK                             4545

HUNGARY                     DATEXL                              2160

HUNGARY                     DATEXL                              2161

ICELAND                     ICEPAC                              2740

INDONESIA                   SKDP                                5101

IRELAND                     IPSS (EIRE)                         2721

IRELAND                     EIREPAC                             2724

ISRAEL                      ISRANET                             4251

ITALY                       DARDO                               2222

ITALY                       ITAPAC                              2227

IVORY COAST                 SYTRANPAC                           6122

JAMAICA                     JAMINTEL                            3380

JAPAN                       DDX-P                               4401

JAPAN                       VENUS-P                             4408

JAPAN                       NISNET                              4406

JAPAN                       NI+CI                               4410

KUWAIT                                                          4263

LEBANON                     SODETEL                             4155

LUXEMBOURG                  LUXPAC                              2704

LUXEMBOURG                  PSTN                                2709

MALAYSIA                    MAYPAC                              5021

MAURITIUS                   MAURIDATA                           6170

MEXICO                      TELEPAC                             3340

NETHERLANDS                 DATANET-1                           2040

NETHERLANDS                 DATANET-1                           2041

NETHERLANDS                 DABAS                               2044

NETHERLANDS                 DATANET                             2049

NETHERLANDS/ANTILLES        UDTS ITT                            3620

NETHERLANDS/MARIANAS        PCINET                              5351

NEW CALEDONIA               TOMPAC NC                           5460

NEW ZEALAND                 PACNET                              5301

NORWAY                      DATAPAK                             2422

PANAMA                      INTELPAQ                            7141

PANAMA                      INTELPAQ                            7142

PHILIPPINES                 CAPWIRE                             5151

PHILIPPINES                 PHILCOM RCA                         5152

PHILIPPINES                 GMCR                                5154

PHILIPPINES                 ETPI-2                              5156

POLYNESIA                   TOMPAC                              5470

PORTUGAL                    TELEPAC                             2680

PORTUGAL                    SABD                                2682

PUERTO RICO                 UDTS- PDIA                          3301

PUERTO RICO                 UDTS- I                             3300

QATAR                       DOHPAC                              4271

REUNION ISLAND              DOMPAC                              6470

SAN MARINO                  X-NET                               2922

SAUDI ARABIA                BAHNET                              4263

SINGAPORE                   TELEPAC                             5252

SINGAPORE                   TELEPAC                             5258

SOUTH AFRICA                SAPONET                             6550

SOUTH AFRICA                SAPONET                             6559

SOUTH KOREA                 DACOM-NET                           4501

SOUTH KOREA                 DNS                                 4503

SPAIN                       TIDA                                2141

SPAIN                       IBERPAK                             2145

SWEDEN                      TELEPAK                             2405

SWEDEN                      DATAPAK                             2402

SWITZERLAND                 TELEPAC                             2284

SWITZERLAND                 DATALINK                            2289

TAHITI                      TOMPAC                              5470

TAIWAN                      UDAS                                4877

TAIWAN                      PACNET                              4872

THAILAND                    IDAR                                5200

TORTOLA                                                         3483

TRINIDAD                    TEXTET                              3740

TRINIDAD                    DATANETT                            3745

TUNISIA                     RED25                               6050

TURKEY                      TURPAC                              2862

TURKS BWI                                                       3763

UNITED ARAB EMIRATES        EMDAN                               4241

UNITED ARAB EMIRATES        TELEX                               4243

UNITED ARAB EMIRATES        TEDAS                               4310

UNITED KINGDOM              IPSS                                2341

UNITED KINGDOM              PSS                                 2342

UNITED KINGDOM              MPDS MERCURY                        2350

UNITED KINGDOM              PSS MERCURY                         2352

U.S.S.R.                    IASNET                              2502

UNITED STATES OF AMERICA    TELENET                             3110

UNITED STATES OF AMERICA    TYMNET                              3106

U.S. VIRGIN ISLANDS