Air-Enhancement Experimentation

By Tim Ventura, Dec 10, 2002

Goal: To increase the conductivity of the air between the wire and the foil in the Lifter design to allow the conduction of higher-volumes ions through the air-gap at lower (non-ionizing) voltages.

# Ultraviolet Stimulation

Method: To apply a source of ionizing ultraviolet light to the air-gap to enhance the conductivity of the air through photoionization. Theoretical indications are that 263nm light is an acceptable frequency in the UV spectrum to use, and previous experimentation has been successfully conducted in other applications using UV lasers in the 190 and 260nm range.

Test Apparatus: Experimental apparatus included the use of (2) G4T5 bulbs and (2) G2T3 “Germicidal Lamps”. These bulbs are designed to produce ionizing ultraviolet radiation for the treatment of air and water in the medical industry. Testing included the monitoring of current through the air-gap at various voltages both in the presence of and the absence of UV-light to test whether current-flow would increase when the uv-light was applied.

Test Results: Experimentation with the germicidal UV-lamps showed no appreciable increase in current-flow across the air-gap at any voltage. Voltages were tried ranging from 115vac to 50kvdc, and the distance between wire and foil on the air-gap was varied from 0.5 to 5 cm in height.

Notes: Initial test results were disappointing – physics theory supports the creation of ionized-region of air from bulbs. Perhaps the wattage output was too low (8 watts for G4T5, and 3 watts for G2T3). Additional tests will include larger UV bulb (G5T8 @ 25 watts) and UV-laser diodes.

# Cox Dipole Thruster

Method: To reduce the energy expenditure in producing a high-voltage DC field by the manipulation and acceleration of electrically charged air via an externally applied magnetic field. In the case of the Cox Dipole Thruster, the method includes using a synchronized high-voltage alternating current applied to a volume of air, which is then accelerated by the magnetic field applied by an electromagnetic. Lorentz-force should provide consistent, directional thrust, as the fields reverse in phase, meaning that the force will not reverse as the cycle changes.

Test Apparatus: An electromagnetic custom-wound to allow the inclusion of a 2-wire air-gap in the central region of the magnet. In this case, the electromagnet includes approximately 200 turns of #26 gauge wire, with an internal resistance of approximately 9 Ohms. The central air-gap is approximately 3 cm from wire to wire, and approximately 5cm in height. Total resistance on the electromagnet’s circuit is approximately 30 Ohms, including balance loading.

Test Results: Initial testing utilizing a high-voltage DC current across the air-gap and a 115vac current applied to the magnetic-coil indicates that a magnetic-displacement of the ion-stream does occur. Accurate measurements of thrust could not be taken, as the ion-wind had a tendency to ground-itself to the electromagnet’s core during operation. A noticeable change in direction on the ion-wind was observed, and with a high-voltage AC power-supply a directional motion of air is expected to be observed as well.

Notes: Per James Cox: “The basic configuration is a discharge electrode pair situated in the middle of a solenoidal coil and in series connection to said coil. Use the neon transformer output and make a many turn amp turn coil with appropriate turn-to-turn insulation with provision for a ball bearing like electrodes in middle so discharge current is perpendicular to magnetic field lines--it should blow the neon discharge downstream if Tesla-frequency-voltage is above a given threshold. Very careful attention to sensitive measurement apparatus could be needed.”

# Microwave Stimulation

Method: Microwave stimulation of the air is intended to capitalize on the well-known effect of electrical-arcing in conventional kitchen microwave devices when a metallic object is inserted. The arcing occurs in the apparatus when a metallic object is introduced during operation because of the high-voltage alternating-current charging on the object by the microwave-beam.

Test Apparatus: A microwave-oven klystron should be connected to the emitter-wire of the Lifter during operation, in order to create a heating effect and increase the conductivity of the air during testing. This test apparatus would consist of the conventional wire/foil apparatus, with the microwave output being directed so as to ionize the water-molecules in the air between the wire and foil. Current-flow should be measured across the air-gap both with and without the microwave in operation to determine if the conductivity of the air has been enhanced.

# Electrical Pre-Ionization

Method: The expectation for this test was that a high-voltage trigger charge crossing between two low-voltage/high-current terminals would provide a plasma for the discharge of high-current bursts between the low-voltage terminals. Such bursts could provide greater thrust than the high-voltage trigger-charge itself could, and would provide a primary source of motive ion-wind thrust in the circuit.

Test Apparatus: In this experiment the apparatus included a cross-shaped layout for 4 electrodes. The North and South facing electrodes were respectively the high-voltage emitter and collector, and E & W were low-voltage/high-current 115 vac terminals.

Test Results: Test results indicated that some conductive-discharge did occur between the low-voltage terminals, although it is difficult to gauge the actual amount of current exchanged between them during testing. Excessive sparking between the high-voltage terminal and the low-voltage terminals prevented accurate measurement of ion-wind flowing between the AC-current emitters.

Notes: Future testing should involve using long, wire-mounted low-voltage terminals. Additionally, high-voltage alternating-current excitation should be attempted to determine if a smaller trigger voltage might be preferable for creating low-voltage, high-current thrust. Another experiment to try involves a high-voltage alternating current being applied to foils mounted on each side of vertically oriented wires connected to a low-voltage, high-current power-supply. In this case, a single plate would be mounted on each side of the two wires, with both plates connected to the high-voltage AC power-supply. The expectation is that the low-voltage would conduct through the air-gap between the plates when the plates are electrostatically charged with the hv-ac current. Other modifications to this design involve putting high-voltage rectifier diodes on the 115 vac power-leads to create a pulsed DC current.

# Conductive-Gas Injection

Method: Saviour at Blaze Labs has measured the electrical permittivity of Carbon-Dioxide (CO2) in parts-per-million (ppm), and graphed the results in a charged of atmospheric-conductivity when CO2 is introduced in several ppm. It has been shown that injecting a conductive gas into the air even in very small concentrations should increase the conductivity of the atmosphere, and should allow a dielectrophoresis-effect to occur between the high-voltage terminals.

Test Apparatus: The test-apparatus should consist of both the conventional emitter and collector used in testing the Lifter apparatus, as well as an emitting-nozzle or other source of CO2 to enhance the air’s conductivity.

Test Results: Tests utilizing copper enamel-coated magnet-wire demonstrate that even very small levels of impurities from the enamel-residue as it burns off during testing have the ability to increase the conductivity for a brief period of time, which in turn creates a greater amount of ion-flow between the emitter and the collector. Additionally, flame-test experiments have shown that CO2 and other combustion-gases introduced into the air-gap will enhance the conducitivity of the air, and allow greater-concentrations of ions to flow between the emitter and collector.

Notes: Future tests should be conducted to determine whether applying a light coating of graphite to the corona-wire will increase the atmospheric-conductivity due to erosion and the subsequent introduction of graphite combustion products into the atmosphere during testing. Additionally, testing should be conducted to determine whether or not the direct introduction of gaseous Carbon-Dioxide into the air-gap will contribute to an appreciable increase in thrust.

# Carbon Dioxide Conductivity Chart

URL: http://bel.150m.com/exp01.htm

| Conductivity mS/cm | ppm |
| --- | --- |
| 0.05 | 0 |
| 0.09 | 0.01 |
| 0.12 | 0.02 |
| 0.16 | 0.03 |
| 0.19 | 0.04 |
| 0.21 | 0.05 |
| 0.24 | 0.06 |
| 0.26 | 0.07 |
| 0.28 | 0.08 |
| 0.3 | 0.09 |
| 0.32 | 0.1 |
| 0.48 | 0.2 |
| 0.61 | 0.3 |
| 0.71 | 0.4 |
| 0.81 | 0.5 |
| 0.89 | 0.6 |
| 0.97 | 0.7 |
| 1.04 | 0.8 |
| 1.11 | 0.9 |
| 1.17 | 1 |
| 1.69 | 2 |
| 2.09 | 3 |
| 2.42 | 4 |
| 2.72 | 5 |