Lifter Required Materials

| Author: Timothy M. Ventura |  | Date: March 09, 2002 |
| --- | --- | --- |
| Warning Notice: This document is intended for educational use only! The project described in this document uses extremely high-voltage power-sources, and is intended for readers over 21 years of age who are experienced working with dangerously high-voltages. If you are not a legal adult, or are not proficient working with dangerously high-voltages, do not try to build this project without expert supervision. The author of this document is not responsible for any death, injury, or property damage resulting from or relating to the procedures shown or devices described in this document. |  |  |
| Purpose of this Document: This document provides additional information on the materials required to successfully build a working prototype Lifter. Many of the materials listed can be dangerous or fatal if abused – please see the above warning for details. |  |  |
| Materials: (photo on left) Construction of a Lifter requires the following materials, shown below: - Balsa Wood strips - Magnet wire - Aluminum Foil - Cyanoacrylate Adhesive - Scotch Tape - Sewing Thread - Hobby Knife - Metric Ruler - Computer-Monitor |  |  |
| 1: Balsa-Wood Strips - Type: 2mm by 6mm strips. - Price: $0.49 each (total of 4) - Notes: I purchase 2mm by 6mm Balsa-Wood strips and split them down the middle length-wise until they measure only 1mm by 3mm. You need probably 1 meter of balsa – buy extra just in case because it breaks very easily. Make sure to buy ‘balsa-wood’ – do not buy ‘bass wood’. Although bass wood is commonly sold next to balsa wood as a substitute, bass-wood is too heavy, and also more difficult to cut. |  |  |
| 2: Magnet Wire - Type: 30-Gauge enamel-coated copper. - Price: $3.99 @ Radio-Shack - Notes: Radio-Shack sells this in a 3 pack containing 26, 28, and 30-gauge magnet-wire. The 30-gauge wire has a red enamel coating, and should be on a spool containing enough to build several Lifter prototypes. Enamel coated wire is not required, but reduces extraneous ion-leakage. |  |  |
| 3: Aluminum Foil - Type: Reynolds or generic brand foil - Price: $2.99 (approximate) - Notes: Make sure to use aluminum foil and not tin-foil. Tin-foil will probably be too heavy to work correctly. You can use regular generic-brand aluminum foil from the grocery store. |  |  |
| 4: Cyanoacrylate Adhesive - Type: Super-Glue or Krazy-Glue - Price: $1.99 - Notes: I recommend using cheap, regular super-glue from a squeeze-tube applicator. Do not use the pen-style applicator as it will probably damage your balsa-wood (The pen-style contains a small plastic nub that requires pressure). Also, do not use the gel-style glue unless you absolutely have to, because it does not hold very well on balsa wood. I use about ½ tube per Lifter. |  |  |
| 5: Sewing Thread - Type: Standard or Extra-Strong Sewing Thread - Price: $1.00 (approximate) - Notes: I use extra-strength sewing thread for the tethers on my Lifters. Make sure to use very lightweight thread – normal strength thread should be fine. |  |  |
| 6: Hobby-Knife - Type: Exacto-Brand or generic - Price: $2.99 (approximate) - Notes: I am using an Exacto-Brand knife for cutting the balsa wood. You will want to make sure that you have a very sharp blade on the knife in order to properly bevel the ends of the balsa support struts. |  |  |
| 7: Scotch-Tape - Type: Scotch-Brand or generic tape (3/4” width) - Price: $1.99 (approx.) - Notes: Scotch tape or generic brand – either clear or translucent tape is fine. Scotch tape is required to anchor the Lifter tether-threads. It is also required to reduce ion-leakage on the folded inside-edge of the foil-anode. |  |  |
| 8: Computer-Monitor Power-Supply - Type: 25,000+ volt DC or higher output - Price: $100 (approximate used price) - Notes: My tests were successful using a computer monitor power-supply, but a monitor is not required – only the approximately 30kV DC power-output. Most 14-inch or larger computer-monitors should provide enough power to work correctly – a tube-style monitor is required (LCD monitors are not high-voltage). |  |  |
| 9: Metric-Scale Measuring Ruler - Type: Any ruler offering metric-scale measuring for millimeter and centimeter scale measurements. - Price: $1.00 (approximate) - Notes: Any ruler should work – the Lifter dimensions are not as critical as the weight is. |  |  |