Soloã

Introduction

Pre-solo Flight Training

Solo Required Knowledge---FAR Part 61.87:

Student Pilot Requirements

Student Pilot Limitations---FAR PART 61.89

Flight Rules---FAR PART 91

Compliance Required for Flight

First Solo

Student Requirements

Instructor Requirements

The Flight

Written Test on Essential Knowledge:

Essential Knowledge

Presolo Test of Student Pilot

Presolo Test of Student Pilot (Instructor’s)

Student Endorsements (Instructor)

Student Pilot Endorsements (Instructor)

Second Solo

Solo Instruction

After Solo-What?

CCR to Local Airports.

Question

A change of pace...

Solo

# Introduction

Solo preparation began on the first flight.  Now, the FARs require specific paper work, study, and testing.  Student skill and knowledge for the flight must meet safety requirements. The instructor will impose limits of visibility and wind conditions.   The extent of the testing of FARs and aircraft knowledge will vary but FARs say the depth must be enough to  determine competency.  Airport and radio procedures must be covered for all anticipated solo situations.  The words, "student pilot," now become part of the aircraft identification whenever the full aircraft identification is appropriate. i.e. "Cessna 6185K student pilot"

The day of possible solo is significant both to student and instructor.  Psychological conditioning begins during the post flight discussions several flights before actual solo.  The parameters of acceptable flying conditions is based primarily on safety considerations. This needs to be clearly understood by the student.  Unsafe solo conditions turns the planned dual/solo lesson into a dual only.  The teaching accountability of the flight instructor is exceeded in no other field and equaled in only a few.

The pre-solo discussion is used to confirm that all required papers are, if not completed, available.  An oral review of the conditions requiring slow flight and the short approach should be included.  The dual is limited by agreement.  The instructor expects that the student will complete at least three safe landings and a go- around in the first half hour of closed traffic.  If this is not accomplished then the remainder of the flight time will be to review or refine landing skills in dual.  The reason for this is that extending the dual time beyond the half hour creates student fatigue that might jeopardize solo safety.  One way to judge the intellectual and emotional load of a student on any pre-solo flight is to see if he can handle the landing process while conversing about some unrelated subject just as he would in an automobile.  It's nice to know that the student has some capacity left for emergencies when the instructor gets out.

If all goes well, the student solo proceeds.  The student is expected to make two touch and go's a full stop in solo flight. A go around does not count.  The instructor will be in the tower to monitor the flight and advise ATC of the student's capability to handle an unexpected situation.  On occasion, the instructor may authorize an extension of the solo flight.

# Pre-solo Flight Training

(1) Preflight, Engine operation, systems

(2) Starting, taxiing, runup

(3) Takeoff & landings, normal and crosswind

(4) Flying straight and level, shallow, medium and steep turns.

(5) Climbs and climbing turns

(6) Traffic pattern entries and departures, collision and large 				aircraft wake avoidance

(7) Descents straight and w/turns, with and without flaps

(8) Speeds cruise through  minimum controllable

(9) Emergencies and malfunctions

(10) Ground reference maneuvers

(11) Power-off landings

(12) Slips to a landing

(13) Go arounds from base turns through final flare

(14) Forced landings from takeoff to anywhere in the pattern

(15) Stall entries w/varied attitudes and power with recovery at 			first sign

# Solo Required Knowledge---FAR Part 61.87:

AIRCRAFT;

Required papers

Required inspections

Oil levels

Fuel consumption/flight time

Weight/balance limits

Rotation/climb speeds

Approach/landing speeds

Flap use/go-around procedures

Effect of bank on stall speed

Emergency operations

# Student Pilot Requirements

License/medical certificate

Knowledge of flight rules--FAR PART 91

Proficient in pre-flight/run up-taxiing

Proficient in climbs/turns/level/descents

Slow flight/stall recognition-recovery

Traffic patterns/ground reference/collision avoidance

Emergencies/priorities

License and Logbook

Endorsements

# Student Pilot Limitations---FAR PART 61.89

(a) (1) No passengers

(2-3)  No flying for hire

(4)    No flying for business

(5)    No international flights

(6)    No flying daytime with visibility below 3 miles, or at night

with less that 5 mile visibility.

(7)    No flying without visual reference to the surface

(8)    No flying that is contrary to instructors logbook endorsements and 			limitations

(b) (1)  Cannot act as required crew member

(2) Cannot fly into certain Class Bs unless endorsed in last 90 days 			(91.95(a)

Cannot be endorsed for San Francisco Class B airspace.

Student must have instructor approval of all flights

Student must have 90 day solo endorsement

Student must have license, medical, and logbook in possession when 			flying.

Student certificate must be signed on back for each aircraft type.

Student certificate must be signed on back for cross country flight.

(This instructor does not allow student night flights)

(The new FARs do not allow student SVFR flights)

The student is not to make overflights such as would attract attention 			of friends or relatives.

# Flight Rules---FAR PART 91

.3 Pilot in command/

.5 Preflight/

.9 Careless operation/

.11 Liquor/

.14 Seat belts

.67 right-of-way/

.71 Acrobatic/

.73 lights

.79 safe altitudes/

.105 VFR minimums/

.107 SVFR/

.109 flight altitudes

# Compliance Required for Flight

1. Emergency Locator Beacon  (some exceptions)

2. Advisory Directive (AD) is due and not complied with

3. Owner/operator responsible for maintaining airworthiness

4. FAA approved flight manual only if flying

5. Engine baffling

6. Airworthiness certificate

7. Pilot may make airworthy by disabling and placarding

8. Only an Airframe and Power mechanic with inspection rights

can modify an aircraft.

9. An aircraft is airworthy only when conforming to type design.

# First Solo

There are many self-induced stresses on the student prior to solo.  The student begins to worry about it never happening, the cost, how others have done it sooner, and a thousand doubts.  As a student your forget that the instructor wants you to solo, also.   Flying as often as you can just prior

to solo is a big help.  Time spent getting appropriate dual is better use of

time and money than scary solo.

A student can help his own situation by treating every pre-solo dual as

though it were going to be a solo.  Take responsibility for all communications and decisions.  Make your own go-around decisions.  By making your own decisions you will be able to show the instructor that you are ready to solo.

Before a student is endorsed for solo he must show both knowledge and compliance with FAR Part 91.  He must have the required papers and endorsements of flight and ground training as required by FAR Part 61.87.  Solo becomes possible only when the student is proficient.  There is no time requirement nor should the student have misconceptions as to time producing proficiency.

Being proficient means that the student can fly the aircraft so as to stabilize climbs, level, and descents.  He uses flaps and trim to secure optimum performance.  While flying he is able to position the aircraft appropriately in the pattern and safely on the runway.  He must understand and correctly respond to ATC communications.  He can accommodate The proficient student has his priorities in order, i.e.; fly the plane, navigate, communicate.  when in doubt, go around.

You and the instructor will know when you are ready for solo.  On the ‘day’ you will fly three touch and go’s or full stops (your option) and a go-around.  If they go well the instructor will direct you to the base of the tower.  The paper work will be completed.  The instructor will go into the tower.  You will proceed to the runway and make three touch and go’s or full stop landings.  Now taxi to the East ramp, it’s over.

Soloing is one of those once-in-a-lifetime experiences that you will always remember.  It is a milestone but the best is yet to come.

# Student Requirements

Have your license/medical

Radio license - no longer required but nice to have

Logbook

Endorsements

# Instructor Requirements

Instructor must sign:

license/logbook/proficiency 61.105a 1-5

The instructor will review and test student's

Use of checklists/verbalize actions

T/O landing procedures/common errors

Use of flaps, carburetor heat

Short approach and go-around

What to do if high/low downwind-base-final

Runway alignment/flare/touchdown

Cross-wind procedures if likely

Radio communications/traffic acknowledgments

Aircraft call up as student pilot

Effect of strong/calm/cross wind

When to go to slow-flight

Essential speeds/use of trim

Clearing/drifting/turns

Traffic procedures

Priorities

# The Flight

Departure Instruction:

Taxiing from tower to longest runway

Clearing approach before entering runway

Rotation/climb-out/heading/wind effect

Airspeed/turns/level-off

Pre-landing/traffic/radio

Expectations the instructor has of the student:

Reasonably smooth ground operations

Tower call up/TO per clearance

No compromises with safe operation at any time

Accurate airspeed control

Adjustment of pattern to conditions/traffic

Acknowledgment of radio calls

Safe controlled landings oriented to runway direction

Smooth/controlled go-around

Smooth/controlled clearing of runway

Ground control call up/taxiing per clearance

INSTRUCTORS APPROVAL OF ALL FLIGHTS

# Written Test on Essential Knowledge:

FAR's Part 91 flight rules

FAR's Part 6l student pilot restrictions

Aircraft operations manual

Airport operational procedures

Airport runways and checkpoints

PIC responsibilities

# Essential Knowledge

1. Fuel:       Octane, useful, preflight, consumption, and time of

flight to FAR required reserve.

2. Engine: Oil, power settings, leaning, runup, magnetos, shut

down, and proper operations.

3. Speeds: Vx, Vy, Va, Vso, best glide, cruise, normal approach,

short approach, and taxiing.

4. Systems:    Electric, vacuum, static, control, trim, brake,

lights, radios.

5. Instruments: Compass, HI, AI, VSI, altimeter, turn coordinator,

airspeed indicator, engine gauges.

6. Maintenance: Log books, required inspections, ADs, papers.

# Presolo Test of Student Pilot

Subsequent tests in other makes and types may be limited to differences in operations and systems.  Instructor copy  required by FAR.

# Presolo Test of Student Pilot (Instructor’s)

Instructor copy required by FAR

C-150/C-172 Operation

1._______ is the instructor determined safe flight time for a C-150 		before being on the ground for fuel.

1a.___ ___  How is this different from the C-172?

2._______  is the minimum oil in a C-150 before adding oil.

2a.______  How is this different from the C-172?

3._______ of Cessna aircraft are prone to slip back.

3a.___ ___  How is this different from the C-172?

4._______ is what you say before starting the engine.

5.____ ____ is where you hold the yoke when the wind is behind you.

6._______  the wind when you run-up.

7._______  toward the final approach course of the runway before you taxi on 			to the runway for takeoff.

8.________________ if the tower uses the word "hold" prior to clearing you for 		takeoff.

9. ___________ is the reason the throttle is full in for takeoff and climbs.

10.____________The term for the direction at right angle to your takeoff

direction.

11. ________ is the  altitude to make your first turn on departure at CCR.

12. ________ is the altitude not to leave before you turn base at CCR.

13,_____________ _____ is used before any power reduction.

14.________ is the count to get 10 degrees of flap before checking flap 			indicator

15._____ _______ is the only solution if the approach is low

16.____ kts is used to get down from a high approach after full flaps and 			power is off.

17.___ ________ is the solution to any touchdown not to be made in the first 			one-third of the runway.

18.__ _____ are used for landing if there is a strong crosswind of 18 kts.

19. __ _____or 2500' is the required distance before tower frequency can be 			changed on departure at CCR.

20.________ must be obtained before making initial radio contact with any 				airport.

21.________ is the emergency radio frequency.

22.________ is the standard VFR transponder code.

23.________ is the emergency transponder code.

24.________ at _________is the instructor assigned minimum altitude and 				airspeed for ground reference practice.

25.___ student flights are to be made without the knowledge and approval of 				the instructor.

26. _____________and ____ _______ are the required documents a student pilot 				must have in possession for any solo flight.

27. Every ______  ____ I must have my log book re-endorsed for solo flight.

28. My instructor has placed the following limits for local solo flight:

Ceiling of _____________

Visibility of _________ miles

Wind velocity of _______ _______

29. The aircraft to the _________has the right of way when aircraft are

converging.

30. I should turn _________if approaching another aircraft head on.

31. An aircraft is at my altitude when it is on the ______________.

32.___ ______________ information is required by the pilot prior to a flight.

33.____________ aircraft always pass on the right.  For  this reason initial 			clearing turns are to the left.

34.________ ____ light from the tower means to move clear of runway.

35.___________ arc of the airspeed indicator should not be flown when in 			turbulence.

36.______ engine power, wing lift and propeller thrust is available when 			there is a high density altitude.

37._______ is the arc color on the airspeed indicator showing the flap 				operating range.

38._____ degrees is the maximum bank to be used in the pattern.

39.____________ is used to improve engine performance and reduce fuel 				consumption at altitude.

40._______ is the word normally omitted when giving distance

41.____________ is the first item on an emergency checklist.

42._____________ wind directions are given by the ATIS to conform with 				runway headings and numbers.

43.__________ indicates that a runway is closed to all but emergencies.

44.________ give an aircraft an increased angle of descent and improve 				landing accuracy.

45._________ is used to keep the tail behind the nose of the aircraft.

46.____________ effect causes control effects to be greatly exaggerated when 			landing.

47._____________ airspeed makes it easier to determine if high or low on 			approach.

48._________ is your first radio word in an emergency.

49. _________ ___________ can cause hard landings on hot days.

50.__ _______ is the reason an aircraft turns to the left when in a climb.

51._____ is a beacon activated when an airplane crashes

52._____________ is always used to check accuracy of heading indicator.

53.________ is responsible for safe operation of an aircraft.

54.___to__ is the mixture ratio of air and fuel that gives best engine 				operation

.

55._______ is the nation wide frequency for Flight Watch.

56. _______ is the altitude AGL that an airplane can proceed in any direction 		without regard to the hemispheric rule..

BRIEFLY EXPLAIN:

1. Solo flight  __________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

2. Run-up

__________________________________________________________________________

___________________________________________________________________________

3.Dutch Roll

(why?)

________________________________________________________________________

________________________________________________________________________

4. See and be seen

________________________________________________________________________

____________________________________________________________________________

5. Slip

________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

6. Go around  ________________________________________________________________________

____________________________________________________________________________

7. Short approach

____________________________________________________________________________

____________________________________________________________________________

8. Stalls

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

9. Airport Class D Airspace

________________________________________________________________________

________________________________________________________________________

10. Uncontrolled airport

____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

11. Pilot in command

____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

12. Visual reference to ground

_________________________________________________________________________

________________________________________________________________________

13. Night

___________________________________________________________________________

___________________________________________________________________________

________________________________________________________________________

14. Logbook endorsement ____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

15. Visual Flight Rules

___________________________________________________________________________

__________________________________________________________________________

________________________________________________________________________

16. ATC light signals

____________________________________________________________________________

__________________________________________________________________________

17. Right-of-way rules

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

18. Traffic pattern

____________________________________________________________________________

____________________________________________________________________________

__________________________________________________________________________

__________________________________________________________________________

19. Noise abatement procedures

____________________________________________________________________________

____________________________________________________________________________

__________________________________________________________________________

20. Wake Turbulence

____________________________________________________________________________

____________________________________________________________________________

__________________________________________________________________________

21. Transponder

____________________________________________________________________________

____________________________________________________________________________

__________________________________________________________________________

22. VASI

____________________________________________________________________________

_________________________________________________________________________

23. Clearance

____________________________________________________________________________

____________________________________________________________________________

____________________________________________________________________________

24. VFR fuel requirement

____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

25. Basic VFR weather minimums

____________________________________________________________________________

____________________________________________________________________________

________________________________________________________________________

26. If there is an aircraft on base while you are flying downwind how do you determine when to turn your base?

________________________________________________________________________

________________________________________________________________________

27.  Describe your flight procedure if you are in the pattern and are told that you are number 3 to land.

________________________________________________________________________

________________________________________________________________________

28.  Describe your landing procedure around a left pattern until clearing the runway if you have a 17 kt right cross wind.

_____________________________________________________________________

________________________________________________________________________

________________________________________________________________________

You are at 3000' and for some reason your engine fails.  Go through the steps of an emergency procedure.

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

30. Explain in your own words what you believe the limitations on a student pilot to be.

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

________________________________________________________________________

31. Explain the need and application of carburetor heat. ___________________________________________________________________

________________________________________________________________________

________________________________________________________________________

Part 91  General Operating and Flight Rules

32. What are minimum safe altitudes: General?  91.119

33. What are VFR fuel requirements? 91.151

34. What is the generalization that applies to flying regarding use of 	alcohol or drugs?  91.17

35. What are the solo flight requirements for a student pilot? 61.87 a., b., 	c., d., m.

36. Has your instruction met these requirements? 61.87 a through m.

37. If not; why not?

38. Does your logbook have the required endorsements?	 61.87 m.

39. If not; why not?

40. Under what conditions might a student pilot fly into Class B  airspace 	in Hawaii but NOT into the San Francisco's Class B? 61.95

41. What papers are a student pilot required to have with him while perating 	an aircraft?

42. When can a student fly between airports?  61.87 k. and 61.95 b. 1. 2. 3.

43. What is the 90 day student endorsement requirement?  61.95 b. 2.

44. How are the radios of the C-172 different from that of the C-150?

45, How does the Power/trim/flap operation of the C-172 differ from the C-	150?

46. What are Safety Belt requirements applicable to Student Pilots?  91.107

47.  What does the FAA consider to be careless or reckless flying?

48. Over populated areas I must fly with what clearances and altitudes?

49. Below 10,000 feet what are the required cloud clearances and visibility?

50. Where must aircraft fly in passing in vicinity of other aircraft at the 	same altitude?

51.  What endorsements are required before I can fly solo?

# Student Endorsements (Instructor)

In addition to the above, the student logbook must be endorsed (Per AC 61-65C  appendix 1 )

NOTE: The general requirements for each solo flight endorsement seem to 		be as follows:

Verified identification and qualification of instructor.

Verified identification and eligibility of student.

Student certificate and medical number.

Is student recreational or private candidate?

Ground instruction in applicable FARs.

Flight instruction in applicable FARs.

Instruction and tests in make and model.

Any required prior endorsements verified.

Statement of competency and safety.

Required 90 day date limits.

Specific instructor restrictions or limitations.

NOTE; The general requirements for ground instruction seem to be as 		 follows:

Verified identification and qualification of instructor.

Verified identification of student.

Is student recreational or private candidate?

Ground instruction in applicable FARs.

# Student Pilot Endorsements (Instructor)

1.  FAR 61.87(b) pre-solo aeronautical knowledge

(name) has satisfactorily completed a presolo written examination demonstrating knowledge of the portions of FAR Parts 61 and 91 applicable to student pilots, and the flight characteristics and operational limitations for a (make and model) aircraft.

Different Make and Model (Sign Certificate and logbook)

I certify that ____#_____ meets the requirements of FAR action 61.87 (m) 1 through 3 and is competent to conduct solo flight in a ___________

REQUIRES WRITTEN TEST AND RESULTS ENTRY

2.  FAR 61.87(c) pre-solo flight training

"I have given (name) the flight instruction required by FAR 61.87(c) in a (make and model).  He has demonstrated proficiency in the applicable maneuvers and procedures listed in FAR Part 61.87(d), and 61.87(e) and is competent to make safe solo flights in a (make and model) airplane.

3. FAR 61.87(m) 90 day solo along with license

I have given (name) the instruction required by FAR 61.87(M).  He as met the requirements of FAR 61.87(m) and is competent to make safe solo flights in a (make and model) aircraft.

Any instructor making this endorsement must

1. Have given student instruction in make and model

2. Found student to meet requirements of FAR

61.87(d) through (e)

3. Found student competent to make a safe solo flight

4.  FAR 61.93(b) Solo landings and takeoffs within 25 miles.

I have flown with (name) and find him competent and proficient to practice landings and takeoffs at the (name) airport.  Landings and takeoffs at (name) are authorized subject to the following conditions.

(visibility, winds, duration) Contingent on 90 day solo endorsement.

5.  FAR 61.(d)(2)(i) each solo cross-country

I have reviewed the preflight planning and preparations of (name) and attest that he is prepared to make the solo flight safely under the known circumstances for (location) to destination via (route) with landings at (airports) in a (make and model) aircraft on (date)  Contingent on 90 day solo endorsement.

61.93 Cross-country student pilot requirements:

The student must have received and have logged instruction in:

Requirements of 61.87(c)(e)

(c)(1)(i) Charts, dead reckoning, pilotage,

compass

(ii) Aircraft performance, all aspects of

weather

(iii) Emergencies, adverse weather, off-

airport options

(iv) Arrivals, departures, patterns,

collision avoidance, and wake

turbulence factors

(v) Terrain as affecting flight

operations

(vi) System operations in aircraft

(2)(i) Short and soft field takeoff/landings

and crosswind procedures

(ii) Best rate and angle takeoff

(iii) Four basics in instrument flight and

use of radio and radar.

(iv) Radio procedures

(v)  Night flying procedures

Private Pilots--40 hours of flight time

20 hours of dual (at least) to include

5 hours of cross country

3 hours of night, including

One cross-country of over 100 NM.

10 takeoffs and ten landings.

3 hours of instrument flight rtraining

3 hours in preparation for the practical test (60 day limit)

10 hours of solo, including

3 hours of soloc cross-country

One x-country flight of at least 150 NM. with three stops and one segment of at least 50 NM between points.

Three takeoffs and three landings at controlled airport.

6.  FAR 61.93(d)(2)(ii) cross-countries not over 50 miles.

I have given (name) flight instruction in both direction over the route between (airport) and (airport), including takeoffs and landings at the airports to be used, and find him competent to conduct repeated solo flight over that route, subject to the following conditions...Contingent on 90 day solo endorsement.

## Other Endorsements (Instructor)

FAR 61.31(e) High performance airplanes (over 200 hp)

I certify that I have given flight instruction in a high performance airplane to (name) holder of pilot certificate (number) and consider him competent to act a PIC in high performance airplanes.

FAR 61.31 Complex aircraft (retractable gear and controllable pitch propeller.

I Certify that I have given flight instruction in a complex aircraft to (name) holder of pilot certificate (number) and consider him competent to act as PIC in a complex aircraft.

FAR 61.39(a)(5) Completion of prerequisites for a practical test

I have given (name), flight instruction in preparation for a (type test) within the preceding 60 days and find him competent to pass the test and to have satisfactory knowledge of the subject areas in which the applicant was shown to be deficient by his airman written test.

FAR 61.47 Retesting within 30 days after first failure

I have given (name) additional (type) instruction and find him competent to pass the (type) test.

Spin Knowledge

FAR 61,105 (6)

I have given _________#____ the flight training and ground instruction in stall awareness, spin entry, spins and spin recovery techniques as required by FAR 61.105 (6) and find him competent in this area. (See SPINS)

Actual spin not required but recommended.

Spin awareness

I certify that I have given _____holder of pilot certificate # ________ ground and flight instruction  in stall awareness, spin entry, spins and spin recovery technique and find that he/she meets the knowledge requirements required under FAR 61....see above.                                            Signed...

# Second Solo

The first solo is deceptive in that it overcomes much of the tension related to, "can I do it?"  The next few lessons need to be tightly controlled since the confidence level is likely to exceed the competence level.  Proudly-poor performance and attitudes require the instructor to be more demanding and critical in order to re-establish attention.

Ask questions that require explanation and insight into systems and procedures.  Supervise the flight preliminaries and carefully review the requirements of the next few flights.  Make it clear that all solo flights require instructor approval.

Once way of retrieving student attention is to introduce some advanced landing and takeoffs.  Do this to establish a line for the student to see how much more there is to know.  Review ground reference and send him out to practice.  Make it so that each flight has a required series of maneuvers with PTS levels required.

# Solo Instruction

When you fly solo you are self-instructing to prevent self-destructing.  All at once you realize that in spite of all you have learned there is even more you don't know and need to know.  Solo flight is truly an eye opener.  Every flight is a new learning experience regardless of your pilot time.

Every flight begins by thinking about everything related to the flight.  Get your priorities in order before going to the airport.  Make the checks of yourself, weather, scheduling and instructor approval required.

Think through the flight by looking over the airport guide, frequencies, checkpoints, what you will say when, course, altitude and alternatives.

Think through similar former flights and think/plan to anticipate events before they happen.  Relive previous mistakes so that they don't become habitual.  Getting away with a mistake is a sure way to have it grow into a bad habit.  Don't 'instruct' yourself into accepting poor performance.  Get the training you are paying for, even when solo.  Every solo flight is a checkride where you are the pilot, instructor, and examiner.  Tape every flight and save the tape.

Every solo flight will have the good and the bad.  You will know some and be deficient in part.  While you may not know what you don't know, talk into the tape as you feel insecure, uncertain, or concerned.  Play the tape back immediately after the flight and again ten years later.  You will learn something new on each playback.  Feel free to call your instructor regarding

your flights.  Cover everything, the good and the bad.

Developing a self improvement flying program as a solo student pilot should carry over as a practice into your flying career.  Some pilots, once they have acquired a license and a few hours seem to quit learning.  It almost as though they have taken a dose of medicine that prevents any further accumulation of knowledge and skill.  Don’t let it happen.

A new pilot today is  entering a world where airplanes are safer, easier to fly, navigate themselves, and often proceed with the pilot only as a monitor of what is happening.  It’s a new world.  If you are not careful you will find some vital habits atrophying such as looking out the window, knowing where you are, seeing traffic, and even flying the airplane.  Watch out or the fun will be gone, too.

# After Solo-What?

Unless you deliberately write out your standards you are apt to be willing to accept less than your best.  Do not make too many solo flights as your own instructor without a phase check by the instructor.  Bad habits are

quick to arise and difficult to eliminate.  Make a list of the skills you

dislike, avoid, or feel insecure doing.  Work on how smoothly you can make

a transitions from one configuration or airspeed to another.

For one thing we will start working out fuel consumption figures for 85K. Top off tanks after landing.  Up to now we have been burning fuel; now we will start managing fuel.  As part of every flight we will fill the tanks up to the departure level and determine consumption.  Then we will take to POH and compute fuel used for taxiing, runup, cruise, and descent.  After doing this a couple of time we want to start estimating (not guessing) fuel burn for our flights and then comparing our estimate with actual.  Make a fuel log for each flight with time for each power setting in every flight regime.  Keep the fights in sequence and you will begin to see a pattern develop.

When solo you are the instructor who much pre-plan the elements that

you expect to accomplish during your solo flights.  Write out the lesson as you expect to fly it.  Airspeed control, altitude parameters, heading variations are all a part of your program.  Slow flight, stalls, steep turns, ground reference, radio procedures and all sorts of arrivals, landings and go-arounds are included.  Locate emergency fields but don’t practice emergencies.  Spiral descents should be planned to come out over a particular point at 1000’.

When you get back to the airport study the area chart and the sectional.

Try to find questions to ask the instructor.  Every new issue has significant

changes.  Read at least one chapter of the POH.  Do a weight and balance sequence by varying the passenger load so that less than full fuel will be required.

## The life of your newly acquired skills is limited by the frequency

with which you provide reinforcement.  How often you fly is more important

than the duration.  If you go through the entire regime of a dozen touch-and-

goes will not provide the skill reinforcement of an inter airport flight.

Every flight should be a skills reinforcement and development flight.

Before you get into the aircraft write out the tolerances you expect to meet.

Select an altitude tolerance of + 20 feet, a heading variation of + 5 degrees,

and + 5 knots of airspeed over ever increasing lengths of time.  try starting at two minutes in climbs, level and descents.  Stick these parameters on an

oversized print-out on the panel.  When you bust a parameter, start over.

Every skill of taxiing should be within one foot of a real or imaginary taxi line,  Every stop should be + 10 degrees of selected heading and + 1 foot

of a selected line.  Takeoff should rotate to attitude that allows  liftoff   + 3 knots of recommended.   Wind correction is applied immediately + 10 degrees margin for parallel runway.  Runway check is made at 300 feet.  Within

100 feet after takeoff aircraft is at Vy + 3 knots and trimmed hands off on heading + 5 degrees.  Ball centered throughout.

Initial level off is anticipated and acquired within + 50 feet and

corrected for hands off within one minute.   Heading throughout level-off is

+ 5 degrees.  For VOR tracking, fly a pre-selected heading and fly it + 0

tolerance long enough to resolve next required heading.  Altitude + 20 feet.

Descents to pattern altitudes should begin early enough to allow retention of power at a reduced level.  Base you selection of when to

initiate your descent on time.  The time will vary with your groundspeed so

always figure in the effect of wind.  Use the VSI.

Landings are performed with pattern altitude + 20 feet and all speeds

past the numbers + 3 knots and correcting.  Trim setting always for hands-off.

All power changes are reductions, all yoke movements are back.  Touch down is

always in the first third of the runway or for accuracy +200 past a point selected abeam the numbers.

# CCR to Local Airports.

The first post-solo flight will be to Napa.  Before departure we will review all radio procedures and visual check points to be used in both directions.  We will make a full stop landing at Napa and a taxi back.

On departing Napa I will advise ATC that you will be coming back in about 30-minutes to do a repeat as a solo student pilot.

When we get back to CCR, we will taxi to the base of the tower and I will do the required paperwork and send you on your way.  I will wait in the tower until your return to the East Ramp.

The process will be repeated to Livermore, Oakland and Rio Vista.

At the end of this series you should have acquired knowledge of the area, its visual checkpoints, airports, and airport procedures sufficient to make you competent for repeated solo flights.  The next training flight will be a cross-country.

# Question

1. What are some of the skills acquired and used when flying between 	airports that are not available when just doing the touch-and-go?

2. What papers must a student pilot have in his possession during solo 	flight?

3. Can you name some of the factors that could prevent a student first solo?

4. Is anything missing in your flight instruction or knowledge requirements?

5. What are the six areas of essential student flight knowledge?

6. What are the six areas of essential student aircraft knowledge?

7. How long is a solo endorsement good for?

8. What two types of instructor endorsements are required on your student 	license?

9. What is required before you can take the Private Pilot Written?

10. What two types of instruction are required before you can be soloed 	between airports?

11. What reference to the ground is required for all student solo 	flights?

12. What is required if you fail a written test?

13. Why is spin awareness required but not actual spins?

Answer

1. Judgment skills related to radio, arrival, departure, and area 	familiarization are improved by flying between airports.

2. Current license, medical, logbook, aircraft papers, sectional and VFR 	area chart.

3. Not having required papers, proper endorsements, reasonable weather/wind, 	health, airworthy aircraft, or confidence in ability to perform 	safely.

4. You don't know what you don't know.

5. Essential knowledge areas are: Aircraft operation, applicable FARs, radio 	procedures, emergency procedures, safety procedures, and 	instructor/club limitations.

6. Essential knowledge area of aircraft knowledge are: Starting, stopping, 	taxiing, flight control, systems, papers, and performance figures.

7. 90 days in the logbook or to the expiration of the student license 	whichever occurs sooner.

8.  Solo in type and cross-country in both logbook and license will complete 	your student license endorsements.

9. Instructor's endorsement of competence to pass test is required before 	you can take the written test..

10. Before you can fly between airports you must have both ground 	instruction and flight instruction in operations and procedures for 	both airports. Endorsements to certify these are required.

11.  No flight is to be made without reference to the ground.  A student is 	not allowed to fly above any cloud condition that prevents 	sight of 	the ground.

12.  The right to retake the test requires a 30 day wait for additional 	study and a new endorsement by an instructor.  (Can't find this in 	FARs)

13.  A pilot must be aware of the flight situations that lead to the stall

and spin.  Recovery from the stall is possible.  The accidental spin 	itself is most likely to occur so close to the ground that no recovery 	is possible.

Question

1. What part of the FARs regarding seat belts do not apply to solo student 	pilots?

2. How do you determine that the fuel shutoff valve is in the off position 	during the exterior preflight?

3. What visual representation on the sectional gives the dimensions of the 	Class D airspace?

4. What makes taxiing seem so difficult?

5. What happens if I dive to get down to a runway?

6. Use your hand to indicate an aircraft's attitude in a power off glide.

7. When can you have too much gas in an airplane?

8. After my power is off and  my flaps full what can I do go get down more 	steeply?

9. How will ATC clear you into Class C airspace?

10. That ATC clearances normally never use the word "cleared" or "clear"?

Answer

1. The part about advising passengers.

2. The sump strainer will not drain.

3. Class D is a communication requirement of an operating control

tower.  It is 4.1 nautical miles in radius from the center of the 	airport up to 2500' AGL (above ground level).  It is shown on the 	sectional by blue dashed circle with extensions up to two miles.

4. The last thing you learn to do well is to taxi.  The classic unlearning 	problem.  You do not drive an airplane on the ground.  It is a 	kiddycar.  Always taxi a though you were in a strong wind.  You will 	need the practice.

5. Diving for the runway may make you miss it.  Increasing approach speeds 	will cause the aircraft to cover more ground getting to the runway and 	will increase the ground effect once at the runway.  To get down, slow 	up.

6. Hold your hand as though the nose were higher than the tail.

7. When you're on fire any gasoline will be too much.

8. Raising the nose can bring you down faster.  Any speed below the best 	glide speed will increase the rate of descent, up to the point of 	stall.  The slower you go with the nose up, the faster you go down.  	At stall you fall down.  Power, without any increase in pitch attitude 	is the only correction to break such a descent.  You are behind the 	power curve when the only way to increase airspeed is by lowering the 	nose.  You are out of useful power.

9. ATC does not give clearances into Class C airspace.  You may be advised 	to remain clear.  You will be addressed by N number or advised of 	radar contact.  The establishment of communications is all that is 	required for entry into an ARSA.  Entry into Class B airspace requires 	a clearance and is not available to students in the Bay area.

10. Taxiing instructions, except where to countermand a taxi restriction, do 	not normally use the term cleared..

Question

1. Under what three circumstances can flight at 4,678 feet be legal 	regardless of direction?

2. What ATC clearance does not normally require acknowledgment.

3. What airport arrival does not have a 2 mile report?

4. What is the last pilot skill to be mastered?

5. What flying operations requires mastery of the Dutch roll?

6. When do altitude limitations regarding flight over residential areas 	not apply?  (3 situations)

7. How is adding power to a airplane different than to a car?

8. How does the horizontal stabilizer affect flight?

9. What action, at altitude, has been known to extend the power off 	emergency glide distance up to 20 percent?

10. What factor must be known or estimated to determine the penetration 	glide speed?

11. Why use flaps for landing?

Answer

1. Climbing, descending, or if that altitude is within 3000' of the ground.

2. "...cleared for takeoff..."  It is not wrong to acknowledge.

3.  45 degree entry into downwind does not need to be requested nor does it 	have a 2-mile report.

4. Taxiing is the most difficult skill to learn and is never mastered.

5. Crosswind landings require all the skills used to perform the Dutch roll.

6. Takeoff, intention to land, emergency situation allow passage within 500 	feet of people.  Making a low approach while not intending to land has 	been construed by the FAA as a violation.

7. Just adding power causes you to go slower.  Try it.  Set up, hands off, 	level flight at a low cruise speed.  Hold heading with rudder while 	applying full power

8. Low pressure in flight is on top of the wing and below the horizontal 	stabilizer.  This means in a conventional aircraft the tail is pulled 	or pushed down as a lever to keep the nose of the aircraft up.  A PTS 	question.

9. At engine failure, slowing the aircraft to near stall will stop the 	propeller from windmilling.  Use any excess airspeed to gain altitude.  	This will increase glide range.  Using the starter to level the 	propeller will reduce ground contact damage.

10. When attempting to glide against a wind it is necessary to add from

1/3 to 1/2 of the headwind component to the best glide speed to gain 	optimum penetration glide range.

11. Flaps are not designed to slow the plane.  Approach landing speeds

are essentially the same for with and without flaps.  Flaps create 	drag that allows the nose to be lowered without increasing airspeed.  	Flaps, thus, improve your over the nose visibility.  The improved 	visibility allows a more accurate determination of whether high or low 	on final.  The steeper descent allows greater accuracy on the 	approach.

Question

1. What can you do to increase your radio's sensitivity for long 	distance 	reception?

2. What is the sequence of three items that can be used to correct a 	high 	landing approach?

3. What is the best correction for being low on a landing approach that 	does not require a go-around?

4. What misconceptions can get a pilot into trouble?

5. What climatic factor can make a final approach to landing very steep?

6. What instinctive reaction makes this steep approach dangerous.

7. What makes a landing in "calm winds" more difficult?

8. Why is the use of trim often confusing?

9. Why are climbs made at full power in fixed pitch aircraft?

10. Four aircraft instruments will work as well off the plane?.

Answer

1.  Turning the squelch knob (if available) to the far right will increase 	the sensitivity of the radio.  This will also increase the unwanted 	'noise'.

2.  First make sure that the fullest application of flaps is being used

as allowed by crosswind conditions; secondly, pull off power; thirdly, 	raise the nose and trim to Vso according to aircraft weight.

3.  Apply full power while holding the yoke forward to keep a constant 	approach speed.  After 10 to 30 seconds reduce back to normal approach 	power.

4.  Fear of height opposes fear of hitting the ground.  At takeoff, keep the 	runway in view because we should see where we are going.  More speed 	 	the runway gives a better feeling of control. If 60 kts is a safe 	approach speed, 80 kts is even safer. The nice flat smooth landing is 	best because it feels so good.  That's the way you land a toy airplane.  	If a moderate bank is safe, them a shallow bank is safer 	and feels 	better.  The feeling that hills and water, in and of 	themselves 	constitute flight hazards to be flown around.  Since the movement of 	the trim can be confusing don't bother with the facts of its use.  This 		plane can be flown without using trim.

5. A very strong head wind.  Be a bit faster in doing the procedures for 	landing.

6. The pilot ignores the required airspeed and incorrectly raises the nose 	to get a desired attitude and an undesired greatly reduced airspeed.

7. The "calm wind" may be a tail wind on high final an a crosswind at the 	runway.  Be alert.  Be a bit slower doing the procedures for landing 	and make rudder/aileron applications immediately in the flare to stop 	drift.

8. The trim wheel is moved 'backwards'.  Moving it down, raises the nose.  	Think of your moving the trim as moving the tail of the aircraft.

9. Gas is added to the engine to make it cooler.  Full throttle opens jets 	to allow excess gasoline to cool engine during climb.  The gasoline 	will actually increase in octane during full power/pressure 	operations.

10. Thermometer, clock (non-electric) compass, ball (inclinometer)would  	also.

Question

1. How does a wing work?

2. Is the tail of an airplane a lifting surface?

3. Why do a full stall landing?

4. How do flaps help landings?

5. What's so different about a crosswind landing?

6. Do we look at the North end of the compass magnet when going North?

7. Why do we normally fly headings by using the heading indicator?

8. It is possible to have too altitude on a flight?

9. When might you turn the yoke to the left while turning the aircraft to 	the right?

Answer

1. The top of the wing lifts the airplane.  The pressure differential 	between the top and bottom curve (if any)  will cause a partial vacuum

over the top.  Lift is really the wing moving into the created vacuum 	space due to air pushing from below.  The weight of the air displaced 	must exceed the weight of the aircraft for it to climb.  When the 	weights are equal the aircraft is in level flight.  When the weight of 	the displaced air is less than the aircraft weight, the aircraft 	descends.

2. Yes, but it 'lifts' downward. The tail of a plane pushes down in flight.  	This is a design and weight and balance contradiction.  If the 	weight/balance is such that airspeed will not give elevator control. 	You will not be able to attain or maintain the nose attitude required 	for landing.  A good reason to know your minimum controllable 	airspeeds.

3. A good landing is made by trying not to land.  Simple answer but correct.  	Only the airplane should know when it is really through flying.  The 	pilot should keep the airplane in the air as long as possible.

4. A steep landing approach gives better aim than a shallow one.  Flaps make 	a steeper approach possible.  Draw a shallow approach and a steep 	approach.  Now make an error angle on each approach line of the same 	size.  Note the difference in runway required.

5. Cross wind landing touchdowns are correctly made on one wheel.  Excess 	side loads are very heavy during one wheel landings.  Be sure to 	eliminate any drift prior to touchdown.  Advise passengers.

6. Yes, the aircraft compass has two magnets balanced on a point.  However, 	the letter N is placed at the compass' magnetic South end. The South 	end of the magnets is attracted to the earth's North magnetic pole.  	While going North and turning right the compass goes left.  Make 	standard rate turns only, when flying the compass.  90 degrees takes 	30 seconds.

7. It's easier.  To go to a heading on the compass you turn away from it.  	Practice flying compass headings and timed turns to other headings to 	appreciate the mental gymnastics required.

8. Only in one situation, when you're on fire.

9. When taxiing you may correctly turn the yoke to the left while turning 	right with the aircraft.  1. When going from directly into the wind 	and turning the aircraft to the right.  2.  When going directly away 	from the wind and turning to the right.

Questions

1. Why are master switches often in two parts?

2. What four aircraft instruments work just as well when not on the 	aircraft?

3. What are some of the skills acquired and used when flying between 	airports that are not available when just doing the touch-and-go?

4. In a properly rigged aircraft where will the yoke and ailerons be 	positioned in a stabilized 30 degree bank?

5. What papers must a student pilot have in his possession during solo 	flight?

6. Can you name some of the factors that could prevent a student first 	solo?

7. Is anything missing in your flight instruction or knowledge 	requirements?

8. What are the six essential areas of student flight knowledge?

9. What are the six essential areas of student aircraft knowledge?

10.  How long is a solo endorsement good for?

11.  What two types of instructor endorsements are required on your 	student license?

12.   What is required before you can take the Private Pilot Written?

Answer

1. One side give battery power to all systems.  The other side activates the 	alternator field coil which enables the alternator to supply power to 	all systems as well as to recharge the battery.  The alternator field 	coil should be turned off in the event of alternator failure or 	malfunction so as to extend battery life.

2. Compass, thermometer, wind-up clock, inclinometer (ball)

3. Judgment skills related to radio, arrival, departure, .

4. Level with cockpit.

5. Current license, medical, logbook, aircraft papers, sectional.

6. Not having required papers, endorsements, weather/wind, health,  aircraft, 	confidence.

7. You don't know what you don't know.

8. Aircraft operation, applicable FARs, radio procedures, emergency 	procedures, safety procedures, and instructor/club limitations.

9. Starting, stopping, taxiing, flight control, systems, papers.

10. 90 days in the logbook and to the expiration of the student license.

11. Solo in type and cross-country.

12. Instructor's endorsement of competence to pass test.

Question

1. What is the last pilot skill to be mastered?

2. What flying operations requires mastery of the Dutch roll?

3. When do altitude limitations regarding flight over residential areas 	not apply?  (3 situations)

4. How is adding power to a airplane different than to a car?

5. How does the horizontal stabilizer affect flight?

6. What action, at altitude, has been known to extend the power off 	emergency glide distance up to 20 percent?

7. What factor must be known or estimated to determine the penetration 	glide 	speed?

8. Why use flaps for landing?

9. What can you do to increase your radio's sensitivity for long 	distance 	reception?

10. What is the sequence of three items that can be used to correct a 	high 	landing approach?

Answer

1. Taxiing

2. Crosswind landings

3. Takeoff, intention to land, emergency

4. Just adding power causes you to go slower.  Try it.  Set up, hands off, 	level flight at a low cruise speed.  Hold heading with rudder while 	applying full power

5. Low pressure in flight is on top of the wing and below the horizontal 	stabilizer.  This means in a conventional aircraft the tail is 	pulled/pushed down as a lever to keep the nose of the aircraft up.  	A PTS question.

6. At engine failure, slowing the aircraft to near stall will stop the 	propeller from windmilling.  Use any excess airspeed to gain altitude.  	This will increase glide range.  Using the starter to level the 	propeller will reduce ground contact damage.

7. When attempting to glide against a wind it is necessary to add from

1/3 to 1/2 of the headwind component to the best glide speed to gain 	optimum penetration glide range.

8. Flaps are not designed to slow the plane.  Approach landing speeds

are essentially the same for with and without flaps.  Flaps create 	drag that allows the nose to be lowered without increasing 	airspeed.  	Flaps, thus, improve your over the nose visibility.  The improved 	visibility allows a more accurate determination of 	whether high or 	low on final.  The steeper descent allows greater accuracy on the 	approach.

9.  Turning the squelch knob (if available) to the far right will increase the 	sensitivity of the radio.  This will also increase the unwanted 'noise'.

10.  First make sure that the fullest application of flaps is being used

as allowed by crosswind conditions; secondly, pull off power; thirdly, 	raise the nose and trim to Vso according to aircraft weight.

Question

1. Is the tail of an airplane a lifting surface?

2. Why do a full stall landing?

3. How do flaps help landings?

4. What's so different about a crosswind landing?

5. Do we look at the North end of the compass magnet when going North?

6. Why do we normally fly headings by using the heading indicator?

7. It is possible to have too much fuel or altitude on a flight?

8. When might you turn the yoke to the left while turning the aircraft 	to the 	right?

9. How can you get ready for an emergency landing.

Answer

1. Yes, but it 'lifts' downward. The tail of a plane pushes down in flight.  	This is a design and weight and balance contradiction.  If the 	weight/balance is such that airspeed will not give elevator control. You 	will not be able to attain or maintain the nose attitude required for 	landing.  A good reason to know your minimum controllable airspeeds.

2. A good landing is made by trying not to land.  Simple answer but correct.  	Only the airplane should know when it is really through flying.  The 	pilot should keep the airplane in the air as long as 	possible.

3. A steep landing approach gives better aim than a shallow one.  Flaps make a 	steeper approach possible.  Draw a shallow approach and a steep 	approach.  Now make an error angle on each approach line of the same 	size.  Note the difference in runway required.

4. Cross wind landing touchdowns are correctly made on one wheel.  Excess side 	loads are very heavy during one wheel landings.  Be sure to eliminate 	any drift prior to touchdown.  Advise passengers.

5. Yes, the aircraft compass has two magnets balanced on a point.  	However, the letter N is placed at the compass' magnetic South end. The 	South end of the magnets is attracted to the earth's 	North magnetic 	pole.  While going North and turning right the compass goes left.  Make 	standard rate turns only when flying the compass.  90 degrees takes 30 	seconds.

6. It's easier.  To go to a heading on the compass you turn away from it.  	Practice flying compass headings and timed turns to other headings to 	appreciate the mental gymnastics required.

7. Only in one situation, when you're on fire.

8. When taxiing you may correctly turn the yoke to the left while turning 	right with the aircraft.  1. When going from directly into 	the wind and 	turning the aircraft to the right.  2.  When going directly away from 	the wind and turning to the right.

9. You will never be ready for an emergency landing.  The decisions leading to 	an accident are usually accumulated from bad to worse.  Begin by 	circling your home.

Question

1. How can wet air weigh less than dry air?

2. Can an aircraft's gross weight for takeoff be greater than its legal

landing weight?

3. How reliable is the functioning of the ailerons?

4. Why is right rudder pressure used more often than left?

5. Do we ever use left ailerons in a right turn?

6. What determines a constant approach angle?

7. On engine failure, what do you want to lose first?

8. What is meant by anticipatory rudder?

9. What will cause you to lose confidence in your flying?

10. What happens when we get behind the power curve?

Answer

1. Wet air is LIGHTER than dry air.  The moisture forces the air 	molecules 	further apart thus decreasing overall weight.

2. Yes, an airplane may be too heavy to safely or legally land.  Excess fuel.

3. Turning the aileron to raise the wing doesn't always work.  If a wing 	is stalled, the application of up aileron usually aggravates the 	stalled condition and does not raise the wing.

4. Right rudder is often used in a climbing left turn.  Once into a climbing 	left turn, either no left or some right rudder will be required to 	center the ball.  P-factor is the reason.

5. Left aileron is often used in a right turn.  At high angles of attack 	the adverse yaw of the opposite aileron is required to coordinate 	the turn against other aerodynamic effects.

6. A constant landing approach angle is determined by airspeed.  Airplanes 	like automobiles have distinct speed sounds, sensations, and 	perceptions.  The more constant your airspeeds the better you will 	recognize these distinctions and any variations.

7. An airplane glides best with its nose up.  On engine failure, if you don't 	hold the nose up you will mostly lose altitude not airspeed.  With the 	nose up you will lose airspeed before you lose altitude.  Take any 	excess airspeed over best glide and change it into altitude.

8. Pilots walk funny because of the rudder.  Increases as well as decreases of 	power, climbs and descent, and in and out of turns require rudder.  	Usually anticipatory right rudder.  Anticipatory right rudder is used 	when entering right turns and when coming out of left turns.

9. When you become confident, be three times as careful.  The surest way to 	lose excess confidence is to have an accident.

10. Back elevator does not always give 'up'.  At very slow airspeeds or 	behind the power curve the aircraft will go slower with back elevator 	but not up.  The proper use of this knowledge makes for good landings.

Question

1. What causes ground effect?

2. What makes a pilot smooth while flying?

3. What is the advantage of a 30 degree bank?

4. Why are pilots poor gamblers?

5. What makes flying unique in a right-handed world?

6.  Why will the instructor make you make mistakes?

7. What is the purpose of the proficiency phase of instruction?

8. When can right be wrong?

9. How does density altitude affect a landing?

10. What makes an airplane fly?

Answer

1. Most poor landings occur because of failure to utilize ground effect.  	Ground effect is a function of a fixed wing span and a fixed airspeed.  	The better your fixed airspeed the better your use of ground effect and 	your anticipation of its effect.

2. Flying straight and level is one of the most difficult skills in flying.  	Smoothness is the proof of flying skill.  Smoothness can only be 	acquired by anticipation.  Anticipation means you are intellectually and 	physically ahead of the aircraft.

3. A well rigged airplane trimmed for a level 30 degree bank is stable in that 	bank with the yoke level to the cockpit.  This is one very good reason 	for making 30 degree banks while in the airport pattern.

4. Pilots under estimate the probability of a particular event happening to 	them.  This is why I never seem to win at Reno.

5. Flying is where left handers of the world get even.

6. You go to school to make mistakes.  If you must make mistakes make them new 	ones.  Make as many mistakes with the instructor as you can so you will 	acquire both the skills of avoidance and 	correction.

7. If the student thinks he ought to know it, he won't ask it.  Proficiency 	phase of flight instruction is not to impart information so much as 	finding out what is not known.

8. Right comes in several different forms.  Your first clearing turn is 	right when it is to the left not the right.  Your most likely error in 	doing stalls is failing to clear.  Right.  You turn the "wrong" way to 	make a compass read "right".

9. A landing attempt under high density altitude conditions is likely to be 	more 'firm' than otherwise because of the relative absence of ground 	effect normally encountered on 'dense' days.  The "plump down" can be 	reduced by flaring closer to the surface or by 	maintaining some power.  	Landing ground speed will be higher and 	roll-out longer.

10. Money.

Question

1. What is the best correction for being low on a landing approach that does 	not require a go-around?

2.  What misconceptions can get a pilot into trouble?

3. What climatic factor can make a final approach to landing very steep?

4. What instinctive reaction makes this steep approach dangerous.

5. What makes a landing in "calm winds" more difficult?

6. Why is the use of trim often confusing?

7. Why are climbs made at full power in fixed pitch aircraft?

8. What four aircraft instruments will work as well off the plane?.

9. How does a wing work?

Answer

1.  Apply full power while holding the yoke forward to keep a constant 	approach speed.  After 10 to 30 seconds reduce back to normal approach 	power.

2.  Fear of height opposes fear of hitting the ground.  At takeoff, keep the 	runway in view because we should see where we are going.  More speed 	along the runway gives a better feeling of control. If 60 kts is a safe 	approach speed, 80 kts is even safer. The nice flat smooth landing is 	best because it feels so good.  That's the way you land a toy airplane.  	If a moderate bank is safe, them a shallow bank is safer and feels 	better.  The feeling that hills and water, in and of themselves 	constitute flight hazards to be flown around.  Since the movement of the 	trim can be confusing don't bother with the facts of its use.  This 	plane can be flown without using trim.

3. A very strong head wind.  Be a bit faster in doing the procedures for 	landing.

4. The pilot ignores the required airspeed and incorrectly raises the nose to 	get a desired attitude and at the same time an undesired greatly reduced 	airspeed.

5. The "calm wind" may be a tail wind on high final an a crosswind at the 	runway.  Be alert.  Be a bit slower doing the procedures for landing and 	make rudder/aileron applications immediately in the flare to stop drift.

6. The trim wheel is moved 'backwards'.  Moving it down, raises the nose.  	Think of your moving the trim as moving the tail of the aircraft.

7. Gas is added to the engine to make it cooler.  Full throttle opens jets to 	allow excess gasoline to cool engine during climb.  The gasoline will 	actually increase in octane during full power/pressure operations.

8. Thermometer, clock (non-electric) compass, ball (inclinometer)would also.

9. The top of the wing lifts the airplane.  The pressure differential between 	the top and bottom curve (if any)  will cause a partial vacuum over the 	top.  Lift is really the wing moving into the 	created vacuum space due 	to air pushing from below.  The weight of 	the air displaced must 	exceed the weight of the aircraft for it to climb.  When the weights are 	equal the aircraft is in level flight.  When the weight of the displaced 	air is less than the aircraft weight, the aircraft descends.

Question

1. What makes taxiing seem so difficult?

2. What happens if I dive to get down to a runway?

3.  Use your hand to indicate an aircraft's attitude in a power off glide.

4.  When can you have too much gas in an airplane?

5. After my power is off and  my flaps full what can I do go get down more 	steeply?

6.  How will ATC clear you into Class C airspace?

7. That ATC clearances normally never use the word "cleared" or "clear"?

8. Under what three circumstances can flight at 4,678 feet be legal 	regardless of direction?

9. What ATC clearance does not normally require acknowledgment.

10. What airport arrival does not have a 2 mile report?

Answer

1. The last thing you learn to do well is to taxi.  The classic unlearning 	problem.  You do not drive an airplane on the ground.  It is a kiddycar.  	Always taxi a though you were in a strong wind.  You will need the 	practice.

2. Diving for the runway may make you miss it.  Increasing approach speeds 	will cause the aircraft to cover more ground getting to the runway and 	will increase the ground effect once at the runway.  To get down, slow 	up.

3. Nose high

4. When you're on fire.

5. Raising the nose can bring you down faster.  Any speed below the best glide 	speed will increase the rate of descent.  Up until the point of stall 	the slower you go with the nose up, the faster you go down.  At stall 	you fall down. Power, without any increase in pitch attitude is the only 	correction to break such a descent.  You are behind the power curve when 	the only way to increase airspeed is by lowering the nose.  You are out 	of useful power.

6. ATC does not give clearances into Class Charlie airspace.  You may be 	advised to remain clear.  You will be addressed by N number or advised 	of radar contact.  The establishment of communications is all that is 	required for entry into an ARSA.  Entry into Class Bravo airspace 	requires a clearance not available to students in the Bay Area.  A 	clearance is required to enter or leave Class Delta airspace.

7. Taxiing, except where to countermand a taxi restriction.

8. Climbing, descending, or if that altitude is within 3000' of the ground.

9. "...cleared for takeoff..."

10.  45 degree entry into downwind

Question

1.  What two types of instruction are required before you can be soloed 	between airports?

2. What reference to the ground is required for all student solo 	flights?

3. What is required if you fail a written test?

4. Why is spin awareness required but not actual spins?

5. How can the differences between adding power to an automobile and an 	airplane get you into trouble?

6. Why does the trim seem to work backwards?

7. What is meant by the stabilized approach?

8. What part of the FARs regarding seat belts do not apply to solo student 	pilots?

9. How do you determine that the fuel shutoff valve is in the off position 	during the exterior preflight?

10. What visual representation on the sectional gives the dimensions of the 	Class D airspace?

Answer

1. Ground instruction and flight instruction in operations and procedures.

2. No flight is to be made without reference to the ground.  A student 	is not 	allowed to fly above any cloud condition that precludes sight of the 	ground.

3. The right to retake the test requires a 30 day wait for additional study 	and a new endorsement by an instructor.

4. A pilot must be aware of the flight situations that lead to the stall and 	spin.  Recovery from the stall is possible.  The accidental spin itself 	is most likely to occur so close to the ground that no recovery is 	possible.

5. The common mental set is that adding power makes things go faster. 	Problems are created when this approach is applied to aircraft.  For a 	moment, the sense will be that the increase in noise is an increase in 	speed.  When power alone is applied to an airplane the nose will tend to 	rise and airspeed will decrease.  Forward yoke pressure must coincide 	with power application for speed to increase or even to be maintained.

6. Moving the trim wheel up lowers the nose; moving it down raises the 	nose.  	It is best to think of trim movement being done by grasping the tail of 	the aircraft.

7. The stabilized approach has several 'constants' airspeed, power setting, 	flap setting, trim setting, control pressures, descent rate, and pilot 	anticipation of what comes next.

8. The part about advising passengers.

9. The sump strainer will not drain.

10. Class D is a communication requirement of an operating control tower.  It 	is 4.1 nautical miles in radius from the center of the airport up to 	2500' AGL (above ground level).  It is shown on the sectional by blue 	dashed circle with extensions up to two miles.  CCR is 3.1 miles+ 	extension

Question

1. How can you get ready for an emergency landing.

2. How can wet air weigh less than dry air?

3. Can an aircraft's gross weight for takeoff be greater than its legal

landing weight?

4. How reliable is the functioning of the ailerons?

5. Why is right rudder pressure used more often than left?

6. Do we ever use left ailerons in a right turn?

7. What determines a constant approach angle?

8. On engine failure, what do you want to lose first?

9. What is meant by anticipatory rudder?

10. What will cause you to lose confidence in your flying?

11. What happens when we get behind the power curve?

Answer

1. You will never be ready for an emergency landing.  The decisions leading 	to an accident are usually accumulated bad to worse.  Begin by 	circling your home.

2. Wet air is LIGHTER than dry air.  The moisture forces the air molecules 	further apart thus decreasing overall weight.

3. Yes, an airplane may be too heavy to safely or legally land.  Excess fuel.

4. Turning the aileron to raise the wing doesn't always work.  If a wing is 	stalled, the application of up aileron usually aggravates the stalled 	condition and does not raise the wing.

5. Right rudder is often used in a climbing left turn.  Once into a climbing 	left turn, either no left or some right rudder will be required to 	center the ball.  P-factor is the reason.

6. Left aileron is often used in a right turn.  At high angles of attack the 	adverse yaw of the opposite aileron is required to coordinate the turn 	against other aerodynamic effects.

7. A constant landing approach angle is determined by airspeed.  Airplanes 	like automobiles have distinct speed sounds, sensations, and 	perceptions.  The more constant your airspeeds the better you will 	recognize these distinctions and any variations.

8. An airplane glides best with its nose up.  On engine failure, if you

don't hold the nose up you will mostly lose altitude not airspeed.  	With the nose up you will lose airspeed before you lose altitude.  	Take any excess airspeed over best glide and change it into altitude.

9. Pilots walk funny because of the rudder.  Increases as well as decreases 	of power, climbs and descent, and in and out of turns require rudder 	applications.  Usually anticipatory right rudder.  Anticipatory right 	rudder is used when entering right turns and when coming out of left 	turns.

10. When you become confident, be three times as careful.  The surest way to 	lose excess confidence is to have an accident.

11. Back elevator does not always give 'up'.  At very slow airspeeds or 	behind the power curve the aircraft will go slower with back elevator 	but not up.  The proper use of this knowledge makes for good landings.

Question

1. When is lift not opposite weight?

2. What does the rudder do?

3. What kinds of aircraft insurance should you carry?

4. What about accidents?

5. What is a clearance?

6. Why climb with wings level?

7. When can oil temperature be more significant than oil pressure?

8. What can I do to extend my glide range?

9. How reliable are forecast winds?

10. What is lost first at engine failure?

11. What do you do if the engine fails on 1/2 mile final?

12. What makes flying unique in a right-handed world?

Answer

1. Lift is not always opposite weight.  In a level turn some lift causes 	the turn.  Some angle of attack increase is required to prevent a 	descent when some lift is used to create the turn moment.

2. The rudder does not 'turn'.  Too much rudder, too soon is just as bad as 	too little, too late.  On approach the rudder is used to keep the nose 	parallel to the runway, the side slip is used then to maintain runway 	alignment.

3. Altitude, like gasoline, airspeed, and runway in front of you is 	insurance.  Very nice to have when you need it.

4. Accidents are the result of avoidable or preventable situations. Bad 	decisions are compounded by bad decisions.  Make your best initial 	decision and stick to it.

5. All tower clearances are only permissive.  The pilot has the final

authority as to the safety of aircraft operations even when permitted 	by ATC.  Don't be hesitant to ask for the most economical, for your, 	departure, taxi route or landing.  All ATC can say is, "NO!"

6. "dragging" a wing low can make 100' a minute climb rate loss.  For 	maximum rate of climb keep the wings level.  This is a performance 	safety factor in addition to Vy takeoff speed to get maximum altitudes 	as quickly as possible.

7. Oil temperature indicates engine temperature.  In the event of an oil 	pressure loss, check the oil temperature.  If temperature is normal, 	there is a possibility that the pressure loss is a gauge problem 	rather 	than an oil problem  If the pressure is down and the 	temperature is up, 	get down fast.

8. If altitude permits, glide range may be increased if propeller is 	stopped.  (See Mooney handbook)

9. No weather conforms to the weatherman's opinion of it.  The wind is never 	as forecast.

10. In the event of engine failure, excess airspeed is lost before altitude.  	Use any excess airspeed over best glide to acquire any altitude you 	can.  Altitude can be traded for airspeed.

11. At engine failure, removing flaps will extend your glide.  Instructor's 	demonstration at 400' with long runway.

12 Flying is where left handers of the world get even.

Question

1. What causes ground effect?

2. What makes a pilot smooth while flying?

3. What is the advantage of a 30 degree bank?

4. Why are pilots poor gamblers?

5.  Why will the instructor make you make mistakes?

6. What is the purpose of the proficiency phase of instruction?

7. How may kinds of right are there?

8. How does density altitude affect a landing?

9. What makes an airplane fly?

10. How are taxiways named?

Answer

1. Most poor landings occur because of failure to utilize ground effect.  	Ground effect is a function of a fixed wing span and a fixed airspeed.  	The better your fixed airspeed the better your use of ground effect 	and your anticipation of its effect.  A multiple effect of a wing 	close to the ground reduces drag, increases lift, decreases required 	power and allows mistakes in landing judgment to be corrected safely.

2. Flying straight and level is one of the most difficult skills in flying.  	Smoothness is the proof of flying skill.  Smoothness can only be 	acquired by anticipation.  Anticipation means you are intellectually 	and physically ahead of the aircraft.  Days without an horizon often 	make it very difficult for inexperienced pilot to level the airplane.

3. A well rigged airplane trimmed for a level 30 degree bank is stable in 	that bank with the yoke level to the cockpit.  This is one very good 	reason for making 30 degree banks while in the airport pattern.

4. Pilots under estimate the probability of a particular event happening to 	them.  This is why I never seem to win at Reno.

5. You go to school to make mistakes.  If you must make mistakes make them 	new ones.  Make as many mistakes with the instructor as you can so you 	will acquire both the skills of avoidance and correction.

6. If the student thinks he ought to know it, he won't ask it.  The 	proficiency phase of flight instruction is not to impart information 	so much as finding out what is not known.

7. Right comes in several different forms.  Your first clearing turn is 	right when it is to the left not the right.  Your most likely error in 	doing stalls is failing to clear.  Right.  You turn the "wrong" way to 	make a compass read "right".

8. A landing attempt under high density altitude conditions is likely to be 	more 'firm' than otherwise because of the relative absence of ground 	effect normally encountered on 'dense' days.  The "plump down" can be 	reduced by flaring closer to the surface or by maintaining some power.  	Landing ground speed will be higher and roll-out longer.

9. Money.

10. Taxiways are named alphabetically with phonetic words for the letters.  	Sometimes the letter/word may result in  easy recognition. i.e. GOLF may 	be by the golf course.  However, Hotel is never a building-see lessons

Question

1. Do we need to ask permission to use a special taxiway?

2. How do you use the yoke when taxiing?

3. What should suction and amperes read during runup?

4. Why is the master switch in two parts.

5. On landing we left the runway rather quickly.  What was turning the 	plane?

6. During the simulated emergency why not dive for the intended landing spot 	to assure reaching it?

7. Why check the ailerons right and left before moving?

Answer

1. ATC or Air Traffic Control does not give permission.  They issue 	CLEARANCES which basically say, "Go ahead but if anything goes wrong 	it your fault."  Oddly enough the word "cleared" is seldom used in 	taxi clearances.  A clearance to taxi clears you via any route as long 	as you do not cross an active runway.  However, if you wish a special 	route or are proceeding on an unusual route it helps to advise the 	controller.  Specific clearance is required to cross an active runway.

2. You move the controls so as to prevent any wind from overturning the 	plane.  Taxiing is the last flying skill acquired.  If you 'dive' away 	from winds behind you and 'climb' into winds coming toward you, the 	controls will be safely positioned.  Use your heading indicator to 	keep in mind the wind direction. Winds are always FROM a direction.

3. Suction may have a GREEN arc or numbers.  In the GREEN or between 4.6 and 	5.4 are normal.  Amperes may have gauges that read on either side of 	zero or may start at zero.  The needle should be on the + side or show 	an increase as alternator load is increased.

4. One half of the switch turns on the battery and makes it possible to 	start the plane and use electrical equipment from the battery.  	The 	other half activates the alternator which provides electricity without 	draining the battery.  In the event of alternator failure you would turn 	off the alternator half of the switch to conserve battery energy.

5. Once on the ground and below 15 miles per hour all steering is with the 	nosewheel and brakes.  Listening to the radio can often suggest to you 	that traffic is close behind making expeditious exiting advisable.  	This awareness will come with experience.

6. This way if you are short the excess speed will allow you to stretch the 	glide. What you suggest is partially correct.  An airplane has two 	best glide speeds.  One is for endurance-time in the air, the other is 	a penetration glide speed. Against  a wind  1/3 to 1/4 of wind 	velocity can be added to glide speed for best penetration.  However it 	is possible to be fooled by nose attitude changes as to whether you 	are high or low.  The best way to determine if a particular point can 	be reached and used for landing is by maintaining a constant airspeed. 	In a given aircraft configuration a constant airspeed sets a constant 	glide angle to a particular point.

7. The fact that the ailerons move up and down is not as critical as whether 	when one moves up that at the same time the other moves down.  The 	most dangerous time to fly an airplane is just after a mechanic has 	worked on it.  More than one accident has been caused by having the 	controls rigged incorrectly.

Question

1. What causes an engine-start fire?

2. Why is it important to keep the ball in the middle?

3. How do you orient yourself at a strange airport?

Answer

1. In colder/wet weather the usual process of pumping the throttle twice 	will not work as adequate prime.  Frequently additional pumps are 	given.  This caused the carburetor to overflow and gasoline to enter 	the air intake system.  If the engine should backfire during the start 	this excess fuel can catch fire. If the engine has started, pull the 	mixture and apply full throttle to consume the fuel as fast as 	possible.  If the engine has not started, pull the mixture and 	continue cranking the engine with the starter to suck the fire into 	the exhaust system.  In either event get on the radio and advise ATC 	of the problem.  This situation can be avoided by using the primer 	pump since fuel is then put directly into the intake manifold.

2. The purpose of the rudder is to keep the tail behind the nose.  If at any 	time of level or turning flight the ball is not in the middle  the 	plane is in an uncoordinated configuration.  For slips and crosswind 	landings this is required and deliberate.  Like flying with one wing 	low, uncoordinated rudder flight is inefficient and in slow flight can 	induce a spin.  Keeping the ball centered in a sign of flying 	proficiency.  This is most noticeable with power applications and in 	slow flight.

Like the thermometer, compass, and electric clock the ball will 	operate completely independent of all aircraft systems.  There is a 	small metal tab on the rudder of most Cessnas which is bent to a set 	position on the ground.  This setting is determined in such a way as 	to keep the ball centered in level cruise configuration.  Any turn, 	climb or other change from normal cruise will require the use of 	rudder 	to keep the ball centered.  Some aircraft have rudder trim 	which 	permits rudder adjustment to keep the ball centered in other than 	cruise.  By performing maneuvers at consistent power settings, 	airspeeds, and angles of bank the pilot can acquire a feel for rudder 	application required to keep the ball properly centered.  In general, 	the higher the nose and greater the power the more right rudder 	required.  Some climbing left turns can be made with no right rudder 	when the angle of bank and nose attitude in combination with "P" 	factor 	combine to keep the ball centered.  Right turns should be "led" with 	right rudder as should recovering from left turns.

3.  As a flying difficulty this is close to #1 being the most common 	regardless of flying hours.  One way to reduce the problem is to pre-	plan an altitude and reporting geographic point that will give a 	selected runway arrival.  If no such point is known, when well away 	from the airport turn the plane to the heading of the runway in use.  	Use this heading referenced to the airport to determine your entry.   	Use the heading indicator matched to runway for straight-in,  90 	degrees for base entry,  and runway heading at right/left rear 45 	degree position for downwind entries.  Practice this at a familiar 	airport before actual use.  Don't hesitate to indicated to ATC that 	you are unfamiliar and request to overfly prior to landing.   Always 	be prepared to advise ATC of your position and altitude in the ATA 	especially if unfamiliar with other traffic reporting positions.

Question

1. If I don't know what to say, what do I say?

2. What do I do if it appears an airplane is coming toward me?

3. While waiting to depart from an uncontrolled airport how do I decide when 	best to communicate?

4. Should I always use a checklist?

5. What do I do if I get "behind" the airplane?

Answer

1. Get on the radio and ignore what may be precise radio procedure.  Use 	plain language to state your message or problem.  Follow directions 	and request clarification if uncertain. Just because you hear

something does not mean that you understand what you hear.  When you 	acknowledge an ATC communication on the aircraft radio, it means that 	you heard it and understand the meaning.

2. Point out the situation to the instructor or if solo communicate. This 	situation may be an illusion if the airport has parallel runways. It 	may not be an illusion but another plane on right base while you are 	on left base for parallel runways.  It may be an airplane on an 	instrument approach executing a missed approach into oncoming traffic 	with tower approval. In any case a student's gut reaction to what is a 	very ordinary situation  usually results in some degree of control 	loss. Remember the first priority is to fly the plane; second, 	navigate (try to miss hazards); third communicate.

3. Exercise your best judgment. You must listen carefully on the radio and 	hopefully determine the intentions and runway operation planned by 	other aircraft. If space permits turn the plane in a complete 360 	degree turn while communicating your intentions to depart.  Don't be 	hurried into a takeoff without clearing both ends of the runway and 	remember communication is the lowest priority at an uncontrolled 	airport.

4. The more professional you are the more you will realize the importance 	of the checklist. However, the precise way a list is used is up to the 	individual. The list may be referred to item by item, in groups, or 	even as a reminder check at the end. Some pilots use a finger count 	system for the items for takeoff or landing if less than ten items.  	However it is not possible to fake the habitual use of a checklist.  	One requirement of a pilots checkride is use of the checklist. If such 	use is not habitual at some point in the checkride things will go 	forgotten.

5. While exact situations will vary some generalizations will apply.  While 	landing--GO AROUND do not hesitate, second guess, or try to salvage a 	poor landing.  Go around and get it right. If lost or disoriented--	slow up, trim for hands off flight and circle the most prominent 	visual 	point in the immediate vicinity. If getting behind in the communications 	required, go to slow flight until things get sorted out.  There is no 	distance too far to contact a tower for clearance into the Class Delta 	airspace. You can wait until too late to make such a contact, however.

Question

1. Why do I pull Carburetor Heat before reducing power?

2. Why should I practice pilotage as opposed to flying the VOR?

3. Why are touch and go's not considered as good use of practice flight 	time?

Answer

1. The design of the carburetor is such that when air and gasoline is mixed 	in the carburetor intake venturi a very abrupt cooling takes place.  	Since most air contains a quantity of moisture, it may freeze on the 	interior parts of the venturi.  In the venturi is a "butterfly" valve, 	like a moveable plate, which controls the amount of air and thereby 	power to the engine.  When the "butterfly is vertical,  maximum air 	and power, the probability of moisture adhering to the interior parts 	is minimized but possible.  When the "butterfly" is less than 	vertical, reduced power, the probability of moisture freezing to the 	interior is greatly increased.  Carburetor Heat is a preventive 	procedure.  Initiated prior to power reduction it brings warm air from 	the heater muff of the exhaust system. This warm air reduces the 	possibility of ice in the carburetor. It should never be partially 	applied.

2. I recently flew from the Bay Area to the L.A Basin with a mid time pilot 	who chose to fly via VORs.  When questioned as to the choice he 	indicated that this followed his flight training and that in the event 	of an accident he was more likely to be found along an airway.  A 	contrary opinion would say that a corresponding pilotage flight could 	be planned where an airport would always be reasonable close. In the 	event of adverse weather, the choice to descend below the clouds could 	well prevent the use of the VOR route. Undue reliance on the airway 	system will degrade the pilotage skills.  On a similar flight on the 	route from Las Vegas to Bakersfield clouds and turbulence caused me to 	opt for a flight over the clouds. Soon there was no ground in sight 	and we were far beyond any VOR coverage. Had to call into play Dead 	Reckoning skills long in disuse. Almost 20 minute "stand-by" by 	Edwards Approach while heading toward restricted area.  Got approach 	clearance just before "panicsville" and then hole opened up to show 	Trona below. D.R works when you are both good AND lucky.

3. In my opinion, touch and go's do not provide the realistic practice

a student requires.  I would rather the time and money be spent flying 	between local (endorsed) airports.  I want my students to develop the 	techniques required for location of call-up checkpoints, a variety of 	arrivals, radio procedures, and planned arrivals at airports under 	varying conditions.  The only time I use touch and go's for 	instruction is when I am trying to expose the student to a variety of 	pattern directions and approach conditions.  Specifically, I want 	crosswinds too be flown in both left and right hand patterns.  I want 	an approach under identical conditions to be encountered both high on 	final and low on final.  I want the alignment with the runway to be 	corrected from both the right and left.  Most of all, I want go-	arounds to be performed from a variety of altitudes and airspeeds.

Question

1. Where do I look to establish a 30 degree bank?

2. How do I determine how far to stay away from a runway on downwind?

Answer

1. It is difficult to determine a 30 degree bank within 3 degrees by eye 	alone.  You should use the available horizon to set, by eye, what 	you think is a 30-degree bank.  A quick reference to the attitude 	indicator should then be used as a check.  The wing strut of a C-150 	is at 30 degrees to the wing.  When the strut is parallel to the 	ground 	you have established a 30 degree bank.  The advantage of making 	a 30 	degree bank is that in a coordinated bank the ailerons can be 	neutralized.  If more than 30 degrees are entered the aircraft tends 	to increase its bank until counter aileron pressure is applied.  If 	less than 30 degrees are entered the aircraft tends to decrease its 	bank until counter aileron pressure is applied to hold the less than 	30-degree bank.  Due to slight variations in aircraft rigging this 	application is not perfect but it is a place to begin.

2. When you try to use visual geometry to find this distance you will find 	that when the length of the runway is less than a mile you will tend 	to be too close.  This excessive closeness will exist at unfamiliar 	airports and where you enter right downwind.  A 45 degree entry to a 	mile long runway would require a mid-field downwind turn to place you 	1/2 mile from the airport.  From this mid-field point to the numbers, 	to the key position, a normal base and 1/2 mile final will equal two 	miles.  If you have organized your landing technique for this two 	mile approach you will find that it fits well with the ATC requests 	for reporting two-mile base and final.  Using the same 45 degree 	approach and mid-field turn to any field less than a mile long will 	bring you too close and give a pattern smaller than two miles.  I 	suggest turning downwind abeam the end of any runways less than a mile 	long.  If you turn right downwind too close you will have difficulty 	seeing the runway.  Every slight dip of the wing to see will only 	serve to move you closer.  Deliberately fly wider downwinds in right 	traffic.

Question

1. How does the pre-flight, power, trim, flap operation of the C-172 differ 	from the C-150?

2.  In C-150 how can you determine fuel selector position from the engine 	compartment?

3. Under new airspace FARs what is standard size of Class D airspace?

4. How is State and Federal minimum altitudes for game reserves depicted 	differently on the Sectional?

5. What is the difference between a Restricted Area and a Warning Area?

6. How can tracking to a VOR cause an accident?

7. What communication is required for flight into Class C airspace?

Answers

1. The C-172 has a luggage door and alternator to check during preflight.

To set power of C-172 to 1500 rpm from cruise speed, you must reduce 	to 1700.  Deceleration will cause 200 rpm drop.  C-172 has less power 	for its weight and will take 2-3 minutes to accelerate to cruise 	unless 	altitude is lost. Initial trim is identical except C-150 slows to 60 	kts; C-172 slows to 70 kts.  Pattern in C-150 is 60 knots after 	turning base.  Once trim is taken off for every notch of flaps.  On 	landing C-150 is trimmed for level and must be re-trimmed for climb.

C-172 flies 70 knots after turning base and 60 kts on final.  No re-	trimming is required when going from 20-degrees flaps to full since 	speed will be reduced to 60 kts.  C-172 is now trimmed for climb.

2. The sump strainer will not drain when the fuel selector is OFF.

3. Class D airspace has a communication requirement because of an operating 	control tower.  It is normally 4.1 nautical miles in radius from the 	center of the airport up to 2500' AGL (above ground level). CCR has 	only 3 nautical mile footprint with extension to VOR.

4. Most federal and state minimum altitudes for game reserves area 2000' 	AGL.  State minimums are on the sectional near the reserve.  Federal 	areas are just marked by ... and minimums are in stated in one corner 	of the white space on the sectional.

5. The only difference between a Restricted Area and a Warning Areas is that 	the latter exists over international waters that cannot be restricted 	from international travel.  The hazards military training operations in 	each are identical.

6. CCR VOR as an example of how tracking to a VOR can be dangerous.  It can be 	received from behind Mt. Diablo in such a manner as to fly the unaware 	aircraft into the mountain.  See the Airport/Facilities Directory.  	Additionally, since VOR focus traffic to one geographic point, mid-air 	incidents are far more likely.

7. ATC does not give clearances into Class C airspace.  You may be advised to 	remain clear.  You will be addressed by N number or advised of radar 	contact.  The establishment of communications is all that is required 	for entry into an ARSA.  Entry into Class B airspace requires a 	clearance.

Question

1. Under what circumstances will Ground Control use the word "cleared"?

2. When does the 'hemispheric rule' for flight altitudes not apply?

3. How are Cessna and Piper nosegear different?

4. What is the brake hazard in transitioning from Cessna to Piper?

5. In the event of engine failure how might I extend my glide?

6. How is a glide against the wind different?

7. How can I increase radio receiver weak signal range?

Answers

1. The word "clearance" is not given for taxi authorizations except where

a specific command to stop or has been previously given.

2. The hemispheric rule does not apply if climbing, descending, or if that 	altitude is within 3000' of the ground.

3. The Cessna nosegear is steerable by means of the rudder pedals through 	springs.  This is only when the strut is depressed.  In flight, the 	Cessna nosegear hangs down and aligns with the relative wind.  This 	means the wheel is aligned with the direction of travel (runway) in a 	crosswind landing even in a cross-control condition.  The Piper nosegear 	is directly linked to the rudder and rudder pedals both in air and on 	the ground.  In a crosswind landing the cross-control approach with the 	Piper results in the nose wheel being at an angle to the runway.  Any 	Piper crosswind landing must be initially on one main wheel only.  The 	nose wheel must be straightened before being allowed to make ground 	contact.

4. The Piper has a cabin support bar across the cockpit just above the 	rudder 	pedals.  Look under panel toward cabin firewall to see.  Inadvertent 	placement of the toes above the pedal toe-stops can bring the feet into 	contact with this bar.  In this situation neither steering or braking is 	possible.  (Hand brake still works)

5. Get flaps off.  If altitude permits, slowing the aircraft to near stall 	will stop the propeller from windmilling.  This will increase glide 	range.  Using the starter to level the propeller will reduce ground 	contact damage.

6. When attempting to glide against a wind it is necessary to add from

1/3 to 1/2 of the headwind component to the best glide speed to gain 	optimum penetration glide range.

7.  Turning the squelch knob (if available) to the far right will increase

the sensitivity of the radio.  This will also increase the unwanted 	'noise'.

Question

1. What can I do to get down if I am high on final?

2. What is the only safe reaction to being low on final?

3. Why do taxiways have yellow lines?

4. When a very strong wind is right down the runway, what are two changes 	from normal landing procedures that will make landing safer?

5. When landing in a 'calm' wind, what 'wind' should you expect?

6. What is confusing about use of the trim?

7. How does full throttle affect fuel octane and engine cooling in fixed 	pitch propeller aircraft?

8. What three instruments function just as well off the aircraft?

Answer

1.  If you are high on final, first make sure that the fullest application 	of flaps is being used 	as allowed by crosswind conditions; secondly, 	pull off power; thirdly, raise the nose and trim to Vso according to 	aircraft weight.  Slips are not allowed in C-150 (C-172) with flaps.

2.  If you are low on final, apply full power while holding the yoke 	forward to keep a constant approach speed.  After 10 to 30 seconds 	reduce back to normal approach power.

3.  The yellow taxiway lines are an implied suggestion that an aircraft 	should taxi on the line to remain clear of obstacles.  The yellow 	helps you to tell a taxiway from a runway.

4. When landing in a very strong head wind.  Be a bit faster in doing the 	procedures for landing on the downwind and avoid using full flaps.  	Full flaps in a very steep approach can affect the airflow over the 	horizontal tail surfaces sufficiently to cause a tail stall.

5. The "calm wind" may be a tail wind on high final an a crosswind at the 	runway.  Be alert.  Be a bit slower doing the procedures for landing 	and make rudder/aileron applications immediately in the flare to stop 	drift.

6. The trim wheel is moved 'backwards'.  Moving it down, raises the nose.

Think of your moving the trim as moving the tail of the aircraft.

7. Gas is added to the engine to make it cooler.  Full throttle opens jets

to allow excess gasoline to cool engine during climb.  The gasoline 	will actually increase in octane during full power/pressure 	operations.

8. Three aircraft instruments will work as well off the plane.  Thermometer, 	clock (non-electric),  compass, (ball (inclinometer)would be also).  	Take the compass with you if walking 'out'.

Question

1. How does 'lift' make an airplane change altitude?

2. Why must the 'lift' of the horizontal tail surfaces be opposite to that 	of the wing?

3. Why should I try not to land when landing?

4. How is a steep approach to a landing better?

5. Why land with crossed controls?

6. How can I make turns using the compass?

Answers

1. The top of the wing lifts the airplane.  The pressure differential 	between the top and bottom curve (if any)  will cause a partial vacuum

over the top.  Lift is really the wing moving into the created vacuum 	space due to air pushing from below.  The weight of the air displaced 	must exceed the weight of the aircraft for it to climb.  Changing 	pitch attitude changes 'lift'.  When the weights are equal the aircraft 	is in level flight.  When the weight of the displaced air is less than 	the aircraft weight, the aircraft descends.  Lift ceases when an airfoil 	exceeds its critical angle of attack.

2. The tail of a plane pushes down in flight.  This is a design and weight

/balance contradiction.  The weight of the engine must be offset by 	some force.  If the weight/balance is such that airspeed will not give 	elevator control.  You will not be able to attain or maintain the nose 	attitude required for landing.  A good reason to know your minimum 	controllable airspeeds.  Below minimum controllable the 'downward 	lift' of the tail surfaces can no longer hold the nose up.

3. A good landing is made by trying not to land.  Simple answer but correct.  	Only the airplane should know when it is really through flying.  The 	pilot should keep the airplane in the air as long as 	possible for the 	best landing.

4. A steep landing approach gives better aim than a shallow one.  Draw a

shallow approach and a steep approach.  Now make an error angle on 	each approach line of the same size.  Note the difference in runway 	required.

5. Cross wind landing touchdowns are correctly made on one wheel.  The rudder 	and aileron are cross controlled.   Be sure to eliminate any drift prior 	to touchdown since side loads are hard on the aircraft during one wheel 	landings.  Advise passengers.

6. Use time.  Standard rate turns only when flying the compass.  90 degrees 	takes 30 seconds.  Turn errors inherent to the compass cause it to 	turn much differently than the aircraft.  To go to a heading on the 	compass you turn away from it.  Practice flying compass headings and 	timed turns to other headings to appreciate the mental gymnastics 	required.

Question

1. Is it possible to have too much altitude on a flight?

2. What do you sometimes turn left when turning right?

3. Can you be 'ready' for an emergency?

4. Why does air during a rain weigh less?

5. Can a VFR airplane takeoff and yet not legally land?

6. When will an ailerons fail to raise a wing?

7. How does P-factor influence the use of rudder?

8. When might left aileron be used in a right turn?

9. What is meant by a stabilized approach?

10. How does an airplane glide best?

Answer

1.  Yes, when you're on fire.

2. When taxiing you may correctly turn the yoke to the left while turning 	right.  In a left climbing turn your yoke can be to the right.

3. You will never be ready for an emergency landing.  The decisions leading 	to an accident are usually progressive bad to worse.  Begin by 	circling your home.

4. Wet air is LIGHTER than dry air.  The moisture forces the air molecules

further apart thus decreasing overall weight.

5. An airplane may be too heavy to safely or legally land.  Excess fuel.

6. Turning the aileron to raise the wing doesn't always work.  If a wing is 	stalled, the application of up aileron usually aggravates the stalled 	condition and does not raise the wing.

7. Right rudder is often used in a climbing left turn.  Once into a 	climbing left turn, either no left or some right rudder will be 	required to center the ball.

8. Left aileron is often used in a right turn.  At high angles of attack 	the adverse yaw of the opposite aileron is required coordinate the 	turn against other aerodynamic effects.  Improper application can 	cause a cross-control stall.

9. A constant landing approach angle is determined by airspeed and 	configuration.  Airplanes like automobiles have distinct speed sounds, 	sensations, and perceptions.  The more constant your airspeeds the 	better you recognize these distinctions and any variations.  Trim!!!!

10. An airplane glides best with its nose up.  On engine failure, if you

don't hold the nose up you will mostly lose altitude not airspeed.

Question

1. Why do pilots walk funny?

2. What should happen when you become confident of your flying ability?

3. When will raising the elevator not cause 'up'?

4. When may lift not be opposite weight?

5. What might be said about rudder being, too much, too little, too early, 	or too late?

6. Fuel, airspeed, and  ???? are forms of flight insurance.

7. Should pilots assume that some accidents are bound to happen?

8. What is the worst ATC can say to a request for preferred handling?

9. Why should all climbs be made with wings level?

Answer

1. Pilots walk funny because of the rudder.  Increases as well as decreases 	of power, climbs and descent, and  in and out of turns require rudder 	applications.  Usually anticipatory right rudder.  Anticipatory right 	rudder is used when entering right turns and when coming out of left 	turns.

2. When you become confident, be three times as careful.  The surest way

to lose excess confidence is to have an accident.

3. Back elevator does not always give 'up'.  At very slow airspeeds or 	behind 	the power curve the aircraft will go slower with back elevator but not 	go 'up'.  The proper use of this knowledge makes for good landings.

4. Lift is not always opposite weight.  In a level turn some lift causes 	the turn.  Some angle of attack increase is required to prevent a 	descent when some lift is used to create the turn moment.

5. The rudder does not 'turn'.  Too much rudder, too soon is just as bad as 	too little, too late.  On landing approach the rudder is used to keep 	the nose parallel to the runway, the slip is used to maintain runway 	alignment.

6. Altitude, like gasoline and airspeed, is insurance.  Very nice to have

when you need it.

7. Accidents are the result of avoidable or preventable situations. Bad 	decisions are compounded by bad decisions.  Make your best initial 	decision and stick to it.

8. All tower clearances are only permissive.  The pilot has the final

authority as to the safety of aircraft operations even when permitted 	by ATC.  Don't be hesitant to ask for the most economical, for your, 	departure, taxi route or landing.  All ATC can say is, "NO!"

9. "dragging" a wing low can make 100' a minute climb rate loss.  For 	maximum rate of climb keep the wings level.  This is a performance 	safety factor in addition to Vy takeoff speed to get maximum altitudes 	as quickly as possible.

Question

1. What should be checked first after noting an oil pressure drop?

2. At CCR when are you most likely to encounter carburetor ice?

3. What can you be certain of regarding weather and wind?

4. In the event of engine failure at cruise, what can be traded?

5. What should you do in the event of engine failure on landing approach?

6. What resource exists during landing but not at altitude?

7. What two features of flying show good pilot skill?

8. Why is it best to make pattern turns at 30-degree bank?

9. Why are pilots poor gamblers?

Answer

1. Oil temperature indicates engine temperature.  In the event of an oil 	pressure loss, check the oil temperature.  If temperature is normal, 	there is a possibility that the pressure loss is a gauge problem 	rather

than an oil problem  If the pressure is down and the temperature is up, 	get down fast.

2. When taxiing.

3. No weather conforms to the weatherman's opinion of it.  The wind is 	never 	as forecast.

4. In the event of engine failure, excess airspeed is lost before altitude.

Use any excess airspeed over best glide to acquire any altitude you 	can.  Altitude can be traded for airspeed.

5. At engine failure, removing flaps will extend your glide.  Fly the same 	indicated airspeed.  Instructor's demonstration at 400' with long 	runway.

6. Most poor landings occur because of failure to utilize ground effect.  	Ground effect is a function of a fixed wing span and a fixed airspeed.

The better your fixed airspeed the better your use of ground effect 	and your anticipation of its effect.

7. Flying straight and level is one of the most difficult skills in flying.

Smoothness is the proof of flying skill.  Smoothness can only be 	acquired by anticipation.  Anticipation means you are intellectually 	and physically ahead of the aircraft.

8. A well rigged airplane trimmed for a level 30 degree bank is stable in 	that bank with the yoke level to the cockpit.  This is one very good 	reason for making 30 degree banks while in the airport pattern.

9. Pilots under-estimate the probability of a particular event happening to 	them.  This is why I never seem to win at Reno and keep going back.

Question

1. Why should left-handed people fly airplanes?

2. Why is the making of mistakes to your advantage while learning to fly?

3. What is the purpose of the proficiency phase of flight instruction?

4. What are some of the forms of 'right' used in flying?

5. Why may summer landings not be as smooth as winter landings?

6. What are some of the 'misconceptions' you have overcome in learning to 	fly?

7. How can adding power in an airplane be different than an automobile?

Answer

1. Flying airplanes is about the only area where left handers of the world 	get even.  PIC in helicopters sits in the right seat.

2. You go to school to make mistakes.  If you must make mistakes make them 	new ones.  Make as many mistakes with the instructor as you can so you 	will acquire both the skills of avoidance and correction.

3. If the student thinks he ought to know it, he won't ask it.  The 	proficiency phase of flight instruction is not to impart information 	so much as finding out what is not known.

4. Right comes in several different forms.  Your first clearing turn is to 	the left not right.  Your most likely error in doing stalls is failing 	to clear.  Right.  You turn the "wrong" way to make a compass read 	"right".  A right turn may require a left turn of the yoke.

5. A landing attempt under high density altitude conditions is likely to be 	more 'firm' than otherwise because of the relative absence of ground 	effect normally encountered on 'dense' days.  The "plump down" can be 	reduced by flaring closer to the surface.

6. Low pressure in flight is on top of the wing and below the horizontal 	stabilizer.  This means in a conventional aircraft the tail is 	pulled/pushed down as a lever to keep the nose of the aircraft up.  A 	PTS question.  Fear of height opposes fear of hitting the ground.  At 	takeoff, keep the runway in view because we should see where we are 	going.  More speed along the runway gives a better feeling of control. 	If 60 kts is a safe approach speed, 80 kts is even safer. The nice 	flat smooth landing is best because it feels so good.  That's the way 	you land a toy airplane.  If a moderate bank is safe, them a shallow 	bank is safer and feels better.  The feeling that hills and water, in 	and of themselves constitute flight hazards to be flown around.  Since 	the movement of the trim can be confusing don't bother with the facts 	of its use.  This plane can be flown without using trim.

7. Just adding power causes you to go slower.  Try it.  Set up, hands off, 	level flight at a low cruise speed.  Hold heading with rudder while 	applying full power

Question

1. What effect will 'diving' for the runway have on your landing?

2. What will any speed below the best glide speed do?

3. What makes an airplane fly?

4. What is the skill about flying you are least likely to master?

5. What are flaps for?

6. How can vertigo be prevented?

7. Why should children get off bicycles and into airplanes?

Answer

1. Diving for the runway may make you miss it.  Increasing approach speeds

will cause the aircraft to cover more ground getting to the runway and 	will increase the ground effect once at the runway.  To get down, slow 	up.

2. Raising the nose can bring you down faster.  Any speed below the best

glide speed will increase the rate of descent.  Up until the point of 	stall the slower you go with the nose up, the faster you go down.  	Power, without any increase in pitch attitude is the only correction.  	You are behind the power curve when the only way to increase airspeed 	is by lowering the nose.

3. Money.

4. The last thing you learn to do well is to taxi.  The classic unlearning

problem.  You do not 'drive' an airplane on the ground.  It is a 	kiddycar.  Where you place your seat and feet will influence how well 	you taxi and how well your learn.

5. Flaps are not designed to slow the plane.  Approach landing speeds

are essentially the same for with and without flaps.  Flaps allow the 	nose to be lowered without increasing airspeed.  Flaps, thus, improve 	your over the nose visibility.  The improved visibility allows a more 	accurate determination of whether high or low on final.  The steeper 	descent allows greater accuracy on the approach.

6. Vertigo sensations cannot be prevented, they can be ignored.  You don't 	appreciate the trained will power required to ignore vertigo until you 	have had it.

7. For every person killed in General Aviation, four children are killed 	while riding bicycles.  Teach children to fly.

PUZZLE

Knowing the right answer makes rearrangement of the letters easier.

1. osthhrreeu (two words) Is the instructor determined flight time for a 	C-150 before being on the ground for fuel; starting with full tanks.

1a. ittons' (two words)  How is this different from the C-172?

2. surtaforqu (two words) Is the minimum oil in a C-150 before adding oil.

2a. surtaixqs  (two words)What is the amount for the C-172?

3. tsase of Cessna aircraft are prone to slip back.

3a. ittons'  How is this different from the C-172?

4. crale  is what you say before starting the engine.

5. aavidewy (two words) Is where you hold the yoke when the wind is behind 	you.

6. cafe  the wind when you run-up.

7. cafe  toward the final approach course of the runway before you taxi 	on to the runway for takeoff.

8. bearkacd (two words) if the tower uses the word "hold" prior to

clearing you for takeoff.

9. inloogc is the reason the throttle is full in for takeoff and climbs.

10. sindcsorw The term for the direction at right angle to your upwind 	takeoff direction.

11. 060 feet is the  altitude to turn on departure at CCR.

12. andouths is the altitude not to leave before turning base at CCR.

13. batherractoure (two words) is used before any power reduction.

14. ourf is the count to get 10 degrees of flap before checking flap 	indicator.

15. pfellurow(two words) is the only solution if the approach is low.

16. aperselerwido (two words) is used to increase the rate of descent from a 	high approach after full flaps and power is off.

17. adoronug (two words) is the solution to any touchdown not to be made 	in the first one-third of the runway.

18. aslofnp (two words) are used for landing if there is a strong 	crosswind over 15 kts.

19. wot thousand vefi hundred(two words) or 3.1 nautical miles is the required 	altitude before tower frequency can be changed on departure.

20. tais  must be obtained before making initial radio contact with the 	     	tower of any airport.

21. 5211. is the emergency radio frequency.

22. 1020  is the standard VFR transponder code.

Answers:

1. three houts - 1a. it’s not -2. four quarts -2a4. six quarts -3. seats -

4. clear -5. dive away -6. face -7. face -8. read back -9. cooling -

10. crosswind -11. 600 -12. thousand -13. carburetor heat - 14. four -

15. go around - 16. lower airspeed - 17. - go around - 18. no flaps -

19. two, five - 20. ATIS -21. 121,5 = 22. 1200

# A change of pace...

1. htetesorudanh (two words) feet is the highest altitude AGL an airplane can 	proceed in any direction without regard to the hemispheric rule.

2. alalalalibev (two words) information is required by the pilot prior 	to a flight.

3. gwlolofni aircraft always pass on the right.  For this reason initial 	clearing turns are to the left.

4. adefghilnrs (two words)light from the tower means to move clear of

runway.

5. leolwy arc of the airspeed indicator should not be flown when in 	turbulence.

6. sesl engine power, wing lift and propeller thrust is available when there is a high density altitude.

7. thewi is the arc color on the airspeed indicator showing the flap 	operating range.

8. rihtty degrees is the maximum bank to be used in the pattern.

9. nneglai is used to improve engine performance and reduce fuel. 		consumption at altitude.

10. semil is the word normally omitted when giving distance.

11. tsilkcehc is the first item on an emergency checklist

12. tegnimca wind directions are given by the ATIS to conform with 			runway headings and numbers.

13. thiew X indicates that the runway is closed to all but emergencies.

14. salfp give an aircraft an increased angle of descent and improve 			landing accuracy.

15. duderr is used to keep the tail behind the nose of the aircraft.

16. rnogud effect causes control effects to be greatly exaggerated when 	landing.

17. ttnncosa airspeed makes it easier to determine if high or low on 	approach.

18. aayydm is your first radio word in an emergency.

19. An aircraft is at my altitude when it is on the ziroohn.

20. pratcof- is the reason an aircraft turns to the left when in a climb.

21. tle is a beacon activated when an airplane crashes

22. smaspoc is always used to check accuracy of heading indicator.

23. tlipo is responsible for safe operation of an aircraft.

24. 16to 1 is the tixmrue ratio of air and fuel that gives best engine 			operation.

25. 0.212 is the nation wide frequency for Flight Watch.

Answer

1. three thousand - 2. all available - 3. following - 4. flashing red-

5. yellow - 6. less - 7. white - 8. thirty - 9. leaning - 10. miles -

11. checklist - 12. magnetic - 13. white - 14. flaps - 15. rudder -

16. ground = 17. constant - 18. mayday - 19. horizon - 20. p factor -

21. ELT - 22. compass - 23. 122.0

24. 700’  at sreciu is the instructor assigned minimum altitude/airspeed 		for ground reference practice.

25. No  sentutd  flights are to be made without the knowledge and

approval of the instructor.

26. senlice and kobolog (two words) are the required documents a student 		pilot must have in possession for any solo flight.

27. Every nsayynetnd I must have my log book endorsed for solo flight.

28. My instructor has placed the following limits for local solo flight:

Ceiling of snthtwoouda

Visibility of  theig miles

Wind velocity of stetenfnfiok (two words)

29. The aircraft to the trigh has the right of way when aircraft are

converging.

30. I should turn grith approaching another aircraft head on.

Answer:

24. cruise - 25, student - 26. license and logbook - 27. ninety days -

28. three thousand, eight, fifteen knots - 29. right - 30. right