Navigation, stalls, hoodã

A.  TASK:  PILOTAGE AND DEAD RECKONING

Common navigation faults are:

Dead Reckoning

B. Task: NAVIGATION SYSTEMS AND RADAR SERVICES

C.  TASK:  DIVERSION

D.  Task:  LOST PROCEDURES

D.  TASK:  MANEUVERING DURING SLOW FLIGHT

B.  Task: POWER OFF STALL:

B.  TASK: POWER-ON STALLS

Imminent stalls -

D. Task: SPIN AWARENESS

Sources of Inadvertent Stalls Becoming Spins

Endorsement For Spin Awareness

A. Task: STRAIGHT AND LEVEL FLIGHT

B. Task: CONSTANT AIRSPEED CLIMBS

C.  Task: CONSTANT AIRSPEED DESCENTS

D.  TASK:  TURNS TO HEADINGS

E.  Task: UNUSUAL FLIGHT ATTITUDES

F.  Task:  RADIO AIDS AND RADAR SERVICES

VII.   AREA OF OPERATION

NAVIGATION

# A.  TASK:  PILOTAGE AND DEAD RECKONING

References AC 61-21, AC 61-23, AC 61-84

P    1. Knows application and distinction between pilotage and dead 			reckoning

P    2. Uses pilotage (landmarks and charts) to navigate plotted course 		line.  Computes speed, times and ETA Compares with ETAs and 		fuel used.

P    3. Identifies landmarks by relating surface features to chart symbols.

P    4. Navigates by means of precomputed headings, ground speeds and 			elapsed time.

P    5. Corrects for and records the differences between preflight fuel,

ground speed, and heading calculations and those determined en 		route.

P    6.  Verifies the airplane’s position within 3 nautical miles of the 		flight-planned route at all times.  All courses maintained 			laterally + 3 miles.

P    7. Arrives at the en route checkpoints and destination ETAs + 5 			minutes,

P    8. Maintains appropriate altitudes + 200’ and headings + 15°.

P    9. Completes appropriate checklists.

EX   Plotting course, selecting checkpoints, computing data based on forecast data, how to establish course and stay on course, locating checkpoints, how to compute ground speed, ETA's, fuel use, correcting ETA's into arrival airports.  Destination and course to be within three nautical miles, altitude + 200 feet, arrival within +5 minutes, headings within + 15 degrees

Pilotage is flying a course line by charted reference to ground points.  Dead reckoning is flying a route based upon course, time, and distance from a known point using estimated winds.  Plotting the course begins with drawing the line on the sectional.  Using a highliter pen or spaced multiple lines makes the course line easily discernible. Determine the True Course, determine the nearest variation (angle between the north and magnetic poles) and compute the Magnetic Course for entry on the course line. Since the Magnetic Course has no wind factor computed, it is a good reference for in flight wind correcting.

You are required to estimate a time of arrival (ETA) at a particular

checkpoint.  You are allowed + 5 minutes error in ETA Vs ATA, + 200' in altitude, + 10 degrees of heading, and  + 3 nautical miles of course.  If you cannot fly pilotage within half of these tolerances you will have difficulty passing the test.  The examiner is allowed to create distractions to determine your ability to divide your attention.

Checkpoint selection is critical since a good checkpoint has three verifiable features, one of which may be a VOR radial.  A day checkpoint may be invalid for night.  There is some advantage in spacing checkpoints evenly from the destination.  Keep points on the pilot's side as much as possible.  Above 3000' don't use power lines or railroads.  Lakes change during the year.  Visibility conditions and altitude can greatly change the value of a checkpoint.  Your skill in correlating a chart-symbol to the landmark seen from the airplane is essential.

The Compass Heading of the aircraft is the way the navigational computations say we should point the plane to allow for the wind, once we are established on course.  Since these computations are based on forecasts they are at best estimates which must be adjusted in actual flight.  The forecast winds are relative to True North in knots.  We use the True Course line drawn on the sectional and True Air Speed in knots from the manual to compute the True Heading and Ground Speed. True heading is then corrected for variation and deviation to obtain Compass Heading which is where the paragraph started.  Fly to your first checkpoint and turn on course.  Re-set your heading indicator and confirm your course.  Sight on something as far ahead as you can see and start looking for your next checkpoint.

The computed Ground Speed is used to compute ETA's (estimated time of arrival) along the route.  These times are in turn used to compute fuel consumption. Estimates of fuel consumption are based upon the manual the actual consumption at the end of the flight can be very educational indeed.  The pilot who records fuel use on all flights gets insight into actual aircraft performance.  Fly fuel use by time.  Most aircraft are NOT topped off to full tanks because of fuel that may be wasted due to heat expansion.

Since wind conditions may vary the departure from an airport, it is vital that the aircraft be established on course as soon as possible.  It is easily possible to depart incorrectly and exceed the 3 mile course error before reaching 1000'.  By telling the tower that you plan to depart on course to your destination you solve a multitude of problems.  Fly directly to your first checkpoint.  Use the checkpoint CHECKLIST Carefully select a point as far ahead on course as you can see and fly to it.

If possible reach your first checkpoint level at cruising altitude.  This makes possible a time check at the second checkpoint and computation of an ETA.  Destination ETA should be estimated at every checkpoint based on checkpoint ATA.  The planning technique that makes ETA at destination very easy is to try to make your checkpoint distances the same measuring backwards from your destination.  The only uneven distance should be from your departure to your first checkpoint. (see instructor)

See training material on use of checkpoint checklist.

# Common navigation faults are:

- Failing to use CHECKLIST for climb, leveling, checkpoints, descent.

- Failing to accelerate to cruise before fine trimming.

- Being off course and not correcting.

- Not setting Com/Nav radios ahead of time.

- Holding head down while reading charts or using computer.

- Failing to estimate heading required to correct wind and then flying

that heading.

See instructional material related to cross country, checkpoints and radio.

The applicant must plan a cross-country to the maximum range of the airplane with required reserve.  This is a test of judgment as well as computational accuracy.  The major judgments involved are weather considerations, aircraft performance, airport requirements, navigational options, considered alternatives, and use of resources.

# Dead Reckoning

A method of navigation without the use of checkpoints predicated on

direction, time, and estimates of speed and wind.  Contrary to popular opinion, it has nothing to do with ded as in deduction.  As 15th century sailing ships began a voyage a rope with evenly spaced knots was tied to a log.  The log was placed 'dead' in the water and the rope was played out

over a fixed amount of time as the passage of knots was counted.  The result of this gave a time over distance which was calculated as knots per hour and recorded in a log book.

Lindbergh used dead reckoning while flying the Atlantic.  The accuracy

of this method depends on several factors:

- Selection of the initial heading based on wind correction angle.

- Estimation of ground speed based on true airspeed and wind effect.

- Accurate timing.

- Luck

# B. Task: NAVIGATION SYSTEMS AND RADAR SERVICES

REFERENCES: AC 61-21, AC 61-23, navigational manuals

P    1. Knows uses and limitations of navigational aids and radar services/

P    2. Selects, identifies and uses appropriate aids and radar services

P    3. Uses facilities to confirm aircraft location.

P    4. Intercepts and tracks a given radial or bearing, if appropriate.

P    5. Recognized and describes the indication of station passage, if 			appropriate.

P    6. Recognizes signal loss and takes appropriate action.

p    7. Uses correct communication procedures and acknowledgments with ATC.

P    8. Flies + 200’ at all times of appropriate altitude.  Uses CHECKLISTS

EX  Use NAV radio and ident code compared with sectional; centers OBS 	from/to; turns plane to course; checks compass, heading indicator, 	OBS readings; limits wind correction to 30 degrees with ever 	decreasing corrections; does not chase needle at station passage; 	watches for red flag; uses CHECKLISTS.  Within + 200 feet of 	altitude, locates aircraft position using radials, intercepts and 	tracks a given radial.  Recognizes signal loss and takes appropriate 	action.  Describes station passage.

The initial selection of a VOR station is dependent upon your location relative to the station, altitude, the distance and the terrain.  40 miles is the usual general aviation distance limit for accurate usability.  Since line-of-sight is a limiting factor the distance may be much less because of the terrain.  It is essential that every VOR signal be identified and confirmed with the sectional for its code.  Lack of identification is the way to tell if the VOR is unusable.  If a service man is making adjustments it may appear to work but accuracy is not assured without the IDENT.  Once identified be sure to turn the IDENT volume down.

Part of the AIM, called the Airports/Facility Directory available at all FSS, lists the terrain/directional limitations of VOR's.  The latest available should be purchased shortly before you plan to take the flight test from a local FBO.  The Concord VOR is unusable in a wide arc toward Mt. Diablo.  The signal may operate the needle, the IDENT work, and you can fly into the mountain. You must know the terrain in order to use a VOR safely.  Using an out of date A/F D is a no-no.

Centering the needle with the OBS requires some degree of dexterity.  The weaker the signal the more the needle waves.  Behind terrain or at maximum line-of-sight ranges for a given altitude the waving may make the VOR unusable. If this is the case learn to recognize it.  Hold the OBS knob between two fingers and move it back and forth in ever decreasing amounts until the needle is centered.  Read the TO/FROM window to confirm setting and heading to turn aircraft to.  Don't delay-turn immediately.

After turning aircraft to initial heading check compass, heading indicator, and OBS for agreement.  They must agree for you to avoid reverse sensing which causes the needle to work backwards.  Any delay in turning the aircraft will cause the needle to swing excessively.  In any event, re-center the needle and fly the corresponding heading.  Watch the needle for movement caused by wind drift.  This movement can only be determined if ACCURATE headings are flown.  After a few minutes if the needle moves to one side, make a heading correction 30 degrees or less designed to re-center the needle keeping in mind a heading that will be required to keep the needle centered.  This is a guessing game similar to finding a number between two other numbers on a computer. With practice and some knowledge of the forecast winds you can become quite accurate. The key to flying the VOR is the ability to fly headings while distinguishing between actual and transient movements of the needle.

Flying to a VOR is like flying into a 40 mile funnel. The closer you get to the station the more sensitive the needle becomes to 1 or 2 degree heading changes.  Recognize that winds will change both with position and altitude. This means that the correction at one point will not necessarily apply further along.  Be prepared to make heading adjustments as necessary, hopefully in ever decreasing amounts until one heading keeps the needle centered until station passage.  Learn to recognize the rather rapid needle movement indicative of passage.  Remember that the higher the crossing altitude the larger will be the cone of confusion. This means that the station passage heading must be held for periods of time, depending on altitude, before the needle will function properly on the other side of the VOR.

The VOR is frequently used as checkpoint backup indicator when map references may be limited.  A VOR is selected that will give radials FROM at an angle 60 to 90 degrees to the course line.  The closer to 90 the more accurate.  Draw a line on the sectional from the VOR to the checkpoint.  Find the VOR frequency, code, and OBS radial from and write them on a navigational log and on the sectional.  As part of your checkpoint CHECKLIST you should have an item specifically for setting and re-setting your VOR.

A properly set OBS (FROM the VOR) for a checkpoint will cause the needle to start moving within 10 degrees of the radial.  The needle will be on the side toward the VOR.  As you proceed toward the checkpoint the needle will move closer toward the center.  It will center at the radial set on the OBS and then move to the side away from the station.  If the station is very close to the course line it is possible for this swing to occur so fast that it will not noticed.  Be sure to observe the side the needle is on since this is the best clue as to the position of the checkpoint.

While it is necessary to demonstrate pilotage as the basic form of navigation, it is also expected that the pilot be able to perform using radio aids.  Over dependence upon radio aids to navigation seems to be a common training error. The fact is that when weather and flight conditions are the poorest the practical use of the VOR decreases.  The VOR range is a function of line of sight which is in turn dependent upon altitude.  If flight must proceed within 700/1200 feet of the surface due to ceilings the VOR range becomes less than 20 miles in level terrain and even less if terrain intervenes.

Flying to VOR's tends to put you in the major fly ways with heavier traffic.  Undue reliance on the VOR reduces pilotage skills and gives a false sense of navigational skill.  Use the VOR only when it is honestly the best route since its limitations become greatest when you need them most.

See instructional material related to checkpoints.

# C.  TASK:  DIVERSION

REFERENCES:  AC 61-21, AC 61-23

P    1. Knows the problems and solutions related to a diversion.

P    2. Selects an appropriate alternate airport and route.

P    3. diverts promptly toward the alternate airport.

P    4. Makes an accurate estimate of heading, groundspeed, arrival time, 		and fuel consumption to the alternate airport.

P    5. Maintains the appropriate altitude, _200’ and + 15°

EX   Factors of selection, present position, location of alternate, sectional use for distance and direction, altitude for visibility and insurance, communication procedures, decision making capability, division of attention, priorities. arrival procedure determined by communications,

Accurate estimate to heading, ground speed figured, estimated time of arrival, fuel consumption, altitude + 200 feet and course held within + 15 degrees.

Reasons for diversion can be numerous such as a kidney stop, airsickness, weather, fuel, mechanical, turbulence, and precautionary.  Every flight should be planned with the possibility of a diversion in mind.  This flexibility should be part of the mental set as an antidote for get-there-itis. Along the route, possibly as part of your checkpoint CHECKLIST you should have noted diversionary airports and their radial from the nearest VOR.  On the sectional write pattern altitudes, frequencies, and other pertinent data. KEEP YOUR CHECKLIST OUT

If the pre-planning has been done correctly a number of the elements of this task are in place.  First you will know where you are.  Visually triangulate your position by using references to sides and front.  This means that while flying up a valley covered with fog, with Mt. Diablo on your right and Grizzley Peak in the Oakland hills to your left, you should approximate your position as Walnut Creek.  This skill can be developed with practice and conscious effort.

Try to keep oriented along your route and have in mind the direction and location of airports.  Practice locating private fields that may not be on the sectional.  Keep an eye out for wind indicators and learn to relate what you see with the winds you find on the ground.  Learn the terrain altitudes, power line locations, and obstructions so that you can make reasonable judgments as to emergency choices.

When required to divert, for whatever reason, turn immediately, maintain altitude as much as possible, fly the airspeed for best range.  Once headed in the selected direction start using the sectional to locate yourself and the emergency destination.  Fold the sectional for your course line or make a line.  Altitude is insurance.  Use the plotter and VOR rose to determine course.  Be sure to correct for variation if using plotter only.  Fly estimated heading.  Estimate time, distance and fuel.  Keep attention divided.

Keep altitude until sure of arrival procedure.  Set radios.  Listen on COM while setting VOR radial for intercept.  Maintain altitude until alternate is located. If contact is established an arrival can be planned, otherwise maintain altitude of at least twice pattern altitude.  Use wind and runway indicators to determine arrival sequence.

Have the manual available but be familiar with the glide range to altitude ratio and short field performance of your plane.  Know the effects of stopping the propeller on glide.  Remember the first item in any emergency is CHECKLIST.

The element of judgment is more difficult to explain. Your choice of where to go and what to do may differ from that of the examiner.  Don't try to second guess, make a considered decision and stay with it if possible.  You would do well to explain the what and why of your actions.  If task requires partial or no power be sure to trim for best glide. Don't let the wind blow you away from your field.  There are far more corrective alternatives for being high than low. Keep your priorities in order.  Fly the plane (airspeed), navigate (to arrive at a pattern altitude abeam numbers on downwind), and then communicate (121.5 or advisory). Use your

CHECKLIST.  See instructional material on emergencies

# D.  Task:  LOST PROCEDURES

REFERENCES: AC 61-21, AC 61-23

P    1. Exhibits knowledge of the flight elements related to getting lost 		and getting found.  Knows the psychology of being lost is 			initially denial, then anger and finally acceptance.

P    2. Selects the best course of action when given a lost situation

Has an options list and makes safest selection.  Uses CCCC if 		in doubt.  Uses ‘lost’ checklist

P    3. Maintains heading of aircraft.  Climbs.

P    4. Identifies the nearest concentration of prominent landmarks.  Seeks 		visual reference.

P    5. Uses navigation systems/facilities and/or contacts an ATC facility 		for assistance as appropriate.  Gets airport information such 		as wind, runway, traffic, and pattern altitude.

P    6. Plans a precautionary landing if deteriorating weather and/or fuel 		exhaustion is imminent.

EX  Being lost effects on thought processes, CHECKLIST meaning of 4 C's, 	using VOR radials, radar assistance, DF steer, limitations, effect of 	one additional problem such as weather, fuel, mechanical, or 	personal, airport Vs off airport landing.  Maintains original 	heading, identifies nearest landmarks, uses navigational aids, 	contacts ATC for assistance, plans a precautionary landing if 	necessary.

The simulation of lost procedures is impossible, simply because of the mentally disabling effects of being lost. At best you must explain the use of the CHECKLIST.  There is a difference between being misplaced and lost.  The misplaced pilot is to one side or the other of course and still may be locating checkpoints and (incorrectly) accepting his position as O.K.  The lost pilot may be holding a course that is incorrect and causing an ever increasing deviation from the checkpoints.  He may locate places and call them checkpoints or locations that are completely wrong.  When identifying a place he shouldn't say, "That's Gotta' be..." because it doesn't.  He may not even recognize that he is off course and misnaming locations.  He is lost and doesn't even know it. Recognition takes place very gradually, sneaking into the consciousness and disabling the brain.  The CHECKLIST is the best way to put things in order.

Habitual reliance on prominent checkpoints such as isolated mountains or lakes leads to a complacency easily shattered by 4 mile visibility.  The distance between checkpoints should be related to aircraft performance and visibility.  Your ability to recognize and correctly identify checkpoints and your relationship to them is the best preventative.  If you know where you are you may have changed "lost' to "misplaced'.  Once 'lost' is changed to 'misplaced', fly directly to your course; intercepting no later than the next checkpoint.

The placement of VOR's in conjunction with altitude and terrain may permit you to obtain a cross bearing of two radials.  If this is the case, use standard procedures of frequency, identification, centering OBS on FROM.  Transfer the VOR information to the sectional.  By using two different VOR's at as near 90 degrees to each other as possible a very accurate position can be determined.  It is important that in doing this procedure the attention be carefully divided in and out of the plane.  Skill in getting two bearings with one VOR requires practice.  Avoid having the aircraft cover too much distance, thus voiding the accuracy. If ceilings force you below 1000' this process most likely will not be available to you.

With good weather, adequate fuel, and a controlled aircraft being lost is not an emergency.  Any one of the above when added as a negative factor to being lost creates an emergency.  The 4 C's, (climb, communicate, confess, comply) makes possible the addition of ATC brains which are free of 'lost' disabilities.  Climbing gives communications range, communication puts skilled ATC brains into the problem, confession relieves much of the pressure and presents others with the problem, compliance directs you toward solution.  If ever you fly off the edge of a sectional the 4 C's is the preferred solution.

One possible unpleasant side effect of being lost is that you may, unknowingly, be flying into a restricted area, ATA, TCA, game refuge, or worse.  It is not uncommon to have such flights met by military aircraft and escorted down for interrogation, tracked by ATC for investigation, or held by police authorities once on the ground.

One major difficulty in locating an airport while lost is that while you see the airport you don't know where it is, its name, or requirements.  Try to over fly in such a way as to avoid the possible pattern altitude.  If it appears to have a tower, try the universal FSS frequency of 122.2,  If you seem to be running out of options, try 121.5 or ground frequencies to get the tower frequency.  If your situation is critical don't hesitate to use a military field.  While technically no communications are required at an uncontrolled airport, some effort should be made.  Use standard patterns and approaches as much as possible.

An off airport landing greatly increases the probability of aircraft damage or injury.  However, if this becomes the only viable alternative do not hesitate.  Plan the landing while visibility, weather, and fuel permit several efforts.  Make low passes over the selected field to determine the best use of terrain and wind.  Use your CHECKLIST and be sure to prepare the cockpit both for landing shock and escape. Make as near normal approach as possible; landing at lowest possible ground speed.

See instructional material on getting found and locating.

VIII.  AREA OF OPERATION

SLOW FLIGHT AND STALLS

# D.  TASK:  MANEUVERING DURING SLOW FLIGHT

REFERENCES:  AC 61-21, OPERATING HANDBOOK, FLIGHT MANUAL

P    1. Knows specifics of entering and flying in slow flight.  Aware of 		reasons for slow flight competence and hazards existing.

P    2. Selects entry altitude that will preclude descent no lower than 		1500’ or as assigned.

P    3. Stabilizes the airspeed at 1.2 Vs1, + 5 knots.

P    4. Makes coordinated straight and level flight and level turns, at 		bank angles and in configurations, as specified by the 			examiner.

P    5. Accomplishes coordinated climbs and descents, straight and turning, 		at bank angles and in configurations as specified by the 			examiner.

P    6. Divides attention between airplane control and orientation.

P    7. Maintains specified altitude +100’, specified  heading + 10°, and 		specified airspeed +10/-5 knots.

P    8. Maintains the specified angle of bank, 30° maximum in level flight, 		+0/-10°, maintains the specified angle of bank, not to exceed 		20° in climbing or descending flight, +0/-10°, rolls out on the 		specified heading, + 10°, and levels off from climbs and 			descents within + 100 feet.

EX    This constitutes flight where any inattention to bank, airspeed, or 	attitude will produce a stall.  A minimum safe altitude might be 	3000'.  Climbs, descents, and level flight with turns may be 	directed with altitudes to be held within 100'.  The examiner may ask 	for either or both explanation and performance.  Not below 1500 feet    	AGL, 1.2 Vso +5 knots airspeed, altitude + 100 feet, heading + 10 	degrees.  Maintains specified bank, not to exceed 30 degrees, + 0 	degrees and -10 degrees, climbing or descending flight not to exceed 	a 20 degree bank at + 0 degrees and -10 degrees.  Rolls out on 	headings + 10 degrees, levels off in climbs, descent within 100 feet.

Initiate the configuration by using CarbHeat, reducing power, holding 	heading and altitude, and trimming for minimum control pressure.  Any 	bank may require 	power to maintain altitude.  Be smooth and careful.  	Full flaps may require full power.

It will be next to impossible to fly this configuration without full use of trim and knowledge of aircraft performance. For the most important element is the smoothness of entry.  You must be able to get to the desired speed while holding heading and altitude, hands off.  Once attained the speed can be held throughout any turn or maneuver if the bank is 30 degrees or less and power is applied in anticipation of loss of lift.

C-150

No flap minimum controllable:    Full flap minimum controllable

CarbHeat                     CarbHeat

1500 RPM                     1500 RPM

Hold Hdg/Alt                 Hold Hdg/Alt

Trim Down 4                  Full Flaps

2000 RPM/Rudder              Full Pwr/Rudder

Power as required            Trim up 1

55 knots or slower           Power as required

40 knots or slower

If a steeper than 20 degree bank is called for then power should be used to compensate for loss of lift.  bank requires careful attention.  You will be constantly making small changes in the yoke and rudder.  More often than not you will be flying in a cross-control condition.  You might find that the examiner will accept a greater speed" like 60 knots without flaps and 45 knots with full flaps.  Remember any change in power will affect the trim and the airspeed unless trim is corrected.  Discuss with the examiner what he feels to be desired before you get into the plane.  You might even ask him to demonstrate.  Can't hurt; might help.  Listen, the sound will be your first indication of things going wrong.

See instructional material on slow flight.

# B.  Task: POWER OFF STALL:

REFERENCES:  AC 61-21, AC 61-67, operating handbook, flight manual

P     1. Knows aerodynamics of power-off stall.  Discusses aerodynamics

of a stall which occurs as a result of uncoordinated flight. 		(Spin entry)  Emphasis shall be placed upon recognition of and 		recovery from a power-off stall.  Recognizes and recovers from 		power-off stall.

P     2. Selects an entry altitude that allows all maneuvers to be above 		1500’ AGL, or as recommended by examiner.

P     3. Establishes a stabilized approach in the approach or landing 			configuration, as specified by examiner.

P     4. Transitions smoothly from the approach or landing attitude to the 		pitch attitude that will induce a stall.

P     5. Maintains a specified heading, + 10°, if in straight flight; 			maintains a specified angle of bank not to exceed 30°, +0/-10° 		in turning flight, while inducing the stall.

P     6. recognized and announces the first aerodynamic indications of the 		oncoming stall, i.e., buffeting or decay of control 				effectiveness.

P     7. Recovers promptly after a stall occurs by simultaneously 			decreasing the pitch attitude, applying power, and leveling the 		wings to return to a straight-and-level flight attitude with 		a minimum loss of altitude appropriate for the airplane.

P     8. Retracts the flaps to the recommended setting, retracts the 			landing gear, if retractable, after a positive rate of climb is 		established; accelerates to Vy before the final flap removal, 		returns to the altitude, heading, and airspeed specified by the 		examiner.

EX  Able to explain what causes an aircraft to stall, drop a wing, spin, and recover.  Able to discuss flight circumstances where an accidental 	or intentional power off stall might occur.  Able to demonstrate power off stall entry and recovery.  Able to demonstrate from level or bank in full or partial stall.  With or without flaps.  All flight to be no lower than 1500 AGL.

Back side of power curve, initiated from stabilized approach landing 	configuration, induce stall while maintaining heading within 10 		degrees,  or in a bank + 0 and -10 degrees with no more than 30 	degree bank.  Recognizes and announces first buffet or decay of 	control authority, recovers promptly after stall occurs with minimum 	loss of altitude.  accelerates to Vy before final flap retraction, 	returns to 	altitude, heading and airspeed specified by examiner.

Establishes landing configuration specified, transitions to stall,

recovers by pitch, power, rudder, minimum loss of altitude.  retracts 	flaps and climbs at Vy as directed.  No secondary stall, excessive 	altitude loss, spin, or flight below 1500 AGL shall occur.

## Stalls

When we do stalls we are seeking recognition and prompt, proper recovery.  You, the student, are being taught awareness.  How you become aware depends on many factors.  Most important are the  variations of the center of gravity that will affect how a stall is entered and recovery accomplished.   As part of the preliminaries the examiner will ask you to perform a weight and balance for the aircraft.  Based on the results you may be expected to discuss how the aircraft will stall and recover based on this information.  You should be able to answer basic questions as to how the weight and C. G. affects the aircraft's stall characteristics.  Answer the question asked.  Don't dig yourself into a hole by trying to explain more than you know.

The examiner is looking for recognition.  He wants to know if you can recognize and distinguish between an incipient, a full stall, and perhaps an aggravated stall.  He will observe your entry into the stall as to speed, heading, bank, and control usage.  Use of the rudder is of Particular interest.

You may be questioned as to your knowledge of stall recovery.  The proper sequence is of particular importance.  Do you try to raise a wing by using aileron?  Did you use rudder to keep a wing from dropping during the incipient phase?  Did you initiate your climb without exceeding Vy?  how do you handle distraction?  Your instructor will try to expose you to all of these learning opportunities.

Don't hurry.  Talk your way into, through, and out of the stall.  At no point in this section does the text discuss CLEARING TURNS.  Yet, failure to clear the area automatically fails the applicant. A very common fault of clearing turns is failure to make at least a 90 degree left and right turn while maintaining the same altitude.  In a C-150 any stall can be achieved with only two fingers behind the yoke.  A full grip on the yoke tends to tighten.  This gives the impression of pull and movement but is deceptive.  It leads to inadvertent steep banks and down pressure on the yoke shaft.  Use two fingers and pull up for the last 4 inches of movement.

Do not use trim during stalls unless the examiner makes a specific request.  Know how to make a smooth entry.  The smoother the entry the more control you will have over the stall.  Without flaps, apply Carb Heat, power off, while holding heading and altitude.  When 60 knots is reached slowly and smoothly apply back pressure, yoke, and rudder.  If a bank is required, do not let the bank exceed 20 degrees.  Use the minimum bank allowed if you can hold it correctly.  A common fault in all stalls that require bank is to let the bank angle increase.  Remember to take out bank pressure on yoke gradually as the pitch and angle of attack increases.  Holding the same yoke angle as entry will cause bank angle to increase as yoke is pulled back.  While in the bank an incorrect application of rudder may cause one wing to drop abruptly.  This is a stall and may occur without the horn going off. The instinctive application of up-aileron will only make the situation worse.

At the stall, regardless of whether its one wing or the nose, relax yoke pressure to allow the nose to go to or slightly below the horizon. Level the wings and apply full power with appropriate rudder.  Set a climb attitude at best rate of climb.  (About 65 knots)  Prior to beginning stalls ascertain how far the examiner wishes you to continue your recovery climb.  If directed to use flaps, put in the assigned amount soon after the white arc is reached.  Hold heading and altitude until 50 knots or lower is reached.  Apply yoke pressure and bank with more generous rudder very gently and slowly.  The stall break will be more abrupt with flaps but the recovery proceeds the same.  As soon as power is applied start bringing up the flaps to 20 degrees and then milking.  The most common faults of the flap stalls is failure to bring up the flaps and entering a secondary stall.  With full power applied the full flaps secondary stall will probably lead immediately to a spin.

A full power, full flap spin requires that power be taken totally off and flaps raised before normal recovery procedures are initiated.  Since a one turn full spin in a C- 150 can lose 1000 feet it is best never to do stalls below 3000 feet.

See instructional material on stalls.

# B.  TASK: POWER-ON STALLS

REFERENCES:  AC 61-21, AC 61-67 operating handbook, flight manual

P    1. Knows aerodynamics of power-on stalls and effect of uncoordinated

use of controls.  This shall include an understanding of the 		aerodynamics of a stall which occurs as a result of 				uncoordinated flight.  (Spin entry)  Emphasis shall be placed 		upon recognition of and recovery from a power-on stall. 			Recognizes and recovers from 	power-on stall.

P    2. Select entry that will complete all maneuvers above 1500’ AGL, or 		recommended altitude, which ever is higher.

P    3. Establishes takeoff or departure , configuration, airspeed and 			power specified by the examiner.

P    4. Transitions smoothly from the takeoff or departure attitude to 			pitch attitude what will induce a stall.

P    5. Maintains a specific heading +10°, if in straight flight; maintains 		a specified angle of bank not to exceed 20°, +0/-10°, if in 			turning flight, while inducing the stall.

P    6. Recognizes and announces the first aerodynamic indications of the 		oncoming stall, i.e., buffeting or decay of control 				effectiveness.

P    7. Recovers promptly after a stall occurs by simultaneously decreasing 		the pitch attitude, applying power as appropriate and leveling 		the wings to return to a straight-and-level flight attitude 		with a minimum loss of altitude appropriate for the airplane.

P    8. Retracts the flaps to the recommended setting, retracts the 			landing gear, if retractable, after a positive rate of climb is 		established; accelerates to Vy before the final flap removal, 		returns to the altitude, heading, and airspeed specified by the 		examiner.

EX Explain the effect of power on the stall.  Show how the failure to 	use adequate rudder to compensate for power affects the stall.  	Discuss the flight situations where such a stall might be expected.  	Explain recovery as being by pitch, power, rudder, minimum loss of 	altitude. retracts flaps and climbs at Vy as directed.  No secondary 	stall, excessive 	altitude loss, spin, or flight below 1500 AGL shall 	occur.

The student will establish the aircraft in a takeoff/departure 	  	configuration at approximately lift-off speed or as specified by 	examiner.  A bank may or may not be required. The amount of power is 	usually specified as full except during entry.

No lower than 1500 feet AGL, takeoff or departure configuration, induce the stall while maintaining heading within 10 degrees or in a turn no more than twenty degrees +0 or -10 degrees.  Recognizes and announces buffet, first signs of control decay, recovers as stall occurs.  To Vy before final flap retraction, returns to altitude, heading, and airspeed specified by the examiner.

This is the stall likely to occur during take off. In higher powered aircraft a trim set for landing assist can precipitate this stall.  This is a good reason to check trim prior to takeoff.  A bank assumes an effort to avoid an obstruction.  Limit bank to 20 degrees since a 30 degree bank is more likely to lead to misuse of the rudder and a wing drop as consequence.  The slower your entry speed the less pitch up when power is applied.  Anticipate all power applications with rudder since applications are more likely to lag than lead.

Again, CLEARING TURNS.  Talk through the entry, stall, and recovery.  Talking explains to the examiner how you understand the process.  CarbHeat, Power to 1500 or as specified, slow to lift off of 50 knots, hold heading and altitude, full power (rudder), and bank as directed.  If you are not directed to bank, don't.  Since the text will allow as little as a 10 degree bank, use that, but no less if so directed.

Slowly, smoothly but constantly, increase the back pressure.  Apply rudder as the pitch attitude increases. The aircraft will gain altitude, may attempt to increase bank if a bank is required.  The important element is to keep applying pressure until the stall occurs.  Any abrupt back pressure will aggravate rudder errors and cause sharp wing drop. All too often pilots fail to get full back and up yoke and so continue to fly the plane at the edge of the stall.  Don't forget the yoke is not all the way back until you pull it up.  As pitch attitude increases so will required rudder pressure.

Since the power dramatically increases the pitch attitude, the stall break will be abrupt.  Allow the nose to drop to or slightly below the horizon, level the wings, confirm full power, and initiate climb at best rate.

See instructional material on stalls.

# Imminent stalls -

The major difference between this stall segment and those preceding is the "depth" of the stall.  In this procedure regardless of the configuration the recovery is initiated at onset.  In the preceding segments no recovery was to be initiated until the actual "break" had occurred.  The initiation as before begins with CLEARING TURNS.  After obtaining the assigned airspeed, flap setting, power, and bank, commence the entry.  The more slowly you enter the stall, while holding heading/altitude as required, the quieter and more easily heard and felt will be the signs of the stall.  If anything, the imminent stall should be approached more gently than a full stall.

The recovery of the imminent stall can be a very smooth gentle process with final recovery in a climb configuration.  Again discuss with the examiner how far he wishes you to hold the climb. All disqualifying elements of the full stall apply to an imminent stall.  Since the emphasis here is on recognition, letting a full stall develop would probably be disqualifying.

Stall Recognition

The stall is because of the angle of attack not the airspeed or attitude.

a. Mushy controls

b. Change in pitch of exterior air flow

c. Buffet, vibration, pitching, sounds

d. Stall warning

e. Body sensing

See instructional material on stalls.

# D. Task: SPIN AWARENESS

REFERENCES;  AC 61-21, AC 61-67, operating handbook, flight manual

P    1. Knows flight situations where unplanned spins may occur.

P    2. Recognizes unintentional incipient spin and knows procedure for 		recovery

P    3. Explains recovery procedure for aircraft used for the practical 			test.

SPIN AWARENESS REQUIREMENT

Flight situations where spins may occur, recognize recovery techniques, recovery procedure for aircraft used on the test.

An established spin is an aggravated stall with developed autorotational forces.  Gravity causes a helical path during the descent.  The aerodynamic and inertial forces are balanced.  You must upset this balance to recover.  The stall which has directional control occurs at an angle of attack near 17 degrees.  In a spin the down wing may have an angle of attack of 70-80 degrees while the up wing will be between 30 and 40 degrees.  The stall prelude to the spin lacks directional control (rudder).

ANGLE OF ATTACK   Angle the wing chord line has when it meets the relative wind.  The CRITICAL ANGLE OF ATTACK  Will always result in a stall.  The flow of air cannot follow the curve of the wing.  For a given flying surface the critical angle of attack never changes.  The airspeed at which a flying surface will stall depends on aircraft weight, balance, and load factor.

RELATIVE WIND   Speed and direction of wind causes by the aircraft movement.  Velocity of relative wind is equal but opposite to speed of aircraft.

Normal category aircraft are placarded against spins but they are factory/FAA tested to one turn (or three seconds) with a one-turn recovery when controls are used correctly.  Speeds and load factors are not to be exceeded.  Flaps may be retracted.  No uncontrollable spins possible.  No acrobatics or spins allowed.  The weight and CG certification of the aircraft will determine its spin characteristics and recovery procedures.  Beyond one turn even correct control input may not work.

Normal category aircraft that are placarded against spins must be considered as being non-recoverable from spins.  Certification today does not include ability to recover from a fully developed stabilized spin.  The required one turn recovery is only the incipient stage of the spin.  Aerobatic aircraft have not been tested beyond six turns.

# Sources of Inadvertent Stalls Becoming Spins

1. Inadequate rudder application in steep climbs.  Climb power raises 	  nose, decreases airspeed and increases P-factor.

2. False concept of air speed when on base due to a tailwind. 	  	 	  Peripheral vision can sense the illusion and send it to the brain.

3. Letting nose pitch up and airspeed drop when applying	power.

4. Having aircraft enter 'reverse command' area in pattern.  A speed so 	  slow and power so high that the only recourse is to lower the nose 	  for flying speed.

5. Use of rudder to increase rate of turn from base to final.  Keep ball 	  centered during turns.

6. Distractions from primary purpose of aircraft control.  This is now a 			required part of the flight test.

7. Trying to stretch a glide at a speed below best glide speed. If you 			don't know best glide, use best climb.  It will be close.

8. Attempting to return to runway after engine failure.  Practice at 			altitude and then add 50% fudge factor.

Spins are not caused by stalls but once a stall occurs where the pilot includes or fails to include, rudder, aileron, or power individually or in combination.  Auto rotation occurs from an asymmetrical stall and a side slip.  There is an abrupt loss of control when leaving the stall and entering the spin.  The untrained pilot will always react instinctively and apply controls incorrectly thus aggravating the spin entry.

An incipient phase occurs when the foregoing  stall is accompanied by uncoordinated yawing.  The yaw induces a roll due to increased asymmetric lift on the wing opposite to the applied rudder.  The aerodynamic differences from uncoordinated stalled flight causes the nose to drop.  The autorotation to follow is quite varied as long as the dynamic and inertial forces are unbalanced.  Airspeed will be changing but the faster the entry the longer it takes to stabilize the spin.  By the second turn we may be in a developed (stabilized) spin.  IAS will be pegged a few miles above 1G stall speed.  Descent will vary but can reach 7500' fpm.

Know how to recognize the beginning of a spin.  Quickly apply the proper control input.  Get out of the incipient spin before it has a chance to develop. In a developed spin an aircraft prohibited for spins may be unrecoverable.  To prevent this from occurring the recognition and recovery from an incipient spin is a desirable training goal.  The first turn of a spin causes the greatest loss of altitude, as much as 800' to 1000'.  High density altitude causes faster rate of spin and greater loss of altitude.

Recovery:  Immediately, power off, opposite rudder, forward on yoke always in this sequence. If rudder is effective yoke forward may not be necessary.  Otherwise, hold full application of controls until recovery.  Check turn coordinator for direction if in doubt. The proper recovery from the incipient spin must be initiated at once or the yaw rate will become faster and the nose more toward the vertical.

# Endorsement For Spin Awareness

(Required logbook endorsement)

I certify that I have given _____holder of pilot certificate # ________ ground and flight instruction  in stall awareness, spin entry, spins and spin recovery technique and find that he/she meets the knowledge requirements required under FAR 61....see above.

Dated...

Signed...

IX.  AREA OF OPERATION

FLIGHT BY REFERENCE TO INSTRUMENTS

# A. Task: STRAIGHT-AND-LEVEL FLIGHT

REFERENCES: AC 61-27, AC 61-23

INSTRUMENT MANEUVERS PERFORMANCE PARAMETERS

Within + 200 feet of altitude, 20 degrees of heading, and 10 knots of airspeed.  Straight and level, constant airspeed climbs and descents with turns to headings.

P    1. Exhibits knowledge of the elements related to attitude instrument 		flying during straight-and-level flight.

P    2. Flies straight-and-level with only reference to instruments. Uses 		basic cross-check	interpretation, scan, trim, control for 			straight and level flight using instruments only. Smooth 			coordinated control

P    3. Maintains altitude + 200’, heading + 20°, airspeed + 10 knots.

EX   Primary instruments are heading indicator, attitude indicator and 	airspeed.  Secondary are turn coordinator, altimeter/vertical speed 	indicator, and power.

A light touch on controls of properly trimmed aircraft is a necessity.

The basic skill of VFR trimmed for straight and level is essential.  Trim must be set in anticipation for acceleration/deceleration and then fine tuned.  A full fist grip on the yoke makes it difficult/impossible to feel yoke pressures requiring trim.  A properly trimmed IFR aircraft requires a locked-elbow light finger pressure pilot.  Changes in power, however slight require retrimming.  You might note the location of the trim indicator as a reference index.

It is important that the eyes not fixate but keep moving.  Use the top center markings of the attitude indicator to level the aircraft.  If a wing is low you are turning. If you find yourself in a turn, first level the wings and then enter the correcting turn.  The correcting turn should be set on the attitude indicator for a standard rate according to airspeed. A C-150 at cruise will be about 12 degrees; in slow flight only 10 degrees of bank.  Check your bank against the turn coordinator to see if it is standard rate.

As a beginner you might try using a count system for returning to desired heading.  If you are 10 degrees off, enter a 10 degree bank for a 4 count and level off, for 20 degrees count 8 etc.  You must be aware that VFR banks are usually steeper than IFR with greater back pressure required. For this reason, inexperience under the hood tends to cause overbanking and climbs.  Just as how you hold a Ping-Pong paddle will set limits to proficiency, so will how you hold the yoke.  Keep it light.

See instructional material on hood.

# B. Task: CONSTANT AIRSPEED CLIMBS

REFERENCES:  AC 61-21, AC 61-27

INSTRUMENT MANEUVERS PERFORMANCE PARAMETERS

Within + 200 feet of altitude, 20 degrees of heading, and 10 knots of airspeed.  Straight and level, constant airspeed climbs and descents with turns to headings.

P    1. Knows basic instrument interpretation, scan, trim, and control for 	 	  straight, constant speed climbs.

P    2. Will establish the specified climb configuration by the examiner.

P    3. Transitions to the climb pitch attitude and power setting on an 		assigned heading using proper instrument cross-check and 			interpretations, and coordinated control application.

P    4. Demonstrates climbs solely by reference to instruments at a 			constant airspeed to specific altitudes in straight flight.

P    5. Levels off at the assigned altitude and maintains that altitude

+ 200’; maintains heading + 20°, maintains airspeed + 10 knots.

EX    Explain how instruments will appear initially and when established.  	Discuss control applications and inputs required to hold airspeed and 	heading.

From level trimmed cruise flight, raise nose 1/2 BAR on attitude indicator, apply full power and right rudder trim down one full turn.  Increase rate of scan to cover top index of attitude indicator and heading indicator.  Correct by count system suggested above.  Check airspeed and fine trim as required.  Do not fixate.  During any change deliberately increase your rate of scan. To level off, first lead altitude by about 50 feet, center BAR trim up one and hold altitude/ BAR while plane accelerates.  Rudder pressure is best for heading control initially.  Fine trim as required at cruise power.  Instructor may be moving seat to affect trim. Distractions are allowed.  A well trimmed plane will not get too far off.  Do not let yourself be hurried.  If you "lose it" level the wings, turn to heading, climb/descend to altitude.  As a beginner do not try to do two of these at once. Fly with a light touch and trim for feel.  A change in sound is one of the first and best clues you have when things are changing. Listen carefully, learn the sound of your plane.

# C.  Task: CONSTANT AIRSPEED DESCENTS

REFERENCES: AC 61-21, AC 61-27

INSTRUMENT MANEUVERS PERFORMANCE PARAMETERS

Within + 200 feet of altitude, 20 degrees of heading, and 10 knots of airspeed.  Straight and level, constant airspeed climbs and descents with turns to headings.

P  1. Knows and can discuss basic instrument interpretation, scan, trim, 		and control for straight, constant speed descents.

P    2. Establishes the descent configuration specified by the examiner.

P    3. Transitions to the descent pitch attitude and power setting on an 		assigned heading using the proper instrument cross-check and 		interpretation, and coordinated control applications.

P    4. Demonstrates descents solely by reference to instruments at a 			constant airspeed to specific altitudes in straight flight.

P    5. Levels off and maintains assigned altitude + 200’, heading + 20°, 		and airspeed  + 10 knots.

EX    Explain how BAR will read differently if aircraft descends at cruise Vs nose high slow flight.  Know how to determine minimum/maximum altitudes.

At cruise speed the easiest descent is made by Carb Heat and 100 RPM changes in power to select the desired rate of descent.  Be careful not to over-react by making power and yoke changes together.  Be patient and let the plane settle down after making a change.  Remember to increase your scan rate.  A half BAR will give about 500 feet per minute.  If the descent is called for at 60 knots, set power and trim as for landing from level cruise, let the plane slow down to 60 by holding altitude.  At 60 ease up on the yoke and maintain 60 just as for landing.  Don't hurry.  Keep a light touch and fine trim as required.

Minimum altitudes are determined by the FAR's or may be assigned by ATC.  Absolute minimum altitude is that which will permit a safe landing.  500' clear of people in farm areas. 1000' in populated areas. 3001' over an ATA.  There is no minimum in an emergency or with intent to land. Maximum altitudes may be assigned by ATC.  700' in magenta area and 1200' in blue areas where visibility of 3 miles and proper cloud clearance cannot be maintained.  The best way to understand how this really works is to make a SVFR flight with an instructor.  Radio and transponder requirements enter in at higher altitudes.

# D.  TASK:  TURNS TO HEADINGS

REFERENCES:  AC 61-21, AC 61-27

INSTRUMENT MANEUVERS PERFORMANCE PARAMETERS

Within + 200 feet of altitude, 20 degrees of heading, and 10 knots of airspeed.  Straight and level, constant airspeed climbs and descents with turns to headings.

P    1. Knows and can discuss basic instrument interpretation, scan, trim, 		and control for turns to headings

P    2. Transitions to the level-turn attitude using proper instrument 			cross-check and  interpretation and coordinated control 			application.

P  3. Demonstrates turns to headings solely by reference to instruments; 		maintains altitude + 200’, maintains assigned final heading + 		20°, maintains airspeed + 10 knots.  25 degree bank under the 		hood is disqualifying in first edition of test.

EX    Explain that the higher your speed the greater degree of bank 	required to make a standard rate turn.

At cruise, in a C-150 about 12 degrees on the attitude indicator will approximate a standard rate turn. Use 1/2 the angle of bank as the lead to catch a desired heading. If improper rudder pressure is applied during or after the turn the yoke input pressures required to execute will be improper, also.

Practice making 30 second standard rate turns from cardinal headings.  Do the turns by time and the turn coordinator.  You may find that the turn coordinator markings are not accurate.  Once you have established the turn coordinator setting relate that to the degrees of bank on the attitude indicator.  You should calibrate each aircraft you fly in this manner for more precise flying.  In any event, set the bank on the attitude indicator according to your airspeed, check the turn coordinator and keep a light touch. Experience has shown that any time a VFR pilot puts more than one finger behind the yoke while on instruments he will exert climb pressure.

During a turn under the hood the first sign of things going wrong will be a change in sound.  Deliberately increase the speed of your scan just before entering a turn and when leveling out.  It helps to develop a sense as to how many degrees your assigned turn has.  This makes it possible to eliminate the heading indicator from your scan for a period of time during the turn and bringing it back when it is about time to level off.

When you level off from a turn you will frequently succumb to the tendency to turn back in the other direction and gaining altitude.  This is a normal reaction to your inner ear sensors.  Prevention is easiest accomplished by momentarily locking on to the top index of the attitude indicator and scanning it with the heading coordinator. This gives the inner ear time to settle down. Then resume normal scan.  Small heading changes can best be made by a count system.  For a ten degree correction, enter a 10 degree bank on the attitude indicator, count 3 or 4, and level off.  Remember the rudder.

# E.  Task: UNUSUAL FLIGHT ATTITUDES

REFERENCES:  AC 61-21, AC 61-23, AC 61-27

RECOVERY FROM UNUSUAL ATTITUDES PARAMETERS

Nose up recovery by using pitch down, add power and level wings.

Nose down recovery by reducing power, leveling wings, and pitch up to level flight.

P    1. Knows and discusses basic instrument interpretation, scan, trim, 		and control for recovery from unusual attitude conditions.

P    2. Recognizes unusual flight attitudes solely by reference to 			instruments; recovers promptly to a stabilized level flight 		attitude using proper instrument cross-check and 				interpretations and smooth, coordinated control application  in 		the correct sequence.  Uses instruments to recognize and 			recover from unusual flight conditions and returning to 			stabilized level flight.

EX Able to explain how vertigo occurs and the use of flight instruments to 	gain/regain control of the aircraft.  Able to interpret aircraft 	sounds and instrument positions indicating a critical attitude. 	Recovers to level flight by prompt, smooth, coordinated control, 	applied in the proper sequence.  No excessive load factors, 	airspeeds, or stalls.

FAR 61.107(a)(6) requires that you receive simulated IFR flight instruction.  It does not state how much.  The examiner is to test you for at least 3 minutes at straight and level. You are expected to maintain heading within 10 degrees altitude within 100', and airspeed within 10 knots.  Since these are maximum limits, cut them in half for training purposes.

As indicated in previous material, there are many ways to create vertigo and illusions.  The examiner will try to have you react instinctively rather than with planned deliberate response.   He cannot pitch more than 10 degrees or bank more than 45.  You are expected to recognize and correct the situation by returning to level cruise flight.  While doing this you are expected to react appropriately, smoothly, and in the correct sequence without developing excess speed, load factors, or stalls.

The student should relax for the ride and listen carefully to the sounds of the aircraft.  When the examiner hands over control of the aircraft the sequence should be.  If airspeed is high, power off.  Low airspeed, full power. Focus on the attitude indicator FIRST.  Level the wings on the BAR.  Check trim and flap positions.  Set flaps and trim as required with power for normal cruise.

# F.  Task:  RADIO AIDS AND RADAR SERVICES

REFERENCES: AC 61-21, AC 61-23, AC 61-27

RADIO, NAVIGATION, AND RADAR PERFORMANCE PARAMETERS

Follows verbal instruction from ATC while flying aircraft  at safe altitude flown + 200 feet, within 20 degrees of heading and 10 knots of airspeed.

P    1. Knows and discusses the uses and limits of radio communications, 		radar and navigational aids available by pilots flying by 			reference to instruments.

P  2. Selects, identifies, and uses ATC facilities.

P  3. Follows verbal instructions and/or navigation systems/facilities for 		guidance.

P    4. Flies safe altitude and headings using radio instructions, radar or 		Nav/Aids

P    5. Levels off and maintains altitude + 200’, assigned final heading + 		20°, airspeed + 10 knots.  (25 degree bank under the hood is 		disqualifying in  first edition of test.)

EX    Show knowledge of com/nav radio frequencies, limitations, and 	 	terminology.  Show similar knowledge of radar/transponder use.

It is in this section of the flight test where bad habits are likely to make the difference.  Do not communicate unless the frequency is clear.  Plan what to say, say what you plan.  Identify navigational aids. (Something you have been neglecting since you first learned why it should be done) Know the basic frequencies.  Know how to get frequencies from the sectional, ATC, or FSS.  Know how to look at the sectional without holding the yoke.  Know how to acknowledge and follow ATC instructions VFR/IFR.

Under stress, bad habits will rise and rise again.  If you have developed a poor radio/radar procedure make and use a check list.  How to use the VOR, the transponder, airport departure/arrival all should be studied to determine if your under stress procedure is acceptable.