Cold conditionsã

Flight in Icing

Tail Stall

Survival Kit

Survival Basics

Fears in Survival Situations

Rules of ABC:

Crash Survival

Winter clothing:

Cold weather Flying

Preflight

- Because of the cold the cold weather pilot tends to abbreviate the preflight.  Don’t.

- Look for fuel dye as indicative of fuel leaks.

- Keeping the tanks full keeps out moisture.

- Ice in fuel looks like floating dust.

- Check engine cylinders and exhaust fittings for white stains.

- Prime 50% more than normal when its cold and 100% if really cold.

- Use manufacturer’s recommendations for cold weather operations

- If heat baffles are used check their security.  Is a baffle called 	for

- over oil cooler?

- Let engine warm up until oil thins out and pressure is normal.

- Crankcase breather hose and system must be free of moisture.

- Hoses, flexible tubing and seals become brittle in cold

- Battery and charging system should be in good condition

- A cold battery will be weakened unless fully charged

- Cold oil will be more viscous (thick)

- Control cables' lubricants will congeal.  Check throttle and CH.

- Don't use anything that works on an automobile to remove ice

- Deicing fluids cause corrosion and leave unpleasant residue

- Keep fuel tanks full and protect against contamination. Super-cooled fuel can create ice crystals and a rough engine.  C.H. helps.

- Preheat engine and cockpit prior to start when 10°F.  Heat under constant attendance and not directly on surfaces.

- Store in heated hanger until just before departure

- Don’t let water get on hinges or moveable parts.

- Clean and dry pitot tubes, heater intake, carburetor intake, control                       surfaces and wheels.

- Melt ice and remove all moisture which may freeze again.  Use hot water bottles or double-bagged plastic bags and plenty of dry towels

- Check weep-holes under aircraft and drain interior moisture.

- Confirm fuel selector is not frozen in one position.

- CO detectors are recommended in addition to heater inspection

- Operations

- Thirty seconds of cranking = 50 hours flight time wear

- Phone your destination just prior to departure

- Plan to walk home.

- Electrical loads will be heavier.

- Don't set parking brakes, hot brakes will freeze solid.

- Idle at higher rpm to keep plugs from fouling and oil hot

- Normal engine temperatures are required to evaporate moisture in crankcase.

- Change tanks more often so that maximum fuel is available.  Fuel

- selector might freeze.

- Flooded?  Mixture out, throttle full.  Crank 20 seconds.

- Fuel/air more reluctant to ignite. Carburetor heat may be required for    takeoff

- Don't use throttle to over prime (two strokes maximum). 	Use engine primer, not throttle.

- Most common cause of aircraft engine fires is prime with throttle

- Prime only with primer.  Over priming  can be harmful.

- If RPM rises when carburetor heat is applied it means that air

- filter is blocked

- Don't try to force the warm-up

- Gyros will need to warm up and speed up, too

- Control cables will be tighter and stiff

- Don't hand prop.

- Keep wheels and brakes as dry as possible

- May need to remove and dry plugs if engine fires only briefly.

- Use radios only after electrical system has run a few minutes.

- Braking may be poor to nil

- Oil must be hot to perform properly. More viscous (thick) oil 	requires more cranking energy

- Fix problems early before they become ‘unfixable’.

- Consider landing gear up

- Use C. H. in icing range

- Use C. H on approach and descent

- Keep power up during descents and extend any drag that may be available.  This will keep the engine warmer.

- Taxi slowly, wheel bearings may be frozen, brakes may not hold.

- Don't use flaps if tailplane icing is at all likely.

- Don't use brakes until tires are on hard surface. Be prepared for

- ineffective braking. Touchdown areas of runways are more slippery.

Performance

- Performance on runway may be less.

- Reduce crosswind capability by 50% for snow and 75% for ice

- On getting airframe ice, change altitude.

- Climb through ice at high speed and shallow angle.

- Descend through ice at lower speed but high rate of descent.

- Climb above wet snow and freezing rain.

- Reduced windshield transparency makes surfaces appear lower than they are.  5-degree slope errors are common.

- If your planned altitude is within 4000' of z-level anticipate ice.

- Plan an alternate route around convective weather.

- Know the "times" as they apply to your flight. (fuel, battery, speed,                      	alternate, etc)

- Set power at cruise and don’t change anything.  Thus, if anything 		changes it means there is a change in power.  Suspect carburetor 		or induction icing.

Conditions

- Ramp operations can cause 'black ice' problems for taxiing.

- Weather systems move more quickly in winter. Fast-moving winter systems create turbulence and hazardous conditions.

- Just because no ice is forecast doesn't mean there won't be any.

- If you are flying in air only 10 degrees above z-level anticipate ice.

- Freezing weather can make a survivable problem non-survivable

- Where is the good weather?

- Expect strong winds from the wrong direction

- Snow changes the way things look

- Light icing over a time is a serious problem

- Ice usually exists under snow.

- Moisture on runway that gets on aircraft may freeze when airborne.

- Clouds dropping rain have less ice potential than those without rain.

- Top portions of clouds contain supercooled droplets.

# Flight in Icing

All exterior sensors, pitot, static, and  stall warning will be degraded as to sensing ability and accuracy.  Antennae efficiency can be changed.  I have had radio transmitter relays freeze in the cockpit so that they would not operate for transmission.  (We used a handheld)  Propeller unbalance is common.  Fly at faster speeds than normal in climb, descent and especially landing.  Most icing happens in visible moisture with temperature between zero and -14 C.  Less than 10% of ice accidents occurred when ice was worse than forecast.  No aircraft is certified to fly in moderate freezing rain.  Any flight into forecast icing is considered by the FAA as a violation of FARs 91.9 and 91.13. as exceeding operating limitations and being careless and reckless.

As ice accumulates and lift decreases, the pilot must increase the angle of attack to retain lift for level flight.  In this condition ice begins to accumulate beneath the wind and tail surfaces.  You can’t see ice below the wing in many aircraft.  Any de-ice capability should be used only to escape the icing, not to continue the flight.  You cannot afford to be casual while operating in ice.  You need to be current in

your skills related to unusual attitude recovery.

Carburetor ice is far more likely to occur and cause an accident than is

airframe icing.  51% of icing accidents are caused by carburetor ice or induction system ice.  The cause of this ice is the failure of the pilot to ANTICIPATE the possibility of ice by applying full carburetor heat and alternate air.  The fixed pitch plane will develop a rough engine while the constant speed plane is going to show a drop in rpm.   Under icing the C. H. will increase the roughtness of the engine.  Leave it on.  Use alternate air if available.

# Tail Stall

This stall can occur when ice accumulates on the bottom of the horizontal tail surfaces to such an extent that the flow of air ceases to give the required download.  The tail stall occurs on the bottom side of the horizontal tail surface.  This lack of air flow can be even further reduced with the application of flaps and an increase in airspeed. Flaps will increase the downwash angle of the over tail airstream and increase the negative angle of attack. This can result in an abrupt nose down attitude and descent.  The recovery is completely different from a normal stall recovery. Flaps are removed.  Power is reduced and the yoke is pulled back.  Pipers are subject to this more than Cessnas.

The do not use flaps if icing exists or is suspected.  You will not be able to see ice on the bottom of the horizontal tail but any ice accumulation will occur there first.  Fly the approach at a speed that will account for ice created drag but not extra fast that will increase the downwash on the tail surface.  More downwash increases the tail stall possibility.  So long as air flow stays attached to the bottom of the tail, it is flying.  You will have control and down pressure from the tail.

The full stall does not occur until both the low-pressure  (top) of the wing and the (bottom) of the tail surfaces lose all their lift.  (I urge you to think through the "download" effect of the tail surfaces since it is often part of the Practical Test oral.) This doesn't happen.  With all low pressure surfaces having some effect the aircraft does not go straight down.  The tail is being held down somewhat by its low pressure.  With ice on this surface it will have reduced effectiveness and power.  If flaps move the center of lift for the wings backward an abrupt nose down pitch is most likely to occur.

The tailplane will collect ice more readily than the wing.  The collected ice will more adversely affect the tailplane's downward loading

than would the same amount of ice on the wing.  Ice may be on the tail before appearing on the wing.  Boot deicing on the tailplane is less effective than on the wing.  An icing tailplane stall may be unrecoverable.  The icing you get may be quite different from the icing used for certification.  the tailplane may be colder than the outside air temperature

(OAT)

Advice:  Don't use flaps if you suspect ice.  Don't mess with ice.

If the freezing level is on the ground, you won't be able to descend to warmer air.

# Survival Kit

1. Textbook				16. Compresses

2. Waterproof matches/tinder	17. Tapes

3. Water-flexible package	18. Aspirin

4. Space blankets/tarp		19. razors

5. Multi-Knife			20. Signal mirror

6. Fire starter			21. 5-day rations

7. Large trash bags		22. Bouillon cubes

8. Flashlight                 23. whistle

9. First aid kit              24. Survival saw

10. Take compass from plane   25. Survival candle

11. Insect repellent          26. Chemical lightsticks

12. Sun screen/balm		27. Gloves

13. Hacksaw blades		28. Hard candy

14. Vice grip/ pump pliers    29. Toilet paper

15. Sealable container        30. Razor blades

# Survival Basics

1. Get out NOW, take what you can.

2. Treat injuries, stop bleeding, keep warm, immobilize

3. STOP

S is for STOP

T is for THINK

0 is for OBSERVE

Knowing where you are is half the solution

Mark any trail used.

P is for PLAN

Select a leader, conserve energy, improvise.

# Fears in Survival Situations

The unknown, darkness, discomfort, being alone, animals, death, punishment, and personal guilt.

Enemies of Survival

Yourself, injuries, temperature, disease.

# Rules of ABC:

Survival is 80% mental, 10% equipment, and 10% skills.

1. Get what you need.  Fire materials off trees not ground - 3 times what is needed.  Get water - vital for survival(Use plastic containers)

Conserve energy lost due to shock of accident.  Rest before making survey of area and making survival preparations.

2. Inventory

Attract attention.  Three fires are distress signal.

3. Select alternative that takes least time, energy, material.

4. Make for safety and durability.

Shelter:

Aircraft hull may not be best choice.

Get off ground or under ground out of wind and wet.

Best bet is to stay with aircraft.

5. Desire for survival is essential.

Salvage aircraft parts as containers, remove compass, gasoline,

oil, upholstery, wiring, battery, hoses.

6. Heat water before drinking to conserve body heat.  Wear dry clothing

as much as possible.

# Crash Survival

#1 survival item is to let a responsible person know where you are 	going, your route, and ETA.  Insurance is to make contact after 		arrival.  If you are not found quickly, you are not likely to be 		found at all.

Survival items that are nice to have, short-term and essential for 		longer are:

Flashlight, digging tool, foul-weather gear, blankets, food, 		water, matches and candles.   A real plus would be a portable GPS.

a handheld radio, or a cellular phone.  Dehydration is the worst

threat usually facing a downed pilot.

# Winter clothing:

Combine high-tech and traditional materials.  It is easier to keep warm by wearing multiple layers.  Inner lawyer should not absorb moisture and wick it away from the body.  1/4 of body head leaves via the head.  waterproof headgear may be the most important single survival item.

Pilots are optimists that believe bad things happen to other people. They believe that flying is so safe that there is no need to be prepared for catastrophe.  Before leaving home you should prepare the 'what if' options.  Whenever you go flying you should be dressed to walk home. You should have required items within cockpit reach.  You should  stay with aircraft and survive until conditions allow foraging.  Short-term rescue is usually within a day.  The greatest danger is exposure to conditions causing hypothermia.  the right condition of wind, temperature, and moisture can expose anyone to hypothermia.  the body loses heat most rapidly when wet.

Rescue can be helped by making your position visible.  Make a clearing, start a fire, make large SOSs.  Use common sense, have confidence in yourself and be patient.  Prior planning, preparedness, practice and training will enable you to make the best decision possible.