Cross countryã

Cross Country Student Pilot Experience Requirements

Cross Country Checkride

Enroute in Brief

Less Than Perfect

Planning a Cross Country

BEGINNING:

Night Before a Cross Country

How to Get a Good Briefing

Using the FSS

Morning of a Cross Country

E-6B

Electronic Calculators

Plotter

Automation

Navigational Log

Suggested log procedure:

A Flight Log

Before a Cross Country

If You Go Weather Services

Flying a Cross Country

The Last Leg

Descent

Pilotage

Dead Reckoning

VFR on Top

Lost Procedures

Common Cross Country errors:

# Cross Country Student Pilot Experience Requirements

Each flight with a landing at a point more than 50 nautical miles from the original departure point counts as cross-country time.  Once flight must have landings at a minimum of three points, one of which is at least 100 nautical miles from the original departure point. FAR 61.109(b)(2) and 61.129(b)(3)(ii).  An instrument student must have 50 hours of cross-country time subsequent to getting the private pilot rating.

# Cross Country Checkride

Test covers pilotage, dead reckoning, radio navigation, diversion to an alternate and lost procedures.  Course must be long enough to check weight/balance and fuel calculations.  No help allowed and complete in 30 minutes.

PTS is said to allow flight to three miles either side of course. Altitude within 200'.  You should hold to less than 1/4 mile off-course before making a correction.  Radio navigation requires you to select and identify a navaid, locate your position, intercept and track a radial/bearing and note station passage.  Diversion to an alternate requires you to pass a knowledge portion during the oral.  Altitude and airspeed control is required as is prompt diversion to alternate with approximate heading, ground speed, fuel consumption, and ETA.  All oral information is covered in Chapter 7 of AC 61-23B and AC 61-21A.

Lost procedures are usually covered during the oral.  There are ways of creating "lost" situations in flight.  Be prepared to demonstrate sufficient skill and knowledge to select the best course of action.

# Enroute in Brief

The pre-departure self briefing suggests that the pilot to talk through and even walk through the flight.  This is even more vital for the inexperienced pilot or where the flight breaks new ground for the experienced pilot.  Make a frequency list of all the frequencies you expect to use in the order you expect to use them.  Mark up your sectional with lines that can be seen, magnetic course arrows, checkpoints, VOR radials, frequencies, pattern entries, obstacles and altitudes.   Weather, fuel, and aircraft factors are mentally and even orally reviewed.  Before entering the aircraft point the direction you expect to fly and recognize that you may be several miles off course on departure unless you manipulate ATC  (ask for a 270 overhead) to your benefit.

## Things to Have with You

Have available those things which may be needed.  Screwdrivers, screws, pliers, wire, crescent wrench, electrical tape, duct tape.  Oddly enough, I have most often needed these items for other pilots in need.  A supply of quarters and a cellular phone often come in handy.  Many airport pay phones will not allow incoming calls.

For cross country flying have a light weight emergency kit, Take appropriate emergency clothing.  Become familiar how various parts of the aircraft can become useful in emergencies.  Develop a checklist of necessary, nice to have, and luxuries for every type of flying you expect to do.  The longer its been since you flew cross-country, the more preparation you need to do.

### Airport/Facilities Directory

You will need a current Airport/Facilities Directory (A/FD).  These are published every 56 days and contains supplemental information about airports and nav aids that cannot be easily places on sectionals.  The FAA does not say that a pilot must have an A/FD, they do imply that you will be sorry if you don't.  1/3 of all VOR have direction/altitude restrictions that are spelled out in the A/FD.  Get one before you begin making your cross-country flights.  FAR 91.103 prescribes preflight action that can only be taken by having an A/FD.  Center frequencies are in the back.

Once Notams have been published in the A/FD they are unavailable in  routine briefings of NOTAMS you get by asking the FSS technician.  Sectional chart changes of frequencies, outages, noise abatement, closures, that become semi-permanent are only available in the A/FD Special Notices section.

Term to know: LOCIDS  = location identifier  every airport and navigational

aid has a series of letters and numbers that are unique.  Be careful with the

0 and O since they often are confused.

### The Pilot Operating Handbook

You will need a Pilot Operating Handbook (POH)specific to the year and model of the aircraft.  You need it to confirm that the airports, runway lengths, elevations, times and distances are within the performance capabilities of your aircraft.  FAR 91.151 indicates that you must have enough fuel to fly to your intended destination  + 30 minutes day and 45 minutes night reserve.   A fuel stop is not a waste of time, it provides time for you to make considered decisions and safer options.

The 30 minute fuel reserve requirement of FAR 91.151 could be considered an emergency reserve but you do not need to declare an emergency to use any of it.  If you have dipped into the 30 minute reserve and you have reason to believe that some FAA type has become aware of it, you might be well advised to file a report with the NASA Aviation Safety Reporting System at PO Box 189, Moffett Field, CA 94035-0189.  If the incident was unintentional such a filing will get you off the FAA hook.

When fuel tanks read empty the FARs say you must be out of gas.  Before reading empty you may have reached a minimum fuel condition.  Minimum fuel means you must proceed to a landing without delay.  	If you should reach a low fuel state, give ATC a minimum fuel advisory.  This is not an emergency and does not give priority ATC handling.  If you require priority you must declare an emergency.  The AIM says that when you have reached the minimum fuel level that requires priority handing you should declare an emergency.  This means the FAA will probably investigate to see if you have the required minimum fuel aboard for day or night.  All of these 'problems' can be avoided by never passing by an opportunity to add fuel.  AIM 5-84

The aircraft performance figures are adjusted for the FAA standard day before entry in the POH. Prior to 1970 most handbooks acquired their performance figures by stripping all possible weight from the aircraft.  Even now it is safest to make a 20% change on the safe-side for anything given in the POH.  The aircraft is no longer new and dirt accumulated over the years will affect performance.

# Less Than Perfect

Making a mistake is permitted and even to be expected as long as it is recognized and corrected.  An unrecognized mistake invites all its relatives to the party.  Every flight has multiple opportunities for error to be compounded by error.  The better we avoid the initial problem the easier it makes the next step and the avoidance of the next problem.

Good risk-management decisions are made on the ground.  Knowing answers is only part of the solution.  Knowing when to get another opinion is another part.  Knowing where to go for the opinion is salvation.  There are few  perfect flights.  Most flights are less than perfect, yet they arrive safely and without damage.  Those pilots who do not learn from their mistakes are most likely to meet the same ones again when they bring their relatives.

# Planning a Cross Country

Pilot judgment is a process which produces a thoughtful, considered and pre-planned decision relating to the aircraft's operation along with the  action/inaction to go with that decision.  The ability to make the best decision involves searching for and establishing the relevance of all available information regarding a flying situation, specifying alternative courses of action and determining expected outcomes from each.  The more knowledge we have, the greater the resources on which we have to draw to evaluate a situation and specify a course of action.  Cross country flight situations are dynamic, and your ability as a pilot to recognize and react depends on how well you can think about what might happen and making your decision accordingly.  Like many other flying skills, your ability to plan a cross country is a gradually acquired skill.  With experience you will enter

into the cross country equation a completely new set of factors.

The range of your aircraft will be determined by flight time which is limited by fuel amount, less reserve, and rate of consumption.   Maximum  endurance is your best glide indicated speed flown at minimum power (about 40%) required to maintain altitude.  This indicated speed, for safety should not be less than 1.3 of stall speed.  This keeps you in the air the longest but to go the farthest you must fly on about 55% power.  From 65 to 75% power you get your optimum cruise speed.  Take your maximum range speed from the POH (best glide speed) multiply it by 1.3 and you get the optimum cruise speed.  Be sure to convert all calibrated speeds to indicated speeds for practical use.

- Maximum range is determined by how well you optimize cruise performance.

- Engine power is proportional to fuel flow.

- Fuel flow is proportional to engine power

- Maximum range varies with weight at a particular angle of attack.

- Maximum endurance varies with weight at a particular angle of attack.

- Maxim range is mostly independent of altitude.

- 99% of maximum range is achieved by flying 7% above maximum range speed.

- Maximum range speed must be adjusted for wind.

- Maximum range is achieved by flying at greatest lift to drag ratio.

- Make a plot of miles per gallon of all TAS in altitude range to be flown.  Convert selected average TAS to average CAS to IAS

- Maximum long range cruise of C-172 is 88 knots.

- 1% reduction in weight, reduces required thrust 1%, increases range 1%, and decreases required speed by 1/2%

- As aircraft weight decreases power must be lowered.

- Maximum endurance is obtained at lower altitudes.

- Fly faster into headwinds.  Make no changes until wind velocity exceeds 25% of maximum range speed.  Then for every 5 knots increase or decrease speed by 2 knots.

- Fly slower with tailwinds.  Take one knot off TAS for every two knots of tailwind.  Do this only until Maximum range speed is reduced by 20%.

- Wind determines a preferred altitude but generally the higher you fly the better your range.  True airspeed increases about 1% for every 1000'.   For short flights it is not economic to fly high because of time to climb factor.  When time to climb is over 15% of enroute time fly lower.  If a headwind reaches 20 to 25% of your true airspeed, increase your speed but expect decreased range.  To stretch your range slow to maximum range speed.

- Plan to navigate in straight lines where terrain and airports make such flight safe.  Often a dog-leg near the beginning or end of the flight will permit a much safer, lower, and airport vicinity routing.  Try to minimize flight time but remember fuel stop time can be avoided by slower flight speeds.  Again, a fuel stop does not waste time.  It buys decision making time and provide safer options.  Evaluate fuel remaining at every third checkpoint during a flight.  Compare your estimates with actual consumption every time you refuel.

- Of all the cross country planning decisions, the no-go decision is the most difficult to make and stick to.  We all want to fly.  The patience to accept that the figures just do not add up to equal a safe flight takes some seasoning to be come part of your planning.  Non-pilots seem all to ready to fly when they have absolutely no comprehension of the hazards involved.  You, the pilot, must recognize that only your judgment, training, and skill can make the safe decision.

# BEGINNING:

1. Locate destinations on sectional

2. Draw multiple lines /// between points

3. Starting from destinations try to locate checkpoints that are:

A. Evenly spaced so that only the first point after

departure is uneven.(This makes ETA easy)

____z-marker instead of straight lines might be a better way for writing info.

/

/___

Departure end                                                                Destination

Some compromise between a good checkpoint and equal spacing

may need to be made.  Usually  a slight change in distance can

make this system work.  Likewise, it is important that the    	    	    distances be measured only by using the scale along the vertical 	    	    lines of longitude.  The checkpoints are not required to be evenly 	    	    spaced but it does make fuel and time estimates easier. It is more 	    	    important that checkpoints be easily identified.

B. Three identifiable elements to confirm point

C. Use 60 degree+ VOR radial as backup for points

Checkpoint     Checkpoint

O.K area

angle too shallow 				angle too shallow

VOR

D. Preferably on pilots side of course line

E. Located so as to account for visibility, speed and

Flight conditions

F. Use VOR to/from departure/destination

1. Account for altitude/range/obstructions

2. Possible malfunction/ident required

3. Double check OBS/compass/heading indicator

4. Enter essentials on log and sectional

G. Fly an airport vicinity route.  Avoid desolate areas.

H. Start with FULL tanks. Use the POH to figure fuel consumption.

refuel to FULL at each stop and compare your estimates with 	 	    	    actual.  Start to revise POH figures to fit actual fuel used as 		    you make other flights until you get more realistic estimates.

### Optional-

Enter VOR frequencies and radials to course line through nearby airports 		as pre-planning for emergency diversion. Mark on sectional.

Use large Z markers

4. Make a scratch log of the checkpoints, distances, times

The distance between checkpoints becomes greater as speed and 	visibility become greater.

The distance between checkpoints is made identical beginning from your 	destination.  The distance to your first checkpoint will vary.  Using 	this system makes ETAs much easier to figure in your head.

Next to each checkpoint make a large 'Z'  with an arrow at each end.

Write the distance in each direction  on the arms of the Z.

Get as much essential information from your work sheets and logs on to 	the sectional

Design your own log sheet using ideas from other sources.  Your own 	log is going to be more practical and useful when you have designed it 	to fit into the way you do your planning and the information you will 	need.  Like a checklist this will take five revisions to develop a

functional log.  Start with everything and then begin to eliminate.

Checkpoints, magnetic courses, radio frequencies, altitudes, and 	pattern altitudes should be transferred to the sectional.

5. Make a scratch radio frequency card

A. Orally review on a tape recorder communications with ground, 				tower, FSS, radar, and flight watch

B. Mark sectional to show initial call up points

C. What if...plans

6. Mark sectional for fuel stops/tank changes, compass course

pattern altitudes, and required altitudes

A. Plan for headwinds and higher fuel consumption

7. Prepare a checkpoint checklist (see page three)

8. Check airport operation hours for lights, fuel, food, etc.

# Night Before a Cross Country

1. Make preliminary FSS contact to determine:

A. Get weather, Z level, fog, visibility, precautions

B, Get forecast winds at 3, 6, & 9 thousand

C. Use information to prepare navigation log

2. Prepare Navigation log:

A. Add 5 minutes to legs for t/o & landing  for flights below 5500'.

B. Compute ETE between points/fuel used

C. Transfer magnetic course to sectional

D. Put pattern altitudes by airports

E. Draw in your expected approach pattern

3. Prepare radio/navigation log

A. Frequencies in expected sequence

B. VOR/OBS radials/mark well on sectional

C. Know 122.1/VOR, 122.0, 122.2, 123.6, 122.9,

122.75, 121.5 and correct uses

# How to Get a Good Briefing

### Standard

Includes

Adverse conditions

Sigmets, Airmets, CWAs

Current conditions

PIREPs

Forecasts

Enroute, destination, winds

### NOTAMS

The Notice to Airmen Publications is now issued every 28 days instead of

every 14 days.  Notams have hundreds of obscure items that must be sifted to find what is important.  What is important to you is your departure point, route, and destination.  Temporary flight restrictions are important if they include your route.  Notams Ds and FDCs will be published in the A/FD and in a biweekly Notices to Airmen.

Notams can be added, changed, or deleted without notice.  The notam briefing of a short flight to another airport will not protect you under FAR 91.103 for your return flight.  The FAA uses the potential endangerment factor to determine grounds for a violation.  If they want you, they can and will get you.

Some briefings are more complete than others.  If you know about the weather and route you can get an abbreviated briefing that will take only a couple of minutes.  A standard briefing covers an entire region and can take fifteen minutes or more.  If you are going any distance beyond 50 miles you should ask for Notam Ds.  If you use DUAT get a route briefing since it will include Notam Ds.  Have the briefer check FDC notams for your route for flight restrictions.

Simply asking for Notices to Airmen (NOTAMS) is not enough.  The information for all 5,600 airports receives full distribution.  Yearly notices exceed 148,000 with 1,400 active on a given day. Taxiway information may be a NOTAM L for local distribution and runway information may be NOTAM D for distant distribution.  DUAT does not do NOTAM L.  When a FSS briefer checks notams, he is referencing Notam Ds. for your route.  Notam Ls are limited in FSS distribution and are not of 'vital' importance.

Flight Data Center NOTAMS (FDC) are regulatory.  FSSs do not provide published of special notices unless specifically asked for them.  An FDC NOTAM could cover such things as the President's position, emergency disaster restrictions or special rules during Olympics.

### Abbreviated

Update of prior briefing

### Outlook

If departure is 6 hours or more away

Followed by standard briefing

### Inflight briefing

Flight Watch-specify type of briefing desired

# Using the FSS

Initially give:

Aircraft identification and type

Your qualifications

Pre-solo as student with instructor

Student solo with/without instructor

Type of flight

Local

Between airports

Cross-country

Night

Departure point and route

Times involved (use local but learn to think Zulu)

Ask for 48 hour outlook for forward planning of next flight

## Get the Weather and More

1. Briefing, synopsis, present, forecast, airmets, sigmets, pireps, 		notams, phone call to destination.

2. Don't fly into possible ice or deteriorating conditions.

3. Don't fly if you can anticipate being in unfamiliar regions.

## Now That You Have the Weather:

1. Is the weather above my 'personal' minimums?

2. How old is the weather information.  Can I get some 'real-time?"

3. If the weather sours what are my alternatives.  Always plan for

alternatives at both ends of the flight.

4. What winds can I expect and plan for takeoff, enroute, landing?

5. Do I have the pilot\aircraft capability to avoid problems?

6. Does the flight look like a 'good' one?

7. Once enroute, how do things look?

8. VFR pilots have more difficulty with low ceilings than storms.

Low ceilings are sneaky.  They don't become a problem until its

too late.

9. For long flights you should record your ETAs in Zulu time since that 		will enable you to compare the ETAs with the time of the weather 			forecasts.

# Morning of a Cross Country

1. Call the FSS

A. Update weather information - get surface winds-NOTAMS

The briefer has a limited time.  He is required to follow a given procedure.  Any mistakes will be on the conservative side.  You have to hear the rehearsed speech before a meaning exchange of information can begin.  The briefer gives you all the bad news first.  The general weather synopsis is of questionable value for local flights.  The FAA briefer is an aeronautical mother-in-law.  Plenty of advice on how to do something without having to deal with the consequences.  A good briefing depends on the skill and experience of the flight service specialist.  You are not required to be a weather expert.  However, you must have the ability to take the information presented and make the go-no-go decision.  Very often FSS information seems designed to keep us from flying instead of giving information sufficiently complete, accurate and understandable enough to make possible an intelligent decision.  Even a student must learn to make intelligent weather decisions.

Questions:

- Describe your experience level and ask the FSS specialist, "If you 		were at my level of experience would you go??"

- Is the weather improving (we only fly into improving weather) at a 		rate faster/slower than forecast?

- Pilot reports?

Turbulence or winds of 30kts?

- Severe weather etc.  Is there a Sigmet/Airmet?

- Base go/no go decision on the actual conditions reported.

Leave yourself an optional way out.

B. File flight plan

C. Finalize planed route & arrivals

2. Tell someone of your plans/aircraft #/route/alternatives

A. Clothes suitable for eventualities/shoes

B. Emergency kit/tools/tape/blankets/water/matches/flashlights\

spares

# E-6B

The E-6B was more created than invented by Phillip Dalton in the early 1930s.   It was initially called "The Dalton Dead reckoning Computer" .  The exact derivation of E6B is not known but the E-6B has become the generic name for a vast number of similar devices which include a circular slide rule and a sliding wind angle ground speed plotter.  The Dalton E-6B was developed from a large shipboard device for handheld use aboard aircraft.  My first E-6B, which is still in the family, is from WWII and made of solid brass with enameled engraving.  A quality piece.  Plastic E-6Bs became common later in the war.  Aluminum and cardboard came later as the E-6B became obsolete with the advent of electronic E-6Bs.

With practice the E-6B can be operated with one hand on both sides once the forecast wind has been plotted.  The E6B is a circular slide rule that has numerous markings for changing from different forms of measurement.  The numerical scales allow relatively rapid solution of proportional problems.  The best use of the E6B can be achieved only when you fully understand how proportions work.  There are two time scales on the inner circles with distance, and measures on the outside.

The simplest analogy is to use the sun, a man, his shadow, a tree, and its shadow.  These can be used to form a proportion since we know the

shadow of a man when made a ratio (fraction) to the height of the man will have the same ratio as the (unknown) height of the tree to its shadow.

There are 8 different ways these numbers can be arranged as a proportion and 32 different combinations to use three of them to find the fourth.

3  =	4   is a proportion made up of two ratios.  They are equal when

6	8   the multiplied product of the diagonal numbers are equal.

3 x 8 = 24  and 6 x 4 = 24

Given any three of the numbers you can find the fourth by simply multiplying the two diagonals and dividing by the remaining number.

Given the 3, 8 and 6 we can find the four by   3 x 8

6

To be able to use the E6B you must understand how these combinations

exist as time, distance, gallons,  Every problem that gives three numbers even with one of them 'understood' is a proportion solvable by the E6B.  In addition the E6B user must confirm that an answer is reasonable.  The outer scale of the E6B is a measure other than time.  The middle scale is time in seconds or minutes, the inner scale is time in hours:minutes.

A problem that asks how far can you go at 90 miles an hour in twenty minutes has two numbers given and that is the E6B speed index or 60 minutes.  As a beginner, you would do well to arrange the numbers as a proportion before doing it on the E6B.  As with the shadow analogy we started with, like or related terms must either go one above the other or across the top or bottom of the proportion.  Any other arrangement will not give a 'reasonable' answer.  There are many ways this can be done but using the E6B format limits our choices.

We have two time numbers.  60 minutes (1 hour) and 20 minutes.  They are on the middle scale.  The distance of 90 miles 'goes' with the 60 minutes so the scale is rotated until 90 is above the 60.  The sought for missing number is a distance on the outer scale above the 20 minutes.

# Electronic Calculators

The E6B in the past few years has been replaced by flight calculators that have indicators and menus to guide you through the processes.  Even so, it is important that you be able to draw out the problem to check the reasonableness of your solution.  Working backwards to find an unknown wind is one of the more difficult required computations.

# Plotter

The first aircraft plotter was invented by a naval officer named Weems.  He adapted the much larger shipboard plotter into a plastic model which was like a ruler with an arc of 180° in the center.  The ruler itself had measures for the WAC and sectional charts in both nautical and statute miles.  A post-WWII improvement was to make a rotating azimuth dial with arrows indicating the route direction.  Until recently all plotters were of a plastic that would

be destroyed by heat.  Protect any plotter from the sun.

# Automation

With the increased use of auto-pilots, RNAV, GPS and Flight Directors there has been a corresponding increase of incidents resulting from misuse

of these devices.  What follows is a compilation of what causes such problems in aviation computers.

- Data Entry

The instruments have no way of knowing the accuracy of input.

An incorrect altitude setting invalidates an altitude alert

warning.

- Monitoring

Where the operator knows not what to look for or where to find it,		there can be no hope for successful interpretation.

- Workarounds

Manual operation can create unwanted effects when the electronics 		refuses to be fooled.

- Misapplication

System complications lead to unexpected and unanticipated effects.

Errors multiply augmented by haste, inattention, fatigue, and 			distraction.  Factors alone or in combination cause adverse 			results.  Misapplication is a cockpit operator problem such as:

Improper Mode:

Putting the correct data into the wrong mode of operation will 			give surprising effects.

Situational Awareness

Usually lost when system operator fixates on a device instead of

on flying the aircraft.

System Awareness

Poor system or operational knowledge of basic systems that may be

usually controlled by automation causes the operator to fail to 			detect system failures.   Most common in hydraulic and fuel 			systems.

Head Down time

Navigation, traffic, and flying are displaced by fascination with

electronic screens.

Over-Dependence

The unnoticed failure of one system is compensated by another with

out input from the pilot.  This results in surprises at some 			point.  Example would be fuel balance and autopilot wing leveling 		operation.

Crew Coordination

Device function attracts and distracts crew from other more basic

flight responsibilities.

Awareness

Ability of device to perform reliably causes excessive pilot 			reliance.  Failures are completely unexpected and at the worst 			possible time.

Training

Initial and recurrent training which includes operational modes,

areas of misapplication, and signs of failure.

Operating Procedures

Having an SOP checklist that is used by all is a good way to avoid

problems made by creative operators.

Crew Coordination

A second crew member is and probably always will be required in a

fully automated cockpit.  The second serves as a backup, shares 			responsibility, and acts as a safety and systems monitor.  The 			second crew member serves as a dog.  The Pilot monitors the 			automated systems.  If he touches anything the dog bites.

# Navigational Log

Mnemonic True Virgins Make Dull Company

TeleVision Makes Dull Company

PREPARING A NAVIGATIONAL LOG

True North - North Pole, the vertical axis of sectional charts, meridians

Magnetic North - Magnetic Pole, earth's magnetic lines of force affecting the 	compass

Variation - angular difference between true north and magnetic north.  The 	angle may be to the east or west side of north.  Eastern variation is 	subtracted from true north (Everywhere west of Chicago) and western 	variation (Everywhere east of Chicago) is added to obtain magnetic 	course.  East is least and West is best: memory aid for whether to add

or subtract variation.  West of Chicago it is always subtracted.

Isogonic lines - Magenta dashed lines on sectional showing variation.  VOR 	roses have variation applied so that variation can be determined by 	measuring the angle of the North arrow on the rose from a vertical line.

Deviation - Compass error.  A compass card in the airplane tells the amount of 	error to be applied to magnetic course to obtain compass course.  Make

a copy to keep at home for planning purposes.

Magnetic Course (MC) minus deviation gives Compass Course (CC). 			 (85K has no deviation on any heading)

07U has one or two degrees on most headings)

True Course - The line drawn on the map.  Draw multiple lines with spaces //// 	from airport center to airport center.  Multiple lines permit chart 	features to be read.

Magnetic Course - True Course (TC) +/- variation = Magnetic Course.  Put 	Magnetic Course on sectional for use while flying. This course 	determines hemispheric direction for correct altitude over 3000’ AGL.

Compass Course - Magnetic Course minus deviation gives Compass Course.  The 	difference is usually only a few degrees.

Course - A route which has no wind correction applied

Heading - a route on which wind correction has been applied to a course.

True Heading - angular difference from true course, the line on the chart, 	caused by the calculated wind correction angle (WCA).

Magnetic Heading - angular difference from magnetic course caused by wind 	correction angle; also, obtained by applying variation to true heading.

Compass Heading - angular difference from compass course caused by wind 	correction angle; also, obtained by applying deviation to magnetic heading.  If wind is AS computed, this is the direction you fly.

True airspeed - Indicated airspeed corrected for pressure, temperature, and 	instrument error.  This is found in the aircraft manual.  Cessna is 	overly optimistic in its figures.

Ground speed - actual speed over the ground. This is the speed on which you 	base your ETA's

Wind Correction angle - angular correction in aircraft heading required to 	compensate for drift caused by wind.  Correctly computed it will allow 	the aircraft to track the line drawn on the chart.

Indicated altitude - Altimeter reading with Kollsman window set for local 	pressure and corrected for instrument error.

Pressure altitude - altimeter reading with Kollsman window set for 29.92. 	(Used for density altitude and true airspeed computations.)  Temperature 	is not used in determining pressure altitude.

True Altitude - distance above datum plane of sea level

Density Altitude - Pressure altitude corrected for temperature.  This is the 	altitude that determines aircraft performance.

# Suggested log procedure:

1. Prepare worksheet for each leg:

Wind direction

Wind speed

Ground Speed                       True airspeed

Wind Correction Angle

True Course                         True Heading

Variation                           Variation

Magnetic Course                     Magnetic Heading

Deviation                           Deviation

Compass Course                      Compass Heading

Wind direction and speed is obtained from the flight service station.

Ground speed is computed.

True airspeed (for flight planning) is obtained from aircraft manual.

Wind correction angle is computed.

True course is the line drawn on a sectional between two points.  A plotter is 	used to determine its direction.

True heading is derived by applying wind correction angle to True course.

Variation is obtained from the sectional by selecting the east/west number 	from the isogonic lines.  Do not use the  30' part of any number. i.e. 	16 degrees 30 minutes is used only as 16 degrees.  Variation may also be 	obtained by measuring the angular difference between true north and the 	0  of a VOR compass rose.  Directions on a VOR rose are magnetic.

Magnetic Course is obtained by adding/subtracting variation from true course.  	This number should be written along the course line on the sectional 	since it determines any application of the Hemispheric rule.  It is the 	starting number from which the pilot determines actual wind correction.

Magnetic heading has no practical application.

Deviation is obtained from the aircraft compass card.

Compass course is obtained by applying deviation to magnetic course.  It is 	seldom used.

Compass Heading is the actual direction you are supposed to fly the plane to 	obtain a computed/desired ground track.  It is the sum/total correction 	of wind, variation, and deviation.

## Simplified form:

Wind

GS                                  TAS

WCA

Memory device

TC                    TH             True

var                   var            Virgins

MC                    MH             Make

dev                   dev            Dull

CC                    CH             Company

# A Flight Log

You can buy flight logs ready made, you can learn to use one in a ground 			school.  In my experience a flight log is great to use for planning the

flight BUT for the actual making of the flight, I prefer to transfer the

essential data of Compass course to the  to the sectional along with all

pertinent frequencies, altitudes, and checkpoints.  Record ETAs and ATAs

on the Z at each checkpoint.  You can transfer the information to the

log sheet at another time.  Save your log sheets.  You may well be going

that way again.

Learn to keep the sectional folded so that information can be used or entered

without lowering your head.  Hold the sectional up not down to maximize your ‘		out of cockpit’ flying time.

Course_________________ to __________________                        Usable Fuel_____________gal

Altitude_______________ft.     True  Airspeed _______K                     Fuel   Burn

Start/Taxi/Takeoff  _________gal

TC   +  WCA  =  TH  +  VAR  =   MH  +  DEV   = CH		Climb

____      __        _____      ___      _____     ___      _____		to ________feet at   _____MH   in  _________min

Depart       ____________                     Airport                           Cruise ____________feet                   ________gal

Elev  _______ft				TOTAL USED  ____________

ATIS _______    FSS  ________        DIAGRAM		  Power

________%     at   __________RPM

Gnd   _______    Appch  _______

_________gal  /  hr     TAS_________

Twr  ________   EMG    121 . 5

Departure time   ______________			Reserve __________Gal ______________Time

________OPEN   FLIGHT   PLAN

Enroute   Time   ______________ minutes

__________________________________________________________________________________________________________

Ground  	Checkpoints	CUM       Total	ETA	ATE	CUM	NOTES	     Fuel   Notes

Speed                                                            DIS        Distance			time

Leg	ATA              ATA

Distance

_________________________________________________________________________________________________________

VOR

-----------------------	----------------------	-----------------------	------------------------	---------------------

FREQ____________________________________________________________________________________________________

Arrive _______________     ____________time

Airport

FSS     ________		      DIAGRAM				              ATIS    DIAGRAM

CTAF  ________  GRN  ________

ATIS    ________  TWR   ________		CLOSE FLIGHT PLAN

# Before a Cross Country

1. BEST FLIGHT INSURANCE IS A GOOD PREFLIGHT

A. Full fuel

l. C-150 3 hours/172 3+ hours-aircraft load a consideration.

2. Refueling plan/fuel tank change plan

3. Preheat if required; clean windows\wings

. What if...plan

B. Minimum safe altitude

1. Hemispheric rule (Use MC, odd people live in the East.

2. Terrain clearance day/night

3. Following roads/valleys- stay on the right side.

4. Restricted flight areas enroute-contact controlling agency.

ZOA is the code for Oakland Center.

C. Emergency procedures

1. Aircraft

2. Weather\avoid surprises

3. Personal

2. ALL AVAILABLE INFORMATION

A. Airplane performance/limitations

1. Weight/Balance

2. Mixture control

3. Climb/descent S-turns

4. Manual and density altitude figures

5. What if...plan

B. Weather

1. Cloud clearances

2. Visibility minimums

3. Turbulence

4. What if...plan

C. Traffic

1. Large/small

2. Jump/glider

3. Airway/approach/patterns

4. What if...plan

3. Airport Considerations

1. Runway length/slope/obstructions

2. Pattern direction/altitude

3. Arrival/call-up/report/radar

4. Taxi plan/request taxi assistance

5. What if...plan

4. Cockpit Organization

1. Checklists

2. Frequency list

How to contact:

FSS, Approach, Flight Watch, Tower, Uncontrolled,

other aircraft.

3. Current sectional with essential information

magnetic course/l/points/pattern altitudes

4. Pens/money/Provisions/flashlight/batteries

4. Communications/FSS/radar/tower

5. What if...plan

5. Don't get into the airplane without pointing the way you

plan to fly.

1. Orally review what to say to tower for each possible runway 				departure/arrival

2. Do everything on the ground that can be done on the ground

3. The behinder you get the slower you should go

4. Allow time for refueling/changing planes etc.

5. If ever you need a reason to cancel a flight, call the FSS.

Somewhere there will be a problem and a reason.

# If You Go Weather Services

Flight Watch (122.0)  EFAS Enroute flight advisory service

ATIS Automatic Terminal Information Service

# Flying a Cross Country

Takeoff planning:

POH data is based on new aircraft, test pilot, and multiple tries.

You should add adequate (?) safety margin to any POH figures to account for any differences.  Factors such as non-current weather, inaccurate thermometer, variations in wind, grade, and surface conditions may not be in the POH.  The time to learn to use the POH is as a student pilot.  It's a little late to do so at Blue Canyon.

The POH will give time, fuel, distance to climb, rate of climb, and climb speed recommendations.  We use the available information to compute

what performance can be expected and reduce the expected performance by such

factors as we deem appropriate.  The last consideration when takeoff conditions approach the marginal is the 'what-if' factor.  Think out your options before you get into the plane.  Plan the departure route, emergency

turns and sites and avoid thinking that you can turn back to the airport.

### Procedures

1. Preset all radios nav/com/OBS/recorder

2. Needed papers/pens open and available

A. Sectional/ checkpoints/magnetic courses marked

B. Checklists

C. Frequency card

3. Belts/harness

4. Taxi as though 40 knot winds

5. Request "time check" prior to takeoff/CHECKLIST

6.  Regardless of takeoff direction climb/fly directly to your first 			checkpoint-OPEN FLIGHT PLAN

Compass check technique:

Have first checkpoint that is clearly visible from the airport.  	Make 270 departure from field that will take you to opposite side of 	airport from checkpoint.  With field and checkpoint clearly in sight 	line up field and checkpoint.  Check compass and sight as far ahead as 	you can on a mountain or cloud.  Go for it.

7. Use checkpoint checklist

A. Time

B. Gauges and instrument check

1. Engine instruments

Check for cabin heat before takeoff.

2. Flight instruments

a. Altitude/setting

b. heading/compass check

C.  Three elements to confirm checkpoint

D. Set VOR/OBS for next checkpoint/IDENT

8.  Once on course pick a visual point as far ahead as possible as a 			course reference

9. Keep your head "out of the cockpit". Hold papers up-not head down

10. If altitude/course/radios are off, don't wait-make everything right 			by the next checkpoint

11. Plan ahead

A. Required/legal/safe climb/descent to altitude

B.  Listening watch on ATIS/flight watch/tower/etc.

C.  Allow time to practice before communicating

D.  If you get behind, go to slow flight

12. Try to get a time check between first two checkpoints and get an ETA

13. You arrive at a checkpoint when airplane is aligned with magnetic 			course and you can sight down the wing directly abeam.  It is 			possible to make large time errors if done otherwise.

Enroute checkpoints should serve as notice to use your checklist.

the checklist should include at least the following:

1. Complete Engine Gauge check

2. Fuel consumption check. (lean and enrich as appropriate)

3. Nearest airport check

4. Emergency landing location

5. Extended route check (Am I heading in the right direction?)

6. Time, time next checkpoint, time of arrival.

7. Setting of radios both com and nav (OBS)

8. Wind adjustment/ground speed confirmation

9. Altitude anticipation (climb or descent)

10. Higher altitudes give better tas, communications, navaid, and

ground clearance.  Visibility is not always improved.

Head winds will be stronger at altitude.

11. Practice of communications.

14. Update your ETA with subsequent time checks.

1. You get a two-percent increase in true airspeed for every 				additional thousand feet of altitude.

15.  Like landings, finding airports requires FAITH

A.  Start your descent at last checkpoint

B.  Runway direction is best determined by turning to runway 				heading well away from the airport and try to pick up visual 			references.

C. Better to communicate early at prominent checkpoint if 					unfamiliar

D. Have a planned arrival

1.  Descent, call up, reporting points

2.  Pattern altitude and arrival

E. Copy ATIS/X-ponder settings and frequency changes.

F.  Repeat back instructions/clearances to confirm

G.  If unfamiliar; so advise.

16.  Emergency planning/CHECKLIST

A. Engine full/partial failure

B. Com/nav full/partial failure

C. Ceiling/visibility low/lowering

D. Lost/misplaced airports

E. Traffic awareness/avoidance

F. Direction and location of nearest airport.

G.  Relative value of NAV aids.

17. Arrival at destination/CHECKLIST

A. Airport in sight-CLOSE FLIGHT PLAN

B. Fill in flight log

1. ATA, fuel consumption

C.  Self analysis of flight -% grade?

1. Use of CHECKLISTS

2.  Accuracy of altitude/navigation/headings

3.  Competence of communications

4. Safety- Never fly through or under a thunderstorm.

18.  What if...

A. No VOR/ATIS/com

B. Unable to understand

C. Disoriented/confused--The 4 C's

D. Assistance desired/required

E. Weather problems/alternatives

F. If in doubt communicate; if in trouble declare an EMERGENCY

# The Last Leg

The last leg of any flight is on a mileage basis the most dangerous because of several factors.  Fatigue contributes to breakdowns in checklist use, procedures and attention.  Some inattention is to be expected at the end of a long flight.  Along with inattention is misinterprets of instruments.  You 'forget" to change frequencies, add or remove flaps, use carburetor heat, or raise/lower the gear.  Get-Home-itis makes you want to go faster.  While get-Home-itis can affect any pilot it is much more likely to become even more dangerous when coupled with fatigue.

The last leg often takes us "home" and into familiar terrain.  Complacency has inherent dangers.  At "home" we experience a psychological letdown that can lead to error, embarrassment or hazard.  We "know" what to do, what to expect and are able to relax at last.  Pilots on the last legs of flights have frequently cited the psychological letdown in vigilance and cross-checking as a precipitating cause for a last leg accident.  Complacency is identified only in post-incident reflection.  No one is immune to complacency.

Fatigue is insidious.  Compounded by a sleep deficiency we find a progressive impact upon alertness and attentiveness.  Don't over schedule your flight if you have not eaten or slept well.  Plan a rest stop every two or three hours.

# Descent

A most neglected part of flying is the arrival.  As soon as a cruise altitude has been selected the pilot should begin planning the arrival.  The ideal is that the arrival should occur with a minimum of control and power changes.  From the Sierras a cruise descent of 2-300 fpm should begin shortly after crossing the summit.  Since most student arrivals occur in the 2-3000’ range we acquire experience and “know” when to start down.  Always try to turn altitude into airspeed for best economy.

Avoid descents over 500 fpm by initiating descent early.  On the other hand, don’t begin so early that you spend too much time beyond safe gliding range of suitable landing sites.  The choice is often a compromise.  Make your choice on the safe side.  Wind is an additional factor.  Your descent gradient will be affected by the headwind\tailwind component.  Your descent gradient is a factor of ground speed not airspeed.  Arriving too high and feeling that you ‘must’ pull the power to get down is going to be damaging to the engine by shock-cooling the cylinders.  The bimetalic composition of the cylinders will cause them to cool at different rates.  Rapid cooling causes warping and

separation of the two metals.

To compute your descent rate and point you must first select a descent speed that you will use and the amount of altitude to the pattern altitude of your destination.  Next determine a desired rate of descent that will be an efficient trade of altitude for airspeed.

A descent of 90 knots covers 1 1/2 miles a minute or 3 miles every 2 minutes.  Descending 250 fpm will lose 500’ every three miles.  Begin descent 18 miles out to lose 3000’, 9 miles out to lose 1500’, 4.5 miles out to lose 750’.  This becomes relatively easy if you have DME, LORAN or GPS capability.

Initiate any descent at a fixed rate (400 fpm)for one DME mile, you will lose 2800’ in seven miles.  For an airport that is 8 miles away you should always be in gliding distance on reaching pattern altitude.

# Pilotage

Pilotage is a big part of the practical flight test.  The oral will check your map reading skills.  The flight will check how well you follow your planned course and relate landmarks to the sectional.  Some pilots find pilotage easier than others due to better spatial relationship skill or ability.  Pilotage can be learned.  It is a matter of instruction, attention and desire.  Pilotage is a low-tech system that began with the Wright brothers.  Pilotage tends to fly less than direct routes.  Good pilotage will always get you where you are going. The backup for pilotage is dead reckoning.

Pilotage requires that you have a prepared route on a sectional, a compass, something to write with and a plan of how to used them if your electronic navigational devices fail.  The spontaneous use of pilotage will not have numbers for deviation or variation, ETA, and fuel consumption.

Your pilotage flight will point the aircraft in a direction that you believe will take you to a given point on the route.  You will make any changes in direction required to get there.  Your compass heading is adjusted for wind as required to follow the map line.  All information related to magnetic course, time, equal spaced checkpoints, and ETA is written directly on the map.  The map and the ground are your references.  Getting  lost is bad.  Running out of fuel is worse.  Missing a checkpoint by five minutes means that something serious is wrong.

Since the line on the map goes from the middle of your departure airport, you will make a 270 departure or whatever it takes to make your initial course correspond with the center of the airport.  Sight to and fly to your first checkpoint.  If visibility is poor, select half-way points to keep you on course.  Don’t time checkpoints for ETA purposes until you are at your cruise altitude.  Once on course make heading changes as required to correct for wind and to keep you on line.  Flying exact headings makes pilotage easier.  The longer the distance between your checkpoints the more important exact headings become.  If you find yourself off-course, make the heading change needed to get on course immediately.  Once on course select a new heading to keep you on course.  For each checkpoint space you should note both the heading and ground speed.  If the spaces are equal then the ETA is just a matter of counting spaces.  You don’t even need the E6B.

I have been flying with a pilot whose aircraft had every instrument and goody you would ever want.  So I make him fly pilotage.  Pilotage is fun and you get to see things you would otherwise never see.  If you can see the ground you can use pilotage.  The most difficult aspect of pilotage is associated with locating airports.  Even large airports can be difficult to see.  New runways

are relatively visible.  Older runways blend in with the terrain unless the local foliage provides contrast.  Green runways in green fields are practically invisible.  One of the advantages of giving student pilots opportunity to

fly as observers.  Give them a sectional and help them to find airports

along the route.

Orient the sectional in the direction of travel so that checkpoints will be properly positioned.  Look for features that will provide continuous orientation.  Look for crossing features that extend well to each side of your route and especially one past your destination if you fly too far.  Time your crossing features so as to keep track of fuel, distance and ground speed.  Every landmark is a potential checkpoint but it may not be a good checkpoint.  Look for the prominent and unique chart and land feature that you can expect to see while flying.  The science of checkpoint selection and verification must be taught, learned, and developed by each pilot.  Being proficient at

pilotage is a parachute to be used if all else fails.  Most of all pilotage

is an enjoyable skill to practice.

A common pilotage fault by the inexperienced pilot is to descend to see better.  You can easily lose the big picture close to the ground.  Select a call-up feature prior to your destination that is prominent enough to let you locate the airport.  Misplaced or lost can become a problem.  Climb.  Relocate yourself at your last known crossing feature.  Use the radio.  Do not fixate on one point to the exclusion of verification points.  Lakes change shape and size during the seasons.

For pilotage always keep the sectional oriented to the direction of travel.  Look in the distance for a course reference.  Find a terrain feature that will take you there.  A river, road, ridge, whatever.  Keep a time record on the sectional as each major feature is passed.  If you get misplaced you will be able to back track.  If you have a destination that is itself hard to locate use a practice known as aiming off.  Fly to a prominent landmark close

to your destination and then aim off that to your destination.  In addition,

have an identifiable landmark near your destination that will aid in its location.

Were land mark configuration allows by having extended rivers, waterways or roads a landfall system may be used.  This means that where you are going is somewhere along a selected road.  You deliberately fly off course enough to

be sure that on reaching the landfall road you will know which way to turn to

your destination.  Not very pretty but very simple.

# Dead Reckoning

At some time during your training you should go under the hood and use dead reckoning to navigate to some obscure destination.   At one time there were no clocks capable of the accuracy required to measure longitude.  Longitudinal distance was measured by dropping a log dead in the water.  A rope with knots tied at regular intervals.  By letting the rope flow by as the ship left the log behind and an minute glass on the ship’s rail the speed of the ship could be calculated as being so many knots of speed.  This data was recorded into a log book.  The effects of winds and water could only be guessed at. Dead reckoning is the basis of all navigation, even that of LORAN and GPS.

Dead reckoning is based on Speed x Time = Distance.  The calculations must begin from a known starting position in a known direction.  In a no wind

condition this can be quite accurate.  Since no wind conditions seldom exist

an estimate is usually made from available forecasts.  Since winds have both direction and speed, dead reckoning accuracy is entirely dependent on how lucky the navigator is in getting forecasts and actual winds to agree.

If the flight planned route allows additional fixes to be determined then new plotting and measuring will be required.  This recalculation includes figuring the actual wind speed and direction as it differs from the forecast.

The re-figuring will require a new wind correcting heading and ETA based upon

the aircraft actual ground speed, heading, and altitude.

You do not need an E6B or other aviation computer to do the figuring.  A protractor, ruler, and pencil will be sufficient.  Correctly used these

will allow you to draw vectors proportional to the directions, distances and

speeds of the six essential elements.

Wind: Wind direction from) and speed will be the shortest vector.

True course is the line direction on the sectional.  True airspeed

is obtained from the POH for a selected power setting and altitude.  Joining these two vectors will form an angle which will show the drift effect of an

hour’s wind on an hour’s true airspeed.  Joining the open ends of the vectors

will give a ground speed per hour vector and a required wind correction angle.

Since the wind is drifting the aircraft to the right the heading angle required to correct this drift must be to the left.

Wind

Vector line

True

Course

Line

Ground Speed

Line

Wind

Correction

Angle

Requirements:

Sectional, compass, watch, known starting point, destination,

estimate of winds, and calculation of wind correction angle.

Components:

Time, distance, heading, put all information on the sectional

Requirements:

Understanding of wind drift, wind correction angle, ground speed 		vs. airspeed.  DR does not often allow a pilot to update his position 		as will pilotage.  Errors in direction will accumulate at a rate 		of 1 mile for every 60 miles of distance multiplied by every 		degree of course error.  Lindbergh was lucky.

Every pilot should make a point to hone pilotage skills by occasional flights into unknown territories at low altitudes.  Total reliance on electronic aids will weaken both pilotage and dead reckoning skills.  The next time you make a flight with one or more other pilots have some sharing of the flight legs.  The flying pilot under the hood but doing all preflight dead reckoning planning.  A suitable reward or punishment can be given at the

end of the flight.

# VFR on Top

Most areas of the world do not allow VFR on Top.  It is allowed in the U. S. but not by student pilots.  The student pilot is required to have the ground in view at all times.  This requirement was not always so but as with most FARs it took accidents to make the rule.

Once you have your pilot's license VFR on Top is allowed.  Weather permitting your instructor will place you into a VFR on Top.  There are occasions when the entire California Central Valley is one huge blanket of fog.  for flights up and down the valley the safest options are down the sides.  The VFR only pilot  should recognize the options limits that exist in crossing flights.  Even an IFR capability may be of little use if the underlying airports are

below minimums.  always keep in mind the location of relatively high-altitude airports and fly a surveillance approach at some time with an instructor.

# Lost Procedures

Denial, anger, acceptance, resignation and one other that escapes me are the mental/emotional stages that a person is supposed to go through when suffering from a terminal illness.  Much the same stages occur when a person in the process of becoming lost.  Becoming lost grows from hints, a sense of  uncertainty, inklings of uneasiness, unwillingness to accept, acceptance, and finally a form of mental paralysis.

There are two kinds of pilots, those who have been lost and those who may expect to be lost.  Perhaps the line should be changed to those who use GPS and those who don’t.  As a pilot who expects to get lost you should now

develop a plan for what to do when lost.  Such a plan should initially depend on your own resources related to points that can be visually recognized, using

the chart and compass to aid orientation.  Consider diversion to a known

alternate by pilotage.  Once you are satisfied that you cannot do it on your

own, get help.

GENERAL RECOMMENDATIONS:

Do not wait for one difficulty-being lost- to combine with another such as low fuel, weather, night, or ?.  One difficulty by itself is solvable but in combination, difficulties cause accidents.  Willingness to communicate on the radio is your best assurance of being helped.  Do not totally rely upon prominent checkpoints such as mountains and lakes.  Pick a prominent feature and make wide circles.  Weather conditions can cause them to disappear.  The easy flight in severe VFR can be very difficult in 10 mile visibility.  At night, clouds or fog layers can distort familiar light patterns so that even an experienced pilot will become disoriented.

One thinking man's procedure makes a five column sheet.  Column one is “known", two is "Unknowns", Third is "What ifs", fourth is "Options", fifth is "Results".  Using this technique may be helpful in keeping you out of the plane in the first place.  Disorientation is an occupational hazard of flying regardless of familiarity and experience.  Some day or better  night, try flying the Bay Area VFR over a thin undercast of fog.  Become a believer.

# Common Cross Country errors:

1.  Failure to get time off.

2.  Failure to plan airport departure to get established on course as soon 		as possible.

3.  Failure to pre-set radios and plan communications.

4.  Failure to reach first checkpoint and note time and use checkpoint 			checklist.  Failure to reset heading indicator

5.  Failure to pick a point/direction as far ahead as visibility allows.

6.  Failure to use sectional to maintain course

7.  Using checkpoints which are not visible or useable

8.  Failure to get time at second checkpoint and to determine ground speed.

9.  Failure to get ETA and re-estimate at each checkpoint.

10. Failure to maintain listening watch on best available frequency.

11. Failure to be aware of controlled areas along or adjacent to route that 		may or may not require action.

12. Failure to see checkpoint that is directly under plane.

13. Failure to climb/descend/turn in anticipation of situation ahead.

14. Failure to communicate if in doubt

15. Failure to anticipate heading changes caused by raising plane's nose.

16. Failure to maintain altitude; being satisfied with 100' high or low.

17. Failure to note VOR needle centering for checkpoint.

18. Looking down in cockpit and turning.

19. Being insecure in releasing yoke and flying with rudder

20. Descending too soon and making destination difficult to locate.

21. Not descending soon enough to reach pattern altitude at destination.

22. Failing to have checkpoints selected near destination at which to 			communicate and descend.

23. Re-trimming during descent which makes leveling off require more than

just adding power.

24.  Making several small turns to downwind instead of selecting a heading 		to turn to.

25. Turning too close to runway on downwind

26. Not knowing where to watch for traffic.

27. Being reluctant to communicate.

28. Continuing to fly toward deteriorating weather instead of making 180 			and flying to light.

29. Allowing clouds and terrain to close in on you instead of making 180.

30. Saying, "That's Gotta be..." when it doesn't have to be and probably 			isn't.

31. Failing to recognize the mentally disabling effect of being lost, in 			bad weather, and disoriented.

32. TLAR navigation.  (That Looks About Right)

33. TGB checkpoints  (That’s Gotta Be)

On dual cross countries the instructor makes a practice of moving his seat to see how long it takes the student to note the change in trim pressure required to hold altitude.  He also may put metal up on the dash to affect the compass.  If the student can't create enough problems then it is up to the instructor to 'help'.

This instructor never looks toward an airport that a student is having difficulty finding.  If anything, I look the other way.