Electricalã

Electrical System

Alternators

Generator

Ammeter

Voltage Regulator

Electrical Failure

Position Lighting

Starters

Precipitation Static

# Electrical System

The master switch or one-half of a split master is a safety device to keep the high battery amperage used in starting out of the cockpit.   The master activates a battery solenoid and relay.  This relay lets electricity go to the primary bus.  The primary bus is a relatively heavy metal bar capable of carrying heavy current loads.  Each electrical circuit is connected to the bus bar as a separate terminal.  The master switch makes it possible to use battery power to start the plane, but has nothing to do with keeping it running.  The magneto system that runs the engine is independent of the master switch and battery.  With the magnetos on, just turning the prop as little as 1/8 turn can cause the engine to start.

In line with this circuit is a fuse or circuit breaker which is sized to the wire used.  Aircraft wiring is specially coated and covered to prevent corrosion. Automotive wire is not suitable for aircraft.  The purpose of a fuse of circuit breaker is to protect the wiring from catching fire.  Co-incidentally equipment is protected.  Some fuses are slowblow.  This allows the fuse to accept a momentary overload without blowing.  Some breakers can be pulled off (gear) but every breaker can be reset.  No breaker should be held closed.  At most, reset a breaker only once before getting expert advice/maintenance.

If the battery relay is not activated because of low voltage in the battery, there is no way for the alternator field coil to develop the voltage needed to recharge the battery or run the electrical system.  When the battery is low some of the alternator energy is used for recharging.  This recharging battery-load is bad for the electrical system.  Having a cigaretty lighter volt meter is desirable.

Aircraft use lead-acid batteries made of a series of such cells to get the desired voltage.  To reduce weight the battery casings are made lighter and smaller for aircraft.  A 12 volt battery has six lead-acid cells giving two volts output each. Positive plates of lead dioxide and spongy lead are kept separated from the negative spongy lead plates in a sulphuric acid/water electrolyte.  Battery capacity is measured in ampere-hours.  Over its there to five year life the battery capacity will decrease.  A charged battery has an unbalance of electrons between the plates.  When an electrical circuit is completed the excess electrons from one plate flow through the electrolyte to the other.  This flow creates lead sulfate which will eventually result in a ‘dead’ battery.  Aircraft batteries can be run down in just a few minutes at the high amperage required for starting.  Endurance is determined by capacity and charge. A load test is used to determine capacity.  The airworthiness of an aircraft battery is about two years.  A dead or weak battery is unairworthy.

The charge of a battery is determined by the density of its electrolyte.  This state is checked with a hydrometer that measures specific gravity as it is floated in the electrolyte.   Hydrometer readings are taken before adding water.  Electrolyte is normally 30% acid by volume.  Never add water to a battery unless it is below the plates.  Charging will cause the level to rise.  A specific gravity reading below 1.24 is considered low for an aircraft because of high load requirements.  Such a battery needs to be charged or replaced.  Normal range of specific gravity is from 1.26 to 1.3.  A three amp charge rate will be best to preserve the battery and prevent overheating.  A weak battery is unairworthy.  A dead battery puts a great load on the electrical system.  Such loads are the number one cause of electrical system failure.  All battery maintenance should be recorded in the airframe logbook per FAR 43.9 as to work performed.

Aircraft battery charging can be done by either current or voltage methods.

A battery must be removed from the aircraft for charging.  Remove and attach the negative cable first and last. Quick chargers will reduce battery life expectancy.  The constant current method requires more time and can result in overcharging if excess time is used.  Vent caps which allow the battery to breathe should be loosened  but not removed during charging.  Avoid sparks which may ignite the hydrogen and oxygen being vented during the charging process. The aircraft alternator/generator system has a constant voltage due to its voltage regulator.  An exploding battery is a terrifying experience.  The battery may be allowed to

become warm during charging but not hot.  Allow the battery to rest for several hours and give final specific gravity test before  reinstallation.

An aircraft battery is kept in a metal box for electrical and mechanical shielding.  Each battery cell is sealed in hard rubber with a non-spill vent cap on top.  A lead weight seals the vent during inverted flight.  The hydrogen gas vented from a battery is highly explosive if ignited.  No smoking around batteries.  The battery box has an exhaust tube through which battery gases are vented.  In some installations intake air and a drain sump neutralize the gasses before venting.  If a cable is loosely connected or corroded, or if there is a direct shorting of the battery terminals the current may burn off the terminal and start a fire.

Rapid discharge or charging ruins a battery and shortens its life.  Heat and cold cause shortened life.  Keeping fluid levels correct in the summer is important.  Batteries have a life potential of thousands of charge

and discharge cycles but longer periods between starts extends life.  A battery with a low charge can be permanently damaged.  The greatest killer of batteries is over-charging.  The colder the temperature the greater charge that can be applied.

A newer type nickel-cadium  (NiCd) are much more expensive but require less maintenance and have a longer service life.  Voltage of the NiCd is constant up to 90% of its discharge cycle.  NiCds use potassium hydroxide electrolyte which as a base instead of acid requires the use of special tools and techniques.  NiCds must be bench checked with special equipment.

A still newer type called a recombinant gas (RG) type is designed so it can’t leak, doesn’t require a sealed box, has greater capacity and gives more 	power.  It uses a glass mat to soak up the acid and hold it in suspension.  The 	mats can be packed closely with more lead plates per cell.  The RG battery has greater capacity and is heavier for its size.  It can be safely shipped by UPS.

Battery maintenance can be done as part of pilots authority under FAR 43.3(g).  For removal always remove the negative lead first and replace it last. Do not charge a battery while it is on the aircraft.  A charging battery will release gases that can be exploded by a spark.  Do not add water prior to charging.  Terminal corrosion after removal can be prevented by coating with a terminal sealer after being reconnected.

# Alternators

Alternator failure is the most frequent cause of electrical system failure.  If you note alternator failure, shut down as much of the system as you can.  My choice would be to leave only the transponder operating.  A popped circuit-breaker will indicate that an overload has occurred.  Reset one time only.  Resetting the breaker must be done with a very firm push.  It is reset when the low-voltage light goes out.  Know where this breaker is and what it feels like when set.

Belt or gear driven.  Gear driven alternators can contaminate engine oil on failure as in C-150.  Proper installation important. Life 700 hours +.  A belt driven alternator must have the belt at the proper tension. This means about 1/2" flex in the belt when pushed with a finger.  Too tight will burn out the bearings of the alternator.  Too loose will cause the belt to slip under load.

An alternator requires a "priming" voltage in the field coil before it can begin to produce electricity.  For this reason, starting an aircraft with a totally dead battery will not enable it to produce electricity for the radios and lights unless it is an older aircraft with a generator.  A quick battery charge of only a few minutes may be sufficient to activate the alternator field coil.

An alternator may work normally until it is subjected to a full load.  At maximum load a failed diode may cause total failure.  An alternator check of this condition could be made by turning everything on prior to shut down so see what happens.

# Generator

Older aircraft may be equipped with a generator to provide electrical power to the systems when the engine in running.  The generator does not need battery power to begin functioning as does the alternator.  This is its only advantage.  The generator will provide insufficient electricity during low power engine operations such as pattern work.  The battery must provide the difference and will become discharged in an hour of such operations.  The increased electrical requirements of modern aircraft often exceed the generating capacity of the generator, hence the alternator.

# Ammeter

Two different types, charge/discharge and load.  The load meter reflects the actual electrical load as it is turned on.  A load ammeter at zero or discharge is saying that you are using battery power.  You should know that radios use .5 amp and receiving and 5 amps when transmitting.  You can tell you have a voltage problem if you can't hear the transmitter relay.

Nav radios and electric gyros use l amp.  Transponder about 2 amps.  You can get an idea of usage by the ammeter reading according to type.  As the electrical load increases the alternator tension belt becomes ever more important.  Under load a loose alternator belt will begin to slip and the alternator will fail to produce the required electricity.  Suspect a loose belt if you get unusual ammeter indications.

# Voltage Regulator

In the circuit with the alternator is a voltage regulator.  It adjusts the field coil electrical flow to provide an output commensurate with the load applied.

If the voltage regulator malfunctions it may activate a high/low voltage light.  This is most likely to occur at low power settings.  If a circuit breaker "pops" reset only once.  If an over-voltage occurs it is important that the alternator be turned off by switch or by circuit breaker.  The circuit breaker may be included as part of the master switch.  Shut down (except the engine) and start over with the alternator.  If the warning light comes on, shut down the alternator.  Run off the battery.  After the alternator cools down you might try again.

# Electrical Failure

There are several forms of electrical failure.  The safest form is the blowing of a fuse or the popping of a circuit breaker.  Never make more than one replacement or reset before getting on the ground.  These protective circuits don't always function

The next likely electrical problem will first appear as a burning odor followed by smoke, an annunciator light coming on, or a failure of some sort.  Wiring and wiring connections are the most likely causes of failure. Your preflight should note any loose wiring in the cockpit. and when they don't fire is not far behind.  Shutting off the master switch is the emergency first step.

Have an electrical failure sequence checklist such as:

Master switch off

All switches off

Try battery switch (ammeter indication)

Try alternator switch (Voltage light)

Try one radio and light at a time to isolate problem

Run on minimum equipment

In the event of smoke, shut everything off and get on the ground. Run through failure sequence only as second choice.  If you have a split master switch, you might use just the battery side and the transponder to 7600.  Don't reset circuit breakers or replace fuses more than once.

# Position Lighting

Wingtip navigation lights can be seen in a 110° arc from front to sides and vertically for 180°.  The rear light need not be on the tail but must be visible 70° to either side of center.  The use of low-powered pulsed landing lights as recognition aids is a $200 self-defense system.  Such lights must not affect pilot vision or position lights.  Wing tip landing lights give greater longevity but make some taxiing more difficult.  Part 91 flight at night does not require landing lights.  Aircraft switches are on when up or in, off then down or out.

The position lights are angled so that they have non-intersecting arcs.  At no time can three position lights be seen at the same time except from above and below.  Two lights can be seen from head-on, 110° to the left rear, and 110° to the right rear.

For collision avoidance if you see a red light give way.  If you see a green light expect traffic to yield but be prepared if he doesn’t.  If you see a white light break right to clear traffic you are passing.   If you see red and green you are head-on to traffic, break right.

All lights, position, strobe, and landing lights lose their intensity over time.  Periodic replacement is a good idea.

# Starters

A student should have knowledge as to the starting mechanism.  The starter motor is activated by a key switch.  The initial spin of the starter usually (Not C-150) uses a bendix spring to engage the starter gear with the engine flywheel gear as with automobiles.  Once the engine starts the gear disengages.  Occasionally the lack of lubrication or battery power will prevent the gears from engaging.  Get help if after four tries the gears do not function.

Starters usually die from overheating while starting hard to start (cold) engines. The Bendix will stick if dirty and fail to retract.  Since it remains engaged with the flywheel the Bendix drive will soon fail.  Lack of use affects the Bendix, brushes and rusts the gears.  These defects require more battery current to operate the starter and cause overheating.  Should be part of engine TBO (time before overhaul) cycle.

# Precipitation Static

As an aircraft flies through air a static charge can be created on the aircraft.  Static dissipation wires are often placed on aircraft trailing edges to remove this static.  When such static exists the radios may make a undesirable noise.