# Instrumentsã

Pitot-Static System

Airspeed indicator (See speeds)

Vertical Speed Indicator (VSI)

Altimeter

Altitudes:

Gyroscopic Instruments

Attitude Indicator

Heading Indicator

Turn Coordinator

Needle and ball

Needle vs. Turn Coordinator

Gyroscopic Failure

Compass

About the Compass

Avionics

# Pitot-Static System

The total or partial failure of any system will be a traumatic occasion.  The static system is the most insidious of systems that fail.  Pilot overconfidence turns to mush when unable to rely on normally reliable instrument indications.  Your survival will depend on how aware you are what indications tell something is wrong or at least going wrong.

The static port(s) provide a way to allow selected instruments to monitor

the atmospheric  pressure known as pressure altitude.  The static system operates the airspeed indicator, altimeter and vertical speed indicator.  During climb the higher air pressure in the instruments and tubes escapes through the static air hole.  During descent the process is reversed.  In level flight the pressures of the altimeter and the VSI equalize.  Any partial blockage of the static system will cause excessive lag in instrument readings.  The altimeter will not catch up with the climb or descents actually occurring.  The airspeed indicator cannot discern the distinction between more pitot pressure or blocked static air.  In either event excess airspeed is indicated.  Thus a partial static failure may

tell you that you are high, fast, and in a slow descent, where in reality you are lower than you think, descending far to rapidly, and near stalling.  These conflicting instrument readings can panic the most experienced pilot.

A blocked static port makes the altimeter cease to operate.  The use of an alternate static air source for the altimeter causes it to read cabin altitude.  Cabin altitude can vary by several hundred feet from  pressure altitude.  These errors are caused by airspeed, cabin venting, and flight attitude.

All static lines provide for drainage of moisture.  Tubes may be metal, plastic or rubber.  If the static hole is plugged the altimeter and VSI will not move during climb or descent. It is possible to bypass the static hole by using alternate air from the interior cabin.  A bypass valve or breaking the glass of the VSI will make this possible.  The use of an alternate static source is an emergency procedure because of the inherent errors in readings derived.  If the static air hole is blocked on takeoff the airspeed indicator will read normal, in the beginning,  but will read lower than it should with an increase in altitude.   The VSI will show a momentary climb.  Indications will vary so no one aircraft’s indication should be considered representative.

# Airspeed indicator (See speeds)

# Vertical Speed Indicator (VSI)

The VSI mechanism works through direct line to the static air outlet on the side of the airplane . The VSI uses this static air source to expand and contract a closed metal aneroid wafer with a small bleed hole which is geared to an indicator needle.  Outside air pressure differences cause the

vertical/descent needle to change.  These changes reflect the differential pressure between the static air and the inside wafer air and the needle moves accordingly to show climb, level or descent.

The VSI case has a bleed hole that slows down the entrance and exit of changing static air pressure.  It may take several seconds for the bleed hole to catch-up with static air changes.  This delay means that the needle

may be reading excessively in the wrong direction, trending correctly but

to the wrong extent.  The calibrated bleed hole needs some time to adjust

to a stabilized condition before the needle will read correctly.  Any temperature corrections are made automatically since the VSI is self-

compensating.  Another type of VSI called an IVSI gives instantaneous indications.  There is also an electronic VSI.  According to “FAR 91.205 the VSI is not a required flight instrument.   The VSI has a ‘zero’ adjustment screw on the lower left side (usually)

The VSI is, after a few seconds a trend indicating instrument.  Stabilized

in climb or descent it can tell a competent pilot the rate.  The better you fly the more you will rely on the VSIs ability to give indications of just how smoothly you are flying.  The altimeter lags the VSI after a short pause.  Pitch changes are reflected on the VSI only to appear later as altitude changes.   Where the VSI needle holds still for any length of time the altimeter will catch up in reading and movement.  You can check VSI operation by lightly pressing the glass face and seeing if the needle will deflect.  A partially blocked system can give a similar indication.

The critical factor in VSI flight and interpretation is the speed of any change.  A typical student reaction to VSI movement is instinctive over-reaction.  The VSI operates best when changes are smooth and gradual. The VSI is the best measure in the aircraft for hands-off trim settings for level, ascending or descending flight in smooth conditions.  Practice flying a fixed rate of climb or descent.  Competence does not come easy.

Trim for a constant level altitude at a set airspeed. Climb and descent rates must be set according to power and speed.  For good IFR basics you should memorize and consistently practice the trim /power settings that give 90 knots in a C-172 or a PA-28 going from level cruise to get a 500 fpm rate on the VSI.

The VSI can be used to track a glide slope from intercept to decision height providing all adjustments are made sm o o thly. Its usefulness today comes into play when pilots are forced into partial panel IFR operations.

In turbulence, ignore the VSI.

It can be used to make the enroute descents that most economically trade altitude for distance.  By knowing your ground speed the distance to go and your rate of descent in feet per minute you can plan your pattern altitude arrival from 60 miles away.  Planning to arrive at a particular altitude point is quite satisfying when you make it work.  You have saved, time,

fuel, and money.

# Altimeter

The misuse of the altimeter is very common.  Half of all accidents involve  altitude difficulties.  It can be set wrong or misinterpreted but it seldom fails.  Only in special situations is the altimeter set to ‘zero’.   Aerobatics and some gliding are such special situations.  The altimeter should not be relied on for the final approach segment of any landing, in narrow mountain passes, in high wind conditions over rough terrain, or when the VSI/airspeed instruments behave erratically.  I have had altimeter settings be off over 500’ when crossing a mountain range.  Variations of 2000’ are not unknown.

Pilots are more likely to run out of altitude than fuel or runway.  If the altimeter is mechanically correct and set correctly the likelihood of an altitude deviation is greatly reduced.  Basic to flying  is the ability to read an altimeter correctly.  Abort 10% of altitude flight errors are due to misreading.  A pilot who fails to reset his altimeter at every opportunity is liable to have be flying at a different true altitude due to pressure change.  A change in altitude can be affected if the temperature of the air lapse rate is different than standard.  I have had an altimeter make a sudden increase in reading just because I flew into a narrow Nevada valley where the bernoulli effect of the parallel winds had lowered the air pressure.

On a recent flight to the L. A. basin we were departing the Class B airspace to the West toward San Luis Obispo and advised SoCal Approach that we were leaving 4,500’ for 8,500’.  Shortly afterwards ATC asked what our altitude was.   We said that we were climbing through 7000 for 8,500.  ATC advised us that our Mode C showed a descent through 4,600’.  We were in severe VFR and could plainly see that we were climbing.  We turned off the Mode C but the conflict of the two altimeter readings could have had very serious consequences at night or in IFR conditions.

The altimeter, invented 150 years ago, consists of a sealed aneroid wafer connected by gears to the Kollsman window knob and to the altimeter needles.  As air pressure changes with changes in altitude the wafer expands and contracts to move the attached gears and needles as calibrated to read barometric pressure.  Where the Kollsman window knob allows you to set the altimeter to the local pressure setting you have a ‘sensitive’ altimeter.  Turning the knob moves the altimeter hands and the Kollsman scale at one inch for every 1000 feet between pressures of 28” to 31” in.Hg.  This setting is required of all pilots in the area much as though all pilots would set their watches to a common time.   No altimeter is capable of correcting for nonstandard temperatures.

Weather services and scientists  use mercury barometers which are about 32" long.  Every FAA tower has such a barometer usually in the stairwell.  An aircraft altimeter becomes "sensitive" when it has a Kollsman window and adjustment knob.  At field elevation your altimeter should show your true altitude above mean sea level.  The altimeter setting converts difference between mean sea level pressure and its sensed pressure into an altitude.

Except for variations caused by lapse rate temperatures, an altimeter will move one inch of mercury for every thousand feet of altitude difference.  As an aircraft gets higher the altimeter has ever more difficulty in accurately converting the difference.  For this reason, pressure altitude of 29.9213 inHg/1013.250 mb is used above 18,000'.  getting a current altimeter setting resolves most errors.  Altimeters use temperature and lapse rate standards.  The Kollsman knob adjusts for pressure but not for temperature.  As temperature changes so should the altimeter change.  But it doesn't.  To the extend that the lapse rate (temperature change per 1000') varies from the standard so will the altimeter become inaccurate even though it is set to the correct surface altimeter setting.  True altitude is affected by temperature and requires a correction when temperature differs from standard.  Standard temperature is 59°F or 15° C along with the above pressures is knows as the international standard atmosphere. (ISA)

Flying at 6000' indicated and altimeter set at 29.92 we fly into colder air.  Cold air is more dense so you will be lower than your altimeter is reading.  When you are flying toward colder air, you are lower than your altimeter indicates.  When you are flying toward lower pressure, you are lower than your altimeter indicates.  If the air temperature is very cold, an altimeter will read higher than its true altitude.  The plane will be closer to the ground than you think.  When flying from high to low look out below applies to both pressure and temperature.  Narrow valley passes have a venturi effect that will lower air pressure and give false altimeter readings.

With the introduction of metric altimeters as well as English system altimeters it is easily possible for altimeter setting errors to occur.  If the ATIS or ATC should cause you to read and set 2991 inches as 991 millibars, as might occur out of the U.S., you would be 640' lower than expected.

All aircraft are required to have an operable altimeter and IFR aircraft

must have one with an accuracy within 75 feet.  Due to inherent lag factors in altimeter construction all pilots learn to lead leveling off in both ascent and descent.  GPS altimeter readings rely on data received from three satellite transmitters.  GPS readings are unaffected by temperature.

# Altitudes:

1. Indicated altitude

The altitude shown on the altimeter (uncorrected for temperature)with the Kollsman window setting.  Most sensitive altimeters contain a bimetalic strip that compensates for temperature changes.  This is the altitude you see with the current sea level pressure setting.  Indicated altitude as read is not affected by temperature.

2. Pressure altitude.

Altitude in relation to a constant pressure plane of 29.92 inches of mercury.  Determined by setting aircraft altimeter to 29,92.

Pressure altitude in combination with temperature is used in the POH to tell the pilot of expected performance parameters.  These combinations are the international standard atmosphere (ISA) standards.  Since the temperatures are standard for the altitude these are density altitudes.  Using our flight altitude and temperature we can calculate the density altitude and by referencing this to the POH we can figure expected performance.  Used for computations.

3. Density altitude

Pressure altitude corrected for nonstandard temperature variations. The altitude at which the airplane "thinks" it is operating at.

It is the mass of a given volume of air rather than its weight.  It is not related to height but rather to aircraft performance potential.

4. True altitude

Actual height above mean sea level.

5. Absolute altitude

Altitude above the surface.

Given to you by a reporting facility corrected for non-				standard surface atmospheric pressure.

7. Ground Zero altitude

Zero set of altimeter at ground level

## Vacuum System

The vacuum pump runs the heading indicator and attitude indicator.  The life expectancy is about 1200 hours.  The suction gauge is very small and usually placed outside the common instrument scan.  All the common vacuum pumps are bi-directional.  One brand has angled vanes that could be damaged if they were turned backwards under load.  This is the only reason an aircraft propeller should not be turned backwards.  On the positive side, it is impossible to have an engine start when turned backwards.   Even the slightest forward motion may start an engine with a shorted P-lead.

# Gyroscopic Instruments

The common instruments using a gyroscope are the needle, the turn coordinator, the heading indicator, and the attitude indicator.  A gyroscope reacts to a force applied as if it were 90 degrees in the direction of spin from where it actually was applied, this reaction is precession.  Precession is used in turn-and-slip indicators and turn coordinators.  Gyros also have inertia, a desire to remain in an established position.  Attitude indicators and heading indicators use this attribute called rigidity in space.

The gyroscope was first invented as a toy by the Frenchman Jean Foucault around 1850.  When any force is applied to an operating gyroscope it attempts to precess.  Precession means that a unique gyroscopic process occurs that gives a force 90° to the direction of the applied force.  Gyroscopes can be powered by either air or electricity to operate autopilots, heading indicators, turn coordinators, attitude indicators,  and horizontal situation indicators.

By various gears and linkages at gyroscope can be installed in a rigid or semi-rigid position.  The rigid mount is used for the attitude indicator with the gyro mounted straight up and linked to always seek that upright position.  The heading indicator has linkage to change its relative movement into dial indications on the vertical card.  The Gyro on the turn coordinator has the gyro set at 30° so that it can react to both roll and yaw.

# Attitude Indicator

The attitude indicator is a vacuum (or electric)driven gyroscopic instrument that obeys the Laws of Newton relating to motion and deflection.  The vacuum pump provides from 3.5 to 5.0 inches of mercury suction power capable of driving the heavy gyro wheel at 17,000 rpm.  At higher altitudes it may be unable to provide sufficient power. It is a required IFR instrument.  The gyro remains in a fixed position due to obeying Newton.  The aircraft moves around the gyro.

Small movements of the airplane cause the AI gyro to react.   The gyro wheel is vacuum driven and quite heavy for its size.  Small cups are cut into the circumference which catch the movement of the vacuum air and spin the wheel at about 17,000 rpm in the vertical axis of the aircraft.  The AI case and its gimbals move around the pitch and roll axes while the high-speed gyroscope remains rigidly vertical.

When making a 180 degree steep turn and then rolling out your attitude indicator will show a nose up and wing down in a direction opposite the turn.  The error is inherent to the erecting mechanism.  The errors tend to cancel in 360 degrees.  Skidding turns cause precession and a proper turn but the recovery will show a turn to the outside that does not exist.

Taxiing turns are always skidding turns.  It is normal for the AI to show up to five degrees of tilt during taxi turns.  Any more requires investigation.  AI gyrations during initial start of the engine is normal.  High thrust takeoffs can cause an acceleration error seen as a false roll of up to five degrees.   In low visibility departures use the turn coordinator and heading indicator during the acceleration period.

The attitude indicator can be checked for correct operation after starting the engine if the horizon bar erects to the horizontal and remains stable.  Some horizon bars will vibrate on erection and gradually slow down and level off as a normal erection.  If the horizon banks over 5-degrees during turns it is unreliable for IFR use.  The AI has weights which sense gravity and will cause precession in an extended moderate banked turn back to a level condition.

# Heading Indicator

The gyro of the heading indicator spins on the aircraft's horizontal axis.  The gimbals suspension connects to the flat face compass card of the instrument through a gearing system.  The compass card turns in reaction to yawing that occurs during an aircraft turn.

The heading indicator is subject to precession and should be reset periodically by reference to the compass.  This resetting can only be done accurately during level and unaccelerated flight.  See the compass material to see how compass errors can occur.  Resetting errors are a common failing in cross country navigation.  Always set during your run-up.  Check the setting on alignment with the runway.  Wait until reaching level and unaccelerated flight for the next set.

# Turn Coordinator

The turn coordinator is usually an electrically driven gyroscope.  It is mounted transversely and parallel to the wing. at an angle of 30 degrees with the back slanted downward.  It is dampened to reduce reaction to turbulence.  It originally served to control single axis autopilots. It senses both yaw mostly and roll rate slightly.  Initially it senses roll and when the bank is established it senses yaw (rudder) input.  Holding the miniature plane's wing on the turn index marks will give a two-minute 360-degree turn.  This is the standard rate turn used in instrument flying.  A pilot who uses the TC as an AI will make a unusual attitude even worse.  The TC is an indicator of yaw and requires application of rudder.  I once discovered this problem with an IFR student who had previously failed another instructors sign-off for the check ride.  I have no doubt that the failure saved his life.

During taxi it will turn in the direction of the turn and the ball will remain centered.  Knowing the airspeed,  the angle of bank can be inferred.  The TC ball (slip-skid) indicator is smaller than that on the older needle and ball instrument.    The TC will indicate the direction of a spin so you will know which rudder application is opposite.  This is not true if the spin is inverted.  A failed TC will "park" level.  A very good reason not to use as a level flight indicator except in emergency.  TC cannot drift or tumble.

Overhaul calibration of both TC and Needle is by adjusting centering springs.  As the springs weaken with age sensitivity to yaw increases so does gyro friction increase with age with a decrease in sensitivity.  Sensitivity is a function of gyro speed.  When the gyro speed falls below 15000 they will precess.  The net effect of these changes are unpredictable.  Failure of gyroscopic instruments begins when gryo speeds fall below 7000.

# Needle and ball

The needle's gyro axis is mounted horizontally and spins up and away from the pilot.  It tilts on the roll axis up to 45 degrees.  It cannot move on the vertical axis.  A yaw of the aircraft causes the gyro to yaw in the opposite direction.  Reversing linkage gives correct direction and amount.

Oscillations are prevented by a dashpot mechanism.

# Needle vs. Turn Coordinator

While these instruments indicate bank angle or turn rate they are designed to initially sense the rate of turn and once stabilized it senses the rate of yaw. (Examiner question)  They contain electrically driven gyros and have a relatively low failure rate due to lack of complexity.  These instruments are the most positive way to determine the direction of a spin by reference to the L/R letters.

# Gyroscopic Failure

The major accident problem with gyros is that their actual failure in IFR

conditions is so rare that the detection, ignoring and partial panel required of the pilot is usually too late and too little to prevent an accident.  Sudden failures are more likely to be non-accident than are subtle failures.  A trained pilot who has enough time to check for contradictory instrument information.  The vacuum system is more apt to fail than is the electric system.  The standard maintenance process is to run the vacuum pump until it fails.  Fortunately most failures occur in VFR conditions.   Electric gyro failures are more likely to result in

serious accidents because the failure rate is so low.

Panel signs of impending gyro failure can be high/low suction readings,

precession that requires constant resetting of HI, gyro noise, erection time, and failure to react during taxi turns.  Gyros operate at speeds up to 20,000 rpm.  Non-use as well as shock can cause the bearings to develop imperfections or gunk detrimental to safe operation.  Older gyros will tumble during maneuvers exceeding 55 degrees unless caged ahead of time.  Modern gyros do not tumble but all gyros are subject to precession caused by either rotor imbalance or bearing friction.

In the event of a gyro failure that is detected by the pilot, he should cover the failed instrument.  I have post-its. always stuck to my lap board for this purpose among others.  Take a look at the compass.  Get it stopped and you are level.  Find the corresponding level gyro, if any.  South is the most stable compass direction in which to fly.  Get straight and level, trim and let go of the yoke.  Fly with rudder.  Now get help from ATC.

# Compass

Mounted on the face of an aircraft compass is a chart.  This chart is

a record of compass error called deviation.  As a pilot of a particular aircraft you should make a copy of this chart for navigational planning.  The upper level of the compass fluid should not be visible.  The maximum deviation allowed by regulation is 10°.

There are three errors made by the compass that are caused by magnetic dip.

Magnetic dip is them vertical attraction of the Earth’s magnetic field.  It exists world wide and is related to your latitude.  The errors caused by magnetic dip are acceleration error which is related to changes in speed when flying east or west.  ANDS is the mnemonic.  Acceleration causes the compass to indicate a turn to the north when no such turn has occurred.  Deceleration causes the compass to indicate a turn to the South when no such turn has occurred.  Northerly turn error is effect of magnetic dip that causes the compass to indicate a turn opposite to the turn actually made when beginning from north.  The compass is then lagging behind the actual rate of turn being made by the aircraft but attempting to catch up.  Heeling error is the tilting of the compass caused by the banking aircraft where it tries to remain horizontal but heels to an angle on the pivot until reaching it’s stop limits.

Many airports have an area set aside for a compass rose.  This rose is

aligned using the variation between the True North and Magnetic North for this specific area.  It shows the magnetic directions such as are used to align the airport runways.  It is a place where aircraft are positioned and 'swung' through the various magnetic directions.  This 'swinging of the compass' is done with all electric circuits functioning so that the operation of the compass will reflect this fact.

As the aircraft is positioned on the eight magnetic courses a small

pair of adjustable magnets are moved by screwdriver so  as to get the most accurate compass reading possible.  As these adjustments are made, a record is made of the compass direction of the aircraft on the rose as compared with the best adjusted reading of the aircraft compass.  This record is transcribed on to the deviation chart affixed to the compass.  If this card is not filled out, your aircraft is not airworthy.

In most cases this deviation is only one or two degrees.  This exceeds the straight line flying ability of a pilot.  There is no minimum deviation.  Whatever the deviation for the cardinal headings may be it must be recorded on the compass deviation card on the compass.  However, with the advent of the Global Position System, it is becoming important to note deviation as a factor in navigation beyond the Practical Test Standards or the FAA written.

The numbers on the compass are opposite in order and direction from the DG.  The reversal of the two numbering systems requires us to be consciously aware of the difference.  Setting the compass to the DG requires that we note the compass lubber line is between two numbers either centered or near one than the other.  These two numbers are then located on the DG and the DG set to correspond.  A very common student error is to make a mistake setting the compass.

Flying with the compass is quite different from using the directional gyro.  The compass has several inherent errors relating to turn, speed and geographic latitude.  The compass should only be "read" in level unaccelerated flight.  It is best (easiest) to make compass turns using the turn coordinator and time.  At 3 degrees a second a turn of two minutes is 360 degrees, one minute 180 degrees, 30 seconds 90 degrees and 10 seconds 30 degrees.  A normal count of 1, 2, 3, (4) will be close to 10 degrees.

The errors of the compass are caused by magnetic dip. The swinging and dipping of the compass during a turn or acceleration due to changes in speed or direction is a physical phenomenon caused when the north seeking end of the compass dives toward the north magnetic pole which is far underground on a northern Canada Island.  The magnetic pole is over 1200 miles from the North Pole.  The higher the latitude the greater the dipping tendency as the compass points down.  Turns from a northerly heading lag behind the turn; turns from a southerly heading lead the turn.  ANDS is the mnemonic for acceleration errors.  See AC 61-27C pg 44.  The best way to avoid magnetic dip errors is to only take magnetic compass readings from level unaccelerated flight.   This is when the compass needle will be most nearly parallel to the earth's surface.

If only the compass is left as an operating flight instrument and you do not have visual reference to the ground, a controlled descent can be done using the compass and holding a heading of 180 degrees.  On the south heading (only) you can keep wings level in clouds by reacting with rudder to small changes in the compass.  When turning to the north the compass leads the turn by the latitude of the aircraft.  When turning to the south the compass lags the turn by the latitude of the aircraft.

Compass turns can be made by time with a standard rate of 3 degrees per second.  In level unaccelerated flight the compass is unaffected by turning or acceleration errors.  The compass is the self contained and independent means of determining flight direction.

Rule of Thumb compass turns:

1. When turning Northerly, undershoot the heading by the latitude

in degrees plus half of the bank angle.

2. When turning Southerly, overshoot the heading by the latitude 				minus half the bank angle.

3. When turning East or West from the South roll out 5 degrees 				early.

4. When turning East or West from the North roll out 10 degrees 				early.

# About the Compass

Northerly turning error is the most noticeable of the compass errors.  When the compass drum is tilted the magnetic attraction of the earth causes the north-seeking end of the compass to dip to the low side.  This causes the compass to read inaccurately.  During a 360 the compass will read most accurately at due East and West headings.

Flying East or West will not have error when turning either North or South.  An increase in airspeed while going East or West will cause the compass to show a turn to the North although the aircraft maintains a constant heading.  A turn to the South is indicated when the aircraft is slowed.

Any turn from a southern heading will show more turn than is actually made.  Any turn from a northerly heading will show the compass indicating a turn in the opposite direction.  The amount of lag decreases as the turn continues.

# Avionics

Moisture, heat, and age will affect your avionics.  You are helping the function and longevity of your equipment if you turn everything on for every flight.  Dust may not be preventable but corrosion of contacts can be prevented by running the knobs through a full cycle of frequencies.  The day of WWII 521 maintenance is no longer with us.  (The 521 was a very early British airborne radar set.  The cannon (screw on) cables plugs were a constant source of failed equipment.  Solution: Lean back and give the

nearest box a kick.)

Don't start your engine with any avionics on-line.  Shut off all radios before stopping the engine.  The starting and stopping of the engine causes the alternator to send voltage 'spikes' or surges through the electrical system.  Most radios now have built-in surge protection but they don't always perform correctly.

If your avionics starts to misbehave, try to isolate the problem by trying different microphones, switches, headsets, or intercoms.  Note the ampere reading changes when going from off-to-on and back.  Try other frequencies.  Check knobs, selectors, squelch, and volume.  Be sure to tell the technician if the problem is intermittent, new, sudden, etc.

Transistor radios do not fail a tenth as much as did tube-type or hybrid of only a few years ago.  NORDO (no radio) procedures have nearly ceased to be a part of flight instruction.  For every transmitter you have two receivers.  At transmitter failure turn up the volume on all your receivers.  You will hear others but no answers to your calls.  Try different microphones or switches.  Listen for the activation of the transmit relay.  Always check to confirm that your headset plugs are fully inserted.  Partial insertion will cause carrier-wave only transmissions.  A loose plug socket may not be making a proper ground connection.  Low voltage will cause the transmit relay not to activate.

If you can’t hear anything, you should initially check the audio panel switch positions of the selector, the frequency, and plug connections.  A stuck transmitter switch will also cause this problem and can be checked by using the VOR ident feature.  The auto switch allows you to compare phones with speaker reception.  Removing all avionics power may free a stuck transmit relay.  Try it.  When all else fails transmit in the blind on FSS frequencies and indicate that you will be monitoring VORs that have voice capability.

Avionics can only be installed by a airframe rated mechanic with a valid inspection authorization or at an FAA-certified Report Station with an avionics rating.  The FAA sets the standards and determines who can make internal adjustments to avionics equipment.  A new aircraft weight and balance must be computed and the equipment list revised when installing or removing avionics.