Speedsã

Airspeed

Pitch vs. Power

Pitch for speed

Airspeeds

V- Speeds

V Speeds and Flaps

Standard performance profiles

Pattern speeds

Variations on a Theme (Instructor)

# Airspeed indicator

The airspeed indicator uses the differences between the static pressure and the pitot pressure to display airspeed.  The pitot tube takes ram air pressure (not a flow of air) from aircraft motion to drive the diaphragm of the airspeed indicator.  The static air hole(s) takes the ambient pressure of the aircraft and registers this pressure on the altimeter, the vertical speed indicator and the side of the airspeed diaphragm opposite the pitot's ram air side.  The IAS requires both the pitot and the static to operate.  The pitot tube usually has a heater element around it to melt ice as an obstruction.  It would be useless against an embedded insect.  Static stystem failure component by component or total is nearly impossible to detect during preflight.  The functioning of the alternate static souce is best checked during climb and descent for detecting a partially blocked system.

The airspeed indicator is color coded to show certain ranges of flight operation.  White is flap range with Vso at the slow end and Vfe the high end.  Cumulative damage will occur if flaps are lowered at speed higher than this range. The green range is normal with Vs1 as the gross stall speed without flaps.  Where the green meets the orange is Vno listed as the maximum structural cruise speed.  The orange range is to be avoided in turbulence.  The high end of the orange range is capped by the red line.  All warranties of structural strength are voided when

speed meet or exceed the redline. All marked color codes are based on indicated airspeeds.

The airspeed will read slightly slower when you climb and slightly faster when you go down.  If the pitot tube is blocked the airspeed indicator will work like an altimeter.  As altitude is gained the ias become s greater than would be expected.  As with the ear the airspeed works best which air pressure is equalized.  There is no required inspection of the pitot-static system. There is a required inspection of the static system which involves the altimeter only.

Many need to know speeds are not shown on the airspeed indicator.  Va the

maneuvering speed at which warranties are voided if abrupt full control movements are made.  Airspeed indicators since 1976 have been standardized as to panel location and to reading in knots with mph as an inner line of readings.

# Airspeed

Every airspeed is the end result of thrust overcoming drag.  The relative movement of the plane to the earth and the air above is the end result of pilot settings of pitch and power.  Airplanes need speed to fly.  However, the recording of this speed does NOT use moving air; instead, it uses air

pressure.  You should know that moving air does not enter the pitot tube.  Only air pressure is applied through the pitot tube.

The pitot tube pressure can be indicated in several ways but the most common is a differential pressure indicator that measures the difference between impact pressure and static pressure on different sides of a flexible air chamber.  A movable arm is geared to the air chamber so that diaphragm movement is measured on the airspeed dial.  The pitot tube measures impact pressure while the static tube measures undisturbed air pressure.  A single static port has inherent errors that occur when uncoordinated flight disturbs the air at the static hole.  The resulting speed measure, called indicated airspeed (ias) is uncorrected for the plumbing installation, air density, or instrument imperfections.

Over the years the markings of airspeed indicators have remained much the same.  However, the source of the markings have varied.  In the 1970 airspeed indicators were usually in miles per hour and in calibrated speeds.  Calibrated airspeed is indicated airspeed adjusted for installation and interment error.  Generally the airspeed correction tables have indicated airspeeds that show lower and higher than calibrated speeds.  This is a built in safety factor in that when you are indicating a slow speed you are not quite as slow as you think you are.  When you are fast you are not quite as fast as you think you are.  Calibrated airspeed should always be used to calculate the 1.3 Vso approach speed and then converted to indicated airspeed for the actual approach.  1.3 Vso is the speed to use if no maneuvering is required on final.  The final authority for any aircraft is the appropriate model and year matching the airplane to the POH.

Calibrated airspeed is different from indicated airspeed in that it makes corrections for installation, density, and instrument errors.  The range of difference between indicated and calibrated airspeeds are shown on a chart in the POH.

The calibration of an airspeed system is based on standard conditions of pressure and temperature.  As density decreases with altitude the speed of the aircraft must be higher in order to achieve the same instrument reading.  While the indicated speed will decrease with altitude due to this decreased impact pressure, the true airspeed will increase.  True airspeed is available in the POH for planning purposes.  True airspeed can also be calculated in flight using E6B calculations.

Instrument aircraft often have an alternate air valve that allows cabin air to replace the blocked exterior static hole.  When being used, the alternate static air causes the altimeter to read high, the airspeed to read high, and the VSI to show a climb for level flight.

# Pitch vs. Power

For years old timers and the FAA have been arguing what controls airspeed and altitude.  'Stick and Rudder' pilots believe that elevator controls airspeed.  The FAA demands that the elevator controls altitude with attitude controlling airspeed.  The conflict has became one of theory against reality.  Will an FAR soon legislate the laws of physics?

The altitude and airspeed performance is not independent of either attitude or power.  The pilot is the controlling factor.  What he does in the cockpit with the elevator set attitude and what he does with the throttle sets the power.  One control or the other is a factor in all flight and only in some situations does one dominate the other.  How well you know your airplane’s flight characteristics is as essential for all flight regimes not just landings.  Every aircraft has idiosyncrasies your checkout must expose you to those.  Otherwise, you become a test pilot.

In cruise flight and constant speed approaches the elevator dominates attitude and altitude while power sets airspeed.  The ability of the elevator to exercise this control exists when power is both variable and available.  It is only when power is not available or considered as a locked constant that the elevator can control airspeed.  The FAA expects you to use the vertical speed indicator (VSI) (elevators) to maintain pitch and the throttle to keep airspeed.  This works for flying the instrument landing system (ILS).  If the throttle is set as a constant 1500 rpm then the trimmed elevator can control airspeed.  The FAA accepts this idea that elevator controls airspeed when power is constant.  The trimmed elevator gives greater control over the glidepath than power.

Power is a relatively coarse adjustment to the glide path.  Massive reductions or increases in power produce illusions of change, especially, if airspeed changes occur simultaneously.  Only by keeping the airspeed constant can the illusions of change due to power application be overcome.  Mistakes, and corrections, in being high and low on approach will be an important part of your landing training.  Reduction of power in increments of 100 rpm can allow slight and smoother changes in the approach path.  Since the effects of power are so variable, due to inertia, most pilots chose to set power at predetermined levels and use the elevator to set the trim to give the attitude most likely to meet the airspeed sought.  Why?  Because it works.

We use pitch as a form of energy control.  We can covert kinetic into potential and potential back to kinetic.  With speed we can use the elevator control to create a climb at a cost of airspeed.  With altitude we can use the elevator control to forgo altitude in exchange for airspeed.

In reality the pilot controls the situation by using a combination of elevator and power which through this combination determines airspeed and altitude.  At any given moment the pilot will make a control decision between altitude or airspeed and use power or elevator, in combination, to meet the needs of that decision.  Where ever the FAA does not have a recommendation as to procedure "...a coordinated combination of both pitch and power adjustments is usually required".  The Flight Training handbook is being rewritten as of 1995.  Hang on.

# Pitch for speed

1. Best angle climb - obstacle clearance - most altitude over distance.

Full power, level wing, set wing angle, hold nose, trim

2. Best rate climb - most altitude over shortest time

Full power, level wing, set wing angle, hold nose, trim

3. Cruise climb - most distance over time + altitude

Full power, level wing, set wing angle, hold nose pitch attitude same

as slow cruise with reduced power, trim

4. Cruise - Most distance over time - select altitude carefully within 3000 	feet above ground.  Same as acceleration for takeoff, best glide at 	idle, slow cruise descent at 2000 rpm, power-on landing at 1500 rpm 	and approach, trim

5. Cruise descent - distance over time minus altitude.  Reduce power but

leaving trim alone will give level flight again just by replacing

the proper reduction.  Called economy of effort.

There are several descent speeds, a cruise descent is best for turning altitude into speed and distance.  You need to plan ahead to select cruise speed descent point.  Plan to lose two to three hundred feet per minute just by using carburetor heat and thumb pressure on the yoke. The way you fly airspeed in any of the foregoing situations is determined by power setting, configuration and trim.  You must know the proper timing for the power setting for each situation.

6. Power-off landing and approach

Not recommended unless engine is cooled down first.  Run at reduced

power in gradual descent.

# Airspeeds

Indicated airspeed:

(common usage) What the airspeed indicator shows it is a raw value. The air pressing on the pitot tube is compared with the pressure on the static port and registered on a dial as miles per hour or knots per hour. (Post-1970 aircraft give performance figures (stall etc) as indicated airspeeds.

Indicated airspeed:

(uncommon usage) To get valid indicated" airspeed you must know what the instrument error of a given reading may be.  Airspeed indicators allow up to 5 mph error on installation as new.  Indicators tend to hang up or ratchet with age.  An ever increasing amount of friction is also expected with age.  This also applies to people.

Calibrated airspeed:

Indicated airspeed corrected for installation errors.  A chart for calibrated airspeed vs. indicated airspeed is in the Pilots Operating Handbook.  (Performance Speeds of pre-1970 aircraft are given as calibrated speed.)  The indicated airspeeds tend to be lower than calibrated airspeeds on the slow end and higher than calibrated airspeeds at the fast end.  Most accurate in mid-ranges.

True airspeed:

Indicated airspeed corrected for temperature and pressure as different from standard. True airspeed increases with altitude.  True airspeed can be calculated on the E6B or in the cruise performance table of the POH.  At best this is a rough estimate.  GPS will give the most accurate true airspeed if the speed is determined over two reciprocal flight courses.  The spread between indicated airspeeds and true airspeeds increase with altitude.  Only Indicated airspeeds are used in taking off or landing an aircraft.

Ground Speed:

True airspeed corrected for wind effect gives ground speed.  Groundspeed is determined on the E6B wind correction slide.  Most radar services can give you a ground speed read-out.  (See DME, Loran, GPS)  At high altitude airports as much as 20% more ground speed will be required for takeoff.  The same 20% increase in ground speed will exist at touch down.

# V- Speeds

These are the "Vital Velocities required for precision flight.  Many V speeds vary with the aircraft weight and not always in the way you might expect.  Va speed will decrease with weight, stall speed decreases with weight as does best glide speed and approach speed.

Vso is the stall speed in landing configuration.  This speed is for level, unaccelerated, 1-G flight and a slow deceleration to stall.

It is at the bottom of the white arc of the airspeed indicator at gross weight.   At lesser weights it will be slower.  The change tends to be proportional, a 5% lower speed for a 5% decrease in weight.  The bottom of the white arc is the stall speed for maximum landing weight at the most unfavorable but allowable center of gravity location.  Depending on year of aircraft this may be either calibrated or indicated airspeed.

Vs 1 is stall speed in a clean configuration and is the bottom of the green arc. This speed as well as Vso is for level, unaccelerated, 1-G flight and a slow deceleration to stall.

Vfe is at the top of the white arc maximum speed for flap extension.  The use of lower flap extension speeds reduces the strain on the system.  As aircraft age this reduction can be important.

Vlo is the maximum landing-gear operating speed.

Vle is the maximum landing-gear extension speed and is based upon the durability of the landing gear doors.

Vne  is the redline speed.  This speed is found by diving 1.4 beyond Vno to attain Vd design diving speed.  .9 times Vd = Vne.  Beyond Vne you become a test pilot.

Va, maneuvering speed is thought of as having to do with control movement.  Va, is the turbulent air penetration speed.  A full deflection of the controls will cause the stall before it folds, spindles or mutilates.  In normal category "clean" aircraft this load limit is 3.8 positive.  The load limit of an aircraft can be exceeded by a PIO (pilot induced oscillation) and turbulence in a seconds.  Get to or below Va even if you must stall.  Va is based on weight, the heavier the weight the higher the Va.  Reduce the maximum gross weight Va by a percentage equal to half the weight reduction.  10% weight reduction reduces Va by 5%.  I find that thinking of it as driving over country railroad tracks.  The lighter the car the higher the bounce.  Know your Va for gross from the POH and how to compute it otherwise.  Once way to determine the approximate Va at below gross weights is to change the Va by half the percentage of weight reduction.  Find the actual weight reduction below gross as a percentage of gross.  Increase the published Va by half of the weight reduction percentage.  A 30% reduction of weight would result in a 15% increase in Va.  Don't even "think" about what happens to Va in over gross situations.  Essential knowledge: Va maneuvering speed decreases with decreasing weight.  Va maneuvering speed is determined by aircraft weight.

When an aircraft flies at a constant speed a straight-line relationship between any increase in the angle of attack and the G-load applied.  At higher speeds an aircraft can reach a higher angle of attack.  With G.A. aircraft by the time the angle of attack becomes great enough to crunch the aircraft with G-load, it will stall.  It will not break at speeds below Va.  The aircraft that is flown in turbulence must fly at slower speeds, specifically Va or lower, so that abrupt G-loads produced by turbulence angle of attack changes will not exceed the structural capability of the aircraft.

When you fly slower you must increase your angle of attack to maintain level flight at 1-G.  A 3.8 increase in G-load is about 3.8 times the higher angle required for the slow speed.  The aircraft will stall before reaching this angle.  Hence, the value of Va in protecting the aircraft.  Additional weight also requires a higher angle of attack for level flight at 1-G.  So additional weight plus a slower speed compounds the G-load protection offered because they require a higher angle of attack.  Flying lighter and faster decreases the protection.  By reducing weight below gross by 2% we can reduced the Va by 1% .

Positive limit load factor for aircraft is +3.8Gs/-1.52 for normal category, 4.4 Gs/-1.76 for utility, and 6.0/-3 for acrobatic.  When you consider that the 'zero' G is actually 1.0 the ability of an aircraft to take G-loads is nearly the same in either direction.

Vno is the top of the green arc known as structural cruise speed.

It is used in turbulence that is different than Va; it is called structural cruise speed or Vno.  Unlike Va this is shown on the airspeed indicator as the meeting point of the orange (yellow)and green.  This is a speed below cruise that is recommended for rough air penetration.  Vno does not offer the structural assurances offered by Va.  At Vno the aircraft, as certified,  should not be structurally damaged by a 35 knot vertical gust.  This is not the same protection given by Va.

Vx is the best angle of climb speed which gives the greatest altitude over horizontal distance.  It is used to climb over FAA trees. (50')

Vy is the best rate climb speed.  It gives the most altitude over time.  It is used in noise abatement situations.  Vy is greater than Vx and decreases with altitude,  while Vx increases  the only time they are equal is at the aircraft's absolute ceiling.

Vref This is a reference speed based on Vso.  We can find the short final approach speed by multiplying Vso x 1.3 + 1/2 wind gust speed.

Vso is also referenced at every 10% reduction in weight gives a %5 reduction in Vso as the full flap landing speed.  This is your ‘over the fence’ speed.   It is a minimum normal final approach speed.  The hydroplaning speed is within a couple knots of this speed using 7 times (smooth) or 9 times(treaded) the square root of the tire pressure.

# V Speeds and Flaps

Vy decreases with flaps applied

Vx increases with flaps applied

Vx and Vy converge with flaps applied

Vx and Vy separations are greatest with flaps up

Vs is slower with flaps applied

1.3 Vs is slower with flaps applied

Climb rate decrease with flaps applied

Vx can be below Vs at maximum flap settings

1.3 Vs is slower than Vy (Behind the power curve)

1.1 Vs is less than Vx (Short field landings)

Sudden retraction below 1.3 results in stall

Flap use is restricted to 2G maneuvers

Deviate from the manufacturers V speeds and you will have reduced performance in every flight regime.  However, if you trust the performance figures in your POH you are an accident waiting to happen.  The book figures are for a new aircraft.  A ten year old plane with a mid-time engine may have a 20% performance deficit.  Instead of the book we must trust our experience and judgment augmented by local experts.  You could develop your own POH book for your aircraft and insert your ‘real’ performance figures.

# Standard performance profiles

You can develop your own standard profile for flying or landing any aircraft.  There are only a few flying profiles required.  They are takeoff, climb with its variations, level cruise with its variations, descent and its variations, missed approach/go-around, and landing.  For each phase there are variations of configuration  and airspeeds which can be checklisted to provide constancy in procedure and performance.  Once you have developed the procedures and profiles for one aircraft it is much

the same process for other types.  Standard profile development is needed

if you expect to have IFR competency.

Fixed pitch aircraft at gross weight climb at full throttle at Vy or Vx from the POH.  Simple, except that most of your flying is not done at gross.  You can roughly figure that for every 10% decrease below gross that the Vx and Vy indicated speeds will decrease by 5%.  For cruise speeds always let the aircraft reach the desired indicated speed before reducing power.  If you reduce power before or afterwards you will begin a cycle of changing speeds, trim, and altitudes that lead to frustration.  To make the required power change you must know the required power.  Cycle through several climbs and cruise changes until you can anticipate your level off at 75% power by knowing where to set power and trim and the sequence required.  Go through the procedure again to determine the requirements to maintain low cruise and slow flight.  Follow the same process for going from level cruise, to low cruise, to slow flight and back again until you can make changes without hesitation.  Make your own checklist.

# Pattern speeds

The next major phase is the descent.  By using the POH Vy speed, as adjusted for weight, and adding about 30 knots we have obtained the approach speed for that weight that will give a 500 fpm descent.  for the desired 3° approach slope at 10 miles we should be a 3000 feet.  At five miles you should be 1500 feet above touchdown.  At your two mile report or downwind entry you should be a pattern altitude but no less than 600 feet.  Every additional drag configuration will contribute a 10 knot reduction in speed.  Appropriate trim adjustments must always be made to maintain the 3° approach slope.  If power alone is used to make a descent, you will find that 500 rpm reduction will approximate a 500 fpm descent.  Pattern descent power setting begin at the numbers.  From full cruise the C-150 first has carburetor heat applied and the throttle reduced to 1500.  You hold heading and altitude until reaching 60 knots.  The same is done with the C-172 except the power is reduced only to 1700 rpm and heading and altitude maintained to reach 70 knots.

Airspeed control begins with knowing the power setting required for any flight condition.  We climb, with full throttle, not with a range of speeds, we climb at a certain speed which the POH says will get us the highest in the least amount of time.  Vy is that speed.  It is 65 in the C-150 and 75 in the C-172.  These are gross weight speeds.  At less than gross a slightly slower speed by two or three knots would be Vy.  Noise abatement requirements and safety say that you should always climb at Vy.  Level cruise is 85 knots in the C-150 and 100 knots in the C-172.  All speeds are indicated airspeeds. The last speeds are landing speeds flown in both types of aircraft with a power setting of 1500 rpm.  The C-150 uses 60 knots in all configurations for normal approaches into the flare.  The C-172 uses 70 knots until final, which is flown at 60 knots.

Full power in all fixed pitch climbs is the simplest application.  When leveling off the power is kept fully applied until the C-150 has reached 85 knots and the C-172 has reached 100 knots.  On reaching these speeds the power is reduced to 2450 rpm for level cruise flight below 5000 feet.  This is very close to 75% power.  Above 5000 the rpm can be advanced 100 rpm for every additional 2500 feet of altitude.  Again, this is about 75% of power.  Leaning for best operation can be done at all altitudes flown with constant power.

As a student you begin your sight profile development by getting the required POH speeds.  You need to write in the changes as affected by weight variations of solo and dual.  Apply these changes as required by your normal operating weight.  Fly the proper Vref airspeed.  A standard profile works for the pilot who can fly the aircraft to the speed and performance determined to be safe.

More to come

Vlof

Vfc

Stall speed

Inverse effect on Va in turbulence

Va	Design maneuvering speed

Vb 	Design speed for maximum gust intensity

Vc	Design cruising speed

Vd 	Design diving speed

Vdf	Demonstrated flight diving speed

Vf	Design flap speed

Vfc	Maximum speed for stability characteristics

Vfe	Maximum flap extended speed

Vh 	Maximum speed in level flight with maximum continous power

Vle	Maximum landing gear extended speed

Vlo	Maximum landing gear operating speed

Vlof	Lift-off speed

Vmc 	Minimum control speed with critical engine inoperative

Vmo 	Maximum operating limit speed

Vmu 	Minimum unstick speed

Vne	Never-exceed speed

Vno	Maximum structural cruising speed

Vr	Rotation speed

Vs	Stalling speed or the minimum steady flight speed at which the airplane

is controllable

Vso	Stalling speed or the minimum steady flight speed obtained in the

landing configuration

Vs1	Stalling speed or the minimum steady flight speed obtained in a

specified configuration

Vtoss	Takeoff safety speed for Category A rotorcraft

Vx	Speed for best angle of climb

Vy	Speed for best rate of climb

V1 	Takeoff decision speed

V2 	Takeoff safety speed

V2min 	Minimum takeoff safety speed