# Uncontrolled airportsã

Non-tower Operation

The callup

Non-Tower Airport Arrival Procedures.

Visual Indicators at Uncontrolled Airports AIM 4-53

Airports Without Operating Control Towers

Common Mistakes at Non-Tower Airports

Uncontrolled Departures

# Non-tower Operation

(See Airspace-low visibility operations)

Most midair collisions and near misses occur within a couple of miles of non-tower airports.  Operations at non-tower airports are more dangerous than an tower controlled airports.  If you habitually fly under ATC control and assistance then you must step up a notch your see-and-be-seen skills when nearing non-tower airports.  There is no more dangerous place to fly than in the vicinity of a non-tower airport.

There are two different kinds of unicom.

1.)  Every tower airport has a unicom on frequency 122.95  The callup gives, "Airport name unicom, aircraft identification and request".  Due to the personnel or physical constraints several calls may be required to establish contact.  Commonly used to order fuel, services, or transportation.  122.95 is the universal nationwide unicom frequency for TOWER fields.  At such an airport unicom is used for ordering fuel, taxis, making phone calls or personal requests.  It is frequently advantageous to contact the unicom a few miles out so that transportation will be there when you land.  Some fields may have additional frequencies but 122.95 is standard.  Only at the very largest airports will this frequency operate on a 24 hour basis.

2.)  Many uncontrolled airports may offer unicom service on the CTAF frequency given on the sectional.  This means that there may be someone on the field to respond to a radio call during normal working hours.

# The callup

"Name of field unicom, aircraft identification, location, altitude, request traffic advisories( or other request)and name of the field."

If there is a unicom response it may be limited to suggesting a runway and traffic direction.  (The Good Samaritan situation has created liability they may not wish to assume with additional advice)  If there is not unicom response all further transmissions should be addressed to "traffic".

The FAA oversight of non-tower operations is there watching you.  I know a pilot who made a low pass over a runway in Oregon and got a 90 day suspension for breaking the 500' FAR.  While some of the FARs are very specific regarding operations much of the non-tower 'regulation' comes from the AIM or Advisory Circulars 90-42 and 90-66A).  FAR 91.127; 91.113 (g); 91.103 and 91.13  should be reviewed by the student before flight to a non-tower airport.  The AIM recommends only the 45 degree entry to a non-tower airport all others are not recommended and or considered as safe\acceptable. Parts of the AIM to be reviewed include  4053; 4-54; 4-8 and parts of AIM Chapter 5, Section 4.  AC 90-42F; 90-66A; and 90-48C are for guidance of operations.  Deviations of any parts of these at a non-tower airport are those which the FAA selects as coming under FAR 91.13, the catch all.  Byron airport has a mix of aircraft, helicopters, gliders, ultralights and parachutes.  This is a mix best avoided by aircraft.

The courts, NTSB have agreed with the FAAs determination that the guidance provided by the FARs, AIM and ACs constitutes acceptable and safe operations.  Everything else is relatively less safe.  Airport owners and operators establish traffic patterns. Standard pattern is always to left barring noise or obstacle requirements.  Arriving aircraft are expected to avoid the flow of traffic until established on the entry leg.

The entry leg on all FAA ACs  appears as a 45 degree entry aimed at the runway landing threshold.  The determination of the entry requires that wind, pattern, and traffic indications be confirmed prior to arrival at pattern altitude.  Plan your descent to arrive at pattern altitude at the downwind turn.  (AC 90-66A suggests that this turn occur at midfield.  This works fine for 5000' runways but a good size pattern for a 2500' runway requires that the turn be abeam the departure end.)  1000' AGL is the new suggested pattern altitude.  Some airports still publish and use the older 800' altitude.

The pattern altitude should be maintained at least until abeam the approach end of the runway.  (CCR authorities require it until turning base.)  Since there are local differences it is well to become familiar with the local requirements.  Base turn is recommended at the 45 (key) point. Turn to final recommended at least 1/4 mile out

The selected runway for common use should be the one most aligned with the wind.  A calm wind runway may be designated.  If a secondary runway is used, flight must not interfere with preferred flow of traffic.  Departing aircraft should not turn until beyond the departure end.  Go arounds should

not turn until well clear of any runway traffic and attempt to maintain visual contact.   While in the pattern turns should not be initiated until within 300' of pattern.  Such an altitude should assure the downwind turn being made at pattern altitude.   Departing aircraft should not make their departure turn until reaching pattern altitude.  (This is a slight change from what I have previously taught.)

FAR 91.1260-7 states that unless otherwise authorized all turns must be

made to the left.  Beyond this there is no ‘regulation’ that prohibits a

base or straight-in entry.  No turns are made in a straight-in approach.  Some airports require only a straight-in (Ruth-northern most airport on the San Francisco Sectional.)  A straight-in is only illegal if it interferes with other traffic.  A straight-in with a two-mile final has been judged a violation of the FAR under this proviso.

Helicopter patterns are expected to be at 500' AGL and inside the aircraft pattern.  Helicopter approaches will be steep.  Glider operations may have a pattern inside the aircraft or with opposite turns.  Ultralights

usually have a 500' pattern inside aircraft and their own runway.  Ultralights take off and land steeply.

# Non-Tower Airport Arrival Procedures.

(SEE RADIO PROCEDURES

1. PLANNING

Runways

Reporting points

45 degree arrivals only

Pattern altitude/direction

Frequency

Look for additional ways to determine

runway in use, such as arriving/departing aircraft, smoke/dust/waves, 	Nearby airports are good indicators.

Try to plan the most efficient arrival conducive to safety.

DON'T FLY INTO THE PATTERN ALTITUDE IN THE BLIND!!!!!

PRIORITIES:

- Fly the plane

- Maintain proper altitudes and headings

- Keep eyes outside cockpit and watch for traffic

- Communicate

122.9 Traffic only advisories at airports without frequency on sectional

123.6 Uncontrolled field with FSS for arrival and departure only

# Visual Indicators at Uncontrolled Airports AIM 4-53

DON'T MISREAD WIND SOCK, TETRAHEDRON, OR SEGMENTED CIRCLE!!!!

a. At those airports without an operating control tower, a segmented circle visual indicator system, if installed, is designed to provide traffic pattern information.  The segmented circle system consists of the following components:

1. The segmented circle--Located in a position affording maximum visibility to pilots in the air and on the ground and providing a centralized location for other elements of the system.

2. The wind direction indicator--A wind cone, wind sock, or wind tee installed near the operational runway to indicate wind direction,  The large end of the wind cone/wind sock points into the wind as does the large end(cross bar) of the wind tee.  In lieu of a tetrahedron and where a wind sock or wind cone is collocated with a wind tee, the wind tee may be manually aligned with the runway in use to indicate landing direction.  These signaling devices may be located in the center of the segmented circle and may be lighted for night use.  Pilots are cautioned against using a tetrahedron to indicate wind direction.

segmented

circle

wind tee    wind sock

3. The landing direction indicator--A tetrahedron is installed when conditions at the airport warrant its use.  It may be used to indicate the direction of landings and takeoffs.  A Tetrahedron may be located at the center of a segmented circle and may be lighted for night operations.  The small end of the tetrahedron points in the direction of landing.  Pilots are cautioned against using the tetrahedron for any purpose other than as an indicator of landing direction.  Further, pilots should use extreme caution when making runway selection by use of a tetrahedron in very light or calm wind conditions as the tetrahedron may not be aligned with the designated calm-wind runway.  At airports with control towers, the tetrahedron is lockable and should only be referenced when the control tower is not in operation.  Tower instructions supersede tetrahedron indications.

tetrahedron

4. Landing strip indicators--Installed in pairs...and used to show the alignment of landing strips (runways)

wind sock

5. Traffic pattern indicators--Arranged in pairs in conjunction with landing strip indicators and used to indicate the direction of turns when there is a variation from the normal left traffic pattern.  (If there is not segmented circle installed at the airport, traffic pattern indicators may be installed on or near the end of the runway.)

b. Preparatory to landing at an airport without a control tower, or when the control tower is not in operation, the pilot should concern himself with the indicator for the approach end of the runway to be used.  When approaching for landing, all turns must be made to the left unless a traffic pattern indicator indicates that turns should be made to the right.  If the pilot will mentally enlarge the indicator for the runway to be used, the base and final approach legs of the traffic pattern to be flown immediately become apparent.  Similar treatment of the indicator at the departure end of the runway will clearly indicate the direction of turn after takeoff.

c. When two or more aircraft are approaching an airport for the purpose of landing, the aircraft at the lower altitude has the right of way, but it shall not take advantage of this rule to cut in front of another which is on final approach to land, or to overtake that aircraft (FAR 91.113(f)).

# Airports Without Operating Control Towers

Traffic Pattern

At most airports and military air bases, traffic pattern altitudes for propeller-driven aircraft generally extend from 600 feet to as high as 1,500 feet above the ground.... Therefore, pilots of enroute aircraft should be constantly on the alert for other aircraft in the traffic patterns and avoid these areas whenever possible.  Traffic pattern altitudes should be maintained unless otherwise required by the applicable distance from cloud criteria (FAR 91.155)

A disproportionate number of near-midair collisions and actual collisions occur at uncontrolled airports.  Any pilot arriving at such an airport should pre-plan his arrival and have the cockpit so organized that maximum attention can be given outside the cockpit.  Seeing is harder in an unfamiliar situation that avoidance of what you see.  My using standard procedures you can better know where to look and what to look for.

As a former school teacher I have memories of playing a visual game with my classes on rainy days instead of going outside for P.E.   I would have one group of children put their heads down.  Another child would take a common object such as a key and place it in an uncommon place such as on a shoe,

head, door knob, or projecting from a crack.  On signal the selected group would raise their head and walk around the room trying to see the object.

As the different children would find the key, they would return to their seats without letting those still looking know that they found the key.  The quietest of the remaining groups would get to put their heads down next.  The children enjoyed the increasing tension and the emotional control required not to reveal they had found the key.  Neat game to tell any teachers you know.

The reason I tell the story of the game is that, always the same children seemed to have the ability and skill to find the keys irst.  There is a special skill in looking, seeing, and then recognizing what you have seen.

In my opinion, you have the required innate ability/skill or not.  Some of my students have consistently exceeded my ability to spot aircraft.  Physical conditions can be a negative/positive factor.  The size, glare, and clarity of the windshield, the existence or non-existence of haze will make a difference. Knowing where to look is an intellectual factor that any pilot can develop.

The most effective scan covers no more than 10 degrees of the window and proceeds in successive 10 degree jumps.  You cannot see when your eyes are

moving.  The eyes must stop and focus for a moment in order to see.   A broad scan must be done by moving the head and keeping the eyes stationary.

Additionally the focus distance (accommodations) must be varied from close to far.  This is most important when haze give an obscure view of distant objects.  Looking too long for distant objects in such conditions will cause the eyes to re-focus at about 20 feet.  Learn to react to things in your peripheral vision even if it is a bug splatt. Any aircraft that balloons in your vision without moving is going to hit you, just like the runway.

## Traffic Indicators

- At night or any other time, an orange light on top of the wind sock.

- The white traffic pattern indicators at right angles to the landing 		strip indicators show that a right turn is required.

- Traffic pattern indicators may be installed on or near the end of the 		runway.

While the 45 degree entry is recommended, the FAA has no suggestion as to how to arrive at the initiating point when no advisory can be obtained.  You should first overfly at twice pattern altitude to confirm wind direction and the pattern direction of the preferred runway. There are two ways I have seen the initial 45 entry reached.  Every key point of each procedure should be identified for other traffic.

1. At twice pattern altitude, cross over the runway at a 90 degree angle toward the downwind leg, cross that leg and fly on this heading at least another mile before turning 90 degrees parallel to the runway toward the upwind end.  At the 90 degree turn descent to the pattern altitude should be initiated. Eyeball for a 45 degree angle to the approach end and make your turn of 135 degrees.  For left traffic this will combine  two right turns, the first of 90 degrees and the second of 135 degrees. Draw this out around an diagram of an airport to see how it put you on a 45 entry.

What I dislike in this option is

a) I have had an instructor use it without allowing for a 				strong cross wind.  His turn to parallel the downwind put 			us right into the downwind traffic pattern.

b) The necessity for right turns restricts the see-and-be-seen 				requirement.

2.  From the approach end of the field depart on a heading that will take you across the pattern side of the field on a 45 degree angle toward the point of entry.  You will be on this heading if the runway # (number) is

on the right front 45 degree mark of your heading indicator.  Proceed on this heading until clear of the pattern and descend 500'   Initiate a course reversal (a required private pilot procedure) first 90 degrees to the right which would be away from the traffic and airport and then a 270 to the left back to the entry position.  On a correct 45 degree heading the runway # number will be on your left rear 45 degree heading marker.  This inbound checking system works for all 45 entries left and right..

# Common Mistakes at Non-Tower Airports

- Not calling up soon enough to get advisory

- Not observing traffic pattern/windsock direction

- Turning downwind too close to runway

- Becoming distracted and not attending to airspeed and

- pattern orientation

- Trying to hit the end of the runway

- Delaying addition of power if low

- Delaying go around if a poor approach/flare

- Not clearing runway immediately

- Discourteous, rude and arrogant behavior

- Flying a DC-8 pattern and not keeping your pattern square and tight.

- Make go-arounds to right side of runway

# Uncontrolled Departures

1. Full 360 degree clearing turn on ground in run-up pad prior departure

2.  Advise traffic of departure and direction

3. Do not make a 45 degree departure until past the departure end of the 	runway and at least half of pattern altitude.  A premature turn can

put you into conflict with traffic inbound on the 45.

4. Make any downwind departures so that the downwind is closer to the 	runway than would normally be used by landing aircraft.  This keeps 	you from crossing their 45 degree entry path.