Papersã

Know the Airplane

Aircraft Papers

Airworthiness

Supplemental Type

Airworthiness Responsibility

Is it Airworthy?

Insurance

Changing Type Certificates

Permanent Records

Engine Logs

Inspections

Operating without a (MEL)

MEL Decision Sequence:

Repairs

Airworthiness Directives (ADs)

Service Difficulty Reports (SDRs)

Service Bulletin

Maintenance Records

Airworthiness Directive

Maintenance Shakedown

Things that go wrong

Question

# Know the Airplane

Once you start flying a particular aircraft you should start learning how it differs from the information in the POH.  The POH is for a new aircraft with the best of pilots the manufacturer could obtain.  Begin by operating with the book figures.  Use radar or a GPS to obtain performance figures at 75% power. Have the aircraft refueled after each flight and develop a consumption table for the flying you do.  Develop your own tool kit as things need fixing.  Learn what is normal for this particular plane so you can become sensitive to such things as carburetor ice, an door ajar, a pending brake problem, control freedom, trim settings, seat problems, etc.

Reading FAR 91.9 says the aircraft must have an AFM/POH but FAR 21.5 says this is only required only for aircraft certified after 1979.  All prior to 1979 aircraft may fly legally with placards.

The aircraft is designed to crumple every place but the cockpit.  Where conditions do not permit an obstacle free landing, you should select obstacles that will absorb impact.  With secure and tight belts you will not bounce into the parts of the cockpit.  You are far more likely to be injured by beating yourself against the inside of the airplane than in any other way.  1/4 of all accidents are caused by some form of engine failure.  Mechanical failure is the form least likely to occur.  running out of fuel is most likely.

# Aircraft Papers

# Airworthiness

FAR 91.7 says the pilot determines the airworthiness of an aircraft.  The ARROW papers must be aboard and displayed in the aircraft.  If used for instruction the 100 hour inspections must be made.  Annual inspections must be made. Inoperative equipment cannot be required by aircraft type certificate or Airworthiness Directive.   Equipment not working must not be required for the type of flight to be made and must be placarded as inoperative by a certified mechanic.  Proof of airworthiness requires proper documentation of required inspections and compliance with all applicable airworthiness directives. Combine all of the above with a conscientious preflight and you have an airworthy aircraft in which to sit.  Technical compliance does not mean safe compliance.  Don't start the engine until you have all available information related to the movement or flight.  Read FAR 91.213.  This is an emotional area for pilots because so little is known about it.

On manufacture, a plane is given an airworthiness certificate that says the plane was made according to approved type design that met safety requirements.  This is attained when the required and proper components are installed and they are consistent with the drawings, specifications, and other data that are part of the type certificate. An airworthiness certificate is valid only if required maintenance has been performed according to FARs 21, 43, and 91.

When inspected in accordance with FAR 43.11(a)(4) says that the AI doing the inspection must certify that a given type of inspection was made and that the aircraft was determined to be airworthy.  Section (a)(5) of this FAR gives all the reasons for which an aircraft may not pass the inspection.

# Supplemental Type

Many aircraft are modified by a STC (Supplemental Type Certificate) which may require different than standard parts.  Both the pilot and mechanics must be made aware of any STC and its requirements.  Larger engines with higher fuel consumption are often installed with no change in fuel capacity and no entry into the Pilot Operating Handbook (POH).  The right to use auto gas is another STC.  The installation of a belly drain in a C-150/2 requires an STC.

The FAA does not require that the POH  be modified to show changes in fuel required by a larger engine.  Accidents cause FARs.  Have an accident and a create an FAR to cover the problem.  If the aircraft can meet the original POH performance figures no changes in those figures are required even though they may be considerably different than actual performance.

Unauthorized changes can affect the safe operation of an aircraft.  A fire extinguisher mounted under a seat in the aircraft can affect collapse design parameters.  Simple modifications to electrical systems can cause system failure.  Wheel fairing can affect performance and flight performance.  The balance envelope usually has a small window that can be drastically affected by movement of a small weight for a long distance.

A major change affects strength, weight, balance, performance, structure, operation, flight characteristics, or airworthiness.  These effects are most likely to occur when changes are made by other than standard practices.  The safest procedure for changes is to contact the FAA Flight Standards District Office before starting.

# Airworthiness Responsibility

Airworthyness is a shared responsibility between the owner, mechanic, pilot, and the maintenance and fueling organizations known as FBOs.  The pilot is responsible for airworthiness from immediately before the aircraft is entered for flight until it is tied down.  The pilot should make a comprehensive review of all the aircraft’s paper work in addition to a thorough preflight.  The mechanic’s responsibility depends of whether an inspection or maintenance was done.  For inspections, the mechanic’s responsibility ends on completion of the logbook entries.  The aircraft is airworthy at that moment.  However, the same mechanic is responsible for any repair or work done on the aircraft back to the original airworthiness certificate until the next  year’s annual inspection. As owner/operator you are primarily responsible for airworthiness including the accuracy and completeness of all logbook entries.

Records review should include verification of current annual inspection, 100 hour inspections for instructional aircraft, correction of any discrepancies found in previous flights, status of airworthiness directives (Ads), any deferred maintenance placarded according to FAR 91.213(d), current weight and balance, current equipment list, ELT battery expiration date, Biannual test of transponder, biannual test of altimeter and static system (IFR), any work done under FAA Form 337.   If the type certificate data sheet indicates the existence of an approved flight Manual then that manual must be in the aircraft.

- 91.403 ...the owner or operator of an aircraft is primarily responsible for maintaining that aircraft in an airworthy condition and for ensuring that maintenance personnel make appropriate entries in the aircraft maintenance records indicating the aircraft has been approved for return to service.  It is the responsibility of the owner and operator to have maintenance performed which may be required between scheduled inspections.

- 91.7 "...the pilot in command of a civil aircraft is responsible for determining whether that aircraft is ready for safe flight." Pilots must take reasonable care to find that an aircraft is airworthy and safe for flight.

- Aircraft must (1) meet the requirements of its type certificate and (2) be in condition for safe operation.  The type certificate verifies the design of the aircraft.

- Proof of airworthiness  requires written proof of required inspections and compliance with airworthiness directives knows as ADs.  “terms and conditions” of the airworthiness certificate requires that all maintenance and changes are performed according to FAR Parts 43 and 91.  FAR 45.11 requires an external data information plate be on the aircraft, usually in the tail section.

- Maintenance must give a detailed description of the work performed, date of completion, signature, certificate number, and kind of certificate of the person approving aircraft’s return to service.

- Records must show total operating time, time on limited life parts, time since overhaul, current inspection status, airworthiness directive compliance and any FAA Form 337’s of major alternations or repairs.  The parts and materials businesses must operate in such a way that expense is not and should not be the primary concern of the pilot or owner.  Safety is.  Where a maintenance problem exists, a passive approach is not going to solve the problem.  A responsible person of the organization must take both liability  and  responsibility for what has occurred and provide the solution.   Supervision of fueling is a most common source of difficulty.

# Is it Airworthy?

- Annual, 100 hour, or progressive inspection

- Any discrepancy corrected found in preflight

- AD status current and in compliance

- FAR 91.213 inoperative items placarded and recorded

- Weight and Balance papers

- Equipment list (see manual)

- ELT expiration

- Biannual on transponder

- Biannual on altimeter(IFR)

- FAA Form 337’s

# Insurance

A standard airworthiness certificate must be in full force and effect for insurance to valid. If the FAA grants a waiver for a ferry flight the insurance company must be informed.  Compliance with the FARs may not satisfy the insurance company.

# Changing Type Certificates

Approved changes to an FAA Type Certificate may be:

Supplemental Type Certificates  (STCs)

FAA Field Approvals (FAA Form 337)

Airworthiness Directives (FAR 39)

# Permanent Records

- Total time on airframe, engine, propeller, since last overhaul of items 	required to be overhauled.

- Status of life-limited parts, inspections, airworthiness directives.

Copies of all FAA Form 337s with duplicate to FAA in OKC

- IFR records on pitot/static system, altimeter, transponder, encoder and VORs.

# Engine Logs

Overhaul

Does not change prior history.  Time is added as "since overhaul".  		Work by FAA licensee.

Rebuild

Time in service begins anew.  Rebuild must be by FAA approved 			depot.

# Inspections

FAR 43.11(a)(b)

Type, description, date, total time, signature, certificate type and 	number; approval or disapproval for return to service

Routine

100 hour, minor alterations.  A & P sign off required

Required

Annual or progressive, major alteration AI sign off required; 				compliance with ADs.

Manufacturer's

Not in FARs but require A & P signoff

# Operating without a (MEL)

Minimum Equipment List

Most older aircraft have been certified without a MEL but everything operational when the aircraft was certified must be operational for flight--including the cigarette lighter and tire pressure.

Without an MEL any inoperative item must be removed or deactivated and then placarded.  An certificated mechanic must do the work, adjust the weight and balance, complete FAA Form 337 and approve for return to service

# MEL Decision Sequence:

The pilot recognizes inoperative instruments or equipment

Question:

Is the item required by MEL or kinds of operations list?

If "yes" aircraft is unairworthy!

If "no" 91.205, 91.213(d)(2)(ii)

Is the inoperative item required for this flight?

If "yes" aircraft is unairworthy!

In "no" 91.213(d)(2)(ii)

Is the item required by airworthiness directive (AD)?

If "yes" the aircraft is unairworthy

If "no"  39, 91.213(d)(2)(iv)

Is the item requires by FAR 91.107, 91.171, 91.185, 91.205, 91.207, 			91.209 etc 91.213(d)(2)(ii)

If "yes" the aircraft is unairworthy

If "no"

The item must be removed or deactivated and placarded inoperative

91.213(a)(3)(i), 91.213(a)(3)(ii)

Pilot must determine that the item does not constitute a hazard under the conditions of the flight.  Pilot may perform work that comes under preventive maintenance.  Item: Under the most strict interpretation of the FARs, it is almost impossible to fly a legal, airworthy aircraft.

FAR 91.213 is distinct from FAR 91.205 which just lists what is required for specific operations.  91.213 says you can’t fly with inoperative instruments or equipment.  Without an MEL, the alternative is in subsection (d) which requires a placard, plus deactivation or removal.  Subsection (d) places total responsibility on the pilot.

1. The pilot must determine hazard potential, if any.

2. Determination may be made by certified mechanics.

3. Owner/operator must confirm if required for selected flight 				operation.

4. Refer to POH for selected flight operation.

5. If not required, deactivate and placard or remove by certified

maintenance person who must make required placard and logbook 			entries.

6. Inoperative placard must be replaced at each required inspection.

# Repairs

The main difference between a 100-hour inspection and annual inspection is that the airframe and powerplant mechanic can sign off the 100 hour but the annual sign-off requires inspection authorization.  While an in-flight operational check may not be required it is very worthwhile.  The logbooks should be signed by the pilot making the flight.

Minor

FAR 43.9(a) compliance

Work done, date completed, certificate type/number, signature.

TT  (total time) not required

FAR 43.9 requires that the person performing preventive maintenance must record the work in the maintenance records.  The record must contain a description of the work, the date of the work, the name of the person doing the work, and approval or disapproval the aircraft for return to service.

Major

Return to service by FAR 65.95

Repair stations

FAR 145 for propellers, instruments and avionics

Discrepancies

Need not be listed in logbook but separate discrepancy list must be signed off

# Airworthiness Directives (ADs)

FAR 91.403(a) requires that an AD compliance record be kept. Some AD are one-time and others recurring.  They notify aircraft owners and others of unsafe conditions and what must be done to operate the aircraft.  AD’s may be of emergency which requires immediate compliance or less urgent nature with some time slack allowed.  AD’s are FAR’s and must be complied with unless exempted.  There is no ‘overfly allowance as may exist with inspections.  FAR 91.417 requires a record be maintained showing status of all AD’s applicable to an aircraft.  This often is on a separate sheet.

# Service Difficulty Reports (SDRs)

SDRs are mechanics’ report on a maintenance problems.  These are collected by the FAA and aircraft manufacturer and passed on to maintenance facilities as ADs (Airworthiness Directive) or as a maintenance suggestion. SDRs make up about 10% of all reported problems.

The most common items reported are those under the greatest stress.  Exhaust valves, crankshaft weights, and valve train problems are usual.  Cylinder reports relate to cracks at exhaust port area.  Magneto problems apply to failed coils.  Certain engine models are subject to specific reports related to inherent weakness such as case cracking in Continentals.  Lycoming has engines with valve wobble problems.  If ever you own an airplane, get an SDR listing for it.

# Service Bulletin

Issued by manufacturer to advise of service problems and solutions.  The solution is not mandatory as with an AD.

# Maintenance Records

Part of every annual or 100 hour inspection is a compression check.  This involves removing the spark plugs, placing a pressure dial in place of the spark plug and watching the pressures change while turning the propeller.  The base pressure is always 80 pounds per square inch in a cylinder.  The legal minimum for an engine is 60 psi.  Readings are usually shown in the log book as 68/80.  When over half of the cylinders read poorly an overhaul is due.

A top overhaul will resurface the cylinders by honing or boring to oversize;

replace the valves, valve seats; and inspect cylinders and install new rings.

A major overhaul does all of the above but reworks the whole engine.  If the

work is done by the factory it is called a remanufactured engine and the

log book begins again at zero just like a new engine.

The owner/operator is responsible for all required maintenance, inspections and logbook entries on an aircraft.  FAR 43 gives entry examples of ‘appropriate” entries.  This includes compliance with the FARs such as FAR 91.403(a), and FAR 91.405.  91.405 requires maintenance sufficient to keep aircraft airworthy.  The owner-operator must retain the log books, be liable  that proper entries are made, and be able to make them available to the FAA. .

All records of maintenance and inspections require that entries say what has been done, the date of completion, signature and certification of the one doing the work and sign-off.  A proper sign-off  applies only to the work done.  An inspection does not make the aircraft airworthy.  An aircraft is airworthy only when work specified in the inspection as being required is completed and signed-off.

FAR 91.213(d)(2) covers inoperative equipment.  This requires that any inoperative instrument or equipment have a signed placard FAR 43.11 as well as logbook entries regarding the action taking.  Approval for return to service is required.  Maintenance is the owner/operator responsibility, airworthiness is the pilot's.  (See MEL above)

FAR 91.7(b) "The pilot in command of a civil aircraft is responsible for determining whether the aircraft is in a condition for safe flight."  If anything untoward happens the FAA gets to second-guess the pilot's decision.  The regulations on maintenance and inspections are in FAR 91.403(a) and  91.405.  91.3 states: "The pilot in command of an aircraft is directly responsible for, and is the final authority as to, the operation of that aircraft.  Again, if anything untoward happens the FAA gets to second-guess the pilot's decision.  The PIC determines airworthiness in the preflight, review of paperwork and checking aircraft maintenance records.  The aircraft must meet and continue to meet it original type design data unless approved changes are made.

# Airworthiness Directive

## (AD)Compliance Record

Aircraft No:                                                                                                Make:                                                                                           Serial Number

Aircraft 			Model:_______________________________________________________________

AD   # Date  Subject    Compliance    Method of   Date  of    Airframe  Component   One-time   Recurring  Next   Authorized and Received                                                      Due          		 Compliance   Compliance Total Time  Total time  in                                                Compliance Signature

Amend                                         Date/Hours                                                            Service at     Service at                                                    Date/Hours Certificate

Number                                              Other                                                                                                                                                        Other         Type & #

Aircraft Record keeping Requirements of Part 91

Regulation	Required 			Required			Recordation	Retention

Records			Entry			Location		Requirement

91.411                                Altimeter and Encoder                Date and maximum altitude                            Maintenance  Log                                     24 months

Signature, #		                    Permanent

91.413                               Transponder tests                         Test, date, signature #		Maintenance log				     24 months

Permanent

91.417(a)(i )  Preventive, alternations                 Work performed, date completed                  Maintenance log	Until repeated,

91.417(b)(i) 100 hr, annuals, required,              Signature, certificate # of person                   Component log                    suspended,

or approved inspections              returning aircraft to service                                                          or one year

91.417(a)(i)     Total time in service            Identification /serial number                                    Maintenance log               Indefinite

91.417(b)(i)     airframe, engines,                Total time since new/overhauled                                Component log

propeller                                                                                 				.

91.417(a)(2)(ii)                 Status of life-limited parts          Identification /serial number          Maintenance log               Indefinite

91.417(b)(2)                      airframe, engine, propeller,         Total time since new/overhauled   Permanent record

or appliance

iii	Time since last overhaul            Identification/serial number	Maintenance log	        Indefinite

as required on time basis           Total time  of service since                  Permanent record

last overhaul						.

iv	Inspection status, time                 Type/time of last inspection                        Maintenance log	         Indefinite

since last                                              defects found, items deferred               Permanent record ____________

v	Status of ADs, time/ date          DA number, revision date,                      Maintenance log                  Indefinite

when next required                            date of compliance, method             AD Compliance record

and time/date of next action                  Permanent record

Signature/ Certificate #		     		.

vi              Form copies  prescribed by             Specified on Form 337	     Form 337 attached                               Indefinite

FAR 43.9(a)  for major

alternation									.

## Pilot/Owner Maintenance

An aircraft will have a long life only if it is operated and maintained according to the manufacturer’s recommendations.  Some service areas require more frequent attention.  Typical of these are:

Wing spar capstrips and attachments

Fuselage carry through structure for landing gear, wing and tail

Tail spars and attachments

Hinge points and attachments for all control and lifting surfaces

Control balance weight attachment

Cabin structural and seat security.

Engine mounts, nose gear and propeller

Keeping an aircraft clean greatly improves both its appearance and its performance.  When you clean an aircraft you learn a great deal about its construction and maintenance.  You become aware of small defects and maintenance problems.  Cleaning protects the thin layer of paint from the corrosion potential of dirt and airborne chemicals.  Detergent and water are the best aircraft cleaners.  If heavy grease must be removed be sure to obtain something other than a household cleaner should be used.  Specifically, get a degreaser designed only for aircraft use.  Pressure washers remove needed lubrication deep inside ball joints.

Only aircraft type cleaners should be used on plastics and Plexiglas.  Using a dry cloth on Plexiglas will cause an electrostatic charge to develop what will attract dust.  Washing is the best cleaning method for Plexiglas.

Oils and solvents from the ground or in the air damage natural rubber more than synthetic.  Light, especially sunlight will affect the durability and elasticity of rubber.  Keep your tires both clean and out of direct sunlight.  Keep all rubber surfaces free of hydrocarbons.

Frequent use of a specific aircraft gives the pilot an opportunity to determine what normal operation feels and sounds like.  Knowing this makes early detection of abnormal conditions more easily detected.   Every part of an aircraft has a finite life, the way the plane is treated has a direct relationship to maintenance costs.   Training flights with their multiple takeoffs and landings are significantly harder on aircraft than normal usage.

WD-40

The use of WD-40 is not recommended on aircraft is not recommended because only a light lubricant is left after the solvents evaporate.  Use Kroil or ACF-50 available through aircraft parts houses.

The use of steam cleaning and pressure washers on aircraft is not good practice since lubrication points are not sealed.  Open bushes, pivot points must be dismantled to grease.  Pressure washes flush out the oil and will wet the interior making it subject to corrosion.

## Most dangerous thing in aviation:

"A pilot with a tool box." or "A politician with a good idea."   FAR 43.7(f) says that a Part 61 pilot may do preventative maintenance on an aircraft used in Part 91 operations provided it does not involve complex assembly work.  The pilot can approve for return to service after work under FAR 43.3(g)  29 items are listed.  The complete index is in FAR Part 43, Appendix A.  Preventative maintenance on one aircraft may not be allowable on another.

The pilot/owner can do the following only if no complex maintenance is required and authorize a return to service only for what they have actually done themselves .  Maintenance must be done in ways acceptable to the FAA that returns the aircraft to at least equal to original condition.

- Replace and service batteries (Not ELT)

- Replenish hydraulic fluid in reservoir

- Replace, clean and space spark plugs

- Remove, install and repair tires and wheels

- Replace defective safety wiring and cotter keys

- Trouble shoot and repair landing light circuits

- Replace bulbs, reflectors and lenses in lights

- Service landing gear by adding oil, grease or air

- Replace safety belts

- Decorate

- Make simple nonstructural repairs

- Replace side windows

- Replace seats and seat parts

- Replace all hoses except hydraulic.

- Install anti-misfueling devices

- Remove and replace plug-in Nav/Com radios

- Remove and replace plug-in software

FAR 91.7 makes the pilot responsible for the airworthiness of an aircraft to be flown.  Assumption of this responsibility carries with it responsibility to become and remain knowledgeable about the aircraft and its systems.  The owner/operator is responsible for the actual financing of the maintenance.  A certified mechanic must inspect any work done and make required entries in the records of the aircraft.  FAR 43.3(d) requires that the mechanic personally observe the work and be available while the job is in progress.  However, the FAA gets to second guess all actions by these parties if something occurs.

Logbook entries must describe the work done, the date of completion, name of

worker, a signature, and certificate number.  This signoff is approval for return to service only for work done by the signer.

The best care you can show toward your engine is to use it along with some good pilot procedures.  A sitting engine will rust and corrode.  Avoid descents with reduced power which may cause shock cooling; high rpm starts which fail to provide required initial lubrication; and, excessive leaning at high rpm which will burn exhaust valves.

# Maintenance Shakedown

The pilot who first flies an aircraft after maintenance actually returns it to service.  A logbook entry under 91.407(b) is only required if flight performance or operation is changed.  You are a test pilot any time you fly after even the most minor aircraft maintenance has taken place.  Don't fly if you do not have the experience and qualifications for the flight.  Just because you are single engine rated does not mean that you are safe to fly an unfamiliar plane.  Find out what maintenance was done and by whom.  Talk to the mechanic so as to separate the flight critical from the cosmetic.

Human error in maintenance is not restricted to the owner/ maintenance.  Accident statistics frequently show that maintenance

is a factor due to:

- Improper component installation

- Use of wrong  or illegal parts

- Improper electrical wiring

- Tools left in aircraft

- Inadequate lubrication

- Aircraft panels improperly secured

- Fuel/oil caps improperly secured

- Gear/control bolts improperly inserted or secured

Maintenance life extends far beyond the end of the job.  Work properly done will have a life of thousands of flight hours and years of time.  Work improperly done on an aircraft is like flying with a time bomb.  Maintenance that deviates from the expected norm or procedures is an accident waiting to happen.   Insignificant problems can have serious consequences.  Aircraft systems are designed to prevent minor maintenance mistakes.  However, human error exists as an unavoidable reality.  Beyond human error lies problems in the organization.  Stress exists in organizations as well and individuals.  Communication reduces stress.

Considerations might be:

- Are the rules there, or there to be evaded?

- Does money make short-cuts easier to justify?

- When a problem occurs, what is done about it?

- Is there a complaint format for all concerned?

- How are mistakes handled?

Engine work is flight critical.  You can determine internal maintenance only by consulting on what was wrong, what was done and comparison with the maintenance manual.  Engine maintenance can be checked by going over all wire and tube connections to make sure that they are tight.  Check all clamps and welds.  Use an inspection mirror if you can.  Make sure all bolts and nuts are secure.  Check engine control linkage for security and wear.  An engine that runs rough at start or has intermittent roughness  may have sticking valves.  Properly operating engines will have exhaust stacks that are powder gray or tan.  Soot deposits show rich operation or excessive oil contamination, fouled plugs, pre-ignition and detonation.  If fuel or hydraulic lines have been repaired it might be well to runup the engine to check the lines.  If work has been done on the aircraft interior be sure to check and wiring, catches or latches.  Preflight all the controls, hinges, lock nuts, cotter pins, safety wire and cable tension.  Shake the plane by the wingtip to see if anything rattles or comes off.

Review the POH on the preflight, emergency procedures and operational limits.  Let ATC or others at the airport know where you will be going and when to expect you back.  Do a careful runup, perhaps with someone outside to keep an eye on things out of the pilot's view.  Plan all your "what ifs"

and depart gaining altitude so as to get back if necessary.	 Accident files show that all too many accidents occur after some kind of maintenance has taken place.  A more careful preflight should be made after all maintenance.   The less an plane flies the more careful should be the preflight inspection.  Become familiar with the papers and logbooks of any aircraft you fly.  As a pilot you should become familiar with the aircraft manual for every plane you fly.  You should first go through it quickly to get an overview of what it contains.  Next you should study it section by section and become familiar with pertinent information and know how to find any that may be required. for a cross country or for a checkride.

# Things that go wrong

A pilot during training should be exposed to recognizing and understanding

the danger signs warning of both eminent and impending operational problems.

There are many visual signs such as smoked rivets, cracks and leaks.  Non-structural cracks can be stop-holed to prevent further growth.  The hole relieves stress.  Leaks can be normal or a problem.  Propeller and hydraulic leaks are serious and should be brought to the attention of a mechanic.

There are tactile and auditory signs from moving the controls, and structure.

Learn to listen for changes in engine, propeller and wind sounds.  The way an

aircraft vibrates is a prime indicator of its health.  any unusual vibration is sufficient warning to land as soon as possible.

Every aircraft has its own smell.  Electrical smells from insulation are distinctive.  Any smell that persists is a warning that a fuse or breaker

has failed or been bypassed.  Turn off the master switch.  You can isolate the problem by turning on one breaker or fuse at a time until you locate the problem.

# Question

1. Your ELT has been removed for repair.  Will a local training  flight be 	allowed if it remains within 50 nm?  Is a pleasure flight allowed?

2. What paper work is required if an aircraft is flown without an ELT?

3. There is an AD (Airworthiness Directive) regarding work due on your 	aircraft.  What is required prior to any flight?

4. Who is responsible for maintaining an aircraft’s airworthiness?

5. What substitute can be used for an FAA approved flight manual?

6. During the preflight you notice that a piece of engine baffle is missing.  	What is the requirement?

7. When can an engine be given 0-time in a logbook.

8. May an A and P alter an aircraft?

Answer

1. Even a local flight cannot be made without an ELT unless it is for

training. There are regulatory exceptions.

2. An aircraft logbook entry must be made by the technician removing the ELT 	with information as to date, serial number, type, and reason for 	removal.  A placard notice must be placed in sight of the pilot about

the ELT being removed.

3. A logbook entry must give the current status of the AD, method of 	compliance, the AD number and revision date.  If the AD is a recurring 	one 	the time and date when the next action is required.  No flight is

allowed in an aircraft with an outstanding AD.

4. The owner and operator is responsible for aircraft airworthiness.

5. There is no substitute for the FAA approved flight manual.  It must

be aboard the aircraft any time it is airborne.

6. Aircraft cannot be flown until in conformance with type design.  Baffling 	is part of the aircraft design.

7. Only the manufacturer can designate an engine as 0-time.

8. Only an A and P with an Inspection license can alter an aircraft.

Question

1. What paperwork is needed when preventive maintenance is done?

2. State VFR fuel requirements for daytime flight.

3. When can you fly in formation?

4. When can something be dropped from an aircraft?

5. when must a pilot have a current pilot certificate and medical

in his possession?

6. What is the minimum safe altitude over congested areas?

7. When may a certified pilot fly as PIC while towing a glider?

8. What is the time lapse required in order to pilot an aircraft after consumption of alcohol?

9. When can a pilot act as PIC of a high performance aircraft?

10. What is required by the FAA when you change living places?

Answer

1. The one doing or approving the work must enter into the aircraft

logbooks and maintenance records his signature, certificate number, kind of

certificate and a description of the work done.

2. You must have enough fuel for daytime flight to fly to initial destination and then for another 30 minutes.

3. Formation flight can be flown only by prior arrangement with the pilot in command.

4. No object can be dropped from an aircraft if care is taken to avoid injury or damage to persons or property.

5. a pilot who is acting as pilot in command must have a current pilot certificate and medical in his possession.

6. Except for takeoff and landing the congested area clearance required is 1000’ above and 2000’ laterally from the highest obstacle.

7. To act as PIC while towing a glider the pilot must have 100 hours in powered aircraft.

8. A pilot may not fly within eight hours of consuming alcohol.

9. To act as PIC in an aircraft of over 200 horsepower, or retractable gear,

flaps, and a controllable propeller the pilot must have flight instruction in such an aircraft and a logbook sign-off by a qualified FAA agent.

10.  The FAA must be notified within 30 days of any permanent change in address.