Hoodã

Hood

Standard Rate Turns

Timed Turns

Unusual Attitudes

VFR to IFR Options

You and Illusions

IFR Flying Faults

Vertigo

Illusions:

Natural Illusions

Night Illusions

Landing Illusions

Black Hole Landing

Aircraft Illusions

Scan (Instructor)

Hood Lessons (Instructor)

Vertigo simulation (Instructor)

Practice Illusions (Instructor)

# Hood

The hood should be fitted prior to engine start.  Note that the attitude indicator has both a wings level and a level index mark at its top.  The 10, 20, and 30 degree bank marks have been pointed out during Dutch-rolls and banks.  A plane will not turn if the wings are level.  This means that whenever the heading indicator is fixed on a heading the plane is not turning and wings can be considered level.  Knowing this will enable you to perform even stalls under the hood.  When the attitude indicator is at level, heading will be constant.  Confirm this with the heading indicator.

The turn coordinator has the ability to show both the direction of the turn and markings that index the 2 minute standard rate turn. The vertical speed indicator will either indicate false or excessive movement under abrupt control input and may have up to a 12 second lag.  The altimeter also may lag.  The accuracy of the instruments is proportional to the smoothness of any change.  The use of sound indexes acquired from VFR flight will help clue you in on airspeeds and changes.

# Standard Rate Turns

All banks under the hood are at standard rate.  Standard rate turns requires two minutes to make a 360 degree circle.  One minute for a 180 and 30 seconds for each 90 degrees.  The standard rate turn for a given airspeed is indicated by the needle or by the turn coordinator. The turn coordinator is the only true indicator of whether the bank is left or right.  (A necessary check during spins)  The angle can be estimated by dividing the speed by ten and adding five. (Speed /10 + 5 = angle of bank)  85 knots airspeed divided by 10 is 8.5 + 5 = 13.5 angle of bank.  Set the estimated angle on the AI and check with the turn coordinator. All climbs and descents are at one bar width on the attitude indicator.  From low cruise both climb and descent at a given rate will closely correspond to RPM changes by 100's.  A power reduction of 500 rpm gives 500 fpm descent.

# Timed Turns

In VFR condition at level altitude cruise the standard rate bank is selected by using the turn coordinator.  The degree of bank is NOTED on the attitude indicator.  This angle of bank becomes the standard to be used.  It is checked for accuracy by making successive timed turns of 90, 180, and 360 degrees.  It takes 30 seconds to turn 90 degrees, one minute to turn 180 degrees and two minutes to turn 360 degrees. In a C-150 at cruise the angle will be about 12 degrees.  After timed turns are introduced at cruise go to VFR slow flight and determine the angle of bank required on the turn coordinator for above timed turns.  The angle will be close to 10 degrees.

# Unusual Attitudes

During the proficiency phase of hood work 'unusual attitudes' should  be practiced.  This means that you will put on the hood, close his eyes, lower or raise his head while the instructor flies.  The instructor will make a series of slow and rapid flight movements designed to disrupt the flow of fluids in the inner ear.  After such a series the student will be told to recover the aircraft through use of the instruments.  When visual cues are missing the inner ear and kinesthetic systems kick in.  When vision is re-admitted confusion occurs.  Panic often ensues when an unusual attitude occurs without warning.  The unusual attitude instruction  must emphasize faith in and use of the instruments, especially the attitude indicator.

If the instructors efforts succeed, the recovery will be difficult for the student due to sensations of vertigo.  Vertigo is a physical condition where the victim suffers symptoms such as spatial disorientation, inability to perform and intellectual confusion.  The instinct to trust our senses causes these problems.  This instinctive trust must be transferred to reliance on the aircraft instruments through training.  Until this transfer is completely accomplished safe IFR flight is not possible.  The ability to recognize and recover from a rudder caused unusual attitude is a good skill best acquired through aerobatic instruction.

The recovery from unusual attitudes proceeds through several steps depending on the aircraft attitude and airspeed.  If the aircraft/engine noise is high, reduce the power, correct any bank, raise and level the wings on the AI horizon.  If the aircraft/engine noise is low, apply full power, correct any bank, lower and level the wings on the AI horizon.  Be aware that the instructor may have applied flaps, trim, or covered instruments to make the recovery more difficult.

Knowing how to correctly recover from non-spinning unusual attitudes is important.  More aircraft and pilots are lost because they try to pull the yoke to recover a nose-down, semi-inverted unusual attitude than from spins.  Use ailerons to roll upright.  A non-aerobatic pilot must avoid making the initial reaction which is pulling back on the yoke.  More often than not this makes the situation worse.  This is specifically true when in a steep bank or inverted.  In a spiral, pull the power off, coordinate aileron and rudder to level wings and recover with less than 2 Gs.  If wake turbulence puts you into a steep bank or turns you upside down you must first reduce power, move the yoke forward and roll the ailerons to get upright.  Any other procedure will over-stress the aircraft.

Unusual attitudes are unintentionally entered by pilots who overestimate

their ability and competence while underestimating the weather conditions.

Begin by being caught on top of a cloud layer.  Follow by a sense of being

lost because you no longer have the same sensory aids you are used to having.  You can’t chart progress except electronically.  Instinctively the pilot will turn and fly to where he wants to be, not necessarily where he wants to go.

A private pilot with the minimum required hood time will maintain control about seven seconds on entering IFR conditions.  This can be prevented only by getting on the instruments and executing a 180° level turn.  Failure to act or to act improperly on entering IFR conditions is very likely to destroy the aircraft in the air.  Slow up on entering turbulence, keep a light touch on the controls and keep the aircraft level, freely allow altitude changes, and don’t attempt any turns.

Below clouds things appear differently, augmented by a desire to keep the surface in sight, the pilot will fly under and around visual obstructions in an ongoing belief that things will get better.  They may, but I you haven’t checked the weather ahead, they probably won’t.  You must know which way the weather is moving in relation to your flight direction.  It forecasts and winds say the weather is better from whence you came, turn around even though the illusions of improvement ahead exist.  Among the disadvantages of flying below the clouds is the instinctive loss of altitude any time you need to improve visiblity.  Visibility may improve but you are in the process of destroying your ability to communicate.  Now a situation that could have been resolved with ATC help is running out of options. As a student this should be a ‘hood’ exercise with your instructor

Add a dose of air contaminated fuel tanks and an approaching need to descend even further and enter clouds and we need to have pre-planned a few years ago what would be the most appropriate thing to do.  Without training and planning stress will so focus your attention that your recovery will be in doubt.  Your arrival on the ground is a certainty.  Getting there safely is dependent you how well you exercise mental and aircraft control.  We should have worked out these options while a student long before reaching this crisis.  Aircraft control is the essential element.

# VFR to IFR Options

What to do?  If you’re on top, do the four C’s.  Climb, communicate, confess, comply.  If fuel and weather limits your ability to find a VFR

escape route.  Don’t delay, find an area in which to descend to VFR below.

ATC vectors may get you to the area but you must know how to make a safe

decent. As a student the maneuvers below  should be a ‘hood’ exercise with your instructor.  Some practice now may solve a future problem by having

a pre-planned solution.  Try it until you can do it.  When the need for

performance occurs you can advise ATC of your ability to perform an IFR

climb or descent in a declared emergency.

One way follows.  There are other ways.  Pay our money and take your

choice.  Assume that all instruments are functional.  Get on a cardinal

heading, C.H. power to 2000 rpm, and trim for 60 knots.   Let go of the

yoke and hold heading with rudder.  Slowly decrease power to 1500 rpm and

descend at 60 knots using only rudder.  If the terrain below is completely

unknown as to cloud clearance and terrain elevation.  Do the process with

full flaps and airspeeds as low as 45 knots.  The worse thing you can do is

a spiraling descent.

If you are below a cloud deck with a known top attainable by your

aircraft and fuel remaining, initiate a VFR to IFR to VFR climb using much

the same technique as the descent.  Well below the cloud base, initiate a

65 knot climb and trim for hands off.  Let go of the yoke and hold a cardinal heading with only the rudder.  Once on top you should know if an escape VFR airport is within range.

Hopefully, you will never need an emergency option.  You should practice recognition of weather so as to initiate the 180 escape before it cant be done.  When the turn is only possible in IFR conditions you must know enough to totally rely on your instruments.  Any effort to mix IFR with VFR is doomed to failure.  Use the turn coordinator to limit the bank to a standard rate.  Better yet learn the amount of bank on the HI that gives the standard rate for your airspeed.

# You and Illusions

1) Illusions can and do occur to everyone

2) Proper planning for strange airport conditions is essential

3) Weather and poor visual conditions make airport conditions 	worse.

4) Rely on your instruments to help your sense perceptions

5) Maintain situational awareness, know where you are.

6) Use airport systems and lights for directional and slope help.

7) Your physical condition can make matters better or worse.

# IFR Flying Faults

Initially, VFR to IFR flight results in over-controlling especially when making corrections.  Non-instrument rated pilots who fly into instrument weather lose control in about a minute of level flight.  The usual VFR movements for climbs, descents, and turns are far too extreme for IFR flight.  One technique used to avoid over controlling on heading changes is to use the AI to set the standard rate of turn.  Count three, five, or six at a steady rate.  Level out from the bank using the attitude indicator top index to level the wings and note the degree of change in the HI.  Do this several times until you can get the count needed to get 10 degrees correction every time.  Focus on the attitude indicator top index when leveling off.  This technique prevents the common fault of reversing the bank back to nearly the original heading.  This reversal is caused by the beginners inability to ignore the turning sense imparted by the fluids of the inner ear.  This problem is normal for the beginner but focusing the eyes on the top index of the AI for a few seconds will give the inner ear fluids time to lose their influence.

# Vertigo

Beware of false sensations.  Your inner ear will give you feelings that

are overpowering.  With low time under the hood you must avoid attempting to extend maneuvers beyond just holding recognize that these a heading or a

standard rate bank.  Any more may exceed your skill capacity for aircraft control in IFR conditions.  Any tenseness will cause the inexperienced to

over-control.  If you have learned to use trim well as a VFR student you may be able to trim successfully for hands-off IFR flight.  An aircraft trimmed for hands-off in pitch can be flown in roll (heading) with just rudder input.  Descents are controlled by slow power reductions, only.

May cause nystagmus (trembling of the eyes) which makes reading of instruments impossible.  Rare but can occur in extremes of weather or flight conditions.  Other types of disorientation are illusions such as caused by runway/cloud sizes, shapes, or slope.

Unrecognized spatial disorientation is caused by a combination of a focused  attention, distraction and instrument fixation.  These most often occur in companion  with loss of situational awareness due to excessive work load.

Recognized spatial disorientation is when the pilot is aware of his disorientation.  Being aware means that the pilot should be able make his

recovery with power and attitude corrections.  Pilots have, over the radio, acknowledged their vertigo and inability to overcome it prior to crashing.

Spatial disorientation is considered incapacitating when the attitude of the aircraft is so unusual that pilot is unable to determine the cause, organizing the instrument information, and making control decisions.

Vertigo is the #1 cause of Air Force fatal accidents.  Vision is the pre-installed vertigo preventative.  A moments glance out-the-window is all it takes.  This will overcome any sensations from other sources.  However, without vision, the organs of balance in the inner ear take over.  The semicircular canals approximate the three axes.   They contain a fluid that stimulates our senses of angular acceleration in these axes.  Our sense of uprightness is done by the otolith organs.  Tiny stones affect hair sensors in reaction to “gravity”.  Otoliths sense linear accelerations, not angular accelerations, and regardless of the direction interpret such accelerations as gravity.  In our muscles and joints we have sensors that give additional information about push or pull.  Unless one or all of these sensors are confirmed by vision we are on our way to vertigo.  The simulation used for vertigo such as the Barany chair, the yardstick on the nose, etc. in no way prepare the pilot for the real occurrence of vertigo.  Unlimited motion in all three axes as well as planetary motion for acceleration are required.  Even with these in place it is additionally necessary to create a work overload. The instructor should expose the student to simulations that closely approximate the real thing.  That is except for safety of altitude, configuration and airspeed.  To train the recovery the same forces and psychological pressures must be recreated over and over by inflight simulations.

# Illusions:

What ever the illusion, it will take a few moments for you to get reoriented.  If the illusion occurs in trimmed flight the problem is not as severe as it might be if it occurs during maneuvers as it usually does.  Spatial disorientation caused by loss of horizon, change in power, banking, or other acceleration forces can lead the pilot to believe that the aircraft is doing something that it is not.  These forces cause the pilot’s vestibular and proprioceptive system to give the somatogravic illusion.

A pilots first reaction will be to over-control in a direction exactly the

opposite to what should be done.  The only safe procedure during the onset of any illusion is to increase your instrument scan rate.

The brain is capable of separating out the conflict of information between the eye and the inner ear when the eyes are looking outside the cockpit.  A

conflict between the sensory inputs of the body can be overwhelmed when the

natural horizon disappears. When the conflict of information comes with the

eyes viewing relatively unfamiliar instruments inside the cockpit, the brain can become confused and then give improper information.  This is the origin of an illusion.  You must trust your instruments in IFR conditions.  (See IFR material)

# Natural Illusions

Rain

Rain gives the illusion of being higher than you actually are.  Combined with haze, you will fly lower approaches during rain.

Visibility

Low visibility can cause the illusion that things are further away.  Entering a fog during descent will give an illusion of pitching up.

Wind

An updraft can cause such vertical sensation as to cause the pilot to put the nose of the aircraft down.  He senses a climb where the attitude of the

aircraft has been level. Avoid the weather conditions that can cause illusions.  Believe your instruments.  Get an instrument rating.

Health

Illness, medicine,  alcohol, fatigue, or hypoxia will make susceptibility to an illusion more likely.  It takes very little to cause disorientation.

# Night Illusions

Runway and approach light illusions will always be a problem if you are in an unfamiliar area.  For this reason it is always desirable to make a daylight familiarization flight to an airport before a first time night arrival.  Night has its own illusions that are covered in the night flight lessons.  The distance of lights is greatly affected by the relative clearness and haze existing.  A region of no lights such as might exist off the end of a runway toward the ocean can cause disorientation because of IFR illusions.  The best solution is to go on instruments until established inland at altitude.

Variable Visibility

If, while on approach, you should suddenly face reduced visibility you will get an illusion of a sudden pitch up in  aircraft attitude.  Failing to recognize this illusion will lead to an instinctive and abrupt descent in the approach flight path.

Your ability to determine distance is greatly affected by haze.  It is not unusual to call a distance at over twice the actual distance.  The sudden appearance of bright lights during an a night approach through haze  will create the illusion that the airport is much closer than previously realized.  This results in a high approach.

Low

If there is no VASI or VAPI for vertical guidance if you get too low the runway lights will begin to disappear.  A steep approach is always better at night. An arrival at an airport with and approach lighting system (ALS) tends to be lower and at a shallower angle than otherwise. Low approach results when runway is approached over dark area.  Don’t use landing light until close to ground.  The visual cues used for a normal night landing seem much the same as you get with a rapid increase in sink rate.

Inverted

If there is a strong crosswind and you are crabbing to the runway instead of slipping you will get the illusion of being inverted.  If the airport is well lighted in a surrounding dark area you will have an illusion of being higher than you actually are.  Again a steep approach has much advantage.

High

Rain on the windshield will give the illusion of being higher than you are.

If you mistake roadways for runways.  It will give illusion of being quite close when lights are bright.  High approach results.

Rising

If you are low and pitch the nose up as a correction or through the use of flaps, the illusion will indicate that you are rising.  Any reduction of power will cause you to land short.

Diving

At night, banking into or away from a line of lights will give the illusion

that a dive or a climb is occurring.  The same dive or climb illusion can

happen by a change in aircraft pitch occurs while flying toward a light.

A 10 degree bank with only the approach lights visible can cause an illusion that the lights are sloping from above.

Distance

Lights that appear dim, as seen through haze, will be reported as more distant than they are.  Example: I once reported an airport as in sight from twelve miles when the tower had me on radar at only five miles.

# Landing Illusions

You and every other pilot is susceptible to illusions.  When any of our three flight senses (visual, auditory and kinesthetic) give erroneous information our mind produces inaccurate information.  Flying illusions are most likely to occur at airports due to visual information.

Illusions do not cause accidents.  Rather, it is a pilots reaction to the illusion that precipitates an accident.  The best way to overcome the negative effects of airport illusions is with experience.  Even the most experienced pilot can be fooled.

Our three flying senses fool us by misinterpreting distances, velocity and

relationship.  Vision is the number one creator of illusion.  Over our life

time of living and flying we have learned that if two things are of the same size and one appears larger it is nearer.  If we know two lines are parallel then when they appear farther apart that is the near end.  As we fly we learn an approach slope for our aircraft at our home airport.  Our brain has developed a data bank of how things are supposed to be at airports.  The terrain around the airport can also be an illusion creating factor.  Use all the electronic and visual (VASI) help you can get.

Width

You should practice landings at airports of varying width.  All too often you become overly familiar with airports of a consistent length and width.  This familiarity interferes with your perception of an airport of different

dimension.

A narrow runway can create the illusion of a high approach.  The AIM Chapter 8-1-5 states that the pilot who does not recognize this illusion will adjust his perceived approach path to fly lower.  Once the pilot realizes that he is low it is vital that full power be applied while maintaining approach speed with yoke pressure.  No trim changes.  Once the proper glide path has been intercepted the power can again be reduced and the stabilized approach resumed.

The illusion associated with a wide runway causes the risk of flaring rather high above the runway with a hard ground contact to follow.

Visibility

The atmosphere can create illusions.  Clear clean air makes everything seem

closer.  Hazy or smog conditions makes things hard to see and apparently

further away.  Note: Be aware of this when reporting distances at strange

airports.  The effect of sunlight in creating shadows is an important part

of our visual data bank.  At night these shadows are not there.  The absence of contrast and background at night is a major cause of night landing and takeoff accidents.  Precipitation will distort visibility through the windshield and make to think you are higher than you actually are.

Speed

Our vision is the major source of our cues related to speed.  However, much of our speed information comes from our peripheral vision and is often not

consciously entered into the brain.  The unfamiliar tangential velocity of the ground passing by in a low level bank  may appear so fast as to make us pull back on the yoke without regard to the air speed.  Stall-spin.  This is the classic sequence of the downwind turn from base to final.  We have learned to judge both the proximity and speed of an known object crossing in front of us.  Closer things seem to move faster.

Expectations

We are all subject to illusion because we have come to expect certain

appearances to occur.  When we are told to look for traffic, we expect to see that traffic.  If we don't see the traffic tension rises; if we see any plane we have our expectation satisfied and we stop looking.  We shouldn't.  The traffic we see does not have to be the same traffic we should be looking for.

Slope

Gives identical illusion as wide runway but is much more likely to cause the pilot to over-shoot the runway.  A runway sloping away will give the illusion that you’re low on the approach.  A runway that slopes toward you can give the illusion of height.  One of the most difficult runways is the one with a mound in the middle.  This gives the worst of both illusions.

In landings where you project the possibility of illusions, it is vital

that you proceed with the landing using a standard procedure to establish

a stabilized approach.  The making of predictable changes of power, flaps,

and trim becomes very important as the landing progresses.  ‘Winging’ it

for changes in these approach elements means that you are subjecting yourself to the effects of illusion.  A pilot who fails to recognize illusion is going to make changes as though what he sees is reality.

Any airport that differs from our experience because of runway dimension shape or slope is going to give us an illusion.  If a runway slopes away from us our perception (illusion) will be that we are too low.

If the runway slopes toward us we will perceive ourselves as being too high.  If we react to the illusion we will find ourselves both high and long from the anticipated touchdown point.  A wide runway or one whose lights make it appear wider will give the illusion of a low approach causing the pilot to stay too high.

A 3 degree approach to an upslope runway and the illusion is that you are high.  The same upslope runway made narrower and you will find yourself low.  A down-slope runway creates a "low" illusion which can cause long landings on short runways.  I have found the best way to counter airport illusions is to have a constant pattern procedure based on a stabilized airspeed and full flaps.  I know that even in the worst conditions of illusion I will be close on final.

Visual deception

Pilots unconsciously make extensive use of there peripheral vision.  Level

flight, banks, climbs, and descents rely 80% on peripheral vision.  (See

downwind turns)  At low levels our peripheral vision gives us a sense of speed.  Over time we develop a peripheral sense as to what "normal" low level speeds are.  Add a tailwind, low altitude, a bank to final approach, and a peripheral sense of a "high" speed.  We now have an illusion causing a pilot response that says to pull back on the yoke to reduce the speed.  The pull merely makes the bank steeper and initiates a low level stall spin.  Recovery not possible.

A final approach over high terrain leading to the runway gives an illusion of a low fast approach.  An approach over terrain that makes the runway seem like an aircraft carrier will give an illusion of too high and too slow.

# Black Hole Landing

When it is very dark you are subject to illusions.  Your seat position may make you more or less subject to these.  Aircraft seats are situated by a "Design Eye reference Point" (DERP) that is supposed to give a view over the cowl and at the instrument panel.  It is not marked on the aircraft but is part of the aircraft design.

If your seat is too low during dark conditions, you will only be able to see runway lights if you are in a descent.  The pilot tends to set up a constant angle for their field of view on approach.   During daylight, you use the "point on the windshield" to maintain this angle.  If the aim point drifts upward you are in an excessive descent; if it drifts downward you are too high.  It works the same at night but the references are fewer.

The black hole illusion begins out a few miles on final.  The field and lights will be foreshortened.  Getting closer the runway should rise in the field of view.  If the aircraft is descending the foreshortened view will remain constant.  The eye/brain interpretation of this is that the constant foreshortened runway is an indication of a constant angle approach.  This is the "everything is fine" illusion of the black hole.  During the final phases the daytime depth perception does not work very well.

When flying into a black hole  or ‘featureless terrain’ you must use a correctly set altimeter to counter the illusion.  Fly a full pattern using your altimeter and a standardized procedure for a stabilized approach.  This is the best way to assure yourself that you will not meet an obstacle on final. This is a combined IFR/VFR approach and requires that you be capable of controlling the aircraft accurately without visual references.  The normal order of viewing runway, lights, and dark areas is so changed illusions of being higher occur.  If the illusions are believed, touchdown can occur before reaching the runway.

The “black hole illusion” has a dramatic effect on straight in approaches to a runway.  The stabilized constant angle approach will appear to exist as an illusion while you fly an arc that flies you into the ground.  Do not fly a straight in approach to a runway at night.  Know the pattern altitude.  Fly somewhat farther out on down wind than appears appropriate.  Use a standardized configuration approach just as you would in daytime.  Fly the VASI or VAPI if available.

Failure to follow the above recommendations may cause you to make a premature descent.  Over flying featureless terrain without lights on an approach that deprives you of the height clues can fly you into the ground.

# Aircraft Illusions

There are some optical illusions that relate to nearby aircraft.  An aircraft below you will appear to be above you.  While getting closer it will appear to descend through your horizon.  All the time it is straight an level below you.  Avoid the temptation to dive.

IFR for VFR Pilots (Instructor)

The purpose of IFR training is to be able to read, interpret the instruments and fly accordingly.  The introduction of the VFR student to the hood should commence with several presumptions.  First the student should be reasonably competent in the four basics VFR.  This means his banks should be performed at 30 degrees and that trim and power changes can be made to set differing speed and flap conditions.  With this foundation the student should be issued some cautions.  His VFR control inputs will have to be reduced to obtain the 3 degree per second turn rate required for IFR.  The student will have to be made aware that the "pop-up" tendency of the plane when leveling off from turns will be less and of necessity require a different pressure and feel.  If VFR instruction has produced a light smooth touch on the controls the addition of a rigid wrist and braced arm will give good IFR control.

The VFR pilot who enters into IFR conditions has an emergency.  First off he should declare that emergency on any frequency he can.  In actual conditions, just reaching for the radio can cause the inexperienced pilot to lose control of the aircraft.

The human eye cannot determine clouds by size or distance.  There are none of the usual references by which we determine size.  Beyond 20 feet the one-eyed person is not handicapped except in terms of peripheral vision.  There is a fascination in flying in proximity to clouds.  Flying toward clouds with the intention of flying over, under, or around can easily lead to flying into them.  You will not be aware of the problem until it is too late.  When a cloud moves you are too close.

We can normally determine our position and direction of motion by use of the eyes, the inner ear, and the seat of our pants which is called the  proprioceptive sense.  We use these in conjunction with the gravitational forces of the earth to know where ‘down’ is when we are on the earth’s surface.  In the air these senses can be deceived and put into conflict.  On water and in the air this can cause motion sickness.  A maneuvering aircraft can create gravitational sensations to our muscles and body.  Our usual feeling of where ‘down’ is becomes confused.

The pilot has no option except to learn to ignore these body senses and

use only the eyes and aircraft instruments to know what is happening.  This ability to ignore and use the eyes is what defines an IFR capable pilot.  A slip or skid gives a sense of tilting in the opposite direction.  A turn will give a sense of increased gravitational force.  This must be visually interpreted as being in a turn through instrument reference and ignored as

being going up as in an elevator.  The VFR pilot will lose altitude going into a turn and gain altitude coming out of a turn.  The body senses want ‘level’ body pressures to remain constant.  This must be ignored and the eyes must be used to enter, hold and leave the turn.  Any increase in body pressures is felt as a climb even though it may be a turn.  Coming out of a turn is felt as a lowering of pressure or a descent even though it is coming out of a turn.  Successful IFR again involves ignoring everything except the eyes interpretation of the aircraft instruments.

# Scan (Instructor)

I suggest a scan process of having the student follow my finger as I point successively from HI to AI, VSI to AI, Altimeter to AI,  etc. with emphasis upon the AI (attitude indicator).  Simulate a change in direction and move the finger faster through the instruments and include the turn coordinator for the standard rate indication.  It is important that the student realize that any change in the aircraft direction, altitude, or airspeed requires an increase in scan speed.  Any fixation on an instrument such as the heading indicator as it progresses in a turn is to be avoided.

# Hood Lessons (Instructor)

Since hood work is emotionally and mentally tiring it is best to limit each session.  Gradually extend the hood time until reaching 20 minute sessions.  Vary the time hood work is done during the lessons.  New skills should be introduced early, while the student is fresh.  Do the new hood work early in the flight for maximum student freshness.  Always review the previous before going on.  Two sessions may be possible if a good sized VFR break is inserted.  On removing the hood it is a good idea to have the student show that he is capable of orienting himself by pointing out landmarks.

Under the hood flying simulated IFR, the student will learn to relate the AI, HI, VSI, turn coordinator, and altimeter to yoke pressures in level flight.  The student should be introduced to doing the four basic procedures under the hood.  Sometime around the fourth or fifth climb out the student should shown the relative position of the attitude indicator and the nose.  He should be placed under the hood and helped maintain the climb attitude, airspeed and heading during airport departures.

Other lessons include  transitioning from cruise into slow flight and back again. Reduce power and do descents VFR and IFR.  IFR hood climbs and climbing turns can be taught and practiced.  It may be best to practice the IFR standard rate turns without the hood and then with the hood.  Level off both VFR and IFR.  These are fundamental IFR skills that can be practiced and transferred to higher performance aircraft.

For the student to maintain straight and level under the hood that aircraft is initially configured for VFR flight.  Then under the hood the student is asked to maintain heading and altitude for one minute.  Do this several times with emphasis upon the lightness of pressures required.  If the arm and wrist are held rigid and the fingers lightly the one minute of constant heading and altitude can be gradually extended to two or more.

Introduce the hood in climb out and have the student note the wing attitude on the attitude indicator.  Have the student attempt left and right 90 degree turns under the hood.  Remove the hood and do the standard rate turns without the hood.  Then back to hood.  Level off under hood and then without hood until the student learns to read the attitude indicator changes as related to aircraft attitude and airspeed.  Do both VFR and IFR standard rate turns to headings up to 360 degree turns with stops at cardinal headings.

# Vertigo simulation (Instructor)

Materials:

1. An open lawn space at least 15' by 15'

2. A yard stick or equivalent light weight stick

Procedure:

1. While standing in middle of open space, have victim hold stick on 		nose.  Hold head as far back as possible and keep eyes focused on 	upper end of stick. Make several (4-5) 360s at a smooth rate.

2. Stop.  Bend down at the hips and while holding the middle of the 		stick with one hand, place (do not drop) stick in front of you.

3. Remain bent over and step across the stick.  Stand up quickly.

Effect:

1. The tilting and turning causes the inner ear canals to receive nerve 	messages in conflict with the normal axis.

2. The changes in position and movements place the inner ear messages in 	conflict to the visual messages.

3. When the exercise is done without pauses, the disrupted sense of 		balance is caused by vertigo.

# Practice Illusions (Instructor)

Autokinesis

On a  dark night, fly toward a single light.  Student is to stare only at the light.  After a while the light will appear to move.  Student should try to keep the light in one place.  Aircraft control will be lost.

Updraft

Have student release yoke and close eyes.  Make slow entry into steep bank.  Student should tell you that you are in a climb.  With eyes closed have student level off.  Climbing sensation should cause student to dive aircraft.  A real downdraft would cause climb reaction on controls.

Bank

A student is asked to fly with eyes closed.  If a bank is entered so slowly

that it does not affect the inner ear, any abrupt correction or abrupt

movement of the head will cause the pilot to re-enter the slight bank.  Since the original bank did not affect the inner ear, the inner ear senses the level condition as being incorrect.  Effort of pilot to correct these will only make them worse.

Spiral

A student is told to tip the head and close the eyes. Have student try to hold aircraft level.  In a few seconds a gradual bank will occur.  Over time, the bank angle will increase. There will be a gradual descent, an increase in airspeed, and an increase in bank angle.  If the student pulls back the inner ear senses turning, spinning, or spiraling in a direction away from the bank.  Ever increasing wrong control inputs will follow.  Effort of pilot to correct will only make things worse.  This is known as the graveyard spiral.

Cover the attitude indicator and have student close eyes and lower head to

one side.  Put aircraft into a shallow constant rate turn for enough time

to allow senses to stabilize (360).  Have student tilt head up quickly while opening eyes. Done properly student will feel tilted to one side

Attempts to correct will be exaggerated in the wrong direction.

Nose up

Have the student enter slow flight eyes closed, without trim , and keep his

eyes closed during acceleration and recovery.  This causes a somatogravic

illusion.  The inner ear will sense a nose-up attitude.  Deceleration reverses the effect.

Falling backwards

Have the student enter a climb with eyes closed.  After 30 seconds give

yoke an abrupt forward movement to slightly nose low attitude while telling student to open eyes.  Student will have sense of falling backwards.

Further forward pressure only increases the effect.  This illusion also has a reverse illusion causing a sensation of falling forward.

Climb

An in flight illusion of climb acceleration by having student close eyes while you fly straight and level at slow flight.  Increase speed while maintaining straight and level.  Student will have illusion of climb until opened eyes prove otherwise.

Climb-dive

Student eyes closed.  From straight and level roll into 45-degree bank for 90-degrees of turn.  Student illusion will be of climb until eyes are opened.  Do same maneuver but have student open eyes when nearly straight and level.  Student illusion will be that aircraft is diving.  Wing-level skids with eyes closed gives tilted sensation.  Steep half Dutch-roll will cause student to sense bank in opposite direction.

Falling

Have student close eyes and move head forward and down in 20-degree stages.

Enter coordinated 30+-degree bank.  Have student turn head quickly and just as quickly bring head up.  Time roll recovery to coincide with heads up.

Illusion is falling to direction of roll.

Lowering Clouds:

At 2000' and under the hood have student note the time and after one minute

descend 500', after two more minutes initiate another 500' descent, after 4

minutes descend still another 500', advise that after 8 minutes he will need to descend 500' more.  You are looking for the student to make a decision to turn back or land.

Icing:

At level cruise and under the hood.  Advise the student that after initial

application of C.H. the power will be reduced 1-200 rpm every minute by the

instructor.  The student is to make a decision as how to proceed.

(Climb while able, descend to warm air, land.)

False horizon

Eyes closed under hood.  On a dark and hazy night fly diagonally across a

freeway where other lights are infrequent.  Problem can be made worse by angled road lights or light patterns.  Angle of lights could cause student to react with dangerous flight attitude. 	Pilot thinks horizon is out of level and attempts to align with illusion.  In IFR conditions this will occur as you break out below a solid cloud ceiling.

Eyes Closed or under hood.  On a dark and hazy night fly diagonally across a freeway where other lights are infrequent.  Light pattern could pilot to react with dangerous flight attitude.

Elevator Illusion

Caused by quick upward acceleration as in updraft.  Climbing sensation causes dive reaction.  Could be caused by downdraft which results in climb reaction on controls.

Eyes closed.  Make 90° turn with very slow bank entry from level

into a 50° bank attitude.  Student should have illusion of a climb.  This same illusion can occur when caught in updraft.  A sudden downdraft can give illusion of dive even though aircraft remains in level attitude.

Coriolis Illusion

Caused by abrupt head movement during a constant rate turn that has existed long enough to stabilize the inner ear.  Gives sensation of motion on another axis.  Attempts to correct

Eyes closed, head down and tilted (Cover AI).  Enter extended constant rate turn until student no longer senses turn and bank.  Have student make sudden head movement to instruments and open eyes.  Student should sense rotation on another axis.  May react to dangerous flight attitude.

Inversion Illusion

Caused by abrupt change from climb to level flight.  The pilot senses that he is falling backwards.  The instinctive response of lowering the nose only makes the sensation worse.

Eyes closed.  Have pilot enter and trim for climb.  When senses

have had settled, have student push yoke forward to nose low attitude while opening eyes.  The pilot should sense feeling of going over backwards.

Leans

Eyes closed. Enter a bank so slowly that it might not trigger the inner ear motion senses.  Once into the bank, make an abrupt correction.  The student will react to his mistaken senses then by re-entering the

previously created bank and will remain there.

Lighting Illusions

Pilot mistakes roadways for runways.  Give illusion of being quite close 		when lights are bright.  High approach results.  Low approach results when 		runway is approached over dark area.  Don’t use landing light until close 		to ground.  The visual cues used for a normal night landing seem much the 		same as you get with a rapid increase in sink rate.

Addendum

Subject: Re: instructional use of MS flight simulator...

To answer your question I will need to tell you a 'war-story'

At the end of WWII I was 21 years old.  I had worked for about

nine months as a LORAN instructor and then as the operator of a

bombing simulator for the use of B-29 crews.  When the japanese

surrendered, I had to wait my turn for the ride back to the U.S.

Those who wished, could go back on a B-29 but military planes are

not as safe as they might be.  I chose to wait for a ship.

In the barn like quanset building next to mine there were eight

flight simulators.  These were the only type in the world at that

time called 'Link trainers".   The Link looked like a stubby wing

airplane about 8' long and 8' of wing.  The cockpit had a cover

that would close and put you in a cockpit with only instrument

lights.

The controls and instruments of the Link were based on the twin

engine DC-3.  With nothing to do for nearly two months except sit in

and fly the Link I became quite capable of flying the Radio Range

system into McCellan Field near Sacraments, CA.

Twenty-one years later I took up flying lessons at Concord, CA.

Because of my prior experience in instrument flying, I had a

terrible time, initially, learing to fly.  I did not know how

to look out of the airplane or even what to look for.  I had to

un-learn my concept of flying and learn another one.

On the other hand, when it came to night and hood work I could

fly quite well because of my hood experience.  To answer your

question, yes the simulator will be of benefit but not where

you expect.  The simulator time will make it necessary for  you

to mentally reject the instruments and learn to look outside.

My telling you this and your accepting it, will make your

learning to fly come easier but not as easy as it would be if

you had spent the time reading "Stick and Rudder", Ron Machado's

book or even running through my disks.