Fuelã

Fuel FARs

Fuel Types

Fuel weight

Fuel Gauge

Fuel Tanks

Fuel Contamination

Fuel vent

Fuel Valve

Shut down

Refueling

Primer

Mixture

Leaning

Benefits of Leaning

Preignition

Enrich on Descent

Fuel Management

Fuel Exhaustion

Exhaust temperature gauge (EGT)

Fuel in Low Wing Aircraft:

Jet Fuel

Oil

# Fuel FARs

FARs require that pilot have fuel for 1/2 hour day and 3/4 hour night flight in aircraft when landing.   Any less is violation of FARs.  A complete flight log with time of takeoff and each fuel change (if any)

needs to be recorded.

# Fuel Types

Most older aircraft fuels have two numbers such as 80/87 (red).  These octane numbers relate to the anti-knock capability of the fuel.  This means the ability of the fuel to resist premature detonation.  The 80 is for normal engine operation.  When the engine is operated under full power and corresponding higher engine pressures, the fuel is rated at 87 octane.  80/87 fuel is colored with a red dye and contains lead as a additive.  100 octane fuel (green) contains even more lead.  100 low lead (blue) has less lead than either of the preceding and can be damaging to engines designed for leaded fuels.  There are additives that can be added to low lead fuels to prevent this damage.   At 1 fuel pound to 11 of air mixture is very rich.  At 1 to 16 fuel is lean.

Lower octane fuels burn more quickly and are subject to detonation.  In an automobile this is called 'pinging' or 'knocking'.  Detonation occurs at high power settings and lean mixtures.  Pre-ignition is the result of hot particles in the cylinder that ignite the fuel without regard to magneto timing.  Because of propeller noise the noise caused by these factors cannot be heard in an airplane.  The damage to the engine cylinders and valves is just a bad, though.  High-octane fuel burns more slowly and predictably.  High octane allows greater compression and power.  130-octane fuel was invented just prior to WWII and was a leading factor in increased aircraft performance that made winning the war possible.

# Fuel weight

An aircraft burns pounds of fuel.  The tank holds gallons.  Each gallon will vary in weight according to the temperature.  A pound of fuel requires less space when it is cold.  Thus, the temperature will affect the amount of useful fuel you have, your range and weight. The more fuel you carry the greater will be the changes.  Standard weight of fuel is 6 pounds per gallon.  A hot day causes fuel to expand.  Refueling in the heat of the day means that you will get a reduced poundage of fuel.  We measure fuel in gallons but an aircraft uses it in pounds.

# Fuel Gauge

The gauge becomes increasingly more accurate as the fuel level reaches empty.  A gauge reading full may be several gallons short of reality.  The more level the wings, the inflation of the nose strut or the slope of the terrain can make a significant difference in the amount of fuel in the tanks and showing on the gauge.  This is the reason a visual check of the tanks is mandatory.  A dip stick long enough not to fall into the tank is a good consumption gauge if marked according to both time and type of flying.  Fuel gauges may have never been recalibrated since new.  Since few airport surfaces are level enough to give gauge accuracy you must rely on developing your own flight-time scale by refueling fully at every home base shut down.  With a few flights you can make a chart for fuel use as your flying dictates.  The reliability of the fuel gauge does not follow

that of the flight instruments.  Flying below 1/4 tank on the gauges takes all the fun out of being in the air.

Light aircraft have fuel gauges that are calibrated to read ‘zero’ during level flight only when the remaining fuel is equal to the unuseable supply remaining according to FAR 23.959.  At no other time is the gauge required to read accurately.

# Fuel Tanks

Where the fuel tank is part of the aircraft structure it is a “wet-wing” aircraft.  This type of wing tend to weep under stress of vibration and age.  Every tank type will have a sump drain near the lowest drain point.  Every fuel tank is required by FAR to have a fuel gauge.

Tanks with filler caps usually have safety chains.  Any fuel cap that is not snug can allow the low pressure over the wing to siphon fuel from the tank.  Overflow and anti-siphon and replacement air ports to tanks are required.  System will have ON/OFF valve and possibly tank selector.  The tanks are interconnected and vented to replace used fuel with air.  In some high wing aircraft only gravity feed is required but most have engine pump and a few have electric pumps to resolve reduced fuel flow at high angles of climb.  Fuel flows from tanks through the selector to the main fuel strainer to the carburetor.

Low wing aircraft must have electric pump as well as engine pump.  Electric pumps have fixed output while engine pumps vary output with engine speed.  With the electric pump fuel flow will be excessive at low power and may require adjustment of the mixture for smoothness.  Fuel selector limits pump operation to tank with fuel.  Otherwise pump could suck air and cause vapor lock.  The cockpit may have fuel flow meter.  System must have sump strainers at lowest point of system as well as for each tank.  Some tanks have markers inside tanks to show less than full levels.  Every POH should have diagram of fuel system.  Know it.  Flush mounted fuel caps are most likely to allow water to enter.  Pour clean fuel sample around cap to see if it drains into tank.  Pump operation and pressure should be confirmed before starting engine.

A fuel tank that is not full is a potential engine stoppage even on takeoff.  A rapid turn on to the runway can cause the fuel in the tank being used to slosh away from the drain port.  This is called unporting.  Baffles may be built into the tank to prevent this undesirable movement of the fuel.  The fuel in the system will allow the engine to run just long enough to get into the air before the air gap caused by the unporting turn causes the engine to give its gasp when ingesting air instead of gasoline.  A fuel tank that is built into the wing is called the 'wet-wing' type.  Every fuel tank is either vented or pressurized and will have a sump drain at its lowest point.

When the temperature gets in the area of 0 degrees Centigrade along with an

even lower dewpoint, ice crystals will develop in the fuel.  These ice crystals can cause a fuel blockage. Ice crystals in the fuel will sparkle in bright lights hone on the fuel sampler.

Bladder Type

In C-182s and C-210s.  20 year life if polyurethane.  Expensive.  Best life if kept full.  Aircraft with bladder type tanks will give full readings if venting does not occur.  The tanks collapse from the bottom so that gauges will be reading full as the tanks are depleted.  Such collapse can be checked by removal of caps immediately after shutdown.  The sucking of air will be apparent.  Bladders should never be left empty or allowed to dry out.

The preflight of bladder types tanks is a two man operation.  First after rocking the wings to move water over the ripples in the bottom of the tank.

Drain the sump.  Then, have the tail lowered as much as possible and again

draqin the sumps.

# Fuel Contamination

Fuel can be made unusable by solid particles.  Tank sealant, rust, and other particles can clog the fuel filters and require that fuel filters be routinely cleaned.  Algae can grow in water fuel conditions and will cause the fuel to appear red, brown, or black.  The existence of algae in the fuel is hazardous.

Biggest culprit is the fuel caps and their seals.  Some fuel caps are vented and some are not.  Care in taking sump samples.  Avoid filling from services that are used infrequently.  Always check color, feel and smell.  Old gas smells stale.  Water will accumulate in partially filled tanks through condensation of air in tanks.  Over several days of hot and cold this can amount to several ounces of water.   Unlike carburetor icing, fuel contamination and low fuel pressure will cause a rough engine without the corresponding loss of RPM.  If you have doubts as to the presence of water, try spinning the sampler.  Any water will show as bright spots in the sample.

# Fuel vent

A blocked fuel vent can prevent the replacement of fuel used with air.  Under such conditions gravity pressure or a fuel pump may be unable to provide fuel to the engine.  There may not be anything you can do about this other than seeking an altitude that will remove the blockage.  Try ground level.  Checking the fuel vent is always part of the preflight.  On a Cessna the functioning of the fuel vent can be verified, beyond doubt, by removing the fuel cap, attaching a piece of medical hose to the vent and blowing through the hose so as to hear the bubbling of fuel in the tank.  Vents are expected to prevent fuel siphoning and blockage.  Interconnected tanks have an common vent system designed to equalize fuel usage.

# Fuel Valve

Valves should be lubricated and lock into their indent positions.  Any stiff or partial positioning should be referred for maintenance.  On two

aircraft checkouts I have found that the fuel valve would not shut off

the fuel sufficient to stop the engine.  The carburetor holds enough fuel

to allow you to taxi to a nearby runway, do a ‘quick’ run-up and perhaps

get airborne.  It is certainly something to be tested.

# Shut down

Use the mixture control to shut down the engine.   This is an important safety procedure which could prevent an engine with a hot magneto from starting.  An engine cannot run without fuel.  Unfamiliar people tend to

touch propellers.  As little as a couple of inches of movement from the right place can cause an engine with fuel to fire and runw.  Don’t touch

propellers unless part of a pre-flight.

# Refueling

The National Fire Prevention Association's Bulletin 407 (1993) advises that it is safer not to ground or bond an aircraft to the refueling source.

Grounding will not prevent sparking at the fuel surface, the wire may not be able to carry the current involved and will not prevent discharge inside of tank.

It is frequently the practice not to completely fill tanks.  This allows an expansion area for the fuel as the temperature increases.  It is important that the pilot realize that under some conditions the amount of fuel available is not the maximum possible.  Not filling the tanks can cause moisture in the air to condense and settle into the gasoline.  This is the most likely cause of water in the aircraft fuel drain sumps.

# Primer

The primer allows the pilot to administer fuel to the intake manifold of the engine before using the starter.  If this is done a minute or so before starting the fuel vapors will circulate more evenly to all cylinders for more even starting.  The primer bypasses the carburetor and draws fuel direct from the fuel line.  Priming makes more certain that a relatively rich fuel-air mixture will enter the intake valves.  The colder it is the more you should prime.

The primer pump may have a long of four inches or short stroke of two inches.  The plunger has a 1/16 pin that fits and locks into a notch.  Unlock the plunger and slowly pull out to its full extension.  Doing this too fast prevents the plunger to develop sufficient suction to draw a full charge of gasoline.  Since we are seeking to atomize the fuel we must apply plenty of pressure when pushing the primer in.  If the primer does not move easily, try a little nose oil.  Oil of the Nose is a private brand easily available in the cockpit.  Be sure to re-latch the primer to prevent an overly rich mixture.  (An unlatched primer once cost hundreds of dollars of mechanic time seeking an engine problem.)

The method to use for priming depends upon engine, aircraft, weather conditions, and your experience.  Some aircraft will start on a warm day with two pumps of the throttle; some require priming every time.  A warm engine may not need any priming.  The difference between priming with the throttle accelerator pump and the primer is essential knowledge.  The accelerator pump fills the carburetor while the primer sprays the intake manifold.  When the weather is cold use the primer not the throttle.  Four or more pumps of the throttle can cause the carburetor to overflow into the air filter system.  Fumes can be ignited by the exhaust.  In this event, and the engine has not started, pull the mixture, apply full throttle and continue cranking with the starter to suck the flames into the exhaust system.   Use the radio, give aircraft and location.  Be prepared to leave the aircraft.  If the engine has started pull the mixture and apply full throttle to use the fuel as rapidly as possible.  Shut off fuel.  Again be prepared to leave aircraft.

The primer injects fuel directly into the intake manifold.  Some engines require such prime at every start.  All engines require use of the primer when temperatures are below 50 degrees F.  Correct priming saves the starter and battery.  Some priming should be done prior to the rest of the preflight.  This allows the fuel/fumes to distribute themselves to all cylinders.

On warm days with most aircraft below 200 hp a couple of pumps of the throttle accelerator pump will suffice for priming.  However, over priming by this method can cause fuel to flow from the carburetor and cause potential for an engine fire.

# Mixture

In the early training it is well to cover the use of the mixture control.  The fuel/air mixture that provides the best power to an airplane engine is about 16 pounds of air (some say 14) to 1 pound of fuel.  The mixture control often in conjunction with an Exhaust Gas Temperature (EGT) is used to adjust the weight of fuel to the weight of air under existing conditions. Not in C-150.  At 800 rpm very little air is entering the carburetor so it is desirable to lean the mixture by pulling the mixture out a bit.  Just how much depends on how the mixture cable is attached to the carburetor.

The mixture control is necessary and used to adjust the weight of the air fuel mixture to 15 to 1 as the air becomes less dense.  Air density is related to altitude, heat and moisture.  The mixture control operates a pencil point probe into a pencil sharpener shaped hole.  While the throttle controls the air the mixture must adjust the fuel for best 15 to 1 engine operation.

Improper setting of the mixture control can cause too much fuel for the air available (over rich) or too much air for the fuel available  (too lean).  The wrong mixture can cause serious engine damage.  Aggressive leaning is a likely damage cause when flying with low fuel.  Poor operational practices can cost you money.

Leaning during taxi is recommended because at low power the heat of the engine will not be enough to vaporize the fuel additives (lead etc.)in a rich mixture.  Unvaporized additives will accumulate in the lower spark plugs and short them out.

When the engine in developing full power as at sea-level and takeoff the mixture control will be fully in and fully rich.  As the air thins with altitude the engine develops less power so the reduced fuel demand is adjusted by the mixture control.  At idle-cutoff the pencil point of the mixture needle completely fills the pencil sharpener hole.

There is a time when the engine may be deliberately operated for a brief time in a lean condition.  Aviation fuel now contains less lead but other ingredients can cause fouling.  Under high power operations these will be vaporized and exhausted from the engine.  During low power operations fouling can occur. If during the magneto check the RPM drop indicates such a fouled plug, it is necessary to increase power (heat) in the engine to a point that will vaporize the accumulation prior to takeoff.

American aircraft have two spark plugs per cylinder, an upper and a lower one.  The fuel induction system of the engine tends to provide more fuel to one cylinder than all the others.  This cylinder gets more fouling particles.  At low power settings the bottom plug of this overly rich cylinder can accumulate unvaporized material.  This will effectively short out the sparkplug and cause the engine to run rough during the magneto check.  When this roughness occurs the mixture should be leaned so as to cause the cylinder to run hotter.  This should (may) cause the particles shorting out the spark plug to vaporize thus removing the short.  If the condition cannot be improved a mechanic should be consulted.  Leaning during low power taxiing operations serves to prevent this condition.

The idle-mixture setting can be checked for both carburetor and fuel-injection by noting the tachometer when you pull the mixture during shutdown. Run the engine at idle, slowly pull the mixture and as you get close to idle-cutoff the engine rpm should increase slightly.  You should see about a 50-rpm rise before the engine quits.  The engine is running too lean at the idle setting if this does not happen.

# Leaning

Climbing to altitude causes the mixture to become increasingly rich because of the decrease in air density.  A descent increases air density so the mixture must be enriched to reduce cylinder temperatures and optimum performance.  Avoid rapid large power changes that are going to shock-cool the engine and warp the bimetalic cylinders.  Such engine operation causes scuffing and lubrication problems of both the pistons and the cylinders.

You are not realy leaning.  You are reducing the weight of the fuel getting to the carburetor to compensate for the reduction in the weight of air as the air density decreases with altitude.  This is done so as to keep the ratio of air to fuel constant for maximum engine performance.  When in doubt, err on the rich side.  Detonation may be caused by apparent proper leaning where the manifold pressure gauge reads low.  You can't hear or feel detonation.  When you lean check the CHT for a rapid rise.  If a CHT rise occurs just after leaning or adding power assume detonation is occurring.

As altitude is gained the weight of the thinner air decreases inside the cylinders.  Unless the mixture is adjusted to maintain the 16 to 1 ratio the engine will operate overly rich and both lose power and waste fuel.  Mixture can be adjusted without an EGT by watching the tachometer and listening to the engine.  From full rich the mixture is incrementally pulled out.  At one point the engine will momentarily show an increase in rpm.  This is the best power 16 to 1 setting by weight.  Incremental pulling of the mixture will cause a decrease in rpm and engine roughness.  The mixture is now on the lean side of the power curve.  For normal conditions the mixture is pushed back in past the best power setting to a slightly rich position.  The engine will smooth out.

At a low power setting, lean without concern of damaging the engine.

When operating below 75% power you can avoid detonation by leaning to peak

EGT temperature.  At 75% or higher power settings lean on the rich side by at least 50°.

With the advent of 100 LL fuel it is advisable to lean immediately after starting the engine at sea level and again shortly after clearing the runway on landing.  This is a preventive for sparkplug fouling and subsequent rough engine operations the next time out.  On a 100 degree day at sea level the density altitude approaches 3000's.  3000' is a good altitude to commence leaning practices with adjustments made every 1000' thereafter.  Leaning the engine does not prevent the full throttle jet for engine cooling from opening during climb.

I once had a C-150 engine quit cold at 800' while taking off from an isolated strip in the Missouri Ozarks.  It was not a very hot day, nor was the airport altitude over several hundred feet.  800' is not much room to get an engine restart but I was able to avoid an emergency landing in a lead mine leaching pond by leaning the mixture to gain a restart.  I also had the engine of a fuel injection Arrow stop during the landing rollout at Albuquerque.  Couldn't get a restart and had to use to tow bar to get clear of the runway.  You don't always land with a rich mixture.  Remember that pulling the carburetor heat in a density altitude situation is going to make a significant increase in the engine's operational density altitude (hot air) and greatly enrich the fuel mixture in that air.

Lycoming Leaning Bulletin 1995

- Any mixture movement should be done slowly.

- 50°F maximum temperature change per minute.

- For long engine life:

65% power or less

Carburetor temperature 400^F or below.

Oil 165-220°F

Lean to 100^ rich side of peak power.

- Lean for best takeoff power above 5000’.

# Benefits of Leaning

1. Lowers operating costs by conserving fuel

2. Reduces roughness caused by running rich.  Smoother operation extends 	life 	of engine mounts and accessories.

3. Slight increase in airspeed

4. Extends range of aircraft and increases fuel reserves

5. Reduces spark plug fouling, extends life of plugs, reduces maintenance.

6. Keeps combustion chambers clear and reduces possibility of pre-ignition 	of deposits in cylinder.

7. Gives normal operating temperatures needed to boil water and acids out 	of the engine oil.

8. Greatly extends endurance and range with a very slight decrease in 	enroute time if lower cruise power used.

9. Leaning reduces the probability of carburetor icing.  This is because 	there is less fuel to vaporize and less heat can be taken from the 	surrounding air.

10. Leaning increases the effectiveness of carburetor heat because it 	makes the engine run hotter and causes the exhaust system to give 	more heat into the carb heat air when applied.

# Preignition

This is automotive 'pinging' but cannot be heard in aircraft.  If the ignition is too far advanced and the engine is put under load as in a climb, preignition can occur.  This can be causes by lower than specified octane fuel and carbonized piston crowns.  Repeated operations can fracture pistons.   Problem can be reduced by reducing power and shock application of throttle.  A problem for a mechanic.

# Enrich on Descent

Be sure to make mixture richer as aircraft descends.  All takeoffs and landings at or near sea level should be full rich setting.  Takeoffs and landings at altitude should be leaned for the performance required.  I once had a fuel injected engine die on the runway at a 6000' airport just after landing.  Too rich.

At altitude the air weighs less so the mixture must be adjusted to correct for the change in weight.  This is done both for economy best engine operation.  Any time engine is being operated at 75% power it is O.K. to lean.  You can lean by engine performance.  By slowly pulling the mixture with brief pauses any changes can be heard and felt.  At best power a gain of one or 2 hundred rpm will be noted.  At overly lean the engine will run rough.  Running  a little on the rich side keeps the engine cooler.

# Fuel Management

- Figure 1/2 gallon per cylinder over normal for climb per hour.

- Figure  maximum endurance to be same as best rate of climb speed.

- Figure  maximum range speed to be best rate of climb speed + 25%.

- Fuel gauge reading can be deceptive.  A pilot should visually check the tanks prior to takeoff.  He should take a worst case fuel consumption rate and base flying time based on this consumption plus required reserve.  Always flight plan for an alternate.

# Fuel Exhaustion

The three kinds of problems with fuel are contamination, starvation, and exhaustion.  There are several ways a pilot can avoid the number one reason for engine failure.  Fuel is time of engine operation, not pounds, gallons, or BTU, just time.  Note the time you start using the engine on your flight log.  This is the only reasonable way to keep tabs on your fuel in most light aircraft.  You should keep a 'stick' for your aircraft which can be marked for time and consumption.  Make a flight from full tanks and then compare the time of flight with the amount of fuel it takes to fill the tanks.  Remember temperature affects both the density and consumption of fuel.  You will never be able to equal POH fuel consumption figures with an old aircraft.  Develop your own consumption figures for every aircraft.

Aircraft gauges are just as unreliable measures of fuel out of tanks as would using another pilot's estimate of fuel remaining prior to your using the plane.  FARs require initial airworthiness gauge accuracy only when full and empty.  No inspections of gauge accuracy is required.  If I cannot touch the fuel with my finger 'stick' I estimate over one hour is out of the tanks. Gauges must by FAR read correctly only at full and empty.  A good rule is to fly to 50/50.  Fly time to half tanks and then get fuel at the nearest airport.  You will see many interesting people and places doing this.  Every three days a plane lands without fuel in the engine.

A pilot’s attitude toward himself, and probability  will determine the

likelihood of a fuel exhaustion accident.  Any time you plan to be flying after four p.m. local you should plan to refuel immediately.  After four p.m. many facilities begin to close down.  If you consider a fuel tank as being half full instead of half empty then your personality defect may lead to a fuel exhaustion accident.  While there are many credit card pumps becoming available, you need to know where they are.  Government ATC agencies may not be of any help unless they have a Airport/Facilities Directory.  Regardless of your politics it is best to be conservative when it comes to figuring available fuel.  If you didn’t start with full tanks you have no way to know how much fuel is really available.

# Exhaust temperature gauge (EGT)

The ideal flight has the pilot operating the engine in accordance with the

manufactures’ recommended operating procedures.  This is especially true when leaning is involved.  In the ideal the EGT is properly installed and calibrated.  With an EGT (Exhaust Gas Temperature) gauge the setting of the mixture can be determined by the needle movements. This probe is adjusted on the richest cylinder.  This cylinder is determined by the engine design, manifold design, and cooling.  Multiple display allows comparison of cylinders.  Fuel injected engines and high power engines will have less variation (100°F) than the (200°F changes between carburated cylinders.  The colder the ambient air the wider the changes.

Usually the mixture is leaned until the needle peaks (reaches its highest point)  of best power.  This is the basic reference for all fuel management and will vary with different power settings, altitude and temperatures.  This can be determined only by when it falls from the high point and marks the best economy fuel mixture setting. EGC has less power than a slightly enriched setting.  Best power is about 100°F to the rich side oof peak.  Peak Engine roughness on the lean side of peak is a function of cylinder fuel differences and not detonation.  The mixture is made richer for a 50 degree drop from the high point on the rich side of best power as a rule of thumb.  The 50°F setting to rich side of peak compormises power

and economy.

Leaning past peak is only feasable on engines over 250 h.p.  Turbo charged engines have a turbine inlet temperature gauge which is usually limited to 1650 degrees F.  In most engines damage is caused when operating at maximum power and agressive leaning.  The engine design limits of compression and temperature can be exceeded by improper fuel settings and high power operation.  Detonation occurs above this limit and may not be noticeable from the cockpit.  Engine roughness, backfiring, and high cylinder temperatures can be indicative of preignition and detonation.  Only an internal inspection of the engine can show for sure the engine damage caused by improper fuel managment.

The EGT uses two dissimilar metal wires to create mini-voltages as heat is

applied.  The EGT gauge is a millivoltemeter calibrated to show temperatures of the exhaust gasses.  The temperatures and the gauge operate linearally through very careful selection of the two metals used.  A Type T thermocouple is used on a Carburetor Heat Temperature (CHT) gauge of very specific metals and lengths.  Any substitution of type or length will invalidate CHT readings.  EGT guages use Type K thermocouples made of chromel and alumel because of the higher temperatures.

These gauges can read incorrectly if an improper thermocouple is used or

has been cout to improper length.  No reading probably indicates a break in the thermocouple.

# Fuel in Low Wing Aircraft:

Confirm pump operation at every use of switch by touching pressure gauge.  Electric pump should be used (normally) 5 times, starting, takeoff, landing, changing tanks and emergency.  Check POH since some aircraft have exceptions were inadvertent use can kill engine. (Bonanzas) The unporting of a fuel tank mentioned above is most likely to occur in low wing aircraft.

The changing of tanks should be recorded by time on a separate chart. the balance of the aircraft for ease of flying suggests that changes every half hour makes for easier flying.  If you always start from full tanks on the same tank it makes it easier to keep track.  Plan your tank changes near airports just in case something goes wrong.

# Jet Fuel

Even a slight mixture of jet fuel will cause trouble.  Kerosene odor, slick feel and will not evaporate.  Fuel tanks should be placarded for type of fuel.  Nozzles of jets are different size and shape.  It happens.

# Oil

Oil does more than lubricate.  It cools, cleans, protect and cushions as well.  Oil prevents damaging metal to metal contact by forming a very thin film.  Oil that is too thin may not prevent sufficient wear.  This film reduces but does not totally prevent frictional build up of heat and engine component wear.  If the oil is too thick, it can’t flow between the parts and prevent wear.  Oil viscosity changes with temperature so a variable viscosity is best for varying conditions.

Carbon on the oral screen is a warning about stuck piston rings.

The cooling function of oil is required to carry away the combustion and friction heat inside the engine.  Additives keep the products of combustion in suspension, thus preventing carbon and ash buildup on the pistons and valves.  Oil is a corrosion preventative that lasts only so long as a surface is oil coated.  An engine left inoperative over a long period of time will have the oil drain off due to gravity until the internal parts are no longer corrosion resistant.

The weight or viscosity of an oil determines its ability to lubricate under specific temperature conditions. Viscosity is the ability of the oil molecules to remain in contact. The lower the viscosity the better the lubrication in cold weather. In hot weather a low viscosity number may be too thin to provide a lubrication shield between moving metals.  The first used oil in a new engine is designed to allow the metal parts to wear and fit together.  Today most oils are of multi-viscosity and of ashless dispersant capability.  This means that the oil will provide proper lubrication over a wide range of temperatures.  Any residue of combustion will be held in suspension until removed by the oil filtering system.  New synthetic oils may be too slick for some operating parts, do not remove lead well, and are harmful to rubber products outside the engine.

Shell 15W-50 is made from a combination of mineral oil base and synthetics.  Mineral oil can absorb and disperse lead from the fuel.  They have recently added an anticorrosion compound to its ashless dispersant (not detergent) additive.  The anticorrison compound works and can be mixed with other types of oils.  Do not mix single grade with multi-grade oils.  Automotive oils should never be used in aircraft since their combustion forms high-melting point byproducts that cause preignition as well as valve sticking.

The taking of an oil reading requires that the pilot be aware of engine characteristics.  A cold engine on a cold morning can cause the oil to creep up the dip stick and give an erroneous reading up to four quarts high.  A hot engine can give a low reading because of oil retained in the engines parts that has yet to affect the stick reading.  Some dip sticks have O-ring seals that will force oil down the dip stick tube and give a very low reading (C-182) when there is actually plenty of oil.  Don't allow oil from the stick to contaminate the ground where people will be walking.

Oil temperature should stay at 180 degrees during climb unless on summer day above 7000'.   Higher temperatures could be due to stuck oil cooler bypass valve or leak.  If oil temperature is rising and pressure is falling, land.  Low pressure without rise in temperature is likely gauge.  Low pressure during idle  usually indicates a leak in the oil-relief valve.  Every pilot should know that an oil pressure and oil temperature gauge are required instruments on aircraft.

Piston-ring problems blacken the oil, raise oil temperature and increases blow-by oil on the aircraft belly.  Ring leakage makes the bottom plugs oily and may not appear in a static compression check since weak rings may not be able to adjust to the cylinder barrel with the engine running.

Engines with small oil sumps tend to have lower consumption than do engines with large sumps.  Oil consumption is proportionate to the number of cylinders and the bore of those cylinders.  Oil consumption must be lower in high compression engines because of the operating octane required.  The harder you run an engine the higher will be the oil consumption.

Some engines can use a quart per hour.  This level of consumption is the point at which an overhaul is due.  Watch for sudden changes in consumption.   A clean engine makes it easy to spot new leaks.  Use oil analysis to determine changes in metal content in the oil.  White smoke through the exhaust system is oil passing rings or valve guides an expensive problem.  Increases in oil consumption iindicates a malfunction or part failure.

Oil sumps are either the more common wet or dry.  Oil is pumped through the oil filter to the bearings and camshaft surfaces.  Wet sumps splash and spray oil on the internal engine parts.  This oil gravitates into the sump where it is sucked up by the pump to repeat the process.  Larger engines use dry pumps.

The most important single thing that can be done to extend the life of an aircraft engine is frequent oil changes.  oil serves as a barier between moving parts of the engine.  Oil contains additives that purposely neutraalize and suspend contaminants.  This is why dirty oil is not a bad thing.  It is just oil doing its cleaning job.