Click on page number in contents for instant scroll.

Emergency and nightã

A. Task: EMERGENCY DESCENT

B.  Task:  EMERGENCY APPROACH AND LANDING

C. Task:  SYSTEMS AND EQUIPMENT MALFUNCTIONS

D. Task: EMERGENCY EQUIPMENT AND SURVIVAL GEAR

A. Task: NIGHT PREPARATION

B. Task: NIGHT FLIGHT

A. Task:  AFTER LANDING

B Task: PARKING AND SECURING

X.  AREA OF OPERATION

EMERGENCY OPERATIONS

# A. Task: EMERGENCY DESCENT

REFERENCES: AC 61-21, operating handbook, flight manual

EMERGENCY DESCENT PROCEDURES

Reasons for an emergency descent.  Leave 3000 feet for 1500 feet in power dive as though having engine or cockpit fire.  Compare with power off nose high slip to lose altitude.

P    1. Knows and can discuss factors of urgency, aircraft limits, and 				techniques relevant to both emergency descent and survival.

P    2. Recognizes the urgency of an emergency descent.

P    3. Establishes the recommended emergency descent configuration and 			airspeed, and maintains that airspeed, + 5 knots.

P    4. Demonstrates orientation, division of attention and, proper planning.   		No descents below 1500’ or above Vne

P    5. follows the appropriate emergency checklist.

EX	The nature of the emergency helps decide the best descent procedure.

A major cockpit or fuel fire would require a Vne descent.  An engine

fire could be kept away from the cockpit by a nose high, power-off,

slipping descent.  A descent through an overcast of unknown base 	might 	require slowing to Vmc with power on to control the descent and hands-	off rudder flying for heading.  A black hole situation would use a 	similar descent where the surface was unknown.  A descent through a hole 	in an overcast could be made with full flaps and power off in a 	descending spiral with flap removal below the bases to expedite the 	descent.  There are probably more.

When you are faced with the need for an emergency descent you have two options that give nearly the same rate of descent for the required 1500’ altitude loss.

Procedure #1

Carburetor Heat, power off and descend at structural cruise speed (where

the yellow and green meet).  Descent should level off after losing 1500’ and

no descent below 3000’.  This descent is at a relatively high speed and covers

considerable distance.  On arrival at the 1500’ descent point you are going

so fast that even more distance (read time) will be required.  This is not the descent you would use to get below a cloud deck.  Since clouds are all of

indeterminate size and distance you are quite likely to reach the clouds before getting below them.

Procedure #2

Carburetor heat, power off, at white arc put in full flaps and descend at

top of the white arc.  This descent will get you down in the same amount of

time as #1 but has the advantage of covering less distance (where distance is

not a factor) and your lower speed will allow rapid deceleration to a landing

speed.

Procedure #3

Carburetor heat, power off, enter a full slip into your best estimate as to

wind direction.  Depending on the wind velocity the descent can be anywhere from straight down to an angle approaching that with full flaps.

# B.  Task:  EMERGENCY APPROACH AND LANDING

REFERENCES:  AC 61-21A, Airplane handbook and Flight Manual

EMERGENCY APPROACH AND LANDING PROCEDURES

Best glide speed + 10 knots.

P    1. Knows and discusses which descent, approach, and landing is 				appropriate to the situation.

P    2. Establishes and maintains the recommended best-glide attitude, 				configuration, and airspeed, + 1`0 knots.

P    3. Selects a suitable emergency landing area within gliding distance.

P    4. Selects best option for field, speed + 10 knots, flies arrival 	  	  		pattern with respect to terrain and wind.

P    5. Contingency options opened and selected, seeks cause of problem,

flaps only when field is certain.

P    6. Maintains positive control of the airplane at all times.

P    7. Follows the appropriate emergency checklist.

EX    Able to explain emergency sequence in a variety of possible situations.  	Be ready to demonstrate emergency process from start up to tiedown.

For single-engine certificates, simulated emergency approaches and landings, as well as system and equipment malfunctions are specified items. Judgment is most evident when a flight doesn't proceed as planned.  Good judgment during aircraft operations that are critical to flight safety must be tested and demonstrated.   An example of a simulated engine failure test of judgment often occurs during low level ground reference maneuvers but can occur over an airport.  The ability to determine relationships and alternatives and then make reasonable decisions is judgment.  The ability to adjust one's actions to meet the surprises that come in the PTS and flying is essential.  The student

/instructor combination must give opportunity to develop this judgment.

Situations

- Because throttle has been misused to prime the engine the carburetor overflows.  When started the exhaust sets fire to the overflow.

Options:

1. If the engine has not started, continue to crank the engine with the mixture out.  This will suck much of the fire into the exhaust, and use up fuel in the system.  If this doesn't work bail out and call authorities.  Use radio?

2. If engine is running, pull mixture and apply full throttle. Use radio to call authorities.

- At application of full power, seat slides back, door opens, or a loud bang occurs.

Options:

1. Pull power and clear runway.

- Shortly after lift off, seat slides back, door opens or a loud bang occurs.

Options:

1. Power off, land on remaining runway.

2. Power as needed to maintain control. Radio.

Altitude needed to safely land. May be off airport.

- During climb out at best rate, the engine fails.

Options:

1. Land off airport within 30 degree of heading

2. If altitude permits (800' or more) make steep turn to airport. 240 			degrees required to make departure runway.

- During climb out, the cockpit fills with smoke.

Options:

1. If electrical, turn off master.  Return for landing.

2. If petroleum, pull mixture, forward nose high slip to landing.

Flaps if possible.

3. In event of fire, get on the ground.

- Engine failure at altitude

Options:

1. CHECKLIST

2. An imminent stall will stop the propeller from windmilling and 	 		improve the glide ratio.  The starter can be used to

level the prop and minimize damage.

3. If descent is being made against the wind, add 1/3 of the wind 			velocity for best penetration glide.

4. If descent is being made with the wind, reduce best glide speed 			slightly.

- Fuel gauge on empty, lost.

Options:

1. Land.  Note: Don't fly below 1/4 tanks.  The gas gauge is the least 		accurate instrument on the aircraft.  Factors such as load, winds, 		and carburation can affect fuel flow.

- Lost-possibly flew off the edge of the sectional.

Options:

1. CCCC  Climb, communicate, confess, comply.  Note: Don't delay. Get 		on the radio and indicate that someone has misplaced your 				destination.  A common fault is to descend in the belief that 			something will appear.  Nothing so interferes with mental 				functioning as being lost or misplaced.

- Partial engine failure.

Options:

1. Your first rule is to maintain positive aircraft control as you 			refer to your printed checklist.  Check the critical items so that 		in an actual failure you will deal with the essentials first.  			Smoothness and airspeed control in obtaining and maintaining the 			best glide airspeed and configuration is essential.  Failure to 			maintain just the best glide speed will prevent accurate decisions 		related to gliding distance and touchdown.   Be sure to consider 	 		factors as altitude, wind, terrain, obstructions in your planning.  		If you have made a poor choice of an emergency landing area, don't 		hesitate to make only one new choice.

- Engine failure with full flaps

Options:

1. Get the flaps up while applying enough back yoke pressure to 	maintain the best glide speed.  Best power off glide speed is usually 	midway between Vx and Vy.  (Check) Trim down four turns from full flap 	descent at 60 knots and 1500 rpm.

# C. Task:  SYSTEMS AND EQUIPMENT MALFUNCTIONS

REFERENCES:  AC 61-21, operating handbook, flight manual

SYSTEMS AND EQUIPMENT MALFUNCTIONS

Partial or complete power loss, engine roughness or overheat, carburetor or induction icing, loss of oil pressure, fuel starvation, electrical system malfunction, flight instrument malfunction, landing gear or flap malfunction, inoperative trim, open door or window, icing, smoke and fire in cabin, smoke and fire in engine compartment, any other emergency, follows prescribed checklist.

P    1. Knows and discusses the systems, malfunctions, and symptoms 				appropriate to the aircraft used.  You are expected to be able to 		draw all the systems given in the POH.

P    2. Takes action for simulated emergencies, such as--

a. degrees of power loss

b. engine roughness or high temperature

c. carburetor or induction ice

d. low oil pressure

e. fuel starvation

f. electrical system problem

g. instrument malfunction

h. gear or flap problem

i. inoperative trim

j. open door or window

k. structural ice

l. any smoke or fire

m. other emergency

P    3. follows the appropriate emergency checklist.

The examiner is not PIC during the flight test.  The examiner test of emergency procedures should be discussed with the applicant before entering the aircraft.  Components should only be disabled by mutual agreement.

- Partial power loss.

Possible causes: Magneto position, Carb Heat position or door, mixture 	position, fuel system problem, shorted spark plug, air filter plugged, 	and many more.

- Rough running engine or overheat

Possible causes: Carburetor ice, fouled or shorted plugs, magneto short 	or lead, fuel system, etc.

Options

Never kill a running engine.  Use what power you have to get to the

nearest landing area.  Descent cools, rich cools.

- Structural icing

Options

Do a 180 at first sign.  Gain altitude for as long as possible using the 	fastest speed possible.  No flaps and do not slow down in the air.  Land 	hot.  the ice on the tailplane is the most dangerous.

- Flight instrument failure
	  	  Options

Cover face of failed instrument(s).  Proceed to destination.  For

pitot you can apply pitot heat.  For altimeter and airspeed you can

break face of VSI or use alternate air.  Knowing where trim is set

`	      and power setting should allow you to proceed to a safe landing if

you have learned how to establish a stabilized approach.

- Carburetor or induction icing

Symptom: rough engine caused by ice blockage of air intake or butterfly 			venturi.  Cure: Full Carb Heat.  Will cause additional power loss and 				then a rise as ice melts.  Carb Heat off will restore normal power.  A 				climb attitude will increase engine heat.  Use magnetos to make engine 				backfire.  May clear ice.   Try to climb, lean the mixture.

- Loss of oil pressure

Symptom:  Gauge, high oil temperature.

If oil temperature is normal it may be gauge.  Make precautionary 	landing.  If oil temperature is high, reduce to minimum power and get 	on ground.  Don't try to make the next airport.  A partial power landing 	is always a better option than a fire or dead engine.  Reduce power, 	check oil temperature for rise that indicates loss of oil, get into 	landing area and down before engine quits.

- Fuel starvation

Cause: Usually pilot error or contamination.  Cure: Get on the ground if 	you have any power at all.  Be sure to get the power off for landing.

Switch tanks, change fuel selector, check mixture movement, rock the

wings.  Once you are committed to landing it is better to forgo any

effort to get engine running.  A sudden full power engine just prior

to touchdown is very dangerous and unlikely to continue. Many accidents 	have been caused by a sudden resumption of power just as the plane hits 	the ground.  Intermittent engine operation will cease at the worst 	possible time.

- Engine compartment fire.

Shut off fuel, pull mixture, full power to use fuel in system. CHECKLIST

Nose high slip to landing as fast as possible.  Another option is to 	dive vertically in an effort to blow out the fire.

- Electrical system malfunction

If fire, kill master. CHECKLIST  Inoperative equipment check fuse/ 	breaker system.  If alternator, try to reset, reduce electric load and

proceed to nearest airport.  Check switches and amp meter readings.

Initially, shut off the master.  Then turn off all electrical equipment.

Turn on master switch check for operation and problem.  Turn on one

electrical element at a time and check for problem.  Try to locate and

isolate problem.  If problem continues shut off master and proceed to

destination using NORDO procedures for IFR or VFR no radio procedures.

- Gear or flap malfunction.

Undo what you did, first.  Aircraft will be controllable if flap

positions are equalized.  Landing gear up need not be an accident.

If flaps are stuck in position, trim for best performance and proceed 	for landing.  If flaps are asymmetric, get them up if possible.

Maintain best control and get to ground in best landing configuration.

CHECKLIST.   Gear, fly to nearest large airport and use your radio.

- Door opening in flight

This will not affect flight or operation except for noise. If you do not 	know how to close the door in the air, don't try.  Land at nearest 	airport.  Not an emergency.  Close the window.  It is possible to close 	door while 	in the air.  For hard to close door, best option is to land 	and close it.

- Trim Inoperative

An inoperative trim in neutral position just makes flying more difficult 	but constitutes no danger.  In any other position it is best to get the 	plane on the ground at reduced power to reduce control pressures and get 	the trim repaired.

Flying an out of trim aircraft is not pleasant but it is possible.

Shifting some weight might help relive pressure.  Work with the

pressure until you can get on the ground.  Not an emergency.

- Vacuum Failure

Vacuum Gauge at zero. Heading indicator begins to spin.  Attitude 	indicator stops in one position.  May be level of cocked.

- Other malfunctions

Loss of throttle control:

Control what ever engine power you have by magneto

switch.

Stuck elevator movement

Use trim and power combination for control

Stuck rudder

Use doors for rudder control

No brakes

Full flaps, yoke back, ground loop

# D. Task: EMERGENCY EQUIPMENT AND SURVIVAL GEAR

REFERENCES:  AC 61-21; operating handbook, flight manual

EMERGENCY EQUIPMENT AND SURVIVAL GEAR

Emergency locator transmitter, battery life, on/off/arm positions, 		required inspections, battery changes, how to activate after 			emergency landing.

P    1. Knows survival gear and equipment available in aircraft

a. location on/in aircraft

b. operation and use

c. required servicing

d. safe storage

e. gear according to climate and topography

P  2. Follows the appropriate emergency checklist.

The ELT must be replaced as placarded on it by date of 1/2 battery life followed by A&Ps signature.  It must have aircraft logbook entries as to date of installation and A&Ps signature.  Since 6-21-94 ELT inspection required annually as part of annual inspection.  It must be replace if activated for one hour.  Data, dates and location required information for PTS. (flight test).  Test of ELT are limited to three audio sweeps during the first five minutes of any hour.  ELTs in storage must have battery removed.

The aircraft shutdown checklist includes putting 121.5 on the com radio prior to turning it off.  This ten seconds is used to save the time and money that would be wasted if an Emergency Locator Transmitter (ELT) false alarm is pursued by the U. S. Air Force Rescue Coordination Center (AFRCC) now located on the East coast.

ELT Update

Only second generation ELTs (TSO-C91a) can be installed after 6-21-95

Battery expiration dates on ELT and maintenance record

New inertia switch will not activate on hard landings.

Can interface with Loran and GPS

Next generation is:

Emergency Position Indicator Radio Beacons (EPIRB's)

Personal Locator Beacon (PLB's)

406.025 MHz ELT's

System can receive and download information such as 				position and identification in one satellite pass.

Crash Survival

#1 survival item is to let a responsible person know where you are 	going, your route, and ETA.  Insurance is to make contact after 		arrival.  If you are not found quickly, you are not likely to be 		found at all.

Survival items that are nice to have, short-term and essential for 		longer are:

Flashlight, digging tool, foul-weather gear, blankets, food, 		water, matches and candles.   A real plus would be a portable GPS.

a handheld radio, or a cellular phone.  Dehydration is the worst

threat usually facing a downed pilot.

Winter clothing:

Combine high-tech and traditional materials.  It is easier to keep warm by wearing multiple layers.  Inner lawyer should not absorb moisture and wick it away from the body.  1/4 if body head leaves via the head.  waterproof headgear may be the most important single survival item.

Pilots are optimists that believe bad things happen to other people. They believe that flying is so safe that there is no need to be prepared for catastrophe.  Before leaving home you should prepare the 'what if' options.  Whenever you go flying you should be dressed to walk home. You should have required items within cockpit reach.  You should  stay with aircraft and survive until conditions allow foraging.  Short-term rescue is usually within a day.  The greatest danger is exposure to conditions causing hypothermia.  the right condition of wind, temperature, and moisture can expose anyone to hypothermia.  The body loses heat most rapidly when wet.

Rescue can be helped by making your position visible.  Make a clearing, start a fire, make large SOSs.  Use common sense, have confidence in yourself and be patient.  Prior planning, preparedness, practice and training will enable you to make the best decision possible.

XI.  AREA OF OPERATION

NIGHT OPERATIONS

# A. Task: NIGHT PREPARATION

REFERENCES: AC 61-21, AC 61-23, AC 67-2, operating handbook, flight 			manual

NIGHT PREPARATION

To be discussed as part of oral, airport lighting, illusions, 			visual acuity and night physiology, chart reading techniques, 			effect of age.

EX  Discuss what you know about:

1. Physiological aspects of illusions, health, visual acuity.

2. Airport lighting systems and controlled lighting.

3. Aircraft light systems

4. Personal lights

5. Night pilotage, navigation, chart reading

6. Night precautions and emergency options

Carry several flashlights.  Physiologically, night vision ability decreases with age, altitudes above 3000' have significant influence.  Mountain flying is quite different from urban area flying.  You should either do a great deal of night flying or none.  Night flight eliminates 90% of your emergency landing options.  Night flight constitutes only 4% of General Aviation flight time by 25% of the accidents.

Know the lighting systems in use at various airports.  Be aware of how to obtain "pilot controlled" lighting and lighting aids not in use at controlled airports.  Know how to obtain ATC/FSS/RADAR assistance in the event of being lost or disoriented.  Be able to determine airport entry and pattern by interpreting ground aids.

The landing light is a useful identification aid at night.  You may be requested to flash your light for identification.  However, when landing, if you can see the light beam ahead while coming in for a landing you would be well advised to turn it off.  Under these circumstances the landing light can create an illusion which results in a far too high flare.  In any event you should always practice using the landing light every other time to maintain no light proficiency.

# B. Task: NIGHT FLIGHT

REFERENCES: AC 61-21, AC 67-2, AIM, operating handbook, flight manual

NIGHT FLIGHT

Preflight, aircraft lighting systems, and checklists to be covered 		as part of the oral.

P    1. Exhibits knowledge of the elements related to night flight.  Since it 		is doubtful that a demonstration will be required, be prepared to 		review your past night flying experience.

P    2. Inspects the interior and exterior of the airplane with emphasis on 			those items essential for night flight.  Preflight inspection for 		night essentials

P    3. Taxies and accomplishes the before takeoff check adhering to good 			operating practice for night conditions.  Night taxiing 				procedures and night operating practices

P    4. Performs takeoffs and climbs with emphasis on visual references.   			Visual references used in night flight

P    5. Navigates and maintains orientation under VFR conditions.

P    6. Approaches, lands, and taxies, adhering to good operating practices 			for night conditions.

P    7. Completes the appropriate checklists.

EX    Be prepared to discuss flight factors of night flying as different from 	day.  This includes equipment, navigation, lighting, physiology, 	weather, terrain, and airport operations.

Be aware of these differences.  Preflight to include interior and exterior lighting system.  Navigation to cover emphasis on terrain clearance, proximity of check points, and 45 minute fuel reserve.  Be able to select checkpoints that will be viable a night.  Severe VFR at night is best flying weather with 1/4 moon or more.  The night pilot is semi-IFR.  Airports at night harbor optical illusions and minimal crosswind references.  Any fog or low clouds can completely change the appearance of an area.

The most difficult aspect of night operations just happens to be taxiing.  Be prepared to ask for assistance or that the lighting be turned up.  The night takeoff becomes critical if disorientation should occur.  You should be reasonably able to fly on instruments any time at night.  Be sure you have set your gyro and predetermined your departure heading.  Know your terrain.  The actual landing is done with power on and not at a full stall.

See instructional material on night flying.

XII. AREA OF OPERATION

POSTFLIGHT PROCEDURES

# A. Task:  AFTER LANDING

REFERENCES: AC 61-21; Airplane Handbook and Flight Manual

P    1. Knows and discusses importance and sequence of after landing 				procedures

P    2. Taxies to the parking/refueling area using the proper wind control 			technique and obstacle avoidance procedures.

P    3. Completes the appropriate checklist.

EX    Able to explain positioning of controls for taxi, reason for power to

taxi 	and position for parking, reasons for shutdown checklist and 	procedure.  Reasons for securing procedure and postflight inspection.

Checklist, clearing runway, cleaning up airplane, clearance and

readback

When landing in a crosswind the controls are correctly positioned for taxiing on the runway.  The controls must be positioned during each turn

from the runway and every taxiway to configure the aircraft for the wind.

Flaps should be removed prior to application of brakes.  It is possible to lock the brakes while the flaps are down at moderate speeds.  Such braking will give a destructive "flat" on the tire in just a few feet.  Carburetor heat should be removed during ground operations since the heated air is unfiltered and potentially damaging to the engine.  The aircraft must be taxied across any hold bars on a taxiway or well clear if on an intersecting runway.  You should acknowledge any change of tower frequency to ground but do not ever change until across the hold bars.  Stop while contacting ground.

Taxi on the center line even around corners.  Remember that an aircraft to your right has the right of way.  Be sure to acknowledge any ground communications directed to you.  On entering the parking area use only the amount of braking/power required to make a smooth turn and smooth stop.

- Straighten the nose wheel during the last few degrees of your parking turn.  Idle at 800 rpm

- Get out your shut down checklist and follow it regardless of how emotionally exhausted you may be.  Radio to 121.5.  Radios, electrical, off.

- Magneto check, mixture out, momentary increase in throttle then retard.

Magnetos off.  Control lock.  Log time.  Exit.

While you push back the plane check tires.  Listen!  Let the examiner know that you are making a visual inspection of the aircraft as to tie down the aircraft.  Check the oil.  With repetition you can learn how much oil a hot engine holds.  Get all your materials organized.  Clean up the cockpit.

Order fuel for the next guy.  You might level the prop if it can be done safely.

# B Task: PARKING AND SECURING

REFERENCES:  AC 51-21, operating handbook, flight manual

P    1. Knows and is able to discuss parking and securing procedures, hand 		signals, precautions

P    2. Parks carefully and properly with consideration of others.

P    3. Uses checklist to shut down engine sequentially and securing the 		cockpit.

P    4. completes the appropriate checklist.

P    5. Makes post flight inspection