# Chapter 2

# Setting up

# the DVD Player

- This chapter shows you how to set up your new DVD player.  You can enjoy full-length DVD’s with true picture quality and multichannel sound.  You can access the on-screen displays with the wireless remote control.

This DVD player supports the following discs.

| - Disc Type | - Contents | - Disc Size | - Maximum Playback Time |
| --- | --- | --- | --- |
| - DVD video disc | - Audio & Video | - 12 cm | - Approx. 4 hours |
| - Audio CD | - Audio | - 12 cm | - Approx. 74 minutes |
|  |  | - 8 cm (CD single) | - Approx. 20 minutes |
| - MP3 CD | - Audio | - 12 cm | - Approx. 600+minutes |
| - Picture CD | - Still Images | - 12 cm | - Variable |

# Emos User Guide                                                                                                                                       6

# Setting up the DVD player___________________________________________________________

# Installing the Batteries in the Remote

To operate the DVD player, you must first place the batteries in the remote control.  You can watch movies in comfort without the hassles of manual operation.

### To install the batteries in the remote control:

- Open the battery compartment cover.

- Insert 2 AA batteries.

Match the + and – on the batteries to the marks inside the cover.

- Close the battery cover back in place.

###  Notes on batteries

Improper use of batteries may cause battery leakage and corrosion.

  - Do not insert the batteries in the wrong direction.

  - Do not charge, heat, or open the batteries.

  - Try not to mix different types of batteries together.

  - If you do not use the remote control for a long time, remove the batteries.

 Notes on the remote control

  - Avoid dropping the remote control.

  - Avoid spilling water or placing anything wet on the remote.

  - Do not leave the remote control near any heat source.

You have now prepared your remote control for later use.  On-screen setup using the remote is detailed at the end of this chapter.

________________________________________________________________________________

7                                                                                                                                             Emos User Guide

# __________________________________________________________Setting up the DVD player_

# Connecting the Audio

The sound signal from your DVD player must travel through an audio cable connection.  Newer TV’s have built-in speakers that play back the stereo signals in two audio channels (left and right).  Some older TV’s can only play back monaural or mono sound.  Mono sound has only one channel so each speaker sounds the same.

### To connect for stereo sound:

  - Connect the audio cables to the ANALOG AUDIO L (left-white) and R (right-red) jacks from the DVD player to the corresponding AUDIO IN L and R jacks on your TV.

###  Notes

  - Place two speakers about one metre (three feet) on either side of a TV set to achieve ideal stereo sound.

  - Connect the DVD player directly to your TV.  If you connect it directly to a VCR (video cassette recorder), the picture may distort.  DVD discs are copy protected.

### To connect for mono sound:

  - Connect the audio cable from the ANALOG AUDIO L and R jacks on the DVD player to the single AUDIO IN jack on the TV.  Do not connect the right audio part of the cable.

In this procedure, you have connected your DVD player’s audio to your TV’s speakers.  You are almost finished attaching all the cables needed.  The next section entitled Connecting the Video will add the picture or video.

________________________________________________________________________________

Emos User Guide                                                                                                                                             8

Setting up the DVD player_________________________________________________________________

Audio and video jacks of DVD player

# Connecting the Video

A video signal splits into the three primary colours of red, green and blue (RGB).  The colour and the black-and-white picture information are combined within the RGB signal.

Composite Video combines signals for black-and-white and color components.  Composite Video provides good picture quality.

S-Video provides better picture quality than Composite Video.  The S-Video connection keeps the black-and-white information separate from the colour information.

Component Video offers the highest video quality available on DVD. Component video is a three channel system with superior colour range.

________________________________________________________________________________

9                                                                                                                                  Emos User Guide

____________________________________________________________Setting up the DVD player

# Connecting for Composite Video

The basic video jack (colour-coded yellow) is also known as Composite Video. It combines black-and-white and colour information into a single video signal.  It is the video standard for broadcast TV, VHS and Laserdisc.

### To produce a basic or Composite Video connection:

  - Connect the video cable (yellow) from the VIDEO OUT jack on the DVD player to the VIDEO IN jack on your TV.

 Notes

  - Turn off and unplug the DVD player and the TV before connecting.

# Connecting for S-Video

The S-Video connection keeps the black-and-white information separate from the colour information. The three colour signals of RGB (red, green, blue) combine into a single colour signal.  This produces a better picture than Composite Video.

### To produce an S-Video connection:

  - Connect the S-Video cable from the S-VIDEO OUT jack on the DVD player to the S-VIDEO jack on your TV.

Notes

- Do not connect the yellow cable to the TV or the DVD player.

- Turn off and unplug the DVD player and the TV before connecting.

# Connecting for Component Video

Component video provides superior quality because the video signal splits into three parts.  The Y signal has black-and-white information while the Pb and Pr signals have colour information.  The converted signal is referred to as Y, Pb, Pr.

### To produce a Component Video connection:

  - Use video-grade cables to connect the TV’s component video jacks (Y, Pb, Pr) to the DVD player’s COMPONENT VIDEO OUT (Y jack to Y, Pb to Pb, and Pr to Pr).

### Notes

- It is important to match the colour coded connectors with a compatible TV.

- Turn off and unplug the DVD player and the TV before connecting.

_______________________________________________________________________________

Emos User Guide_                                                                                                                          10

Setting up the DVD player__________________________________________________________________

With all the wires secured firmly in place, you can set up the different features on your DVD player.  To find out more, go to Customizing the Function Settings at the end of this chapter.  To connect components to your player, continue to Connecting to Optional Audio Equipment.

# Connecting to Optional Audio Equipment

Stereo covers about a 120-degree frontal perspective.  You perceive sounds in a much wider perspective (even from behind).

Not many homes are equipped with surround-sound, yet many films are made in this format.  All DVD players on the market today offer Dolby 5.1 Sound.

## Connecting to an Amplifier for Digital Stereo Sound

Digital Stereo is a four channel surround sound used in movie theatres in the 1970’s.  The four channels of sound information convert into two channels.

Enjoy it with an amplifier equipped with digital audio input and a speaker system (right and left front speakers).

### To produce Digital Stereo Sound:

  - Use a digital coaxial cable to connect the DVD player’s COAXIAL jack to the DIGITAL AUDIO IN jack of your amplifier.

Notes

  - Turn off and unplug both units before making any connections.

  - Adjust the amplifier’s volume to a comfortable listening level to avoid speaker damage.

  - If you place the DVD player near a tuner or radio, the radio broadcast may distort.  If this happens, place the DVD player away from these units.

________________________________________________________________________________________

11                                                                                                                                              Emos User Guide

__________________________________________________________________Setting up the DVD player

## Connecting to an Amplifier for Dolby Digital Surround Sound

Surround sound systems reproduce sounds in front of and in back of the listener, resulting in 360 degree sound.  The Dolby 5.1 Digital Surround Sound system has six discrete channels of audio.  The left, center, and right channels are in front while the left and right surround channels are in back.  You need a compatible amplifier, receiver, and speakers to enjoy it.  Playing back true surround sound requires at least five speakers.

### To produce Digital Surround Sound:

  - Connect the DIGITAL OUT jack of the DVD player to the OPTICAL DIGITAL IN jack of the Dolby Digital or DTS receiver using an optical digital cable.

Notes

  - Adjust the volume of your receiver to a moderate listening level to avoid any speaker damage.

  - When connecting or disconnecting the DVD player, turn off the amplifier to avoid speaker damage.

With all the equipment attached and ready, you can start popping the popcorn and selling the tickets!  Your home theatre is open for business.  For some final tweaking, continue to the next section Customizing the Function Settings.

________________________________________________________________________________

Emos User Guide                                                                                                                                            12

Setting up the DVD player__________________________________________________________________

# Customizing the Function Settings

You can customize the DVD player’s performance to your own personal preference.  Use your remote control to access setup menus and to select different options.

### To customize the function settings:

- If you are playing a disc, press STOP.

The Setup Menu does not function during playback.

- To display the Setup Menu Screen, press SETUP.

An on-screen list of General Setup, Video Setup and Preferences appears.

- To highlight a selection in the list, press / , and then press OK.

A second list appears on the right.

- To highlight a selection in the second list, press , and then press .

The first list disappears.

- Press OK to record your choice and resume normal playback.

The second list disappears.

Notes

- To quit the procedure at any time, press the SETUP button.

- This procedure takes care of the main function settings of your DVD player.  You can start watching discs now or after you set the parental controls.  Go to the next section Setting the Parental Control Menu to find out more.

________________________________________________________________________________________

13                                                                                                                                          Emos User Guide

________________________________________________________________Setting up the DVD player

# Setting the Parental Control

You can control the type of content you watch on your DVD player with rating limits and parental control.  DVD discs equipped with the parental control function are rated according to their content.  The content allowed by a rating level and the way a disc is controlled may vary.

Setup Menu Screen

### To set the parental control:

- If you are playing a disc, press STOP.

The Setup Menu does not function during playback.

- To display the Setup Menu, press SETUP.

An on-screen list appears.

- To select Preferences, press /, and then press OK.

The Preferences list appears.

- To select Parental Control, press , and then press OK.

The Enter Password Screen appears.

- Enter a four-digit password of your choosing by using the number buttons.

A Confirm Password Screen appears.

- To confirm your password, press OK.

The Setup Menu disappears.

Notes

  - Enter your password to play unrated discs or to change your control options.

  - Some DVD discs may not respond to parental control settings.

  - To quit the procedure at any time, press the SETUP button.

_________________________________________________________________________________________

Emos User Guide                                                                                                                                      14

Setting up the DVD player__________________________________________________________________

## Setting the Rating Limits

Block discs with an assigned rating (R, 14A, G, etc.) from playing unless you enter a password.  The ratings range from 1 to 8 and include the rating systems used in Canada, USA, and Europe.

### To set the rating limits:

  - If you are playing a disc, press STOP.

The Setup Menu does not function during playback.

  - To display the Setup menu, press SETUP.

An on-screen list appears.

  - To select Preferences, press the /, and then press OK.

The Preferences list appears.

  - To select Rating Limits, press , and then press .

  - Press OK.

The 8-level rating list appears.

  - Select a rating level by pressing  and  to the level you find acceptable.

  - Press OK.

The Enter Password screen appears.

  - Enter your four-digit password with the number buttons, and press OK.

The Setup Menu disappears.

Notes

- Some unrated discs are blocked automatically unless you enter your password.

- To quit the procedure at any time, press the SETUP button.

Now you can sit back and enjoy a movie.  Leave all your worries about child appropriate material behind you and relax.

________________________________________________________________________________15                                                                                                                                           Emos User Guide