July 1998

Internet Solutions Business Unit

Compaq Computer Corporation

Compaq ProLiant servers deliver proven performance for Microsoft Exchange Server customers

Compaq achieved record-breaking Exchange Server scalability by supporting 15,000 users on a ProLiant 7000 with four – 400MHz Intel Pentium II Xeon processors. Using Microsoft’s Load Simulation utility, the ProLiant 7000 with four 400 MHz processors, was tested with user loads of 15,000 simultaneous MAPI e-mail users at Compaq’s Microsoft Competency Center in Redmond, Washington. These favorable results demonstrate the superior performance capabilities of four processor servers based on the Intel Pentium II Xeon processor as compared to eight processor servers based on the Intel Pentium Pro available from competitors.

Compaq and Microsoft have worked to optimize Microsoft Exchange Server performance on Compaq server products to provide an optimal balance between performance, availability, manageability, and cost.  Compaq not only provides world-class server platforms but the experience necessary for the successful deployments of messaging and collaborative applications.

### Compaq ProLiant 7000

The Compaq ProLiant 7000 is the ultimate standards-based server delivering the most scalable performance and the highest levels of availability.  The ProLiant 7000 supports up to four Pentium II Xeon processors and 8GB of system memory.  The architecture supports Enhanced PCI Hot-Plug and a Triple Peer PCI bus design (five 64-bit PCI, four 32-bit PCI and one ISA slot). Standard with the ProLiant 7000 is the 64-bit PCI SMART 3100ES Array Controller that supports 3 Wide Ultra SCSI-3 channels and 64MB of cache.  Leveraging Fibre Channel, The Compaq ProLiant 7000 can support nearly 7TB of external storage.  Combined with the latest high-availability features and processor technology, the ProLiant 7000 is designed for the most demanding and mission-critical applications.

### Exchange Server Performance Test Results

The tests were conducted using Microsoft’s Load Simulator tool. For each test, the Load Simulator Medium MAPI canonical profile was chosen. The Medium MAPI profile reflects the task workload of a typical corporate e-mail user, including common daily mail tasks such as send, browse, read, and forward, as well as scheduling tasks and distribution list usage. The Response Time score represents a 95th-percentile score of the measured test run. The score is expressed in milliseconds (ms). A Response Time score of 1000 ms or less is considered an acceptable response time for e-mail users utilizing Exchange Server’s MAPI protocol.  The tests were completed in accordance with Microsoft’s Exchange Server OEM Benchmarking Policy Guidelines version 1.1.

### Performance Highlights (ProLiant 7000, (4) Pentium II Xeon 400MHz, 4GB RAM)

| User Load | 15,000 |
| --- | --- |
| Response Time (milliseconds) | 343 |
| Messages Submitted (8-hour period) | 213,021 |
| Messages Delivered (8-hour period) | 260,707 |
| Messages Recipients Delivered (8-hour period) | 1,179,107 |
| Messages Sent (8-hour period) | 57,315 |

Note: Complete Test results disclosure can be found in Appendix A.

- compaq ProLiant 7000 Tested configuration

- (4) Pentium II Xeon 400-MHz – 1MB L2 cache per processor

- 4GB RAM

- (1) SMART 3100ES Array Controller with 64MB Cache (configured for 100% WB) - OS/Pagefile/Exchange DS/MTA Files/Exchange Log Files: (2) 4.3-GB Drives – RAID1 - Exchange Information Store Files: (16) 4.3-GB Drives – RAID0

- Compaq Netelligent (100BaseTX) network interface card (NIC)

- Windows NT Server v4.0 Enterprise Edition (using /3GB BOOT.INI switch) - Exchange Server v5.5 – Enterprise Edition

NOTE:  Complete Test results disclosure can be found in Appendix A.

### What the Benchmarks Don’t Tell You

It is important to understand that benchmarks such as these are designed to give Exchange Server implementation planners baseline references for understanding the capabilities of hardware platforms from a single vendor such as Compaq or other competing hardware vendors. When interpreting these benchmarks, two things should be kept in mind.

First, consider whether benchmarks are performed on what can be referred to as customer-deployable configurations. A hardware vendor may publish a result that is based on a platform or configuration that should not be deployed in a real-world Exchange Server deployment. For example, many vendors (including Compaq) publish results using disk subsystems configured with RAID0. While RAID0 does provide the highest levels of disk subsystem performance, it fails to provide any protection against data loss.  Compaq recommends a deploying Exchange Server with disk fault tolerance such as RAID1 or RAID5 for the highest levels of data protection.

Second, most vendors, including Compaq, conduct benchmarks for Exchange Server that are single-server in nature.   Also, keep in mind that benchmarks do not account for issues such as backup and disaster recovery or information store maintenance sizing. Whatever the issue, care must be taken when interpreting benchmarks to ensure that they represent useful information for your Exchange Server deployment and are based on valid simulation methodologies.

While it is significant that the ProLiant 7000 server can successfully scale to 15,000 medium MAPI e-mail users in a single server benchmark exercise, Compaq recommends careful evaluation of all issues involved in real-world Exchange Server deployments such as management, administration, and disaster recovery.

### Load Simulator

The main tool used in generating the workload for this scalability demonstration was the Microsoft Exchange Server Load Simulation utility called Load Simulator. Load Simulator is a tool for simulating a client user load on an Exchange Server. Its purpose is to enable a single Windows NT machine called a LoadSim client to simulate multiple Microsoft Exchange client users.

The operation of Load Simulator users is governed by a Load Simulator profile. This profile controls factors such as how long a Load Simulator "day" is, how many e-mail messages to send in a day’s time, how many times to open and read e-mail, whether to use distribution lists, whether to use public folders, etc.

Load Simulator creates a highly accurate simulation of reality. It mimics the full Microsoft Exchange Client in many respects. First, it uses .MSG files, the same format used by the Exchange Client. This guarantees that messages generated by Load Simulator have the same properties as those sent by real users of the Exchange Client. Second, Load Simulator uses the same MAPI remote procedure call (RPC) semantics as those used by the Client. Third, Load Simulator registers MAPI change notifications in the same manner as they are registered by the Client. Finally, Load Simulator even emulates the Microsoft Exchange Client list box cache, which the Client uses for folder and message panes in the viewer when a user browses and selects messages on the server.

Several steps are necessary to performing a successful simulation. The Load Simulator setup and initialization process comes first. Load Simulator first, creates the test topology by first generating the user directory entries. Next, the test store is initialized and populated with the test messages and folder items. The tests are typically run for up to 8 hours depending upon the user load simulated and amount of time required to reach a steady state for measurement purposes. During a test run, users log on to the Exchange Server and begin processing various messaging tasks. Task response time data is logged to the LSPERF.LOG file and client messages and error logging are stored in the LOADSIM.OUT file. To produce test scores, the LSLOG utility is used to parse out the LSPERF.LOG file and calculate the response time score. By default, 95th- and 50th-percentile response time scores are calculated. Ninety-fifth-percentile response time scores for the MAPI/RPC protocol should be less than 1000 ms, according to Microsoft. Also, the Exchange Server IS Send Queue and the MTA Work Queue (other message and delivery queues should also be considered, depending on the protocol) must consistently return to near zero during the steady-state period for which test measurements are taken. Queues that continue to grow and fail to return to near zero indicate that the server is not sustaining the required workload.  There should be no errors logged by the LoadSim clients during the test. When these conditions are met, a successful test run has been completed. For more information on LoadSim Medium canonical profiles, please refer to the LoadSim documentation at http://www.microsoft.com/exchange/library/loadsim55x86.exe.

# APPENDIX A: TEST DISCLOSURE

| LoadSim Clients | Configuration |
| --- | --- |
| Network Topology (100Base T, Token Ring, etc.) | 100 Base-TX |
| Number and type of clients | (40) 2x5/133, 128 MB RAM ( 400 users each) or better (indicates minimum configuration) |
| Number and type of hubs/concentrators (full duplex, switching, etc.) | Compaq Netelligent 5708 Switch and Netelligent 2624 Hubs |
| Number of clients/segment | 20 |
| Client CPU type and speed in percentages | 2P/133-MHz Pentium processors or better |
| Client network controller broken down by percentages | Compaq Netelligent 10/100 |
| Client network software name and version (drivers, protocols, redirector) | Microsoft Windows NT Workstation 4.0 with SP3 TCP/IP |
| Size of any client network cache | None |
| Network controller software | Compaq Netelligent 10/100 driver |
| LoadSim version | 5.5 (Build 2187) |

NOTE: Response time measurements were taken from a LoadSim Control Client simulating 100 users configured with 96 MB RAM and a Pentium/166 CPU. The client is located on an isolated network segment connected to a 100-Mb/s switch.

### Benchmark Tuning

The table below shows tuning parameters for Exchange Server that varied from default settings for purposes of the benchmark exercise.  Compaq recommends using the default settings specified by the Microsoft Exchange Performance Optimizer for production server deployments.

| Parameter | Default Setting | Benchmark Setting |
| --- | --- | --- |
| IS Buffers (4K each) | 524,000 | 375,000 |
| Max Open Tables | 32,000 | 200,000 |
| IS Send Threads | 4 | 8 |
| IS Deliver Threads | 4 | 8 |
| MTA Submit/Deliver Threads | 2 | 8 |
| MTA TCP Threads | 2 | 6 |
| MTA TCP Control Blocks | 20 | 25 |

### Version Information (Server)

| Name | Type | Version or Date |
| --- | --- | --- |
| Compaq NT SSD | Software/Drivers | 2.09 |
| SMART ECI Driver | Software/Drivers | 1.06 |
| System Firmware | ROM | 7/8/98 |
| SMART 3100ES Firmware | ROM | 2.72 |
| Windows NT Server EE | Operating System | 4.0 SP3 |
| Exchange Server EE | Application | 5.5 (Build 1960.8) |

### Performance Data Disclosure

15,000 Users (Measured during test run at steady state)

| Summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Supported Benchmark Load |  |  |  | 15,000 Users |  |
| Weighted 95th Percentile Score |  |  |  | 343 milliseconds |  |
| Benchmark Profile |  |  |  | Loadsim Medium User |  |
| Protocol |  |  |  | Exchange MAPI |  |
| Length of Steady State |  |  |  | 4 hours |  |
| Length of Test |  |  |  | 8 hours |  |
| Transaction Load (hourly) |  |  |  |  |  |
| Messages Submitted |  |  |  | 26627.69 |  |
| Message Recipients Delivered |  |  |  | 147388.46 |  |
| Messages Sent |  |  |  | 7164.49 |  |
| Transaction Load (per Second) |  |  |  |  |  |
| Message Opens/Sec |  |  |  | 53.75 |  |
| Folder Opens/Sec |  |  |  | 14.05 |  |
| RPC Read Bytes/Sec |  |  |  | 47250.31 |  |
| RPC Write Bytes/Sec |  |  |  | 383111.15 |  |
| Transaction Queues |  |  |  |  |  |
| IS Send Queue Average Length |  |  |  | 5.50 |  |
| MTA Work Queue Average Length |  |  |  | 8.04 |  |
| Processor Utilization |  |  |  |  |  |
| System Processor Utilization (%) |  |  |  | 83.74% |  |
| System Processor Queue Length |  |  |  | 4.29 |  |
| System Context Switches/Sec |  |  |  | 5438 |  |
| Process % CPU Time - Store |  |  |  | 272.33% |  |
| Process % CPU Time - DS |  |  |  | 25.38% |  |
| Process % CPU Time - MTA |  |  |  | 15.91% |  |
| Memory Utilization |  |  |  |  |  |
| Available Bytes |  |  |  | 1838MB |  |
| Pages/Sec |  |  |  | 0.015 |  |
| Process Working Set Bytes - Store |  |  |  | 1.99GB |  |
| Process Virtual Bytes - Store |  |  |  | 2.78GB |  |
| Logical Drive Utilization |  |  |  |  |  |
| IS Database Disk Reads/Sec |  |  |  | 409.40 |  |
| IS Database Disk Writes/Sec |  |  |  | 244.01 |  |
| IS Database Average Disk Queue Length |  |  |  | 11.36 |  |
| IS Log Disk Reads/Sec |  |  |  | 0.93 |  |
| IS Log Disk Writes/Sec |  |  |  | 159.82 |  |
| IS Log Average Disk Queue Length |  |  |  | 0.34 |  |

NOTE: Performance results measured using Microsoft NT Performance Monitor. Measurements were obtained by measuring averages for the period of steady-state activity (i.e. after 15, 000 users were successfully logged on). Tests measure the messaging throughput of a single-server, single-site topology. For deployment-specific information contact a Microsoft or Compaq representative. More information can be found at: http://www.microsoft.com/exchange/support/deployment/planning/deploy.asp?A=5&B=1

## User Response Times (Latencies) from Load Simulator

| Client Actions |  | ### 95th Percentile Response Time (in Milliseconds) |  |  |
| --- | --- | --- | --- | --- |
| Read | 260 |
| Send |  | 531 |
| Delete | 421 |
| Move | 550 |
| Submit |  | 441 |

# APPENDIX B: RELATED DOCUMENTS

These documents are available on the Compaq website.

Compaq and Microsoft Demonstrate Enterprise Scalability with Exchange Server 5.5,

http://www.compaq.com/support/techpubs/whitepapers/ECG00961197.html

Microsoft Exchange Server 5.5 on the Compaq ProLiant 850R,

http://www.compaq.com/support/techpubs/whitepapers/ECG0710698.html

Microsoft Exchange Server 5.5 on the Compaq ProLiant 3000,

http://www.compaq.com/support/techpubs/whitepapers/ECG0720698.html

Microsoft Exchange Server 5.5 on the Compaq ProLiant 6000 Class Servers,

http://www.compaq.com/support/techpubs/whitepapers/ECG0730698.html

Compaq Deployment and Configuration Guide: Microsoft Exchange Server on Compaq ProLiant Servers,

http://vcmproapp02.compaq.com/

Performance of Exchange Server 4.0 on Compaq ProLiant Servers,

http://www.compaq.com/support/techpubs/whitepapers/444A0696.html

“Deschutes” Family Processor Technology,

http://www.compaq.com/support/techpubs/whitepapers/ecg0500698.html

Disk Subsystem Performance and Scalability,

http://www.compaq.com/support/techpubs/whitepapers/ECG0250997.html

Configuring Compaq RAID Technology for Database Servers,

http://www.compaq.com/support/techpubs/technotes/184206-1html

Compaq SMART Array Controller Technology,

http://www.compaq.com/support/techpubs/whitepapers/667A0697.html

Hardware vs. Software Fault Tolerance,

http://www.compaq.com/support/techpubs/whitepapers/ECG0660298.html

Compaq Pentium Pro Processor-based Servers,

http://www.compaq.com/support/techpubs/whitepapers/308A0496.html

Configuring the Compaq ProLiant 5000 Server for Peak Performance,

http://www.compaq.com/support/techpubs/whitepapers/679A0697.html

Compaq White Paper Index,

http://www.compaq.com/support/techpubs/whitepapers

Notice

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

.

The information in this publication is subject to change without notice.

COMPAQ COMPUTER CORPORATION SHALL NOT BE LIABLE FOR TECHNICAL OR EDITORIAL ERRORS OR OMISSIONS CONTAINED HEREIN, NOR FOR DIRECT, INCIDENTAL OR CONSEQUENTIAL DAMAGES RESULTING FROM THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL.

This publication does not constitute an endorsement of the product or products that were tested. The configuration or configurations tested or described may or may not be the only available solution. This test is not a determination of product quality or correctness, nor does it ensure compliance with any federal, state or local requirements. Compaq does not warrant products other than its own strictly as stated in Compaq product warranties.

Compaq, Contura, Deskpro, Fastart, Compaq Insight Manager, LTE, PageMarq, Systempro, Systempro/LT, ProLiant, TwinTray, LicensePaq, QVision, SLT, ProLinea, SmartStart, SMART-2/DH, NetFlex, DirectPlus, QuickFind, RemotePaq, BackPaq, TechPaq, SpeedPaq, QuickBack, PaqFax, registered United States Patent and Trademark Office.

Aero, Concerto, QuickChoice, ProSignia, Systempro/XL, Net1, SilentCool, LTE Elite, Presario, SmartStation, MiniStation, Vocalyst, PageMate, SoftPaq, FirstPaq, SolutionPaq, EasyPoint, EZ Help, MaxLight, MultiLock, QuickBlank, QuickLock, and UltraView, CompaqCare and the Innovate logo, are trademarks and/or service marks of Compaq Computer Corporation.

Other product and company names mentioned herein may be trademarks and/or registered trademarks of their respective companies.

©1998 Compaq Computer Corporation. Printed in the U.S.A.

Microsoft, Windows, Windows NT, Windows NT Enterprise Edition, and Exchange Server are trademarks and/or registered trademarks of Microsoft Corporation.

Intel, Pentium, Pentium II, Xeon, and Pentium Pro are trademarks and/or registered trademarks of Intel Corporation.

Cheyenne, ArcServe and Computer Associates are the trademarks and/or registered trademarks of Computer Associates, Inc.

Microsoft Exchange Server 5.5 On the Compaq ProLiant 7000 - Pentium II Xeon Server

First Edition (July 1998)

Document Number: ECG0XXXXXX